August 22, 2023

Version 1.0

2D Base Level Engineering Report

Lower Canadian – Deer Watershed (HUC 8 11090201)

Report Prepared by:

Compass PTS JV

3101 Wilson Boulevard, Suite 900

Arlington, VA 22201

Report Prepared for:

FEMA (Federal Emergency Management Agency)

Region 6, Risk Analysis Branch

FRC 800 North Loop 288

Denton, TX 76209-3698

Document History

Document Location

K:/FY2021/21-06-0060S/Hydraulics - Lower Canadian - Deer (Dewey County, OK)|BLE (FY21) 03/Hydraulic Data Capture - Hydraulic Data Capture|Lower Canadian - Deer (Dewey County, OK) - 01

Revision History

| Version Number | Version Date | Summary of Changes | Author |
| --- | --- | --- | --- |
| 1.0 | August 2023 | Deliverable to FEMA Region VI | Compass PTS JV |

Table of Contents

Executive Summary	1

2D Base Level Engineering (BLE) Methodology	3

Topographic Data	5

Inventory	5

Evaluation	5

Lower Canadian – Deer Watershed Source Terrain Data	8

Data Development Methodology	8

DEM Quality Assurance/Quality Control (QA/QC)	8

2D BLE Parameters	10

2D Computational Mesh and Settings	11

Model and Boundary Condition Setup	11

Hydrology	13

Hydraulics	22

Model Results	23

Floodplain Mapping	25

Model Outputs	25

Methodology	25

Flood Hazard Area Layer	25

CNMS Validation	26

Initial Assessment A1 – Significant Topography Update Check	26

Initial Assessment A2 – Check for Significant Hydrology Changes	26

Initial Assessment A3 – Check for Significant Development	26

Validation Check A4 – Check of Studies Backed by Technical Data	27

Validation Check A5 – Comparison of BLE and Effective Zone A	27

Validation Results	28

Flood Risk Assessment	32

Quality Assurace & Quality Control	32

Deliverables	33

Considerations	33

Challenges	33

Recommendations	34

References	35

Appendix A – Exhibits – Base Level Engineering Results	36

# Executive Summary

FEMA Region 6 contracted Compass to complete a Base Level Engineering (BLE) analysis for the Lower Canadian Deer Watershed in Oklahoma and Texas to support FEMA's Discovery process and validation of effective Zone A Special Flood Hazard Areas (SFHAs). The BLE process involves using the best available data and incorporating automated techniques with traditional model development procedures to produce regulatory-quality flood hazard boundaries for the 1-percent annual chance event and estimate flood hazard boundaries for multiple recurrence intervals.

The source digital terrain data used for surface model development in support of hydrologic and hydraulic analysis as well as mapping activities were leveraged from various local, State, and Federal partners. Details regarding the different datasets used are provided in the respective sections throughout this report.

Flood discharges for this study were calibrated using both the United States Geological Survey (USGS) regression equations and gage analysis, where stream gages with sufficient record exist. Regression equation discharges were obtained using the USGS StreamStats online application for locations in Oklahoma.

The Hydrologic Engineering Center’s River Analysis System (HEC-RAS) program version 6.1.0 was used to compute water surface elevations using 2D analysis.

The stream mile network that was validated for this watershed was compiled using FEMA’s Community Needs Management Strategy (CNMS) inventory. CNMS is an inventory of flood hazard studies and flood hazard mapping needs for areas where a study is needed. This data is helpful for community officials in analyzing and depicting flood hazards to enhance the understanding of flood risks. Communities may use this information to make informed decisions on their planning and flood mitigation efforts. Table 1 lists the Zone A stream miles associated with this validation analysis.

Table 1: Summary of Stream Miles

| Source | Stream Miles |
| --- | --- |
| Current Inventory – CNMS (S_Studies_Ln) | 2,041.4 |
| New Study Streams – CNMS (S_Unmapped_Ln) Informed by NHD 1:100 | 665.4 |
| Total (BLE) Stream Miles | 2,706.8 |

The full inventory of Zone A studies in the watershed were classified in CNMS. Total miles validated in CNMS are summarized in Table 2 and illustrated in Figure 1 below.

Table 2: Zone A Validation Results

| Validation Status | CNMS Validation Status | CNMS Project Start | CNMS Post-BLE Assessment/Total Miles |
| --- | --- | --- | --- |
| VALID | BEING STUDIED | 65 | 0 |
| UNVERIFIED | BEING STUDIED | 185.4 | 250.4 |

Figure 1: Lower Canadian Deer - CNMS Validation Results

# 2D Base Level Engineering (BLE) Methodology

Recent innovations and efficiencies in floodplain mapping have allowed the U.S. Department of Homeland Security’s Federal Emergency Management Agency (FEMA) to develop a process formerly known as First Order Approximation (FOA), now labeled Base Level Engineering (BLE), which can be used to address current program challenges, including the validation of Zone A studies and the availability of flood risk data in the early stages of a Flood Risk Project. The BLE process involves using best available data and automated techniques to produce estimates of flood hazard boundaries for multiple recurrence intervals.  The Lower Canadian – Deer Watershed BLE documented here was designed to use 2-dimensional (2D) modeling efforts with enhancements and calibration to develop products intended to be transitioned into regulatory data development workflows.

As described in Title 42 of the Code of Federal Regulations, Chapter III, Section 4101(e), once every five years, FEMA must evaluate whether the information on Flood Insurance Rate Maps (FIRMs) reflects the current risks in flood prone areas. FEMA makes this determination of flood hazard data validity by examining flood study attributes and change characteristics, as specified in the Validation Checklist of the Coordinated Needs Management Strategy (CNMS) Technical Reference. The CNMS Validation Checklist provides a series of critical and secondary checks to determine the validity of flood hazard areas studied by detailed methods (e.g., Zone AE, AH, or AO). While the critical and secondary elements in CNMS provide a comprehensive method of evaluating the validity of Zone AE studies, a cost-effective approach for evaluating Zone A studies has been lacking.

In addition to the need for Zone A validation guidance, FEMA standards require flood risk data to be provided in the early stages of a Flood Risk Project.  FEMA Program Standard Identification (SID) #29 requires that during Discovery, data must be identified that illustrates potential changes in flood elevation and mapping which may result from the proposed project scope.  If available data does not clearly illustrate the likely changes, an analysis is required that estimates the likely changes.  This data and any associated analyses should be shared, and results should be discussed with stakeholders.

An important goal of the BLE process is the scalability of the results.  Scalability means that the results of a BLE should not only be used for CNMS evaluations of Zone A studies but can also be leveraged throughout the Risk Mapping, Assessment and Planning (Risk MAP) program. The large volume of data resulting from a BLE can be updated as needed and used for the eventual production of regulatory and non-regulatory products, outreach and risk communication, and MT-1 processing.  Leveraging this data outside the Risk MAP program may also be valuable to external stakeholders.

In an effort to increase and enhance the flood risk products in Texas and Oklahoma, FEMA Region VI contracted the Compass PTS JV to perform a BLE analysis for the Lower Canadian – Deer Watershed. This report documents the BLE process, products, and results for this watershed.  Figure 2 depicts the Lower Canadian – Deer Watershed footprint.

Figure 2: Lower Canadian – Deer HUC 8

The BLE assessment prepared Zone A analyses for approximately 2,050 miles of stream reaches within the Lower Canadian - Deer Watershed with a minimum drainage area tolerance of one square mile. The selection and extent of stream reaches studied was based upon the number of stream miles with minimum drainage area of one square mile AND the number of effective Zone A stream miles. Study reaches were extended above this one square mile threshold as appropriate to ensure all effective Zone A floodplain received an updated analysis. The following sections will summarize the BLE process and will discuss the results along with their recommended use.

## Topographic Data

A high-resolution Digital Elevation Model (DEM) is a fundamental component for 2D engineering analyses by providing a detailed representation of the surface for hydraulic routing through the model area.  As such, DEMs were developed for the Lower Canadian – Deer BLE project by leveraging available high-resolution gridded elevation data derived from Light Detection and Ranging (LiDAR) collections throughout the entire State of Oklahoma and Texas.  The 10-foot DEM developed to support the 2D BLE modeling and analysis, within the Lower Canadian – Deer Watershed, was executed using the following steps:

- Available elevation data for the project area were inventoried and collected.

- Leveraged elevation data were evaluated and prioritized based on source vertical accuracy, year of collection and resolution.

- Seamless DEMs were processed using GIS.

- Quality was assured using quantitative and qualitative assessment.

Documentation regarding leveraged data including coverage, accuracy, acquisition dates, and source contact/agency are presented in the figures, tables and text below.  All vertical accuracy specifications were obtained from the metadata or survey reports provided with the leverage datasets.  All available metadata, survey reports, and other leverage documentation are included in the FEMA Data Capture Technical Reference compliant submittal content for the Lower Canadian – Deer Watershed.

### Inventory

An inventory of existing topographic data was conducted for the Lower Canadian – Deer BLE project footprint.  Figure 3  depicts the datasets identified for leverage across the project area.  FEMA, National Oceanic and Atmospheric Administration (NOAA), USGS, and other State and Federal agencies were queried to build the inventory with the most current and available data sources.

### Evaluation

A data coverage assessment was conducted to check for data gaps, extent, accuracy, and completeness.  A review of related documentation, reports, indexes, and metadata associated with the leveraged datasets ensured each dataset meets FEMA accuracy requirements for topographic data.  Decisions to leverage or exclude a dataset (or portion of it), were based generally on the following criteria coupled with engineering judgment:

- Data meet FEMA vertical accuracy standards

- Date of origination

- Data density and coverage

Table 3 depicts the Risk MAP SID #42 vertical accuracy requirements based on flood risk and terrain slope within the floodplain being mapped.

Table 3:  FEMA Vertical Accuracy Requirements for Leveraged Data

| Level of Flood Risk | Typical Slopes | Specification Level | Vertical Accuracy* | LiDAR Nominal Pulse Spacing (NPS) |
| --- | --- | --- | --- | --- |
| High (Deciles 1,2,3) | Flattest | Highest | 24.5 cm / 36.3 cm | ≤ 2 meters |
| High (Deciles 1,2,3) | Rolling or Hilly | High | 49.0 cm / 72.6 cm | ≤ 2 meters |
| High (Deciles 2,3,4,5) | Hilly | Medium | 98.0 cm / 145 cm | ≤ 3.5 meters |
| Medium (Deciles 3,4,5,6,7) | Flattest | High | 49.0 cm / 72.6 cm | ≤ 2 meters |
| Medium (Deciles 3,4,5,6,7) | Rolling | Medium | 98.0 cm / 145 cm | ≤ 3.5 meters |
| Medium (Deciles 3,4,5,6,7) | Hilly | Low | 147 cm / 218 cm | ≤ 5 meters |
| Low (Deciles 7,8,9,10) | All | Low | 147 cm / 218 cm | ≤ 5 meters |

*Vertical Accuracy at 95% Confidence Level (FVA or NVA)/(CVA or VVA)

Table 4 depicts the complete list of source elevation data and attributes leveraged for the Lower Canadian – Deer Watershed BLE project.  All datasets used for hydraulic analyses and mapping meet the highest specification level defined.  Further explanation of the datasets can be referenced below.

Table 4: LiDAR Data Sources

| Description | Year | Accuracy | Source/ Contact | Approximate Footprint (mi2) |
| --- | --- | --- | --- | --- |
| FEMA Texas Panhandle LiDAR | 2018 | 4.3 cm | FEMA | 793.9 |
| FEMA Region 6 OK – Woodward LiDAR | 2017 | 4.7 cm | FEMA | 729.7 |
| 2012 OK LiDAR – NRCS Priority Project | 2012 | 18.5 cm | NRCS | 514.1 |
| 2010 NRCS – Washita River Plus, OK | 2010 | 18.5 cm | NRCS | 0.04 |

Figure 3: Extent of LiDAR Data

### Lower Canadian – Deer Watershed Source Terrain Data

The source elevation data for the Lower Canadian – Deer Watershed are DEMs derived from the the 2018 FEMA Texas Panhandle LiDAR dataset; the 2017 FEMA Region 6 OK – Woodward LiDAR dataset; the 2012 OK LiDAR – NRCS Priority Project; and the 2010 NRCS – Washita River Plus, OK.  Only points classified as “ground” points (i.e., bare earth) were imported from the LiDAR and used for development of the project DEMs.  Bare-earth LiDAR data are typically made by filtering non-ground returns (e.g. buildings, vegetation, etc.) from the raw laser returns.  Figure 3 depicts the extent of the data defined in Table 4. The 2018 FEMA Texas Panhandle LiDAR was collected with a nominal pulse spacing of 0.7 meters. The data was compiled to meet 19.6 cm vertical accuracy at 95% confidence level (RMSE * 1.96).  The 2017  FEMA Region 6 OK – Woodward LiDAR was collected with a nominal pulse spacing of 0.35 meters. The data was compiled to meet 19.6 cm vertical accuracy at 95% confidence level (RMSE * 1.96). The 2012 OK LiDAR – NRCS Priority Project was collected with a nominal pulse spacing of 1.4 meters. The data was compiled to meet 19.6 cm vertical accuracy at 95% confidence level (RMSE * 1.96). The 2010 NRCS – Washita River Plus, OK LiDAR was collected with a nomial pulse spacing of 1.4 meters. The data was compiled to meet 19.6 cm vertical accuracy at 95% confidence level (RMSE * 1.96).

### Data Development Methodology

The source topographic data were processed for an area covering the Lower Canadian – Deer Watershed and contributing drainage areas for the Lower Canadian – Deer BLE modeling efforts.  The topographic data for Lower Canadian – Deer Watershed was projected horizontally, as needed, to North American Datum of 1983 (NAD83), State Plane Coordinate System (SPCS) Oklahoma North in feet (3501-SPC83).  All topographic data were adjusted vertically, as needed, to North American Vertical Datum of 1988 (NAVD88) in feet.  Compass used a combination of ArcGIS and other software tools to apply any vertical datum shifts and/or any horizontal projection transformations to the topographic data.

### DEM Quality Assurance/Quality Control (QA/QC)

DEMs developed for use in the Lower Canadian – Deer Watershed BLE analysis were developed and independent assured to meet quality standards of the project.  The data were developed using a controlled process, were evaluated and assured by a topographic data development team and were evaluated and assured by the engineering team. QA during the data development process includes, but is not limited to the following QC checks:

•	Horizontal Projection Check

•	Vertical Datum Check

•	Resolution Check

•	Format Check

•	Seamless Data Check to ensure the DEM files are consistent and seamless along source data edges

The QC after the development process by the DEM development team included visual observations using hillshade, contouring, color rendering, and/or other visual aids to review and identify potential impactful anomalies within the DEM surface.  This QC step included, but were not limited to the following QC checks:

•	Seamless Data Check to ensure no voids along the edges and between the prioritized datasets

•	NoData Value Check to ensure no null values

•	Manual Elevation Check using hillshade rasters to find erroneous elevation issues

•	Unit Consistency Check

•	Legacy Cell Value Anomalies

QA conducted after the seamless DEM development conducted by the engineering team included visual or automated assessments to identify potentially impactful anomalies or slope changes that may adversely impact hydraulic calculations.

The final DEM data developed for the Lower Canadian – Deer Watershed is assured to meet FEMA standards and present a representative surface developed from leveraged elevation data for the purposes of this BLE project.

## 2D BLE Parameters

The following sections describe the 2D computational mesh and program setting considerations, followed by discussion and tabulation of hydrologic and hydraulic engineering methods and model inputs. HEC-RAS version 6.1.0 was used for this project. This HUC 8, as shown in Figure 4, was split into four modeling areas based on HUC 10 watersheds (Town of Arnett – Canadian River, Town Camargo – Canadian River, Town of Taloga – Canadian River, and Deer Creek).

Figure 4:  HEC-RAS 2D BLE Model Areas

### 2D Computational Mesh and Settings

The HEC-RAS 2D computational mesh defines the extents of the 2D flow and can affect the accuracy of the 2D calculations.  A denser mesh may provide more accurate results, but it can dramatically increase computation times. The 2D mesh for the models was set as evenly spaced cells at 200 feet.  The mesh was further refined by placing breaklines along roads, berms, ridges, and other high ground that can influence the flow. To ensure the smooth mapping and tie ins, a 500-foot to 1,000-foot buffer was added to the HUC boundary.

The HEC-RAS 2D computational meshes were created for the Lower Canadian – Deer models based on HUC boundaries using ArcGIS toolsets, such as smoothing and simplification routines; ultimately, significantly reducing the need for manual edits to mesh cells within HEC-RAS that happen to generate errors. The 2D mesh count for each work area is outline below in Table 5, with a 200-foot nominal mesh cell size for all; there are factors that could result in either larger or smaller cell sizes including proximity to the edge of the 2D mesh or the presence of breaklines.  An adaptive time step based on a maximum Courant of 1 and minimum Courant of 0.4 was used in the HEC-RAS model, applying the Diffusion Wave (simplified Full Momentum) equations.

Table 5: Work Area Cell Count

| Work Area | Cell Count |
| --- | --- |
| Town of Arnett – Canadian River | 494,562 |
| Town of Camargo – Canadian River | 336,100 |
| Town of Taloga – Canadian River | 537,748 |
| Deer Creek | 544,236 |

### Model and Boundary Condition Setup

Using HEC-RAS rain-on-grid modeling requires establishing a 2D computational mesh boundary. For this project, precipation was applied to the mesh and losses were calculated within HEC-RAS using the Soil Conservation Service (SCS) Curve Number (CN) method.

Outflow boundary conditions (from the computational 2D mesh) were utilized along basin boundaries using normal depths.  Unique outflow boundaries were established for obvious riverine outflows, while the remaining boundaries were defined as continuous boundaries to allow drainage from adjacent basins to leave the model area freely. Normal depth was used for all non-unique outflow boundary conditions using approximate energy grade-line slopes estimated from the LiDAR terrain data. Figure 5 shows the mesh of each model area along with the gages in this study area.

Figure 5:  HEC-RAS 2D Computational Mesh and USGS Peak Streamflow Gages

## Hydrology

Precipitation data for this study were referenced from NOAA’s Precipitation Frequency Data Server (PFDS), and appropriate values for all sub-basin elements were determined considering spatial variation. USGS streamflow gage data were leveraged to develop the inflow hydrograph for the Town of Arnett  – Canadian River model (USGS gage 07228000) and to estimate peak flows for downstream gages within each work area for calibration purposes.

The gages utilized for this study are presented in Table 6. Gage 07228280 was not utilized in this study because of its minimal period of record. For Gage 07228290, the gage analysis was unable to calculate a reasonable 1% peak flow value compared to the USGS published results. This gage also had a relatively short, (22 year), length of record, and the the data was older (1964-1985). Because of these factors, a weighted average of the gage value and streamstats value was used for calibration.

Table 6:  USGS Peak Streamflow Gages

| Gage ID | Flooding Source and Location | Published Drainage Area (mi2) | Period of Record |
| --- | --- | --- | --- |
| 07228290 | Rough Creek near Thomas, OK | 10.4 | 1964-1985 |
| 07228450 | Deer Creek Trib near Hydro, OK | 2.31 | 1964-1975 |

Annual chance peak flows were calculated at each gage using USGS Bulletin 17C methodology.  The 84% confidence interval was used to determine the 1% plus and minus chance events.  Calculated discharges for the 1%, 1% plus, and 1% minus events are presented in Table 7 for each gage utilized in this study

Table 7: USGS Peak Streamflow Gage Analysis Results

| Gage ID | Flooding Source and Location | 1% Peak Q (cfs) | 1%- Peak Q (cfs) | 1%+ Peak Q (cfs) |
| --- | --- | --- | --- | --- |
| 07228290 | Rough Creek near Thomas, OK | 25,000* | 13,440 | 61,740 |
| 07228450 | Deer Creek Trib near Hydro, OK | 1,800 | 1,219 | 3,362 |
| *Gage analysis value |  |  |  |  |

#### Excess Precipitation for 2D Computational Mesh

Precipitation frequency data published in NOAA Atlas 14, Volume 9 Version 2.0 (NOAA, 2013) were used for this study.  These data were obtained from NOAA’s PFDS (http://hdsc.nws.noaa.gov/hdsc/pfds/index.html). The 10%, 4%, 2%, 1%, and 0.2% annual chance 24-hour precipitation depths for all sub-basin elements were determined by area-weighted intersection of gridded PFDS data with sub-basins.

In addition to recurrence interval precipitation estimates, NOAA Atlas 14 provides 90% confidence intervals of reported precipitation values.  On a normal distribution curve, 90% confidence intervals (i.e. the upper and lower 95% confidence limits) correspond to +/- 1.645 standard deviations.  The 1% plus and minus events are defined to be one standard deviation above and below the 1% event.

The SCS CN method was used to calculate losses and define excess precipitation for these watersheds. Curve numbers were computed by intersecting the National Land Cover Dataset (NLCD) 2019 coverage and Natural Resources Conservation Service (NRCS) soils data based on land cover and soils matrices.

During the calibration of the model, curve numbers were adjusted to achieve flows that are closer to regression equation and flood frequency analysis estimates. Initial CNs for this study are provided in Table 8.

Table 8:  Land Use-Soils-CN Matrix for Computing Initial Curve Numbers

| LU_GridCode | NLCD LU Description | Hydrologic Soil Group |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | A | B | C | D |
| 11 | Open Water | 96.6 | 96.6 | 96.6 | 96.6 |
| 21 | Developed Open Space | 38.9 | 59.6 | 71.1 | 77.2 |
| 22 | Developed Low Intensity | 50.9 | 66.4 | 75.9 | 80.9 |
| 23 | Developed Medium Intensity | 73.5 | 82.2 | 86.0 | 88.6 |
| 24 | Developed High Intensity | 83.4 | 87.3 | 89.9 | 91.2 |
| 31 | Barren Land | 38.9 | 59.6 | 71.1 | 77.2 |
| 41 | Deciduous Forest | 22.2 | 37.9 | 46.8 | 53.0 |
| 42 | Evergreen Forest | 26.3 | 47.8 | 64.1 | 72.3 |
| 43 | Mixed Forest | 27.1 | 49.8 | 64.1 | 71.1 |
| 52 | Shrub Scrub | 26.3 | 45.7 | 60.7 | 68.7 |
| 71 | Herbaceous | 44.7 | 61.8 | 73.5 | 83.4 |
| 81 | Hay Pasture | 38.9 | 59.6 | 71.1 | 77.2 |
| 82 | Cultivated Crops | 53.0 | 64.1 | 72.3 | 75.9 |
| 90 | Woody Wetlands | 62.9 | 72.3 | 80.9 | 88.6 |
| 95 | Emergent Herbaceous Wetlands | 62.9 | 72.3 | 80.9 | 88.6 |

NRCS rainfall-runoff methods were used to define excess precipitation applied to the 2D mesh, including CNs for defining rainfall losses. The precipitation was applied using a set of nested frequency distributions published by the NRCS based on Atlas 14 statistical data; all work areas were within the MSE3, MSE4 and TX C temporal regions. Table 9 provides the precipitation totals used for determining excess precipitation.

Table 9: Precipitation Totals for HEC-RAS 2D Computational Mesh (post ARF)

| Sub-basin Description | Percent Annual Chance Precipitation Total (in) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 4 | 2 | 1 | 0.2 | 1% Minus | 1% Plus |
| Town of Arnett – Canadian River | 4.3 | 5.2 | 6.0 | 6.8 | 9.0 | 5.7 | 8.1 |
| Town of Camargo – Canadian River | 4.5 | 5.5 | 6.3 | 7.1 | 9.4 | 6.0 | 8.5 |
| Town of Taloga – Canadian River | 4.7 | 5.9 | 6.8 | 7.8 | 10.4 | 6.6 | 9.2 |
| Deer Creek | 4.9 | 6.1 | 7.1 | 8.1 | 11.0 | 6.9 | 9.6 |

Figure 6, Figure 7, Figure 8 and Figure 9 show the hyetographs applied to the 2D computational mesh.

Figure 6: Precipitation Hyetographs applied to the Town of Arnett – Canadian River Computational Mesh

Figure 7: Precipitation Hyetographs applied to the Town of Camargo – Canadian River Computational Mesh

Figure 8: Precipitation Hyetographs applied to the Town of Taloga – Canadian River Computational Mesh

Figure 9: Precipitation Hyetographs applied to the Deer Creek Computational Mesh

#### Hydrograph Boundary Conditions

The Town of Arnett – Canadian River model utilized an inflow hydrograph (

Figure 10), which was estimated based on data recorded by USGS Gage 07228000. A dimensionless unit hydrograph was estimated based on historical storm events, which was further scaled and interpolated to estimate the respective inflow for various recurrence intervals and to accommodate the distance between the gage location and the model inflow boundary condition. Stage hydrographs were used as the inflow into the Town of Camargo – Canadian River and Town of Taloga – Canadian River models while a flow hydrograph was utilized for the Deer Creek model to allow for water surface elevations to be within 0.5 feet for each event. These hydrographs are shown in Figure 11, Figure 12, and Figure 13.

Figure 10:  Flow Hydrograph for Town of Arnett – Canadian River

Figure 11:  Stage Hydrograph for Town of Camargo – Canadian River

Figure 12:  Stage Hydrograph for Town of Taloga – Canadian River

Figure 13:  Flow Hydrograph for Deer Creek

## Hydraulics

This section describes the remaining hydraulic modeling considerations, including implementation of Manning’s roughness, breaklines, and hydraulics structures within the 2D computational mesh.

#### Roughness Coefficients

A Manning’s n roughness coverage was developed for the 2D computational mesh using typical values for roughness for given NLCD land classifications. A buffer zone was created around each stream centerline to create a main channel roughness zone.  The channel roughness zone was given a channel Manning’s n value that is dependent on the NLCD Classification to ensure the channel 2D cells have appropriate conveyance  The table below shows a typical land use-roughness matrix used in defining the roughness coverage for the study area.

Table 10:  NLCD 2019-Manning’s N Roughness Matrix

| NLCD Classification | Overland Manning’s N | Channel Manning’s N | Normal | Source |
| --- | --- | --- | --- | --- |
| Open Water | 0.03 | 0.03 | 0.03 | Ven Te Chow 1959 |
| Developed, Open Space | 0.04 | 0.035 | 0.04 | 2D User’s Manual, 2016 |
| Developed, Low Intensity | 0.1 | 0.035 | 0.1 | 2D User’s Manual, 2016 |
| Developed, Medium Intensity | 0.08 | 0.035 | 0.08 | 2D User’s Manual, 2016 |
| Developed, High Intensity | 0.15 | 0.035 | 0.15 | 2D User’s Manual, 2016 |
| Barren Land | 0.03 | 0.03 | 0.03 | Ven Te Chow 1959 |
| Deciduous Forest | 0.16 | 0.05 | 0.16 | Ven Te Chow 1959 |
| Evergreen Forest | 0.16 | 0.05 | 0.16 | Ven Te Chow 1959 |
| Mixed Forest | 0.15 | 0.05 | 0.15 | Ven Te Chow 1959 |
| Scrub/Shrub | 0.1 | 0.04 | 0.1 | Ven Te Chow 1959 |
| Grassland/Herbaceous | 0.035 | 0.04 | 0.035 | Ven Te Chow 1959 |
| Pasture/Hay | 0.03 | 0.04 | 0.03 | Ven Te Chow 1959 |
| Cultivated Crops | 0.035 | 0.04 | 0.035 | Ven Te Chow 1959 |
| Woody Wetlands | 0.16 | 0.04 | 0.16 | Ven Te Chow 1959 |
| Emergent Herbaceous Wetland | 0.07 | 0.04 | 0.07 | Ven Te Chow 1959 |

#### Breaklines

Breaklines align grid cell faces and were used within the 2D mesh area to define prominent features including, road embankments and hydraulic structures as well as stream centerlines.  Road embankments were identified in ArcGIS and imported into HEC-RAS as breaklines to ensure that water was not routed past roads until it was deep enough to overtop the road. Similarly, bridge/culvert crossings that were not processed out of the terrain data were modeled by adjusting breaklines adjacent to the road embankment to align grid cells around the embankment and allow water to be routed across the embankment without creating artificial backwater.

County, interstate, common name, state recognized, and U.S. roads were incorporated for road breaklines from the U.S. Census Bureau TIGER/Line road layer.  These were then used to identify points where streams intersect road embankments and v-notch breaklines were added to ensure flow was passing the embankments in a way that was reflective of the culverts or bridges similar to the process for highways as outlined above, but only being implemented at the streams.

Private dams were not modeled as showing protection and no major dams or reservoirs are present in this HUC 8 nor does this watershed include any FEMA accredited levees.

The stream centerlines were input as breaklines to ensure cells lined up perpendicularly to the stream and to capture stream conveyance.

# Model Results

The 2D BLE results for the study provide additional estimated SFHA in areas that do not currently have a SFHA mapped. While the results provide context for flood risk communication as part of the Discovery process and are scalable, the results require further analysis to be used for regulatory purposes.  The validity of the 2D BLE results should be verified through community work map meetings before being applied to a regulatory product.

Two USGS gages within the model areas had more than 10 years of flow record and were used for calibration and validation of the 1% annual chance event. Peak flows within the model were compared with a Bulletin 17C analysis of the gage data using PeakFQ. Several other locations were selected throughout each model area to compare model flows with the published USGS regional regression equations. At certain calibration locations where the upstream drainage area was significantly developed, adjustments were made to the regression equation estimates based on urbanization. Final results of the flow comparisons are presented in Table 11.

Table 11:  Model Calibration Results

| Work Area | Calibration Location | Drainage Area (mi2) | Lower Limit (cfs) | Q1% (cfs) | Upper Limit (cfs) | 2D Model flow (cfs) |
| --- | --- | --- | --- | --- | --- | --- |
| Town of Arnett – Canadian River | Streamstats TA_CR_1 | 8 | 2,007 | 3,340 | 4,673 | 3,278 |
|  | Streamstats TA_CR_2 | 84 | 3,233 | 5,380 | 7,527 | 5,179 |
|  | Streamstats TA_CR_3 | 11 | 2,596 | 4,320 | 6,044 | 4,938 |
|  | Streamstats TA_CR_4 | 13 | 2,662 | 4,430 | 6,198 | 5,518 |
|  | Streamstats TA_CR_5 | 40 | 2,446 | 4,070 | 5,694 | 5,688 |
| Town of Camargo – Canadian River | Streamstats TC_CR_1 | 526 | 45,375 | 75,499 | 105,623 | 69,048 |
|  | Streamstats TC_CR_3 | 14 | 3,239 | 5,390 | 7,541 | 7,506 |
|  | Streamstats TC_CR_4 | 5 | 1,677 | 2,790 | 3,903 | 3,110 |
| Town of Taloga – Canadian River | Streamstats TT_CR_2 | 44 | 6,791 | 11,300 | 15,809 | 11,505 |
|  | Streamstats TT_CR_3 | 26 | 4,850 | 8,070 | 11,290 | 9,259 |
|  | Streamstats TT_CR_4 | 7 | 2,723 | 4,530 | 6,337 | 4,829 |
|  | Streamstats TT_CR_5 | 14 | 3,281 | 5,460 | 7,639 | 5,911 |
|  | Streamstats TT_CR_6 | 11 | 2,963 | 4,930 | 6,897 | 6,465 |
|  | Streamstats TT_CR_7 | 6 | 2,314 | 3,850 | 5,386 | 3,139 |
|  | Streamstats TT_CR_8 | 13 | 3,456 | 5,750 | 8,044 | 6,606 |
|  | Streamstats TT_CR_9 | 19 | 4,171 | 6,940 | 9,709 | 5,166 |
|  | UGSS Gage 07228290 | 10 | 6,581* | 10,950* | 15,319* | 13,947 |
| Deer Creek | Streamstats DC_1 | 20 | 4,934 | 8,210 | 11,486 | 5,545 |
|  | Streamstats DC_2 | 160 | 14,364 | 23,900 | 33,436 | 30,433 |
|  | Streamstats DC_3 | 16 | 4,057 | 6,750 | 9,443 | 5,211 |
|  | Streamstats DC_6 | 18 | 4,706 | 7,830 | 10,954 | 4,820 |
|  | USGS Gage 07228450 | 2 | 1,082 | 1,800 | 2,518 | 1,838 |

*Weighted average of gage analysis value and streamstats value

## Floodplain Mapping

The following sections provide a synopsis of how raw modeled depths were translated into SFHAs.  In addition to developing a new SFHA, the BLE model data was leveraged to validate the effective zone A studies within the project footprint.

### Model Outputs

The floodplains are derived from the raw modeled depth grids using the maximum value.  These depth grids are exported from HEC-RAS as TIFF format rasters with an interpolated rendering that slope values at the center and along the faces/edges of the computational mesh cells.  Using GIS, the TIFF rasters are post processed into 1% SFHA and 0.2% shaded X polygons.

### Methodology

The use of 2D modeling methods results in water surface elevation values at every cell in the model’s computational mesh.  In order to represent the desired model results and eliminate extraneous disconnected cells, post processing of the depth grids is required.  For the purposes of the Lower Canadian – Deer BLE project, floodplain mapping delineation was completed using connected raster cells at the extent of the CNMS mapped and unmapped features in the project footprint with raster depths greater than 0.5 feet.  Converting the raster data to polygon features enabled an intersection of modeled results to the CNMS and effective zones to create the SFHA and 0.2% shaded X features.  Because the new mapping, based on gridded engineering, retains the blocky shape of a raster, a simplification process was applied using GIS to smooth the boundaries.  These processes remove unnecessary points, bends, and angles while preserving the natural shape of the polygon.  Furthermore, small voids, or “holes” inside of the floodplain were aggregated with the larger surrounding polygons to merge them and make the floodplain complete.  Mapping polygons were cleaned against the criteria of being larger than 2 acres. For example, if flooding polygons were disconnected from a flooding source and less than two acres, the polygon was removed from the SFHA. Flood polygons less than 2 acres that intersected a flooding source were assessed individually. These edits adhere to traditional and approved floodplain mapping approaches.

In addition to the SFHA, all other flooding associated with the 1% and 0.2% raw results were retained as “on the shelf” data that may be leveraged for future needs and analysis.

### Flood Hazard Area Layer

Special Flood Hazard Areas, as noted above, were developed to the extent of the CNMS features or up to 1 square mile drainage area and effective zone A study locations.  The Regional CNMS database, National Flood Hazard Layer, and paper inventory were used as reference data to ensure extent of the BLE results represents appropriate flooding extent.

The 0.2% flood areas were produced using the same methods as the 1% SFHA.  After both layers were developed, a union of the two products was performed to develop the deliverable format EBFE_FLD_HAZ_AR.

# CNMS Validation

The following summarizes the results of the CNMS validation assessments for the effective Zone A studies in the Lower Canadian – Deer Watershed.

The inventory of Zone A studies (250.4) in the Lower Canadian – Deer Watershed were classified in CNMS with validation status of “UNVERIFIED” (250.4 miles) and with status type of “BEING STUDIED.” The following is a summary of the results of the CNMS validation assessment for the effective Zone A studies in the study area. Initial Assessment checks A1-A3 were evaluated for the CNMS inventory of Zone A studies.

### Initial Assessment A1 – Significant Topography Update Check

This check involves determining whether a topographic data source is available that is significantly better than what was used for the effective Zone A modeling and mapping. For effective Zone A studies within Roger Mills County, LiDAR was used and, therefore, these streams pass this check. For all other streams throughout the watershed, lower quality topographic sources were used in the effective studies and the topographic sources used in this BLE study are considered significant improvements. Therefore, the remainder of the streams fail this check.

### Initial Assessment A2 – Check for Significant Hydrology Changes

This check involves first determining if regression equations were used for the effective study. Next, it must be determined whether new regression equations have become available from the USGS since the date of the effective Zone A study.  If newer regression equations exist for the area of interest, then an engineer must determine whether these regression equations would significantly affect the 1-percent annual chance flow.

For streams in Roger Mills County, 2010 regression equations were used. New equations were published in 2019. For the Canadian River, a comparison between these two equations was completed and shows that the new equations significantly impact BFEs, but neither the effective nor new equation is suitable due to the large drainage area of the stream. Therefore, this stream fails this check. For the Powwow River, a comparison between these two equations was completed as well and shows that the new equations do not cause a significant change in BFEs, therefore this stream passes this check. All other effective Zone A studies in this watershed did not use regression and automatically pass this check.

### Initial Assessment A3 – Check for Significant Development

This check involves using the National Urban Change Indicator (NUCI) dataset to assess increased urbanization in the watershed of the Zone A study.  If the percentage of urban area within the HUC 12 watershed containing the effective Zone A study is 15% or more, and has increased by 50% or more since the effective analysis, the study would fail this check.  Although the NUCI data provide year-to-year changes in urbanization, the NLCD also is needed to establish a baseline of urban land cover for this analysis.  The check for significant development in this watershed was completed by evaluating percentage of urban change at the HUC 12 level.

Three streams within Custer County are classified as urban, but have had no significant increase in impervious area since the effective analysis. Therefore, these streams pass this check. All other effective Zone A studies within the watershed are classified as rural and, therefore, also pass this check.

All of the initial assessment results are shown in Table 12.

Table 12:  A1-A3 Validation Results

| Assessment Checks | Pass / Fail | Notes |
| --- | --- | --- |
| A1 – Topography | Pass/Fail | High quality topographic data was used in the effective study / LiDAR sources available are considered significant improvements from the effective Zone A topographic sources |
| A2 – Hydrology | Pass/Fail | Regression equations were either not used OR new equations do not create a significant change in BFE / New equations that create a significant change in BFE are now available |
| A3 – Development | Pass | Watershed does not meet urban threshold OR has had no significant increase in impervious area since effective analysis |

### Validation Check A4 – Check of Studies Backed by Technical Data

Zone A studies that pass all initial assessment checks described above may be categorized as “VALID” in the CNMS Inventory only if the effective Zone A study is supported by modeling or sound engineering judgment and all regulatory products are in agreement.  If the effective Zone A study passes all initial assessment checks, but is not supported by modeling, or if the original engineering method used is unsupported or undocumented, a comparison of the BLE results and effective Zone A’s is performed.

All effective Zone A studies within Blaine County, Custer County, and Dewey County are not supported by technical data and fail this check. All other Zone A studies within this watershed are model-backed studies and, therefore, pass this check.

### Validation Check A5 – Comparison of BLE and Effective Zone A

The effective Zone A comparison was performed at the full extent of the Lower Canadian – Deer Watershed.  The validation of the effective Zone A boundaries using 2D flood hazard products differ from the standard 1D methods due to the lack of cross sections and their use with standard Floodplain Boundary Standards (FBS) methodology.  For this 2D study, the effective A zone boundaries were compiled using the National Flood Hazard Layer and Core Logic effective digital uplift product.  These data were dissolved to one continuous A-zone layer, which then had points placed along its perimeter every 500 feet.

For each test point, a 75-foot buffer was created.  Using this buffer, the minimum and maximum values of the DEM were extracted, as a proxy for the effective BFE.  The minimum value of the 1% minus raster, and the maximum value of the 1% plus raster are also extracted.  These 1% plus maximum and 1% minus minimum values are products of the new 2D BLE study and act as the vertical tolerance.  The test point passes if the DEM maximum value is less than or equal to the 1% plus maximum value and the DEM minimum value is greater than or equal to the 1% minus minimum value. This can be visualized as a short 75-foot radius cylinder, with a height of 1% plus maximum – 1% minus minimum.  This test verifies that at least one point from the ground surface (i.e. proxy BFE) falls both vertically and horizontally within this range.

### Validation Results

All 250.4 total miles of available CNMS features representing the effective Zone A studies were categorized as VALID – BEING STUDIED or UNVERIFIED – BEING STUDIED.  Total miles in each of these categories are summarized in Table 13 and illustrated in Figure 14 below.  Table 14 summarizes the validation results based on the individual HUC 12 watersheds within Lower Canadian – Deer Watershed.

Table 13: Zone A Validation Results

| Validation Status | CNMS Validation Status | CNMS Project Start | CNMS Post-BLE Assessment/Total Miles |
| --- | --- | --- | --- |
| VALID | BEING STUDIED | 65 | 0 |
| UNVERIFIED | BEING STUDIED | 185.4 | 250.4 |

Table 14: BLE Comparison Results

| HUC-12 Watershed |  | Total FBS Points | Fail | Pass | % Pass | BLE Comparison Pass? (>85%) | Priority Score |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Watershed Name | HUC 12 ID |  |  |  |  |  |  |
| Lower Canadian Deer | All Streams | 10,531 | 8,599 | 1,932 | 18% | Fail |  |
| 110902010101 Unnamed Trib-Canadian River | 110902010101 | 119 | 90 | 29 | 24% | Fail | 8.0 |
| 110902010103-Cimarron River | 110902010103 | 185 | 151 | 34 | 18% | Fail | 15.7 |
| 110902010110-Canadian River | 110902010110 | 289 | 218 | 71 | 25% | Fail | 15.4 |
| 110902010606 Unnamed Tributary | 110902010606 | 380 | 310 | 70 | 18% | Fail | 32.2 |
| 110902010703-Canadian River | 110902010703 | 528 | 449 | 79 | 15% | Fail | 39.7 |
| American Horse Creek-Canadian River | 110902010705 | 112 | 84 | 28 | 25% | Fail | 37.9 |
| Bois d' Arc Creek-Canadian River | 110902010204 | 207 | 184 | 23 | 11% | Fail | 11.1 |
| Cottonwood Creek-Canadian River | 110902010201 | 125 | 96 | 29 | 23% | Fail | 20.4 |
| Dead Woman Creek | 110902010610 | 466 | 327 | 139 | 30% | Fail | 8.1 |
| Dead Woman Crossing-Deer Creek | 110902010608 | 1,697 | 1,299 | 398 | 23% | Fail | 26.1 |
| Deer Creek | 110902010607 | 838 | 770 | 68 | 8% | Fail | 43.7 |
| Dugout Creek-Canadian River | 110902010203 | 143 | 96 | 47 | 33% | Fail | 14.7 |
| Flying Creek-Canadian Creek | 110902010303 | 112 | 96 | 16 | 14% | Fail | 2.4 |
| Horse Creek-Deer Creek | 110902010602 | 809 | 643 | 166 | 21% | Fail | 21.3 |
| Lariat Creek Canadian River | 110902010706 | 321 | 243 | 78 | 24% | Fail | 33.5 |
| Little Deer Creek | 110902010604 | 1,030 | 906 | 124 | 12% | Fail | 42.0 |
| Powwow Creek-Canadian River | 110902010306 | 200 | 146 | 54 | 27% | Fail | 20.3 |
| Spotted Deer Creek-Canadian River | 110902010206 | 206 | 189 | 17 | 8% | Fail | 38.0 |
| Town of Fay-Canadian River | 110902010507 | 495 | 435 | 60 | 12% | Fail | 46.4 |
| Town of Foley | 110902010605 | 726 | 615 | 111 | 15% | Fail | 40.8 |
| Town of Hydro-Deer Creek | 110902010609 | 215 | 148 | 67 | 31% | Fail | 10.7 |
| Town of Independence-Deer Creek | 110902010603 | 619 | 512 | 107 | 17% | Fail | 26.8 |
| Town of Putnam-Deer Creek | 110902010601 | 465 | 386 | 79 | 17% | Fail | 24.1 |
| Trail Creek-Canadian River | 110902010205 | 211 | 177 | 34 | 16% | Fail | 17.8 |
| Troll Creek | 110902010308 | 33 | 29 | 4 | 12% | Fail | 32.7 |

Figure 14: CNMS Validation Results

An overall risk for each HUC 12 watershed was calculated using the National Flood Risk Percentages Dataset and its proportional area.  The weighted risk was multiplied by the percentage of points in the watershed that failed the CNMS comparison to effective to determine the priority score.  Figure 15 below shows the range of the Lower Canadian – Deer HUC 12 priority scores which can be used to initiate discussions during the Discovery phase.  Town of Fay – Canadian River HUC 12 was determined to have the highest priority score and the most need while Flying Creek – Canadian Creek HUC 12 has the lowest score.  CNMS priority was calculated for all HUC 12s in the Lower Canadian – Deer Watershed in both the states of Oklahoma and Texas.

Figure 15: Ranking of Lower Canadian – Deer Watershed HUC-12s

# Flood Risk Assessment

A flood risk analysis was performed for this project.  The updated 1-percent annual chance depth grid was used to calculate the potential flood losses.  The loss results are stored in the S_FRAC_AR spatial file within the FRD geodatabase.  All results are reported in whole dollar values.

Hazus version 6.1 was used for the basic and refined loss analysis.

The losses are reported via census blocks.  It is important to note that the Hazus version used utilizes dasymetric census blocks.  Dasymetric mapping removes undeveloped areas (such as areas covered by other bodies of water, wetlands, or forests) from the census blocks, changing their shape and reducing their size in these areas.  For more information on dasymetric data visit FEMA’s Media Library for the Hazus-MH Data Inventories: Dasymetric vs. Homogenous, or Hazus 3.0 Dasymetric Data Overview.

Hazus analysis was performed by county within the project watershed extents for each return period to ensure proper model processing. A summary of results per county for the 1-percent annual chance scenario are shown in Table 15.

Table 15: Hazus 6.1 Results for 1-percent-annual-chance scenario for Lower Canadian - Deer Watershed, Oklahoma/Texas

| County | Full Replacement – Total Loss | Dollar Exposure (Replacement Value) - Buildings | Dollar Exposure (Replacement Value) - Contents |
| --- | --- | --- | --- |
| Roger Mills, OK | $302,000 | $18,336,000 | $9,162,000 |
| Lipscomb, TX | $28,000 | $6,284,000 | $6,176,000 |
| Hemphill, TX | $126,000 | $7,815,000 | $4,282,000 |
| Ellis, OK | $2,064,000 | $71,217,000 | $44,907,000 |
| Dewey, OK | $23,607,000 | $202,397,000 | $135,239,000 |
| Custer, OK | $78,243,000 | $1,379,763,000 | $1,120,771,000 |
| Caddo, OK | $6,490,000 | $120,617,000 | $100,181,000 |
| Blaine, OK | $554,000 | $31,555,000 | $17,480,000 |
| Total | $111,414,000 | $1,837,984,000 | $1,438,198,000 |

# Quality Assurace & Quality Control

The 2D BLE results were independently assured to meet quality standards of the project. Standard checklists compling with FEMA standards were used to review hydrology and hydraulics results, the resulting flood hazard area delineations and CNMS dataset. QA during the data development process includes, but is not limited to the following QC checks:

- Hydrologic QC – included checks for curve number calculation, input precipitation, flood events, areal reduction factor and human error

- Hydraulic QC – included checks for HEC- RAS geometry, breakline placement, v-notch placement to ensure water conveyance at crossings, refinement regions, roughness coefficients, boundary/initial conditions, timestep and mapping outputs

- Mapping – included checks for spot checking flood inundation accuracies based on floodplain mapping criteria, ensuring flood extents from higher frequency events did not exceed lower frequency extents due to GIS smoothing algorithms, and topological overlap and cluster tolerance errors were resolved

- CNMS - Included review of the A1-A5 analysis for all effective approximate reaches in the HUC, geometry updates to newly studied reaches added as Zone X, and removal of scoped areas which lie outside of the final BLE footprint

# Deliverables

All information, data and files for the Lower Canadian – Deer, Oklahom Watershed BLE process were uploaded to the FEMA Mapping Information Platform (MIP). Items included with this deliverable include:

- Base Level Engineering Summary Report

- Certificate of Compliance and Completeness forms

- Hydraulic and Hydrologic Models (HEC-RAS and HEC-HMS, respectively)

- Spatial Files (EBFE File Geodatabase and MetaData)

- Supplemental Data (CNMS and Hazus)

# Considerations

## Challenges

Major challenges included a lack of peak streamflow records at the outlet, though sufficient data was available to provide confidence in the results. Streamstats data was used in addition to USGS gage data for the parts of the models within Oklahoma to increase confidence in model results.

Calibrating the models in this area was a challenge because of the karst features present. Curve numbers were adjusted to represent karst features and used to calibrate the models.

There was a challenge in calibrating a tributary in the Town of Arnett model. An internal boundary condition line was added along this tributary to calibrate it to the Streamstats peak flow value.

Another major challenge included incorporating the upstream HUC 8 into the Town of Arnett – Canadian River model. The inflow hydrograph was estimated based on data recorded by USGS Gage 07228000. A dimensionless unit hydrograph was estimated based on historical storm events, which was further scaled and interpolated to estimate the respective inflow for various recurrence intervals and to accommodate the distance between the gage location and the model inflow boundary condition.

The tie-in between the Town of Taloga and Deer Creek models was a challenge. The 0.2-percent-annual-chance tie-in to the existing (Lower Canadian Walnut) 1D model downstream of this study area was also a challenge. These tie-in challenges were addressed  by changing the model inflow and outflow slopes.

Structures have not been added to the BLE model despite evidence of cross structures which allow the streams to convey water through numerous structures. An estimated structure opening was modeled however, the model may be overestimating the water surface elevation upstream of the road until the road is overtopped and underestimating the water surface elevation downstream of the road.

## Recommendations

This study provides significant information useful for flood identification and communication among those affected.  The study is highly scalable, and stakeholder input and further analysis would enhance the product and inform implementation of regulatory flood hazard areas.  In addition, the validity of the 2D BLE results should be verified through community work map meetings before being applied as a regulatory product.

# References

Bibliography list of references used and referred to throughout document.

Chow, Ven T. "Development of Uniform Flow and It Formulas." Open Channel Hydraulics. Caldwell, NJ: Blackburn, 1959. 109-113. Print.

FEMA, “Guidance for Flood Risk Analysis and Mapping – Base Level Engineering (BLE) Analyses and Mapping”, November 2021. (https://www.fema.gov/sites/default/files/documents/fema_base-level-engineering-guidance_112021.pdf).

NOAA, “NOAA Atlas 14 Precipitation-Frequency Atlas of the United States”, 2013. Volume 9, Version 2.0. (https://www.nws.noaa.gov/oh/hdsc/PF_documents/Atlas14_Volume9.pdf).

U.S. Army Corps of Engineers, Hydrologic Engineering Center. (September 2021). HEC-RAS River Analysis System, Version 6.1.0 Davis, California.

USGS, “Estimating Magnitude and Frequency of Floods Using PeakFQ Program: USGS Fact Sheet”, 2006.

USGS.  Multi-Resolution Land Characteristics Consortium. National Land Cover Database 2019.  (National Land Cover Database 2019 - Landcover & Imperviousness (NLCD2019) | Multi-Resolution Land Characteristics (MRLC) Consortium).

USGS. “Statewide Analysis of the Drainage-Area Ratio Method for 34 Streamflow Percentile Ranges in Texas”. 2006.

USGS. StreamStats V4.15.0. (https://streamstats.usgs.gov/ss/)

# Appendix A – Exhibits – Base Level Engineering Results