Internal Review of BLE Submittals

HOW2

Reviewer Name:  Brian Bradach

BLE Watershed:  Lower Wolf

Date Reviewed:  July 17, 2023

Status of Review:  Passed

FEMA Region 6 has prepared submittal templates and provided guidance materials for use. This handout provides some high-level data checks that should be performed ahead of a BLE data submittal to FEMA Region 6 for review.

Please assure your BLE submittals use the most up-to-date templates when preparing the submittals.  The most up-to-date templates (File Geodatabase – zipped, BLE Report Template and 2D File Inventory excel) and Submittal Guidance (Submittal Guidance) are available at:

https://rmd.msc.fema.gov/Regions/VI/Ph0_Investment/1/Base%20Level%20Engineering/Submittal%20Guidance

Report File(s)

Report should provide all information identified in BLE Report Template

Report should include a CNMS/NVUE evaluation map (where does BLE support FIRMs, based on Zone A review criteria – similar floodplain width)

Report should include an image with HUC10 areas defined – may be labeled with Name or HUC10 number – selection should match naming of Model File folders.

Report should include information about how hydrology and resultant flow values were defined.

Hydrology should indicate if data was available for any reservoirs, if so, provide details on how flow values were adjusted and placed in the hydraulic models.

Report should be provided in Word (.doc) and PDF file formats.

2D model areas shall include a note to indicate the optimal time step used to prepare the analysis.

☐ 	OPTIONAL – if work maps are prepared, they will be included in Report file folder when made available on the site.  Work maps may be included in a ZIP file or folder.

☐ 	OPTIONAL – if change area maps are prepared, they will be included in Report file folder when made available on the site.  Work maps may be included in a ZIP file or folder.

☐ 	OPTIONAL – if KML/KMZ files are prepared, they will be included in Report file folder when made available on the site.  Work maps may be included in a ZIP file or folder.

AR CTP includes workmaps that should be ZIPPED up and submitted to USGS for download with the REPORT.zip file

Model Files

☐ 	If hydrology is prepared it should be provided in submittal.

☐ 	1D – Excel files with Regression Equations (and gage verification) should be provided

☐ 	2D – HEC-HMS files for loss calculation should be included

☐ 	2D – HEC-HMS files for rain-on grid (7 events – 10%, 4%, 2%, 1%, 1% plus, 1% minus, and 0.2%)

☐ 	1D Hydraulic Models should be broken up into folders by HUC10 area.  Naming may be numerical (HUC10 number) or include the name of the HUC10 - should match the naming for HUC10 graphic shown in BLE Report. An index map should be provided in the Models folder that matches the HUC10 graphic in the report.

☐ 	Number of Model folders (1D) should match the number of HUC10 areas.  There is one case where the folder count will increase by one. If one mainline stream traverses multiple HUC10 areas – in this case, the mainline stream model should be placed in its own folder.

☐ 	2D Model areas will have their materials compiled in one folder and be organized in agreement with Submittal Guidance document, pages 25, 26, and 27.

☐ 	2D models should be accompanied by spatial information for Land Use/Manning’s N values

☐ 	2D model submittals must include terrain files to allow operation of the model by others.

☐ 	2D model deliveries should also include the 2D Model Inventory File.

☐ 	2D model  OUTPUT should be run with a time step identified and documented by the Mapping Partner; this (optimized) time step should be documented in the BLE report.

☐ 	2D model  Each of the seven frequencies should then be run once more, these model outputs will be run with a time step of 6‐hours or 12‐hours, this longer time step will allow the output file to be significantly reduced.

Spatial Files (General)

☐ 	Spatial information should be delivered in agreement with the Spatial Reference System detailed on page 28 of the Submittal Guidance document.

☐ 	Rules for “null” values are detailed on page 29 of the Submittal Guidance

☐ 	Topology rules detailed on page 30 of the Submittal Guidance should be reviewed and checked ahead of submittal.

Spatial Files (Base – S_POL_AR  Political Areas).

☐ 	Spatial information should be submitted in agreement with overview provided on page 35 in Submittal Guidance.

☐ 	File should include all communities that intersect and include some community area within the HUC8 being addressed – this may be submitted one of three ways, shown from most preferable (left) to least preferable (right) in the images below.

Include all counties that intersect HUC8 area and sustain all communities within the county, even if not in the HUC8.

Clip all community polygons to HUC8 boundary.

Include all communities within HUC8 Boundary and keep county/parish areas whole.

1

Three communities within the HUC8 are missing in the image to the left.  This political area submittal will be returned for additional work.

2

3

Spatial Files (Base – S_HUC8_AR  Watershed Boundaries)

- Spatial information should be submitted in agreement with overview provided on pages 11 and 12 in Submittal Guidance.

- File should include HUC8 watershed boundary based on the USGS Watershed Boundary Dataset, minimum.

- OPTIONAL – It is beneficial if the HUC10 watershed areas are also included in the file.  Polygon overlaps (Between HUC8 and HUC10) are acceptable.

Spatial Files (CNMS – S_Studies_Ln  Studied Streams)

- Current CNMS coverage may be collected by contacting the FEMA Regional Support Center (RSC) CNMS Lead, Michael Johnson (Michael.Johnson@aecom.com)

- The S_Studies_Ln coverage should include stream centerlines for ALL streams that have been studied in the BLE effort – if the stream has a model, it should have a CNMS line.

- If a tributary is 100% inundated and has no model created, then it is not required to be included in the S_Studies_Ln file.

- Stream segments upstream of all study areas should NOT be included in the S_Studies_Ln file.

- If you require back up review, the CNMS lead at the RSC can be contacted, he has some documentation and training materials that could support this review effort.

The watershed to the left is missing several studied streams (green).  The submittal only updated the current mileage on FIRMs with assessment results (orange).  The S_Studies_Ln submitted should include all newly studied streams too.

Spatial Files (CNMS – S_Unmapped_Ln  Unstudied Streams)

- CNMS data provided by the Regional Support Center will include an S_Unmapped_Ln coverage.

- Streams that are newly studied should be MOVED to the S_Studies_Ln file.

- Stream segments upstream of all study areas should be included in the S_Unmapped_Ln

The watershed to the left shows the extended streamlines that exist upstream of the floodplains created during the BLE assessment.

Note – WTR_LN and S_Studies_Ln will have the same extent. S_Unmapped_Ln should contain anything not included in the BLE analysis.

Spatial Files (EBFE – BFE_2D  2D BFE Contours)

- If study is prepared in 2D environment, file should be included in the database delivered.

- Create contours using the 1% annual chance WSEL event raster, in low lying areas, create 1- or 2-foot contours. Where more elevation is included in the study dataset, prepare 5-foot contours.

Spatial Files (EBFE – DTL_STUD_AR  Detailed Study Area)

- Review NFHL/FIRMs/Study for detailed study areas (Zone AE, AH, AO and Zone V/VE) flood designations to create this coverage.  Note, Zone V areas are included in the detailed study as the coastal influence generally details a different BFE than the riverine analysis alone.

- Detailed Study Areas should be intersected with the DFIRM coverage to provide FIRM panel numbers as a reference.

- Review NFHL and/or County DFIRM Databases to assure all detailed floodplain areas (S_FLD_HAZ_AR) are included in the areas provided.

- Use GIS to georeference/review paper FIRMs to provide full coverage where FIRMs have not yet been modernized.

- Assure that the polygons are not significantly larger than the BLE and the Detailed study areas.

The detail study areas on the FIRM are shown in dark blue, BLE floodplains (1%) are teal.  Yellow was the submitted study area and the pink is the updated detail study area polygons.

In Area 1 – the polygons were too wide.

In Area 2, the coverage did not include all detailed study areas.

2

1

Should NEW study areas be part of the project being delivered, these should be added to the coverage.

Spatial Files (EBFE – DTL_STUD_LN  Detailed Study Streams)

- Include stream centerline for all detailed study streams, leverage DFIRM database (S_WTR_LN and/or S_Profil_Basln to provide the stream centerline for the studied stream.

- Use GIS to georeference/review paper FIRMs to provide full coverage where FIRMs are not modernized  CoreLogic dataset can provide coverage (detailed/floodway) where FIRMs are not modernized.

- Should NEW study segments be included to the existing FIRM coverage (New Zone AE, AO, AH, V, VE), please add these to the coverage.

Spatial Files (EBFE – FLD_HAZ_AR  Floodplains 1% and 0.2%)

- Floodplains should be provided in polygon format for a seamless coverage of the study area.

- 1D BLE floodplains should not extend beyond flooding source’s upstream cross section.

- 1% and 0.2% floodplains should NOT have overlapping polygon areas.

- Floodplains and rasters should be reviewed against one another. The floodplain coverage and grids must agree at the extents (e.g. 1% floodplain matches 1% raster), this is needed to assure that the viewer and report function will be supported. Review EST_RISK = HIGH against 1% grids.  Review EST_RISK = MODERATE against 0.2% grids.

- 1% polygons and rasters should be smaller than 0.2% polygons and rasters.

In this case, the floodplains (pink) have been cleaned/filled-in and the grids (black overlay) do not agree and require update.

EITHER Floodplains need to match to grids

OR grids screened to match the floodplains.

In this case, the floodplains (pink) and the grids (black overlay) do not agree and require update.

EITHER Floodplains need to match to grids

OR grids screened to match the floodplains.

In this case, the 1% floodplains (blue) and 0.2% floodplains (pink) and 10% floodplains (yellow) have different coverage areas.  There is a 1% floodplain polygon missing that does not cover the yellow (10%).

Spatial Files (EBFE – TENPCT_FP  Ten Year Floodplain)

- Floodplains should be reviewed against the TENPCT_FP.  Review of the 1% and 10% floodplains against one another to assure that the TENPCT_FP floodplain coverage is similar and does not expand beyond the 1% extent.

- In some BLE submittals, the 10% floodplains do not provide complete coverage.

The 10% floodplain (yellow) is larger than the 1% floodplain (blue) in the areas shown here.  Ultimately, this required update of a model and floodplains too.

In this case, the 1% floodplains (blue) highlight that the 10% floodplains are missing some coverage.

IF this occurs within a DETAILED study area, it is can be acceptable.

Spatial Files (EBFE – WTR_LN  Water Line)

- WTR_LN and S_Studies_Ln should have the same extent once the BLE is complete, but the geometry may not align with each other.  WTR_LN may include terrain-based profile baselines from the BLE that may not align with the effective lines from S_Studies_Ln.

- Field WTR_NM (in WTR_LN) should include a name that will coincide with a MODEL folder within the modeling delivery.  Only those related to S_Unmapped_Ln may have no data entered in this field.

Spatial Files (EBFE – WTR_AR  Water Area)

- WTR_AR should include lake/pond/reservoir boundaries available in the NHD Waterbody coverage.

- If the dataset is refined, there is no need to include water areas that are smaller than an acre in size.

Spatial Files (EBFE – SUB-BASINS)

- Sub-basins are required for 1D submissions (NOT required in 2D submittals – but may be included)

- Sub-basin file should include a coverage for the complete HUC8 area. Review may need to make feature class transparent to view all features.

- In addition, the subbasin file may/should include additional drainage areas that are upstream of the HUC8 being studied.

Spatial Files (EBFE – XS  1D analysis cross-section)

- Cross-section files are only included in 1D BLE submittals.

- Review cross-section files to assure that they visually appear when added to GIS.

- Coverage may be reviewed against the EBFE- WTR_LN file and/or the CNMS-S_Studies_Ln

- EXPORT all entries to an excel file.

  - Review 10% and 1% water surface elevation values, in all cases the 1% elevation should be HIGHER than the 10% elevation included.

  - Review 1% and 0.2% water surface elevation values, in all cases the 0.2% elevation should be HIGHER than the 1% elevation included.

Spatial Files (Mitigation Files – S_AOMI_AR). Areas of mitigation interest should include polygons to define areas that:

- Culvert/bridge/structure data would better define the flood extent.

- Terrain processing may not have been complete, survey may be needed to better define floodplain.

- 1D or 2D analysis may provide better result.

- Indicates an overflow area between multiple streams, sub-basins, or basins.

- Tie in issues at basin extents that are not resolved.

Spatial Files (Mitigation Files – S_FRAC_AR). Census Block polygons with risk assessment loss results.  The Hazus (one HPR file) information is submitted, check to assure it is included in the Supplemental file folders that are submitted.

Spatial Files (L_Source_CIT). Table includes references for all source information used and included in the submittal information.

Spatial Files – WSEL Grids (500>100>50>25>10). As outlined in the floodplain section above – the review of the grids is to assure that the 1% WSEL grid coverage agrees with the EST_RISK = HIGH floodplain.  Also review the 0.2% WSEL grid coverage for agreement with the entire coverage included in FLD_HAZ_AR.

Elevations should be rounded to 0.0. Some partners also include 50/25/10 year water surface elevation grids.

Spatial Files – Flood Depth Grids (500>100>50>25>10). As outlined in the floodplain section above. Review of the grids is to assure that the 1% flood depth grid coverage agrees with the EST_RISK = HIGH floodplain.  Also review the 0.2% flood depth grid coverage for agreement with the entire coverage included in FLD_HAZ_AR.

Depths should be rounded to 0.0. Some partners also include 50/25/10 year water surface elevation grids.

The image to the right shows where both the 1% floodplain/grids and the 0.2% floodplain/grids both require resolution prior to finalizing the data for the USGS.

HUC8 Basin Connections. If HUC8 information has been released, review floodplains and/or grids against each other at the US and DS ends. The mismatch is best resolved prior to submittal to the Estimated BFE Viewer.

Spatial Data Submittals. As a part of the review, historically, I have reviewed the incoming feature classes and then loaded them to the most up-to-date template.  In some cases, errors will arise if partners have not used the most up-to-date template.  The templates for BLE and the submittal guidance are included on the RMD SharePoint at the following location:

https://rmd.msc.fema.gov/Regions/VI/Ph0_Investment/1/Base%20Level%20Engineering/Submittal%20Guidance