# Texas 2D BLE Analysis QA/QC Checklist

# Project Information

| HUC-8 Watershed Name & ID: | 11120102 Palo Duro |
| --- | --- |
| HUC-10 Watershed Name & ID: | 1112010201 Upper Palo Duro Creek 1112010202 North Palo Duro Creek 1112010203 Lower Palo Duro Creek |
| Modeler: | FTN Associates |
| Reviewer: |  |

## Terrain and Modeling Review (QC Items 1-6)

| Modeler Initials: | LB | Reviewer Initials: | WF/AC |
| --- | --- | --- | --- |
| Submittal Date: | 3/7/2023 | Review Submitted Date: | 3/20/23 |
| Review Response Date: |  | Backcheck Complete Date: | 4/24/23 |

## Mapping Review (QC Item 7)

| Modeler Initials: |  | Reviewer Initials: | WF/KJ |
| --- | --- | --- | --- |
| Submittal Date: |  | Review Submitted Date: | 06/30/23 |
| Review Response Date: |  | Backcheck Complete Date: |  |

## Completeness Review (QC Item 8)

| Modeler Initials: |  | Reviewer Initials: | WF/KJ |
| --- | --- | --- | --- |
| Submittal Date: |  | Review Submitted Date: | 06/30/23 |
| Review Response Date: |  | Backcheck Complete Date: |  |

(Modeler to populate with an “X” for items included in the QC submittal)

| Level of Study |  |  | Flood Events |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| X | 2D BLE |  | X | 10% ACE (10-year) | X | 0.2% ACE (500-year) |
|  | Enhanced BLE |  | X | 4% ACE (25-year) | X | 1% plus |
|  |  |  | X | 2% ACE (50-year) | X | 1% minus |
|  |  |  | X | 1% ACE (100-year) |  | Observed Rainfall |

Instructions

Each section includes instructions to the modeler and reviewer regarding their responsibilities in the completion of this checklist. In the Methodologies / Data Sources tables, grey italic text is added as an example response. Additional sample responses are located in the Texas 2D BLE Guidelines Quick Guide.

Reviewer to indicate if Technical Review items generally comply using a “Pass”. If the technical review observes an issue of noncompliance or an item that requires additional attention, an “Fail” should be utilized.

## Submittal Checklist

Data submitted for each milestone should be carried through each subsequent milestone. All geospatial layers and models must have same defined projection system as terrain. Modeler to populate with an “X” for items included in the QC submittal.

| Base Map Data (QC items 1-6) |  |
| --- | --- |
| X | Location Map (PDF): HUC-8 and HUC-10 boundaries and/or model domain boundary if different from HUC-10s, stream gages, dams (if applicable), mainstem streams (for reference), background map |
| X | GIS Map (MXD or Map Package) with the following data included but not limited to: Stream centerlines, model breaklines, rainfall data, WSEL and Depth grids (10, 100, 500-yr), adjacent HUC mapping (available on the BLE viewer or currently studying), 2D connections for inflows/outflows, gages or points of flow comparisons (BFEs, highwater marks, etc.), profile lines, boundary condition lines, refinement regions, model domain boundary, model mesh, DFIRM mapping panels, base map data (roads, aerials, etc.), soils data, land use data, terrain |
| Technical Report / Documentation (QC items 1-6) |  |
| X | Technical Report / Project Notebook: assumptions, special considerations, and modeling concerns |
| X | Detailed explanation and support data of variances in methodologies and approaches from recommended guidelines (Should be included in Technical Report / Project Notebook) |
| X | Internal QA/QC Documentation: comments and responses |
| X | Flow comparison tables within the technical report |
| Hydrology (QC items 1-6) |  |
| X | Hydrologic Model (HEC-HMS) model with all associated DSS files for 7 required storm events |
| X | Hydrologic Data and Parameter Spreadsheets (rainfall calculations, curve number table, gage locations, gage analysis and flow validation) |
| See report | Hydrologic modeling notebook including assumptions, notes, and special conditions (if not provided in report) |
| Hydraulics (QC items 1-6) |  |
| X | Hydraulic Model (HEC-RAS) with all associated files and 7 required storm events (remove non-final files)d |
| See report | Hydraulic Data and Parameter Spreadsheets (Manning’s n-value table, infiltration calculations support data-Soils, Land Use, Curve Number table, other calculations/data used in model) |
| See report | Manning’s n-value table |
| X | Precipitation data correctly linked to hydrologic model or hard coded from provided spreadsheet |
| See report | Hydraulic modeling notebook including assumptions, notes, and special conditions |
| Mapping (QC item 7) |  |
| - | Flood risk database |
| - | Modeler to complete topology validation as outlines in the guidelines |
| - | Modeler to verify mapping meets deliverable requirements as outlined in guidelines |
| Completeness Review (QC item 8) See Texas 2D BLE Guidelines Chapter 4 – Deliverables |  |
| - | Technical BLE Report |
| - | Final cleaned hydrologic model |
| - | Final cleaned hydraulic model |
| - | Flood risk database |
| - | CMNS updates |
| - | HAZUS results |
| - | Additional supplemental data |

# 1 – Model Terrain

Methodologies / Data Sources (Modeler to populate)

| Terrain Source(s): | USGS 2018 Texas Panhandle LiDAR, 2017 NRCS-FEMA Northeast NM LIDAR, 2016 Roosevelt and Curry Counties, NM LIDAR |
| --- | --- |
| Data Gaps: | N/A |
| Source Resolution: | 1-meter |
| Model Terrain Resolution: | 10 foot |
| Terrain Horizontal Projection: | NAD83 SPCS 4201 FT |
| Terrain Vertical Projection: | NAV88 FT |
| Z Values: | 4849.42 – 3461.68 |
| Terrain Buffer: | 1 mile |
| Other: |  |

## Technical Review (Reviewer to confirm with Pass/Fail)

| General Review |  |
| --- | --- |
|  | LiDAR source matches the latest available data at time of project initiation |
|  | Large reservoir or lake extents included in model domain |
|  | Check visible seams, gaps, or holes of LiDAR sources (if more than one source was used) |

## Review Comments

- No comments

# 2 – Hydrology

## Methodologies / Data Sources (Modeler to populate)

| Software: | HEC-HMS v4.8 |
| --- | --- |
| Rainfall Source: | Atlas 14 |
| Rainfall Duration: | 24-hr |
| Depth Selection: | 5-minute Intensity Duration |
| Rainfall Temporal Distribution: | Frequency Storm 50% Intensity Position |
| Areal Reduction: | None |
| Loss Methodology: | CN – HEC-RAS Rain on Mesh |
| Unit Hydrograph Transform Method: | HEC-RAS Rain on Mesh |
| Inflow Hydrograph Source: | - |
| Other: |  |

## Statistical Data (Modeler to populate)

No gages within watershed.

## Technical Review (Reviewer to confirm with  Pass/Fail)

| Excess Rainfall |  |
| --- | --- |
| P | Precipitation values are an aerial average of Atlas 14 values for the entire 2D area boundary for each storm event or other appropriate rainfall application |
| F | CN values appropriately reflect land use and soils for entire 2D area boundary |
| F | Losses are correctly calculated to establish the excess rainfall hyetographs by checking land use, soils and infiltration layers or other relative information |
| F | Total precipitation depths for 1%+ and 1%- are calculated correctly |
| P | Excess rainfall hyetographs are calculated for all storm events (10%, 4%, 2%, 1%, 0.2%, 1%+, and 1%-) |
| NA | Cumulative excess rainfall values are in increasing order from 10%, 4%, 2%, 1%, to 0.2% |
| NA | Cumulative excess rainfall values for the 1%+ and 1%- fall in order with the storm events matching the total precipitation values |
| Inflow Hydrographs |  |
| NA | Shape and time-to-peak of any inflow hydrographs resemble past documented large storm events |
| P | Hydrographs reasonable when compared to the other return periods. The shapes should be similar, and hydrographs from different events should not intersect unless there is an engineering reason. Peak flows from a lower return period event should not be higher than a higher return period event |
| P | Contributing hydrographs from another RAS model applied at coincident locations (Provide locations in base map data) |
| General Hydrology |  |
| P | Any flow regulating dams in the model area? (Yes/No) |
| F | Dam modeling methodologies applied and explained? (Yes/No). If yes, provide discharge rating curve calculations and gage analysis documenting regulated versus unregulated records, if applicable. |
| F | Model aligns with documentation of dam modeling approaches. |
| Other Notes: NA |  |

## Review Comments

- The project narrative under the hydrology submittal only shows the terrain section. Please provide necessary documentation and methodology details of hydrologic parameters.

Updated.

OK.

- No files were provided to support the precipitation data and any other hydrologic parameters.

All files can be found be found in the map package. A copy has been placed in the supplemental data folder.

OK.

- Please add a note in the report stating that the HMS model is created solely for the purpose of creating precipitation data for RAS 2D modeling.

Updated.

OK.

- No aerial reduction factor applied to the HUCs as per the BLE report? Please provide documentation and supporting information to validate.

FTN believes that, although using rainfall reduction may be appropriate when computing flows at the outlet of a HUC-8 or the total contributing drainage area to a gage, reducing the rainfall applied over the model meshes would lead to an underestimation of the flood risk in the upland portions of the models. For this reason, use of areal reduction was not retained.

OK. Please make sure to document assumptions made.

Will update report.

Okay.

- Suggest keeping the same geometry for all events, this includes 1% plus & minus. Check page no. 22-23 of the TWDB Guidelines. The use of AMC III is not recommended for BLE modeling.

Updated.

OK.

- The infiltration values within RAS are all blank. Please provide documentation to show those numbers or update accordingly.

Minimum infiltration rate is an optional value that can be used to calibrate models. Since there are no gages withing Palo Duro Watershed, FTN coordinated with FNI for the gage analysis in Tierra Blanca Watershed. It was decided during the calibration process to use the default value of 0.

OK.

# 3 – Hydraulic Geometry

## Methodologies / Data Sources (Modeler to populate)

| Software: | HEC-RAS 6.3.0 |
| --- | --- |
| Loss Methodology: | CN |
| Soils Source: | USGS |
| Land Cover Source: | NLCD 2019 – verified to Aerial |
| Manning's Roughness Source: | NLCD 2019– verified to Aerial |
| Nominal Mesh Cell Sizes: | 200’x200’ |
| Other: |  |

## Technical Review (Reviewer to confirm with Pass/Fail)

| 2D Mesh |  |
| --- | --- |
| F | 2D Boundary delineated along watershed boundary or buffer to ensure all contributing area is included |
| F | Nominal cell size appropriate for model detail (generally 200 feet, minimum 50 feet for refinement areas) |
| P | Computation points are at default locations or changes documented |
| P | Mesh Cell and Face tolerances are default or changes documented |
| P | No mesh errors |
| P | Cell face relationships are calculated |
| Breaklines |  |
| P | Breaklines are consistent with the terrain data |
| P | Breaklines sufficiently detailed to capture features (thalweg, banks, berms, etc.) |
| P | Cells do not overlap terrain features where appropriate (i.e., unwanted leaking) |
| P | Breakline cell sizing and parameters are appropriate |
| F | 2D Area Connections placed at applicable benchmarks, gages, or dams |
| P | 2D Area Connections set to use normal 2D equation domain for benchmarks and gages. Appropriate modeling approach used for dams. |
| Manning’s N-Values |  |
| P | Manning’s n-values associated to land cover data provided |
| P | Land Cover classifications are sufficiently descriptive within the data table |
| F | Manning’s n-value classifications generally match current aerial imagery |
| P | Manning’s n dataset is associated with the geometry files |
| General |  |
| P | All model datasets each have unique descriptive names |

## Review Comments

- 1112010203 Mesh doesn’t cover full HUC-8 extents. Black is HUC8 and Pink is the mesh boundary. Few examples are below. Others are marked in the Palo_QC_03172023.shp provided.

Boundaries have been updated to include all areas of the watershed.

OK.

- No buffer created for the 2D mesh boundary within RAS models.

1000 ft buffer has been added.

OK.

- 200x 200 cell size is not used  for model 201 and 202.

Corrected.

OK.

- 2D connections not placed at gages. Add profile lines (if used) for the gages.

There are no gages within the HU08 watershed.

Since there are no gages, please revise report to state that. Section 2.3 Model Results says that discharge rates compare reasonably well with the stream gage analysis. Please update Figure 5 name to not have USGS Stream Gages in the name.

Will update report.

Okay.

- The calibration regions added to update n values do not correspond to aerials correctly. For example, model 203 has a calibration region added of 0.035 but this area shows developed low-intensity area, and it should be 0.06. The refinement regions are over-applying the manning n values and the buffer applied seems a lot wider (around 400 feet wide) than needed. Check the TWDB Guidelines for manning n values. 0.035 represent clean maintained channels.

Calibration area has been corrected to follow the banks of the channel closer. Manning’s n values have been updated to follow the channel values listed in the TWDB Guidelines.

OK.

The image below  is more of a mix of deciduous forests or woody wetlands. The original land cover should be used for most places.

- Suggest keeping the same geometry for all events, this includes 1% plus & minus. Check page no. 22-23 of the TWDB Guidelines. The use of AMC III is not recommended for BLE modeling.

Updated.

OK.

- Suggest extending the downstream mesh to overlap with the upstream model. This will create a seamless floodplain.

Updated.

OK.

- The Draft BLE report identified Bivins Lake Dam was modified since the City of Canyon FIS. Additional data on this modification may be obtained from the TCEQ Central Registry and should be clearly documented in the report. Additionally, any assumptions made to model this structure must be documented with consideration for future FEMA regulatory process. If modeling assumptions could impact flood inundation extents, please discuss in Section 2.3.4 – Challenges of the BLE Report.

Spillway was confirmed by comparing the data from TCEQ Central Registry and the terrain. Additional detail added to report. No additional data was found through the TCEQ Central Registry for the central outlet.

The 100yr event is overtopping the top of dam. Verify that this is correct/add write up about it in the report.

Upon review, it appears there are approximately 2 feet of standing water remaining at the completion of the playa hot start file run. The lake appears to normally be empty from review of historic aerial imagery. Therefore, the hot start run appears to be artificially occupying some of the lake’s total storage. While there is no information indicating that the central outlet functions during routine events (FIS text indicates no discharge until the emergency spillway elevation), the small culvert invert has been altered to drain the standing water left after the hot start run in an attempt to start event modeling with no water behind the dam.

The results were not able to be viewed in the model. Not checked.

# 4 – Hydraulic Boundary Conditions

## Technical Review (Reviewer to confirm with Pass/Fail)

| Boundary Review |  |
| --- | --- |
| P | External boundaries connected to 2D mesh |
| F | Boundary conditions everywhere flow could build up at the boundary of the 2D mesh |
| P | Boundary conditions have a hydrograph or elevation/slope data assigned |
| F | Input hydrographs consistent with hydrologic data (DSS, manual entry) |
| P | Precipitation data is consistent with hydrologic data (DSS, manual entry) |
| F | Downstream boundary consistent with ground slope or known downstream WSEL |
| F | Flow hydrographs applied at appropriate locations |
| F | Energy grade slope for hydrograph distribution is reasonable |
| NA | Internal rating curves appropriate and documented for control structures (if applicable) |

## Review Comments

- Model 201: Flow is building up along the edge of the mesh.

Updated.

OK.

- Model 201: Split the outflow Boundary condition between the main channel and overbank. The normal depth slope will be different for each part. This is because it doesn’t have a connected floodplain within the entire extent.

Updated.

OK.

- Model 201: Boundary condition lines are missing from the northern edge of the boundary.

Updated.

OK.

- Model 202: Split the inflow boundary condition line, and add normal depth boundary condition lines at the outer edge of the boundary.

Updated.

OK.

- Model 202: Revise the outflow boundary line placement to cover the full floodplain extents. Suggest looking at 0.2% storm event WSEL grid for reference.

Updated.

OK.

- All models: the normal depths have the same slope of 0.001  entered for all BC lines and outfall. Please verify and revise accordingly. The slopes will vary based on the terrain.

Slopes have been updated accordingly.

OK.

- Inflow DSS links are not connected within flow file. Show a graph of all events in the report.

Inflows are now connected. Because peak discharges were already included in the results section of the report, we decided to not include graphs in the reports.

OK.

# 5 – Hydraulic Model Execution

## Methodologies / Data Sources (Modeler to populate)

| Unsteady Plan Names |  |  |  |
| --- | --- | --- | --- |
| 10% ACE (10-year): | 10PCT | 0.2% ACE (500-year): | 002PCT |
| 4% ACE (25-year): | 04PCT | 1% plus: | 01PCT_plus |
| 2% ACE (50-year): | 02PCT | 1% minus: | 01PCT_minus |
| 1% ACE (100-year): | 01PCT |  |  |
| Other: |  |  |  |

## Statistical Data (Modeler to populate)

| Time Step: | 1 sec – 60 sec (Time Divisors) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Courant Value: | ~<2, Max 5 |  | Simulation Length: | 144 | hours |
| Computation Interval: | 60 | seconds | Model Runtime: | ~6 – 10 hr /model | hours |
| Hydrograph Output Interval: | 15 | minutes | Mapping Output Interval: | 15 | minutes |

## Technical Review (Reviewer to confirm with Pass/Fail)

| Execution Review |  |
| --- | --- |
| P | Appropriate 2D parameters are used (Boundary Conditions, Time Step, Simulation Length, etc.) |
| P | Hydrograph Output Interval and Mapping Output Interval set at a reasonable interval to capture peaks |
| P | Model runtime is reasonable (< 12 hours, but dependent upon model size) |

## Review Comments

- No comments.

# 6 – Hydraulic Model Results

## Methodologies / Data Sources (Modeler to populate)

| Flow Validation/Gage Analysis |  |
| --- | --- |
| Software: | No Gages present within watershed |
| Methodology |  |
| Removal of Outliers and Justification |  |
| Validation Notes (describe parameter adjustments): |  |
| Other: |  |

## Technical Review (Reviewer to confirm with Pass/Fail)

| Computation Checks |  |
| --- | --- |
| P | Model runs with no cell errors |
| P | Volume balance is within tolerance (ΔV < 1% is ideal, however 2% to 3% can be acceptable for rain on mesh) |
| Results Spot Checks Check at all critical structures, major confluences, a handful of headwater tributaries, downstream end, at benchmarks, and a handful of randomly selected locations. |  |
| P | No visible spikes in WSEL visibly present with a single cell |
| P | WSEL and depth grids appear reasonable (no artificial dips or spikes) |
| P | Visual verification flooding is connected downstream for all significant contributing areas (> 64 acres) |
| P | Velocity grids appear reasonable (no unusually high velocity areas or Courant values greater than 2) |
| P | No crossing profiles between all events |
| P | Inflow and outflow hydrograph timing is reasonable as compared to the peak |
| P | Spills into/from adjacent HUCs appropriately accounted for |
| F | WSEL tie-in between model domains and HUCs along major rivers/streams where inflows are applied |
| Results Full Checks |  |
| P | Computational time step meets ideal Courant condition |
| P | Diffusion Wave: V∆T∆X ≤2 (Up to 5) (Results are unit-less) |
| P | Model simulation is long enough to pass the entire hydrograph through the model (minimum of 12 hours past the peak, preferably 24 hours) |
| P | Hydrographs at internal and external boundaries are smooth |
| P | Contributing areas are contained with 2D mesh or accounted for with inflow hydrographs |
| F | No flow trapped at 2D boundary |
| F | Visual verification mapping is reasonable compared to effective |
| Flow Verification |  |
| N/A | Peak flow gage analysis performed on all gages in the watershed with sufficient data |
| N/A | All gages used have an appropriate period of record (10 to 20+ years) |
| N/A | All gages used have no extensive gaps in record which would impact the analysis |
| N/A | Gage analysis results validated to other sources (Regression, USGS, etc.) |
| N/A | Results at applicable benchmarks justified (gages, HWMs, effective BFEs) |
| P | Peak discharges comparable to gages, effective/prior studies, regional regression equations |

## Review Comments

- Unable to validate timing as the inflow and outflow hydrograph placement needs revision. Based on their current placement, they are fine.

Noted.

OK.

- All tie-ins, WSEL grids, courant, crossing profiles, and velocities will need to be verified again as the model will change after addressing the comments from checkpoints 1 through 5.

Noted.

Model 201 does not have results for the 100yr storm event. Could not verify courant, crossing profiles or velocities

Upon review, the 100-year event may have been corrupted during the submittal process as the original files contained this information. A revised model has been resubmitted, which includes the current plan.

Model is still corrupted and not showing results.

Multiple attempts have been made, which includes downloading the modeling from the links previously provided. These attempts, both by FTN and FNI personnel, lead to complete model downloads and no files missing or corrupted. As no changes have been made to the modeling, the model links are considered valid, as we cannot duplicate the corrupted file comments.

- Spills into adjacent HUC are not accounted for. Please coordinate with the surrounding HUCs to ensure the spills coming in and going out from this HUC are accounted for in order to produce seamless mapping.

Updated.

Spill from same HUC are accounted for. Spill areas between Palo Duro and other HUC 08s is still in question. Everything on the HUC 08 boundaries is set as an outflow with normal depth. Please verify that there are no inflows from external HUC 08s on the northern or southern boundaries that need to be accounted for to provide seamless mapping.

Coordination with adjacent HUC08 watershed modelers have been performed. As part of this process, there were no significant inflows founds. Boundary conditions have been left as is.

Okay.

- The WSEL is within the appropriate range but the floodplain width is not aligned. Please refer to my comment under hydraulics above to overlap the mesh to create a seamless floodplain. Few examples shown below:

Updated.

Model 201 does not have results for the 100yr storm event. Used the grids in the map package to run a subtraction. Most areas look like they are tying in. Suggest running a subtraction to verify and checking with correct depths removed.

Per the comment, the subtraction calculation was performed. As a result of this calculation, the mapped results listed tie-in within the 0.5 feet threshold.

Please see included shapefile “QC_Shapefile.shp” for areas where the inundation areas are not tying in. A couple of examples are shown below as well:

Areas are within the 0.5 ft tolerance. Mapping has been manually adjusted for tie-in appearance.

- The stage from the inflow hydrograph for the BC line along the over banks is jumping pretty high within the first few seconds. See the stage and flow hydrographs  under geometry. For example, see below for one of the hydrographs which shows approximately 1-foot elevation change immediately. Please ensure appropriate application of the restart file is being used or remove restart file.

Restart file is being used appropriately. The restart file assigns every cell to a single water surface elevation based off the Hot Start model run. This is why there is a sudden jump in water surface elevation.

OK.

- Please provide effective mapping panels to validate against BLE mapping.

Data should be found in the map package under

Panels were in map package, but not S_FLD_HAZ_AR. Downloaded recent mapping, and it is reasonable in comparison. If there are paper panels, please verify against those as well.

Noted.

All bounding panels are in progress. Resolved.

- No gages present or data provided to validate flows. If no gages are available, please provide available data/studies (FIS, local studies, Regression) to validate model flows.

Model compared to regression and FIS. Details can be found in the report.

OK.

- New comment based on updated modeling: Velocities – In model 203 at Bivins Lake, there are velocities upwards of 1,000 ft/s

Upon review, the high velocities occur near the end of the simulation once the modeled time steps begin to increase and after the lake has hit the max WSE values. Adjustments to the modeled simulation time steps have been made to reduce these high values.

The high velocities may be an indication that the model is unstable at the end of the simulation but haven’t been able to view the results. Provide files to review these results.

Multiple attempts have been made, which includes downloading the modeling from the links previously provided. These attempts, both by FTN and FNI personnel, lead to complete model downloads and no files missing or corrupted. As no changes have been made to the modeling, the model links are considered valid, as we cannot duplicate the corrupted file comments.

# 7 – Mapping Results

## Technical Review (Reviewer to confirm with Pass/Fail)

| Hydraulic Model |  |
| --- | --- |
|  | HEC-RAS model with results, pertinent data and required storm events |
| Flood Risk Database |  |
| F | All features fully populated per current FEMA guidelines |
| F | Mapping consistent between grids to polygons for all three required mapped events |
| P | 10yr smaller than 100yr smaller than 500yr |
| P | Holes filled in polygon, depth and WSE grids |
| F | Boundary interfaces align matching or identified as AOMI |

## Review Comments

- Comments provided in the shapefile “ QC_Comments”

Noted.

- Check the tie-ins as mentioned in the Hydraulic comments section.

Areas are within the 0.5 ft tolerance. Mapping has been manually adjusted for tie-in appearance.

- Unable to verify against raw mapping as the model doesn’t show results.

FTN and FNI have both downloaded the data submitted and no plans appear to be missing. It should be noted that the input and output folders will need to be combined for the results to show up.

- 100-yr and 500-yr are not tying in in several places, few examples are shown below. Please see shapefile “QC_comments.shp”for all areas.

Updated. Areas are within the 0.5 ft tolerance. Mapping manually adjusted for tie-in appearance.

# 8 – Completeness Review

## Technical Review (Reviewer to confirm with Pass/Fail)

| File Structure and Naming |  |
| --- | --- |
|  | Deliverables provided in TWDB/FEMA required file folder structure and naming convention |
|  | Data missing (if any) documented in report and ReadMe file within the required folder |
| Report |  |
|  | Technical 2D BLE report documents all assumptions, special considerations, etc. |
|  | Flow validation and difference discussed in report |
|  | Flow comparisons discussed in report |
| Hydrologic Model |  |
|  | HEC-HMS model with results, pertinent data and required storm events |
| Hydraulic Model |  |
|  | HEC-RAS model with results, pertinent data and required storm events |
| Flood Risk Database |  |
|  | All features fully populated per current FEMA guidelines |
| CNMS |  |
|  | Updated correctly with new centerlines |
|  | A1-A5 checks complete |
|  | CNMS Post- Processing Complete |
| HAZUS |  |
|  | Run per FEMA guidance |

## Review Comments

Folder Structure comments:

- Add a pdf of the report in the “Reporting and Documentation” folder

Updated.

- Please remove one of the “Supplemental_Folder” under geospatial data since there are two.

Updated.

PaloDuro_BLE_Database.gdb comments:

- In DTL_STUD_AR, CASE_NO field is not populated

Updated.

- In DTL_STUD_LN, CASE_NO field is not populated

Updated.

- In FLD_HAZ_AR, add leading zeros to the EST_AR_ID field

Updated.

- In the same file, the coded value didn’t work for DUAL_ZONE and needs to be populated with “False (No)”

Updated.

- In TENPCT_FP, EST_ID, V_DATUM, LEN_UNIT, ZONE_SUBTY, AR_REVERT, and AB_AUBTRV are unpopulated

- In S_AOMI_Ar, AOMI_ID is not populated for most features. Please add polygons around the HUC boundary for areas that spill outside the HUC 8.  See few examples in QC_Comments.shp.

Updated.

- WTR_AR,  TENPCT_FP, DTL_STUD_AR, DTL_STUD_LN, S_POL_AR, S_HUC_AR,  and S_FRAC_Ar   When zoom to full extents its not zooming to full extents. Check there are no blank polygons added and that it zooms to the correct extents.

Shapefiles updated and should have the correct extent.

- In S_FRAC_Ar, CASE_NO field is not populated.

Updated.

- S_FRAS_Pt: Feature class is empty

Shapefile is not required.

QA/QC Checklist Approval

This 2D Base Level Engineering Analysis QA/QC Review is in compliance with the contract requirements and all task checkpoints are complete.  The independent QC Team has reviewed the analysis, presented review comments to the Production Team Task Leader, and discussed any problems or issues.  The Task Leader, QC Manager, QC Reviewer, and Project Manager have signed the QA/QC Checklist to confirm that all comments have been received, addressed, and documented appropriately.

| QA/QC Team – [TWDB or company name] |  |  |  | Production Team – [add company name] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| QC Manager |  | Date |  | Task Lead |  | Date |
| QC Reviewer |  | Date |  | Project Manager |  | Date |