DRAFT August 2023

Version 3.0

Base Level Engineering Report

Palo Duro (HUC8 11120102)

Report Prepared by:

Freese & Nichols, Inc Team

FTN Associates, Ltd.

3 Innwood Circle, Suite 220

Little Rock, AR, 72211

Report Prepared for:

Texas Water Development Board

1700 North Congress Avenue

Austin, TX 78701

Document History

Document Location

Enter MIP URL (K: Drive Location) here for future use.

Revision History

| Version Number | Version Date | Summary of Changes | Author |
| --- | --- | --- | --- |
| 1.0 | June 6, 2023 | First draft to TWDB for review | FTN/FNI |
| 2.0 | July 21, 2023 | Revised for backcheck submittal | FTN/FNI |
| 3.0 | August 11, 2023 | Revised for backcheck submittal | FTN/FNI |

Table of Contents

Executive Summary	1

1.0 Introduction	3

2.0 2D BLE Modeling Inputs and Controls 	5

2.1 Topographic Data	5

2.1.1 Inventory	5

2.1.2 Evaluation	6

2.1.3 Data Development Methodology	7

2.1.4 DEM QA/QC	7

2.2	2D BLE Methods	8

2.2.1 Model Areas	8

2.2.2 Model and Boundary Condition Setup	9

2.2.3 2D Computational Mesh and Settings	11

2.2.4 Hydrology	11

2.2.5 Hydraulics	14

2.3 Model Results	17

2.3.1 Gage Analysis	17

2.3.2 Regression Analysis	17

2.3.2 Sensitivity Analysis	20

2.3.3 Frequency Event Results	20

2.3.4 Challenges	22

2.3.5 Recommendations	23

3.0 Quality Control	23

4.0 Floodplain Delineation (10%, 1% and 0.2% events)	23

5.0 CNMS Validation of Effective Zone A Special Flood Hazard Area (SFHA)	24

References	25

List of Tables

Table 1: Source Topographic Data (DEMs) Leveraged for Palo Duro Watershed	6

Table 2: Palo Duro Model Areas	8

Table 3: HEC-HMS Frequency Storm Parameters	12

Table 4: PaloDuro_01_02_UP– Precipitation-Frequency Depths	12

Table 5: PaloDuro_01_02_DOWN – Precipitation-Frequency Depths	13

Table 6: PaloDuro_03– Precipitation-Frequency Depths	13

Table 7: Land Use-Soils-CN Matrix	14

Table 8: NLCD 2019 Manning’s Roughness	15

Table 9: Regression Equations	18

Table 10: Regression Equations – Calculated Results	18

Table 11: Regression Equations – Model Results	18

Table 12: Frequency Event Peak Discharge Summary (cfs)	20

Table 13: Detail Study Area – Model Comparison	21

List of Figures

Figure 1: Palo Duro Watershed HUC-10 Basins	3

Figure 2: Palo Duro Watershed BLE Source Terrain Data	5

Figure 3: Palo Duro Model Domain	8

Figure 4: Palo Duro Creek Inflow to PaloDuro_01_02_Down Model	9

Figure 5: North Palo Duro Creek Inflow to PaloDuro_01_02_Down Model	9

Figure 6: Palo Duro Creek Inflow to PaloDuro_03 Model	9

Figure 7: Palo Duro Watershed Breaklines	15

Figure 8: Terrain Modification Example	15

Figure 9: Regression Reference Lines in Palo Duro Watershed	18

Figure 10: Comparison at Regression Locations	18

Figure 11: Drainage Areas vs. Discharges	21

# Executive Summary

The Texas Water Development Board (TWDB) contracted with the Freese and Nichols, Inc. (FNI) team in September 2021 to develop Base Level Engineering for multiple HUC8 Watersheds within the State. FTN Associates, Ltd., as a subcontractor and member of the FNI Team, will complete a Base Level Engineering (BLE) analysis for the Palo Duro HUC-8 in Northwest Texas, to support FEMA’s Discovery process and validation of effective Zone A Special Flood Hazard Areas (SFHAs). This BLE study will provide significant data for several Texas counties previously lacking modernized flood models.

The BLE process involves using best available data and incorporating automated techniques with traditional model development procedures to produce regulatory-quality flood hazard boundaries for the 1-percent annual chance event as well as estimates of flood hazard boundaries for multiple recurrence intervals.

The source digital terrain data used for surface model development in support of hydrologic and hydraulic analysis as well as mapping activities were leveraged from various local, State, and Federal partners.

Flood discharges and water surface elevations for this study were calculated by utilizing the rain-on-grid two-dimensional (2-D) modeling capabilities of the Hydrologic Engineering Center’s River Analysis System (HEC-RAS), which translates rainfall into runoff based on an underlying terrain and various other physically based parameters. 2D HEC-RAS models were developed in accordance with the 2019 FEMA Region 6 Base Level Engineering Submittal Guidance, which relies heavily on HEC-RAS River Analysis System, 2D Modeling Users’ Manual 5.0 (Bruner, 2016). All models were developed in the HEC-RAS program version 6.1.0.

The stream mile network that was validated for these watersheds was compiled using FEMA’s Community Needs Management Strategy (CNMS) inventory. CNMS is an inventory of flood hazard studies and flood hazard mapping needs for areas where a study is needed. This data is helpful for community officials in analyzing and depicting flood hazards to enhance the understanding of flood risks. Communities may use this information to make informed decisions on their planning and flood mitigation efforts. Stream miles from FEMA’s Community Needs Management Strategy (CNMS) inventory were not reviewed as part of this submittal and will be validated for these watersheds at a later date

# 1.0 Introduction

Recent innovations and efficiencies in floodplain mapping have allowed the U.S. Department of Homeland Security’s Federal Emergency Management Agency (FEMA) to develop a process called Base Level Engineering (BLE), which can be used to address current program challenges, including the validation of Zone A studies and the availability of flood risk data in the early stages of a Flood Risk Project. The BLE process involves using best available data and incorporating automated techniques with traditional model development procedures to produce regulatory quality flood hazard boundaries for the 1-percent annual chance event as well as estimates of flood hazard boundaries for multiple recurrence intervals. The cost for developing the data and estimates resulting from the BLE process are lower than standard flood production costs. The BLE results may be used for eventual production of regulatory and non-regulatory products.

As described in Title 42 of the Code of Federal Regulations, Chapter III, Section 4101(e), once every five years, FEMA must evaluate whether the information on Flood Insurance Rate Maps (FIRMs) reflects the current risks in flood prone areas. FEMA makes this determination of flood hazard data validity by examining flood study attributes and change characteristics, as specified in the Validation Checklist of the Coordinated Needs Management Strategy (CNMS) Technical Reference. The CNMS Validation Checklist provides a series of critical and secondary checks to determine the validity of flood hazard areas studied by detailed methods (e.g., Zone AE, AH, or AO). While the critical and secondary elements in CNMS provide a comprehensive method of evaluating the validity of Zone AE studies, a cost-effective approach for evaluating Zone A studies has been lacking.

In addition to the need for Zone A validation guidance, FEMA standards require flood risk data to be provided in the early stages of a Flood Risk Project. FEMA Program Standard SID #29 requires that during Discovery, data must be identified that illustrates potential changes in flood elevation and mapping that may result from the proposed project scope. If available data does not clearly illustrate the likely changes, an analysis is required that estimates the likely changes. This data and any associated analyses should be shared, and results should be discussed with stakeholders. An important goal of the BLE process is the scalability of the results. Scalability means that the results of a BLE analysis can not only be used for CNMS evaluations of Zone A studies but can also be leveraged throughout the Risk MAP program. The data resulting from a BLE analysis can be updated as needed and used for the eventual production of regulatory and non-regulatory products, outreach and risk communication, and MT-1 processing. Leveraging this data outside the Risk MAP program may also be valuable to external stakeholders.

TWDB contracted the Freese and Nichols team to complete a BLE analysis for the Palo Duro Watershed in Northwest Texas to support FEMA’s Discovery process and validation of effective Zone A Special Flood Hazard Areas (SFHA). The study extents include portions of Curry, Deaf Smith, Oldham, Potter, Quay, and Randall Counties, and include the following communities: the City of Canyon and the Town of Vega. The study area consists of three HUC-10 basins: Upper Palo Duro Creek, North Palo Duro Creek, and Lower Palo Duro Creek. Figure 1 shows the location of the Palo Duro HUC-10 basins with respect to the counties and communities mentioned above.

Figure 1: Palo Duro Watershed HUC-10 Basins

The Base Level Engineering assessment prepared cursory engineering analysis for approximately 257 miles of stream reaches within the Palo Duro Watershed with a minimum drainage area tolerance of 0.1 square mile. The Palo Duro Watershed contains distinct topography for its watershed. The distinct topography correlates to unique flood infrastructure for each of the subregions. The watershed is made up of a combination of playas and riverine characteristics (e.g., comprised primarily of rivers and streams). Due to the varied natural infrastructure, stream reach delineations were only extended through the riverine subregions.  The playa lakes are generally hydraulically disconnected, and therefore, stream reaches were not created for these features. These regions of the watershed are colloquially referred to as “on the caprock” to describe the geographical transition. The BLE analysis was completed using a 2D grid to represent the watershed and therefore the entire watershed was included in hydrologic and hydraulic modeling. The following sections will summarize the BLE process and will discuss the results along with their recommended use.

# 2.0 2D BLE Modeling Inputs and Controls

Section 2.0 discusses the fundamental components required to execute a 2D BLE analysis for the Palo Duro (HUC 11120102) Watershed. Inputs including elevation data, hydrology from rain-on-grid and inflow hydrographs, and hydraulic analyses and variables are defined in the following sections. The HEC-RAS 6.3.0 rain-on-grid utility was applied in the production of these Texas 2D BLE products.

## 2.1 Topographic Data

A high-resolution Digital Elevation Model (DEM) derived from Light Detection and Ranging (LiDAR) provides a detailed representation of the Earth’s surface. DEMs are one of the fundamental components for 2D engineering analyses and are used to direct the hydrology through the model area. The seamless DEMs developed to support the 2D BLE modeling and analysis, within the Palo Duro watershed, were executed using the following steps:

- Available 1-meter DEMs within each watershed were gathered from the Science Base Catalog and USGS National Map;

- DEMs were reprojected into NAD 1983 State Plane Texas North (EPSG 2275), re-sampled to 10-foot resolution, and mosaicked with GDAL, GRASS, and ArcGIS geoprocessing tools; and,

- Quality Assurance (QA) and Quality Control (QC) measures were implemented to ensure topographic data meets quality standards of the project.

Additional documentation regarding DEMs including coverage, accuracy, acquisition dates, and source contact/agency are presented in the figures, tables, and text in the sections below.

### 2.1.1 Inventory

DEMs were inventoried for the Palo Duro Watershed, including buffer of at least 1000-foot around the watershed boundary. Figure 1 depicts the datasets identified and leveraged for the watershed. Data from FEMA and USGS were queried to build the inventory with the most current and available data resources.

Figure 2: Palo Duro Watershed BLE Source Terrain Data

### 2.1.2 Evaluation

A data coverage assessment was conducted to check for data gaps, extent, accuracy, and completeness. A review of related documentation, reports, indexes, and metadata associated with the leveraged datasets ensured each dataset meets FEMA accuracy requirements for topographic data as defined in the FEMA 2020 BLE Region 6 Submittal Guidance document. Table 1 presents the complete list of source elevation data and attributes leveraged for the Palo Duro watershed.

Table 1: Source Topographic Data (DEMs) Leveraged for Palo Duro Watershed

| Counties | Year | Description | Data Type | 3DEP Quality Level | Vertical Accuracy (RMSE) | Source/Owner | Approximate Footprint (Sq. Mi.) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Curry | 2016 | Roosevelt and Curry Counties, NM LIDAR | Airborne LiDAR | 2 | 11.9 cm | USGS | 291 |
| Quay | 2017 | NRCS-FEMA Northeast NM LIDAR | Airborne LiDAR | 2 | 9.8 cm | USGS | 28 |
| Deaf Smith, Oldham, Potter, and Randall | 2018 | TX Panhandle LiDAR | Airborne LiDAR | 2 | 7.2 cm | USGS | 1491 |

#### 2.1.2.1 Watershed Source Terrain Data

The source elevation data for the Palo Duro watershed were 1-meter DEMs derived from the 2018 FEMA Texas Panhandle LiDAR data, 2017 NRCS Northeast New Mexico LiDAR, and 2016 Roosevelt & Curry Counties, NM LiDAR. Figure 2 depicts the extent of the data defined in Table 1.

2018 Texas Panhandle LiDAR

The 2018 FEMA Texas Panhandle LiDAR was acquired between February 9, 2018, to September 3, 2019. The vertical accuracy for the dataset is reported to be 7.2 cm RMSEz at a 95% confidence level, which meets project accuracy specifications of the National Standard for Spatial Data Accuracy (NSSDA). These accuracies meet FEMA standards for floodplain reevaluation studies and map modernization programs designed to update the Flood Insurance Rate Maps (FIRM).

2017 NRCS-FEMA Northeast NM LiDAR

The 2017 New Mexico NRCS-FEMA Northeast NM LiDAR was acquired between November 20, 2017, and February 11, 2018. The vertical accuracy for the dataset is reported to be 9.8 cm RMSEz at a 95% confidence level, which meets project accuracy specifications of the National Standard for Spatial Data Accuracy (NSSDA). These accuracies meet FEMA standards for floodplain reevaluation studies and map modernization programs designed to update the Flood Insurance Rate Maps (FIRM).

2016 Roosevelt & Curry Counties, NM LiDAR

The 2015 Roosevelt & Curry Counties, NM LiDAR was acquired between September 30, 2015, and March 25, 2016. The vertical accuracy for the dataset is reported to be 11.9 cm RMSEz at a 95% confidence level, which meets project accuracy specifications of the National Standard for Spatial Data Accuracy (NSSDA). These accuracies meet FEMA standards for floodplain reevaluation studies and map modernization programs designed to update the Flood Insurance Rate Maps (FIRM).

### 2.1.3 Data Development Methodology

DEMs were combined into a single mosaic file for each collection that represented the footprint of each data source within the project area. Tools within ArcGIS were used to apply horizontal projection transformations and vertical datum shifts. The DEMs were projected horizontally to North American Datum of 1983 (NAD83), State Plane Coordinate System (SPCS) Texas North (4201) in feet. Additionally, all DEMs were adjusted vertically to NAVD88 in feet.

### 2.1.4 DEM QA/QC

#### 2.1.4.1 DEM Quality Assurance

Seamless DEMs used in the BLE analyses were developed and independently assured to meet quality assurance (QA) standards of the project. Quality assurance during the data development process included, but was not limited to:

- Horizontal Projection Check (NAD83 SPCS 4202 FT)

- Vertical Datum Check (NAVD88 FT)

- Resolution Check (1-meter to 10-foot resolution)

- Seamless Data Check to ensure the DEM files are consistent and seamless along source data edges

#### 2.1.4.2 DEM Quality Control

Quality control (QC) measures were implemented following the development process of the seamless DEMs and included visual observations using hillshade, color rendering, and/or other visual aids to review and identify potential impactful anomalies within the DEM surfaces.  This QC step included, but was not limited to:

- Seamless Data Check to ensure no voids along the edges and between the prioritized datasets.

- NoData Value Check to ensure no null values.

- Manual Elevation Check using hillshade rasters to find erroneous elevation issues.

Following these QA/QC steps, the final DEM data developed for the Palo Duro Watershed are assured to meet FEMA standards and present a representative surface developed from leveraged elevation data for the purposes of this BLE project.

Stream centerlines were delineated to 0.1 square mile within each watershed and were cutoff at the caprock and/or first playa. Stream centerlines were smoothed and generalized.

## 2.2	2D BLE Methods

The following sections describe the methodology used in the 2D BLE analysis of the Palo Duro Watershed. This section includes discussion regarding the overall approach along with hydrologic and hydraulic methodologies and results.

### 2.2.1 Model Areas

The Palo Duro Watershed covers a total area of approximately 999 square miles. While possible to model the entire HUC8 in a single 2D HEC-RAS model, the large area would lead to excessively long model run times and inhibit the model’s usability for future users. To combat this issue and simplify model development, the study team divided the Palo Duro HUC8 into several smaller models, consisting of a single HUC10 or group of HUC10s within the larger HUC8. It was decided to combine the two upstream HUC10 watersheds (1112010201 and 1112010202) due to water artificially building up at the boundary between the two models. Since the two HUC10 watershed run parallel to each other, external boundary conditions couldn’t be placed between the two. The combined model was then split into an upstream and downstream model to reduce run times. Table 2 and Figure 3 present the grouping of HUC10s for each 2D model. As described in Section 2.2.2 Model and Boundary Condition Setup, these models remain interconnected via external boundary conditions, so no runoff volume is lost through the subdivision of the HUC8.

Table 2: Palo Duro Model Areas

| Model | HUC10(s) | HUC10 Name(s) | Area (Sq. Mi.) |
| --- | --- | --- | --- |
| PaloDuro_01_02_UP | 1112010201,1112010202 | North Palo Duro Creek, Upper Palo Duro Creek | 496 |
| PaloDuro_01_02_DOWN | 1112010201,1112010202 | North Palo Duro Creek, Upper Palo Duro Creek | 320 |
| PaloDuro_203 | 1112010203 | Lower Palo Duro Creek | 185 |

Figure 3: Palo Duro Model Domain

### 2.2.2 Model and Boundary Condition Setup

HEC-RAS 2D rain-on-grid modeling requires two main components, a 2D computational mesh and inflow hydrology. The 2D computational mesh boundaries were established within the Palo Duro HUC8 watershed according to the groupings presented in Table 2. Inflow hydrology can be applied through various methods in HEC-RAS 2D modeling, but hydrology methods are limited to two categories for the Palo Duro HUC8. These are flow hydrographs and precipitation time-series. Section 2.2.4.1 Precipitation for HEC-RAS 2D describes the development the precipitation data applied to each model. Precipitation, rather than excess precipitation, was applied directly to the 2D mesh because simplistic hydrologic losses can be accounted for entirely within HEC-RAS version 6.3.0.

A normal depth boundary condition was utilized along all basin outflows that uses an approximate energy grade-line slope estimated from the LiDAR terrain data but were adjusted as necessary to ensure a tie-in within 0.5 ft of the surrounding models.

External boundary conditions were applied along each model area’s boundary where a transfer of flow (inflow or outflow) may occur. The upstream area of each model, where applicable, has an inflow hydrograph applied that accounts for outflow from the upstream model area. For example, the PaloDuro_01_02_Down model has an inflow hydrograph at its upstream end that applies the outflow from the PaloDuro_01_02_Up model. Figures 4 – 6 show the major inflows (Palo Duro Creek and North Palo Duro Creek) applied within the models.

Figure 4: Palo Duro Creek Inflow to PaloDuro_01_02_Down Model

Figure 5: North Palo Duro Creek Inflow to PaloDuro_01_02_Down Model

Figure 6: Palo Duro Creek Inflow to PaloDuro_03 Model

### 2.2.3 2D Computational Mesh and Settings

The HEC-RAS 2D computational mesh defines both the extent and level of detail of 2D hydraulic results. The density and detail of the 2D mesh is the primary component that defines the resolution of hydraulic results (e.g., more detail-finer results, less detail-coarser results). A more detailed mesh may provide finely resolved and accurate results, but it can lead to excessively long computation times which are prohibitive to future use of BLE models. To provide a balance between mesh detail and model performance, the base mesh for each model was developed using 200’x200’ cells. The mesh was refined through the addition of breaklines along roadways, reservoirs, stream centerlines, and other topographically significant features. The mesh for each model area was developed in a manner that allows for automatic regeneration of the mesh by future users, without the need for manual cell edits.

Hydraulic results are also influenced by the many computational settings used in each HEC-RAS simulation. Of the available settings, the two most influential to results are the computational timestep and the equation set. Models use the “Adjust Time Step Based on Time Series of Divisors” function. The simulation begins with a timestep of 60 seconds and is lowered to 15 seconds when rainfall begins to rise. Each model applies the Diffusion Wave equation set. All other computational settings were kept at their default values as sensitivity analyses were conducted early in the modeling process and showed little to no impact on model results.

### 2.2.4 Hydrology

Hydrologic data for the Palo Duro Watershed was derived from NOAA Atlas 14, Volume 11 Version 2.0 precipitation data, which was obtained from NOAA’s Precipitation Frequency Data Server. Spatial variation of precipitation depths was accounted for by developing area-weighted average depths for each model area presented in Table 2. For contributing basins outside of the study area, inflows were applied through an external boundary condition at the upstream boundary Palo Duro watershed. The flow hydrograph for the contributing basin was obtained from the 2D BLE model of the respective watershed. The 10%, 4%, 2%, 1%-minus, 1%, 1%-plus, and 0.2% annual exceedance probability events were simulated in the Palo Duro Watershed.

#### 2.2.4.1 Precipitation for HEC-RAS 2D

The average NOAA Atlas 14 precipitation-frequency depths for each model area in Table 2 were converted to a precipitation time-series dataset within a simplified HEC-HMS v4.8 model.  Default parameters were assigned to a single basin to extract the precipitation time-series data (infiltration was not accounted for in the rainfall data because it will be calculated within the hydraulic models). A frequency storm was created within HEC-HMS for each event listed in Section 2.2.4 Hydrology. Each frequency storm utilized the parameters listed in Table 3. The precipitation-frequency depths for each model area and simulation event are summarized in Tables 4 -6. A precipitation time-series was then extracted from the HEC-HMS models and utilized in the HEC-RAS 2D simulations.

Areal reduction was not utilized within this study. Although using rainfall reduction may be appropriate when computing flows at the outlet of a HUC-8 or the total contributing drainage area to a gage, reducing the rainfall applied over the model meshes would lead to an underestimation of the flood risk in the upland portions of the models.

In addition to the recurrence interval precipitation depths, NOAA Atlas 14 also provides 90% confidence limits for each reported precipitation depth. The 90% confidence intervals correspond to +/- 1.645 standard deviations, which was determined using the probability table for the standard normal distribution. The 1%-minus and 1%-plus event depths utilize the following equation:

Table 3: HEC-HMS Frequency Storm Parameters

| Parameter | Value |
| --- | --- |
| Annual-Partial Conversion | None |
| Storm Duration | 1-day |
| Intensity Duration | 5-minute |
| Intensity Position | 50% |
| Area Reduction | None |
| Storm Area | - |

Table 4: PaloDuro_01_02_UP– Precipitation-Frequency Depths

| Duration (min) | Percent Annual Chance Precipitation Depth (in) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10% | 4% | 2% | 1%- | 1% | 1%+ | 0.2% |
| 5 | 0.60 | 0.74 | 0.84 | 0.74 | 0.95 | 1.16 | 1.23 |
| 15 | 1.19 | 1.45 | 1.65 | 1.45 | 1.87 | 2.28 | 2.42 |
| 60 | 1.99 | 2.43 | 2.77 | 2.44 | 3.13 | 3.83 | 4.08 |
| 120 | 2.34 | 2.87 | 3.30 | 2.95 | 3.75 | 4.56 | 4.93 |
| 180 | 2.52 | 3.12 | 3.60 | 3.25 | 4.12 | 4.99 | 5.44 |
| 360 | 2.87 | 3.55 | 4.11 | 3.76 | 4.72 | 5.68 | 6.29 |
| 720 | 3.23 | 3.98 | 4.58 | 4.21 | 5.24 | 6.27 | 7.05 |
| 1440 | 3.65 | 4.47 | 5.12 | 4.72 | 5.83 | 6.93 | 7.86 |

Table 5: PaloDuro_01_02_DOWN – Precipitation-Frequency Depths

| Duration (min) | Percent Annual Chance Precipitation Depth (in) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10% | 4% | 2% | 1%- | 1% | 1%+ | 0.2% |
| 5 | 0.62 | 0.74 | 0.84 | 0.73 | 0.94 | 1.16 | 1.21 |
| 15 | 1.21 | 1.47 | 1.66 | 1.44 | 1.86 | 2.27 | 2.38 |
| 60 | 2.05 | 2.48 | 2.80 | 2.44 | 3.14 | 3.85 | 4.05 |
| 120 | 2.42 | 2.96 | 3.38 | 3.00 | 3.83 | 4.66 | 4.99 |
| 180 | 2.63 | 3.24 | 3.73 | 3.34 | 4.24 | 5.15 | 5.57 |
| 360 | 3.00 | 3.71 | 4.29 | 3.89 | 4.91 | 5.92 | 6.49 |
| 720 | 3.38 | 4.16 | 4.79 | 4.38 | 5.47 | 6.56 | 7.26 |
| 1440 | 3.81 | 4.66 | 5.34 | 4.90 | 6.06 | 7.23 | 8.05 |

Table 6: PaloDuro_03– Precipitation-Frequency Depths

| Duration (min) | Percent Annual Chance Precipitation Depth (in) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10% | 4% | 2% | 1%- | 1% | 1%+ | 0.2% |
| 5 | 0.61 | 0.74 | 0.85 | 0.74 | 0.95 | 1.16 | 1.22 |
| 15 | 1.22 | 1.47 | 1.67 | 1.45 | 1.87 | 2.29 | 2.40 |
| 60 | 2.09 | 2.53 | 2.87 | 2.50 | 3.22 | 3.94 | 4.15 |
| 120 | 2.49 | 3.03 | 3.46 | 3.07 | 3.92 | 4.77 | 5.11 |
| 180 | 2.71 | 3.33 | 3.81 | 3.41 | 4.33 | 5.25 | 5.68 |
| 360 | 3.11 | 3.83 | 4.40 | 3.98 | 5.01 | 6.04 | 6.61 |
| 720 | 3.52 | 4.31 | 4.95 | 4.50 | 5.62 | 6.74 | 7.40 |
| 1440 | 3.97 | 4.84 | 5.53 | 5.06 | 6.26 | 7.46 | 8.19 |

#### 2.2.4.2 Hydrologic Losses

Hydrologic losses were calculated within HEC-RAS according to the SCS Curve Number method. Curve numbers were calculated by intersecting the National Land Cover Dataset (NLCD) 2019 with soils data obtained from the USDA Soil Survey Geographic Database (SSURGO). A curve number was then assigned to each resultant value of the intersected land cover classification and hydrologic soil group, as presented in Table 7. These parameters represent the Curve Numbers (CN) applied to the HEC-RAS 2D models, which correspond to ARC II, or average antecedent moisture conditions. The values presented in Table 7 represent adjusted CN values that were utilized in the 10%, 4%, 2%, 1%, 1% -, 1%+, and 0.2% events. CN values were originally selected using results from a previous study in a nearby watershed (Running Water Draw). These values were determined by Soil Conservation Service’s (SCS) Technical Release 55 (TR-55). Since no gages were present in the watershed, FTN coordinated with FNI to calibrate the models by adjusting CN values as necessary, so that gages in the downstream watershed could be matched. The initial abstraction ratio for the default CN layer was set to 0.20. The minimum infiltration rate was set to 0 in/hr.

Table 7: Land Use-Soils-CN Matrix

| Land Use Grid Code | NLCD Land Use Description | Hydrologic Soil Group |  |  |  | TR 55 Cover Type and Hydrologic Condition |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | A | B | C | D |  |
| 11 | Open Water | 99 | 99 | 99 | 99 | N/A |
| 21 | Developed Open Space | 39 | 61 | 74 | 80 | Open Space (Good) |
| 22 | Developed Low Intensity | 46 | 65 | 77 | 82 | Residential (2 acres) |
| 23 | Developed Medium Intensity | 61 | 75 | 83 | 87 | Residential (1/4 acre) |
| 24 | Developed High Intensity | 89 | 92 | 94 | 95 | Commercial |
| 31 | Barren Land | 77 | 86 | 91 | 94 | Urban (Newly graded) |
| 41 | Deciduous Forest | 30 | 55 | 70 | 77 | Woods (Good) |
| 42 | Evergreen Forest | 45 | 66 | 77 | 83 | Woods (Poor) |
| 43 | Mixed Forest | 36 | 60 | 73 | 79 | Woods (Fair) |
| 52 | Shrub Scrub | 30 | 48 | 65 | 73 | Brush (Good) |
| 71 | Herbaceous1 | 49 | 62 | 74 | 85 | Herbaceous (Good) |
| 81 | Hay Pasture | 39 | 61 | 74 | 80 | Pasture (Good) |
| 82 | Cultivated Crops | 64 | 75 | 82 | 85 | Row Crop (SR + CR - Good) |
| 90 | Woody Wetlands | 30 | 55 | 70 | 77 | Woods (Good) |
| 95 | Emergent Herbaceous Wetlands | 30 | 58 | 71 | 78 | Meadow |

1TR 55 did not include a value for soil group A. Desert shrub (good) was used for that group instead.

#### 2.2.4.3 Reservoirs

There are no USACE regulated reservoirs in this watershed. The City of Canyon FIS report included an Elevation-Storage-Discharge table for Bivins Lake, which can be found in the most downstream HUC10 (1112010203). It was noted that the emergency spillway had been updated since the effective date of the FIS Report, so the rating curve was not included in the 2D/SA connection.  The dimensions of the spillway were confirmed by comparing data found on the TCEQ Central Registry and the terrain. The Spillway includes a 200 ft wide earth cut channel, with a 450 ft and 7 ft tall fuse plug. No details were provided on the central outlet of the dam. While there is no information indicating that the central outlet functions during routine events (FIS text indicates no discharge until the emergency spillway elevation), the small culvert invert has been altered to drain the standing water left after the hot start run in an attempt to start event modeling with no water behind the dam.

All other reservoirs were determined to be not hydrologically significant. Breaklines were placed on top of the dams and terrain modifications were added as necessary.

### 2.2.5 Hydraulics

Hydraulic analysis of the Palo Duro Watershed was conducted in HEC-RAS v6.3.0 using a rain-on-grid modeling approach. This section describes the development of hydraulic input data and its implementation in the 2D model.

#### 2.2.5.1 Roughness Coefficients

A manning’s roughness layer was developed for the 2D mesh by applying typical values associated with NLCD land cover classifications. In addition to the base manning’s roughness layer from the NLCD data, manning’s roughness values were refined along stream centerlines to override excessively high channel roughness values resulting from the NLCD layer. A calibration area was added to the model to incorporate Channel Manning’s Values. An initial buffer of 25 feet was found to be appropriate for most streams but was adjusted as necessary. Channel roughness values were assigned according to a review of aerial imagery and generally range from 0.03 – 0.04. documents the typical roughness values assigned to each NLCD land cover classification, which correspond with the 2019 FEMA Region 6 Base Level Engineering Submittal Guidance document. Table 8 documents the typical roughness values assigned to each NLCD land cover classification, which correspond with the 2019 FEMA Region 6 Base Level Engineering Submittal Guidance document.

Table 8: NLCD 2019 Manning’s Roughness

| Land Use Grid Code | NLCD Land Use Description | Minimum Manning’s N | Maximum Manning’s N | Selected Manning’s N |
| --- | --- | --- | --- | --- |
| 11 | Open Water | 0.025 | 0.05 | 0.03 |
| 21 | Developed Open Space | 0.04 | 0.05 | 0.04 |
| 22 | Developed Low Intensity | 0.06 | 0.12 | 0.06 |
| 23 | Developed Medium Intensity | 0.08 | 0.16 | 0.08 |
| 24 | Developed High Intensity | 0.12 | 0.2 | 0.12 |
| 31 | Barren Land | 0.3 | 0.03 | 0.03 |
| 41 | Deciduous Forest | 0.1 | 0.20 | 0.1 |
| 42 | Evergreen Forest | 0.1 | 0.16 | 0.1 |
| 43 | Mixed Forest | 0.1 | 0.20 | 0.1 |
| 52 | Shrub Scrub | 0.07 | 0.16 | 0.07 |
| 71 | Herbaceous | 0.035 | 0.05 | 0.035 |
| 81 | Hay Pasture | 0.045 | 0.05 | 0.045 |
| 82 | Cultivated Crops | 0.045 | 0.05 | 0.045 |
| 90 | Woody Wetlands | 0.1 | 0.15 | 0.10 |
| 95 | Emergent Herbaceous Wetlands | 0.07 | 0.085 | 0.07 |

#### 2.2.5.2 Breaklines and Hydraulic Structures

Breaklines were used within the 2D mesh to align cell faces along stream centerlines, roadway and railroad embankments, and other hydraulically significant features. Roadway centerlines were initially identified within ArcGIS and then imported into HEC-RAS as breaklines, see Figure 7 for locations of the breaklines.

Figure 7: Palo Duro Watershed Breaklines

#### 2.2.5.3 Initial Conditions

Initial conditions in the Palo Duro Watershed are influenced by the dominance of playa lakes in overland areas along the caprock. The applicability of an initial water surface elevation within the playas was investigated during a sensitivity analysis. The study team referenced City of Lubbock criteria and Playa Lakes in the Southern High Plains: Runoff, Infiltration, and Recharge (Weinberg et.al, 2021) when determining an appropriate initial condition for the playa lakes. The study team created an initial condition run that applies 2” of rainfall that is then allowed to infiltrate and flow completely out of each model area. A restart file was generated at the end of this simulation, which was then applied as the initial conditions of each frequency event simulation.

#### 2.2.5.4 Terrain Modifications

Hydraulically significant culvert crossings at roadways were identified based on embankment height, volume behind the embankment, and contributing area. For hydraulically significant features, terrain modifications were used to burn a channel into the terrain within HEC-RAS, shown in Figure 8. Terrain modification parameters were estimated using aerial imagery.

Figure 8: Terrain Modification Example

## 2.3 Model Results

The results of this study produced an SFHA that provides additional and/or new mapping across most of the Palo Duro HUC8. The estimated SFHA boundaries can serve as an estimate for flood risk in the area, which has not been previously studied. The discharge rates measured at key locations in the watershed compare reasonably well with the stream gage analysis. These results provide context for flood risk within the watershed and should be verified through community meetings before being applied to a regulatory product.

### 2.3.1 Gage Analysis

There were no known gages located within the Palo Duro Watershed, but there are gages present in the downstream HUC08 watershed (Tierra Blanca). FTN coordinated with FNI during the calibration process to ensure discharges in Terra Blanca matched USGS gages.

### 2.3.2 Regression Analysis

To generate a point of comparison, the study team completed a regression analysis according to equations created by USGS listed in Table 9. The equations were derived from “Regression Equations for Estimation of Annual Peak-Streamflow Frequency for Undeveloped Watersheds in Texas Using an L-moment-Based, PRESS -Minimized, Residual-Adjusted Approach” (Asquith and Roussel, 2009). Reference lines (shown in Figure 9) were created in ArcGIS for comparison with results from model runs. Locations of reference lines were determined by terrain data on primarily the mainstream line.

The flows calculated from the regression equations can be found in Table 10 and the modeled results can be found in Table 11. No changes were made according to regression results because downstream gages took precedence over regression equations. Figure 10 shows the comparison between Regression equations and model results for the 1% event. The error bars represent the HEC-RAS 1% Plus and 1% Minus events. It was noted that results for reference lines found upstream in areas dominated by playas had large percent errors and results for reference lines found downstream in areas with riverine characteristics results were very similar to calculated flows (less than a 10% difference). It is likely most of the data used to determine the regression equations was not collected from areas with playas, and therefore is not a good estimate for that type of terrain. Additionally, the terrain type makes it difficult to determine the drainage area, which is one of the parameters in the equation.

Table 9: Regression Equations

| Regression Equation | RSE | Adj. R-squared | AIC statistic | PRESS statistic |
| --- | --- | --- | --- | --- |
| Q10=P1.203S0.403×100.918Ω+13.62-11.97A-0.0289 | 0.25 | 0.89 | 86.5 | 46.6 |
| Q25=P1.140S0.446×100.945Ω+11.79-9.819A-0.0374 | 0.26 | 0.89 | 140 | 49.5 |
| Q50=P1.105S0.476×100.961Ω+11.17-8.997A-0.0424 | 0.28 | 0.87 | 220 | 55.6 |
| Q100=P1.071S0.507×100.969Ω+10.82-8.448A-0.0467 | 0.30 | 0.86 | 320 | 64.8 |
| Q500=P0.988S0.569×100.976Ω+10.40-7.605A-0.0554 | 0.37 | 0.81 | 591 | 98.7 |

RSE, residual standard error in log10-units of cubic feet per second; Adj., adjusted; AIC , Akaike Information Criterion; PRESS , Prediction Error Sum of Squares; QT, peak streamflow for T -year recurrence interval in cubic feet per second; P, mean annual precipitation in inches; S, dimensionless main-channel slope; Ω ,OmegaEM parameter that represents a generalized terrain and climate index that conveys relative differences in peak-streamflow potential across the study area; and A, drainage area in square miles (Asquith and Roussel, 2009).

Table 10: Regression Equations – Calculated Results

| Reference Line | Latitude | Longitude | Drainage Area (Sq. Mi.) | 10% (cfs) |  | 4% (cfs) | 2% (cfs) | 1% (cfs) | 0.2% (cfs) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HUC01 _Reference Line 1 | 34.9604 | -102.908 | 106.8 |  | 5,413 | 8,965 | 12,417 | 16,962 | 32,225 |
| HUC01 _Reference Line 2 | 35.0059 | -102.742 | 181.5 |  | 7,056 | 11,546 | 15,869 | 21,490 | 40,067 |
| HUC02 _Reference Line 1 | 35.134 | -102.842 | 7.0 |  | 512 | 752 | 958 | 1,204 | 1,937 |
| HUC02 _Reference Line 4 | 35.0857 | -102.22 | 285.0 |  | 5,544 | 8,527 | 11,225 | 14,522 | 24,673 |
| HUC03 _Reference Line 1 | 35.0297 | -102.132 | 485.2 |  | 10,157 | 16,256 | 22,026 | 29,387 | 53,134 |
| HUC03 _Reference Line 2 | 35.0601 | -102.053 | 53.1 |  | 3,712 | 6,024 | 8,225 | 11,068 | 20,376 |
| HUC03 _Reference Line 3 | 35.0165 | -101.983 | 548.4 |  | 11,356 | 18,050 | 24,356 | 32,362 | 57,968 |
| HUC03 _Reference Line 4 | 35.0021 | -101.912 | 625.4 |  | 11,625 | 18,459 | 24,891 | 33,044 | 59,107 |

Table 11: Regression Equations – Model Results

| Reference Line | Latitude | Longitude | Drainage Area (Sq. Mi.) | 10% (cfs) |  | 4% (cfs) | 2% (cfs) | 1% (cfs) | 0.2% (cfs) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HUC01 _Reference Line 1 | 34.9604 | -102.908 | 106.8 |  | 2,256 | 3,766 | 5,063 | 6,566 | 11,006 |
| HUC01 _Reference Line 2 | 35.0059 | -102.742 | 181.5 |  | 1,164 | 2,136 | 3,127 | 4,360 | 8,564 |
| HUC02 _Reference Line 1 | 35.134 | -102.842 | 7.0 |  | 2,635 | 3,821 | 5,133 | 6,483 | 10,422 |
| HUC02 _Reference Line 4 | 35.0857 | -102.22 | 285.0 |  | 10,152 | 14,586 | 18,480 | 22,867 | 35,575 |
| HUC03 _Reference Line 1 | 35.0297 | -102.132 | 485.2 |  | 14,119 | 20,331 | 25,764 | 32,088 | 50,988 |
| HUC03 _Reference Line 2 | 35.0601 | -102.053 | 53.1 |  | 2,144 | 3,842 | 5,530 | 7,543 | 13,022 |
| HUC03 _Reference Line 3 | 35.0165 | -101.983 | 548.4 |  | 13,395 | 20,320 | 26,106 | 33,151 | 55,690 |
| HUC03 _Reference Line 4 | 35.0021 | -101.912 | 625.4 |  | 12,858 | 20,963 | 27,087 | 34,160 | 57,885 |

Figure 9: Regression Reference Lines in Palo Duro Watershed

Figure 10: Comparison at Regression Locations

### 2.3.2 Sensitivity Analysis

To evaluate which model parameters significantly impact results, multiple sensitivity analyses were run. These sensitivities included variation of model equations (e.g., Diffusion Wave vs. Full Momentum), variation of Minimum Infiltration Rates, and varying CN values. While many of the analyses yielded minimal changes in the model results, it was noted that changing from the default Curve Numbers made large changes in model results.

### 2.3.3 Frequency Event Results

Peak outflows from each HUC10 watershed were extracted from the model using reference lines (see Figure 9) and are summarized for each simulated frequency event in Table 12. Palo Duro Watershed is made up of three HUC10 watersheds. Upper Palo Duro Creek and North Palo Duro Creek both drain into Lower Palo Duro Creek. Figure 11 shows the relationship between Drainage Area and Discharges within the watershed. The relationship appears to be mostly linear, with a slight jump representing the Upper Palo Duro Creek Watershed. The west part of the HUC-10 watershed is made up of primarily playas. Rather than flowing downstream, water gets caught within the playa and infiltrated into the soil, therefore it is not contributing to the discharges seen at the outlet of the HUC-10 watershed.

There was one detailed study area present, which is in Canyon, Texas. A comparison was made between the lettered cross sections and the BLE model results, which can be found in Table 13.  Some differences should be expected due to the changes in methodology and updates in modeling technology. The effective models are 1D steady flow models, meaning one discharge is applied to the model for the duration of the simulation. The BLE study utilizes 2D unsteady-flow models. Additionally, the BLE analysis does not include detailed structure data. Differences between the effective data and BLE data may indicate a need for an updated Detailed Zone AE study.

Table 12: Frequency Event Peak Discharge Summary (cfs)

| Name | HUC-10 Area (sq. mi.) | Drainage Area (sq. mi.) | 10% | 4% | 2% | 1%- | 1% | 1%+ | 0.2% |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| North Palo Duro Creek Flow Check | 166 | 166 | 707 | 1,460 | 2,238 | 1,744 | 3,233 | 5,093 | 6,709 |
| Upper Palo Duro Creek Flow Check | 331 | 331 | 7,620 | 10,871 | 13,779 | 11,850 | 17,122 | 23,116 | 27,321 |
| Upper Palo Duro Creek Outlet | 357 | 357 | 9,093 | 12,709 | 15,783 | 13,830 | 19,477 | 26,127 | 31,182 |
| North Palo Duro Creek Outlet | 457 | 457 | 10,341 | 14,777 | 18,686 | 15,936 | 23,057 | 30,922 | 35,729 |
| Lower Palo Duro Creek Outlet | 185 | 999 | 13,611 | 20,918 | 27,047 | 23,114 | 34,107 | 47,734 | 57,835 |

Table 13: Detail Study Area – Model Comparison

| Stream Name | Lettered Cross Section | Effective 1% Annual-Chance WSEL (ft) | Modeled 1% Annual-Chance WSEL (ft) | Difference Effective – Modeled (ft) |
| --- | --- | --- | --- | --- |
| Spring Draw | A | 3496.5 | 3495.2 | 1.3 |
| Spring Draw | B | 3505 | 3503 | 2.0 |
| Spring Draw | C | 3512.7 | 3511 | 1.7 |
| Spring Draw | D | 3512.9 | 3514 | -1.1 |
| Spring Draw | E | 3526.7 | 3523 | 3.7 |
| Spring Draw | F | 3552 | 3546.2 | 5.8 |
| Palo Duro Creek | A | 3484.4 | 3487.3 | -2.9 |
| Palo Duro Creek | B | 3484.7 | 3487.7 | -3.0 |
| Palo Duro Creek | C | 3496.4 | 3495.1 | 1.3 |
| Palo Duro Creek | D | 3496.4 | 3495.2 | 1.2 |
| Palo Duro Creek | E | 3497 | 3498.5 | -1.5 |
| Palo Duro Creek | F | 3497.2 | 3499.2 | -2.0 |
| Palo Duro Creek | G | 3501.52 | 3503.5 | -2.0 |
| Palo Duro Creek | H | 3508.39 | 3509.1 | -0.7 |
| Palo Duro Creek | I | 3512.66 | 3513.8 | -1.1 |
| Palo Duro Creek | J | 3515.26 | 3516.1 | -0.8 |
| Palo Duro Creek | K | 3526.14 | 3526.5 | -0.4 |
| Palo Duro Creek | L | 3531.7 | 3535.5 | -3.8 |
| Palo Duro Creek | M | 3539.8 | 3543.3 | -3.5 |
| Palo Duro Creek | N | 3546.8 | 3549.4 | -2.6 |
| Palo Duro Creek | O | 3550 | 3552.6 | -2.6 |
| Palo Duro Creek | P | 3552.4 | 3555.7 | -3.3 |
| Palo Duro Creek | Q | 3557 | 3560.9 | -3.9 |
| Palo Duro Creek | R | 3562.3 | 3565 | -2.7 |

Figure 11: Drainage Areas vs. Discharges

### 2.3.4 Challenges

BLE studies are high level and low detail. Structure data was not provided in most of the watershed, so assumptions had to be made about culvert/bridge parameters. Structure data for Bivins lake with included but was outdated. These factors can affect the validity of the model. For future studies, structure survey data should be incorporated to refine results and more accurately estimate discharge out of the bridge/structure.

The dominance of playa lakes throughout the watershed and lack of stream gages present challenges for the 2D BLE analysis. The precise initial conditions of playa lakes are difficult to simulate within a HEC-RAS 2D model and their natural seasonal variability adds a level of uncertainty to model results.

The lack of stream gage records within the Palo Duro watershed reduces comparison points for the analysis and limits the ability to simulate historical events which could verify the 2D BLE results and approach. FTN coordinated with FNI to calibrate discharges to gages in the downstream watershed (Terra Blanca Watershed) and compared results within the watershed to regional regression equations.

Additionally, the watershed’s terrain is very flat and does not contain one outlet. Water spills into surrounding watersheds in many different areas. To ensure water was not artificially built-up along the watershed edges, boundary conditions had to be added around the entirety of the watershed.

### 2.3.5 Recommendations

This study provides useful information for flood risk identification and communication among stakeholders within the watershed. This study is scalable, and additional analysis will enhance the product and further inform the implementation of regulatory flood hazard areas. The validity of the 2D BLE analysis should be verified through community meetings prior to being applied as a regulatory product.

# 3.0 Quality Control

Throughout the BLE analysis, multiple quality checks were performed. These included checks to review the terrain data processing prior to hydraulic model development; hydrologic discharge and model parameter checks; hydraulic checks for complete model coverage, addition of terrain modification locations to allow for hydraulic connectivity through structures, revisions to dam spillways and top of dams, and revisions to Manning’s “n” values; floodplain mapping cleanup; database quality checks and review; and final submittal reviews. Significant efforts were made to reasonably resolve all items noted prior to submittal. Please see the information included in the Reporting and Documentation > Internal Reviews folder for additional details on the hydraulic review documentation.

# 4.0 Floodplain Delineation (10%, 1% and 0.2% events)

Following the HEC-RAS analysis, the model results were then imported into the HEC-RAS 6.3 RAS Mapper tool to map floodplain boundaries for the model extent. This tool uses a routine that develops water surface elevation grids based on the 10-foot cell size DEM. The sloping rendering mode was used with the “Depth-Weighted Faces” mapping option selected to reduce the effect of cupping and skewed results. For this BLE analysis, mapping results were developed for three (3) events. These events were the 10-, 1-, and 0.2-percent-events. The results of these analyses were used to perform an updated Flood Loss Analysis using the HAZUS program.

The 10-, 1-, and 0.2-percent-event mapping was refined by removing a depth of 1/3 feet or less. Mapping pieces smaller than 1/3 acre were removed. The resulting floodplain spatial data was generalized using a 5-foot tolerance, then smoothed with smoothing tolerance 2.5x input raster cell size. To avoid topology errors, the floodplain spatial data was generalized once again with a 0.3-foot tolerance. After the initial boundary edits, the resulting floodplain boundaries were merged into a single watershed map boundary. For the 10-percent-event, minor areas of high ground (islands) less than 0.5 acres were filled in. For the 1-, 0.2-percent-event, islands less than 2 acres were filled in.

Once the map boundaries were cleaned, the resulting rasters (Water Surface Elevation, Depth, etc.) were developed with the raster set to correspond in extent to the cleaned polygon boundary. This ensures that the water surface raster and the floodplain boundary are consistent with each other. The backwater WSE at HUC10 watershed boundaries was extended upstream to produce clean tie-ins, with no drawdowns. The depth raster product was created at the end of the process by performing a raster subtraction with the water surface elevation raster and the ground DEM. Depth rasters were populated with the value 0.1 feet for areas where holes were filled.

# 5.0 CNMS Validation of Effective Zone A Special Flood Hazard Area (SFHA)

CNMS lines were not reviewed as part of this submittal and will be validated for these watersheds at a later date coordinated with TWDB.

# References

Bibliography list of references used and referred to throughout document.

- USGS. Multi-Resolution Land Characteristics Consortium. National Land Cover Database 2019. (https://s3-us-west-2.amazonaws.com/mrlc/nlcd_2019_land_cover_l48_20210604.zip).

- Chow, Ven T. "Development of Uniform Flow and Its Formulas." Open Channel Hydraulics. Caldwell, NJ: Blackburn, 1959. 109-113. Print.

- FEMA, “Guidance for Flood Risk Analysis and Mapping – First Order Approximation”, November 2014. (http://www.fema.gov/media-library-data/1420849667914-c1656173985d814d0e62f80818505969/FOA_Guidance_Nov_2014.pdf).

- FEMA Region 6, “Base Level Engineering Submittal Guidance”, June 2017. (https://rmd.msc.fema.gov/Regions/VI/Ph0_Investment/1/Base%20Level%20Engineering/Submittal%20Guidance/20170615_BLE%20Submittal%20Guidance_R6.pdf)

- USDA-SCS. Urban Hydrology for Small Watersheds. Technical Release No. 55 (TR-55), 1986. (https://www.nrc.gov/docs/ML1421/ML14219A437.pdf)

- Asquith, W.H., and Roussel, M.C., 2009, Regression equations for estimation of annual peak-streamflow frequency for undeveloped watersheds in Texas using an L-moment-based, PRESS -minimized, residual-adjusted approach, U.S. Geological Survey Scientific Investigations Report 2009–5087, 48 p, (http://pubs.usgs.gov/sir/2009/5087)