| Submitted By: | FTN | Date: | 12/20/2022 |
| --- | --- | --- | --- |
| Reviewed By: | Doucet | Date: | 01/03/2022 |

Technical Review – TULE (HUC 8 # 11120102)

HUC 10 # 1112010202

| Y | N | N/A | Item Description | Comments |
| --- | --- | --- | --- | --- |
| Data Completeness Check |  |  |  |  |
| ☒ | ☐ | ☐ | Hydrology model and/or spreadsheets included | Basin for HMS model is mislabeled; Corrected |
| ☒ | ☐ | ☐ | Terrain dataset included |  |
| ☒ | ☐ | ☐ | Projection file included |  |
| ☐ | ☒ | ☐ | 2D boundaries included | In model but not exported; added to supplemental files ; Good |
| ☐ | ☒ | ☐ | 2D Breaklines included | In model but not exported; added to supplemental files; Good |
| ☒ | ☐ | ☐ | Land Cover dataset (Manning’s n-values) Included |  |
| ☒ | ☐ | ☐ | Infiltration dataset (Composite CN values) Included |  |
| ☐ | ☒ | ☐ | Regression analysis Included |  |
| ☐ | ☒ | ☐ | Gage analysis Included |  |
| ☒ | ☐ | ☐ | Preliminary floodplain runs Included in RAS-Mapper |  |
| ☒ | ☐ | ☐ | Terrain, land cover, and infiltration data have been associated with correct geometry(s) |  |
| ☐ | ☐ | ☐ | All flood events included in the HEC-RAS model (10%, 4%, 2%, 1%, 0.2%, 1%+, 1%-, and any observed rainfall events) | Model Start and End Date will need to be updated to 2022; updated to 2022; Good |
| ☒ | ☐ | ☐ | Modeling narrative included |  |
| Hydrology |  |  |  |  |
| Rainfall |  |  |  |  |
| ☒ | ☐ | ☐ | Precipitation values are an aerial average of Atlas 14 values for the entire 2D area boundary for each storm event |  |
| ☒ | ☐ | ☐ | Total precipitation depths for 1%+ and 1%- are calculated correctly |  |
| ☒ | ☐ | ☐ | Rainfall hyetographs are calculated for all storm events (10%, 4%, 2%, 1%, 0.2%, 1%+, and 1%-) |  |
| ☐ | ☐ | ☒ | Cumulative excess rainfall values are in increasing order from 10%, 4%, 2%, 1%, to 0.2% |  |

| Y | N | N/A | Item Description | Comments |
| --- | --- | --- | --- | --- |
| ☐ | ☐ | ☒ | Cumulative excess rainfall values for the 1%+ and 1%- fall in order with the storm events matching the total precipitation values |  |
| Inflow Hydrographs (if any) |  |  |  |  |
| ☐ | ☐ | ☒ | Correct DSS file paths are being used in HEC-RAS |  |
| ☐ | ☐ | ☒ | Contributing hydrographs from another HEC-RAS model applied at coincident locations |  |
| HEC-RAS Geometry |  |  |  |  |
| Terrain |  |  |  |  |
| ☒ | ☐ | ☐ | 10ft Imported Grid resolution used |  |
| ☒ | ☐ | ☐ | Topographic abnormalities corrected and no drops or rises from underlying data |  |
| Land Cover (Manning’s n) |  |  |  |  |
| ☒ | ☐ | ☐ | Land Cover classifications are sufficiently descriptive within the data table |  |
| ☒ | ☐ | ☐ | Manning’s n values generally match current Land Cover classifications | There are a couple that are Min or Max of the Guideline N Values; just be aware; Noted; Good |
| Infiltration |  |  |  |  |
| ☒ | ☐ | ☐ | CN values generally match current soils and land use data |  |
| ☒ | ☐ | ☐ | Appropriate abstraction ratio and minimum infiltration used for CN classifications |  |
| 2D Mesh |  |  |  |  |
| ☐ | ☐ | ☐ | 2D Boundary delineated along watershed boundary or buffer to ensure all contributing area is included | No buffer used in some areas; seem to not be needed; make sure all HUC 10 is included; any comments about mesh border is in QAQC Comments shapefile; Model boundary was merged and split to ensure flow not being trapped at edge. See shapefile. ; Not sure why BCs are missing along North portion of mesh, this may get called out in External QAQC. Also there are areas of floodplain buildup along borders where BCs are missing (example 1pctPlus) : Corrected. |
| ☒ | ☐ | ☐ | Nominal cell size appropriate for model detail? (generally 200 feet) |  |
| ☒ | ☐ | ☐ | No mesh errors |  |

| Y | N | N/A | Item Description | Comments |
| --- | --- | --- | --- | --- |
| ☐ | ☐ | ☐ | Where appropriate cells overlap terrain features (i.e., V-notch at bridges) | Comments in QAQC shapefile; See shapefile; See shapefile for backcheck comments Corrected. |
| ☐ | ☐ | ☒ | 2D Area Connections placed at applicable benchmarks and set to use normal 2D equations domains |  |
| ☐ | ☐ | ☒ | Refinement zone assigned appropriately |  |
| Breaklines |  |  |  |  |
| ☐ | ☐ | ☐ | Breaklines are spaced at least 200 ft (cell spacing) apart or breakline spacing parameters adjusted as necessary? | Found a lot of breaklines within 200 ft and breaklines not being enforced; examples given but model will need thorough review Comments in QAQC shapefile; See shapefile; See shapefile for backcheck comments Corrected. |
| ☐ | ☐ | ☐ | Breaklines sufficiently detailed and consistent with the terrain data? | Comments in QAQC shapefile Stream breaklines will need to be extended to the 0.1 sq. mile limits. Stream breaklines can end at the caprock where flow transitions from streams to playas and overflows; See shapefile; See shapefile for backcheck comments Corrected. |
| 2D Boundary Conditions |  |  |  |  |
| ☐ | ☐ | ☒ | Initial conditions used and set appropriately |  |
| ☒ | ☐ | ☐ | Internal and external boundaries connected to 2D mesh |  |
| ☐ | ☒ | ☐ | Boundary conditions everywhere flow could build up at the boundary of the 2D mesh | Need to extend/add BCs throughout Perimeter; I recommend setting plot tolerance to default to see how a lot of water trapped at boundary No need for short BCs in most areas, can have a few long BCs; BCs have been edited to reflect resultant water entrapment.; See above comment Corrected. |
| ☒ | ☐ | ☐ | Boundary conditions have a precipitation or hydrograph data and assigned at correct locations | Normal Depth applied if not precip or hydrograph Model has the same 0.001 ND slope applied to all BCs, seems inaccurate, I am getting 0.0014 for EG Slope (100 yr storm) for the Outflow BC; this will not change the results drastically. Keep slope as is; Good |
| ☒ | ☐ | ☐ | Downstream boundary consistent with ground slope / known WSEL |  |
| 2D Model Execution |  |  |  |  |
| ☒ | ☐ | ☐ | Appropriate 2D parameters and options used |  |
| Y | N | N/A | Item Description | Comments |
| ☒ | ☐ | ☐ | Hydrograph Output Interval and Mapping Output Interval set at a reasonable interval to capture peaks |  |
| ☒ | ☐ | ☐ | Model runtime is reasonable |  |
| 2D Model Results |  |  |  |  |
| Computation Checks |  |  |  |  |
| ☒ | ☐ | ☐ | Model runs with no cell errors |  |
| ☒ | ☐ | ☐ | Volume balance is within tolerance (ΔV < 1% is ideal, however 2% to 3% can be acceptable for rain on mesh) |  |
| Full Checks |  |  |  |  |
| ☐ | ☐ | ☐ | Computational time step meets ideal Courant condition (diffusive wave ≤ 2, up to 5) |  |
| ☒ | ☐ | ☐ | Model simulation is long enough to pass the entire hydrograph through the model (minimum of 12 hours past the peak, preferably 24 hours) |  |
| ☐ | ☒ | ☐ | Hydrographs at internal and external boundaries are smooth |  |
| ☒ | ☐ | ☐ | Contributing areas are contained within 2D mesh or accounted for with inflow hydrographs |  |
| ☐ | ☐ | ☐ | No Flow trapped at 2D Boundary | Consistent Mapping on edge of Mesh; will need to make sure they tie in in mapping or will need to be removed; Examples in QAQC_Comments shapefile; Model boundary was merged and split to ensure flow not being trapped at edge. See shapefile.;; Good |
| Spot Checks |  |  |  |  |
| ☒ | ☐ | ☐ | WSELs appear reasonable within a single cell (check for visible spikes) | Saw some spikes and ‘cupping’ but that could be due to terrain and model; likely due to flat terrain.; Good |
| ☒ | ☐ | ☐ | Depth grids appear reasonable (no artificial dips or spikes) |  |
| ☒ | ☐ | ☐ | Flooding is connected downstream for all significant contributing areas (> 64 acres) | Some gaps, but seems to just be a due to flat terrain, playas, etc; likely due to flat terrain. .; Good |

| Y | N | N/A | Item Description | Comments |
| --- | --- | --- | --- | --- |
| ☒ | ☐ | ☐ | Velocity grids appear reasonable (no unusually high velocity areas / hot spots) |  |
| Flow Checks |  |  |  |  |
| ☒ | ☐ | ☐ | DSS files contain runs consistent with project |  |
| ☐ | ☐ | ☒ | Peak modeled discharges comparable to gages, and regional regression equations |  |
| 2D Model Verification |  |  |  |  |
| Stream Gage Analysis |  |  |  |  |
| ☐ | ☐ | ☒ | Regression analysis performed at select locations in the watershed |  |
| ☐ | ☐ | ☒ | Stream gage analysis performed on all gages in the watershed with sufficient data | Not enough Data; there were no existing gages in this huc 10 for analysis.; Good |
| Sensitivity Analysis |  |  |  |  |
| ☒ | ☐ | ☐ | Sensitivity analysis performed to evaluate which model parameters significantly impact results |  |
| ☐ | ☐ | ☐ | Based on the sensitivity analysis results did the results of the selected computation settings resulted in acceptable outcomes | Recommend shortening Reference Lines to prevent capturing too much flow and focus on main channel; corrected .; Inflow/Outflow BCs seem to be pretty long, if FPs are tying in to each other within 0.5 feet and no flow leaving this is fine Corrected. Results do not seem to be matching well with regression analysis results; No changes were made according to regression results. They were just used as a comparison, since no gage was present.; Good |
| Additional Comments |  |  |  |  |