# Texas 2D BLE Analysis QA/QC Checklist

# Project Information

| HUC-8 Watershed Name & ID: | 11120103 Upper Prairie Dog Town Fork Red |
| --- | --- |
| HUC-10 Watershed Name & ID: | 1112010301 Headwaters 1112010302 Happy Draw 1112010303 Gip Creek 1112010304 Battle Creek 1112010305 Headwaters Mulberry Creek 1112010306 Mulberry Creek |
| Modeler: | Freese & Nichols |
| Reviewer: |  |

## Terrain and Modeling Review (QC Items 1-6)

| Modeler Initials: | HA / OT | Reviewer Initials: | AC |
| --- | --- | --- | --- |
| Submittal Date: | 3/24/2023 | Review Submitted Date: | 4/19/2023 |
| Review Response Date: | 8/15/23 | Backcheck Complete Date: | 6/28/23 |

## Mapping Review (QC Item 7)

| Modeler Initials: |  | Reviewer Initials: |  |
| --- | --- | --- | --- |
| Submittal Date: |  | Review Submitted Date: |  |
| Review Response Date: |  | Backcheck Complete Date: |  |

## Completeness Review (QC Item 8)

| Modeler Initials: |  | Reviewer Initials: |  |
| --- | --- | --- | --- |
| Submittal Date: |  | Review Submitted Date: |  |
| Review Response Date: |  | Backcheck Complete Date: |  |

(Modeler to populate with an “X” for items included in the QC submittal)

| Level of Study |  |  | Flood Events |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| X | 2D BLE |  | X | 10% ACE (10-year) | X | 0.2% ACE (500-year) |
|  | Enhanced BLE |  | X | 4% ACE (25-year) | X | 1% plus |
|  |  |  | X | 2% ACE (50-year) | X | 1% minus |
|  |  |  | X | 1% ACE (100-year) |  | Observed Rainfall |

Instructions

Each section includes instructions to the modeler and reviewer regarding their responsibilities in the completion of this checklist. In the Methodologies / Data Sources tables, grey italic text is added as an example response. Additional sample responses are located in the Texas 2D BLE Guidelines Quick Guide.

Reviewer to indicate if Technical Review items generally comply using a “Pass”. If the technical review observes an issue of noncompliance or an item that requires additional attention, an “Fail” should be utilized.

## Submittal Checklist

Data submitted for each milestone should be carried through each subsequent milestone. All geospatial layers and models must have same defined projection system as terrain. Modeler to populate with an “X” for items included in the QC submittal.

| Base Map Data (QC items 1-6) |  |
| --- | --- |
| QC Files | Location Map (PDF): HUC-8 and HUC-10 boundaries and/or model domain boundary if different from HUC-10s, stream gages, dams (if applicable), mainstem streams (for reference), background map |
| GIS Files | GIS Map (MXD or Map Package) with the following data included but not limited to: Stream centerlines, model breaklines, rainfall data, WSEL and Depth grids (10, 100, 500-yr), adjacent HUC mapping (available on the BLE viewer or currently studying), 2D connections for inflows/outflows, gages or points of flow comparisons (BFEs, highwater marks, etc.), profile lines, boundary condition lines, refinement regions, model domain boundary, model mesh, DFIRM mapping panels, base map data (roads, aerials, etc.), soils data, land use data, terrain |
| Technical Report / Documentation (QC items 1-6) |  |
| See report | Technical Report / Project Notebook: assumptions, special considerations, and modeling concerns |
| See report | Detailed explanation and support data of variances in methodologies and approaches from recommended guidelines (Should be included in Technical Report / Project Notebook) |
| QC Files/Internal QC | Internal QA/QC Documentation: comments and responses |
| See report | Flow comparison tables within the technical report |
| Hydrology (QC items 1-6) |  |
| Hydrology Models | Hydrologic Model (HEC-HMS) model with all associated DSS files for 7 required storm events |
| Hydrology Models | Hydrologic Data and Parameter Spreadsheets (rainfall calculations, curve number table, gage locations, gage analysis and flow validation) |
| See report | Hydrologic modeling notebook including assumptions, notes, and special conditions (if not provided in report) |
| Hydraulics (QC items 1-6) |  |
| Hydraulic Models | Hydraulic Model (HEC-RAS) with all associated files and 7 required storm events (remove non-final files) |
| Hydraulic Models & GIS Files | Raw mapping grids for 10, 1, and 0.2% storm events |
| See report | Hydraulic Data and Parameter Spreadsheets (Manning’s n-value table, infiltration calculations support data-Soils, Land Use, Curve Number table, other calculations/data used in model) |
| See report | Manning’s n-value table |
| X | Precipitation data correctly linked to hydrologic model or hard coded from provided spreadsheet |
| See report | Hydraulic modeling notebook including assumptions, notes, and special conditions |
| Mapping (QC item 7) |  |
| - | Flood risk database |
| - | Modeler to complete topology validation as outlines in the guidelines |
| - | Modeler to verify mapping meets deliverable requirements as outlined in guidelines |
| Completeness Review (QC item 8) See Texas 2D BLE Guidelines Chapter 4 – Deliverables |  |
| - | Technical BLE Report |
| - | Final cleaned hydrologic model |
| - | Final cleaned hydraulic model |
| - | Flood risk database |
| - | CMNS updates |
| - | HAZUS results |
| - | Additional supplemental data |

# 1 – Model Terrain

Methodologies / Data Sources (Modeler to populate)

| Terrain Source(s): | USGS 2018 Texas Panhandle LiDAR |
| --- | --- |
| Data Gaps: | N/A |
| Source Resolution: | 1-meter |
| Model Terrain Resolution: | 10 foot |
| Terrain Horizontal Projection: | NAD83 SPCS 4201 FT |
| Terrain Vertical Projection: | NAV88 FT |
| Z Values: | 2245.37-4056.07 |
| Terrain Buffer: | 1 mile |
| Other: |  |

## Technical Review (Reviewer to confirm with Pass/Fail)

| General Review |  |
| --- | --- |
| P | LiDAR source matches the latest available data at time of project initiation |
| P | Large reservoir or lake extents included in model domain |
| P | Check visible seams, gaps or holes of LiDAR sources (if more than one source was used) |

## Review Comments

No Comments

# 2 – Hydrology

## Methodologies / Data Sources (Modeler to populate)

| Software: | HEC-HMS v4.8 |
| --- | --- |
| Rainfall Source: | Atlas 14 |
| Rainfall Duration: | 24-hr |
| Depth Selection: | 5-minute Intensity Duration |
| Rainfall Temporal Distribution: | Frequency Storm 50% Intensity Position |
| Areal Reduction: | None |
| Loss Methodology: | CN – HEC-RAS Rain on Mesh |
| Unit Hydrograph Transform Method: | HEC-RAS Rain on Mesh |
| Inflow Hydrograph Source: | - |
| Other: |  |

## Statistical Data (Modeler to populate)

| ID | Description | Gage Name | Notes | Years | Valid Years | Used? |
| --- | --- | --- | --- | --- | --- | --- |
| 7297500 | Trib - Headwaters | Pr Dog Twn Fk Red Rv nr Canyon, TX | Location not suitable for validation | 13 | 13 | No |
| 7297910 | Main Stem – Happy Draw | Pr Dog Twn Fk Red Rv nr Wayside, TX | 1967-2022 | 55 | 55 | Yes |
| 7299000 | Trib – Mulb Creek | Mulberry Ck nr Brice, TX | Not enough data | 2 | 2 | No |
| 7298500 | Main Stem – Mulb Creek | Pr Dog Twn Fk Red Rv nr Brice, TX | 1938-1944, 1949-1951, 1960-1963, 2000 -2022 | 32 | 32 | Yes |

## Technical Review (Reviewer to confirm with Pass/Fail)

| Excess Rainfall |  |
| --- | --- |
| P | Precipitation values are an aerial average of Atlas 14 values for the entire 2D area boundary for each storm event or other appropriate rainfall application |
|  | CN values appropriately reflect land use and soils for entire 2D area boundary |
| P | Losses are correctly calculated to establish the excess rainfall hyetographs by checking land use, soils and infiltration layers or other relative information |
|  | Total precipitation depths for 1%+ and 1%- are calculated correctly |
| P | Rainfall hyetographs are calculated for all storm events (10%, 4%, 2%, 1%, 0.2%, 1%+, and 1%-) |
| P | Rainfall values are in increasing order from 10%, 4%, 2%, 1%, to 0.2% |
| P | Rainfall values for the 1%+ and 1%- fall in order with the storm events matching the total precipitation values |
| Inflow Hydrographs |  |
| P | Shape and time-to-peak of any inflow hydrographs resemble past documented large storm events |
| F | Hydrographs reasonable when compared to the other return periods. The shapes should be similar, and hydrographs from different events should not intersect unless there is an engineering reason. Peak flows from a lower return period event should not be higher than a higher return period event |
| F | Contributing hydrographs from another RAS model applied at coincident locations (Provide locations in base map data) |
| General Hydrology |  |
| No | Any flow regulating dams in the model area? (Yes/No) |
| NA | Dam modeling methodologies applied and explained? (Yes/No). If yes, provide discharge rating curve calculations and gage analysis documenting regulated versus unregulated records, if applicable. |
| F | Model aligns with documentation of dam modeling approaches. |
| Other Notes: NA |  |

## Review Comments

M1 Audit shapefile & responses in each model folder and included in Mapper for each model.

- Check page no. 22-23 of the TWDB guidelines. The use of AMC III is not recommended for BLE modeling.

1% plus and minus run geometries have been revised.

No further comments.

- There is nothing in the report documenting the inflows from external HUC08s (Palo Duro, Tierra Blanca, Tule).

Discussion of these external hydrographs has been included in section 2.2.2 Model and Boundary Condition Setup.

Please include inflow hydrograph figures from contributing HUCs.

Section 2.2.2 Model and Boundary Condition Setup now includes inflow hydrograph figures from external HUCs.

- Inflows from upstream models are crossing.

Due to the nature of unsteady modeling there may be instances of “crossing” hydrographs. We have reviewed the model results are have determined all models are appropriate.

Please see comments added below under Comment 4, where there are still instances of crossing WSELs.

CompareWSE Raster Calculates were generated in RASMapper and used to check all models for instances of crossing WSELs. Adjustments were made to breaklines in the instances under Comment 4 and a couple other areas to no longer have crossing WSELs.

- There are crossing flow profiles. Some of the places where flows were pulled from are in the QC shapefile.

It is more appropriate to compare flow through reference or boundary conditions. When creating a profile line, results are limited by the timestep and can be skewed by the cell faces RAS selects. Some of the “crossings” below are related to the additional factor of losses that were previously included in our approach. We’ve included a CompareWSE Raster Calculate in mapper of the 100/500YR which we feel is an appropriate check of unsteady results.

  - Example for Headwaters Prairie

- ID 11 in QC shp

- No further comments

  - Example for Happy Draw

- ID 2 in QC shp

- Response states WSE comparison is appropriate, but RAS still shows crossing WSELs.

Crossing WSELs in Happy Draw have been fixed.

  - Example for Gip Creek

- ID 2 in QC shp

- No further comments

  - Example for Headwaters Mulberry

- ID 1 in QC shp

- Areas around here have 100yr larger than 500yr. It shows up in exported grids. This is a representative area and checks across all modeling areas should be performed.

Crossing WSELs in Headwaters Mulberry have been fixed.

# 3 – Hydraulic Geometry

## Methodologies / Data Sources (Modeler to populate)

| Software: | HEC-RAS 6.3.0 |
| --- | --- |
| Loss Methodology: | CN |
| Soils Source: | USGS |
| Land Cover Source: | NLCD 2019 – verified to Aerial |
| Manning's Roughness Source: | NLCD 2019– verified to Aerial |
| Nominal Mesh Cell Sizes: | 200’x200’ with refinement region 100’x100’ |
| Other: |  |

## Technical Review (Reviewer to confirm with Pass/Fail)

| 2D Mesh |  |
| --- | --- |
| P | 2D Boundary delineated along watershed boundary or buffer to ensure all contributing area is included |
| P | Nominal cell size appropriate for model detail (generally 200 feet, minimum 50 feet for refinement areas) |
| P | Computation points are at default locations or changes documented |
| P | Mesh Cell and Face tolerances are default or changes documented |
| P | No mesh errors |
| P | Cell face relationships are calculated |
| Breaklines |  |
| F | Breaklines are consistent with the terrain data |
| F | Breaklines sufficiently detailed to capture features (thalweg, banks, berms, etc.) |
| P | Cells do not overlap terrain features where appropriate (i.e., unwanted leaking) |
| P | Breakline cell sizing and parameters are appropriate |
| F | 2D Area Connections of Reference Lines placed at applicable benchmarks, gages, or dams |
| F | 2D Area Connections set to use normal 2D equation domain for benchmarks and gages. Appropriate modeling approach used for dams. |
| Manning’s N-Values |  |
| P | Manning’s n-values associated to land cover data provided |
| P | Land Cover classifications are sufficiently descriptive within the data table |
| P | Manning’s n-value classifications generally match current aerial imagery |
| P | Manning’s n dataset is associated with the geometry files |
| General |  |
| P | All model datasets each have unique descriptive names |

## Review Comments

- There are no Reference Lines to show where the gages are located.

Reference lines provided at gage locations.

No further comments.

- Shapefiles are provided for each area showing recommendations on breakline edits.

Revisions have been made to breaklines as appropriate.

No further comments.

- New Comment. Channel n-value regions are fairly large. About 200’ on each side of centerline. Please verify this is right. If so, make sure to document why this buffer size was chosen in the report.

This buffer size is intentional. This size was chosen to ensure that one cell on either side of the centerline was captured in the channel n-value region. This has been documented in report section 2.2.4.1 Roughness Coefficients.

# 4 – Hydraulic Boundary Conditions

## Technical Review (Reviewer to confirm with Pass/Fail)

| Boundary Review |  |
| --- | --- |
| P | External boundaries connected to 2D mesh |
| P | Boundary conditions everywhere flow could build up at the boundary of the 2D mesh |
| P | Boundary conditions have a hydrograph or elevation/slope data assigned |
| P | Input hydrographs consistent with hydrologic data (DSS, manual entry) |
| P | Precipitation data is consistent with hydrologic data (DSS, manual entry) |
| P | Downstream boundary consistent with ground slope or known downstream WSEL |
| F | Flow hydrographs applied at appropriate locations |
| P | Energy grade slope for hydrograph distribution is reasonable |
| P | Internal rating curves appropriate and documented for control structures (if applicable) |

## Review Comments

- Hydrographs from upstream models are applied at a point where they overlap with the mesh, but they are not tying in. Recommend changing the way the inflow is applied to help with tie-ins.

Adjustments for tie ins are being made concurrent to this review.

Still having some tie-in issues between Battle Creek and Mulberry, Happy Draw and Gip Creek. Subtraction grids provided for the 100year. Please check 500year as well for all modeling domain tie-ins and external tie-ins.

Tie- ins were checked for the 100- and 500-year. BC lines were adjusted where appropriate.

- Some inflows have longer run times than others. Suggest keeping them the same.

Noted.

No further comments.

# 5 – Hydraulic Model Execution

## Methodologies / Data Sources (Modeler to populate)

| Unsteady Plan Names |  |  |  |
| --- | --- | --- | --- |
| 10% ACE (10-year): | 10PCT_min15 | 0.2% ACE (500-year): | 002PCT_min15 |
| 4% ACE (25-year): | 04PCT_min15 | 1% plus: | 01PCT_plus_min15 |
| 2% ACE (50-year): | 02PCT_min15 | 1% minus: | 01PCT_minus_min15 |
| 1% ACE (100-year): | 01PCT_min15 |  |  |
| Other: |  |  |  |

## Statistical Data (Modeler to populate)

| Time Step: | 30 sec |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Courant Value: | ~<2, Max 5 |  | Simulation Length: | 144 | hours |
| Computation Interval: | 30 | seconds | Model Runtime: | ~2hr /model | hours |
| Hydrograph Output Interval: | 15 | minutes | Mapping Output Interval: | 60 | minutes |

## Technical Review (Reviewer to confirm with Pass/Fail)

| Execution Review |  |
| --- | --- |
| P | Appropriate 2D parameters are used (Boundary Conditions, Time Step, Simulation Length, etc.) |
| P | Hydrograph Output Interval and Mapping Output Interval set at a reasonable interval to capture peaks |
| P | Model runtime is reasonable (< 12 hours, but dependent upon model size) |

## Review Comments

- Some plans are running for 6 days, while others are running for 7 days throughout modeling domains. Recommend running all plans for the same amount of time.

Updated for consistent run settings

No further comments

- Please clean up runs/plans in RAS to only have the ones that are going to be used as final left.

Model runs have been cleaned.

Plans have been cleaned. Consider cleaning Map Layers for final submittal as well.

Map layers were cleaned up in all models.

- Was not able to load the 100-year results for Gip Creek. When trying to extract profiles, RAS mapper would crash. Might need to be reran.

All models have been run in HEC-RAS v6.3

No further comments.

# 6 – Hydraulic Model Results

## Methodologies / Data Sources (Modeler to populate)

| Flow Validation/Gage Analysis |  |
| --- | --- |
| Software: | HEC-SSP 2.2 |
| Methodology | Bulletin 17B |
| Removal of Outliers and Justification | ID; Description; Gage Name; Notes; Years; Valid Years; Used?. 7297500; Trib - Headwaters; Pr Dog Twn Fk Red Rv nr Canyon, TX; Location not suitable for validation; 13; 13; No. 7297910; Main Stem – Happy Draw; Pr Dog Twn Fk Red Rv nr Wayside, TX; 1967-2022; 55; 55; Yes. 7299000; Trib – Mulb Creek; Mulberry Ck nr Brice, TX; Not enough data; 2; 2; No. 7298500; Main Stem – Mulb Creek; Pr Dog Twn Fk Red Rv nr Brice, TX; 1938-1944, 1949-1951, 1960-1963, 2000 -2022; 32; 32; Yes. |
| Validation Notes (describe parameter adjustments): |  |
| Other: |  |

## Technical Review (Reviewer to confirm with Pass/Fail)

| Computation Checks |  |
| --- | --- |
| P | Model runs with no cell errors |
| P | Volume balance is within tolerance (ΔV < 1% is ideal, however 2% to 3% can be acceptable for rain on mesh) |
| Results Spot Checks Check at all critical structures, major confluences, a handful of headwater tributaries, downstream end, at benchmarks, and a handful of randomly selected locations. |  |
| P | No visible spikes in WSEL visibly present with a single cell |
| P | WSEL and depth grids appear reasonable (no artificial dips or spikes) |
| P | Visual verification flooding is connected downstream for all significant contributing areas (> 64 acres) |
| F | Velocity grids appear reasonable (no unusually high velocity areas or Courant values greater than 2) |
| F | No crossing profiles between all events |
| P | Inflow and outflow hydrograph timing is reasonable as compared to the peak |
| P | Spills into/from adjacent HUCs appropriately accounted for |
| F | WSEL tie-in between model domains and HUCs along major rivers/streams where inflows are applied |
| Results Full Checks |  |
| F | Computational time step meets ideal Courant condition |
| P | Diffusion Wave: V∆T∆X ≤2 (Up to 5) (Results are unit-less) |
| P | Model simulation is long enough to pass the entire hydrograph through the model (minimum of 12 hours past the peak, preferably 24 hours) |
| P | Hydrographs at internal and external boundaries are smooth |
| P | Contributing areas are contained with 2D mesh or accounted for with inflow hydrographs |
| P | No flow trapped at 2D boundary |
| F | Visual verification mapping is reasonable compared to effective |
| Flow Verification |  |
| P | Peak flow gage analysis performed on all gages in the watershed with sufficient data |
| P | All gages used have an appropriate period of record (10 to 20+ years) |
| P | All gages used have no extensive gaps in record which would impact the analysis |
| P | Gage analysis results validated to other sources (Regression, USGS, etc.) |
| F | Results at applicable benchmarks justified (gages, HWMs, effective BFEs) |
| F | Peak discharges comparable to gages, effective/prior studies, regional regression equations |

## Review Comments

- There are crossing WSEL profiles between events:

Previous submittal included varied losses for 1%+/- event. The Compare WSE has been reviewed and provided and is appropriate for all models.

  - Example for Happy Draw:

- ID 2 in QC shapefile

- The compare WSE file shows some issues with crossing profiles as well. Suggest looking throughout modeling areas.

The Compare WSE Rasters were used to check all models for instances of crossing WSELs. Adjustments were made to breaklines to minimize differences.

  - Example for Headwaters Mulberry

- ID 1 in QC shp

- Same as comment 4d in Hydrology.

Fixed Happy Draw and Headwaters Mulberry instances of WSELs crossing.

- There are spikes in velocity. Major spikes are noted in shapefile but recommend looking throughout all modeling areas.

M1 Audit shapefile & responses in each model folder and included in Mapper for each model.

No further comments.

- Gage 07297910 in Happy Draw has a 41% difference when compared to the gage analysis. Is there some reasoning behind why it is so far off? If so, please provide documentation of differences.

There is significant variability between the observed data and the computed curve for the Wayside gage in Happy Draw. For this reason, gage comparison the Brice gage in Mulberry Creek was prioritized for model validation.

No further comments.

- There is effective mapping in Headwaters Prairie. There are some big differences in mapping extents. Please provide explanation.

This discussion and comparison will be included in the mapping portion of the report.

Not currently included in report.

This has been added to Section 4.0 Floodplain Delineation (10%, 1% and 0.2% events).

# 7 – Mapping Results

## Technical Review (Reviewer to confirm with Pass/Fail)

| Hydraulic Model |  |
| --- | --- |
| P | HEC-RAS model with results, pertinent data and required storm events |
| Flood Risk Database |  |
| P | All features fully populated per current FEMA guidelines |
| P | Mapping consistent between grids to polygons for all three required mapped events |
| F | 10yr smaller than 100yr smaller than 500yr |
| NA | Holes filled in polygon, depth and WSE grids |
| NA | Boundary interfaces align matching or identified as AOMI |

## Review Comments

- There are areas between modeling domains that do not tie-in within half a foot. Ran a subtraction of the grids for the 100yr event to show where the tie-in issues are. Adjusting the shape of inflows and location could help with tie ins.

Adjustments for tie ins are being made concurrent to this review.

Tie-in issues identified. 100yr subtractions ran and submitted with this review. Please check for 500year.

Tie- ins were checked for the 100- and 500-year. BC lines were adjusted where appropriate.

- New Comment: Was not able to verify the tie-in with Tierra Blanca. Make sure to run subtractions to ensure tie-ins.

The tie-in had not been accounted for correctly on Palo Duro and Tierra Blanca. See below – UPD is in grey, Palo Duro in green, and Tierra Blanca in blue. This will not impact the UPD mapping, but this will be addressed in mapping for the other HUCs. We will add a polygon to the AOMI.

# 8 – Completeness Review

## Technical Review (Reviewer to confirm with Pass/Fail)

| File Structure and Naming |  |
| --- | --- |
|  | Deliverables provided in TWDB/FEMA required file folder structure and naming convention |
|  | Data missing (if any) documented in report and ReadMe file within the required folder |
| Report |  |
|  | Technical 2D BLE report documents all assumptions, special considerations, etc. |
|  | Flow validation and difference discussed in report |
|  | Flow comparisons discussed in report |
| Hydrologic Model |  |
|  | HEC-HMS model with results, pertinent data and required storm events |
| Hydraulic Model |  |
|  | HEC-RAS model with results, pertinent data and required storm events |
| Flood Risk Database |  |
|  | All features fully populated per current FEMA guidelines |
| CNMS |  |
|  | Updated correctly with new centerlines |
|  | A1-A5 checks complete |
|  | CNMS Post- Processing Complete |
| HAZUS |  |
|  | Run per FEMA guidance |

## Review Comments

QA/QC Checklist Approval

This 2D Base Level Engineering Analysis QA/QC Review is in compliance with the contract requirements and all task checkpoints are complete.  The independent QC Team has reviewed the analysis, presented review comments to the Production Team Task Leader, and discussed any problems or issues.  The Task Leader, QC Manager, QC Reviewer, and Project Manager have signed the QA/QC Checklist to confirm that all comments have been received, addressed, and documented appropriately.

| QA/QC Team – [TWDB or company name] |  |  |  | Production Team – [add company name] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| QC Manager |  | Date |  | Task Lead |  | Date |
| QC Reviewer |  | Date |  | Project Manager |  | Date |