May 15, 2023

Version X.X

Base Level Engineering Report

Upper Prairie Dog Town Fork Red (HUC-8 11120103)

Report Prepared by:

Freese and Nichols Inc.

801 Cherry St.

Suite 2800

Fort Worth, Texas 76102

Report Prepared for:

FEMA (Federal Emergency Management Agency)

Region 6, Risk Analysis Branch

FRC 800 North Loop 288

Denton, TX 76209-3698

Document History

Document Location

Enter MIP URL (K: Drive Location) here for future use.

Revision History

| Version Number | Version Date | Summary of Changes | Author |
| --- | --- | --- | --- |

Table of Contents

Executive Summary	4

1.0 Introduction	6

2.0 2D BLE Modeling Inputs and Controls	8

2.1 Topographic Data	8

2.1.1 Inventory	8

2.1.2 Evaluation	8

2.1.3 Data Development Methodology	10

2.1.4 DEM QA/QC	10

2.2 2D BLE Methods	10

2.2.1 Model Areas	10

2.2.2 Model and Boundary Condition Setup	12

2.2.3 2D Computational Mesh and Settings	12

2.2.3 Hydrology	13

2.2.4 Hydraulics	16

2.3 Model Results	18

2.3.1 Stream Gage Analysis	18

2.3.2 Sensitivity Analysis	19

2.3.3 Frequency Event Results	21

2.3.4 Challenges	21

2.3.5 Recommendations	22

3.0 Quality Control	23

4.0 Floodplain Delineation (10%, 1% and 0.2% events)	24

CNMS Validation of Effective Zone A Special Flood Hazard Area (SFHA)	24

References	25

Appendix A – Summary of Peak Flow Values (Q)	26

Appendix B – Exhibits – Base Level Engineering Results	27

# Executive Summary

The Texas Water Development Board (TWDB) contracted with Freese and Nichols, Inc. in September 2021 to complete a Base Level Engineering (BLE) analysis for the Upper Prairie Dog Town Fork Red HUC-8 in Northwest Texas, to support FEMA’s Discovery process and validation of effective Zone A Special Flood Hazard Areas (SFHAs). This BLE study will provide significant data for several Texas counties previously lacking modernized flood models.

The BLE process involves using best available data and incorporating automated techniques with traditional model development procedures to produce regulatory-quality flood hazard boundaries for the 1-percent annual chance event as well as estimates of flood hazard boundaries for multiple recurrence intervals.

The source digital terrain data used for surface model development in support of hydrologic and hydraulic analysis as well as mapping activities were leveraged from various local, State, and Federal partners.

Flood discharges and water surface elevations for this study were calculated by utilizing the rain-on-grid two-dimensional (2-D) modeling capabilities of the Hydrologic Engineering Center’s River Analysis System (HEC-RAS), which translates rainfall into runoff based on an underlying terrain and various other physically based parameters. 2D HEC-RAS models were developed in accordance with the 2019 FEMA Region 6 Base Level Engineering Submittal Guidance, which relies heavily on HEC-RAS River Analysis System, 2D Modeling Users’ Manual 5.0 (Bruner, 2016). All models were developed in the HEC-RAS program version 6.3.0.

The stream mile network that was validated for these watersheds was compiled using FEMA’s Community Needs Management Strategy (CNMS) inventory. CNMS is an inventory of flood hazard studies and flood hazard mapping needs for areas where a study is needed. This data is helpful for community officials in analyzing and depicting flood hazards to enhance the understanding of flood risks. Communities may use this information to make informed decisions on their planning and flood mitigation efforts.

The full inventory of Zone A studies in the watershed were classified in CNMS. Total miles currently validated in CNMS are summarized in Table 1 and illustrated in Figure 1 below.

Table 1: Zone A Validation Results

| Validation Status | Status Type | Total Mileage |
| --- | --- | --- |
| Valid | Being Studied | 56.9 |
| Unverified | Being Studied | 387.6 |
| Assessed | Being Studied | 1048.0 |

Figure 1: UPDT HUC-8 CNMS Validation Status

# 1.0 Introduction

Recent innovations and efficiencies in floodplain mapping have allowed the U.S. Department of Homeland Security’s Federal Emergency Management Agency (FEMA) to develop a process called Base Level Engineering (BLE), which can be used to address current program challenges, including the validation of Zone A studies and the availability of flood risk data in the early stages of a Flood Risk Project. The BLE process involves using best available data and incorporating automated techniques with traditional model development procedures to produce regulatory quality flood hazard boundaries for the 1-percent annual chance event as well as estimates of flood hazard boundaries for multiple recurrence intervals. The cost for developing the data and estimates resulting from the BLE process are lower than standard flood production costs. The BLE results may be used for eventual production of regulatory and non-regulatory products.

As described in Title 42 of the Code of Federal Regulations, Chapter III, Section 4101(e), once every five years, FEMA must evaluate whether the information on Flood Insurance Rate Maps (FIRMs) reflects the current risks in flood prone areas. FEMA makes this determination of flood hazard data validity by examining flood study attributes and change characteristics, as specified in the Validation Checklist of the Coordinated Needs Management Strategy (CNMS) Technical Reference. The CNMS Validation Checklist provides a series of critical and secondary checks to determine the validity of flood hazard areas studied by detailed methods (e.g., Zone AE, AH, or AO). While the critical and secondary elements in CNMS provide a comprehensive method of evaluating the validity of Zone AE studies, a cost-effective approach for evaluating Zone A studies has been lacking.

In addition to the need for Zone A validation guidance, FEMA standards require flood risk data to be provided in the early stages of a Flood Risk Project. FEMA Program Standard SID #29 requires that during Discovery, data must be identified that illustrates potential changes in flood elevation and mapping that may result from the proposed project scope. If available data does not clearly illustrate the likely changes, an analysis is required that estimates the likely changes. This data and any associated analyses should be shared, and results should be discussed with stakeholders. An important goal of the BLE process is the scalability of the results. Scalability means that the results of a BLE analysis can not only be used for CNMS evaluations of Zone A studies but can also be leveraged throughout the Risk MAP program. The data resulting from a BLE analysis can be updated as needed and used for the eventual production of regulatory and non-regulatory products, outreach and risk communication, and MT-1 processing. Leveraging this data outside the Risk MAP program may also be valuable to external stakeholders.

TWDB contracted Freese and Nichols, Inc. to complete a BLE analysis for the Upper Prairie Dog Town Fork Red Watershed, shown in Figure 2, in Northwest Texas to support FEMA’s Discovery process and validation of effective Zone A Special Flood Hazard Areas (SFHA). The study extents include portions of Armstrong, Briscoe, Carson, Donley, Hall, Potter, Randall, and Swisher Counties, and include the following communities: the Cities of Amarillo, Canyon, Claude, Happy, Lake Tanglewood, Palisades, and Timbercreek Canyon. The study area consists of six HUC-10 basins: Headwaters Prairie Dog, Happy Draw, Gip Creek, Battle Creek, Headwaters Mulberry, and Mulberry Creek.

Figure 2: Upper Prairie Dog Town Fork Red HUC-10 Basins

The Base Level Engineering assessment prepared cursory engineering analysis for approximately 4,340 miles of stream reaches within the Upper Prairie Dog Town Fork Red Watershed with a minimum drainage area tolerance of one square mile. The watershed is divided into two subregions with distinct topography for each subregion. The distinct topographies correlate to unique flood infrastructure for each of the two subregions. The upper region of the basin is dominated by the playa lake system while the lower region of the basin is comprised primarily of rivers, streams, and lakes. Due to the varied natural infrastructure, stream reach delineations were only extended through the lower regions of rivers and streams.  The playa lakes are generally hydraulically disconnected and therefore stream reaches were not created for these features. The BLE analysis was completed using a 2D grid to represent the watershed and therefore the entire watershed was included in hydrologic and hydraulic modeling. The following sections will summarize the BLE process and will discuss the results along with their recommended use.

# 2.0 2D BLE Modeling Inputs and Controls

Section 2.0 discusses the fundamental components required to execute a 2D BLE analysis for the Upper Prairie Dog Town Fork Red Watershed. Inputs including elevation data, hydrology from rain-on-grid and inflow hydrographs, and hydraulic analyses and variables are defined in the following sections. The HEC-RAS 6.3.0 rain-on-grid utility was applied in the production of these Texas 2D BLE products.

## 2.1 Topographic Data

A high-resolution Digital Elevation Model (DEM) derived from Light Detection and Ranging (LiDAR) provides a detailed representation of the Earth’s surface. DEMs are one of the fundamental components for 2D engineering analyses and are used to direct the hydrology through the model area. The seamless DEMs developed to support the 2D BLE modeling and analysis, within the Upper Prairie Dog Town Fork Red watersheds, were executed using the following steps:

- Available 1-meter DEMs within each watershed were inventoried and sourced from FEMA, NOAA, and USGS;

- Seamless DEMs were produced using GIS spatial analyst and 3D analyst tools and resampled to 10-foot resolution; and,

- Quality Assurance (QA) and Quality Control (QC) measures were implemented to ensure topographic data meets quality standards of the project.

Additional documentation regarding DEMs including coverage, accuracy, acquisition dates, and source contact/agency are presented in the figures, tables, and text in the sections below.

### 2.1.1 Inventory

DEMs were inventoried for each HUC8 watershed, including a 1-mile buffer around each watershed boundary. Figure 3 depicts the datasets identified and leveraged for each watershed. Data from USGS were queried to build the inventory with the most current and available data sources.

### 2.1.2 Evaluation

A data coverage assessment was conducted to check for data gaps, extent, accuracy, and completeness. A review of related documentation, reports, indexes, and metadata associated with the leveraged datasets ensured each dataset meets FEMA accuracy requirements for topographic data as defined in the FEMA 2020 BLE Region 6 Submittal Guidance document. Table 2 depicts the complete list of source elevation data and attributes leveraged for the Upper Prairie Dog Town Fork Red watershed.

Table 2: Source Topographic Data (DEMs) Leveraged for Upper Prairie Dog Town Fork Red Watershed

| Watershed | Counties | Year | Description | Data Type | 3DEP Quality Level | Vertical Accuracy (RMSE) | Source/ Owner | Approx. Footprint (Sq. Mi.) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Upper Prairie Dog Town Fork Red (HUC 11120103) | Armstrong, Briscoe, Carson, Donley, Hall, Potter, Randall, Swisher | 2018 | USGS Texas Panhandle LiDAR | Airborne LiDAR | 2 | 4.3 cm | USGS | 2,152 |

#### 2.1.2.1 Watershed Source Terrain Data

The source elevation data for the Upper Prairie Dog Town Fork Red watershed was 1-meter DEMs derived from the 2018 USGS Texas Panhandle LIDAR data. Figure 3 depicts the extent of the data defined in Table 2.

Figure 3: Upper Prairie Dog Town Fork Red Watershed BLE Source Terrain Data

USGS 2018 FEMA Texas Panhandle LiDAR

The 2018 FEMA Texas Panhandle LiDAR was acquired between February 9, 2018, to September 9, 2019. The vertical accuracy for the dataset is reported to be 4.3 cm RMSE at a 95% confidence level which meets project accuracy specifications of the NSSDA. These accuracies meet FEMA standards for floodplain reevaluation studies and map modernization programs designed to update the Flood Insurance Rate Maps (FIRM).

### 2.1.3 Data Development Methodology

DEMs were processed for the Upper Prairie Dog Town Fork Red watershed for the BLE modeling efforts. Tools within ArcGIS were used to apply horizontal projection transformations and vertical datum shifts. The DEMs were projected horizontally to North American Datum of 1983 (NAD83), State Plane Coordinate System (SPCS) Texas North Central (4201) in feet. Additionally, all DEMs were adjusted vertically to NAVD88 in feet.

### 2.1.4 DEM QA/QC

#### 2.1.4.1 DEM Quality Assurance

Seamless DEMs developed for use in the Upper Prairie Dog Town Fork Red watershed BLE analyses were developed and independently assured to meet QA standards of the project. QA standards during the data development process included, but were not limited to the following:

- Horizontal Projection Check (NAD83 SPCS 4201 FT)

- Vertical Datum Check (NAVD88 FT)

- Resolution Check (1-meter to 10-foot resolution)

Seamless Data Check to ensure the DEM files are consistent and seamless along source data edges

#### 2.1.4.2 DEM Quality Control

QC measures were implemented following the development process of the seamless DEMs and included visual observations using hillshade, color rendering, and/or other visual aids to review and identify potential impactful anomalies within the DEM surfaces.  This QC step included, but was not limited to the following:

- Seamless Data Check to ensure no voids along the edges and between the prioritized datasets

- NoData Value Check to ensure no null values

- Manual Elevation Check using hillshade rasters to find erroneous elevation issues

Following these QA/QC steps, the final DEM data developed for the Tule and Upper Prairie Dog Town Fork Red watersheds are assured to meet FEMA standards and present a representative surface developed from leveraged elevation data for the purposes of this BLE project.

## 2.2 2D BLE Methods

The following sections describe the methodology used in the 2D BLE analysis of the Upper Prairie Dog Town Fork Red Watershed. This section includes discussion regarding the overall approach along with hydrologic and hydraulic methodologies and results.

### 2.2.1 Model Areas

The Upper Prairie Dog Town Fork Red Watershed covers a total area of 2,152 square miles. While possible to model the entire HUC8 in a single 2D HEC-RAS model, the large area would lead to excessively long model run times and inhibit the model’s usability for future users. To combat this issue and simplify model development, the study team divided the Upper Prairie Dog Town Fork Red HUC8 into six models, one for each HUC10 area. Table 3 presents each 2D model and Figure 4 provides a visual of the model boundaries. As described in Section 2.2.2 Model and Boundary Condition Setup, these models remain interconnected via internal boundary conditions, so no runoff volume is lost through the subdivision of the HUC8.

Table 3: Upper Prairie Dog Town Fork Red Model Areas

| Model | HUC10 | HUC10 Name | Area (Sq. Mi.) |
| --- | --- | --- | --- |
| 1112010301 | 1112010301 | Headwaters Prairie Dog | 601 |
| 1112010302 | 1112010302 | Happy Draw | 319 |
| 1112010303 | 1112010303 | Gip Creek | 218 |
| 1112010304 | 1112010304 | Battle Creek | 350 |
| 1112010305 | 1112010305 | Headwaters Mulberry | 299 |
| 1112010306 | 1112010306 | Mulberry Creek | 365 |

Figure 4: Upper Prairie Dog Town Fork Red HUC-10 Model Boundaries

### 2.2.2 Model and Boundary Condition Setup

HEC-RAS 2D rain-on-grid modeling requires two main components, a 2D computational mesh and inflow hydrology. HUC10 boundaries with a 1000’ buffer applied were used to establish the 2D computational mesh boundaries within the Upper Prairie Dog Town Fork Red HUC8. Inflow hydrology can be applied through various methods in HEC-RAS 2D modeling, but hydrology methods are limited to two categories  for the Upper Prairie Dog Town Fork Red HUC8. These are flow hydrographs and precipitation time-series. Section 2.2.3.1 Precipitation for HEC-RAS 2D describes the development of the precipitation data applied to each model. Precipitation, rather than excess precipitation, was applied directly to the 2D mesh because hydrologic losses can be accounted for entirely within HEC-RAS version 6.3.0.

Boundary conditions were applied along each model area’s boundary where a transfer of flow (inflow or outflow) may occur. The upstream area of each model, where applicable, has an inflow hydrograph applied that accounts for outflow from the upstream model area. For example, the 1112010302 (Happy Draw) model has an inflow hydrograph at its upstream end that applies the outflow from the 1112010301 (Headwaters Upper Prairie Dog Town) model.

Inflow hydrographs from external HUC-8 watersheds Palo Duro, Tierra Blanca, and Tule were applied as boundary conditions at the appropriate confluence locations. These HUC-8 watersheds are being studied under the same task order contract as this HUC-8 and the 2D Base Level Engineering computed flows were used as inflow hydrographs.

A normal depth boundary condition was utilized along all basin outflows that uses an approximate energy grade-line slope estimated from the LiDAR terrain data. There were also normal depth boundaries applied along all other sections of the model perimeter to prevent ponding. These normal depth boundaries were split up by topography (i.e. riverine, caprock) to estimate the energy grade-line.

### 2.2.3 2D Computational Mesh and Settings

The HEC-RAS 2D computational mesh defines both the extent and level of detail of 2D hydraulic results. The density and detail of the 2D mesh is the primary component that defines the resolution of hydraulic results (e.g., more detail-finer results, less detail-coarser results). A more detailed mesh may provide finely resolved and accurate results, but it can lead excessively long computation times which are prohibitive to future use of BLE models. To provide a balance between mesh detail and model performance, the base mesh for each model was developed using 200’x200’ cells. To add detail to dense urban areas, refinement regions that enforced 100’x100’ cells were added in over urban areas in TxDOT’s municipalities layer. The mesh was further refined through the addition of breaklines along roadways, reservoirs, stream centerlines, and other topographically significant features. The mesh for each model area was developed in a manner that allows for automatic regeneration of the mesh by future users, without the need for manual cell edits.

Hydraulic results are also influenced by the many computational settings used in each HEC-RAS simulation. Of the available settings, the two most influential to results are the computational timestep and the equation set. The models in the Upper Prairie Dog Town Fork Red Watershed, with the exception of 1112010302, use a 30 second timestep with the Diffusion Wave equation set. The 1112010302 (Happy Draw) HUC10 contains many steep drops in terrain that resulted in model instabilities. In response to these instabilities, the Happy Draw HUC10 uses a courant varied timestep. The varied timestep allows the model to determine the appropriate timestep to keep the 2D courant residence time in the ideal range between 0.5 and 2. All other computational settings were kept at their default values. As described in Section 2.3.2, the sensitivity of results to additional computational settings was preliminarily evaluated, but the initial settings listed in this section were maintained following the sensitivity analysis.

### 2.2.3 Hydrology

Hydrologic data for the Upper Prairie Dog Town Fork Red Watershed was derived from NOAA Atlas 14, Volume 11 Version 2.0 precipitation data, which was obtained from NOAA’s Precipitation Frequency Data Server. Spatial variation of precipitation depths was accounted for by developing area-weighted average depths in ArcGIS with NOAA Atlas 14 precipitation raster files for each model area presented in Table 5-Table 10. The 10%, 4%, 2%, 1%-minus, 1%, 1%-plus, and 0.2% annual exceedance probability events were simulated in the Upper Prairie Dog Town Fork Red Watershed.

#### 2.2.3.1 Precipitation for HEC-RAS 2D

The average NOAA Atlas 14 precipitation-frequency depths for each model area in Table 5-Table 10 were converted to a precipitation time-series dataset within a dummy HEC-HMS v4.9 model. A frequency storm was created within HEC-HMS for each event listed in Section 2.2.3 Hydrology. Each frequency storm utilized the parameters listed in Table 4 . A precipitation time-series was then extracted from the HEC-HMS models and utilized in the HEC-RAS 2D simulations.

In addition to the recurrence interval precipitation depths, NOAA Atlas 14 also provides 90% confidence limits for each reported precipitation depth. The 90% confidence intervals correspond to +/- 1.645 standard deviations. The 1%-minus and 1%-plus events utilize the 1% precipitation depths +/- 1.645 standard deviations.

Table 4: HEC-HMS Frequency Storm Parameters

| Parameter | Value |
| --- | --- |
| Annual-Partial Conversion | None |
| Storm Duration | 1-day |
| Intensity Duration | 5-minute |
| Intensity Position | 50% |
| Area Reduction | None |
| Storm Area | - |

Table 5: 1112010301 – Precipitation-Frequency Depths

| Duration (min) | Percent Annual Chance Precipitation Depth (in) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10% | 4% | 2% | 1%- | 1% | 1%+ | 0.2% |
| 5 | 0.61 | 0.74 | 0.86 | 0.76 | 0.97 | 1.19 | 1.26 |
| 15 | 1.20 | 1.47 | 1.69 | 1.50 | 1.93 | 2.36 | 2.49 |
| 60 | 2.07 | 2.53 | 2.92 | 2.58 | 3.32 | 4.07 | 4.27 |
| 120 | 2.47 | 3.02 | 3.48 | 3.10 | 3.96 | 4.82 | 5.12 |
| 180 | 2.70 | 3.30 | 3.80 | 3.41 | 4.33 | 5.25 | 5.61 |
| 360 | 3.11 | 3.80 | 4.37 | 3.95 | 4.97 | 6.00 | 6.47 |
| 720 | 3.54 | 4.33 | 4.97 | 4.52 | 5.65 | 6.77 | 7.39 |
| 1440 | 4.02 | 4.91 | 5.62 | 5.15 | 6.37 | 7.59 | 8.32 |

Table 6: 1112010302 – Precipitation-Frequency Depths

| Duration (min) | Percent Annual Chance Precipitation Depth (in) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10% | 4% | 2% | 1%- | 1% | 1%+ | 0.2% |
| 5 | 0.60 | 0.73 | 0.85 | 0.76 | 0.98 | 1.20 | 1.26 |
| 15 | 1.18 | 1.45 | 1.69 | 1.50 | 1.94 | 2.38 | 2.50 |
| 60 | 2.02 | 2.48 | 2.89 | 2.57 | 3.32 | 4.07 | 4.27 |
| 120 | 2.42 | 2.97 | 3.44 | 3.07 | 3.94 | 4.80 | 5.08 |
| 180 | 2.66 | 3.25 | 3.76 | 3.36 | 4.29 | 5.21 | 5.55 |
| 360 | 3.07 | 3.76 | 4.33 | 3.89 | 4.93 | 5.96 | 6.41 |
| 720 | 3.51 | 4.31 | 4.96 | 4.50 | 5.65 | 6.80 | 7.44 |
| 1440 | 4.01 | 4.92 | 5.66 | 5.17 | 6.44 | 7.71 | 8.51 |

Table 7: 1112010303 – Precipitation-Frequency Depths

| Duration (min) | Percent Annual Chance Precipitation Depth (in) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10% | 4% | 2% | 1%- | 1% | 1%+ | 0.2% |
| 5 | 0.61 | 0.76 | 0.88 | 0.78 | 1.01 | 1.24 | 1.32 |
| 15 | 1.22 | 1.50 | 1.74 | 1.55 | 2.00 | 2.45 | 2.60 |
| 60 | 2.09 | 2.57 | 2.99 | 2.66 | 3.44 | 4.21 | 4.47 |
| 120 | 2.52 | 3.10 | 3.60 | 3.22 | 4.12 | 5.03 | 5.40 |
| 180 | 2.77 | 3.41 | 3.95 | 3.54 | 4.52 | 5.50 | 5.95 |
| 360 | 3.21 | 3.96 | 4.58 | 4.14 | 5.24 | 6.34 | 6.93 |
| 720 | 3.67 | 4.55 | 5.27 | 4.82 | 6.05 | 7.28 | 8.05 |
| 1440 | 4.17 | 5.18 | 6.00 | 5.53 | 6.89 | 8.24 | 9.15 |

Table 8: 1112010304 – Precipitation-Frequency Depths

| Duration (min) | Percent Annual Chance Precipitation Depth (in) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10% | 4% | 2% | 1%- | 1% | 1%+ | 0.2% |
| 5 | 0.63 | 0.77 | 0.88 | 0.76 | 0.99 | 1.21 | 1.28 |
| 15 | 1.25 | 1.52 | 1.74 | 1.52 | 1.96 | 2.41 | 2.53 |
| 60 | 2.16 | 2.63 | 2.99 | 2.60 | 3.36 | 4.13 | 4.36 |
| 120 | 2.60 | 3.17 | 3.62 | 3.19 | 4.09 | 5.00 | 5.36 |
| 180 | 2.85 | 3.50 | 4.00 | 3.55 | 4.53 | 5.52 | 5.97 |
| 360 | 3.30 | 4.07 | 4.68 | 4.20 | 5.32 | 6.45 | 7.06 |
| 720 | 3.76 | 4.67 | 5.42 | 4.94 | 6.22 | 7.49 | 8.28 |
| 1440 | 4.27 | 5.32 | 6.19 | 5.70 | 7.12 | 8.54 | 9.47 |

Table 9: 1112010305 – Precipitation-Frequency Depths

| Duration (min) | Percent Annual Chance Precipitation Depth (in) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10% | 4% | 2% | 1%- | 1% | 1%+ | 0.2% |
| 5 | 0.63 | 0.77 | 0.88 | 0.78 | 1.01 | 1.23 | 1.32 |
| 15 | 1.24 | 1.52 | 1.75 | 1.56 | 2.00 | 2.43 | 2.61 |
| 60 | 2.14 | 2.62 | 3.01 | 2.68 | 3.43 | 4.19 | 4.48 |
| 120 | 2.55 | 3.12 | 3.59 | 3.22 | 4.09 | 4.96 | 5.34 |
| 180 | 2.79 | 3.41 | 3.92 | 3.53 | 4.46 | 5.39 | 5.82 |
| 360 | 3.20 | 3.93 | 4.51 | 4.09 | 5.12 | 6.15 | 6.69 |
| 720 | 3.64 | 4.46 | 5.13 | 4.70 | 5.83 | 6.96 | 7.65 |
| 1440 | 4.12 | 5.05 | 5.79 | 5.36 | 6.58 | 7.80 | 8.62 |

Table 10: 1112010306 – Precipitation-Frequency Depths

| Duration (min) | Percent Annual Chance Precipitation Depth (in) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10% | 4% | 2% | 1%- | 1% | 1%+ | 0.2% |
| 5 | 0.63 | 0.77 | 0.87 | 0.76 | 0.98 | 1.20 | 1.27 |
| 15 | 1.26 | 1.52 | 1.74 | 1.52 | 1.96 | 2.40 | 2.52 |
| 60 | 2.20 | 2.66 | 3.02 | 2.64 | 3.40 | 4.16 | 4.40 |
| 120 | 2.65 | 3.23 | 3.68 | 3.25 | 4.16 | 5.06 | 5.42 |
| 180 | 2.92 | 3.56 | 4.07 | 3.63 | 4.61 | 5.60 | 6.04 |
| 360 | 3.40 | 4.16 | 4.78 | 4.30 | 5.43 | 6.55 | 7.16 |
| 720 | 3.89 | 4.80 | 5.54 | 5.06 | 6.33 | 7.60 | 8.44 |
| 1440 | 4.43 | 5.47 | 6.32 | 5.84 | 7.24 | 8.64 | 9.68 |

#### 2.2.3.2 Hydrologic Losses

Hydrologic losses were calculated within HEC-RAS according to the SCS Curve Number method. Curve numbers were calculated by intersecting the National Land Cover Dataset (NLCD) 2019 with soils data obtained from the USDA Gridded Soil Survey Geographic Database (gSSURGO). A curve number was then assigned to each resultant value of the intersected land cover classification and hydrologic soil group. Based on results from the sensitivity analyses and stream gage analysis, and adjustment of minus 15 was applied to the Antecedent Runoff Condition II (ARC II), or average antecedent moisture conditions, as presented in Table 11. A minimum curve number of 30 was utilized in all scenarios. These parameters represent the default Curve Numbers applied to the HEC-RAS 2D models. The values presented in Table 11 were utilized in all studied events. The initial abstraction ratio for the default CN layer was set to 0.20.

Table 11: Land Use-Soils-CN Matrix

| Land Use Grid Code | NLCD Land Use Description | Hydrologic Soil Group |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | A | B | C | D |
| 11 | Open Water | 84 | 84 | 84 | 84 |
| 21 | Developed Open Space | 34 | 54 | 64 | 69 |
| 22 | Developed Low Intensity | 46 | 60 | 68 | 72 |
| 23 | Developed Medium Intensity | 66 | 73 | 76 | 78 |
| 24 | Developed High Intensity | 74 | 77 | 79 | 80 |
| 31 | Barren Land | 30 | 46 | 59 | 65 |
| 41 | Deciduous Forest | 30 | 40 | 55 | 62 |
| 42 | Evergreen Forest | 30 | 40 | 55 | 62 |
| 43 | Mixed Forest | 30 | 40 | 55 | 62 |
| 52 | Shrub Scrub | 30 | 33 | 50 | 58 |
| 71 | Herbaceous | 34 | 47 | 59 | 70 |
| 81 | Hay Pasture | 30 | 46 | 59 | 69 |
| 82 | Cultivated Crops | 36 | 52 | 61 | 65 |
| 90 | Woody Wetlands | 57 | 65 | 72 | 78 |
| 95 | Emergent Herbaceous Wetlands | 57 | 65 | 72 | 78 |

### 2.2.4 Hydraulics

Hydraulic analysis of the Upper Prairie Dog Town Fork Red Watershed was conducted in HEC-RAS v6.3.0 using a rain-on-grid modeling approach. This section describes the development of hydraulic input data and its implementation in the 2D model.

#### 2.2.4.1 Roughness Coefficients

A manning’s roughness layer was developed for the 2D mesh by applying typical values associated with NLCD land cover classifications. In addition to the base manning’s roughness layer from the NLCD data, manning’s roughness values were refined along stream centerlines to override excessively high channel roughness values resulting from the NLCD layer. Channel roughness values were assigned according to a review of aerial imagery and generally range from 0.035 – 0.06. Table 12 documents the typical roughness values assigned to each NLCD land cover classification.

Table 12-NLCD 2019 Manning's Roughness Values

| Land Use Grid Code | NLCD Land Use Description | Minimum Manning’s N | Maximum Manning’s N | Selected Manning’s N |
| --- | --- | --- | --- | --- |
| 11 | Open Water | 0.025 | 0.05 | 0.03 |
| 21 | Developed Open Space | 0.03 | 0.05 | 0.04 |
| 22 | Developed Low Intensity | 0.06 | 0.12 | 0.06 |
| 23 | Developed Medium Intensity | 0.08 | 0.16 | 0.08 |
| 24 | Developed High Intensity | 0.12 | 0.2 | 0.12 |
| 31 | Barren Land | 0.023 | 0.03 | 0.03 |
| 41 | Deciduous Forest | 0.10 | 0.20 | 0.10 |
| 42 | Evergreen Forest | 0.08 | 0.16 | 0.10 |
| 43 | Mixed Forest | 0.08 | 0.20 | 0.10 |
| 52 | Shrub Scrub | 0.07 | 0.16 | 0.07 |
| 71 | Herbaceous | 0.025 | 0.05 | 0.035 |
| 81 | Hay Pasture | 0.025 | 0.05 | 0.045 |
| 82 | Cultivated Crops | 0.02 | 0.05 | 0.045 |
| 90 | Woody Wetlands | 0.045 | 0.15 | 0.10 |
| 95 | Emergent Herbaceous Wetlands | 0.05 | 0.085 | 0.07 |

#### 2.2.4.2 Breaklines and Hydraulic Structures

Breaklines were used within the 2D mesh to align cell faces along stream centerlines, roadway and railroad embankments, and other hydraulically significant features. Stream centerlines generated from the terrain data were enforced to align cell faces perpendicular to flow. These breaklines were delineated to 0.1 square mile by automated process and manually cleaned to meet modeling needs. With the unique topography of West Texas, stream centerlines were ended when the flow regime changed from riverine to a network of playas and overflows.

Roadway centerlines were initially identified within ArcGIS and then imported into HEC-RAS as breaklines. Culvert crossings at roadways with an embankment height greater than 2m, a volume behind the embankment greater than 5 ac-ft, and a contributing drainage area greater than 1 sqmi were identified as hydraulically significant. For hydraulically significant features, v-notches were added at the culvert crossing to approximate the capacity of the culvert. Adding v-notches to the significant roadway embankments aids in appropriately accounting for storage within the watershed. Figure 5 shows an example of a v-notch breakline. For crossings through multi-lane highways that span more than the length of a cell, the v-notch approach is insufficient, and thus a channel terrain modification was applied. Additionally, a reference line was added to every dam in the TCEQ inventory within the model boundaries to improve model usability for future users.

Figure 5: V-Notch Breakline Example

## 2.3 Model Results

The results of this study produced a SFHA boundary that can serve as an estimate for flood risk in areas which have not been previously studied. The discharge rates measured at key locations in the watershed compare reasonably well with the stream gage analysis. These results provide context for flood risk within the watershed and should be verified through community meetings before being applied to a regulatory product.

### 2.3.1 Stream Gage Analysis

There are a total of four USGS gages within the Upper Prairie Dog Town Fork Red watershed. Of the four, only two USGS gages within the Upper Prairie Dog Town Fork Red Watershed had valid statistical peak flow data available for an adequate length of record. The statistical peak flow data from the 1% AEP storm at USGS Stream Gages 07298500 and 07297910 were determined to be the most relevant values on which to focus model validation efforts. Details about the relevant Upper Prairie Dog Town Fork Red gages and their locations can be seen in Table 13 and Figure 6. The statistical peak flows for the USGS Stream Gages were produced using HEC-SSP 17B and can be found in Table 14.

Table 13: USGS Stream Gages in Tule HUC8

| ID | Gage Name | Notes | Years | Valid Years | Used? |
| --- | --- | --- | --- | --- | --- |
| 7297500 | Pr Dog Twn Fk Red Rv nr Canyon | Misc. years available in 1920s and 1940s – Not Valid | 13 | 0 | No |
| 7297910 | Pr Dog Twn Fk Red Rv nr Wayside | Current Buffalo Lake control structure completed 1992 | 55 | 30 | Yes |
| 7299000 | Mulberry Ck nr Brice | Not enough data | 2 | 2 | No |
| 7298500 | Pr Dog Twn Fk Red Rv nr Brice | Valid Years: 2003 -2022 | 32 | 19 | Yes |

Figure 6: USGS Gages within UPDT HUC8

Table 14: USGS Gage Statistical Peak Streamflows

| Location | 1% AEP Peak Flow (cfs) |
| --- | --- |
| 07297910 – Pr Dog Twn Fk Red Rv nr Wayside, TX | 41,605 |
| 07298500 – Pr Dog Twn Fk Red Rv nr Brice, TX | 75,683 |

### 2.3.2 Sensitivity Analysis

To evaluate which model parameters significantly impact results, multiple sensitivity analyses were run with varied parameters. Table 15 presents the parameters varied for each sensitivity run. All sensitivity analyses were computed for the 1% frequency event.

Table 15: Sensitivity Analysis Parameters

| Sensitivity Run | Category | Infiltration Layer | Manning’s Roughness | Equation Set | Computational Timestep | Initial Conditions |
| --- | --- | --- | --- | --- | --- | --- |
| Base Model | - | CN | Base | Diffusion Wave | Varied | Dry |
| Full Momentum | Numerical Sensitivity | CN | Base | Full Momentum | Varied | Dry |
| Reduced Timestep (1/3 Base) | Timestep Sensitivity | CN | Base | Diffusion Wave | Varied | Dry |
| CN1.5 | Physical Parameter Sensitivity | CN1.5 | Base | Diffusion Wave | Varied | Dry |
| CNIII | Physical Parameter Sensitivity | CNIII | Base | Diffusion Wave | Varied | Dry |
| Manning’s n - 50% | Physical Parameter Sensitivity | CN | N*50% | Diffusion Wave | Varied | Dry |
| Manning’s n + 50% | Physical Parameter Sensitivity | CN | N*150% | Diffusion Wave | Varied | Dry |

The results of the sensitivity analyses at selected USGS gage locations are summarized within the watershed in Table 16 through Table 18.

Table 16: Sensitivity Analysis Results – Peak Flow Results (cfs)

| Location | Base Model | Full Momentum | Reduced Timestep (1/3 Base) | CN1.5 | CNIII | Manning’s n – 50% | Manning’s n + 50% |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 07297910 – Happy Draw | 54,906 | 55,457 | 54,652 | 45,113 | 103,450 | 73,921 | 52,169 |
| 07298500 – Mulb Ck | 120,512 | 111,347 | 111,610 | 70,228 | 179,026 | 123,658 | 109,272 |

Table 17: Sensitivity Analysis Results – Time of Peak (hr)

| Location | Base Model | Full Momentum | Reduced Timestep (1/3 Base) | CN1.5 | CNIII | Manning’s n – 50% | Manning’s n + 50% |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 07297910 – Happy Draw | 19.00 | 19.00 | 19.00 | 31.00 | 17.00 | 17.00 | 31.00 |
| 07298500 – Mulb Ck | 18.00 | 19.00 | 19.00 | 21.00 | 18.00 | 18.00 | 19.00 |

Table 18: Sensitivity Analysis Results - Peak Water Surface Elevation (ft)

| Location | Base Model | Full Momentum | Reduced Timestep (1/3 Base) | CN1.5 | CNIII | Manning’s n – 50% | Manning’s n + 50% |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 07297910 – Happy Draw | 2481.6 | 2481.9 | 2481.6 | 2481.0 | 2481.1 | 2480.7 | 2482.1 |
| 07298500 – Mulb Ck | 2067.1 | 2067.3 | 2067.1 | 2065.6 | 2069.0 | 2066.1 | 2067.8 |

The sensitivity analysis revealed the base model assumptions for equations selection and time step did not significantly impact model results and therefore no changes were made to the selected computational settings. The sensitivity analysis demonstrated the potential impacts to the flooding results based on curve number and manning’s n values. These parameters were considered in the model refinement when validating to available data.

### 2.3.3 Frequency Event Results

The results of the analysis at selected USGS gage locations are summarized in Table 19 for each simulated frequency event and are compared against the USGS gage values for the 1% event in Table 20. There is significant variability between the observed data and the computed curve for the Wayside gage in Happy Draw. For this reason, gage comparison the Brice gage in Mulberry Creek was prioritized for model validation.

Table 19: Frequency Event Peak Discharge Summary at Selected USGS Gages (cfs)

| Location | 10% | 4% | 2% | 1%- | 1% | 1%+ | 0.2% |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 07297910 – Happy Draw | 24,547 | 36,578 | 44,306 | 11,863 | 58,721 | 170,590 | 102,905 |
| 07298500 – Mulb Ck | 24,700 | 42,293 | 55,464 | 26,285 | 73,727 | 207,956 | 139,218 |

Table 20: Difference in 1% Event Discharges at Selected USGS Gages

| Location | Statistical Peak Flow (cfs) | Model Peak Flow (cfs) | Percent Difference (%) |
| --- | --- | --- | --- |
| 07297910 – Happy Draw | 41,605 | 58,721 | 34 |
| 07298500 – Mulb Ck | 75,683 | 73,727 | 03 |

### 2.3.4 Challenges

The lack of stream gages present challenges for the 2D BLE analysis. The scarcity of recent stream gage records within the Upper Prairie Dog Town Fork Red and surrounding watersheds reduces comparison points for the analysis and limits the ability to simulate historical events which could verify the 2D BLE results and approach. Without many statistically significant gages in the upstream HUC 8s, there is very little way to validate the inflows into the UPDT watershed. This is further complicated by the presence of reservoirs in the Tule and Tierra Blanca HUC 8s that could significantly impact the Upper Prairie Dog Town Fork Red flows.

### 2.3.5 Recommendations

This study provides useful information for flood risk identification and communication among stakeholders within the watershed. This study is scalable, and additional analysis will enhance the product and further inform the implementation of regulatory flood hazard areas. The validity of the 2D BLE analysis should be verified through community meetings prior to being applied as a regulatory product.

# 3.0 Quality Control

Text

# 4.0 Floodplain Delineation (10%, 1% and 0.2% events)

Text

Challenges

Text

Results and Recommendations

Text

## CNMS Validation of Effective Zone A Special Flood Hazard Area (SFHA)

Text

Initial Assessment A1 – Significant Topography Update Check

Text

Initial Assessment A2 – Check for Significant Hydrology Changes

Text

Initial Assessment A3 – Check for Significant Development

Text

Initial Assessment A4 – Check of Studies Backed by Technical Data

Text

Initial Assessment A5 – Comparison of BLE and Effective Zone A Floodplains

Text

CNMS Validation Results

Text

# References

Bibliography list of references used and referred to throughout document.

- USGS. Multi-Resolution Land Characteristics Consortium. National Land Cover Database 2011. (http://www.mrlc.gov/nlcd2011.php).

- Chow, Ven T. "Development of Uniform Flow and Its Formulas." Open Channel Hydraulics. Caldwell, NJ: Blackburn, 1959. 109-113. Print.

- FEMA, “Guidance for Flood Risk Analysis and Mapping – First Order Approximation”, November 2014. (http://www.fema.gov/media-library-data/1420849667914-c1656173985d814d0e62f80818505969/FOA_Guidance_Nov_2014.pdf).

- FEMA Region 6, “Base Level Engineering Submittal Guidance”, June 2017. (https://rmd.msc.fema.gov/Regions/VI/Ph0_Investment/1/Base%20Level%20Engineering/Submittal%20Guidance/20170615_BLE%20Submittal%20Guidance_R6.pdf).

# Appendix A – Summary of Peak Flow Values (Q)

| Stream Name | Drainage Area (Sq. Miles) | 1% Peak Q (cfs) | 1% minus Peak Q (cfs) | 1% plus Peak Q (cfs) |
| --- | --- | --- | --- | --- |

# Appendix B – Exhibits – Base Level Engineering Results

Add exhibits here to depict results, recommendations, etc… that may be useful to any community/end user of the BLE data for further refinement in the future.