# Texas 2D BLE Analysis QA/QC Checklist

# Project Information

| HUC-8 Watershed Name & ID: | Upper Prairie Dog Town Fork Red (11120103) |
| --- | --- |
| HUC-10 Watershed Name & ID: |  |
| Modeler: | Holly Ahumada |
| Reviewer: | Leah Horick |

## Terrain and Modeling Review (QC Items 1-6)

| Modeler Initials: |  | Reviewer Initials: |  |
| --- | --- | --- | --- |
| Submittal Date: |  | Review Submitted Date: |  |
| Review Response Date: |  | Backcheck Complete Date: |  |

## Mapping Review (QC Item 7)

| Modeler Initials: | HA | Reviewer Initials: | LH |
| --- | --- | --- | --- |
| Submittal Date: | 8/23/23 | Review Submitted Date: | 8/30/23 |
| Review Response Date: |  | Backcheck Complete Date: |  |

## Completeness Review (QC Item 8)

| Modeler Initials: |  | Reviewer Initials: |  |
| --- | --- | --- | --- |
| Submittal Date: |  | Review Submitted Date: |  |
| Review Response Date: |  | Backcheck Complete Date: |  |

(Modeler to populate with an “X” for items included in the QC submittal)

| Level of Study |  |  | Flood Events |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 2D BLE |  |  | 10% ACE (10-year) |  | 0.2% ACE (500-year) |
|  | Enhanced BLE |  |  | 4% ACE (25-year) |  | 1% plus |
|  |  |  |  | 2% ACE (50-year) |  | 1% minus |
|  |  |  |  | 1% ACE (100-year) |  | Observed Rainfall |

Instructions

Each section includes instructions to the modeler and reviewer regarding their responsibilities in the completion of this checklist. In the Methodologies / Data Sources tables, grey italic text is added as an example response. Additional sample responses are located in the Texas 2D BLE Guidelines Quick Guide.

Reviewer to indicate if Technical Review items generally comply using a “Pass”. If the technical review observes an issue of noncompliance or an item that requires additional attention, an “Fail” should be utilized.

## Submittal Checklist

Data submitted for each milestone should be carried through each subsequent milestone. All geospatial layers and models must have same defined projection system as terrain. Modeler to populate with an “X” for items included in the QC submittal.

| Base Map Data (QC items 1-6) |  |
| --- | --- |
|  | Location Map (PDF): HUC-8 and HUC-10 boundaries and/or model domain boundary if different from HUC-10s, stream gages, dams (if applicable), mainstem streams (for reference), background map |
|  | GIS Map (MXD or Map Package) with the following data included but not limited to: Stream centerlines, model breaklines, rainfall data, WSEL and Depth grids (10, 100, 500-yr), adjacent HUC mapping (available on the BLE viewer or currently studying), 2D connections for inflows/outflows, gages or points of flow comparisons (BFEs, highwater marks, etc.), profile lines, boundary condition lines, refinement regions, model domain boundary, model mesh, DFIRM mapping panels, base map data (roads, aerials, etc.), soils data, land use data, terrain |
| Technical Report / Documentation (QC items 1-6) |  |
|  | Technical Report / Project Notebook: assumptions, special considerations, and modeling concerns |
|  | Detailed explanation and support data of variances in methodologies and approaches from recommended guidelines (Should be included in Technical Report / Project Notebook) |
|  | Internal QA/QC Documentation: comments and responses |
|  | Flow comparison tables within the technical report |
| Hydrology (QC items 1-6) |  |
|  | Hydrologic Model (HEC-HMS) model with all associated DSS files for 7 required storm events |
|  | Hydrologic Data and Parameter Spreadsheets (rainfall calculations, curve number table, gage locations, gage analysis and flow validation) |
|  | Hydrologic modeling notebook including assumptions, notes, and special conditions (if not provided in report) |
| Hydraulics (QC items 1-6) |  |
|  | Hydraulic Model (HEC-RAS) with all associated files and 7 required storm events (remove non-final files)d |
|  | Hydraulic Data and Parameter Spreadsheets (Manning’s n-value table, infiltration calculations support data-Soils, Land Use, Curve Number table, other calculations/data used in model) |
|  | Manning’s n-value table |
|  | Precipitation data correctly linked to hydrologic model or hard coded from provided spreadsheet |
|  | Hydraulic modeling notebook including assumptions, notes, and special conditions |
| Mapping (QC item 7) |  |
|  | Flood risk database |
|  | Modeler to complete topology validation as outlines in the guidelines |
|  | Modeler to verify mapping meets deliverable requirements as outlined in guidelines |
| Completeness Review (QC item 8) See Texas 2D BLE Guidelines Chapter 4 – Deliverables |  |
|  | Technical BLE Report |
|  | Final cleaned hydrologic model |
|  | Final cleaned hydraulic model |
|  | Flood risk database |
|  | CMNS updates |
|  | HAZUS results |
|  | Additional supplemental data |

# 1 – Model Terrain

Methodologies / Data Sources (Modeler to populate)

| Terrain Source(s): |  |
| --- | --- |
| Data Gaps: |  |
| Source Resolution: |  |
| Model Terrain Resolution: |  |
| Terrain Horizontal Projection: |  |
| Terrain Vertical Projection: |  |
| Z Values: |  |
| Terrain Buffer: |  |
| Other: |  |

## Technical Review (Reviewer to confirm with Pass/Fail)

| General Review |  |
| --- | --- |
|  | LiDAR source matches the latest available data at time of project initiation |
|  | Large reservoir or lake extents included in model domain |
|  | Check visible seams, gaps or holes of LiDAR sources (if more than one source was used) |

## Review Comments

# 2 – Hydrology

## Methodologies / Data Sources (Modeler to populate)

| Software: |  |
| --- | --- |
| Rainfall Source: |  |
| Rainfall Duration: |  |
| Depth Selection: |  |
| Rainfall Temporal Distribution: |  |
| Areal Reduction: |  |
| Loss Methodology: |  |
| Unit Hydrograph Transform Method: |  |
| Inflow Hydrograph Source: |  |
| Other: |  |

## Statistical Data (Modeler to populate)

| Stream Gage Name & ID: |  |
| --- | --- |
| Stream Gage Name & ID: |  |
| Stream Gage Name & ID: |  |

## Technical Review (Reviewer to confirm with  Pass/Fail)

| Excess Rainfall |  |
| --- | --- |
|  | Precipitation values are an aerial average of Atlas 14 values for the entire 2D area boundary for each storm event or other appropriate rainfall application |
|  | CN values appropriately reflect land use and soils for entire 2D area boundary |
|  | Losses are correctly calculated to establish the excess rainfall hyetographs by checking land use, soils and infiltration layers or other relative information |
|  | Total precipitation depths for 1%+ and 1%- are calculated correctly |
|  | Excess rainfall hyetographs are calculated for all storm events (10%, 4%, 2%, 1%, 0.2%, 1%+, and 1%-) |
|  | Cumulative excess rainfall values are in increasing order from 10%, 4%, 2%, 1%, to 0.2% |
|  | Cumulative excess rainfall values for the 1%+ and 1%- fall in order with the storm events matching the total precipitation values |
| Inflow Hydrographs |  |
|  | Shape and time-to-peak of any inflow hydrographs resemble past documented large storm events |
|  | Hydrographs reasonable when compared to the other return periods. The shapes should be similar, and hydrographs from different events should not intersect unless there is an engineering reason. Peak flows from a lower return period event should not be higher than a higher return period event |
|  | Contributing hydrographs from another RAS model applied at coincident locations (Provide locations in base map data) |
| General Hydrology |  |
|  | Any flow regulating dams in the model area? (Yes/No) |
|  | Dam modeling methodologies applied and explained? (Yes/No). If yes, provide discharge rating curve calculations and gage analysis documenting regulated versus unregulated records, if applicable. |
|  | Model aligns with documentation of dam modeling approaches. |
| Other Notes: NA |  |

## Review Comments

# 3 – Hydraulic Geometry

## Methodologies / Data Sources (Modeler to populate)

| Software: |  |
| --- | --- |
| Loss Methodology: |  |
| Soils Source: |  |
| Land Cover Source: |  |
| Manning's Roughness Source: |  |
| Nominal Mesh Cell Sizes: |  |
| Other: |  |

## Technical Review (Reviewer to confirm with Pass/Fail)

| 2D Mesh |  |
| --- | --- |
|  | 2D Boundary delineated along watershed boundary or buffer to ensure all contributing area is included |
|  | Nominal cell size appropriate for model detail (generally 200 feet, minimum 50 feet for refinement areas) |
|  | Computation points are at default locations or changes documented |
|  | Mesh Cell and Face tolerances are default or changes documented |
|  | No mesh errors |
|  | Cell face relationships are calculated |
| Breaklines |  |
|  | Breaklines are consistent with the terrain data |
|  | Breaklines sufficiently detailed to capture features (thalweg, banks, berms, etc.) |
|  | Cells do not overlap terrain features where appropriate (i.e., unwanted leaking) |
|  | Breakline cell sizing and parameters are appropriate |
|  | 2D Area Connections placed at applicable benchmarks, gages, or dams |
|  | 2D Area Connections set to use normal 2D equation domain for benchmarks and gages. Appropriate modeling approach used for dams. |
| Manning’s N-Values |  |
|  | Manning’s n-values associated to land cover data provided |
|  | Land Cover classifications are sufficiently descriptive within the data table |
|  | Manning’s n-value classifications generally match current aerial imagery |
|  | Manning’s n dataset is associated with the geometry files |
| General |  |
|  | All model datasets each have unique descriptive names |

## Review Comments

# 4 – Hydraulic Boundary Conditions

## Technical Review (Reviewer to confirm with Pass/Fail)

| Boundary Review |  |
| --- | --- |
|  | External boundaries connected to 2D mesh |
|  | Boundary conditions everywhere flow could build up at the boundary of the 2D mesh |
|  | Boundary conditions have a hydrograph or elevation/slope data assigned |
|  | Input hydrographs consistent with hydrologic data (DSS, manual entry) |
|  | Precipitation data is consistent with hydrologic data (DSS, manual entry) |
|  | Downstream boundary consistent with ground slope or known downstream WSEL |
|  | Flow hydrographs applied at appropriate locations |
|  | Energy grade slope for hydrograph distribution is reasonable |
|  | Internal rating curves appropriate and documented for control structures (if applicable) |

## Review Comments

# 5 – Hydraulic Model Execution

## Methodologies / Data Sources (Modeler to populate)

| Unsteady Plan Names |  |  |  |
| --- | --- | --- | --- |
| 10% ACE (10-year): |  | 0.2% ACE (500-year): |  |
| 4% ACE (25-year): |  | 1% plus: |  |
| 2% ACE (50-year): |  | 1% minus: |  |
| 1% ACE (100-year): |  |  |  |
| Other: |  |  |  |

## Statistical Data (Modeler to populate)

| Time Step: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Courant Value: |  |  | Simulation Length: |  | hours |
| Computation Interval: |  | seconds | Model Runtime: |  | hours |
| Hydrograph Output Interval: |  | minutes | Mapping Output Interval: |  | minutes |

## Technical Review (Reviewer to confirm with Pass/Fail)

| Execution Review |  |
| --- | --- |
|  | Appropriate 2D parameters are used (Boundary Conditions, Time Step, Simulation Length, etc.) |
|  | Hydrograph Output Interval and Mapping Output Interval set at a reasonable interval to capture peaks |
|  | Model runtime is reasonable (< 12 hours, but dependent upon model size) |

## Review Comments

# 6 – Hydraulic Model Results

## Methodologies / Data Sources (Modeler to populate)

| Flow Validation/Gage Analysis |  |
| --- | --- |
| Software: |  |
| Methodology |  |
| Removal of Outliers and Justification |  |
| Validation Notes (describe parameter adjustments): |  |
| Other: |  |

## Technical Review (Reviewer to confirm with Pass/Fail)

| Computation Checks |  |
| --- | --- |
|  | Model runs with no cell errors |
|  | Volume balance is within tolerance (ΔV < 1% is ideal, however 2% to 3% can be acceptable for rain on mesh) |
| Results Spot Checks Check at all critical structures, major confluences, a handful of headwater tributaries, downstream end, at benchmarks, and a handful of randomly selected locations. |  |
|  | No visible spikes in WSEL visibly present with a single cell |
|  | WSEL and depth grids appear reasonable (no artificial dips or spikes) |
|  | Visual verification flooding is connected downstream for all significant contributing areas (> 64 acres) |
|  | Velocity grids appear reasonable (no unusually high velocity areas or Courant values greater than 2) |
|  | No crossing profiles between all events |
|  | Inflow and outflow hydrograph timing is reasonable as compared to the peak |
|  | Spills into/from adjacent HUCs appropriately accounted for |
|  | WSEL tie-in between model domains and HUCs along major rivers/streams where inflows are applied |
| Results Full Checks |  |
|  | Computational time step meets ideal Courant condition |
|  | Diffusion Wave: V∆T∆X ≤2 (Up to 5) (Results are unit-less) |
|  | Model simulation is long enough to pass the entire hydrograph through the model (minimum of 12 hours past the peak, preferably 24 hours) |
|  | Hydrographs at internal and external boundaries are smooth |
|  | Contributing areas are contained with 2D mesh or accounted for with inflow hydrographs |
|  | No flow trapped at 2D boundary |
|  | Visual verification mapping is reasonable compared to effective |
| Flow Verification |  |
|  | Peak flow gage analysis performed on all gages in the watershed with sufficient data |
|  | All gages used have an appropriate period of record (10 to 20+ years) |
|  | All gages used have no extensive gaps in record which would impact the analysis |
|  | Gage analysis results validated to other sources (Regression, USGS, etc.) |
|  | Results at applicable benchmarks justified (gages, HWMs, effective BFEs) |
|  | Peak discharges comparable to gages, effective/prior studies, regional regression equations |

## Review Comments

# 7 – Mapping Results

## Technical Review (Reviewer to confirm with Pass/Fail)

| Hydraulic Model |  |
| --- | --- |
|  | HEC-RAS model with results, pertinent data and required storm events |
| Flood Risk Database |  |
| Y | All features fully populated per current FEMA guidelines |
| Y | Mapping consistent between grids to polygons for all three required mapped events |
| Y | 10yr smaller than 100yr smaller than 500yr |
| Y | Holes filled in polygon, depth and WSE grids |
| Y | Boundary interfaces align matching or identified as AOMI |

## Review Comments

- S_HUC_Ar does not match USGS HUC8 boundary; see Base_QAQC_Areas polygon. Make sure to use most updated WBD and re-clip layers (note: as there is a spot where current HUC boundary extends beyond the correct one, clip original layers to avoid leaving a data gap)

Boundary updated

- Layers not clipped to correct HUC boundary (see Base_QAQC_Areas and EBFE_QAQC_Areas):

  - S_Pol_Ar

  - BFE_2D

  - TENPCT_FP

  - FLD_HAZ_AR

Clipped to correct HUC boundary

- Review WTR_AR for waterbodies that have disconnected sections that should be included despite being < 20 acres (see EBFE_QAQC_Areas object IDs 4 and 5)

All disconnected waterbodies now included

- Waterbodies >= 20 acres included, but some smaller waterbodies at NID dams with a 200-acre foot or greater capacity were missed and should be included. See EBFE_QAQC_Areas 6-8.

Noted NID dams included

- All layers do not start at Object ID 1, re-export to fix.

Changes have not been implemented to expedite submittal. Please confirm whether this comment is required for BLE complete status.

- DTL_STUD_AR and DTL_STUD_LN are incorrect. Detailed study area features should not be as extensive as shown. Detailed study area polygons should cover only detailed study zones shown on FIRM + BLE floodplain extents that fall within FIRM detailed study AND extend beyond. Detailed study polygons should not cover blank areas of the map or follow FIRM panels. As this watershed has extensive shallow flooding, many polygons that should be included in DTL_STUD_AR will be connected to other polygons that should not be included. In these cases, DTL_STUD_AR polygons will not exactly match FLD_HAZ_AR polygons.

DTL_STUD_AR and DTL_STUD_LN updated based on comment

  - In image 1 below, the light blue is FIRM 1% and orange is FIRM 2%. The dark blue is the submitted FLD_HAZ_AR. The area circled in green (roughly) would be included as detailed study, as it includes and expands the detailed study on the FIRM.

  - In image 2 below (same symbology), the entirety of the selected feature (selection outlined in light blue) to the left of the green line would be included in DTL_STUD_AR. Please reach out with any questions.

Image 1

Image 2

- WTR_LN extends outside FLD_HAZ_AR. Adjust WTR_LN features so they are contained within FLD_HAZ_AR (see WTR_LN_OutofBounds).

Corrected

- BLE_WSE01PCT and BLE_DEP01PCT extents are different. (See RasterMismatch100yr; very small areas, 1-2 pixels, are fine and don’t need to be fixed).

Errors were primarily located outside of the HUC-8. Correcting the boundary clip fixed this issue.

- BLE_WSE0_2PCT and BLE_DEP0_2PCT extents are different. (See RasterMismatch500yr; very small areas, 1-2 pixels, are fine and don’t need to be fixed).

Errors were primarily located outside of the HUC-8. Correcting the boundary clip fixed this issue.

- 10% floodplain is smoothed in some areas but not others. Please clarify why this is. See image below (dark blue = FLD_HAZ_AR, light blue = TENPCT_FP).

10% floodplain only appears smoothed where the WTR_LN 5’ buffer has been applied. However, the model results have not been smoothed. This approach has been approved in June 2023 and is documented in the report.

- WTR_AR features extend beyond FLD_HAZ_AR in some places. Merge WTR_AR and FLD_HAZ_AR. (Dark blue = FLD_HAZ_AR, red/selected areas = WTR_AR extending beyond FLD_HAZ_AR.) (It seems waterbodies were merged into 10pct polygons rather than 100yr.)

WTR_AR features are merged into the FLD_HAZ_AR feature prior to cleaning. The approved smoothing and simplifying tolerances cause slight discrepancies in extent within these established tolerances. Changes have not been implemented to expedite submittal. Please confirm whether this comment is required for BLE complete status.

- WSE grids have a strange, scalloped pattern that is not present in the depth grids and has impacted the BFE_2D contours. Please provide an explanation for this phenomenon.

HEC-RAS calculates a single WSE for each cell (200x200). Each calculated WSE spans about 400 terrain cells (10x10). RAS Mapper interpolates between cell faces and can sometimes cause these scalloped patterns for shallow depths. The location of the screenshot above is unclear, but other “scalloped” WSEs we found all represent very shallow depths (0.1-0.2’) that HEC-RAS cannot accurately model using large cells.

The reason this phenomenon doesn’t show up in the depth rasters is because the depth rasters are created by subtracting the terrain from the WSE rasters. This effectively adds noise that masks this interpolation pattern.

Using the horizontal render mode in RAS Mapper will show how these shallow flow areas are being modeled hydraulically - as a series of shallow strips on the downhill side with large gaps in between. This is an inherent limitation of 2D BLE models with cells that are very large relative to the shallow depths being modeled.

# 8 – Completeness Review

## Technical Review (Reviewer to confirm with Pass/Fail)

| File Structure and Naming |  |
| --- | --- |
| Y | Deliverables provided in TWDB/FEMA required file folder structure and naming convention |
| Y | Data missing (if any) documented in report and ReadMe file within the required folder |
| Report |  |
|  | Technical 2D BLE report documents all assumptions, special considerations, etc. |
|  | Flow validation and difference discussed in report |
|  | Flow comparisons discussed in report |
| Hydrologic Model |  |
|  | HEC-HMS model with results, pertinent data and required storm events |
| Hydraulic Model |  |
|  | HEC-RAS model with results, pertinent data and required storm events |
| Flood Risk Database |  |
| Y | All features fully populated per current FEMA guidelines |
| CNMS |  |
|  | Updated correctly with new centerlines |
|  | A1-A5 checks complete |
|  | CNMS Post- Processing Complete |
| HAZUS |  |
| Y | Run per FEMA guidance |

## Review Comments

QA/QC Checklist Approval

This 2D Base Level Engineering Analysis QA/QC Review is in compliance with the contract requirements and all task checkpoints are complete.  The independent QC Team has reviewed the analysis, presented review comments to the Production Team Task Leader, and discussed any problems or issues.  The Task Leader, QC Manager, QC Reviewer, and Project Manager have signed the QA/QC Checklist to confirm that all comments have been received, addressed, and documented appropriately.

| QA/QC Team – [TWDB or company name] |  |  |  | Production Team – [add company name] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| QC Manager |  | Date |  | Task Lead |  | Date |
| QC Reviewer |  | Date |  | Project Manager |  | Date |