# Texas 2D BLE Analysis QA/QC Checklist

# Project Information

| HUC-8 Watershed Name & ID: | 11120201 Upper Salt Fork Red |
| --- | --- |
| HUC-10 Watershed Name & ID: | 1112020101 Greenbelt Reservoir – Salt Fork Red River 1112020102 Whitefish Creek – Salt Fork Red River |
| Modeler: | Gavin Wood/ Gabrielle Carranza / Travis Ortega |
| Reviewer: | Lauren Fox |

## Terrain and Modeling Review (QC Items 1-6)

| Modeler Initials: | GW | Reviewer Initials: | LF |
| --- | --- | --- | --- |
| Submittal Date: | 12/09/22 | Review Submitted Date: | 01/25/2023 |
| Review Response Date: | 05/02/2023 | Backcheck Complete Date: | 06/13/2023 |

## Mapping Review (QC Item 7)

| Modeler Initials: |  | Reviewer Initials: | GP, |
| --- | --- | --- | --- |
| Submittal Date: |  | Review Submitted Date: | 4/18/24 |
| Review Response Date: |  | Backcheck Complete Date: | 3/17/25 |

## Completeness Review (QC Item 8)

| Modeler Initials: |  | Reviewer Initials: | GP |
| --- | --- | --- | --- |
| Submittal Date: |  | Review Submitted Date: | 4/18/24 |
| Review Response Date: |  | Backcheck Complete Date: | 3/17/25 |

(Modeler to populate with an “X” for items included in the QC submittal)

| Level of Study |  |  | Flood Events |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| X | 2D BLE |  | X | 10% ACE (10-year) | X | 0.2% ACE (500-year) |
|  | Enhanced BLE |  | X | 4% ACE (25-year) | X | 1% plus |
|  |  |  | X | 2% ACE (50-year) | X | 1% minus |
|  |  |  | X | 1% ACE (100-year) |  | Observed Rainfall |

Instructions

Each section includes instructions to the modeler and reviewer regarding their responsibilities in the completion of this checklist. In the Methodologies / Data Sources tables, grey italic text is added as an example response. Additional sample responses are located in the Texas 2D BLE Guidelines Quick Guide.

Reviewer to indicate if Technical Review items generally comply using a “Pass”. If the technical review observes an issue of noncompliance or an item that requires additional attention, an “Fail” should be utilized.

## Submittal Checklist

Data submitted for each milestone should be carried through each subsequent milestone. All geospatial layers and models must have same defined projection system as terrain. Modeler to populate with an “X” for items included in the QC submittal.

| Base Map Data (QC items 1-6) |  |
| --- | --- |
| X (included in report) | Location Map (PDF): HUC-8 and HUC-10 boundaries and/or model domain boundary if different from HUC-10s, stream gages, dams (if applicable), mainstem streams (for reference), background map |
|  | GIS Map (MXD or Map Package) with the following data included but not limited to: Stream centerlines, model breaklines, rainfall data, WSEL and Depth grids (10, 100, 500-yr), adjacent HUC mapping (available on the BLE viewer or currently studying), 2D connections for inflows/outflows, gages or points of flow comparisons (BFEs, highwater marks, etc.), profile lines, boundary condition lines, refinement regions, model domain boundary, model mesh, DFIRM mapping panels, base map data (roads, aerials, etc.), soils data, land use data, terrain |
| Technical Report / Documentation (QC items 1-6) |  |
| X | Technical Report / Project Notebook: assumptions, special considerations, and modeling concerns |
| X | Detailed explanation and support data of variances in methodologies and approaches from recommended guidelines (Should be included in Technical Report / Project Notebook) |
| X | Internal QA/QC Documentation: comments and responses |
| X | Flow comparison tables within the technical report |
| Hydrology (QC items 1-6) |  |
|  | Hydrologic Model (HEC-HMS) model with all associated DSS files for 7 required storm events |
| X | Hydrologic Data and Parameter Spreadsheets (rainfall calculations, curve number table, gage locations, gage analysis and flow validation) |
|  | Hydrologic modeling notebook including assumptions, notes, and special conditions (if not provided in report) |
| Hydraulics (QC items 1-6) |  |
| X | Hydraulic Model (HEC-RAS) with all associated files and 7 required storm events (remove non-final files) |
| X | Raw mapping grids for 10, 1 and 0.2% storm events |
| X | Hydraulic Data and Parameter Spreadsheets (Manning’s n-value table, infiltration calculations support data-Soils, Land Use, Curve Number table, other calculations/data used in model) |
| X | Manning’s n-value table |
| X | Precipitation data correctly linked to hydrologic model or hard coded from provided spreadsheet |
| X | Hydraulic modeling notebook including assumptions, notes, and special conditions |
| Mapping (QC item 7) |  |
|  | Flood risk database |
|  | Modeler to complete topology validation as outlines in the guidelines |
|  | Modeler to verify mapping meets deliverable requirements as outlined in guidelines |
| Completeness Review (QC item 8) See Texas 2D BLE Guidelines Chapter 4 – Deliverables |  |
|  | Technical BLE Report |
|  | Final cleaned hydrologic model |
|  | Final cleaned hydraulic model |
|  | Flood risk database |
|  | CMNS updates |
|  | HAZUS results |
|  | Additional supplemental data |

# 1 – Model Terrain

Methodologies / Data Sources (Modeler to populate)

| Terrain Source(s): | United States Geological Survey (Texas Panhandle 2018 D18 Lidar) |
| --- | --- |
| Data Gaps: | N/A |
| Source Resolution: | 1 meter |
| Model Terrain Resolution: | 3.28084 ft |
| Terrain Horizontal Projection: | NAD83 Texas North FIPS 4201 (US Feet) NAVD 1988 |
| Terrain Vertical Projection: | NAVD88 height |
| Z Values: |  |
| Terrain Buffer: | 1 mile |
| Other: |  |

## Technical Review (Reviewer to confirm with Pass/Fail)

| General Review |  |
| --- | --- |
| P | LiDAR source matches the latest available data at time of project initiation |
| P | Large reservoir or lake extents included in model domain |
| P | Check visible seams, gaps or holes of LiDAR sources (if more than one source was used) |

## Review Comments

- The model description of the HEC-RAS model states a topographic source data that is 3.28 feet.  Please revise this description.  It implies that the source data was a continuous 3.28 foot terrain.  Typically source data descriptions would detail the original data sources (USGS [TERR PROJ NAME] 2 meter data, YYYY).

  - Model description revised to reflect source data topographic resolution. Source data was mix of 1 and 2-meter resolution, so overall terrain was resampled to the finest resolution of inputs (1 meter). Please review report and LiDAR Source: United States Geological Survey (USGS). Panhandle LiDAR, 2018-05-20. Web. 2023-04-13. (TNRIS DataHub)

Backcheck: 	OK

- It is stated that a base lidar dataset of 2 meters was resampled down to a 1 meter dataset.  The minimum raster resolution of the final lidar dataset should be 2 meters since this is the source data.  If a larger cell size is desired, the base terrain data may be resampled up to a coarser resolution of 10 feet.

  - 1-meter terrain was used for model. Please review report and LiDAR Source: United States Geological Survey (USGS). Panhandle LiDAR, 2018-05-20. Web. 2023-04-13. (TNRIS DataHub)

Backcheck: 	It is understood that the 1m terrain was used, however, this is not appropriate based upon the statements made regarding the input datasets.  The cell size of the base data with the largest cell size should be maintained in order to avoid potentially inaccurate data extrapolations created by resampling.

  - The data included at the header of this page was filled out incorrectly for this HUC8. The terrain sources for this HUC8 are correctly documented in the report – there is a singular LiDAR source for this HUC8 and it has a base resolution of 1 m. Header has been revised appropriately on this checklist.

Backcheck: OK

# 2 – Hydrology

## Methodologies / Data Sources (Modeler to populate)

| Software: | HEC-RAS 6.3 |
| --- | --- |
| Rainfall Source: | NOAA Atlas 14 |
| Rainfall Duration: | 24 hours |
| Depth Selection: | 10-year, 25-year, 50-year, 100-year, 500-year, 100-Plus, 100-minus |
| Rainfall Temporal Distribution: | NOAA Atlas 14 Volume 11, Region 2, 1st quartile, 50% event hyetographs |
| Areal Reduction: | No |
| Loss Methodology: | SCS Curve Number |
| Unit Hydrograph Transform Method: | Atlas 14 Regional Curves |
| Inflow Hydrograph Source: | N/A |
| Other: |  |

## Statistical Data (Modeler to populate)

| Stream Gage Name & ID: | Greenbelt Lk nr Clarendon, TX – 07299840 (not used for calibration) |
| --- | --- |
| Stream Gage Name & ID: | Lelia Lk Ck bl Bell Ck nr Hedley, TX - 07299890 |
| Stream Gage Name & ID: |  |

## Technical Review (Reviewer to confirm with  Pass/Fail)

| Excess Rainfall |  |
| --- | --- |
| F | Precipitation values are an aerial average of Atlas 14 values for the entire 2D area boundary for each storm event or other appropriate rainfall application |
| P | CN values appropriately reflect land use and soils for entire 2D area boundary |
| P | Losses are correctly calculated to establish the excess rainfall hyetographs by checking land use, soils and infiltration layers or other relative information |
| P | Total precipitation depths for 1%+ and 1%- are calculated correctly |
| - | Excess rainfall hyetographs are calculated for all storm events (10%, 4%, 2%, 1%, 0.2%, 1%+, and 1%-) |
| P | Cumulative excess rainfall values are in increasing order from 10%, 4%, 2%, 1%, to 0.2% |
| P | Cumulative excess rainfall values for the 1%+ and 1%- fall in order with the storm events matching the total precipitation values |
| Inflow Hydrographs |  |
| - | Shape and time-to-peak of any inflow hydrographs resemble past documented large storm events |
| F | Hydrographs reasonable when compared to the other return periods. The shapes should be similar, and hydrographs from different events should not intersect unless there is an engineering reason. Peak flows from a lower return period event should not be higher than a higher return period event |
| - | Contributing hydrographs from another RAS model applied at coincident locations (Provide locations in base map data) |
| General Hydrology |  |
| Y | Any flow regulating dams in the model area? (Yes/No) |
| Y | Dam modeling methodologies applied and explained? (Yes/No). If yes, provide discharge rating curve calculations and gage analysis documenting regulated versus unregulated records, if applicable. |
| Y | Model aligns with documentation of dam modeling approaches. |
| Other Notes: NA |  |

## Review Comments

- Include page numbers in the report documents.

  - Page numbers added.

Backcheck: 	OK

- Throughout all sections of the report, there are many indications of placeholders of a template report not being updated.  Please review the report and ensure missing information or information from old reports are removed or adjusted for relevancy to this project.

  - Report sections updated/place holders removed.

Backcheck: 	OK. Only remaining indications of placeholders at discussion of bounding models; acceptable prior to model finalizations.

- Based upon the workshop held on January 18th and the TWDB BLE Guidelines, to justify differing methodologies it is required to submit a comparison of traditional BLE rainfall methodology and any alternative rainfall approach.

  - These draft models were submitted for initial review on 12/13/2023, one month prior to the referenced workshop where this requirement was communicated. To date, TWDB has conditionally approved the rainfall method hinging on model calibration to historic data in the watershed. A memorandum of this decision is being drafted by TWDB and will be included as an appendix to the report for the M3 review.

Backcheck: 	Per a memorandum provided by TWDB on June 1, 2023, TWDB has conditionally accepted the rainfall approached utilized for this HUC with conditions included in the memorandum and provided below.

# Conditions

- Without a full comparative gage analysis between precipitation methods as requested by Halff Associates in the M1 review comments, TWDB considers Stantec to be taking an “at-risk” modeling approach by maintaining this methodology.  Stantec will still be held to the standard of satisfactory model validation with gages in the study HUCs. (See Addendum)

- Significant delays to schedule, budget overages, or requests for change order to satisfy the requirement described in Condition #1 will not be accepted by TWDB.

- Additional calibration tools (such as evapotranspiration) must be justified with requested documentation and should be applied as consistently as possible across the task order watersheds.

- Wherever possible, the model should strive to represent actual physical processes (such as diversions) in preference of proxy calibration tools.

- All comments generated by the external review process regarding calibration/validation must be adequately addressed.

- Additional analysis or data (within reason) may be required to satisfactorily meet condition #5.

- All TO2 Models should utilize the same precipitation approach for consistency unless a compelling argument for variation in approach is presented.

- The report provided states that a Bulletin 17C gage analysis was completed for this area.  Please include this model in the submittal.

  - Analysis added to report appendix.

Backcheck: 	Please include the model, inputs, and outputs in the submittal as previously requested.

  - Bulletin 17C files have been Included in “Hydrology Submittal > Supplemental Data” data per the MIP/ TWDB BLE folder delivery structure. We have also updated the appendix to include the B17C output for the gage within the HUC8.

Backcheck: 	OK – 17C Files included with Backcheck Supplemental.

- The report states that “This gage, 07299890 was determined to have inconsistent data to draw meaningful comparisons for calibration.” Please elaborate further.

  - Report language has been edited – please see “challenges” section and calibration section.

Backcheck: 	Please include the 17c inputs, model, and outputs for review as indicated in comment 4.

Response 2.     Bulletin 17C files have been Included in “Hydrology Submittal > Supplemental Data” data per the MIP/ TWDB BLE folder delivery structure.

Backcheck: 	Bulletin 17C Files for Gage 07299890 were not included. Please provide them with the final submittal.

3/14/2025 - Backcheck comment: Files are not provided in most recent submittal. Please confirm they are provided in final deliverables.

5/6/2025 – PeakFQ Bulletin 17C analysis provided in the Engineering Models/Supplemental_Data folder.

- For tables 4-8, 4-9, and 4-10, please include the PeakFQ values for gage 07299840 as comparison points.

  - Peak FQ analysis was not performed for gage 07299840 as discussed in the report. This gage only contains WSEL for the Greenbelt Reservoir as discussed in report.

Backcheck: 	OK. Were comparisons made to WSELs based on available historic data?

  - It is beyond scope of BLE to provide a comparison of stage within this reservoir for a specific flood frequency. USGS records of WSE levels in the lake were reviewed for comparison to the LiDAR elevation and the discussion has been added to the report, including the following: “Gage 07299840 contains only water surface elevation data for the Greenbelt Reservoir. This data was not used for calibration as it is not flow data. Additionally, historical reservoir storage according to the gage data has a downwards trend since the pond’s inception in the 1970’s, making direct comparisons between modeled and historical results difficult.”

  - Stantec has been in contact with the owner of the Greenbelt Reservoir (Greenbelt Municipal and Industrial Water Authority) multiple times and have been unable to obtain any information more detailed than what is publicly available. The available data from the USGS gage (Gage 07299840) at the dam is WSEL only, period of record from (1996-present). Historical reservoir storage data indicates a downwards trend in WSEL since the pond’s inception in the 1970’s. The qualitative information provided by the owner indicated that the watershed has a long history of drought conditions (ongoing drought since early 90’s) and that the reservoir levels have been impacted by heavy agricultural irrigation since reservoir inception, making the WSEL records difficult to apply for model validation. Model flows were validated at USGS gage 07299890 (tributary) within the Upper Salt Fork Red Watershed, and gage 07300000 in the Lower Salt Fork Red Watershed, downstream of the reservoir. Further discussion is included in section 4.6 of the updated report.

Backcheck: Comment resolved

- The first sentence of Appendix A states that 11 gages are present in the project area, while it was previously established that only 2 gages are present in the project area.  Please clarify this section to show that details within Appendix A cover several project areas, not only the project area for this report.

  - Report section clarified.

Backcheck: 	OK

- The report states that for gage 07300000 only data from water years 1953 to 1997 were used, despite records being available through 2022 due to skew values being excessively positive.  Please provide further reasoning and comparison between use of the full dataset and truncated dataset.  USGS data show the area to be regulated from 1967 to the present, therefore it seems more reasonable to truncate to that range.  This area does not appear to be relevant to HUC 8 11120201 (project area) so it is unclear why this discussion is present in the report.

  - The Bulletin 17C analysis for USGS 07300000 was truncated at 1997 because a drought has stricken the region from 1998 through present (see figure below). The fundamental theory behind Bulletin 17C methods is that a univariate probability distribution can describe the data; however, at this site we have a classic mixture model problem (i.e., there are numerous mechanisms governing floods). We chose to exclude observations from 1998 through present as they are potentially influential low outliers that will impact the estimate of large/rare floods in the region. Though the region is currently in a drought, we wish to represent the flood risk as it was in the previous epoch, which is represented using the data from 1953 to 1997.

  - In addition, this gage is downstream of the mainstem outflow of HUC 11120201 (ie peak flows at the gage are directly affected by outflow of the HUC). Additional clarifying text has been added to the report.

Backcheck: 	Please truncate the analysis of gage 07300000 to 1967 as this is when regulation begins in the USGS gage record.  With the reasoning provided, ending at 1997 is acceptable. Page 35 of the report states “…HUC 8 11120201 (downstream of the Greenbelt Reservoir) contributes approximately 40% of the drainage area for the gage.”  For clarity, please also indicate within this section of the report an approximate stream distance from the ending of model 102 to gage 07300000.

  - We have completed a Pettitt test (a non-parametric test for change point detection) on the full time series of annual maximum peak streamflow. The results of the Pettitt test (Figure 1) indicate that a statistically significant (p-value = 5.281e-06) change point exists. The test output indicates that the probable change point occurs at time K = 45, which corresponds to the 1997 peak. We performed a follow-up Pettitt test on the subset time series (i.e., 1953-1997), which did not result in a statistically significant change point, with tolerance of alpha=0.05. This further justifies using the 1953-1997 period.

Figure 1. Screenshot depicting the Pettitt test for USGS 07300000 annual maximum time series.

We have completed the analysis for the requested subset time series (1953-1967). However, note that an incredible amount of uncertainty will be introduced when using only 15 years of observations. This uncertainty directly affects the range of the 95% confidence interval (which is nearly an order of magnitude, see results below. This uncertainty also directly affects the “best estimate” because of the rarity of the flood quantile (i.e., 100-year return period, 1% chance annual exceedance). Also note that the resulting estimate will be based upon data that is over 55 years old. These estimates are not representative of the current hydrology, climate, nor channel geometry. We recommend using the PeakFQ results included in our report which correspond to 1953-1997.

We present the 1953-1967 results alongside those from the 1953-1997 subset for comparison.

|  | Estimate (cfs) | Lower 95% CI (cfs) | Upper 95% CI (cfs) |
| --- | --- | --- | --- |
| 1953 - 1967 | 180,400 | 97,430 | 896,500 |
| 1953 – 1997 | 151,400 | 86,010 | 362,100 |

We have added an approximate distance from model outlet to gage 0730000 to the report (24 miles).

  - The backcheck did not clearly state the subset to be used, our sincerest apologies for misinterpreting the request. We have completed the analysis on the 1967-1997 subset, summarized below. The values have been updated in the report.

Backcheck: Comment resolved.

| AEP 0.01 | Estimate (cfs) | Lower 95% CI (cfs) | Upper 95% CI (cfs) |
| --- | --- | --- | --- |
| 1967 – 1997 | 99,210 | 53,220 | 275,100 |

- Include the NOAA Atlas 14 precipitation data for the events modeled in the report.

  - Clarification on this comment was requested via email on 5/3. Please clarify what you want to see in the report and we can update accordingly for M3 submittal.

Backcheck: 	Indicate the point precipitation values at the approximate centroid of the project area. It is understood that gridded precipitation is being used in the model, however, it would be ideal to include an approximation of precipitation values used in the modeling in the report.

  - We’ve added a table of average precipitation across each HUC10 basin for each frequency storm in Section 3.1.

Backcheck: 	Comment resolved.

- Curve numbers used in the models are much higher than the suggested values from the TWDB guidance document “Texas Two-Dimensional Base Level Engineering Guidelines, version 1.0” dated May 21, 2022. Discuss in the hydrologic section of the report. Additionally, TWDB guidance suggests the use of a separate impervious layer. No impervious layer is present in the models submitted.

  - The curve numbers are a calibration feature of the model; the CN's chosen for this model are in "normal" range and the model is calibrated to the gages in the HUC, and the downstream mainstem gages, within reason with the applied CN's. Comparatively at gage 07300000, the downstream mainstem peak flow is low, compared to 1% median 17C, and decreasing the CN's in this model would contribute to pulling the modeled flows below the 95% CI for the statistical analysis at the gage.

Backcheck: 	CN values that differ from the TWDB BLE Guidelines must be clearly documented within the BLE report. Additionally, the use of composite CNs is not recommended and separate impervious layer should be used. Provide clear documentation in report of the CN adjustments made during model validation with justification for those changes.

  - We have updated the report to include additional justification for the use of the current infiltration methodology (which results in reasonable agreement in the modeled results and the bulletin 17C analyses at the gages). An agreement to allow the use of this methodology was made with TWDB and communicated to Halff via email on 7/14/2023.  Please see the report edits in section 3.2.

Backcheck: 	Comment Resolved

- It is understood that the primary weir for the reservoir was removed for the final modeling due to extended run times. However, please provide this model and any supporting calculations as a supplemental file for review in the next submittal.

  - Model with primary weir submitted.

Backcheck: 	Model 101 with the morning glory weir was not found in the submittal folder.

  - We included this model in the revised submittal at the following path: UpperSaltForkRed_11120201\Hydraulics_Submittal\UpperSaltForkRedRiver_11120201\Supplemental_Data\GreenbeltReservoir_1112020101_Outflow

Backcheck: 	Comment Resolved.

- The report states that the flow calculated through the primary weir during the 0.2 percent event was transferred downstream for the 1 percent, 1 percent plus, and 0.2 percent events.  The reported water surface elevation of the 0.2 percent event is approximately 2 feet above the primary weir elevation.  The 1 percent event is approximately 10 feet below the weir elevation and the 1 percent plus is approximately 3 feet below the elevation, therefore it is unclear why this assumption was made.

  - Models have been updated and flow transfer to basin 102 is only occurring during the 0.2 percent storm event. The 0.2% inflow hydrograph coming from the dam weir has been removed for the other storm events, as the upstream WSE does not reach the crest elevation of the dam weir in the smaller storm events. See updated 102 runs.

Backcheck: 	OK

- Flow from the morning glory weir is not being fully transferred to the downstream basin based on the inflow hydrograph.

  - Peak flow leaving the dam via morning glory outfall to the downstream HUC is captured. Generally, flow leaving the morning glory are negligible and only for the 500-year. Simulation Time Window is sufficient to transfer hydrograph of the entire downstream HUC10 model.

Backcheck: 	OK

# 3 – Hydraulic Geometry

## Methodologies / Data Sources (Modeler to populate)

| Software: | HEC-RAS 6.3 |
| --- | --- |
| Loss Methodology: | SCS Curve Number |
| Soils Source: | USGS Soils Survey Geographic Database (SSURGO) (SSURGO 2020; MRCL 2019) |
| Land Cover Source: | National Land Cover Database |
| Manning's Roughness Source: | 2019 National Land Cover Database (NLCD) Land Use Classification raster |
| Nominal Mesh Cell Sizes: | 200x200ft |
| Other: Refinement Region cell size 50x50 |  |

## Technical Review (Reviewer to confirm with Pass/Fail)

| 2D Mesh |  |
| --- | --- |
| P | 2D Boundary delineated along watershed boundary or buffer to ensure all contributing area is included |
| P | Nominal cell size appropriate for model detail (generally 200 feet, minimum 50 feet for refinement areas) |
| P | Computation points are at default locations or changes documented |
| P | Mesh Cell and Face tolerances are default or changes documented |
| P | No mesh errors |
| P | Cell face relationships are calculated |
| Breaklines |  |
| F | Breaklines are consistent with the terrain data |
| F | Breaklines sufficiently detailed to capture features (thalweg, banks, berms, etc.) |
| F | Cells do not overlap terrain features where appropriate (i.e., unwanted leaking) |
| P | Breakline cell sizing and parameters are appropriate |
| P | 2D Area Connections placed at applicable benchmarks, gages, or dams |
| P | 2D Area Connections set to use normal 2D equation domain for benchmarks and gages. Appropriate modeling approach used for dams. |
| Manning’s N-Values |  |
| P | Manning’s n-values associated to land cover data provided |
| P | Land Cover classifications are sufficiently descriptive within the data table |
| P | Manning’s n-value classifications generally match current aerial imagery |
| P | Manning’s n dataset is associated with the geometry files |
| General |  |
| P | All model datasets each have unique descriptive names |

## Review Comments

- Supplemental comments regarding hydraulic geometry and results included in shapefiles provided.

  - Shape files reviewed, and changes responded to and implemented into the updated model meshes. Responses to each comment added to the shapefiles. See revised shapefiles.

Backcheck:	OK

- According to Table 4-2 of the provided report, Developed, Medium Intensity is using a lower manning’s value than Developed, Low Intensity.  Revise or explain.

  - Mannings Values adjusted. Medium Intensity development was changed from 0.08 to 0.12.

Backcheck:	OK

- Include the state plane projection used in the HEC-RAS model description.

  - State plane projection included.

Backcheck:	OK

- Consolidate Manning’s values to only use the land use type, rather than land use type and hydric soil group. This will make the model cleaner.

  - Agree. Addressed as suggested.

Backcheck:	OK

- Two terrains are present in the models.  Please remove extraneous terrains.

  - Original terrains removed. Resampled terrains with HC modifications remain as only terrain for each model.

Backcheck:	OK

- Review terrain along “HC” polyline areas. It is stated in other modeling logs that these files represent areas of terrain adjustment, however, there are no apparent terrain adjustments made at many of these locations. Alternately, there are locations with terrain adjustments but no HC indicator line.  Individual comments not made at these locations.  Terrain adjustment comments made in shapefile where no HC line is present.

  - Incorrect HC shapefile provided previously. Correct HC shapefile included now, with resampled & modified terrains that include HC mods. No terrain adjustment comments were found in the provided shapefiles. Please provide, if any.

Backcheck:	HC Lines were not found in this submittal.

  - HC shapefile can be found in revised submittal in the Features folder. UpperSaltForkRed_11120201\Hydraulics_Submittal\UpperSaltForkRedRiver_11120201\Simulations\GreenbeltReservoir_SaltForkRed_1112020101\Features

Backcheck: 	Comment Resolved.

- Many refinement regions have highly angular edges and many do not appear to be enforced into the mesh.  Review and revise.

  - Refinement regions updated and re-enforced into updated model meshes. See updated model meshes.

Backcheck:	OK

- Section 4.5 of the report suggests that overlap exists between the mesh regions of the 2 basins and that staggered SA2D connection locations were used between basin 101 and 102.  This is not the case for the models submitted.  Please alter the text in this section to reflect the model conditions of basins 101 and 102. It is later stated that the methodology used in this section was not used; it is unclear why this section remains in the report.

  - Report updated.

Backcheck:	The report provided is still discussing overlaps and usage of staggered SA2D connections.

  - Sections 4.5 and 4.5.1 have been updated accordingly.

3/14/2025 - Backcheck comment: Please review section 4.5. Section starts with describing a process and then section 4.5.1 explains that that process is not used. Can be misleading. This section should only focus on the practices and processes used for modeling the 2 domains: 1112020101 and 1112020102.

- Instabilities exist at the SA2D connection “Outflow_Leila”.  The weir and cell elevations are significantly different. Try reconciling these differences and/or adjusting mesh size to resolve.

  - Leila connection updated. Flows and velocities appear much more normal in the updated model.

Backcheck:	OK

- Section 4.5.1 of the report states that the overflow weir is at an elevation of 2674 feet, however the terrain at this location is approximately 2676.  Water surface elevations at this location are just above 2674 feet in higher events, indicating overflows would occur if the terrain represented the stated elevations. Please revise the terrain or modeling in this location to represent the stated dam elevations.

  - The crest of the dam spillway is given at 2,674 feet elevation, according to the USGS data and description about the dam. It is also stated by the USGS that “The spillway is an uncontrolled open cut through natural ground, 1,450 ft wide and located at the left end of dam…” indicating natural ground which is likely to vary slightly from the provided 2,674-foot elevation of the spillway crest. Overflows are not occurring here, per the modeling results. BC line ‘BC_01’ is set up in the model to allow any flows to leave the model if overflow occurred, but as the results show, no flow makes it to this boundary line in any of the storm simulations. Any ponding along the spillway that is at or greater than 2674 is localized ponding due to the terrain and rain on grid occurring here, and not caused by actual spilling of the reservoir, according to the modeling.

Backcheck: 	Based on the response it appears the modeler agrees that the BC line does not represent the spillway elevations stated in the report, however, believes that the emergency spillway is not utilized for any of the storms and is inconsequential to results.  Based on documents for the site available on the TWDB site, it is proposed that the 2674.0’ elevation stated is NGVD29.  Vertical transformation on the NOAA site shows an approximate 1.3’ difference between NGVD29 and NAVD88 at this location.  Based on this assumption, the reviewer agrees usage of the lidar data is sufficient for modeling the emergency spillway. However, please research and discuss the discrepancy between the report’s stated elevation of 2674.0’ and the modeled elevation of approximately 2676.0’.

  - As mentioned in the report, the 500-year WSEL of the reservoir is approximately 2,666 feet and does not reach the documented spillway elevation of neither 2,674 or 2,676 (explained in text excerpt from report below). The discrepancy between the two elevations of 2,674 and 2,676 maybe be due to vertical transformation difference as stated above, or due to Highway 70 which cuts through the spillway area and is the highest elevation on the spillway. The following language has been added to the report: “A discrepancy exists between the modeled spillway elevation of 2,676 feet and the recorded dam spillway elevation of 2,674 feet. This may be due to two factors. State Highway 70 runs through this dam spillway and since the dam’s impoundment date of January 01, 1968, roadway improvements to this State Highway 70 may have occurred. Historical TxDOT records of SH 70 improvements were searched to no conclusion. In addition, the original USGS Geological Survey records the dam as of October 8, 1967 at 2,674 using the vertical transformation at the time, NGVD29. This modeling scenario uses the current vertical transformation NAVD 88. The difference between NGVD29 and NAVD88 at this point according to the NOAA Online Vertical Datum Transformation web tool, is +0.40 meters, or +1.32 feet. A combination of these factors may explain the spillway elevation difference.”

Backcheck:	Comment Resolved.

# 4 – Hydraulic Boundary Conditions

## Technical Review (Reviewer to confirm with Pass/Fail)

| Boundary Review |  |
| --- | --- |
| P | External boundaries connected to 2D mesh |
| F | Boundary conditions everywhere flow could build up at the boundary of the 2D mesh |
| P | Boundary conditions have a hydrograph or elevation/slope data assigned |
| P | Input hydrographs consistent with hydrologic data (DSS, manual entry) |
| - | Precipitation data is consistent with hydrologic data (DSS, manual entry) |
| P | Downstream boundary consistent with ground slope or known downstream WSEL |
| P | Flow hydrographs applied at appropriate locations |
| P | Energy grade slope for hydrograph distribution is reasonable |
| - | Internal rating curves appropriate and documented for control structures (if applicable) |

## Review Comments

- Review the perimeters of the 2D mesh.  There is flow stacking up along the mesh in a significant number of areas.  Boundary conditions should be applied in these locations.

  - Boundary condition lines added for areas of ponding against model perimeters. See revised models.

Backcheck:	OK

# 5 – Hydraulic Model Execution

## Methodologies / Data Sources (Modeler to populate)

| Unsteady Plan Names |  |  |  |
| --- | --- | --- | --- |
| 10% ACE (10-year): | 10pct | 0.2% ACE (500-year): | 0.2pct |
| 4% ACE (25-year): | 4pct | 1% plus: | 1pctPLUS |
| 2% ACE (50-year): | 2pct | 1% minus: | 1pctMINUS |
| 1% ACE (100-year): | 1pct |  |  |
| Other: |  |  |  |

## Statistical Data (Modeler to populate)

| Time Step: | Variable Courant-Based Time Step |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Courant Value: | <2.0 |  | Simulation Length: | 96 | hours |
| Computation Interval: | 30 (base) | seconds | Model Runtime: | <12 hours | hours |
| Hydrograph Output Interval: | 15 | minutes | Mapping Output Interval: | 15 | minutes |

## Technical Review (Reviewer to confirm with Pass/Fail)

| Execution Review |  |
| --- | --- |
| P | Appropriate 2D parameters are used (Boundary Conditions, Time Step, Simulation Length, etc.) |
| P | Hydrograph Output Interval and Mapping Output Interval set at a reasonable interval to capture peaks |
| P | Model runtime is reasonable (< 12 hours, but dependent upon model size) |

## Review Comments

- Extend the run time of basin 102 to fully pass the hydrograph during the run duration.

  - Peak flow leaving the dam via morning glory outfall to the downstream HUC is captured. Generally, flow leaving the morning glory are negligible and only for the 500-year. Simulation Time Window is sufficient to transfer hydrograph of the entire downstream HUC10 model.

Backcheck: OK

# 6 – Hydraulic Model Results

## Methodologies / Data Sources (Modeler to populate)

| Flow Validation/Gage Analysis |  |
| --- | --- |
| Software: | HEC-RAS 6.3 |
| Methodology | Rain on Grid |
| Removal of Outliers and Justification |  |
| Validation Notes (describe parameter adjustments): |  |
| Other: |  |

## Technical Review (Reviewer to confirm with Pass/Fail)

| Computation Checks |  |
| --- | --- |
| F | Model runs with no cell errors |
|  | Volume balance is within tolerance (ΔV < 1% is ideal, however 2% to 3% can be acceptable for rain on mesh) |
| Results Spot Checks Check at all critical structures, major confluences, a handful of headwater tributaries, downstream end, at benchmarks, and a handful of randomly selected locations. |  |
| P | No visible spikes in WSEL visibly present with a single cell |
| P | WSEL and depth grids appear reasonable (no artificial dips or spikes) |
| P | Visual verification flooding is connected downstream for all significant contributing areas (> 64 acres) |
| P | Velocity grids appear reasonable (no unusually high velocity areas or Courant values greater than 2) |
| P | No crossing profiles between all events |
| P | Inflow and outflow hydrograph timing is reasonable as compared to the peak |
| P | Spills into/from adjacent HUCs appropriately accounted for |
| P | WSEL tie-in between model domains and HUCs along major rivers/streams where inflows are applied |
| Results Full Checks |  |
| P | Computational time step meets ideal Courant condition |
| P | Diffusion Wave: V∆T∆X ≤2 (Up to 5) (Results are unit-less) |
| F | Model simulation is long enough to pass the entire hydrograph through the model (minimum of 12 hours past the peak, preferably 24 hours) |
| F | Hydrographs at internal and external boundaries are smooth |
| P | Contributing areas are contained with 2D mesh or accounted for with inflow hydrographs |
| F | No flow trapped at 2D boundary |
| P | Visual verification mapping is reasonable compared to effective |
| Flow Verification |  |
| P | Peak flow gage analysis performed on all gages in the watershed with sufficient data |
| P | All gages used have an appropriate period of record (10 to 20+ years) |
| P | All gages used have no extensive gaps in record which would impact the analysis |
| P | Gage analysis results validated to other sources (Regression, USGS, etc.) |
| F | Results at applicable benchmarks justified (gages, HWMs, effective BFEs) |
| P | Peak discharges comparable to gages, effective/prior studies, regional regression equations |

## Review Comments

- Verify there are no crossing profiles in the results using a sampling of results using SA2D connection lines in the HEC-RAS model and discuss the analysis within the hydraulic section of the report.

  - Profiles verified. 100yr minus flows are generally lower than 50yr flows.

Backcheck: 	Profile lines are not seen in the models provided and no discussion was found in the report.

  - Section 4.7.3 was added to the report and includes profile plots for all frequency events. The profile lines from these checks have been saved in the models and can be provided as a shapefile as needed.

Backcheck: 	Profile lines received with Supplemental and verified. Comment Resolved.

- Repetitive cell errors in basin 102. Review and revise.

  - 102 mesh updated, no cell errors.

Backcheck:	OK

# 7 – Mapping Results

## Technical Review (Reviewer to confirm with Pass/Fail)

| Hydraulic Model |  |
| --- | --- |
|  | HEC-RAS model with results, pertinent data and required storm events |
| Flood Risk Database |  |
| FAIL | All features fully populated per current FEMA guidelines |
| FAIL | Mapping consistent between grids to polygons for all three required mapped events |
| FAIL | 10yr smaller than 100yr smaller than 500yr |
| FAIL | Holes filled in polygon, depth and WSE grids |
| FAIL | Boundary interfaces align matching or identified as AOMI |

## Review Comments

Please add the following verbiage to the report if applicable. This is regarding the irregularities that is seen when filling holes:

The Elevation Void Fill routine is used to fill holes in the rasters. For the 1% and 0.2% ACEs, holes less than 2 acres were filled, while for the 10% ACE, holes less than 0.5 acres were filled. Occasionally, this routine produces WSELs that are below ground elevation. In such cases, the WSELs were not revised, but the depths at these locations were set to 0.1 feet. The routine fills the holes via an interpolation process that uses the WSELs around the edge of the hole to estimate the WSELs within the hole. Minor discontinuities may appear around the hole’s perimeter, especially near the maximum fill area of 2 acres. These discontinuities typically occur where cupping is present in the raster near the edge of the hole or when the WSELs near the hole are changing rapidly. This issue is not present in all holes and is usually limited to those approaching the maximum fill area.

Mapping Geometry Comments:

- FLD_HAZ_AR, TENPCT_FP, BLE_WSE01PCT, BLE_WSE0_2PCT, BLE_DEP01PCT, BLE_DEP0_2PCT

  - Islands under 2 acres of disconnected floodplain polygons meeting a matrix criteria developed by the TWDB and uncontained holes under 2 acres need to be removed.

  - Response #1: No Change. Coastal and High Plains (TWDB Guidance 3.2.1) Matrix criteria was used for removing 0.33 acres AND 0.33 ft depth polygons. Data needs to be within both of those parameters to be removed. No additional removal of disconnected floodplains outside of what the matrix tool removes was performed. No additional guidance or clarification was mentioned within the TWDB/FEMA R6 Guidance documents or TWDB Log of Revisions regarding manual removal of 2 acre disconnected floodplain polygons. See screenshot below from TWDB Log of Revisions

  - Backcheck: In discussion with TWDB, Stantec will not resolve small, disconnected pixel mapping for Task Order 2 HUCs. No further comment.

- BLE_DEP01PCT, BLE_DEP0_2PCT

  - BLE_DEP01PCT should not extend outside of BLE_DEP0_2PCT.

  - Response#1: Agree, we revised so 01PCT extents do not extend beyond 0_2PCT.

  - Backcheck: no comments

- BLE_WSE01PCT, BLE_WSE0_2PCT

  - BLE_WSE01PCT should not extend outside of BLE_WSE0_2PCT.

  - Response#1: Agree, we revised so 01PCT extents do not extend beyond 0_2PCT.

  - Backcheck: no comments

- BLE_WSE01PCT, BLE_WSE0_2PCT

  - 2-4 feet jump in cell values

  - 100.8143246°W 35.0103726°N

  - 100.9280920°W 34.9834610°N

  - Response#1: See QC.shp for specific response to each area.

  - Backcheck: no comments

2679.6

2682.3

2479.3

2483.5

- BLE_DEP01PCT, BLE_WSE01PCT

  - BLE_DEP01PCT and BLE_WSE01PCT extents should be the same.

  - Response#1: Agree, we revised so 01PCT extents match.

  - Backcheck: no comments

  - 3/27/2025 Backcheck #2: BLE_DEP01PCT extends outside of BLE_WSE01PCT. Please see QC_Comments_20250327 for the locations where that occurs.

  - Response#2: Agree, we revised so 01PCT extents match.

- BLE_DEP0_2PCT, BLE_WSE0_2PCT

  - BLE_DEP0_2PCT and BLE_WSE0_2PCT extents should be the same.

  - Response#1: Agree, we revised so 01PCT extents do not extend beyond 0_2PCT.

  - Backcheck: no comments

  - 03/27/2025 Backcheck #2: BLE_DEP0_2PCT extends outside of BLE_WSE0_2PCT. Please see QC_Comments_20250327 for the location where that occurs.

  - Response#2: Agree, we revised so 0_2PCT extents match.

  - BLE_DEP0_2PCT and BLE_WSE0_2PCT have missing data

  - Response#1: Agree, we revised to populate missing data.

  - Backcheck: no comments

- TENPCT_FP

  - TENPCT_FP should not extend out of 100-yr floodplain.

  - Response#1: Agree, we revised so TENPCT_FP doesn’t extend outside 100-yr floodplain.

  - Backcheck: no comments

# 8 – Completeness Review

## Technical Review (Reviewer to confirm with Pass/Fail)

| File Structure and Naming |  |
| --- | --- |
|  | Deliverables provided in TWDB/FEMA required file folder structure and naming convention |
|  | Data missing (if any) documented in report and ReadMe file within the required folder |
| Report |  |
|  | Technical 2D BLE report documents all assumptions, special considerations, etc. |
|  | Flow validation and difference discussed in report |
|  | Flow comparisons discussed in report |
| Hydrologic Model |  |
|  | HEC-HMS model with results, pertinent data and required storm events |
| Hydraulic Model |  |
|  | HEC-RAS model with results, pertinent data and required storm events |
| Flood Risk Database |  |
|  | All features fully populated per current FEMA guidelines |
| CNMS |  |
|  | Updated correctly with new centerlines |
|  | A1-A5 checks complete |
|  | CNMS Post- Processing Complete |
| HAZUS |  |
| PASS | Run per FEMA guidance |

## Review Comments

Flood Risk Database:

- S_Pol_Ar

  - 03/27/2025 Backcheck #2: S_Pol_Ar is not continuous throughout entire HUC. See QC_Comments_20250327.shp

  - Response#1: Agree, we revised so S_Pol_Ar is one continuous feature.

- S_HUC_Ar

  - No comments

- WTR_LN

  - Geometry comment on streams, please verify streams meet 1 SQM for rural and 0.5 SQM for urban areas per Region/FEMA requirements.

  - Response#1: No change. Streams verified to meet 1 sqmi for rural and 0.5sqmi for urban area per Region/FEMA requirements. Data is within Supplemental_Folder --> WaterLines.

  - Backcheck: no comments

  - Review QC polygons for stream centerline containment issues, etc.

  - Response#1: We reviewed stream centerline 1% FP containment and made necessary updates. There will not be full containment for all streams due to disconnected floodplains due to roads/embankments.

  - Backcheck: no comments

- WTR_AR

  - Please drop extra attribute.

  - Response#1: Agree we removed “area” field.

  - Backcheck: no comments

  - Three named waterbodies are missing from WTR_AR feature class. Please review the NHD waterbodies dataset for missing features.

  - Response#1: We added named waterbodies regardless of size to the WTR_AR feature.

  - Backcheck: no comments

- DTL_STUD_AR

  - No comments

- DTL_STUD_LN

  - No comments

- BFE_2D

  - BFE_2D lines should not have closed boundaries. Please remove closed features. A review of entire HUC required to remove all BFEs that are circular. Recommended running topology on feature to locate and remove.

  - Response#1: We removed circular BFEs.

  - Backcheck: BFE_2D lines values failed to match 100-yr WSE grid values by more than 1 ft. Failed lines appear as dense clusters. Please see the QC shapefile for examples.

  - Response#2: Fixed, we revised the BFEs to match the WSE 1% Grid values.

  - 03/27/2025 Backcheck #2: BFE_2D features are not snapped to HUC boundary. QC_Comments_20250327.shp

  - Response#3: BFEs have been clipped to the HUC boundary and are fully contained within the HUC8 boundary using the FEMA resolution and tolerance.

  - Coded domains not applied correctly to the BFE_LN_TYP field please review all fields that use coded domains and apply correctly.

  - Response#1: Agree, attributes revised with coded domains.

  - Backcheck: no comments

- TENPCT_FP

  - No comments on Attribute Table

- FLD_HAZ_AR

  - Coded domains not being applied correctly for the ZONE_SUBTY please review and populate using coded domain values for flood zone X. Review that all fields are properly applying coded domains is required.

  - Response#1: Agree, attributes are updated with coded domain values.

  - Backcheck: no comments

  - 03/27/2025 Backcheck #2: FLD_HAZ_AR is not snapped to HUC boundary. See QC_Comments_20250327.shp

  - Response#2: FLD_HAZ_AR has been clipped to the HUC boundary and is fully contained within the HUC8 boundary using the FEMA resolution and tolerance.

- S_Studies_Ln and S_Unmapped_Ln

  - CNMS features are reviewed by Michael Johnson, Region 6 CNMS Lead. Please provide documentation of his review and acceptance in your next submittal.

  - Response #1: CNMS documentation was provided within the submittals Supplemental Data Folder --> CNMS

  - Backcheck: no comments

- S_FRAC_Ar

  - Please review that all features are clipped to the BLE study area HUC there are some census blocks outside of the HUC boundary.

  - Response #1: No change.  FRAC Ar data is to not be clipped to HUC8 boundary, if touching the boundary the polygon features are to be left in its entirety. This is documented in our report. Any polygons fully outside the boundary were removed.  This is in Flood Risk Assessment Guidance, Section 7.0 Dataset Spatial Extents

  - Backcheck: no comments

- Updated TOT_LOSS01 and BL_TOT01 fields for consistency in use of 0 and -9999 in the field. It is best to stick to 0 for any census blocks that had loss information rather than -9999 which normally represents NULL values.

  - Response #1: No change. Null values have been populated with –9999 and output data from Hazus listed as Zero (0) left as is. This approach has been approved by TWDB.

  - Backcheck: no comments

- S_AOMI_AR

  - Missing AOMI polygons where there will be tie-ins to adjacent watersheds that are not studied by Stantec. Please note, tie-ins with in +/- 0.5 ft are required between internal model domains to adjacent study HUCs for HUCs studied by your team.

  - Response #1: Agree, we added appropriate AOMIs for adjacent watersheds that are not part of our Task Orders.

  - Backcheck: no comments

- L_Source_Cit

  - No comments

QA/QC Checklist Approval

This 2D Base Level Engineering Analysis QA/QC Review is in compliance with the contract requirements and all task checkpoints are complete.  The independent QC Team has reviewed the analysis, presented review comments to the Production Team Task Leader, and discussed any problems or issues.  The Task Leader, QC Manager, QC Reviewer, and Project Manager have signed the QA/QC Checklist to confirm that all comments have been received, addressed, and documented appropriately.

| QA/QC Team – [TWDB or company name] |  |  |  | Production Team – [add company name] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| QC Manager |  | Date |  | Task Lead |  | Date |
| QC Reviewer |  | Date |  | Project Manager |  | Date |