# Texas 2D BLE Analysis Audit Checklist

# Project Information

| HUC-8 Watershed Name & ID: | 11120202 Lower Salt Fork Red |
| --- | --- |
| HUC-10 Watershed Name & ID: | Richardson Creek-Salt Fork Red River 1112020201 Sand Creek-Salt Fork Red River 1112020202 Cave Creek-Salt Fork Red River 1112020203 Turkey Creek 1112020204 Bitter Creek-Salt Fork Red River 1112020205 |
| Modeler: | Ian Hudson, Shiva Sandrana, Pedro Silva, Gokgoz Kilic |
| Reviewer: | Sara McMahan (Halff) |

This audit process includes only high-level review for items identified in the Detailed Review for another HUC for the same consultant. The consultant is responsible for performing a detailed review internally for base data, hydrology, hydraulics, and modeling results.

## Audit 1: Terrain, Modeling, Preliminary Mapping Review (QC Items 1-7)

| Modeler Initials: | DC | Reviewer Initials: | SM |
| --- | --- | --- | --- |
| Submittal Date: | 6/30/2023 | Review Submitted Date: | 7/18/2023 |
| Review Response Date: |  | Backcheck Complete Date: | 10/31/2023 |

## Audit 2: Completeness Review (QC Item 8)

| Modeler Initials: | DC | Reviewer Initials: |  |
| --- | --- | --- | --- |
| Submittal Date: | 6/30/2023 | Review Submitted Date: |  |
| Review Response Date: |  | Backcheck Complete Date: |  |

(Modeler to populate with an “X” for items included in the QC submittal)

| Level of Study |  |  | Flood Events |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| X | 2D BLE |  | X | 10% ACE (10-year) | X | 0.2% ACE (500-year) |
|  | Enhanced BLE |  | X | 4% ACE (25-year) | X | 1% plus |
|  |  |  | X | 2% ACE (50-year) | X | 1% minus |
|  |  |  | X | 1% ACE (100-year) |  | Observed Rainfall |

Instructions

Each section includes instructions to the modeler and reviewer regarding their responsibilities in the completion of this checklist. In the Methodologies / Data Sources tables, grey italic text is added as an example response. Additional sample responses are located in the Texas 2D BLE Guidelines Quick Guide.

## Submittal Checklist

Data submitted for each milestone should be carried through each subsequent milestone. All geospatial layers and models must have same defined projection system as terrain. Modeler to populate with an “X” for items included in the QC submittal.

| Base Map Data (QC items 1-6) |  |
| --- | --- |
| X | Location Map (PDF): HUC-8 and HUC-10 boundaries and/or model domain boundary if different from HUC-10s, stream gages, dams (if applicable), mainstem streams (for reference), background map |
|  | GIS Map (MXD or Map Package) with the following data included but not limited to: Stream centerlines, model breaklines, rainfall data, WSEL and Depth grids (10, 100, 500-yr), adjacent HUC mapping (available on the BLE viewer or currently studying), 2D connections for inflows/outflows, gages or points of flow comparisons (BFEs, highwater marks, etc.), profile lines, boundary condition lines, refinement regions, model domain boundary, model mesh, DFIRM mapping panels, base map data (roads, aerials, etc.), soils data, land use data, terrain |
| Technical Report / Documentation (QC items 1-6) |  |
| X | Technical Report / Project Notebook: assumptions, special considerations, and modeling concerns |
| X | Detailed explanation and support data of variances in methodologies and approaches from recommended guidelines (Should be included in Technical Report / Project Notebook) |
| X | Internal QA/QC Documentation: comments and responses |
| X | Flow comparison tables within the technical report |
| Hydrology (QC items 1-6) |  |
|  | Hydrologic Model (HEC-HMS) model with all associated DSS files for 7 required storm events |
| X | Hydrologic Data and Parameter Spreadsheets (rainfall calculations, curve number table, gage locations, gage analysis and flow validation) |
|  | Hydrologic modeling notebook including assumptions, notes, and special conditions (if not provided in report) |
| Hydraulics (QC items 1-6) |  |
| X | Hydraulic Model (HEC-RAS) with all associated files and 7 required storm events (remove non-final files)d |
| X | Hydraulic Data and Parameter Spreadsheets (Manning’s n-value table, infiltration calculations support data-Soils, Land Use, Curve Number table, other calculations/data used in model) |
| X | Manning’s n-value table |
| X | Precipitation data correctly linked to hydrologic model or hard coded from provided spreadsheet |
| X | Hydraulic modeling notebook including assumptions, notes, and special conditions |
| Preliminary (Raw) Mapping (QC item 7) |  |
| X | Preliminary 10%, 1%, 0.2% ACE mapping results (raster grids from HEC-RAS) |
|  | Modeler to complete topology validation as outlines in the guidelines |
|  | Modeler to verify mapping meets deliverable requirements as outlined in guidelines |
| Completeness Review (QC item 8) See Texas 2D BLE Guidelines Chapter 4 – Deliverables |  |
|  | Technical BLE Report |
|  | Final cleaned hydrologic model |
|  | Final cleaned hydraulic model |
|  | Flood risk database |
|  | CMNS updates |
|  | HAZUS results |
|  | Additional supplemental data |

QC shapefiles provided for each model for SA2D and mesh enforcement adjustments. There are no comments in these, they are just for spatial comments. Other shapefile provided to show issues that occur with boundaries between models and added breaklines.

# 1 – Model Terrain

Methodologies / Data Sources (Modeler to populate)

| Terrain Source(s): | United States Geological Survey (Texas Panhandle 2018 D18 Lidar Oklahoma Woodward 2017 Lidar Oklahoma Dam Rehab Assessment 2011) |
| --- | --- |
| Data Gaps: | N/A |
| Source Resolution: | 2 meter |
| Model Terrain Resolution: | 3.28084 ft |
| Terrain Horizontal Projection: | NAD83 Texas North FIPS 4201 (US Feet) NAVD 1988 |
| Terrain Vertical Projection: | NAVD88 height |
| Z Values: |  |
| Terrain Buffer: | 1 mile |
| Other: |  |

## Technical Review (Reviewer to provide comments)

Reviewer is to utilize detailed QA/QC to identify general review items for this audit.

## Review Comments

- Model 203’s terrain is not clipped to a 1-mile buffer. Causes the model to open very slowly and it’s less efficient when reviewing while the terrain loads.

Response: Model 203 terrain clipped in re-submitted model.

Backcheck: No further comments.

- Review terrain mods. Many unnecessary ones such as the one below. Channel was already burned through the bridge; terrain mod did not add any detail or allow flow to go anywhere it couldn’t already go.

Response: Redundant terrain modifications eliminated from models.

Backcheck: No further comments.

- Excerpt from report below. Not sure this is the best method to address the irrigation canals. Using “take higher” to completely remove the irrigation channels also artificially raises the berms. A breakline on the berms to prevent water from spilling until it reaches berm height makes more sense. Or if terrain mods are preferred, polygons on the berms using controls point (berm elevation points) would block the channel without artificially raising the berms.

Response: Levee terrain modifications were replaced with polygon terrain modifications using the berm elevations as control points.

Backcheck: I don’t see discussion of the actual method used in the report. Please add to terrain modification section.

Response: Discussion added in Section 4.4 describing the terrain modifications.

# 2 – Hydrology

## Methodologies / Data Sources (Modeler to populate)

| Software: | HEC-RAS 6.3 |
| --- | --- |
| Rainfall Source: | NOAA Atlas 14 |
| Rainfall Duration: | 24 hours |
| Depth Selection: | 10-year, 25-year, 50-year, 100-year, 500-year, 100-Plus, 100-minus |
| Rainfall Temporal Distribution: | NOAA Atlas 14 Volume 11, Region 2, 1st quartile, 50% event hyetographs |
| Areal Reduction: | No |
| Loss Methodology: | SCS Curve Number |
| Unit Hydrograph Transform Method: | Atlas 14 Regional Curves |
| Inflow Hydrograph Source: | N/A |
| Other: |  |

## Statistical Data (Modeler to populate)

| Stream Gage Name & ID: | Salt Fork Red River near Wellington, TX (07300000) |
| --- | --- |
| Stream Gage Name & ID: | Salt Fork Red River at Mangum, OK (07300500) |
| Stream Gage Name & ID: | Salt Fork Red River near Elmer, OK (07301110) |

## Technical Review (Reviewer to provide comments)

Reviewer is to utilize detailed QA/QC to identify general review items for this audit.

## Review Comments

- Stated CN values are quite a bit higher than those in TWDB guidelines.

Response: Deviation of CN and impervious from TWDB guidance to be highlighted in the report.

Backcheck: No further comments.

- TWDB guidelines require three layers when using SCS Curve Number method – soils, landcover, and infiltration.

Response: Deviation of CN and impervious from TWDB guidance to be highlighted in the report.

Backcheck: No further comments.

# 3 – Hydraulic Geometry

## Methodologies / Data Sources (Modeler to populate)

| Software: | HEC-RAS 6.3 |
| --- | --- |
| Loss Methodology: | SCS Curve Number |
| Soils Source: | USGS Soils Survey Geographic Database (SSURGO) (SSURGO 2020; MRCL 2019) |
| Land Cover Source: | National Land Cover Database |
| Manning's Roughness Source: | 2019 National Land Cover Database (NLCD) Land Use Classification raster |
| Nominal Mesh Cell Sizes: | 200x200ft |
| Other: |  |

## Technical Review (Reviewer to provide comments)

Reviewer is to utilize detailed QA/QC to identify general review items for this audit.

## Review Comments

- Update model folder and project names to have consistent naming conventions between the five HUC-10s. Use TWDB naming guidelines.

Response: All model files renamed to the HUC10 number.

Backcheck: No further comments.

- There are areas where revised boundary is narrower than official USGS boundary. Confirm if this is okay since it’ll cut off some mapping. Example below – blue areas would likely have mapping if USGS boundary was used.

Response: All models were modified to add a buffer of overlap to allow for perimeter tie-ins. The gaps in USGS HUC8 coverage have been corrected.

Backcheck: Still a sizeable gap at the US end of model 201. If covered by US model, then okay. Confirm complete coverage and tie-ins are met between all model domians prior to final mapping.

Response: Section 4.5 in the report on the modeling coverage in this area from the upstream model. Supplementary DSS continuity and depth raster checks have been provided in the submission along with the upstream model.

Note for future reviews: Please provide model domain boundaries as shapefiles for review in GIS.

Response: Noted.

Also a gap in the northeast part of model 201. Response: Model domain extended to address gap in coverage.

- Not all HUC10s edge matched. There are gaps that aren’t covered by any model. Check Lower_QAQC.shp for locations. Double check adjacent HUC8s for edge matching as well.

Response: All models were modified to add a buffer of overlap to allow for perimeter tie-ins. There are no gaps between models remaining.

Backcheck: No further comments.

- Most of the SA2D flow transfer lines should be truncated to cover the width of the highest flood event (without pulling excess flow) and adjusted to stay perpendicular to the floodplain. Check individual shapefiles for locations.

Response: no change. SA/2D connectors are delineated perpendicular to the water surface elevation contours and ensure flow continuity between models, as reflected in the flow continuity spreadsheets included.

Backcheck: Noted, no further comments.

- Cells are often not properly enforced around breaklines, particularly at intersections. Can cause unwanted leaking across cells. Example below, see model shapefiles for specific locations.

Model 202 – specific attention needs to be given to the breaklines in Wellington; did not call out all of the enforcement issues because almost all breaklines had issues.

Response: Enforcement errors addressed and documented in QA/QC shapefiles.

Backcheck: See comments for any outstanding backcheck comments.

Response: Enforcement errors corrected in model and documented in shapefile.

- Some areas have conflicting breaklines. One of the breaklines should be trimmed back to help with cell enforcement. Particularly when a road is intersecting another road at an angle; incoming road should be trimmed back. Can also adjust near spacing around particular breaklines (for the smaller breaklines used for berms this may be most effective). Example below, see model shapefiles for specific locations. Example below should just be realigned into a single breakline. No reason for the breaklines to be separate and bend upwards.

Response: Breakline alignment corrected where applicable.

Backcheck: See comments for any outstanding backcheck comments.

Response: Enforcement errors corrected in model and documented in shapefile.

- Some breaklines not enforced at all. Example below, see model shapefiles for specific locations.

Response: Breakline enforcement reviewed and corrected in updated models.

Backcheck: See comments for any outstanding backcheck comments.

Response: Enforcement errors corrected in model and documented in shapefile.

- Some computation points not contained within their cells. Example below, see model shapefiles for specific locations.

Response: Cell placement and enforcement revisited in updated models.

Backcheck: See comments for any outstanding backcheck comments.

Response: Enforcement errors corrected in model and documented in shapefile.

- Refinement regions generally, particularly along the mainstem, are not well defining the channel/floodplain. They’re just buffered regions around the centerline. More effort should be made to have them represent the channel geometry. Example below – blue makes more sense; current boundary cuts into high overbanks and leaves out other areas of similar elevation in the main channel area.

Good example of a refinement region that has been updated to follow the bank better below. Would recommend matching the other bank though.

Response: The Refinement regions have been adjusted along the main stem to follow the main conveyance path.

Backcheck: Refinement regions looks better but are still inconsistent along banks. Example below – blue looks good but other areas (red) of similar elevations are left out of refinement regions. A lot of areas are clearly still buffered centerlines.

Response: The current refinement region resolution is consistent with the four other task order 2 watersheds which have all been approved for mapping. The refinement regions are not manually delineated along the bank lines but coarsely delineated based on modeled inundation results from a 10-year event, as described in the report. We believe the refinement regions in these models should be maintained for consistency across the task order.

- Not all SA2D connections are enforced.

Response: SA/2D connectors enforced in updated models.

Backcheck: No further comments.

- A lot of breaklines, particularly in Model 203, are so tiny they don’t affect the cell enforcement at all and should be deleted. Example below.

Response: Eliminated/merged all breaklines less than 100ft in updated HUC203 model.

Backcheck: No further comments.

- Trim breaklines and refinement regions within a reasonable distance to edge of boundary.

Response: Breaklines in all models trimed closed to the perimeter edge in updated models.

Backcheck: No further comments.

- Use a full breakline rather than these small curved breaklines in these areas.

Response: Corrected in HUC203 model, noted in shapefile response column.

Backcheck: No further comments.

# 4 – Hydraulic Boundary Conditions

## Technical Review (Reviewer to provide comments)

Reviewer is to utilize detailed QA/QC to identify general review items for this audit.

## Review Comments

- Model 203 and 204 – use same boundary condition method as other models rather than the small segments of boundary conditions. Missing some areas where water builds up at the boundary.

Response: As model perimeters are now buffered, generic boundary condition lines have been applied along the entire perimeter excluding the inflow/outflow locations.

Backcheck: If using a generic boundary condition (outside of outflow channels) all models should use the same value. 0.01 and 0.001 are both being used throughout the models. Update models to all use the generic boundary condition. 205 still uses normal depth slopes based off terrain rather than any generic value.

Response: All models updated to 0.01 slope for perimeter boundary conditions.

- Normal depth boundary condition lines should be consistent with ground slope, not all using the same value. Some models using same slope for all boundary conditions aside from the downstream channel segment.

Response: As model perimeters are now buffered, generic boundary condition lines have been applied along the entire perimeter excluding the inflow/outflow locations.

Backcheck: If using a generic boundary condition (outside of outflow channels) all models should use the same value. 0.01 and 0.001 are both being used throughout the models. Update models to all use the generic boundary condition.

Response: All models updated to 0.01 slope for perimeter boundary conditions.

- New comment: Model 204 has no outflow channel boundary condition (using channel slope). Please confirm this is correct / appropriate.

Response: Outflow boundary condition line adjusted and slope re-measured to reflect the ground slope.

# 5 – Hydraulic Model Execution

## Methodologies / Data Sources (Modeler to populate)

| Unsteady Plan Names |  |  |  |
| --- | --- | --- | --- |
| 10% ACE (10-year): | 10pct | 0.2% ACE (500-year): | 0_2pct |
| 4% ACE (25-year): | 4pct | 1% plus: | 1pctPLUS |
| 2% ACE (50-year): | 2pct | 1% minus: | 1pctMINUS |
| 1% ACE (100-year): | 1pct |  |  |
| Other: |  |  |  |

## Statistical Data (Modeler to populate)

| Time Step: | Variable Courant-Based Time Step |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Courant Value: | <2.0 |  | Simulation Length: | 96 | hours |
| Computation Interval: | 30 (base) | seconds | Model Runtime: | <12 hours | hours |
| Hydrograph Output Interval: | 15 | minutes | Mapping Output Interval: | 15 | minutes |

## Technical Review (Reviewer to provide comments)

Reviewer is to utilize detailed QA/QC to identify general review items for this audit.

## Review Comments

- No comments.

# 6 – Hydraulic Model Results

## Methodologies / Data Sources (Modeler to populate)

| Flow Validation/Gage Analysis |  |
| --- | --- |
| Software: | HEC-RAS 6.3 |
| Methodology | Rain on Grid |
| Removal of Outliers and Justification |  |
| Validation Notes (describe parameter adjustments): |  |
| Other: |  |

## Technical Review (Reviewer to provide comments)

Reviewer is to utilize detailed QA/QC to identify general review items for this audit.

## Review Comments

- Was removal of outliers considered during gage analysis? Based on the plots in Appendix B, it looks like Gage 07300000 might have a couple of high outliers that could skew the flows. This may help bring modeled peak flows closer to the gage analysis values.

Response: Analysis for gage 07300000 was revised as part of the approval of HUC8-11120201. The results presented in the report reflect the approved analysis. Updated gage analysis has been included in hydrology submittal folder.

Backcheck: No further comments.

- Adjust SA2D lines for gages. Current line is perpendicular to channel and flow but should be across the bridge opening to accurately reflect the location of the gage.

Response: SA/2D connectors are extended along the bridge to capture overtopping of the embankment which is not reflected in the inundation mapping due to cell resolution.

Backcheck: Noted, no further comments.

- There are still areas with courant higher than 5. If an adaptive time step is already being used, look at mesh sizing and breaklines near the problem areas.

Response: Noted and revised as needed in revised models.

Backcheck: No further comments.

- Effective mapping is generally wider than modeled floodplains.

Response: Model is performing within Bulletin 17C confidence intervals and are considered calibrated.

Backcheck: Noted, no further comments.

# 7 – Preliminary (Raw) Mapping Results

## Technical Review (Reviewer to confirm with Pass/Fail)

| Hydraulic Model |  |
| --- | --- |
|  | HEC-RAS model with results, pertinent data and required storm events |
| Preliminary Mapping 1% ACE |  |
|  | Preliminary 10%, 1%, 0.2% ACE mapping results (raster grids from HEC-RAS) to meet FEMA mapping requirements |
|  | Boundary interfaces align matching or identified as AOMI |

## Review Comments

- See review comments under Checkpoint 8 below

# 8 – Completeness Review

## Technical Review (Reviewer to confirm with Pass/Fail)

| File Structure and Naming |  |
| --- | --- |
|  | Deliverables provided in TWDB/FEMA required file folder structure and naming convention |
|  | Data missing (if any) documented in report and ReadMe file within the required folder |
| Report |  |
|  | Technical 2D BLE report documents all assumptions, special considerations, etc. |
|  | Flow validation and difference discussed in report |
|  | Flow comparisons discussed in report |
| Hydrologic Model |  |
|  | HEC-HMS model with results, pertinent data and required storm events |
| Hydraulic Model |  |
|  | HEC-RAS model with results, pertinent data and required storm events |
| Flood Risk Database |  |
|  | All features fully populated per current FEMA guidelines |
|  | Mapping consistent between grids to polygons for all three required mapped events |
|  | 10yr smaller than 100yr smaller than 500yr |
|  | Holes filled in polygon, depth and WSE grids |
|  | Boundary interfaces align matching or identified as AOMI |
| CNMS |  |
|  | Updated correctly with new centerlines |
|  | A1-A5 checks complete |
|  | CNMS Post- Processing Complete |
| HAZUS |  |
|  | Run per FEMA guidance |

## Review Comments

- FRD

  - Base_Dataset

    - S_HUC_Ar

Geometry:

- No Comments

Attribute Table:

- No Comments

    - S_Pol_Ar

Geometry:

- No Comments

Attribute Table:

- The feature with ObjectID 72, which is Wellington, should be classified as a city, not an Unincorporated Area. Please update POL_NAME1 and POL_NAME2 to reflect this and make sure attribution is consistent with other cities.

Response#1:  Agree, revised attribution of Wellington.

Halff Backcheck 8/28/2024: Corrected. No further comments

  - CNMS_Dataset (Reviewed separately by region 6 CNMS lead)

  - EBFE_Dataset

    - BFE_2D

Geometry:

- Some issues with the BLE_WSE01PCT grid are reflected in the BFE lines. See QC polygon and Grids Comments.

Response#1:  Agree, WSE grids were recreated to reflect the island filling provided in the TWDB guidance table.

Halff Backcheck 8/28/2024: Halff Backcheck 8/28/2024: Unresolved. Steep and sudden drops in Water Surface Elevation still exist. Please see ‘QC_Polygon_20240828’ for example.

Response#2: No action needed. Alignment w/topography achieved; steep change in WSE   	       due to the filling of a long thin hole (e.g. a roadway or berm) where the original WSE 	           	        was different on either side before filling.

Attribute Table:

- No Comments

    - DTL_STUD_AR

Geometry:

- No Comments

Attribute Table:

- No Comments

    - DTL_STUD_LN

Geometry:

- No Comments

Attribute Table:

- No Comments

    - FLD_HAZ_AR

Geometry:

- Floodplain polygons smaller than 0.33acres exist. This is smaller than the allowable mapping threshold defined by page 36 of 48 in the TWDB BLE Guidelines. Please remove areas of shallow or small flood risk according to the Guidance outlined by TWDB. Please see ‘QC_Polygon_20240627’ for examples and search for other instances throughout the HUC. 
Response#1: No action. All TO2 HUCs have not performed additional disconnect removal after the Matrix Tool Coastal and High Plains removal criteria (TWDB approved). 
An additional removed criteria is performed in TO3 HUCs.

Halff Backcheck 8/28/2024: Ok

- Zone ‘A’ floodplain polygons exist that are not reflected in the 1% Grids. FLD_HAZ_AR should match 1% and 0.2% grids extents. Please see ‘QC_Polygon_20240627’ for example and search for other instances throughout the HUC. 
Response#1: Agree, we corrected to ensure gridded data and vector data are consistent between event types (1% WSE, Depth Grid and Vector data; 0.2% WSE, Depth Grid, and Vector data).

Halff Backcheck 8/28/2024: Corrected. No further comments

Attribute Table:

- No Comments

    - SUBBASINS

Geometry:

- No Comments

Attribute Table:

- No Comments

    - TENPCT_FP

Geometry:

- Features smaller than 0.5 acres should be deleted. This is based on comments TWDB received from region on previously submitted HUCs. Please see ‘QC_Polygon_20240627’ for example and search for other instances throughout the HUC. 
Response#1: No action. All TO2 HUCs have not performed additional disconnect removal after the Matrix Tool Coastal and High Plains removal criteria. Islands were filled up to 0.5acres for 10pct FP. An additional removed criteria is performed in TO3 HUCs.

Halff Backcheck 8/28/2024: Ok

Attribute Table:

- No Comments

    - WTR_AR

Geometry:

- No Comments

Attribute Table:

- No Comments

    - WTR_LN

Geometry: No Comments

Attribute Table: No Comments

    - XS_1D

Geometry: No Comments

Attribute Table: No Comments

  - Mit_Haz_Datasets

    - S_AOMI_Ar

Geometry: No Comments

Attribute Table:

- Feature AOMI_ID = 206 is missing CID and POL_NAME_1. Please populate accordingly. Response#1: Agree, revised attribution of AOMI_ID = 206.

Halff Backcheck 8/28/2024: Corrected. No further comments

    - S_FRAC_Ar

Geometry: No Comments

Attribute Table:

- Fields TOT_LOSS01, BL_TOT01, CL_TOT01 should be using -8888 for null values not -9999 since they are “required” fields. Please refer to page 29 of the FEMA Region 6 BLE Guidelines for reference.

Response#1: Agree, these three fields are noted as Required. -8888 was applied.

Halff Backcheck 8/28/2024: Corrected. No further comments

    - S_FRAS_Pt

Geometry: No Comments

Attribute Table: No Comments

- Grids

    - The 1% WSE Grid is using a custom coordinate system. I recommend making sure all grids are consistent and are using the same coordinate system. 
Response#1: Agree, we revised the 1% WSE grid coordinate system to reflect the TX State Plane North.

Halff Backcheck 8/28/2024: Corrected. No further comments

    - Sudden changes in elevation exist in WSE grids throughout HUC. these sudden changes are not reflected in the depth grids. Also affecting BFE lines. Please see ‘QC_Polygon_20240627’ for example and search for other instances throughout the HUC.

Response#1:  Agree, confirmed and achieved elevations are smooth and in alignment with topography.

Halff Backcheck 8/28/2024: Unresolved. Steep and sudden drops in Water Surface Elevation still exist. Please see ‘QC_Polygon_20240828’ for example.

Response#2: No action needed. Alignment w/topography achieved; steep change in WSE due to the filling of a long thin hole (e.g. a roadway or berm) where the original WSE was different on either side before filling.

    - Internal tie-in between model boundaries 2 and 3 in 1% grids exceed 0.5ft tolerance. Please see ‘QC_Polygon_20240627’ for location.

Response#1: Confirmed and achieved 0.5ft tie-in.

Halff Backcheck 8/28/2024: Corrected. No further comments

    - Internal tie-in between model boundaries 4 and 5 in 1% grids exceed 0.5ft tolerance. Please see ‘QC_Polygon_20240627’ for location. 
Response#1: Confirmed and achieved 0.5ft tie-in.

Halff Backcheck 8/28/2024: Corrected. No further comments

    - Internal tie-in between model boundaries 3 and 5 in 0.2% grids exceed 0.5ft tolerance. Please see ‘QC_Polygon_20240627’ for location.

Response#1: Confirmed and achieved 0.5ft tie-in.

Halff Backcheck 8/28/2024: Unresolved. Tie-in still exceed 0.5ft tolerance. Please see ‘QC_Polygon_20240828’ for location.

Response#2: Agree, confirmed and achieved 0.5ft tie-in.

QA/QC Checklist Approval

This 2D Base Level Engineering Analysis QA/QC Review is in compliance with the contract requirements and all task checkpoints are complete.  The independent QC Team has reviewed the analysis, presented review comments to the Production Team Task Leader, and discussed any problems or issues.  The Task Leader, QC Manager, QC Reviewer, and Project Manager have signed the QA/QC Checklist to confirm that all comments have been received, addressed, and documented appropriately.

| QA/QC Team – [TWDB or company name] |  |  |  | Production Team – [add company name] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| QC Manager |  | Date |  | Task Lead |  | Date |
| QC Reviewer |  | Date |  | Project Manager |  | Date |