# Texas 2D BLE Analysis QA/QC Checklist

# Project Information

| HUC-8 Watershed Name & ID: | Southern Beaver: 11130207 |
| --- | --- |
| HUC-10 Watershed Name & ID: | Headwaters Beaver Creek (1113020701) -SBVR_701 Upper Beaver Creek (1113020702) -SBVR_702 Middle Beaver Creek (1113020703) -SBVR_703 Lower Beaver Creek (1113020704) -SBVR_704 |
| Modeler: | Watearth/Atkins |
| Reviewer: | Halff Associates |

## Terrain and Modeling Review (QC Items 1-6)

| Modeler Initials: | SGK/FA/CQ | Reviewer Initials: | NSR |
| --- | --- | --- | --- |
| Submittal Date: | April 21, 2023 | Review Submitted Date: | June 10, 2023 |
| Review Response Date: |  | Backcheck Complete Date: | August 2, 2023; August 20, 2024 |

## Mapping Review (QC Item 7)

| Modeler Initials: | PC/RF | Reviewer Initials: | NB, LB |
| --- | --- | --- | --- |
| Submittal Date: | 9/20/23 | Review Submitted Date: | 2/28/2024, 7/5/2024 |
| Review Response Date: | 10/9; 10/31; 11/14; 11/28; 12/4; 2/24/2024; 6/25/2024 | Backcheck Complete Date: | 2/29/2024, 7/5/2024, 8/19/2024, 1/2/2025 |

## Completeness Review (QC Item 8)

| Modeler Initials: | PC/RF/SA | Reviewer Initials: | NB, LB |
| --- | --- | --- | --- |
| Submittal Date: | 2/24/2024 | Review Submitted Date: | 2/28/2024, 7/5/2024 |
| Review Response Date: | 6/25/2024 | Backcheck Complete Date: | 2/29/2024, 7/5/2024, 8/19/2024, 1/2/2025 |

(Modeler to populate with an “X” for items included in the QC submittal)

| Level of Study |  |  | Flood Events |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| X | 2D BLE |  | X | 10% ACE (10-year) | X | 0.2% ACE (500-year) |
|  | Enhanced BLE |  | X | 4% ACE (25-year) | X | 1% plus |
|  |  |  | X | 2% ACE (50-year) | X | 1% minus |
|  |  |  | X | 1% ACE (100-year) |  | Observed Rainfall |

Instructions

Each section includes instructions to the modeler and reviewer regarding their responsibilities in the completion of this checklist. In the Methodologies / Data Sources tables, grey italic text is added as an example response. Additional sample responses are located in the Texas 2D BLE Guidelines Quick Guide.

Reviewer to indicate if Technical Review items generally comply using a “Pass”. If the technical review observes an issue of noncompliance or an item that requires additional attention, an “Fail” should be utilized.

## Submittal Checklist

Data submitted for each milestone should be carried through each subsequent milestone. All geospatial layers and models must have same defined projection system as terrain. Modeler to populate with an “X” for items included in the QC submittal.

| Base Map Data (QC items 1-6) |  |
| --- | --- |
| X | Location Map (PDF): HUC-8 and HUC-10 boundaries and/or model domain boundary if different from HUC-10s, stream gages, dams (if applicable), mainstem streams (for reference), background map |
| X (RAS Mapper) | GIS Map (MXD or Map Package) with the following data included but not limited to: Stream centerlines, model breaklines, rainfall data, WSEL and Depth grids (10, 100, 500-yr), adjacent HUC mapping (available on the BLE viewer or currently studying), 2D connections for inflows/outflows, gages or points of flow comparisons (BFEs, highwater marks, etc.), profile lines, boundary condition lines, refinement regions, model domain boundary, model mesh, DFIRM mapping panels, base map data (roads, aerials, etc.), soils data, land use data, terrain |
| Technical Report / Documentation (QC items 1-6) |  |
| X | Technical Report / Project Notebook: assumptions, special considerations, and modeling concerns |
| X | Detailed explanation and support data of variances in methodologies and approaches from recommended guidelines (Should be included in Technical Report / Project Notebook) |
| X | Internal QA/QC Documentation: comments and responses |
| X | Flow comparison tables within the technical report |
| Hydrology (QC items 1-6) |  |
| X | Hydrologic Model (HEC-HMS) model with all associated DSS files for 7 required storm events |
| X | Hydrologic Data and Parameter Spreadsheets (rainfall calculations, curve number table, gage locations, gage analysis and flow validation) |
| X (Report/ Results spreadsheet) | Hydrologic modeling notebook including assumptions, notes, and special conditions (if not provided in report) |
| Hydraulics (QC items 1-6) |  |
| X | Hydraulic Model (HEC-RAS) with all associated files and 7 required storm events (remove non-final files) |
| X | Hydraulic Data and Parameter Spreadsheets (Manning’s n-value table, infiltration calculations support data-Soils, Land Use, Curve Number table, other calculations/data used in model) |
| X | Manning’s n-value table |
| X | Precipitation data correctly linked to hydrologic model or hard coded from provided spreadsheet |
| X | Hydraulic modeling notebook including assumptions, notes, and special conditions |
| Mapping (QC item 7) |  |
|  | Flood risk database |
|  | Modeler to complete topology validation as outlines in the guidelines |
|  | Modeler to verify mapping meets deliverable requirements as outlined in guidelines |
| Completeness Review (QC item 8) See Texas 2D BLE Guidelines Chapter 4 – Deliverables |  |
|  | Technical BLE Report |
|  | Final cleaned hydrologic model |
|  | Final cleaned hydraulic model |
|  | Flood risk database |
|  | CMNS updates |
|  | HAZUS results |
|  | Additional supplemental data |

# 1 – Model Terrain

Methodologies / Data Sources (Modeler to populate)

| Terrain Source(s): | Please see technical memo for more detail: Texas West Central LiDAR (2018), Brown County LiDAR (2019), Hurricane LiDAR (2019), Lower Colorado San Bernard LiDAR (2018), and Lampasas LiDAR (2014) |
| --- | --- |
| Data Gaps: | None |
| Source Resolution: | Cell size of 1 x 1 meter |
| Model Terrain Resolution: | Cell size of 5 x 5 feet |
| Terrain Horizontal Projection: | North American Datum (NAD) 1983 (2011) Texas State Plane Zone 3 (Central) |
| Terrain Vertical Projection: | NAVD88 |
| Z Values: | Units of feet |
| Terrain Buffer: | 2,500 feet |
| Other: |  |

## Technical Review (Reviewer to confirm with Pass/Fail)

| General Review |  |
| --- | --- |
| P | LiDAR source matches the latest available data at time of project initiation |
| P | Large reservoir or lake extents included in model domain |
| P | Check visible seams, gaps or holes of LiDAR sources (if more than one source was used) |

## Review Comments

- No comments

# 2 – Hydrology

## Methodologies / Data Sources (Modeler to populate)

| Software: | HEC-RAS 6.3.1 |
| --- | --- |
| Rainfall Source: | NOAA Atlas 14 |
| Rainfall Duration: | 24 hours |
| Depth Selection: | Domain Average |
| Rainfall Temporal Distribution: | HEC-HMS |
| Areal Reduction: | None |
| Loss Methodology: | Spatially varied CN |
| Unit Hydrograph Transform Method: | N/A |
| Inflow Hydrograph Source: |  |
| Other: Precipitation spreadsheets are included with the submittal |  |

## Statistical Data (Modeler to populate)

| Stream Gage Name & ID: | Beaver Creek near Electra, TX (07312200) |
| --- | --- |
| Stream Gage Name & ID: | Beaver Creek Tributary near Crowell, TX (07312140) |

## Technical Review (Reviewer to confirm with Pass/Fail)

| Excess Rainfall |  |
| --- | --- |
| F | Precipitation values are an aerial average of Atlas 14 values for the entire 2D area boundary for each storm event or other appropriate rainfall application |
| P | CN values appropriately reflect land use and soils for entire 2D area boundary |
| P | Losses are correctly calculated to establish the excess rainfall hyetographs by checking land use, soils and infiltration layers or other relative information |
| P | Total precipitation depths for 1%+ and 1%- are calculated correctly |
| P | Excess rainfall hyetographs are calculated for all storm events (10%, 4%, 2%, 1%, 0.2%, 1%+, and 1%-) |
| F | Cumulative excess rainfall values are in increasing order from 10%, 4%, 2%, 1%, to 0.2% |
| F | Cumulative excess rainfall values for the 1%+ and 1%- fall in order with the storm events matching the total precipitation values |
| Inflow Hydrographs |  |
| NA | Shape and time-to-peak of any inflow hydrographs resemble past documented large storm events |
| P | Hydrographs reasonable when compared to the other return periods. The shapes should be similar, and hydrographs from different events should not intersect unless there is an engineering reason. Peak flows from a lower return period event should not be higher than a higher return period event |
| P | Contributing hydrographs from another RAS model applied at coincident locations (Provide locations in base map data) |
| General Hydrology |  |
| y | Any flow regulating dams in the model area? (Yes/No) |
| NA | Dam modeling methodologies applied and explained? (Yes/No). If yes, provide discharge rating curve calculations and gage analysis documenting regulated versus unregulated records, if applicable. |
| NA | Model aligns with documentation of dam modeling approaches. |
| Other Notes: NA – PLEASE PROVIDE SOME ADDITIONAL INFORMATION REGARDING THE USE OF KNOWN WSE AT DAM IN MODEL 702 |  |

## Review Comments

- All models: Was unable to review the HEC-HMS model as the model delivered was built/run in HMS 4.11 BETA. Please use the current non-beta version.

Atkins response (07/11/23): Re-created the HMS model in HEC-HMS 4.10.

Half Backcheck: Comment Resolved

- Consider adding a figure to the BLE Report with the Precipitation Hyetographs for each model area.

Atkins response (07/11/23): In our Middle Pease and North Pease reports, we have included incremental and cumulative hyetographs from one of the domains in our write-up describing how the precipitation inputs were determined. We will add similar graphics to the Southern Beaver and Little Wichita reports. This approach was approved by the ITR team during our call on June 23, 2023.

Half Backcheck: Method approved. Will resolve comment when BLE report is submitted.

Atkins Response (8/27/23): We have added the information graphics described in our response dated 7/11/23.

- Model 702: Please add documentation as to use of initial condition points at the lake.

Atkins response (07/11/23): Santa Rosa Lake in the Red River Basin has a conservation pool elevation of 1167 ft above mean sea level (NAVD 29), which was adjusted by subtracting 0.2 ft to convert to NAVD 1988. (ref: Santa Rosa Lake (Red River Basin) | Texas Water Development Board)

Halff Backcheck: Comment not resolved.

Atkins Response (8/27/23): Use of the IC point to set the initial WSE in the lake has been documented in the report in Subsection 1.2.5.

- Model 703: No Initial Conditions Point is used in Model domain 703 for the downstream region. This model domain overlaps with 702 where a downstream Initial Condition point is located. These should correspond.

Atkins response (07/11/23): The initial condition point for the Santa Rosa Reservoir (the conservation pool value of 1167 ft (1166.80 ft when converted to NAVD 88) has been added to 703 in the reservoir

Half Backcheck: Comment Resolved

# 3 – Hydraulic Geometry

## Methodologies / Data Sources (Modeler to populate)

| Software: | HEC-RAS 6.3.1 |
| --- | --- |
| Loss Methodology: | Spatially varied CN |
| Soils Source: | SSURGO |
| Land Cover Source: | NLCD 2019 |
| Manning's Roughness Source: | Assigned based on land use. Please see table in Southern Beaver Model Overview and Results.xlxs |
| Nominal Mesh Cell Sizes: | 200 x200 except for channel refinement regions, breaklines and urban regions, where 100 ft x100 ft was considered. |
| Other: |  |

## Technical Review (Reviewer to confirm with Pass/Fail)

| 2D Mesh |  |
| --- | --- |
| F | 2D Boundary delineated along watershed boundary or buffer to ensure all contributing area is included |
| P | Nominal cell size appropriate for model detail (generally 200 feet, minimum 50 feet for refinement areas) |
| P | Computation points are at default locations or changes documented |
| P | Mesh Cell and Face tolerances are default or changes documented |
| P | No mesh errors |
| P | Cell face relationships are calculated |
| Breaklines |  |
| F | Breaklines are consistent with the terrain data |
| F | Breaklines sufficiently detailed to capture features (thalweg, banks, berms, etc.) |
| P | Cells do not overlap terrain features where appropriate (i.e., unwanted leaking) |
| P | Breakline cell sizing and parameters are appropriate |
| F | 2D Area Connections placed at applicable benchmarks, gages, or dams |
| NA | 2D Area Connections set to use normal 2D equation domain for benchmarks and gages. Appropriate modeling approach used for dams. |
| Manning’s N-Values |  |
| P | Manning’s n-values associated to land cover data provided |
| P | Land Cover classifications are sufficiently descriptive within the data table |
| P | Manning’s n-value classifications generally match current aerial imagery |
| P | Manning’s n dataset is associated with the geometry files |
| General |  |
| P | All model datasets each have unique descriptive names |

## Review Comments

- See QC Comments Shapefile SouthernBeaver_QC_Comments.shp. Comments note which model they are in reference to.

Atkins response (07/11/23): All comments have been responded to in the QC_shapefile.

Half Backcheck: Comment Resolved

- Model 701: Consider buffering the HUC boundary to ensure that it is fully covered by the model domain and so that the domain captures all topographic high points of the contributing study area. In the screen shot below the green is the HUC10 boundary and the black is your model domain.

Atkins response (07/11/23): We do not think any changes are needed for the following reasons:

  - We use the HUC-8 watershed boundary and the LiDAR terrain to set the external boundary extents so that they capture all terrain that drains to the watershed.

  - Our internal domain extents follow the high ground in the LiDAR data. No overlap is needed between internal domain extents.

In the following snip, the red line is USGS HUC10 boundary, orange is computed (hydro-conditioning) HUC8 boundary, and black is Model boundary. We followed the computed HUC8 boundary to make sure all flow is contained within the model.

Half Backcheck: Comment Resolved

- Looking at the terrain and contours (orange lines in screenshot) I would suggest revising your mesh boundary so that it fully contains all of the contributing area.

Atkins response (07/11/23): We do not think any changes are needed for the following reasons:

  - The model domain is within the USGS HUC8 boundary (orange line in our snip) and follows the high ground

  - The model boundary follows the roadway which would defer the flow towards west of the boundary

  - There is no ponding along the perimeter at this location (the snip is showing 100-year depth results) 

Half Backcheck: Comment Resolved

- It was noted that many streamline breaklines had gaps along streams, were floating (i.e. not connected to anything downstream) or had breaks when crossing roadway breaklines. Many examples of this are included in the QC shapefile. Please review streamline breaklines to ensure that there is appropriate connectivity and that streamline breaklines do not break or have gaps. See screenshot for an example.

Floating breakline not connected

Gap at structure

Atkins response (07/11/23): We reviewed the shapefile and add stream centerline breaklines where the gap is longer than 1000 ft (an example snip is below). The process we used to set breaklines prioritizes the roadway breaklines. When the stream centerlines are within 100 ft of the roadway breaklines, there is a break in the stream centerlines. The streams are continuous in the Manning’s classification polygons. This approach leads to cells that align better with roadways when the stream centerline breaklines are near roads. These stream centerline breaklines are not used as water lines in the mapping.

This below is an example where streamline breakline (and accompanying refinement region) is added to the model geometry:

Half Backcheck: Comment Resolved

- Noted many streamlines in the QC shapefile where the streamline breakline does not follow the terrain. Please review breaklines and correct streamlines to follow terrain.

Atkins response (07/11/23): Stream centerlines are within the 100-yr floodplain so we are not planning to make any changes. Stream centerlines were prepared as part of hydro-conditioning.  Cell sizes in the model in the stream refinement regions is 100 ft. When the stream centerline is highly sinuous, as it is here, this large cell size is not able to capture the sinuousity of the centerline. Adjusting the breakline will not improve the model.
Half Backcheck: Comment Resolved

- Reinforce Breakline 256 so that cells are aligned.

Atkins response (07/11/23): Cells are aligned along Breakline 256, see snip:

Half Backcheck: Comment Resolved

- Noted in QC Shapefile an area where there was some attempted leakage in the 500year mapping. Please review.

Atkins response (07/11/23): Reviewed and fixed. The snip below shows the depth for 500-year event:

Half Backcheck: Comment Resolved

- Would consider revising the alignment of the SA/2D Connection used to connect the upstream and downstream models so that it is more perpendicular to the main channel and floodplain and also so that it does not cut through the tributary. See QC shapefile.

Atkins response (07/11/23): The SA2D connection is adjusted so it does not cross the tributary and is perpendicular to the main channel and floodplain. The snip below shows the revised SA2D connection and the WSE results.

Half Backcheck: Comment Resolved

- Model 702: Noted that there are very few breaklines representing roadways throughout the model domain. Consider adding breaklines at roadways. Some examples of missing roadway breaklines are below.

Atkins response (07/11/23): These watersheds are not highly developed so there are not many roads. Many roads are at-grade. All roadways and railroads within the TxDOT databases have been used as breaklines. We will review and add if necessary, but the TWDB guidelines were followed.

Half Backcheck: Comment Resolved

- Noted several locations where streamlines do not intersect at confluences. Please review and correct these. –

Atkins response (07/11/23): Reviewed and made changes based on each case and made responses on the QC shapefile. Here are some examples of revisions:

Half Backcheck: Comment Resolved

- Noted many locations with disconnected streamlines, please ensure that there is a single consistent line for each streamline breakline.

Atkins response (07/11/23): The gaps in stream centerline breaklines occur when they are within 100 ft of road breaklines, to prioritize roadway breaklines and properly align cells. However, the Manning’s N channels are continuous and waterlines are not the same as stream centerlines

Half Backcheck: Comment Resolved

- Noted several locations where the streamline breakline does not follow the terrain and does not cross structures at the culvert location. Please review streamlines to ensure that follow the topography and cross roadway embankments at the correct location.

Atkins response (07/11/23): reviewed and made revisions based on each case and noted on QC shapefile. Here are some examples of revisions:

Half Backcheck: Comment Resolved

- Consider adding some additional breaklines around the spillways. See QC Shapefile.

Atkins response (07/11/23): Added

Half Backcheck: Comment Resolved

- Suggest modifications to the SA/2D Connections for the spillways of the reservoir.

  - For spillway 1, suggest shortening the length of the connection so that it only represents the actual spillway.

Atkins response (07/11/23): Shortened the length of SA2D connection

Half Backcheck: Comment Not Resolved. 2D Connection no longer is across the spillway. Intent of original comment was to shorten the 2D Connection so that it ran high point to high point and just contained the spillway section and not the embankment. Please revise

Atkins response (8/27/23): The SA2D Connection has been revised. See the screenshot below.

  - For spillway 2, suggest moving it downstream slightly so that it is located at the location of the weir according to aerial imagery and the terrain.

Atkins response (07/11/23): Moved the SA2D slightly downstream, the new spillways looks like this:

Half Backcheck: Comment Resolved

- Model 703:  Enforce SA/2D Connection Outflow_BVRC_703

Atkins response (07/11/23): All SA2D connections (including the new additions at validation points) are enforced

Half Backcheck: Comment Resolved

- Noted several locations where streamline breaklines did not match the streamline in terrain. See QC Shapefile and example below.

Atkins response (07/11/23): Please see comment #4 on Page #6.

Half Backcheck: Comment Resolved

- Noted some areas where the breakline was not enforced. Cells were not enforced with how the current breakline was drawn.

Atkins response (07/11/23): All breaklines are enforced in the model.

Half Backcheck: Comment Resolved

- Noted an area where cells are enforced but there in no breakline. See QC Shapefile FID 67.

Atkins response (07/11/23): There is no breakline at this location, it is noted on the QC Shapefile as well. All mesh has been recreated and all necessary breaklines are enforced during this revision.

Half Backcheck: Comment Resolved

- Please review these cells in the upstream end of the model. Looks like something is off from the transition out of the refinement region.

Atkins response (07/11/23): Reviewed and corrected the cells as seen below:

Half Backcheck: Comment Resolved

- Model 704: Noted many breaklines with gaps or the streamlines/breaklines did not intersect at the confluence. See QC Shapefile for reference.

Atkins response (07/11/23): Some of the gaps were connected and noted on the QC shapefile on a case by case basis. When the roadways and stream centerline breaklines are within 100-ft of each other, there is a gap in the stream centerline breaklines to prioritize roadway breaklines. The Manning’s n polygon for streams are continuous on the other hand.

Half Backcheck: Comment Resolved

- Noted many locations where breakline did not follow the channel as defined in the terrain. Please review and correct where necessary

Atkins response (07/11/23): Stream centerlines are within the 100-yr floodplain so we are not planning to make any changes. Stream centerlines were prepared as part of hydro-conditioning.  Cell sizes in the model in the stream refinement regions is 100 ft. When the stream centerline is highly sinuous, as it is here, this large cell size is not able to capture the sinuousity of the centerline. Adjusting the breakline will not improve the model.

Half Backcheck: Comment Resolved

- Noted some locations where a roadway breakline should be applied. These were many roadways and some requiring terrain modifications.

Atkins response (07/11/23): Breaklines are added on a case by case basis, some are shown below:

Half Backcheck: Comment Resolved

- Suggest adding a breakline to this dam embankment.

Atkins response (07/11/23): added, see snip below:

Half Backcheck: Comment Resolved

- FID 97 – Review this streamline alignment.

Atkins response (07/11/23): Stream centerlines are within the 100-yr floodplain so we did not make any changes. Stream centerlines were prepared as part of hydro-conditioning.  Cell sizes in the model in the stream refinement regions is 100 ft. When the stream centerline is highly sinuous, as it is here, this large cell size is not able to capture the sinuousity of the centerline. Adjusting the breakline will not improve the model.

Half Backcheck: Comment Resolved

- New Comment SBVR_701: Enforce 2D Connections as breaklines.

Atkins Response (8/27/23): 2D connections have been enforced. The next three screenshots show the upper, middle, and lower areas in the screenshot included with the ITR comment.

Upper:

Middle:

Lower:

Halff Backcheck (08202024): Comment resolved.

- SBVR_704: Enforce 2D connections.

Atkins Response (8/27/23): This comment was inadvertently overlooked while addressing the ITR comments. Leaving the 2D connection as is will not cause any adverse effects in the BLE outcomes.

Halff backcheck (08202024): Understood. Comment resolved.

# 4 – Hydraulic Boundary Conditions

## Technical Review (Reviewer to confirm with Pass/Fail)

| Boundary Review |  |
| --- | --- |
| P | External boundaries connected to 2D mesh |
| P | Boundary conditions everywhere flow could build up at the boundary of the 2D mesh |
| P | Boundary conditions have a hydrograph or elevation/slope data assigned |
| P | Input hydrographs consistent with hydrologic data (DSS, manual entry) |
| P | Precipitation data is consistent with hydrologic data (DSS, manual entry) |
| F | Downstream boundary consistent with ground slope or known downstream WSEL |
| F | Flow hydrographs applied at appropriate locations |
| P | Energy grade slope for hydrograph distribution is reasonable |
| P | Internal rating curves appropriate and documented for control structures (if applicable) |

## Review Comments

- Model 701: Consider adjusting downstream mesh boundary and boundary condition line so that the boundary is more perpendicular to floodplain and less rounded out. There is divided flow at the outflow for the 100yr and lower storm events. This may be remedied if the boundary is moved upstream or downstream some. Would also review the normal depth slope to ensure that the appropriate slope is applied.

Atkins response (07/11/23): We changed the SA2D connection and extended the model domain as can be seen in the snip on the right hand side. We are using stage hydrograph boundary conditions and not normal depth, so slope is not a concern.

Half Backcheck: Comment Resolved

- Model 702. Consider moving the inflow hydrograph to be external to the mesh boundary.

Atkins response (07/11/23): The process we follow uses internal boundary conditions to transfer flow from upstream model to downstream model, and has worked very well for tie ins in the past. We are not planning to make changes to this process.

Half Backcheck: Comment Resolved

- Consider shortening the BC Line for the outlet of 702. It should contain all storm event and tie in at high points but does not necessarily need to be drawn out so far.

Atkins response (07/11/23): BC line is shortened as shown in the snip on the right hand side:

Half Backcheck: Comment Resolved

- Models 702/703. Consider moving the overlap area between the models to be fully below the reservoir embankment and spillways. That may make for a cleaner tie in and overlap area. It would also allow the inflows from 702 to be applied at the upstream end/exterior of the mesh.

Atkins response (07/11/23): Initial conditions will be added to the reservoir in the overlap area, and the use of reservoir in the overlap area does not affect the downstream model so we are not making any changes to the model perimeters.

Half Backcheck: Comment Resolved

- Model 703: Consider splitting the downstream boundary condition line into two lines since there is divided flow here. The area in the left overbank will have a different slope that that of the main channel and floodplain.

Atkins response (07/11/23): Split the BC lines as seen in snip on the right hand side:

Halff Backcheck. While this response is valid for the above comment. The above comment, item 5, was made based upon results using a single outlet with a normal depth slope boundary condition. The current model provided has two outflows, each with their own stage hydrograph downstream boundary condition. This has changed the results and there is now a widened floodplain that extends across the entire downstream area. Since the conditions have changed since the previous submittal, it is recommended that a single BC line be used to represent the stage hydrograph. The input stage hydrographs were reviewed for the 1pct and it was noted that the shape and peak were generally consistent. The starting elevations were considerably different, but this is likely due to the starting ground surface elevation differences between the two hydrograph locations. Please review the downstream boundary and ensure that an appropriate boundary condition is applied.

New results:

500yr  100yr

Atkins Response (8/27/23): After examining the terrain and channels near the downstream end of 703 and the flow out of 703, some revisions were made. The SA2D connection has been split into three sections and there are still two stage hydrograph boundary conditions as the elevations are very different for the main channel and the tributary leaving 703.

Halff backcheck (08202024): Review of the two hydrographs as described. Comment resolved.

- Has consideration been given to moving the downstream boundary either more upstream away from the roadway or further downstream past the roadway? The way the boundary is cut and the proximity to the roadway may cause some unanticipated results along the boundary.

Atkins response (07/11/23): Yes, we have considered this. We are not planning to change the model boundary as the road does not have any effect on the flow in this location.

Half Backcheck: Comment Resolved

- Model 704: Consider moving the upstream inflow boundary condition line to the upstream end of the model domain so that it is an external boundary condition.

Atkins response (07/11/23): The process we follow uses internal boundary conditions to transfer flow from upstream model to downstream model, and has worked very well for tie ins in the past. We are not planning to make changes to this process.

Half Backcheck: Comment Resolved

- Consider adjusting the downstream model domain and boundary condition line so that it extends further past the confluence with the Wichita River. Model results can be clipped to the HUC boundary afterwards.

Atkins response (07/11/23):  Model domain is extended to include Wichita River

Half Backcheck: Comment Resolved

# 5 – Hydraulic Model Execution

## Methodologies / Data Sources (Modeler to populate)

| Unsteady Plan Names |  |  |  |
| --- | --- | --- | --- |
| 10% ACE (10-year): | 10pct | 0.2% ACE (500-year): | 0.2pct |
| 4% ACE (25-year): | 4pct | 1% plus: | 1plu |
| 2% ACE (50-year): | 2pct | 1% minus: | 1min |
| 1% ACE (100-year): | 1pct |  |  |
| Other: |  |  |  |

## Statistical Data (Modeler to populate)

| Time Step: | Advanced time step based on Courant condition (7.5-60 Seconds) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Courant Value: | Min: 0.45 | Max: 2 | Simulation Length: | 96 | hours |
| Computation Interval: | 30 | seconds | Model Runtime: | 1-8 | hours |
| Hydrograph Output Interval: | 10 | minutes | Mapping Output Interval: | 60 | minutes |

## Technical Review (Reviewer to confirm with Pass/Fail)

| Execution Review |  |
| --- | --- |
| P | Appropriate 2D parameters are used (Boundary Conditions, Time Step, Simulation Length, etc.) |
| P | Hydrograph Output Interval and Mapping Output Interval set at a reasonable interval to capture peaks |
| P | Model runtime is reasonable (< 12 hours, but dependent upon model size) |

## Review Comments

- No comments

# 6 – Hydraulic Model Results

## Methodologies / Data Sources (Modeler to populate)

| Flow Validation/Gage Analysis |  |
| --- | --- |
| Software: | N/A |
| Methodology | Regression flows were used in the absence of gage data and FIS flows; Draft InFIRM report was used to develop the inflow hydrograph for O.H. Ivie reservoir, pool frequency information obtained from RFA report is leveraged for reservoir modeling. Refer to the BLE report and the Model Overview and Results Spreadsheet. |
| Removal of Outliers and Justification | N/A |
| Validation Notes (describe parameter adjustments): Refer to the BLE Report and the MidCo Model Overview and Results.xlsx for more details of validation |  |
| Other: |  |

## Technical Review (Reviewer to confirm with Pass/Fail)

| Computation Checks |  |
| --- | --- |
| P | Model runs with no cell errors |
| P | Volume balance is within tolerance (ΔV < 1% is ideal, however 2% to 3% can be acceptable for rain on mesh) |
| Results Spot Checks Check at all critical structures, major confluences, a handful of headwater tributaries, downstream end, at benchmarks, and a handful of randomly selected locations. |  |
| P | No visible spikes in WSEL visibly present with a single cell |
| P | WSEL and depth grids appear reasonable (no artificial dips or spikes) |
| P | Visual verification flooding is connected downstream for all significant contributing areas (> 64 acres) |
| F | Velocity grids appear reasonable (no unusually high velocity areas or Courant values greater than 2) |
| F | No crossing profiles between all events |
| F | Inflow and outflow hydrograph timing is reasonable as compared to the peak |
| F | Spills into/from adjacent HUCs appropriately accounted for |
| F | WSEL tie-in between model domains and HUCs along major rivers/streams where inflows are applied |
| Results Full Checks |  |
| F | Computational time step meets ideal Courant condition |
| F | Diffusion Wave: V∆T∆X ≤2 (Up to 5) (Results are unit-less) |
| P | Model simulation is long enough to pass the entire hydrograph through the model (minimum of 12 hours past the peak, preferably 24 hours) |
| F | Hydrographs at internal and external boundaries are smooth |
| P | Contributing areas are contained with 2D mesh or accounted for with inflow hydrographs |
| F | No flow trapped at 2D boundary |
| P | Visual verification mapping is reasonable compared to effective |
| Flow Verification |  |
| P | Peak flow gage analysis performed on all gages in the watershed with sufficient data |
|  | All gages used have an appropriate period of record (10 to 20+ years) |
|  | All gages used have no extensive gaps in record which would impact the analysis |
|  | Gage analysis results validated to other sources (Regression, USGS, etc.) |
|  | Results at applicable benchmarks justified (gages, HWMs, effective BFEs) |
|  | Peak discharges comparable to gages, effective/prior studies, regional regression equations |

## Review Comments

- General: Were 2D Connections used at validation points to compare flows to Regression equations? Per BLE guidance, it is recommended that 2D Connection lines be used at Validation points.

Atkins response (07/11/23): SA2D connections are added to the validation locations

Half Backcheck: Comment Resolved

- Model 701. Please review the downstream boundary condition Outflow_SBVR_701_BC_STG. Water is ponding during the rising limb of the stage hydrograph. Would consider revising how the mesh boundary is drawn so that it is less rounded out.

Atkins response (07/11/23): The issue is resolved after making changes in the model and the boundaries. The new results are shown below:

Half Backcheck: Comment Resolved

- Model 702. Some instability noted at the SA/2D connections  that are used to represent the reservoir spillways. Consider some refinements to the connections as mentioned above. Would also consider some smaller cells at these locations.

Atkins response (07/11/23):  These minor instabilities do not affect model outcomes. Using smaller cell sizes is not certain to improve the model and undermine stability. No changes will be made to the model. 
Half Backcheck: Comment Resolved

- Courant does get to 5 at the 2nd spillway location. This indicates it may need some cell refinement.

Atkins response (07/11/23): Courant number may get slightly higher over spillways due to increased velocity during rain events.

Half Backcheck: Comment Resolved

- Review Outflow/Spillway 1. The HW Stage for the 1% is at one point higher than the HW Stage for the 0.2%.

Atkins response (07/11/23): We will investigate this issue.

Halff Backcheck: Issue not resolved. Crossing profile for 100yr/500yr at this 2D Connection.

Atkins response (8/27/23): The crossing profiles have been eliminated. Please see the screenshot below:

Halff backcheck (08202024): Comment resolved.

- There is a mapping bust between the 1pct events at the boundary between Model domains 702 and 703. Please review and revise. Results from 702 are approximately 2ft higher than those of 703.

Atkins response (07/11/23): This is remedied when tie ins are completed.

Half Backcheck: Comment Resolved

- Model 703: Please review the outflow boundary condition line for 703. But with two peaks in the flow and stage hydrographs, it could use another look to make sure nothing else is influencing the outflow.

Atkins response (07/11/23): Will review 
Half Backcheck: This Comment Resolved. There are additional comments for this downstream boundary below in comment 13.

- Model 704: There is some instability occurring in the rising limb of the stage hydrograph at the outflow boundary condition of 704. Please investigate.

Atkins response (07/11/23): These minor instabilities do not affect model outcomes. No changes will be made to the model.

Halff Backcheck: Comment Resolved.

- New Comment: SBVR_701: Flow at Outflow BC line is dipping and there is negative flow. Please review. 

Atkins Response (8/27/23): The negative flow has been reduced to only a few cfs. The stage hydrograph boundary condition is causing some slight backflow at the start of the rain event due to the very flat terrain of the watershed This negative flow has been documented in the report.

Halff backcheck (08202024): Comment resolved.

- SBVR_702: Cross profiles on the rising limb of stage hydrograph for Outflow2.

Atkins Response (8/27/23): None of the profiles are crossing on the rising limb of the hydrograph. See the screenshot below.

Halff backcheck (08202024): Comment resolved.

- SBVR_702: The 1pct results are higher at the beginning of the stage hydrographs for the validation point 2D connections. Screen shot is from PT3. Please review and revise.

Atkins Response (8/27/23): All plans, including the 1pct, start from the same starting stage, and then the stages do not cross each other. Please see the screenshot below.

Halff backcheck (08202024): Comment resolved.

- There is some backflow occurring at validation location SBVR_702_PT1 2D Connection. Please review this area and revise if necessary.

Atkins Response (8/27/2023): the negative flows at the beginning of the simulation are still in the model. However, this is just a random validation location and the small dip in the flows are insignificant compared to overall volume of water at this location. This issue has been documented in the report.

Halff backcheck (08202024): Comment resolved.

- SBVR_703: There are crossing profiles of the flow and stage hydrographs across the various BC lines. Please review the inflow conditions, unsteady flow files, etc. anything that may be causing these issues. Screen shot examples below.

  - Inflow From SBVR 702 2 Boundary Condition.

Atkins response (8/27/2023): The profiles are no longer crossing. See the 3 screenshots below.

  - Outflow SBVR 703 STG 2

Atkins response (8/27/2023): The profiles are no longer crossing. Please see the screenshot below.

  - Outflow SBVR 703 STG1

Atkins response (8/27/2023): The profiles are no longer crossing. Please see the screenshots below.

Halff backcheck (08202024): All Comments resolved.

- SBVR 704: Review Inflows from 703. There is a dip in the 0.2pct Inflow. Please make sure this is valid.

Atkins response (8/27/23): We have split the SA2D connection transferring flow from 703 into 704 into three segments since there a main channel and two tributaries flow out of 703. The split SA2D connection in 703 and the new inflows into 704 are shown below. The dip has been eliminated:

Inflow from 703_1

Inflow from 702_2:

Inflow from 703_3

Halff backcheck (08202024): Comment resolved.

# 7 – Mapping Results

## Technical Review (Reviewer to confirm with Pass/Fail)

| Hydraulic Model – Not included |  |
| --- | --- |
| FAIL | HEC-RAS model with results, pertinent data and required storm events |
| Flood Risk Database |  |
| FAIL | All features fully populated per current FEMA guidelines |
| FAIL | Mapping consistent between grids to polygons for all three required mapped events |
| FAIL | 10yr smaller than 100yr smaller than 500yr |
| FAIL | Holes filled in polygon, depth and WSE grids |
| FAIL | Boundary interfaces align matching or identified as AOMI |

## Review Comments

Mapping Geometry Comments:

- TENPCT_FP

  - Areas of the TENPCT_FP are outside of the 100yr and 500yr please review screenshots below and QC polygons. These are just representative examples a full review of the entire HUC required.

  - Many small fragments of the 10yr outside of 100yr and 500yr, review all fragments and consider an erase on any pieces of 10yr outside of FLD_HAZ_AR, if appropriate.

Atkins Response (10/30/2023): 10yr and 100yr floodplains are now contained inside the 100yr and 500yr floodplains, respectively.

Halff Backcheck 11/10/2023

  - No Further Comments

- GRIDs & FLD_HAZ_AR

  - BLE_WSE01PCT, BLE_WSE0_2PCT, BLE_DEP01PCT and BLE_DEP0_2PCT naming conventions incorrect please remove _tif

Atkins Response (10/30/2023): Raster names have been revised to remove _tif.

Halff Backcheck 11/10/2023

  - No Further Comments

  - Areas of the 100yr outside of the 500yr in Grids and Mapping (FLD_HAZ_AR)

Atkins Response (10/30/2023): 100yr grid has been updated to be contained inside the 500yr grid.

Halff Backcheck 11/10/2023

  - No Further Comments

  - Holes in the 500yr FLD_HAZ_AR and Grids. Ensure holes are filled up 2 acres in the 100 and 500 yr and up to 0.5 acres in the 10 year.

Atkins Response (10/30/2023): Holes have been filled in the 10yr, 100yr and 500yr grids.

Halff Backcheck 11/10/2023

  - No Further Comments

Model Boundary Tie-Ins

- Issue at Models SBVR 703 and 704 tie-ins greater than 0.5ft in the 100yr and 500yr. Screenshots below referencing 500yr

Atkins Response (10/30/2023): Raster tie-ins have been updated for the 100yr and 500yr grids.

Halff Backcheck 11/10/2023

  - No Further Comments

- Mapping and Grids in a few locations cut off at tie-in boundaries. Ensure requirement of +/- 0.5 ft tie is met.

Atkins Response (10/30/2023): For FIDs 16 and 17, after reviewing these areas, we decided to remove the  mapping that was not tieing in for two reasons. First, it was incredibly shallow, with an average depth of approximately 0.15 ft.  Second, in each case, there was no mapping on the other side of the tie-in boundary to tie-in to.  We felt this approach produced the cleanest mapping in these two areas.

Halff Backcheck 11/10/2023

- Tie in areas should be reviewed to ensure mapping tie ins are meeting FEMA's requirement of +/- 0.5 ft. If areas have been reviewed and confirmed but do not align in floodplain width to the adjacent study domain/HUC, please identify with AOMI_Ar for follow-up in future studies.

Atkins Response (11/27/2023): Polygons added back to feature class and rasters.  AOMI_Ar polygon has been added to indicate tie-in was not possible here since there was no flooding in the bordering domain.

Halff Backcheck 11/30/2023

  - S_AOMI_Ar polygons added to areas near border of tie-ins with adjacent HUCs correctly. No further comments.

- Issues with Tie-Ins to adjacent HUCs please review screenshots and QC polygons, these are just representative examples. Blue China is available on the BLE viewer to use as a reference to tie into. For other HUCs that aren’t available please create AOMI polygons.

Tie-In greater than 0.5ft/issues with mapping

Atkins Response (10/30/2023): We downloaded the Blue China data from the eBFE viewer. We also reached out to Jenna Rao (i.e., our client PM at the TWDB) to seek her feedback on how to approach this comment.

After our discussion with Jenna and our research into the Blue China modeling/mapping methdology, we have not revised our floodplain mapping to tie-in to the Blue China mapping because it would be challenging and time-consuming to do so, and the effort may not result in tie-ins that are within spec. The reasons why this is the case include:

- The BLE methodologies used in each watershed are different. The Blue China modeling applied an area-reduction factor to the rainfall totals. It also computed rainfall losses in HEC-HMS. The Southern Beaver modeling did not apply an area-reduction factor to the rainfall, and rainfall losses were computed in HEC-RAS. Plus, the Blue China HEC-RAS model and the Southern Beaver HEC-RAS model used different Manning’s n values. Because of these differences, there is no guarantee clean tie-ins could be developed.

- A spatial review of the data revealed that at most of the tie-in locations, the floodplain mapping in each HUC-8 watershed differed significantly from one another. For instance, widths were often very different. Furthermore, sometimes there was floodplain mapping along one watershed boundary but not the other. Fixing these tie-ins would require extensive re-work and may not lead to tie-ins that are within spec.

Instead of revising the mapping, we have added AOMI polygons along the border between the Southern Beaver and Blue China watersheds.

Halff Backcheck 11/10/2023

- Please ensure S_AOMI_Ar polygons are added to all areas along all adjacent HUC boundaries that tie-ins will not be done for due to the issues listed above.

Atkins Response (11/27/2023):  Polygons added along adjacent HUCs to indicate further study required for tie-in

Halff Backcheck 11/30/2023

  - S_AOMI_Ar polygons added to areas near border of tie-ins with adjacent HUCs correctly. No further comments.

Halff Backcheck 11/30/2023

  - Please review area below appears grid was removed and there is no 500 year FLD_HAZ_AR only 100yr.

Atkins Response (1/15/24):  Removed the 100yr polygon.

L Source Cit:  Updated the scale of vector sources to be 1:24,000.  REF1 is 1:100,000.  For BASE1, updated title from “TxDOT City Boundaries” to “TxDOT City and County Boundaries”.

Deleted the HUC8 FC from the EBFE dataset, as it is redundant of the S HUC AR in the Base dataset.

Political Area:  Features that intersect the effective floodplain have their ANI_TF as False.  All others remain as Unknown.

AOMI Area:  Clipped polygons that bordered the HUC, to only around where the floodplain abruptly exits the HUC.

Renumbered Fld Haz Ar and AOMI Ar unique IDs.

FRAC Area:  set Scenario ID to blank.  Added dam IDs.  No levees in HUC.

Fld Haz Ar:  Multipart to Singlepart, repaired geometry, pairwise integrate, ran topology.  Re-attributed Est_Risk to be the coded value words, instead of just letters M or H.

Ran topology for political areas, BFEs, water areas, water lines, 10%.

Tenpct FP:  Added domains for vertical datum, length unit, and Est_Risk.  Populated fields as NAVD88, Feet, and Extreme.  Also corrected one SFHA_TF to True.

Water Areas/Lines:  Any named feature is True for Shown_FIRM and Shown_Index.  Any NP name is False.

Updated 100yr grids for Depth and WSE.

BFEs:  Added ‘_BLE’ to Est_ID.

Object IDs in sequential and consecutive order.

No 500yr FLD_HAZ_AR and no grids for either event. If grids were removed due to shallow flooding please remove 100yr FLD_HAZ_AR.

Mapping Backcheck 2/29/2024

  - No further comments

Halff Backcheck 11/30/2023

  - Issues with tie-ins at model boundaries still occurring, please review screenshot below and QC polygons to update and assure the 0.5ft threshold is met. A full review of the HUC is required to ensure no more tie-in issues along other model boundaries.

Atkins Response (1/15/24):  This tie-in has been cleaned up. A global check was completed to confirm all other tie-ins are acceptable.

Mapping Backcheck 2/29/2024

  - Please review QC polygon and screenshot below issue still occurring along SBVR_702 and SBVR_703 WSE01PCT/DEP01PCT grids are higher than the WSE0_2PCT/DEP0_2PCT grid.

  - Additionally, a difference raster (0.2% WSE – 1% WSE) has been provided to identify these areas throughout the watershed. It appears in many instances that a hole exists in the raw grids and was filled with depths greater than 0.1 ft in both the 1% and 0.2% grids. Please review your process for filling these holes/meeting mapping tie ins to ensure 1% WSE is less than (or equal to if filling a hole) the 0.2% WSE.

Atkins Response  06/24/2024

- Grids have been reviewed and WSE01PCT/DEP01PCT are less than WSE0_2PCT/DEP0_2PCT grids.

- Tie in issue at the location identified have been resolved and a global review of the watershed confirms that no additional tie in issues are present.

Halff Backcheck 7/05/2024

- Mapping comment addressed, no further comments.

Halff Backcheck 7/05/2024

- Issue with BFEs, please review screenshots below and QC polygons. There are now rounded/closed loop BFE’s

Circular BFE’s mostly occurring in or near areas where holes were filled in Grids causing anomalies:

AtkinsRealis Response (08/08/2024) – BFE feature class has been reviewed. Circular BFE lines have been deleted.

Halff Backcheck 8/19/2024-

- BFE comments have been addressed and cleared.

# 8 – Completeness Review

## Technical Review (Reviewer to confirm with Pass/Fail)

| File Structure and Naming – Not Full Deliverable |  |
| --- | --- |
| FAIL | Deliverables provided in TWDB/FEMA required file folder structure and naming convention |
| FAIL | Data missing (if any) documented in report and ReadMe file within the required folder |
| Report– Not Included |  |
| FAIL | Technical 2D BLE report documents all assumptions, special considerations, etc. |
| FAIL | Flow validation and difference discussed in report |
| FAIL | Flow comparisons discussed in report |
| Hydrologic Model – Not Full Deliverable, Not Included |  |
| FAIL | HEC-HMS model with results, pertinent data and required storm events |
| Hydraulic Model– Not Full Deliverable, Not Included |  |
| FAIL | HEC-RAS model with results, pertinent data and required storm events |
| Flood Risk Database |  |
|  | All features fully populated per current FEMA guidelines |
| CNMS – Not Included |  |
| FAIL | Updated correctly with new centerlines |
| FAIL | A1-A5 checks complete |
| FAIL | CNMS Post- Processing Complete |
| HAZUS |  |
| PASS | Run per FEMA guidance |

## Review Comments

Deliverables:

- Please provide final Deliverables in the EBFE File Format.

- Please include report in next deliverable for review of mapping sections.

Backcheck 2/29/2024:

Please provide submittal in the EBFE format as provided by TWDB.

  - Reporting and Documentation

    - Final deliverable will include report (Word/PDF) and Hydraulics Metadata

  - Geospatial Data

    - Supplemental Folder should include CNMS and HAZUS. Hydraulics, Hydrology, and Terrain do not belong in this folder. If additional information is to be included here, please create an “Other” folder and save in there.

  - Engineering Models

    - This folder was not included in the deliverable. Engineering Models is required to have the following folders as shown below. A Supplemental folder may be included for additional supporting data (ex. Gage analysis, regression calculations, etc.). Within the Hydraulics Model Folder – the Inputs and Outputs may be placed in one folder as shown to prevent broken links in the model.

Atkins Response  06/24/2024

Comment 2.1: This submittal includes the Geospatial Data folder and the Report-Documentation folder. We will submit the full eBFE package for this watershed following M2 approval.

Comment 2.2: The Geospatial/Supplemental Data folder has been revised as requested. CNMS data will be added once CNMS has been completed.

Comment 2.3: We will submit the full eBFE package (included the Engineering Models folder) for this watershed following M2 approval.

Halff Backcheck 7/05/2024

- No further comments.

Flood Risk Database:

- S_Pol_Ar

  - SOURCE_CIT- References BASE1 this source is not in the L_Source_Cit table please update or add to table.

Atkins Response (10/30/2023): Added BASE1 Source Cite.

Halff Backcheck 11/10/2023

  - No Further Comments

- S_HUC_Ar

  - Currently DIGITS, CASE_NO, VERSION_ID, SOURCE_CIT empty or missing please update and add appropriate field information.

Atkins Response (10/30/2023): Updated the S_HUC_Ar attribute table.

Halff Backcheck 11/10/2023

  - No Further Comments

- WTR_LN

  - Geometry comment on streams, please verify streams meet 1 SQM for rural and 0.5 SQM for urban areas per Region/FEMA requirements.

  - Review QC polygons for stream centerline containment issues, etc.

Atkins Response (10/30/2023): WTR_Ln has been reviewed and all urban areas with a population greater than 1,000 have 0.5 sq mi lines (only the City of Crowell met the population criterion).

Halff Backcheck 11/10/2023

  - Please ensure updates made make sense for mapping. A buffer appears to have been added to the stream centerline for 1% containment, however, there is no 0.2% mapping. Ensure where there is 1% mapping, there should also be 02% mapping as well.

Atkins Response (11/27/2023): For areas where polygons buffered to ensure Wtr_Ln are contained within 100yr floodplain, the same was done on 0.2%.

0.2% floodplain buffered to contained Wtr_Ln

1% floodplain buffered to contained Wtr_Ln

Halff Backcheck 11/30/2023

  - No further comments.

- WTR_AR

  - No comments

- S_AOMI_AR

  - Currently empty, just verifying there are no areas such as land use changes, areas along adjacent HUC 8 boundaries or streamflow constrictions that should be included?

Atkins Response (10/30/2023): Updated S_AOMI_Ar.

Halff Backcheck 11/10/2023

    - Loaded feature class correctly, please also include S_AOMI_Ar polygons along the boundary for Pease to the North all other HUCs that Southern Beaver does not tie into due to availability on the BFE viewer or studies that were not completed by Atkins and readily available. Other features that should be added include stream flow constrictions since S_AOMI_Pts are no longer included as a deliverable. Stream flow constrictions should be created as points and buffered then loaded into S_AOMI_Ar.

Atkins Response (11/27/2023):  Added new polygons S_AOMI_Ar feature classes as requested.

Halff Backcheck 11/30/2023

  - Correctly added and attributed, No further comments.

- DTL_STUD_AR

  - No comments

- DTL_STUD_LN

  - No comments

- BFE_2D

  - No comments

- S_FRAC_Ar

  - Missing/empty please populate with Hazus results

Atkins Response (10/30/2023): Added Hazus results to S_FRAC_AR.

Halff Backcheck 11/10/2023

- Loaded feature class correctly, no further comments.

- CNMS Feature Classes

  - S_Studies_Ln and S_Unmapped_Ln- Missing/empty coordinate with Halff and TWDB for CNMS submittal on a later date.

Atkins Response (10/30/2023):  The TWDB has directed us to submit our CNMS updates to Michael Johnson for review instead of the ITR team. Please close this comment.

Halff Backcheck 11/10/2023

    - No Further Comments

- TENPCT_FP

  - No comments on Attribute Table

- FLD_HAZ_AR

  - No comments on Attribute Table

- L_Source_Cit

  - Sources mentioned in other features such as BASE1 are not in the L_Source_Cit table please review all sources mentioned throughout other feature classes and those used for data and add to table.

Atkins Response (10/30/2023): Added BASE1 to L_Source_Cit table.

Halff Backcheck 11/10/2023

    - No Further Comments

Halff Backcheck 8/19/2024-

    - L_Source_Cit table currently has duplicate sources – STUDY1, BASE1, BASE2 are being used for different references. Source citations must be unique. For example BASE1 is referenced as Texas Statewide National Flood Hazard Layer Publisher FEMA and then referenced again as TxDOT City and County boundaries Publisher TxDOT:

Atkins Response (10/23/2024): L_Source_Cit was updated to remove the entries inadvertainly included from the template geodatabase

Halff Backcheck 1/2/2025 – Addressed, no further comments

QA/QC Checklist Approval

This 2D Base Level Engineering Analysis QA/QC Review is in compliance with the contract requirements and all task checkpoints are complete.  The independent QC Team has reviewed the analysis, presented review comments to the Production Team Task Leader, and discussed any problems or issues.  The Task Leader, QC Manager, QC Reviewer, and Project Manager have signed the QA/QC Checklist to confirm that all comments have been received, addressed, and documented appropriately.

| QA/QC Team – [TWDB or company name] |  |  |  | Production Team – AtkinsRealis |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Shrinivas Kaulgud, Rachel Forster |  | 4/8/25 |
| QC Manager |  | Date |  | Task Lead |  | Date |
|  |  |  |  | Chris Wright |  | 4/8/25 |
| QC Reviewer |  | Date |  | Project Manager |  | Date |