# Texas 2D BLE Analysis QA/QC Checklist

# Project Information

| HUC-8 Watershed Name & ID: | Little Wichita: 11130209 |
| --- | --- |
| HUC-10 Watershed Name & ID: | North Fork Little Wichita River – Lake Kickapoo (1113020901) -LWIC_901 South Fork Little Wichita River – Little Wichita River (1113020902) -LWIC_902 Lake Arrowhead – Little Wichita River (1113020903) -LWIC_903 East Fork - Little Wichita River (1113020904) -LWIC_904 Dry Fork Little Wichita River – Little Wichita River (1113020905) -LWIC_905 |
| Modeler: | Watearth/Atkins |
| Reviewer: | Halff Associates |

## Terrain and Modeling Review (QC Items 1-6)

| Modeler Initials: | SGK/FA/CQ/RK/SD | Reviewer Initials: | NSR |
| --- | --- | --- | --- |
| Submittal Date: | August 25, 2023 | Review Submitted Date: | 6/19/23 |
| Review Response Date: |  | Backcheck Complete Date: | 7/27/23 |

## Mapping Review (QC Item 7)

| Modeler Initials: | SGK/FA/CQ/RK/SD | Reviewer Initials: | LH |
| --- | --- | --- | --- |
| Submittal Date: | August 31, 2023 | Review Submitted Date: | 10/30/23 |
| Review Response Date: |  | Backcheck Complete Date: |  |

## Completeness Review (QC Item 8) – not included in this submittal

| Modeler Initials: |  | Reviewer Initials: |  |
| --- | --- | --- | --- |
| Submittal Date: |  | Review Submitted Date: |  |
| Review Response Date: |  | Backcheck Complete Date: |  |

(Modeler to populate with an “X” for items included in the QC submittal)

| Level of Study |  |  | Flood Events |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| X | 2D BLE |  | X | 10% ACE (10-year) | X | 0.2% ACE (500-year) |
|  | Enhanced BLE |  | X | 4% ACE (25-year) | X | 1% plus |
|  |  |  | X | 2% ACE (50-year) | X | 1% minus |
|  |  |  | X | 1% ACE (100-year) |  | Observed Rainfall |

Instructions

Each section includes instructions to the modeler and reviewer regarding their responsibilities in the completion of this checklist. In the Methodologies / Data Sources tables, grey italic text is added as an example response. Additional sample responses are located in the Texas 2D BLE Guidelines Quick Guide.

Reviewer to indicate if Technical Review items generally comply using a “Pass”. If the technical review observes an issue of noncompliance or an item that requires additional attention, an “Fail” should be utilized.

## Submittal Checklist

Data submitted for each milestone should be carried through each subsequent milestone. All geospatial layers and models must have same defined projection system as terrain. Modeler to populate with an “X” for items included in the QC submittal.

| Base Map Data (QC items 1-6) |  |
| --- | --- |
| X | Location Map (PDF): HUC-8 and HUC-10 boundaries and/or model domain boundary if different from HUC-10s, stream gages, dams (if applicable), mainstem streams (for reference), background map |
| X (RAS Mapper) | GIS Map (MXD or Map Package) with the following data included but not limited to: Stream centerlines, model breaklines, rainfall data, WSEL and Depth grids (10, 100, 500-yr), adjacent HUC mapping (available on the BLE viewer or currently studying), 2D connections for inflows/outflows, gages or points of flow comparisons (BFEs, highwater marks, etc.), profile lines, boundary condition lines, refinement regions, model domain boundary, model mesh, DFIRM mapping panels, base map data (roads, aerials, etc.), soils data, land use data, terrain |
| Technical Report / Documentation (QC items 1-6) |  |
| X | Technical Report / Project Notebook: assumptions, special considerations, and modeling concerns |
| X | Detailed explanation and support data of variances in methodologies and approaches from recommended guidelines (Should be included in Technical Report / Project Notebook) |
| X | Internal QA/QC Documentation: comments and responses |
| X | Flow comparison tables within the technical report |
| Hydrology (QC items 1-6) |  |
| X | Hydrologic Model (HEC-HMS) model with all associated DSS files for 7 required storm events |
| X | Hydrologic Data and Parameter Spreadsheets (rainfall calculations, curve number table, gage locations, gage analysis and flow validation) |
| X (Report/ Results spreadsheet) | Hydrologic modeling notebook including assumptions, notes, and special conditions (if not provided in report) |
| Hydraulics (QC items 1-6) |  |
| X | Hydraulic Model (HEC-RAS) with all associated files and 7 required storm events (remove non-final files) |
| X | Hydraulic Data and Parameter Spreadsheets (Manning’s n-value table, infiltration calculations support data-Soils, Land Use, Curve Number table, other calculations/data used in model) |
| X | Manning’s n-value table |
| X | Precipitation data correctly linked to hydrologic model or hard coded from provided spreadsheet |
| X | Hydraulic modeling notebook including assumptions, notes, and special conditions |
| Mapping (QC item 7) |  |
| X | Flood risk database |
| X | Modeler to complete topology validation as outlines in the guidelines |
| X | Modeler to verify mapping meets deliverable requirements as outlined in guidelines |
| Completeness Review (QC item 8) See Texas 2D BLE Guidelines Chapter 4 – Deliverables |  |
|  | Technical BLE Report |
|  | Final cleaned hydrologic model |
|  | Final cleaned hydraulic model |
|  | Flood risk database |
|  | CMNS updates |
|  | HAZUS results |
|  | Additional supplemental data |

# 1 – Model Terrain

Methodologies / Data Sources (Modeler to populate)

| Terrain Source(s): | Please see technical memo for more detail: Texas West Central LiDAR (2018), Brown County LiDAR (2019), Hurricane LiDAR (2019), Lower Colorado San Bernard LiDAR (2018), and Lampasas LiDAR (2014) |
| --- | --- |
| Data Gaps: | None |
| Source Resolution: | Cell size of 1 x 1 meter |
| Model Terrain Resolution: | Cell size of 5 x 5 feet |
| Terrain Horizontal Projection: | North American Datum (NAD) 1983 (2011) Texas State Plane Zone 3 (Central) |
| Terrain Vertical Projection: | NAVD88 |
| Z Values: | Units of feet |
| Terrain Buffer: | 2,500 feet |
| Other: |  |

## Technical Review (Reviewer to confirm with Pass/Fail)

| General Review |  |
| --- | --- |
| P | LiDAR source matches the latest available data at time of project initiation |
| P | Large reservoir or lake extents included in model domain |
| F | Check visible seams, gaps or holes of LiDAR sources (if more than one source was used) |

## Review Comments

- BLE Report has conflicting information regarding the terrain sources. There are two sources listed in the writeup, but the figure shows 4 sources. The descriptions for the two sources do no reference counties within this study region. Please review the topographic data sources section and update.

Atkins response (07/18/23): Atkins will revise and clarify the sources in report accordingly.

Halff Backcheck: Unresolved

Atkins Response (08/23/2023): Atkins has updated the report to provide information on the 4 terrain data sources.

- There is a terrain bust in model 903. This is likely due to the change in terrain source. Please provide documentation on this bust in the terrain.

Atkins response (07/18/23): Atkins will provide supporting documentation. 
Halff Backcheck: Unresolved

Atkins Response (08/23/2023): This area is located in the backwater of Lake Arrowhead, where two LIDAR sources meet. We have set the initial lake elevation for the lake equal to the conservation pool elevation, which is higher than the water surface elevation in both LiDAR datasets. Therefore, this discontinuity does not adversely affect BLE outcomes. This issue is documented in the report in Subsection 1.1.1 and in the Challenges section.

# 2 – Hydrology

## Methodologies / Data Sources (Modeler to populate)

| Software: | HEC-RAS 6.3.1 |
| --- | --- |
| Rainfall Source: | NOAA Atlas 14 |
| Rainfall Duration: | 24 hours |
| Depth Selection: | Domain Average |
| Rainfall Temporal Distribution: | HEC-HMS |
| Areal Reduction: | None |
| Loss Methodology: | Spatially varied CN |
| Unit Hydrograph Transform Method: | N/A |
| Inflow Hydrograph Source: |  |
| Other: Precipitation spreadsheets are included with the submittal |  |

## Statistical Data (Modeler to populate)

| Stream Gage Name & ID: | N Fk Little Wichita Rv Trib nr Archer City, TX (07314200) |
| --- | --- |
| Stream Gage Name & ID: | Little Wichita Rv nr Archer City, TX (07314500) |
| Stream Gage Name & ID: | Little Wichita Rv nr Henrietta, TX (07314900) |
| Stream Gage Name & ID: | E Fk Little Wichita Rv nr Henrietta, TX (07315200) |
| Stream Gage Name & ID: | Little Wichita Rv nr Ringgold, TX (07315400) |

## Technical Review (Reviewer to confirm with Pass/Fail)

| Excess Rainfall |  |
| --- | --- |
| P | Precipitation values are an aerial average of Atlas 14 values for the entire 2D area boundary for each storm event or other appropriate rainfall application |
| F | CN values appropriately reflect land use and soils for entire 2D area boundary |
| F | Losses are correctly calculated to establish the excess rainfall hyetographs by checking land use, soils and infiltration layers or other relative information |
| P | Total precipitation depths for 1%+ and 1%- are calculated correctly |
| P | Excess rainfall hyetographs are calculated for all storm events (10%, 4%, 2%, 1%, 0.2%, 1%+, and 1%-) |
| P | Cumulative excess rainfall values are in increasing order from 10%, 4%, 2%, 1%, to 0.2% |
| P | Cumulative excess rainfall values for the 1%+ and 1%- fall in order with the storm events matching the total precipitation values |
| Inflow Hydrographs |  |
| NA | Shape and time-to-peak of any inflow hydrographs resemble past documented large storm events |
| P | Hydrographs reasonable when compared to the other return periods. The shapes should be similar, and hydrographs from different events should not intersect unless there is an engineering reason. Peak flows from a lower return period event should not be higher than a higher return period event |
| F | Contributing hydrographs from another RAS model applied at coincident locations (Provide locations in base map data) |
| General Hydrology |  |
| Y | Any flow regulating dams in the model area? (Yes/No) |
| F | Dam modeling methodologies applied and explained? (Yes/No). If yes, provide discharge rating curve calculations and gage analysis documenting regulated versus unregulated records, if applicable. |
| F | Model aligns with documentation of dam modeling approaches. |
| Other Notes: NA |  |

## Review Comments

- Please add documentation to the BLE report concerning the Initial elevations of Kikapoo Pool and the downstream point Downstream 902. There is not documentation on these values.

Atkins response (07/18/23):  – Atkins will add information on the initial elevations of reservoirs.
Halff Backcheck: Unresolved

Atkins Response (08/23/2023): Via an online search we have obtained limited reservoir operation information from the City of Wichita Falls and the TWDB. The report has been updated. See Subsection 1.2.5 and References Section.

- Please add documentation to the BLE report on the use of initial elevations points at the lake/dam in model 903.

Atkins response (07/18/23):  Atkins will add information on the initial elevations of reservoirs
Halff Backcheck: Unresolved

Atkins Response (08/23/2023): Via an online search we have obtained limited reservoir operation information from the City of Wichita Falls and the TWDB. The report has been updated. See Subsection 1.2.5 and References Section.

- It was noted that the Kikapoo pool elevation for the 100-yr event is 1045.318 while the elevation is 1045 for all other storm events. Please review and update. If no change is made, provide documentation explaining this.

Atkins response (07/18/23):  Atkins will investigate the source of conservation pool elevation and make adjustments if a different reference was used. 
Halff Backcheck: Unresolved

Atkins Response (08/23/2023): The initial elevation at Kickapoo pool is 1044.8ft and it has been applied to all storm events. The report has been updated. See Subsection 1.2.5.

- The initial elevation at Arrowhead pool is greater for the 100-yr event than all other events. Please review and update. If no change is made, provide documentation explaining this.

Atkins response (07/18/23):  Atkins will review and update

Halff Backcheck: Item Resolved

- There is overlap between model domains of 901/902; however, the Initial Elevation points from 901 model are not in the corresponding locations of 902 model. Please review the use of the Downstream point in 901 but not 902 or provide documentation as to why the decisions were made.

Atkins response (07/18/23):  The stage hydrograph was not reflecting the pool elevation and was showing a drop at the beginning of the model and oscillations in the flow hydrograph; however, after removing the initial conditions point the flow hydrograph at the spillway is consistent with the outflow from upstream model at the same location.

Half Backcheck: Item Resolved

- Comment 5 can be applied to the overlap between 903 and 905 and use of initial elevation points in 903 but not in 905.

Atkins response (07/18/23):  See response in 5.

Half Backcheck: Item Resolved

# 3 – Hydraulic Geometry

## Methodologies / Data Sources (Modeler to populate)

| Software: | HEC-RAS 6.3.1 |
| --- | --- |
| Loss Methodology: | Spatially varied CN |
| Soils Source: | SSURGO |
| Land Cover Source: | NLCD 2019 |
| Manning's Roughness Source: | Assigned based on land use. Please see table in Southern Beaver Model Overview and Results.xlxs |
| Nominal Mesh Cell Sizes: | 200 x200 except for channel refinement regions, breaklines and urban regions, where 100 ft x100 ft was considered. |
| Other: |  |

## Technical Review (Reviewer to confirm with Pass/Fail)

| 2D Mesh |  |
| --- | --- |
| P | 2D Boundary delineated along watershed boundary or buffer to ensure all contributing area is included |
| P | Nominal cell size appropriate for model detail (generally 200 feet, minimum 50 feet for refinement areas) |
| P | Computation points are at default locations or changes documented |
| P | Mesh Cell and Face tolerances are default or changes documented |
| P | No mesh errors |
| P | Cell face relationships are calculated |
| Breaklines |  |
| F | Breaklines are consistent with the terrain data |
| P | Breaklines sufficiently detailed to capture features (thalweg, banks, berms, etc.) |
| F | Cells do not overlap terrain features where appropriate (i.e., unwanted leaking) |
| P | Breakline cell sizing and parameters are appropriate |
| F | 2D Area Connections placed at applicable benchmarks, gages, or dams |
| P | 2D Area Connections set to use normal 2D equation domain for benchmarks and gages. Appropriate modeling approach used for dams. |
| Manning’s N-Values |  |
| P | Manning’s n-values associated to land cover data provided |
| F | Land Cover classifications are sufficiently descriptive within the data table |
| P | Manning’s n-value classifications generally match current aerial imagery |
| P | Manning’s n dataset is associated with the geometry files |
| General |  |
| P | All model datasets each have unique descriptive names |

## Review Comments

- General: See QC Shapefile LWIC_QC_Comments.shp. Specific domain name is included in comments.

  - Comments regarding breakline alignment, additional terrain mods

Atkins response (07/18/23):  Thank you for providing the shapefile, all comments in the shapefile are responded and recorded.

Halff Backcheck: Items Resolved

- LWIC_901: Enforce 2D Area Connection LWIC_901_PT1, PT2 and PT3.

Atkins response (07/18/23):  2D area is enforced.

Halff Backcheck: Item not resolved in model provided. PTs 1 and 2 removed from model. PT3 not enforced.

Atkins Response (08/23/2023): SA2D connections have been enforced. See screenshots below of PT1, PT2 and PT3.

- Enforce 2D Area Connection Outflow_LWI_901

Atkins response (07/18/23):  Atkins has enforced the 2D area connection, see figure in comment 4.

Halff Backcheck: Item not resolved in model provided.

Atkins Response (08/23/2023): SA2D connections have been enforced. See screenshots below of the Outfall for 901.

- Correct breaklines Spillway 3 and Breakline 322 to have a single streamline

Atkins response (07/18/23): Revised

Halff Backcheck: Item Resolved

Updated model conditions.

- General Comment for all models: There are three land cover layers in RASMapper. LandCover (1) is empty, Mannings (2) has the Manning’s n values used and LandCover (3) is empty. Remove any layers not needed or empty.

Atkins Response (7/18/2023): Deleted the unused/not needed layers.
Halff Backcheck: Item Resolved

- Consider adjusting Refinement Region around lake surface to better reflect the lake surface per underlying terrain.

Atkins response (07/18/23): Revised

Halff Backcheck: Item Resolved

- LWIC_902: 2D Connections LWIC_902_PT2 and Outflow_LWIC_902 are duplicates. Remove one connection.

Atkins response (07/18/23): One of the validation locations is the outflow location, Atkins will delete the connection LWIC_902_PT2

Halff Backcheck: Item Resolved

- Enforce 2D Connections as breaklines in model. Applies to all 2D Connections in 902.

Atkins response (07/18/23): updated

Halff Backcheck: Item Resolved

- Please add documentation to the use of smaller than 100 foot cells along breaklines to the BLE Report. Noted that 50 foot was used in the model 902.

Atkins response (07/18/23): Atkins applied the 100-foot cells; smaller cell size is not used.
Halff Backcheck: Item Resolved

- Several breaklines that are short and are possibly very small tributaries within a wide floodplain. Suggest removing these.

Atkins response (07/18/23): Removed
Halff Backcheck: Item Resolved

- Correct these breaklines through this section as noted in the QC Shapefile.

Atkins response (07/18/23): Model is updated

Halff Backcheck: Item Resolved

- LWIC_903: There should not be both a breakline and 2D Connection at the dam. Please remove the breakline and review the 2D connection to ensure that it runs across the top of embankment and the line is enforced.

Atkins response (07/18/23): Kept the 2D connection, and removed the breakline

Halff Backcheck: Item Resolved

- Enforce all 2D connections as breaklines.

Atkins response (07/18/23): Enforced the 2D connection as breakline

Half Backcheck: Item Not Resolved. Enforce LWIC_903_PT1 and Outflow_LWIC_903

Atkins Response (08/23/2023): SA2D connections have been enforced. See screenshots below of PT1 and the Outflow of 903.

- Adjust 2D Line for LWIC_903_PT2 so that it cuts across the channel perpendicular.

Atkins response (07/18/23): – Atkins will revise the SA2D line to make more perpendicular to the overland flow. The team used the vector velocity an WSE to trace the 2D line

Half Backcheck: Item resolved

- Would recommend using a refinement region for the entire spillway and not just a small buffered portion of the breakline. Also would draw breakline in center of spillway.

Atkins response (07/18/23): Model is updated

Halff Backcheck: Item Resolved

- Consider shortening the line for the 2D Connection LWIC_903_PT2 in right overbank.

Atkins response (07/18/23): shortened the 2D connection line

Halff Backcheck: Item Resolved

- 903 and 904: Curve Number values in Infiltration layer do not correspond with TWDB values.

Atkins response (07/18/23):  CN values are following the TWDB guidelines, but were revised (adjusted between AMCI and AMCII values) during validation and documented in the report

Halff Backcheck: Item Resolved

- LWIC_904: 2D Connection LWIC_904_PT2 and a breakline are duplicates. Remove the breakline, ensure 2D connection is drawn on high point of roadway and enforce.

Atkins response (07/18/23):  One of the validation points is the outflow, duplicate one (LWIC_904_PT2) is deleted

Halff Backcheck: 2D Connection was removed. Item resolved

- Enforce 2D Connection LWIC_904_PT3 as if it were a breakline.

Atkins response (07/18/23):  2D connection line is enforced

Halff Backcheck: PT3 is now named PT1. Line is enforced. Item Resolved.

- There are two 2D Connections at the outflow, LWIC_901_PT1 and Outflow_LWIC_904. Remove one of these. There should not be duplicate 2D connections.

Atkins response (07/18/23): One of the validation points is the outflow, Atkins will delete the duplicate one (LWIC_901_PT1)

Halff Backcheck: Item Resolved

- LWIC_905: Enforce 2D Connections as breaklines.

Atkins response (07/18/23):  enforced

Halff Backcheck: Item Not resolved. LWIC_905_PT3 is not enforced.

Atkins Response (08/23/2023): SA2D connections have been enforced. See screenshot below of PT3 in 905.

# 4 – Hydraulic Boundary Conditions

## Technical Review (Reviewer to confirm with Pass/Fail)

| Boundary Review |  |
| --- | --- |
| P | External boundaries connected to 2D mesh |
| P | Boundary conditions everywhere flow could build up at the boundary of the 2D mesh |
| P | Boundary conditions have a hydrograph or elevation/slope data assigned |
| P | Input hydrographs consistent with hydrologic data (DSS, manual entry) |
| P | Precipitation data is consistent with hydrologic data (DSS, manual entry) |
| F | Downstream boundary consistent with ground slope or known downstream WSEL |
| P | Flow hydrographs applied at appropriate locations |
| NA | Energy grade slope for hydrograph distribution is reasonable |
| NA | Internal rating curves appropriate and documented for control structures (if applicable) |

## Review Comments

- General comment concerning use of 2D Connections and Boundary Condition Lines. While the 2D Connections (Outflows) and corresponding BC Lines (inflows) are at the same location, they are drawn differently, thus the inflow may not be applied in the same way the outflow is applied. Please review all corresponding connections to ensure that they match. See example below from 902/903 where the red line is the 2D Connection from 902 and the blue line is the BC Line from 903.

Atkins response (07/18/23):  The SA2D line only covers the width of the river so the flow in the overbanks is not accounted during tie in process.  Atkins has applied this principle in prior models giving appropriate outcomes at the 2D location.

Halff Backcheck: Item Resolved

- LWIC_901: Consider straightening/smoothing out the downstream mesh perimeter and boundary condition line.

Atkins response (07/18/23):  Smoothed/straightened the mesh perimeter and boundary condition
Halff Backcheck: Item Resolved

- LWIC_902: Consider adjusting model perimeter so that the BC line for inflow from 901 is applied at the perimeter/outside of the model instead of inside the mesh.

Atkins response (07/18/23):   We are using internal boundary conditions to transfer flow, so we will not be making changes to change the BC lines

Halff Backcheck: Item Resolved

- Adjust model perimeter and BC line of outlet so that is more perpendicular to channel.

Atkins response (07/18/23):  Adjusted the BC line to make it more perpendicular

Halff Backcheck: Item resolved

- General comment: The normal depth slope for all downstream outlets is set to 0.002. A spot check of outlet stream channels shows that the channel slope at outlets varies. Please review normal depth slopes at outlets and update.

Atkins response (07/18/23):  the tie ins for this watershed are completed using stage hydrographs and slopes are no longer used.

Halff Backcheck: Item Resolved

- LWIC_903: Review Normal Depth slope for boundary conditions line LWIC_903_BC1.

Atkins response (07/18/23): –the tie ins for this watershed are completed using stage hydrpgraphs. 
Halff Backcheck: Item Resolved

- Shorten BC line for Outflow_LWIC_903_BC_STG.

Atkins response (07/18/23):  updated the model at that section.

Halff Backcheck: Item Resolved

- A boundary condition line should be applied at the upstream end of the model extents. Currently water is ponding and may impact results at the Inflow hydrograph that is immediately downstream. Other alternative would be to change mesh so that the inflow BC line is applied on the exterior.

Atkins response (07/18/23):  We are not considering changing to external boundary conditions. We did not apply upstream boundary conditions to make sure that no flow is lost through the upstream boundary condition.

Halff Backcheck: Item Resolved

- LWIC_905: It is recommended that the inflow hydrograph boundary condition be applied as an external bc line instead of internal. Internal BC lines may have unintended effects to the flows.

Atkins response (07/18/23): – We are not considering changing the internal boundary conditions to external ones. We will make sure there will be no loss of flow from upstream boundaries.

Half Backcheck: Item Resolved

- Consider extending the downstream mesh extent and boundary condition line further downstream into the main stream floodplain. Currently, the mesh is not fully perpendicular to the floodplain and is sharply cutting off the flows. The Velocity Static Vector and Particle Tracing features are good tools for helping to visualize the flowpaths.

Atkins response (07/18/23):  extended the model domain downstream as specified.

Half Backcheck: Item Resolved

- LWIC_904 and 905 New comment added to a new QC Shapefile LWIC_QC_Backcheck.shp. Noted an embankment that does not have a breakline. They are significant embankments and should have an enforced breakline.

Atkins Response (08/23/2023): QC shapefile comments have been addressed and responses have been added to the attribute table.

# 5 – Hydraulic Model Execution

## Methodologies / Data Sources (Modeler to populate)

| Unsteady Plan Names |  |  |  |
| --- | --- | --- | --- |
| 10% ACE (10-year): | 10pct | 0.2% ACE (500-year): | 0.2pct |
| 4% ACE (25-year): | 4pct | 1% plus: | 1plu |
| 2% ACE (50-year): | 2pct | 1% minus: | 1min |
| 1% ACE (100-year): | 1pct |  |  |
| Other: |  |  |  |

## Statistical Data (Modeler to populate)

| Time Step: | Advanced time step based on Courant condition (7.5-60 Seconds) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Courant Value: | Min: 0.45 | Max: 2 | Simulation Length: | 96 | hours |
| Computation Interval: | 30 | seconds | Model Runtime: | 1-8 | hours |
| Hydrograph Output Interval: | 10 | minutes | Mapping Output Interval: | 60 | minutes |

## Technical Review (Reviewer to confirm with Pass/Fail)

| Execution Review |  |
| --- | --- |
| P | Appropriate 2D parameters are used (Boundary Conditions, Time Step, Simulation Length, etc.) |
| P | Hydrograph Output Interval and Mapping Output Interval set at a reasonable interval to capture peaks |
| P | Model runtime is reasonable (< 12 hours, but dependent upon model size) |

## Review Comments

- No comments

# 6 – Hydraulic Model Results

## Methodologies / Data Sources (Modeler to populate)

| Flow Validation/Gage Analysis |  |
| --- | --- |
| Software: | N/A |
| Methodology | Regression flows were used in the absence of gage data and FIS flows; Draft InFIRM report was used to develop the inflow hydrograph for O.H. Ivie reservoir, pool frequency information obtained from RFA report is leveraged for reservoir modeling. Refer to the BLE report and the Model Overview and Results Spreadsheet. |
| Removal of Outliers and Justification | N/A |
| Validation Notes (describe parameter adjustments): Refer to the BLE Report and the MidCo Model Overview and Results.xlsx for more details of validation |  |
| Other: |  |

## Technical Review (Reviewer to confirm with Pass/Fail)

| Computation Checks |  |
| --- | --- |
|  | Model runs with no cell errors |
|  | Volume balance is within tolerance (ΔV < 1% is ideal, however 2% to 3% can be acceptable for rain on mesh) |
| Results Spot Checks Check at all critical structures, major confluences, a handful of headwater tributaries, downstream end, at benchmarks, and a handful of randomly selected locations. |  |
| P | No visible spikes in WSEL visibly present with a single cell |
| F | WSEL and depth grids appear reasonable (no artificial dips or spikes) |
| P | Visual verification flooding is connected downstream for all significant contributing areas (> 64 acres) |
| P | Velocity grids appear reasonable (no unusually high velocity areas or Courant values greater than 2) |
| F | No crossing profiles between all events |
| F | Inflow and outflow hydrograph timing is reasonable as compared to the peak |
| P | Spills into/from adjacent HUCs appropriately accounted for |
| F | WSEL tie-in between model domains and HUCs along major rivers/streams where inflows are applied |
| Results Full Checks |  |
| P | Computational time step meets ideal Courant condition |
| P | Diffusion Wave: V∆T∆X ≤2 (Up to 5) (Results are unit-less) |
| F | Model simulation is long enough to pass the entire hydrograph through the model (minimum of 12 hours past the peak, preferably 24 hours) |
| P | Hydrographs at internal and external boundaries are smooth |
| P | Contributing areas are contained with 2D mesh or accounted for with inflow hydrographs |
| F | No flow trapped at 2D boundary |
| NA | Visual verification mapping is reasonable compared to effective |
| Flow Verification |  |
| P | Peak flow gage analysis performed on all gages in the watershed with sufficient data |
| P | All gages used have an appropriate period of record (10 to 20+ years) |
| P | All gages used have no extensive gaps in record which would impact the analysis |
| P | Gage analysis results validated to other sources (Regression, USGS, etc.) |
| P | Results at applicable benchmarks justified (gages, HWMs, effective BFEs) |
| P | Peak discharges comparable to gages, effective/prior studies, regional regression equations |

## Review Comments

- LWIC_901: Noted Courant values greater than 5 at the spillway of the lake.

Atkins response (07/18/23):  Highest velocity are being develop at the spillway, however, the face Courant value is less than 5.
Halff Backcheck: Item Resolved

- Missing rasters. Please include depth and wse rasters for all events with backcheck.

Atkins response (07/18/23):  Added these in the backcheck submittal

  - 100-YR: LWIC_901

  - 500-YR: LWIC 902, 903 and 904

Halff Backcheck: Item Resolved

- At LWIC_901_PT1 and Outflow_LWI_901 the 100-yr elevation  is higher than the 500-yr elevation. This is likely due to the difference in starting elevation for Kikapoo Lake. Please review and update.

Atkins response (07/18/23): The statement is correct. After setting the conservation pool, current hydrographs are accurate.

Halff Backcheck: Item Resolved

- Stage and elevation at the downstream boundary condition Outflow_LWIC_901_BC_STG is higher for the 100-yr than the 500-yr at the beginning of the simulation. Please review and update.

Atkins response (07/18/23): Updated stage hydrographs are consistent

Halff Backcheck: Item Resolved

- There is some instability in the falling limb of BC3. Noted, but no changes necessary.

Atkins response (07/18/23):  Noted, thanks.

Halff Backcheck: Item Resolved

- General Comment: Spot check of 100yr results.

  - There is no overlap on edges of 901/902 resulting in a bust of WSE. This is located at the far upstream point of 902 and north side of 901.

Atkins response (07/18/23):  The model is updated  at that area, so the overlap is smoother.

  - There is a mapping bust at the interface between 903 and 905. Do you plan to map 903 to the dam and spillway and then immediately begin 905 mapping? The wse is drastically difference across both models for the same locations.

Atkins response (07/18/23):  After tie ins are completed, we are observing a good agreement in the tie-in location at the overlap area between 903 and 905.

Halff Backcheck: 6a and 6b resolved.

- LWIC_902: Inflow hydrograph from Inflow_From_LWICRv_901 is showing the 100yr stage as higher than the 500yr stage.

Atkins response (07/18/23): The updated inflow hydrographs for LWIC are more consistent with higher inflows.

Halff Backcheck: Item Resolved

- LWIC_903: At 2D Connection Outflow_LWIC_903, the 100yr and 500yr TW stages cross. Please review and correct. –

Atkins response (07/18/23): The hydrographs do not intercept in the updated model.

Halff Backcheck: Item Not Resolved. There is still a crossing water surface elevation at the Outflow. 


Atkins Response (08/23/2023): The model has been updated. The revised results do not have crossing profiles in the Stage hydrographs for the SA2D Outflow of 903. Screenshot below aids review.

- Flows do not peak at the 2D Connection for Outflow_lwic_903 line. Please review this outlet and the simulation time.

Atkins response (07/18/23): Simulation time is extended

Half Backcheck: Item not resolved. Simulation Time extended only for 1pct.

Atkins Response (08/23/2023): Simulation time was extended 1-day for all storm events. The flow hydrograph of the 903 outflow is shown in the screenshot below to aid the review.

- Flows do not peak at BC Line Outflow_LWIC_903_BC_STG. –

Atkins response (07/18/23): Simulation time is extended

Half Backcheck: Item not resolved. Simulation Time extended only for 1pct.

Atkins Response (08/23/2023): The simulation run was extended by 1 day. Revisions were made to the stage hydrographs in LWIC_903 model based on the updated LWIC_905 model results. Stage and flow hydrographs for the downstream stage boundary in LWIC_903 are shown below.

- *New Comment* LWIC_902, 903: Model simulation time for 1pct event is longer than the other events. 1pct has end time of 07JAN while all other events have simulation end time of 04JAN. Please update events to be consistent in runtime unless there is a need to extend or shorten. If there is a situation which requires variability in the simulation run time, please add an explanation to the BLE Report. Screenshot below from Model 903

Atkins Response (08/23/2023): All events for the LWIC_903 model have been extended uniformly and have a simulation end time of 05JAN 24:00. The screenshot below of the flow hydrograph outflow aids the review.

- LWIC_903. Review Boundary Condition Outflow_LWIC_903_BC_STG. There is negative flow at this BC line. This is likely being caused by the stage hydrograph in the spillway.

Atkins Response (08/23/2023): The ground elevations downstream of the dam spillway outlet are higher than the channel elevations in the spillway. See the screenshot below. This reverse gradient causes negative flows in the flow hydrograph. This issue has been documented in the report in the reservoir modeling section (1.2.5) and the Challenges section.

- Spot check of 1pct WSE Grids.

  - Bust in overlap between LWIC 901 and 902. Screen shot black is 901 wse grid and blue is 902 wse grid. The lake wse is 1ft greater in the 901 results.

Atkins Response (08/23/2023): The models have been updated to tie in within 0.5’. Please see the screenshot below from the revised model results for the WSE tie-in area for LWIC_901 with LWIC_902. The first screenshot shows the 100-YR FP comparison and the second screenshot shows the WSE differences between the two domains.

- LWIC_905: Review BC Line: Inflow From LWICRV 903 Stage and Flow Hydrograph. The stage for the 50yr is higher than all events. There appears to be some issue with instability in the 500yr.

Atkins Response (08/23/2023): The screenshot below shows the updated model results for the Inflow from the LWIC_903 stage hydrograph.

# 7 – Mapping Results

## Technical Review (Reviewer to confirm with Pass/Fail)

| Hydraulic Model |  |
| --- | --- |
| NA | HEC-RAS model with results, pertinent data and required storm events |
| Flood Risk Database |  |
| F | All features fully populated per current FEMA guidelines |
| P | Mapping consistent between grids to polygons for all three required mapped events |
| F | 10yr smaller than 100yr smaller than 500yr |
| P | Holes filled in polygon, depth, and WSE grids |
| F | Boundary interfaces align matching or identified as AOMI |

## Review Comments

TWDB (3/11/24)

On this backcheck I was able to properly review tie-ins and determined that the raw rasters need to be adjusted. There are tie-in errors as well as areas where the raw rasters extend beyond the final rasters in locations that are not explained by the pond and hole cleanup process accomplished by the matrix. Because of this, I did not completely re-review the HUC, as any layers derived from the rasters will need to be redone. I sent the following comments to Seth Ahrens via email on 3/21/24:

Hello again Seth,

See the link below for 4 shapefiles (2 for the 1% and 2 for the 0.2%) with locations for 1) tie-in errors and 2) areas where the raw rasters overshoot the final rasters.

In your previous email, you mentioned that tie-ins were only done where the domains overlap. As far as I am aware, horizontal tie-ins and 0.5-foot vertical tie-ins must be achieved between all model domain boundaries (internally), which is why it is recommended that each model domain be buffered so there is overlap on all sides, allowing tie-ins to be achieved around the whole perimeter. Some of my tie-in error polygons are in non-overlap regions for this reason. If there is a reason to allow tie-ins to be completed differently for this watershed, please let me know, I may be missing something.

For the raw raster overshoots, I suspect that pond and hole cleanup were done on each model domain individually and that is why the raw rasters overshoot the finals in some areas. Most of the overshoots I marked occur at tie-in error locations, which supports this explanation. If this is the case, this is an error that needs correcting, as removing ponds and holes on each model domain individually will remove areas that would not qualify for removal based on size if merged into one large raster. Besides these larger overshoot areas, there are also 1-2 pixel overshoots and undershoots throughout the watershed where the raw rasters are just a tiny bit off from the finals on the edges. These I don’t have an explanation for, except that perhaps these are not the exact raw rasters that were used to create the final rasters.

Please let me know if you would like to meet and discuss these issues further. I am out of the office tomorrow, but I will be back next week. You should also know that next week is my final week at the TWDB, so should further review be needed past next week, we’ll need to transition to a new reviewer.

Best,

Leah

TWDB (8/31/23) Provided QC layer: QAQC_Areas_LittleWichita

For the attached QAQC layer, the locations provided are, in most cases, only representative of a given error, not an exhaustive list of all the places it occurs. Each error should be fixed in all places it occurs, not just in the areas marked.

- Topology checks

  - S_Pol_Ar – Has gaps and overlaps

  - BFE_2D – Has overlaps, self-overlaps, and self-intersections

  - FLD_HAZ_AR – Has overlaps

  - WTR_AR – Must be larger than cluster tolerance

Atkins response (11/18/2023): Topology has been checked and errors have been resolved.

TWDB response (12/5/23): Did not check as errors in multiple layers remain and topologies will need to be re-run after corrections.

Atkins response (1/15/24):  Topology run after all other fixes made.  Found no water area issues, Fld Haz Ar issues, BFE issues, Pol Ar issues, 10% issues, or FRAC Ar issues.

TWDB response (3/11/24): Did not check as errors in rasters necessitate corrections and will need to be re-reviewed upon backcheck.

Atkins response (7/31/24): Topology has been validated for all polygons

- Feature classes present/missing

- S_FRAS_Pt missing from FRD. Must be included even if empty. (Due to incorrect schema)

Atkins response (11/18/2023): BLE database schema has been updated.

TWDB response (11/29/23): Fixed

- S_AOMI_Pt is no longer included in FRD. All features in S_AOMI_Pt should instead be included in S_AOMI_Ar. As S_AOMI_Ar is a polygon feature class, take point features from S_AOMI_Pt and add buffers to create polygons that can be merged with S_AOMI_Ar. Appropriate buffer radius should be determined by size of AOMIs.

Atkins response (11/18/2023): Information in AOMI_Pt is now in AOMI_Ar.

TWDB response (11/29/23): Fixed, just make sure Object IDs for S_AOMI_Ar start at 1 and do not skip.

S_FRAC_AR is not populated. Must be populated with data from HAZUS run.

Atkins response (11/18/2023): The FRAC_AR feature class has been populated.

TWDB response (11/29/23): Fixed. Different issue noticed during backcheck: for SCENAR_ID, if there is an NID dam or NLD levee that intersects with the S_FRAC_Ar feature, put the ID for that dam or levee here. If there are none, populate with “Null” or the empty string.

Atkins response (1/15/24):  Object IDs for all FCs are sequential and consecutive.  For Frac Ar, Scenario ID is now blank for most features, and brought in Dam/Levee IDs for any dam or levee associated (in or near) a Frac Ar polygon.

TWDB response (3/11/24): Fixed

- Schema

  - V5 schema was used when V6 was available at time of contract initiation.

Atkins response (11/18/2023): BLE database schema has been updated.

TWDB response (12/5/23): Fixed for everything except FLD_HAZ_AR. One domain does not match, which will not affect the BLE data but needs to be fixed for acceptance by FEMA. Re-load FLD_HAZ_AR data into copy of layer from most recent V6 template: https://fema.maps.arcgis.com/home/item.html?id=50ef6dd229464fc8905b7e54ecb4d55a

Atkins response (1/15/24):  Downloaded and used the V6 template.

TWDB response (3/11/24): FLD_HAZ_AR now has no domains enabled. Please ensure that the domains enabled for each layer match that of V6 schema.

Atkins response (7/31/24): All domains match the V6 schema.

- Attribute table population

- L_Source_Cit

    - Base studies should start at 1

Atkins response (11/18/2023): Source_Cit has been revised to start with BASE1.

TWDB response (11/29/23): Fixed

- S_Pol_Ar

    - SOURCE_CIT references sources not listed in L_Source_Cit

Atkins response (11/18/2023): Updated L_Source_Cit table to include references.

TWDB response (11/29/23): SOURCE_CIT issue fixed. Different issue noticed during backcheck: Within a county, each community should only have one entry in S_Pol_Ar. If a community is not continuous, either naturally or because it has been split at a county boundary in order to correctly populate CO_FIPS, it should be multi-part. See R6V6 guidance, page 34. For example, the image below shows the City of Scotland. The areas highlighted in yellow should be one multi-part feature in Archer County. The unhighlighted portion of the city would be a second entry in S_Pol_Ar with the information for Clay County.

Atkins response (1/15/24):  Merged features of the same name together.  Multipart features are acceptable.

TWDB response (3/11/24): Merging features together does not resolve the issue. However, whether communities should be split at county boundaries is a question FEMA has not yet addressed. Because of that, I think it’s fine to leave as-is.

    - ANI_TF should be True for Baylor and Clay Counties as portions of these counties fall within FIRM panels but do not have any effective mapping/flood risk identified within those panels. For these two rows, populate ANI_FIRM according to guidance.

Atkins response (11/18/2023): Updated ANI_TF to be True for Baylor and Clay counties.

TWDB response (11/29/23): Fixed

    - ANI_TF should be False for the City of Petrolia, City of Henrietta, and City of Bellevue.

Atkins response (11/18/2023): Updated ANI_TF to be False for Petrolia, Henrietta and Bellevue.

TWDB response (11/29/23): Fixed

- BFE_2D

    - EST_ID should be HUC8#_BLE (e.g. 12345678_BLE). This differs from Region 6 Version 6 BLE Submittal guidance. The TWDB, in conjunction with Halff, have opted to use the HUC8#_BLE format for all EST_ID fields within the BLE FRD for consistency and because the DFIRM_ID format currently in guidance does not work well with BLE projects. Guidance will be updated to reflect this change.

Atkins response (11/18/2023): EST_ID updated with “_BLE”.

TWDB response (11/29/23): Fixed

    - BFE_LN_TYP – Contours outside of DTL_STUD_AR polygons should be "MAPPED" or 1010, contours inside of DTL_STUD_AR polys should be "NOT MAPPED" or 1020.

Atkins response (11/18/2023): This change has been made.

TWDB response (11/30/23): Some features still incorrectly attributed.

Atkins response (1/15/24):  Only BFEs that are fully within the DSA are ‘Not Mapped’.  Those that are partially in the DSA are left as ‘Mapped’

TWDB response (3/11/24): Some features still incorrectly attributed. For example, BFE_2D object IDs 97225, 97246, and 97247.

Atkins response (07/31/2024): features correctly attributed

- DTL_STUD_AR

    - DTL_AR_ID should be HUC8#_BLE (e.g. 12345678_BLE). This differs from Region 6 Version 6 BLE Submittal guidance. The TWDB, in conjunction with Halff, have opted to use the HUC8#_BLE format for this field for consistency and because the format currently in guidance does not work well with BLE projects. Guidance will be updated to reflect this change.

TWDB response (11/29/23): Not fixed

Atkins response (1/15/24):  Added _BLE to the end.

TWDB response (3/11/24): Fixed

    - Fields CD_POC through CD_EMAIL are required if applicable and should be “Null” or the empty string

TWDB response (11/29/23): Not fixed

Atkins response (1/15/24):  Converted all NPs to blank fields, except for CD_YN field.

TWDB response (3/11/24): Fixed

    - SOURCE_CIT references a source not listed in L_Source_Cit. Each FIRM panel should also receive its own entry in L_Source_Cit (FIRM1, FIRM2, etc.)

TWDB response (11/29/23): Not fixed

Atkins response (1/15/24):  Per internal QC, all sources should be FIRM1.  FIRM1 has been added to L_Source_Cit.

TWDB response (3/11/24): FEMA guidance is unclear, approach accepted.

    - LOMR_ID should be sequential numbers with leading zeros

TWDB response (11/29/23): Not fixed

Atkins response (1/15/24): Per internal QC, LOMR_ID, Case_No, Scale, and Status are only populated when a LOMR is incorporated into the DSA, but not yet effective.  Otherwise they will be blank (empty string).

TWDB response (3/11/24): FEMA guidance is unclear, approach accepted.

    - STATUS is required if applicable and should be “Null” or the empty string

TWDB response (11/29/23): Not fixed

Atkins response (1/15/24): Made blank.

TWDB response (3/11/24): Fixed

    - Extra fields: OBJECTID_1* and SHAPE_Leng

TWDB response (11/29/23): Fixed

Atkins response (11/18/2023): Updated DTL_STUD_AR as directed.

TWDB response (11/29/23): Not all changes made

Atkins response (1/15/24):  All DSA changes made.  Ensured that effective Zone AE was included, but effective Zone A was excluded.  Used both NFHL and RMSI data.

TWDB response (3/11/24): Fixed

- DTL_STUD_LN

    - OBJECTID should start at 1

TWDB response (3/11/24): Fixed

    - DTL_LN_ID should be HUC8#_BLE (e.g. 12345678_BLE)

TWDB response (11/29/23): Not fixed

TWDB response (3/11/24): Fixed

    - Fields CD_POC through CD_EMAIL are required if applicable and should be “Null” or the empty string

TWDB response (11/29/23): Not fixed

TWDB response (3/11/24): Fixed

    - SOURCE_CIT references a source not listed in L_Source_Cit. Each FIRM panel should also receive its own entry in L_Source_Cit (FIRM1, FIRM2, etc.)

TWDB response (11/29/23): Not fixed

TWDB response (3/11/24): FEMA guidance is unclear, approach accepted.

    - LOMR_ID should be sequential numbers with leading zeros

TWDB response (11/29/23): Not fixed

TWDB response (3/11/24): FEMA guidance is unclear, approach accepted.

    - STATUS is required if applicable and should be “Null” or the empty string

TWDB response (11/29/23): Not fixed

TWDB response (3/11/24): Fixed

    - Extra fields: OBJECTID_1 and SHAPE_Leng

TWDB response (11/29/23): Fixed

Atkins response (11/18/2023): Updated DTL_STUD_LN as directed.

TWDB response (11/29/23): Not all changes made

Atkins response (1/15/24):  All changes made in DSA have been made in DSL too.  Ensured that DSL was clipped to DSA boundaries.

TWDB response (3/11/24): Fixed

- FLD_HAZ_AR

    - EST_ID should be HUC8#_BLE (e.g. 12345678_BLE)

    - AR_REVERT and AR_SUBTRV are required if applicable and should be “Null” or the empty string

Atkins response (11/18/2023): Updated EST_ID, AR_REVERT and AR_SUBTRV as directed.

TWDB response (11/29/23): Fixed

- TENPCT_FP

    - EST_ID should be HUC8#_BLE (e.g. 12345678_BLE)

    - SFHA_TF should be true, as all 10% flood areas fall within the 100yr, which is considered an SFHA as it is an A zone.

    - AR_REVERT and AR_SUBTRV are required if applicable and should be “Null” or “” (the empty string)

Atkins response (11/18/2023): Updated EST_ID, SFHA_TF, AR_REVERT and AR_SUBTRV as directed.

TWDB response (11/29/23): Fixed

- WTR_AR

    - EST_ID should be HUC8#_BLE (e.g. 12345678_BLE)

    - SHOWN_FIRM – If the water feature is shown on the FIRM, this field would be True, otherwise, False. If there is no FIRM in a watershed, this field will be False. Water features that obscure a profile baseline feature for the same reach should be attributed as False. If null, "Null" (or “”, the empty string)

    - SHOWN_INDX – If the water feature is shown on the Index Map, this field would be True, otherwise, False. If there is no FIRM in a watershed, this field will be False. Due to the scale of the Index Map format, lower order and overly detailed water features would be attributed as False to avoid clutter on the map.  If null, "Null" or “”, the empty string

Atkins response (11/18/2023): Updated EST_ID, SHOWN_FIRM and SHOWN_INDEX as requested.

TWDB response (12/5/23): Fixed

- WTR_LN

    - EST_ID should be HUC8#_BLE (e.g. 12345678_BLE)

    - SHOWN_FIRM – If the water feature is shown on the FIRM, this field would be True, otherwise, False. If there is no FIRM in a watershed, this field will be False. Water features that obscure a profile baseline feature for the same reach should be attributed as False. If null, "Null" (or “”, the empty string)

    - SHOWN_INDX – If the water feature is shown on the Index Map, this field would be True, otherwise, False. If there is no FIRM in a watershed, this field will be False. Due to the scale of the Index Map format, lower order and overly detailed water features would be attributed as False to avoid clutter on the map.  If null, "Null" or “”, the empty string

Atkins response (11/18/2023): Updated EST_ID, SHOWN_FIRM and SHOWN_INDEX as requested.

TWDB response (12/5/23): Fixed

- S_AOMI_Ar

    - AOMI_TYP – Where possible, “Other / Misc Structure” should be used instead of “NP”. Note: The list of possible AOMI_TYP values within the template domain is longer than those listed in the FEMA Domain Tables Technical Reference (2020). See screenshots below for how to access the full list of domain values in ArcPro. Choose the most applicable value.

Atkins response (11/18/2023): Updated AOMI_TYP to appropriate type.

TWDB response (11/29/23): Fixed

    - AOMI_INFO – If this field is left empty, the AOMI_TYP should not be NP or Other / Misc Structure. At least one of these fields should provide information on why the point requires an AOMI.

Atkins response (11/18/2023): Updated blank AOMI_TYP to NP.

TWDB response (11/29/23): Fixed

    - SOURCE_CIT references source not listed in L_Source_Cit

Atkins response (11/18/2023): Add source references to L_Source_Cit table.

TWDB response (11/30/23): Fixed

- S_FRAC_Ar – Empty. Must be populated with HAZUS run data.

Atkins response (11/18/2023):  Updated S_FRAC_Ar feature class.

TWDB response (11/30/23): Fixed

- HUC boundary

- Incorrect HUC boundary. Replace with current HUC boundary from NHD Water Boundary Dataset. See QAQC_Areas_LittleWichita, object IDs# 1 and 2.

TWDB response (11/30/23): Fixed

- Layers not clipped to HUC8 boundary:

    - TENPCT_FP - TWDB response (11/30/23): Still not clipped (3/11/24): Fixed

    - FLD_HAZ_AR - TWDB response (11/30/23): Fixed

    - WTR_LN - TWDB response (11/30/23): Fixed

    - S_FRAC_Ar - TWDB response (11/30/23): Still not clipped (3/11/24): Still a tiny bit (< 0.5 ft) over the HUC boundary in some places but good enough. Upon discussion with AECOM, we learned that these tiny slivers are a drawing issue within ArcGIS and not actually a data issue. Further slivers of this size will not be marked as incorrect.

Atkins response (07/31/2024): S_FRAC_Ar clipped to HUC boundary

    - S_UnMapped_Ln - TWDB response (11/30/23): N/A, not included in submittal

    - S_Studies_Ln - TWDB response (11/30/23): N/A, not included in submittal

Atkins response (11/18/2023):  Clipped TENPCT, FLD_HAZ_AR, WTR_LN and S_FRAC_AR to updated HUC boundary.

Atkins response (1/15/24):  Clipped Ten Percent and Frac Ar to HUC.

TWDB response (3/11/24): Fixed

- Modeling domains tie-ins

  - Tie-in issues, see QAQC_Areas_LittleWichita Object IDs #14-24 and 49-70. Modeling domains must tie-in within 0.5 ft.

Atkins response (11/18/2023):  Tie-ins has been revised as needed to meet the 0.5-foot criterion.

TWDB response (12/4/23): Cannot verify without terrain and raw grids. Please submit terrain and raw grids for backcheck.

Atkins response (3/1/24): Outside of eBFE folder structure, see ForLeah folder for Terrain DEM as well as raw WSE and depth rasters as exported from HEC-RAS. Split line approach was not used to develop this mapping so no split lines or difference rasters have been included in this submittal.

TWDB response (3/11/24): Terrain and raw grids provided and tie-ins reviewed. See note on page 35.

- Floodplain polygon containment

  - TENPCT_FP extends significantly beyond 100. See QAQC_Areas_LittleWichita Object IDs #4-7.

Atkins response (11/18/2023):  Updated TENPCT to be contained inside of 100yr polygon.

TWDB response (11/30/23): Better but numerous slivers of 10pct still remain outside 100yr. See LW_QAQC_Backcheck OIDs 2-4.

Atkins response (1/15/24):  Already fixed by updating 10% into the floodplain.

TWDB response (3/11/24): Did not check as errors in rasters necessitate corrections and will need to be re-reviewed upon backcheck.

Atkins response (07/31/2024): Rasters corrected, tenpct_fp clipped to 100 year

  - 500yr overlaps 100yr. Likely a boundary tie-in issue as overlaps occur at/near model domain boundaries. See QAQC_Areas_LittleWichita Object IDs #14-24 and images below.

Atkins response (11/18/2023):  Tie-in issue resolved.

TWDB response (11/30/23): Overlaps fixed. Cannot verify tie-ins without terrain.

    - Pink = model domain boundaries, Blue = areas where 500yr overlaps 100yr

    - Close-ups of overlaps. Dark blue = 100yr, Light blue = 500yr

Atkins response (3/1/24): Outside of eBFE folder structure, see ForLeah folder for Terrain DEM as well as raw WSE and depth rasters as exported from HEC-RAS.

- Small ponding area removal

  - Some ponding areas remain that should be removed per mapping thresholds. See QAQC_Areas_LittleWichita Object IDs #85-94 and screenshot of inland mapping threshold below. (Note: Areas marked are representative, not exhaustive.)

Atkins response (11/18/2023):  Removed ponding less than 0.33 acre.

TWDB response (12/4/2023): Numerous ponding areas within inland mapping thresholds remain in the DEP01 and DEP02 grids (and, by extension, the corresponding WSE grids and FLD_HAZ_AR). See LW_QAQC_Backcheck OIDs 295-1394. (Note: The areas included in the QAQC shapefile are only for a subsection of the DEP01 and DEP02 grids. In the interest of time, the entire watershed was not reviewed for this issue. However, given that there are so many errors in just this subsection, it should be assumed that there are errors in the entire watershed).

Could not verify if 10pct was processed properly without the 10pct raster. Please submit the 10% raster upon backcheck.

For reference: Ponds (disconnected areas of shallow depth and low flood risk) should be removed per inland mapping thresholds shown above. Islands (holes within floodplain polygons) should be removed/filled in based on the following criteria: Area <= 2 acres for 1% and 0.2% events, Area <=0.5 acres for 10% events. Area calculations should be geodesic.

On this topic, I wanted to clarify something from your report. On page 40, the report states “Fill all holes in the 10% floodplain polygon less than 0.5 acres. Remove all islands in the 10% floodplain polygon that were less than 0.5 acres.” Based on my definition of holes and islands, this is a redundant statement, but I wanted to make sure we are using the same definitions. If you are defining holes and islands differently, this may be incorrect.

Please reach out with any questions.

Atkins response (02/23/2024):  Small islands that should have been removed according to the inland mapping 	threshold matrix in the 100 and 500 year rasters and FLD_HAZ_AR have been removed.

Ponds in the TENPCT_FC were removed according to previous QC comments.

TWDB response (3/11/24): Did not check as errors in rasters necessitate corrections and will need to be re-reviewed upon backcheck.

Atkins response (07/31/2024): Model data remapped using the inland mapping threshold.

- Raster containment

  - DEP01 extends beyond DEP02. See QAQC_Areas_LittleWichita Object IDs #8-13.

  - WSE01 extends beyond WSE02. See QAQC_Areas_LittleWichita Object IDs #8-13.

Atkins response (11/18/2023):  Updated 1% WSE and Depth to be contained inside of 0.2% floodplain.

TWDB response (11/30/23): Areas marked in initial review have been fixed, but DEP01/WSE01 still extend beyond DEP02/WSE02 in many small areas (1-3 cells). See LW_QAQC_Backcheck OIDs 8-294 (Note: this is a complete list of all uncontained areas).

Atkins response (02/23/2024): DEP01/WSE01 Rasters have been clipped so that they no longer extend beyond the  the DEP02/WSE02 rasters.

TWDB response (3/11/24): Did not check as errors in rasters necessitate corrections and will need to be re-reviewed upon backcheck.

Atkins response (07/31/2024): DEP01/WSE01 are contained within DEP02/WSE02

- Centerline containment

  - 1 centerline going outside of 100yr. See QAQC_Areas_LittleWichita, Object ID #3.

Atkins response (11/18/2023):  Updated Wtr_Ln to be contained inside of 1% floodplain.

TWDB response (12/4/2023): 2 segments still outside 1% floodplain. See LW_QAQC_Backcheck OIDs 5 and 6.

Atkins response (1/15/24):  OID is only for FCs in a GDB.  The ITR comments are a shp.  Either way, checked for water lines that are outside of the floodplain, and found none.  We do not know what Halff OIDs 5 and 6 are, but would like to see them.

TWDB (3/11/24): Apologies for the confusion. In the LW_QAQC_Backcheck shapefile, there is a “Layer” column and a “Description” column (the n got cut off in the shapefile). The two segments outside the 1% floodplain are the two features where Layer = WTR_LN and Description = Outside of 100yr (I misspelled “outside” in the shapefile). On the most recent submittal, these two segments still fall outside the 1% floodplain.

Atkins response (07/31/2024): Centerlines are contained within the 100 year floodplain except at conveyance structures

WTR_LN

  - Straight lines in lakes generated by ArcHydro should be manually adjusted. The aim is for centerlines to appear more natural. In image below, pink = WTR_LN, blue = FLD_HAZ_AR, green = rough suggestion for line adjustment.

TWDB response (12/4/2023): Fixed

  - WTR_LN does not have coverage for FEMA effective. See QAQC_Areas_LittleWichita Object IDs #26-48. Use FIRM Flood Hazard Zones and Profile Baseline layers to determine additional coverage needed.

Atkins response (11/18/2023):  Revised Wtr_Ln to include coverage for effective Zone A and used CNMS to update straight water lines in the lake.

TWDB (12/4/23): Just one spot of effective zone A missing a waterline, rest looks good. See LW_QAQC_Backcheck OID 7.

Atkins response (1/15/24):  Again, no OIDs means that we can’t find this problem.

TWDB (3/11/24): Apologies for the OID issue. In this case, column names can be used to pinpoint error as described above (Layer = WTR_LN and Descriptio = Add coverage for effective zone A)

- WTR_AR

  - Included all NHD waterbodies. Should only include waterbodies that are:

    - 20 acres or larger and/or

    - Named and/or

    - Formed by an NID dam with a capacity of 200 acre-ft or more (see QAQC_Areas_LittleWichita Object ID #25)

Atkins response (11/18/2023):  Adding dams with a storage area of 200 ac-ft is not documented in the guidance. After seeking clarification from Jenna Rao (TWDB) and Erika Nordstrom (Halff), the WTR_AR feature class has been populated as follows:

- Included all NHD waterbodies features that are 20 acres or larger or are named.

- Included all waterbodies larger than 20 acres captured in the LiDAR breaklines.

- Downloaded all dams from the NID with a surface area larger than 20 acres. If the ponded area shown on the aerial photography (i.e., base map aerials available in ArcPro), was 20 acres or larger, it was added to WTR_AR.

TWDB (12/4/23): Fixed. Apologies for the confusion, I was mistaken about the dams, the 200-acre figure is for spillways.

- AOMIs

  - Remove duplicate AOMIs (Identify using Must Not Overlap topology. Some overlap on distinct AOMIs is acceptable.)

  - Missing AOMIs on boundaries with adjacent 1D studies. See QAQC_Areas_LittleWichita Object IDs #71-84. (Note: The locations marked in QAQC_Areas_LittleWichita are only representative; more AOMIs are needed for interfaces with 1D studies or adjacent HUCs with studies not yet completed/available on EBFE viewer. An AOMI polygon should be placed wherever water stops abruptly at the HUC boundary and does not continue in the adjacent 1D study. These AOMI polygons should stay within the HUC boundary.)

Atkins response (11/18/2023):  Updated AOMI_Ar to include dams, high-water marks and low-water crossings.  We also clipped the polygons noting that the tie-ins along the watershed perimeter should be reviewed so they fall within the Little Wichita watershed. We did not revise this polygon to include just the areas where the floodplain stops abruptly at the watershed boundary. We have received approval from Jenna Rao at TWDB to keep the polygon along the entire perimeter. However, she has directed us to revise our approach in future task orders so that it is aligned with the request in the comment to only draw polygons where flooding abruptly stops at the watershed boundary.

TWDB response (11/30/23): Duplicate AOMIs still present (see screenshot below). Some AOMIs are now multi-part and should be single part. Consolidate duplicates and make all AOMIs single-part.

Though previously approved to meet Nov 30th deadline, one watershed boundary-wide AOMI for tie-ins will likely be flagged by FEMA reviewers and should now be updated.

For reference: Along interfaces with 1D studies (in this case, Farmer’s Mud and Upper West Fork Trinity), an AOMI polygon of AOMI_TYP = NP and AOMI_INFO = “Spill area. In need of further engineering analysis.” should be placed wherever water stops abruptly at the HUC boundary and does not continue in the adjacent 1D study. These AOMI polygons should be clipped to the HUC boundary. Until adjacent 2D studies become available, the same approach should be applied to AOMIs along those study interfaces.

Atkins response (1/15/24):  Removed duplicate polygons, and those that were just redundant (different geometry, but same location and attributes, clearly for the same real-life feature).  Added the redundant attributes to the kept features, for a better AOMI Info.  Multipart-to-Singlepart all features.  Trimmed all AOMI Ar polygons along the HUC boundary.  Now only around locations where the floodplain abruptly exits the HUC.

TWDB response (3/11/24): Duplicate and redundant AOMIs removed, multipart AOMIs converted to single-part, and HUC-boundary-wide tie-in AOMI polygon replaced as directed. Did not check the accuracy of the trimmed AOMIs for external tie-ins due to the need for raster corrections and re-review.

- Oddly distinct/segmented depth values. See QAQC_Areas_LittleWichita Object IDs# 53 and 60. May not be incorrect but check for accuracy.

Atkins response (11/18/2023):  Please see the QC shapefile response column for our responses.

TWDB response (11/30/23): Cannot verify without terrain.

Atkins response (3/1/24): Outside of eBFE folder structure, see ForLeah folder for Terrain DEM as well as raw WSE and depth rasters as exported from HEC-RAS.

TWDB response (3/11/24): The change in depth does not appear to match the terrain, which shows a constant value in this area as the depth changes. This is a modeling QAQC question, as I’m not sure if this is an issue (the depth changes that cause the visual segmentation are relatively small).

Atkins response (07/09/2024):  All tie in issues resolved in accordance with TWDB guidance.

# 8 – Completeness Review

## Technical Review (Reviewer to confirm with Pass/Fail)

| File Structure and Naming |  |
| --- | --- |
|  | Deliverables provided in TWDB/FEMA required file folder structure and naming convention |
|  | Data missing (if any) documented in report and ReadMe file within the required folder |
| Report |  |
|  | Technical 2D BLE report documents all assumptions, special considerations, etc. |
|  | Flow validation and difference discussed in report |
|  | Flow comparisons discussed in report |
| Hydrologic Model |  |
|  | HEC-HMS model with results, pertinent data and required storm events |
| Hydraulic Model |  |
|  | HEC-RAS model with results, pertinent data and required storm events |
| Flood Risk Database |  |
|  | All features fully populated per current FEMA guidelines |
| CNMS |  |
|  | Updated correctly with new centerlines |
|  | A1-A5 checks complete |
|  | CNMS Post- Processing Complete |
| HAZUS |  |
|  | Run per FEMA guidance |

## Review Comments

QA/QC Checklist Approval

This 2D Base Level Engineering Analysis QA/QC Review is in compliance with the contract requirements and all task checkpoints are complete.  The independent QC Team has reviewed the analysis, presented review comments to the Production Team Task Leader, and discussed any problems or issues.  The Task Leader, QC Manager, QC Reviewer, and Project Manager have signed the QA/QC Checklist to confirm that all comments have been received, addressed, and documented appropriately.

| QA/QC Team – [TWDB or company name] |  |  |  | Production Team – [add company name] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| QC Manager |  | Date |  | Task Lead |  | Date |
| QC Reviewer |  | Date |  | Project Manager |  | Date |