Lower Trinity-Tehuacana Watershed, TX

Base Level Engineering (BLE) Results

Contract #HSFE60-15-D-0003

Task Order #70FA6019F00000035

February 2021

Prepared for:

DHS/FEMA (Federal Emergency Management Agency)

Region VI

FRC 800 North Loop 288
Denton, TX 76209-3698

Submitted by:

Compass PTS JV

3101 Wilson Boulevard

Suite 900

Arlington, VA 22201

Document History

Document location

J:/FY2020/20-06-0052S/Hydraulics - Hydraulics|Lower Trinity - Tehuacana|BLE 03/Hydraulic Data Capture - Hydraulics|Lower Trinity - Tehuacana|BLE – 01

Revision History

| Version Number | Version Date | Summary Changes | Team/Author |
| --- | --- | --- | --- |
| 1 | January 2021 | Initial Base Level Engineering Draft | Compass PTS JV |

Approvals

This document requires the approval of the following persons:

| Role | Name | Phone Extension | Title (CLIN/RMC) | Review Date | Approved Date |
| --- | --- | --- | --- | --- | --- |
| Project Manager | Brian Downing | 817.767.7458 | Project Manager | 2/12/2021 | 2/12/2021 |

Client distribution

| Name | Title/Organization | Location |
| --- | --- | --- |
| Diane Howe | FEMA Region VI | Denton, TX |
| FEMA’s MIP | Region VI | See path above |

Table of Contents

Executive Summary	i

Base Level Engineering (BLE) Methodology	3

1.1	Topographic Data	6

1.1.1	Evaluation	7

1.2	Hydrology	9

1.2.1	Flood Frequency Analyses	11

1.3	Hydraulics	13

1.4	Quality Control	15

1.5	One-Percent Special Flood Hazard Area Delineation	16

Challenges	17

Results and Recommendations	17

3.1	CNMS Validation of Effective Zone A SFHA	18

3.2	Flood Risk Analysis	23

References	25

Appendix A Summary of Peak Qs	A-1

Appendix B BLE Map	B-1

# Executive Summary

FEMA Region VI contracted Compass to complete a Base Level Engineering (BLE) analysis for the HUC-08 watershed Lower Trinity-Tehuacana (LTT) in Central Texas, to support FEMA’s Discovery process and validation of effective Zone A Special Flood Hazard Areas (SFHAs).  The BLE process involves using best available data and incorporating automated techniques with traditional model development procedures to produce regulatory-quality flood hazard boundaries for the 1-percent annual chance event as well as estimates of flood hazard boundaries for multiple recurrence intervals.

The source digital terrain data used for surface model development in support of hydrologic and hydraulic analysis as well as mapping activities were leveraged from various local, State, and Federal partners.  Details regarding the different datasets used are provided below in Section 1.1.

Flood discharges for this study were calculated the United States Geological Survey (USGS) regression equations.  Regression equations were obtained from the USGS Scientific Investigations Report (SIR) 2009-5087, Regression Equations for Estimation of Annual Peak-Streamflow Frequency for Undeveloped Watersheds in Texas Using an L-moment Based, PRESS-Minimized, Residual-Adjusted Approach (2009).

The Hydrologic Engineering Center’s River Analysis System (HEC-RAS) program version 4.1.0 was used to compute water surface elevations on a stream by stream basis.  All hydraulic models were computed using 1-D steady state analysis.

The stream mile network that was validated for these watersheds was compiled using FEMA’s Community Needs Management Strategy (CNMS) inventory.  CNMS is an inventory of flood hazard studies and flood hazard mapping needs for areas where a study is needed.  This data is helpful for community officials in analyzing and depicting flood hazards to enhance the understanding of flood risks.  Communities may use this information to make informed decisions on their planning and flood mitigation efforts.  Table ES-1 lists the stream miles associated with this validation analysis.

Table ES-1: Summary of Stream Miles

| Source | Trinity River Stream Miles |
| --- | --- |
| CNMS | 1,596.2 |

The full inventory of Zone A studies in the watershed were classified in CNMS.  Total miles validated in CNMS are summarized in Table ES-2 and illustrated in Figure ES-1 below.

Table ES-2: Aggregated Zone A Validation Results

| Validation Status | Status Type | Total Miles |
| --- | --- | --- |
| Valid | Being Studied | 188.1 |
| Unverified | Being Studied | 1,408.1 |

Figure ES-1: Lower Trinity-Tehuacana Watershed CNMS Validation Results

An overall risk for each HUC-12 watershed was calculated using the National Flood Risk Percentages Dataset and its proportional area.  The weighted risk was multiplied by the percentage of points in the watershed that failed the CNMS comparison to effective to determine the priority score.  Figure ES-2 below shows the range of the Lower Trinity-Tehuacana Watershed HUC-08 priority scores which can be used to initiate discussions during the Discovery phase.

Figure ES-2: Ranking of Lower Trinity-Tehuacana Watershed HUC-12

# Base Level Engineering (BLE) Methodology

Recent innovations and efficiencies in floodplain mapping have allowed the U.S. Department of Homeland Security’s Federal Emergency Management Agency (FEMA) to develop a process called Base Level Engineering (BLE), which can be used to address current program challenges, including the validation of Zone A studies and the availability of flood risk data in the early stages of a Flood Risk Project.  The BLE process involves using best available data and incorporating automated techniques with traditional model development procedures to produce regulatory quality flood hazard boundaries for the 1-percent annual chance event as well as estimates of flood hazard boundaries for multiple recurrence intervals.  The cost for developing the data and estimates resulting from the BLE process are lower than standard flood production costs.  The BLE results may be used for eventual production of regulatory and non-regulatory products.

As described in Title 42 of the Code of Federal Regulations, Chapter III, Section 4101(e), once every five years, FEMA must evaluate whether the information on Flood Insurance Rate Maps (FIRMs) reflects the current risks in flood-prone areas.  FEMA makes this determination of flood hazard data validity by examining flood study attributes and change characteristics, as specified in the Validation Checklist of the Coordinated Needs Management Strategy (CNMS) Technical Reference.  The CNMS Validation Checklist provides a series of critical and secondary checks to determine the validity of flood hazard areas studied by detailed methods (e.g., Zone AE, AH, or AO).  While the critical and secondary elements in CNMS provide a comprehensive method of evaluating the validity of Zone AE studies, a cost-effective approach for evaluating Zone A studies has been lacking.

In addition to the need for Zone A validation guidance, FEMA standards require flood risk data to be provided in the early stages of a Flood Risk Project.  FEMA Program Standard SID #29 requires that during Discovery, data must be identified that illustrates potential changes in flood elevation and mapping that may result from the proposed project scope.  If available data does not clearly illustrate the likely changes, an analysis is required that estimates the likely changes.  This data and any associated analyses should be shared, and results should be discussed with stakeholders.

An important goal of the BLE process is the scalability of the results.  Scalability means that the results of a BLE analysis can not only be used for CNMS evaluations of Zone A studies but can also be leveraged throughout the Risk MAP program.  The data resulting from a BLE analysis can be updated as needed and used for the eventual production of regulatory and non-regulatory products, outreach and risk communication, and MT-1 processing.  Leveraging this data outside the Risk MAP program may also be valuable to external stakeholders.

FEMA Region VI contracted Compass to complete a BLE analysis for the Lower Trinity-Tehuacana Watershed in Central Texas to support FEMA’s Discovery process and validation of effective Zone A Special Flood Hazard Areas (SFHA).  The study extents include portions of Leon, Freestone, Limestone, Navarro, Henderson, Anderson, and Houston Counties, and include the following communities: The Cities of Athens, Buffalo, Crockett, Fairfield, Grapeland, Jewett, Latexo, Mexia, Palestine, and Teague.  As well as the towns of Elkhart, Kirvin, Oakwood, Streetman, Tehuacana, and Wortham.  The study area consists of 7 HUC-10 basins: Big Elkhart Creek-Trinity River, Box Creek-Trinity River, Buffalo Creek, Caney Creek-Tehuacana Creek, Catfish Creek, Lake Creek-Trinity River, and Upper Keechi Creek. Figure 1 shows the orientation of the Lower Trinity-Tehuacana HUC-10 basins with respect to the counties.

Figure 1: Lower Trinity-Tehuacana Watershed HUC-10 Basins

Compass studied approximately 2,409 miles of stream reaches within the Lower Trinity-Tehuacana Watershed with a minimum drainage area tolerance of one square mile.  The selection and extent of stream reaches studied were based upon the number of stream miles with a minimum drainage area of one square mile and not the number of effective Zone A stream miles.  Study reaches were extended above this threshold as appropriate to ensure all effective Zone A flood areas received an updated analysis.  Topographic data from multiple sources were used to determine the hydrologic and hydraulic characteristics of the watershed.  The following sections summarize the BLE process and discuss the results along with their recommended use.

## Topographic Data

An inventory of existing topographic data was conducted for the Lower Trinity-Tehuacana BLE project footprint.  Figure 2 depicts the datasets identified and leveraged across the project area.  FEMA, TNRIS, and other State and Federal agencies were queried to build the inventory with the most current and available data sources.

Figure 2: Extent of LiDAR Data for Lower Trinity-Tehuacana Watershed

### Evaluation

A data coverage assessment was conducted to check for data gaps, extent, accuracy, and completeness.  A review of related documentation, reports, indexes, and metadata associated with the leverage datasets ensured each dataset meets FEMA accuracy requirements for topographic data.  Decisions to leverage or exclude a dataset (or portion of it), were based generally on the following criteria coupled with engineering judgment:

- Data meet FEMA vertical accuracy standards (Table 1)

- Date of origination

- Data density and coverage

Table 1 depicts the Risk Map SID 43 vertical accuracy requirements based on flood risk and terrain slope within the floodplain being mapped.

Table 1:  FEMA Vertical Accuracy Requirements for Leveraged Data

| Level of Flood Risk | Typical Slopes | Specification Level | Vertical Accuracy* | LiDAR Nominal Pulse Spacing (NPS) |
| --- | --- | --- | --- | --- |
| High (Deciles 1,2,3) | Flattest | Highest | 24.5 cm / 36.3 cm | ≤ 2 meters |
| High (Deciles 1,2,3) | Rolling or Hilly | High | 49.0 cm / 72.6 cm | ≤ 2 meters |
| High (Deciles 2,3,4,5) | Hilly | Medium | 98.0 cm / 145 cm | ≤ 3.5 meters |
| Medium (Deciles 3,4,5,6,7) | Flattest | High | 49.0 cm / 72.6 cm | ≤ 2 meters |
| Medium (Deciles 3,4,5,6,7) | Rolling | Medium | 98.0 cm / 145 cm | ≤ 3.5 meters |
| Medium (Deciles 3,4,5,6,7) | Hilly | Low | 147 cm / 218 cm | ≤ 5 meters |
| Low (Deciles 7,8,9,10) | All | Low | 147 cm / 218 cm | ≤ 5 meters |

*Vertical Accuracy at 95% Confidence Level (FVA or NVA)/(CVA or VVA)

Table 2 depicts the complete list of source elevation data and attributes leveraged for the Lower Trinity-Tehuacana Watershed BLE project.  All the Terrain datasets used for hydrology, hydraulic analyses, and mapping meet the highest specification level defined in Table 1.  Further explanation of the Table 2 datasets can be referenced in section 1.1.1.1.

Table 2:  Source Topographic Data Available for the Lower Trinity-Tehuacana Watershed

| Year | Description | Data Type | RMSE | Source/Owner |
| --- | --- | --- | --- | --- |
| 2016 | Texas Neches LiDAR | Airborne LiDAR | 12.9 cm | USGS |
| 2017 | Texas Robertson-Freestone-Brazos LiDAR | Airborne LiDAR | 24.5 cm | USGS |
| 2018 | EASTERN TX QL2 LiDAR | Airborne LiDAR | 5.9 – 11.6 cm | USGS |

#### Lower Trinity-Tehuacana Watershed Source Terrain Data

The source elevation data for the Lower Trinity-Tehuacana Watershed are DEMs derived from the FEMA/USGS and NRCS LiDAR data.  Only points classified as “ground” points (i.e., bare earth) were imported from the LiDAR and used for development of the project DEMs.  Bare-earth LIDAR data are typically made by filtering non-ground returns (e.g. buildings, vegetation, etc.) from the raw laser returns.  Figure 2 depicts the extent of the data defined in Table 2.

2017 USGS Neches River Basin LiDAR - The 2017 Neches River Basin LiDAR was acquired between March 2016 and October 2017. The LIDAR were collected with a nominal pulse spacing of 0.52 meters. The RMSEz reported for the dataset was 12.9 cm at the 95% confidence level which meets project accuracy specifications of the NSSDA.

2017 USGS LiDAR (Brazos, Freestone, & Robertson) - The 2017 Brazos-Freestone-Robertson LiDAR was acquired between January 2017 and March 2017. The LiDAR were collected with a nominal pulse spacing of 0.63 meters. The RMSEz reported for the dataset was 24.5 cm at the 95% confidence level which meets project accuracy specifications of the NSSDA.

2018 USGS Eastern Texas LiDAR - The 2018 Eastern Texas LiDAR was acquired in March 2018. The LIDAR were collected with a nominal pulse spacing of 0.63 meters. The RMSEz reported for the dataset was between 5.9 – 11.6 cm at the 95% confidence level which meets project accuracy specifications of the NSSDA.

#### 1.1.1.2 Terrain Data Processing

The LiDAR data were processed in Environmental System Research Institute (ESRI) ArcGIS software to create a composite Digital Elevation Model (DEM) dataset for the project area.  The DEM was used to generate hydrologically correct stream centerlines and served as a source for horizontal alignment and vertical elevation during manual revisions.

The Watershed Information System (WISE) Terrain Analyst (WTA) module software platform was utilized in order to create a Hydrologically correct DEM (HDEM) for the project area.  WTA is used to identify natural sinks, peaks and flat areas in the DEM surface.  The WTA module uses several routines that take localized elevations from the source topographic data and apply them to the streams.  This ensures that the streams in the watershed are lower in elevation than any overbank sumps and also descend in height down to an outfall.  The final generated streams file is then “burned” into DEMs to force flow through structures while preventing it from jumping out of the channel banks.

During the hydro enforcement of DEM, elevations of the cells were algorithmically calculated and the best path to route flow was determined without filling sinks in the DEM.  Once all calculations were completed, the flow was checked confirming that all drainage flowed downstream correctly and routed to outside of the HUC-08 basin.  The outputs from the processing described above, which included composite DEMS and HDEMs (10-ft and 50-ft cell-size), were used during the hydrology, hydraulics, and floodplain mapping processes.

In addition to the quantitative assessment of the source digital terrain, a qualitative visual inspection of the composite DEM was performed using a hillshade derived from the 10-foot DEM. The visual inspection indicated no unusual or non-terrestrial features were observed in the composite DEM, assuring the surface files used for hydrologic, hydraulic analyses, and floodplain mapping activities are sufficient for BLE analysis.

## Hydrology

Flood discharges for this study were calculated using the United States Geological Survey (USGS) regression equations.  Regression equations were obtained from the USGS Scientific Investigations Report (SIR) 2009-5087, Regression Equations for Estimation of Annual Peak-Streamflow Frequency for Undeveloped Watersheds in Texas Using an L-moment Based, PRESS-Minimized, Residual-Adjusted Approach (2009).

The Watershed Information System (WISE) software was used to delineate drainage basins in shapefile format using the 50-foot DEM. WISE was also used to calculate the main-channel slope for each basin.  The basin shapefile attribution was automated by WISE with drainage area, main-channel slope, and precipitation.

Table 3 presents the published equations used in this study.  In these equations, Qi represents peak streamflow for i-recurrence interval (annual chance exceedance) in cubic feet per second (cfs), P represents mean annual precipitation in inches, S represents dimensionless main-channel slope, Ω represents the OmegaEM parameter, and A represents cumulative drainage area in square miles.

Table 3:  Summary of Regression Equations in Texas (SIR 2009-5087)

| Recurrence Interval | Equation |
| --- | --- |
| Q10% | P1.203 x S0.403 x 10[0.918Ω + 13.62 – 11.97A^(-0.0289)] |
| Q4% | P1.140 x S0.446 x 10[0.945Ω + 11.79 – 9.819A^(-0.0374)] |
| Q2% | P1.105 x S0.476 x 10[0.961Ω + 11.17 – 8.997A^(-0.0424)] |
| Q1% | P1.071 x S0.507 x 10[0.969Ω + 10.82 – 8.448A^(-0.0467)] |
| Q0.2% | P0.988 x S0.569 x 10[0.976Ω + 10.40 – 7.605A^(-0.0554)] |
| Variables: Qi, peak flow for i recurrence interval annual chance exceedance in cubic feet per second; P, mean annual precipitation, in inches; S, Main-channel slope (dimensionless); Ω, OmegaEM parameter; and A, cumulative drainage area, in square miles. |  |

Discharges for the 1-percent plus and 1-percent minus annual chance event were calculated as Q1%+/- = Q1% ± 101.6*0.30, where 0.30 is the mean residual standard error for the Q1% equation.

The mean annual precipitation values for the Texas regression equations were determined based on gridded coverage obtained from the PRISM Climate Group and available for download from the following location: http://prism.oregonstate.edu/normals/.  The annual precipitation values reflect data for the climatological period 1981-2010 as recorded by the Natural Resources Conservation Service (NRCS).

Main-channel slope was calculated in WISE.  An automated routine determined the longest flowpath from upstream of a reach to the outlet of the sub-basin of interest.  The elevations for those endpoints were based on the 10-foot DEM developed from the LiDAR.

From USGS SIR 2009-5087, the OmegaEM parameter is a generalized terrain and climate index that expresses relative differences in peak-streamflow potential.  A shapefile was developed and populated with OmegaEM values based on Figure 2 in SIR 2009-5087.  This shapefile was used, along with a python script in ArcCatalog, to determine OmegaEM values on a sub-basin basis.  For sub-basins spanning more than one OmegaEM grid, the sub-basin’s centroid determined its OmegaEM parameter.

Automated basin delineations were performed in WISE.  Basin break points were set by the user with a sub-basin target size of one square mile.  This criterion was adjusted for the main stem of the Trinity River to avoid excessive and unnecessary discharge breaks.  Break points were also set immediately upstream of stream confluences.  Cumulative drainage area was determined based on these automated delineations performed by WISE in combination with a stream connectivity routine that defined the stream reach segments with upstream and downstream neighbors.

The sub-basin shapefile was attributed with the computed discharges, and those discharges were incorporated into the HEC-RAS models using an automated routine in WISE.  Discharges, as well as water surface elevation results, were associated with the hydraulic cross-sections prior to generation of floodplain boundaries and grid mapping.  Those results are available in GIS format as part of this BLE submittal package.

### Flood Frequency Analyses

A flood frequency analysis was prepared for the Lower Trinity-Tehuacana watershed.  Peak annual discharge from five USGS gages were processed to generate frequency discharges at each gage.  Table 4 presents the gages included in the flood frequency analysis including the source of the data, location of the gage, period of record, and stream the gage is located on.  Figure 3 shows the locations of USGS gages utilized in the flood frequency analyses.

Table 4: Flood Frequency Analysis Stream Gages

| Gage Number | Flooding Source and Location | Published Drainage Area (mi2) | Period of Record |
| --- | --- | --- | --- |
| USGS 08064700 | Tehuacana Ck nr Streetman, Tx | 142 | 1969-2018 |
| USGS 08064800 | Catfish Ck nr Tennessee Colony, Tx | 207 | 1962-1989 |
| USGS 08065000 | Trinity Rv nr Oakwood, Tx | 12833 | 1923-2019 |
| USGS 08065200 | Upper Keechi Ck nr Oakwood, Tx | 150 | 1962-2019 |
| USGS 08065350 | Trinity Rv nr Crockett, Tx | 13911 | 1964-2019 |

Figure 3: Locations of USGS Gages Utilized in Flood Frequency Analysis

A flood frequency analysis was computed for each USGS gage to determine the peak discharges for the Lower Trinity-Tehuacana Watershed using the USGS “Guidelines for Determining Flood Flow Frequency, Bulletin #17B”, revised September 1982.  The 2009 regression models predict lower 1-percent annual chance event streamflow values than the flood frequency analyses at gage 08065350 located along the Trinity River and gage 08064700 located in the Tehuacana-Caney Creek HUC-10.  Therefore, a frequency adjustment factor is recommended for the 2009 regression equations for the Trinity River and Tehuacana-Caney Creek HUC-10.  The adjustment factor used for gage 08065350 was only utilized between gages 8065000 and 08065350 along the Trinity River. No adjustment factor was used for the rest of the Lower Trinity-Tehuacana HUC-08.

The 1-percent annual chance event peak streamflow comparisons as calculated with the 2009 regression model and from gage data for Lower Trinity-Tehuacana mainstem and tributaries are shown in Figure 4 and Figure 5, respectively.  The frequency analysis results indicated that adjustment factors would be required for some HUC-10s.  Adjustment factors of 1.3 and 1.06 were computed for Tehuacana-Caney Creek and the Trinity River, respectively.

Figure 4: 1-Percent Annual Chance Event Flood Frequency Analysis for Lower Trinity-Tehuacana Mainstem

Figure 5: 1-Percent Annual Chance Event Flood Frequency Analysis for Lower Trinity-Tehuacana Creek Tributaries

## Hydraulics

The hydraulic approach for this BLE analysis of the Lower Trinity-Tehuacana Watershed consisted of using the terrain model described in Section 1.1 in combination with the hydrologic outputs from Section 1.1.1 to establish water surface elevations using 1-D steady state analysis.  The Hydrologic Engineering Center’s River Analysis System (HEC-RAS) program version 4.1.0 was chosen as the computer model to compute water surface elevations on a stream by stream basis.  The WISE software was used to establish model stream orientation, generate initial hydraulic cross-section layout and stationing, assign n-values to cross-sections, and develop all input files for the HEC-RAS program.  ESRI’s ArcMap program was used to review and refine cross-section layout orientation.

First pass cross-section layout was performed using an automated routine in WISE based on the cumulative drainage area at the cross-section location.  A first draft model was created based on this initial cross-section layout, and draft boundaries were developed.  At this stage, a second pass inspection for cross-section placement occurred.  Significant refinement occurred during this step.  To improve the hydraulic models, additional cross-sections were added as needed to better define the BLE floodplain boundary.  Cross-sections were extended in locations where overtopping occurred.  Orientation of cross-sections was refined to improve on the perpendicular orientation to flow.  Additional cross-sections were added at floodplain constrictions and at downstream portions of tributaries to ensure a proper tie-in with receiving streams.  Cross-sections were adjusted to remove sections that intersected hydraulic crossings in the floodplain.  For some of the largest studied streams, cross-sections were laid out manually to have more reasonable spacing and better capture the constrictions in the floodplain.

Cross-sections were not drawn on top of roadways or railroads but were placed at the upstream and downstream face of major roads and railroads.  Ineffective flow stations were placed in the hydraulic models as appropriate to account for flow constrictions and other locations deemed by the engineer to be ineffective at conveying flow downstream.

Cross-sections were drawn on dam tops for significant dams with well-defined spillways to better represent ponded water upstream of the structures.  In so doing, it was assumed that most of the flow during a flood event would pass the spillway and that the hydraulic model would reasonably estimate flow across the spillway as represented in the hydraulic cross-section.  The elevations used in the modeling were checked against effective Zone A boundaries, and the results were deemed reasonable.

The relationship between cumulative drainage area and assigned channel geometry is shown in Table 5.  These default values for dimensions and spacing were subject to change based on engineering judgment.

Table 5: Cross-Section Default Parameters

| Drainage area (upper limit) (mi2) | XS Spacing (ft) | Channel Top Width (ft) |  | Channel Bottom Width (ft) |  | Channel Depth (ft) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Minimum | Maximum | Minimum | Maximum |  |
| 1 | 500 | 5 | 12 | 4 | 11 | 0.1 |
| 2 | 500 | 5 | 14 | 4 | 13 | 0.1 |
| 4 | 500 | 9 | 14 | 8 | 13 | 0.3 |
| 6 | 500 | 11 | 16 | 10 | 15 | 0.3 |
| 10 | 500 | 12 | 17 | 11 | 16 | 0.5 |
| 20 | 500 | 14 | 22 | 13 | 21 | 0.5 |
| 30 | 500 | 16 | 24 | 15 | 23 | 0.5 |
| 40 | 500 | 17 | 26 | 16 | 25 | 0.5 |
| 50 | 600 | 17 | 27 | 16 | 26 | 0.5 |
| 75 | 600 | 19 | 30 | 18 | 29 | 0.5 |
| 100 | 750 | 19 | 32 | 18 | 31 | 0.5 |
| 200 | 750 | 22 | 36 | 21 | 35 | 0.5 |
| 300 | 1000 | 23 | 38 | 22 | 37 | 0.5 |

In typical BLE projects, Manning’s roughness coefficients (n-values) are determined using the 2016 National Land Cover Data (NLCD) dataset in combination with n-values from Chow (1959) and Calenda et al. (2005).  The association between the n-values and the NLCD Classification is shown in Table 6.  Manning’s n-value takeoffs are performed by WISE (default values taken from the “Normal” column).  N-values within channel banks are constrained by the automated routine to a range of 0.030 to 0.070. Then, overbank and channel n-values are manually adjusted in certain locations based on engineering judgment.  Under the current BLE task order, the n-value assignments were modified in a calibration process detailed in Section 1.1.

Table 6: Manning's "n" Roughness Based on 2016 NLCD Classification (Moore, 2011)

| NLCD Classification | Minimum | Normal | Maximum | Source |
| --- | --- | --- | --- | --- |
| Open Water | 0.025 | 0.03 | 0.033 | Chow 1959 |
| Developed, Open Space | 0.01 | 0.013 | 0.016 | Calenda et al. 2005 |
| Developed, Low Intensity | 0.038 | 0.05 | 0.063 | Calenda et al. 2005 |
| Developed, Medium Intensity | 0.056 | 0.075 | 0.094 | Calenda et al. 2005 |
| Developed, High Intensity | 0.075 | 0.1 | 0.125 | Calenda et al. 2005 |
| Barren Land | 0.025 | 0.03 | 0.035 | Chow 1959 |
| Deciduous Forest | 0.1 | 0.12 | 0.16 | Chow 1959 |
| Evergreen Forest | 0.1 | 0.12 | 0.16 | Chow 1959 |
| Mixed Forest | 0.1 | 0.12 | 0.16 | Chow 1959 |
| Scrub/Shrub | 0.035 | 0.05 | 0.07 | Chow 1959 |
| Grassland/Herbaceous | 0.025 | 0.03 | 0.035 | Chow 1959 |
| Pasture/Hay | 0.03 | 0.04 | 0.05 | Chow 1959 |
| Cultivated Crops | 0.025 | 0.035 | 0.045 | Chow 1959 |
| Woody Wetlands | 0.08 | 0.1 | 0.12 | Chow 1959 |
| Emergent Herbaceous Wetland | 0.075 | 0.1 | 0.15 | Chow 1959 |

The boundary condition used for most of the study streams was normal depth with a default value of 0.005 ft/ft.  For streams with large drainage areas (generally greater than 8 square miles), the normal depth slope was calculated based on the channel inverts of the downstream cross-sections.

In cases where streams tie into a lake, a normal depth slope was calculated based on the channel inverts of the downstream cross-sections (typically between 0.0001 and 0.001 ft/ft).  Several HUC-10s within this watershed are located in urban areas with storm drain systems, which are unaccounted for in the BLE models. Simplifications of these systems may considerably affect perceived risk.

## Quality Control

Following the initial BLE analysis in each watershed, the flood hazard area delineations created by the BLE process were reviewed for areas where the results were not ideal.

Quality Control (QC) results indicated that some of the models should be extended to cover the scope of effective flood hazard data.  Those streams were extended farther upstream to match the extents of the SFHA data.

Typical manual editing resulting from reasonability checks included adding cross-sections, adjusting orientation of cross-sections, trimming cross-sections and reduction of the default “V” angle of cross-sections.  Examples of default “V” angled cross-sections are shown in Figure 6.  The majority of the cross sections were adjusted. Other examples of manual editing included the addition of cross-sections at confluence areas (see Figure 7 below), modifications to improve perpendicular orientation at the channel, adjustment of discharge breaks to better represent flow addition points, revisions to dam spillways and dam tops, additional cross-sections bounding major hydraulic structures, and revisions to n-values.

A major component of the QC process was an automated check that identified locations where the 1-percent and 0.2-percent annual chance event profiles were crossed by any other frequency profile.  Significant effort was made to reasonably resolve all of these instances. Another automated check identified locations where there was a drawdown of greater than 0.5 foot on the 1-percent and 0.2-percent annual chance event water surface profiles.  This check is particularly useful for identifying errors in the model such as a channel that is too wide, a poorly placed cross-section, or a need for additional cross-sections.  Again, significant effort was made to reasonably resolve these drawdown situations.

Figure 6: Default "V" angle cross-sections automated by WISE (left).  Manually edited cross-sections to more accurately capture terrain (right).  Resulting flood boundaries shown in gold (left) or purple (right) for clarity.

Figure 7: Manually added cross-sections (green) to improve accuracy of tie-ins at confluences.

## One-Percent Special Flood Hazard Area Delineation

The 10-percent, 1-percent and 0.2-percent annual chance event boundaries were mapped using a routine that develops water surface elevation grids based on the 10-foot cell size DEM developed from the LiDAR dataset used for this project (see Section 1.1). This product was converted to a polygon for cleaning.  The cleaning routine involved manual inspection of the polygons to identify and remove areas of disconnected flooding.  In general, areas with a size of less than 5,000 square feet were removed and all others were investigated to determine whether they should be considered as potentially part of the SFHA.  This investigation was aided by the ground DEM and aerial imagery.  Manual adjustments to the polygons were made to account for spillways on dams which could not be accurately modeled using HEC-RAS as well as disconnected areas along the flooding source that should reasonably be connected.

Following the removal of disconnected flooding areas and other boundary adjustments, the small islands in the floodplain were filled.  Islands with a size between roughly 5,000 and 30,000 square feet were inspected and, in general, islands that were less than 10,000 square feet were filled.

Once the island filling process was complete, the water surface raster mapping routine was run and set to conform to the polygon boundary.  This ensures that the water surface raster and the floodplain boundary are consistent with each other.  The depth raster product was created at the end of the process by performing a raster subtraction with the water surface elevation raster and the ground DEM.

# Challenges

Challenges encountered during BLE analyses vary based on available data on which to run the analysis.  The Lower Trinity-Tehuacana Watershed presented challenges as summarized in the following paragraphs.

A significant effort was made to improve water surface elevation tie-ins at stream confluences.  Cross-sections were manually added on the downstream end of nearly every stream, some of which traverse the receiving stream, so that the water surface elevation of the tributary would be lower than that of the receiving stream.

There are a significant number of small-sized dams on the tributaries throughout the watershed.  Discharges calculated with the regression equations do not take into account the impact of these structures. There may be need for further investigation when upgrading these models for detailed studies.

A significant effort was made to maintain consistent discharges along the Trinity River mainstem between the Lower Trinity-Tehuacana and Lower Trinity-Kickapoo Watershed BLE studies. Revisions were made to the regression equation parameters in this study to ensure consistent discharges between the two BLE studies mentioned.

# Results and Recommendations

The BLE results for this study produced a SFHA that compares reasonably well with the effective SFHA.  These boundaries provide an additional estimated SFHA in areas that do not currently have an SFHA mapped.  These results provide context for flood risk communication as part of the Discovery process and should be verified through community work map meetings before being applied to a regulatory product.

A map showing the BLE results is included as Appendix B.

## CNMS Validation of Effective Zone A SFHA

The following summarizes the results of the CNMS validation assessments for the effective Zone A studies in the Lower Trinity-Tehuacana Watershed.

#### Initial Assessment A1 – Significant Topography Update Check

The significant topography update check determines whether a topographic data source is available that is significantly better than what was used for the effective Zone A modeling and mapping.  For most of the study area the effective Zone A topographic data source is unknown, but most likely would have leveraged contours from USGS 24K map products.  This caused most reaches to FAIL this element because the elevation data leveraged in the BLE study represents a significant improvement from the USGS 24K map products.  The exception to this is the streams in Leon County, which were restudied more recently in 2011 and utilized LiDAR.

#### Initial Assessment A2 – Check for Significant Hydrology Changes

The significant hydrology changes check determines whether new regression equations have become available from the USGS since the date of the effective Zone A study.  If newer regression equations exist for the area of interest, then an engineer must determine whether these regression equations would significantly affect the 1-percent annual chance flow.  A majority of the reaches being assessed did not use regression methods or the hydrologic methods are unknown/undocumented.  The reaches in Leon County used the most current regression equations (SIR 2009-5087).  Reaches in Freestone County and Leon County PASS this check.  Reaches in Anderson, Henderson, Houston, and Limestone Counties have several streams that used regression methods prior to 2003 and new regression methods have since been published.  Therefore, the reaches in these counties FAIL this check.

#### Initial Assessment A3 – Check for Significant Development

The significant development check, using the National Urban Change Indicator (NUCI) dataset, assesses increased urbanization in the watershed of the BLE.  If the percentage of urban area within the HUC-12 watershed containing the effective Zone A study is 15 percent or more and has increased by 50 percent or more since the effective analysis, the study would fail this check.  Although the NUCI data provide year-to-year changes in urbanization, the NLCD also is needed to establish a baseline of urban land cover for this analysis.  The check for significant development in the Lower Trinity-Tehuacana study area was completed by evaluating percentage of urban change at the HUC-12 level.  All of the watershed is classified as rural and, therefore, all reaches PASS this assessment.

Table 7 presents the summarized results of checks A1 through A3.

Table 7: A1-A3 Validation Results

| Assessment Checks | Pass / Fail | Notes |
| --- | --- | --- |
| A1 - Topography | Fail/Pass | Available LiDAR sources in areas of effective studies that did not utilize LiDAR/Studies within Leon County utilized LiDAR for effective study |
| A2 - Hydrology | Fail/Pass | Regression was not used for effective studies or the most current regression equations were used; Regression was used for effective studies and new equations now exist. |
| A3 - Development | Pass | Watershed is classified as Non-Urban |

#### Validation Check A4 – Check of Studies Backed by Technical Data

Zone A studies that pass all initial assessment checks described above may be categorized as “Valid” in the CNMS Inventory only if the effective Zone A study is supported by modeling or sound engineering judgment and all regulatory products are in agreement.  If the effective Zone A study passes all initial assessment checks, but is not supported by modeling, or if the original engineering method used is unsupported or undocumented, a comparison of the BLE results and effective Zone A’s is performed.  Due to lack of documentation of the original engineering methods in Freestone County, check A4 has been marked as FAIL for these reaches.  The majority of reaches have some documentation of engineering methods, and therefore, PASS this check.

#### Validation Check A5 – Comparison of BLE and Effective Zone A

The BLE/effective Zone A comparison method leverages the existing Floodplain Boundary Standard (FBS) certification procedures described in FEMA SID 113, but with a slight modification.  This modified FBS comparison approach uses the 1-percent plus and 1-percent minus flood profiles and horizontal and vertical tolerances described in FEMA’s Automated Engineering guidance document dated May 2016.  For the comparison of BLE and effective Zone A in the Texas study area, the following vertical and horizontal tolerances were used to conduct the modified FBS procedure.  One point was placed every 200 feet along the floodplain boundaries for comparison.

Vertical Tolerance:  +/- 10 feet (one-half contour interval of assumed effective topographic source).
Horizontal Tolerance:  +/- 75 feet (standard horizontal tolerance for BLE comparison testing).

Of the 1,930 modeled BLE streams in the study area, 731 were found to correspond (within the tolerance limits) with effective Zone A flood zones.  Comparison results for these streams were grouped at the HUC-12 level and are summarized in Table 8 to better understand the general health of the HUC-12 watershed, but the validation check was performed at the stream level.  Streams where the percentage of passing FBS sample points is greater than or equal to 85% are marked as “Pass”, otherwise marked as “Fail”.

Table 8: HUC-12 Zone A Validation Results

| HUC-12 Watershed |  | Total FBS points | Fail | Pass | %Pass | BLE Comparison Pass? (>85%) | Priority Score |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Watershed Name | Watershed Number |  |  |  |  |  |  |
| Lower Trinity-Tehuacana | All Streams | 81,026 | 28,022 | 53,004 | 65.4% | Fail |  |
| Elm Creek-Tehuacana Creek | 120302010101 | 2,930 | 1,168 | 1,762 | 60.1% | Fail | 21.0 |
| Cedar Creek | 120302010102 | 782 | 372 | 410 | 52.4% | Fail | 38.7 |
| Little Tehuacana Creek-Tehuacana Creek | 120302010103 | 1,687 | 888 | 799 | 47.4% | Fail | 39.4 |
| Upper Caney Creek | 120302010104 | 1,482 | 609 | 873 | 58.9% | Fail | 20.5 |
| Lower Caney Creek | 120302010105 | 1,071 | 742 | 329 | 30.7% | Fail | 43.4 |
| Sloan Creek-Tehuacana Creek | 120302010106 | 855 | 590 | 265 | 31.0% | Fail | 55.2 |
| Pin Oak Creek-Cottonwood Creek | 120302010107 | 2,370 | 1,343 | 1,027 | 43.3% | Fail | 34.2 |
| Sandy Creek-Tehuacana Creek | 120302010108 | 1,580 | 735 | 845 | 53.5% | Fail | 27.7 |
| Big Brown Creek | 120302010109 | 1,523 | 733 | 790 | 51.9% | Fail | 38.5 |
| Bear Creek-Tehuacana Creek | 120302010110 | 509 | 237 | 272 | 53.4% | Fail | 37.2 |
| Shelton Mill Branch | 120302010201 | 823 | 128 | 695 | 84.4% | Fail | 7.8 |
| Upper Coon Creek | 120302010202 | 1,806 | 332 | 1,474 | 81.6% | Fail | 10.0 |
| Upper Catfish Creek | 120302010203 | 1,199 | 184 | 1,015 | 84.7% | Fail | 9.4 |
| Lower Coon Creek | 120302010204 | 1,543 | 296 | 1,247 | 80.8% | Fail | 11.5 |
| Middle Catfish Creek | 120302010205 | 1,434 | 277 | 1,157 | 80.7% | Fail | 15.5 |
| Otter Creek-Beaver Creek | 120302010206 | 1,547 | 340 | 1,207 | 78.0% | Fail | 17.6 |
| Holly Spring Branch-Beaver Creek | 120302010207 | 1,381 | 291 | 1,090 | 78.9% | Fail | 16.9 |
| Lower Catfish Creek | 120302010208 | 2,796 | 984 | 1,812 | 64.8% | Fail | 28.2 |
| Saline Branch-Trinity River | 120302010301 | 2,337 | 630 | 1,707 | 73.0% | Fail | 21.6 |
| Edwards Creek-Trinity River | 120302010302 | 2,133 | 945 | 1,188 | 55.7% | Fail | 35.4 |
| Spring Lake-Spring Creek | 120302010303 | 629 | 183 | 446 | 70.9% | Fail | 23.3 |
| Town of Montalba-Lake Creek | 120302010304 | 1,976 | 566 | 1,410 | 71.4% | Fail | 23.5 |
| Indian Creek-Trinity River | 120302010305 | 1,532 | 655 | 877 | 57.2% | Fail | 32.5 |
| Town Creek | 120302010401 | 3,701 | 605 | 3,096 | 83.7% | Fail | 14.2 |
| Keechie Creek | 120302010402 | 2,769 | 707 | 2,062 | 74.5% | Fail | 22.4 |
| Gaston Branch-Trinity River | 120302010403 | 2,043 | 666 | 1,377 | 67.4% | Fail | 26.5 |
| Manson Creek | 120302010404 | 2,567 | 583 | 1,984 | 77.3% | Fail | 18.2 |
| Box Creek | 120302010405 | 2,125 | 547 | 1,578 | 74.3% | Fail | 20.6 |
| Caney Creek-Trinity River | 120302010406 | 2,381 | 855 | 1,526 | 64.1% | Fail | 28.7 |
| Alligator Creek | 120302010501 | 335 | 182 | 153 | 45.7% | Fail | 38.0 |
| Linn Creek-Buffalo Creek | 120302010502 | 1,496 | 663 | 833 | 55.7% | Fail | 35.5 |
| Browns Creek-Buffalo Creek | 120302010503 | 1,921 | 674 | 1,247 | 64.9% | Fail | 26.3 |
| Bliss Creek-Buffalo Creek | 120302010504 | 1,532 | 282 | 1,250 | 81.6% | Fail | 12.9 |
| Wheelock Creek | 120302010505 | 1,249 | 229 | 1,020 | 81.7% | Fail | 14.7 |
| Resaca Creek-Buffalo Creek | 120302010506 | 621 | 98 | 523 | 84.2% | Fail | 12.6 |
| Mims Creek-Upper Keechi Creek | 120302010601 | 2,172 | 915 | 1,257 | 57.9% | Fail | 32.4 |
| Hog Creek-Upper Keechi Creek | 120302010602 | 992 | 522 | 470 | 47.4% | Fail | 42.1 |
| Jelly Slough-Upper Keechi Creek | 120302010603 | 1,684 | 694 | 990 | 58.8% | Fail | 33.0 |
| Birch Creek-Upper Keechi Creek | 120302010604 | 2,075 | 544 | 1,531 | 73.8% | Fail | 21.0 |
| Haley Creek-Upper Keechi Creek | 120302010605 | 2,063 | 929 | 1,134 | 55.0% | Fail | 36.0 |
| Boons Creek-Trinity Creek | 120302010701 | 2,416 | 1,384 | 1,032 | 42.7% | Pass | 45.8 |
| Upper Big Elkhart Creek | 120302010702 | 1,656 | 425 | 1,231 | 74.3% | Fail | 20.5 |
| Little Elkhart Creek | 120302010703 | 2,409 | 796 | 1,613 | 67.0% | Fail | 24.7 |
| Lower Big Elkhart Creek | 120302010704 | 906 | 334 | 572 | 63.1% | Fail | 29.5 |
| Upper Hurricane Bayou | 120302010705 | 2,368 | 912 | 1,456 | 61.5% | Fail | 33.3 |
| Lower Hurricane Bayou | 120302010706 | 2,290 | 909 | 1,381 | 60.3% | Fail | 32.3 |
| Cedar Branch-Trinity River | 120302010707 | 1,330 | 339 | 991 | 74.5% | Fail | 20.4 |

Validation Results

Based on the validation assessments and BLE comparison results described above, the CNMS inventory of Zone A studies in the Lower Trinity-Tehuacana Watershed has been updated as summarized in Table 9 and illustrated in Figure 8 below.  The priority scores are illustrated in Figure 9.

Table 9: Aggregated Zone A Validation Results

| Validation Status | Status Type | Total Miles |
| --- | --- | --- |
| Valid | Being Studied | 188.1 |
| Unverified | Being Studied | 1,408.1 |

Figure 8: Lower Trinity-Tehuacana Watershed CNMS Validation Results

Figure 9: Ranking of Lower Trinity-Tehuacana Watershed HUC-12s

## Flood Risk Analysis

A flood risk analysis was performed for this project.  The updated 1-percent annual chance grid (known as the ‘refined’ grid) was used to perform the flood risk analysis to produce the flood losses.  The refined grid loss results are stored in the S_FRAC_Ar feature class.

Hazus version 4.2 was used for the basic and refined loss analysis.

The losses are reported via census blocks.  It is important to note that Hazus version 4.2 uses dasymetric census blocks.  Dasymetric mapping removes undeveloped areas (such as areas covered by other bodies of water, wetlands, or forests) from the Census blocks, changing their shape and reducing their size in these areas.  For more information on dasymetric data visit FEMA’s Media Library for the Hazus-MH Data Inventories: Dasymetric vs. Homogenous, or Hazus 3.0 Dasymetric Data Overview.

Hazus analysis was performed on a watershed level for the study area extents to ensure proper model processing.  A summary of results per county for the 1-percent annual chance event scenarios are shown in Table 10.

Table 10: Hazus 4.2 Results for 1-Percent Annual Chance Scenario for Lower Trinity-Tehuacana

| County | Full Replacement - Total Loss | Dollar Exposure (Replacement Value) - Buildings | Dollar Exposure (Replacement Value) - Contents |
| --- | --- | --- | --- |
| Anderson | $89,823,000 | $2,390,199,000 | $1,432,729,000 |
| Freestone | $39,179,000 | $1,569,922,000 | $937,910,000 |
| Henderson | $14,349,000 | $930,728,000 | $602,141,000 |
| Houston | $19,778,000 | $1,057,484,000 | $663,051,000 |
| Leon | $40,718,000 | $544,966,000 | $340,692,000 |
| Limestone | $1,991,000 | $362,133,000 | $239,512,000 |
| Navarro | $14,000 | $37,052,000 | $19,335,000 |
| Total | $205,852,000 | $6,892,484,000 | $4,235,370,000 |

# References

Asquith, W. and Roussel, M. (2009). Regression Equations for Estimation of Annual Peak-Streamflow Frequency for Undeveloped Watersheds in Texas Using an L-moment-Based, PRESS Minimized, Residual-Adjusted Approach SIR 2009-5087, United States Geological Survey.

- Chow, Ven T. (1959). "Development of Uniform Flow and Its Formulas." Open Channel Hydraulics, Blackburn, Caldwell, NJ, 109-113.

- Federal Emergency Management Agency (FEMA) (2016). “Guidance for Flood Risk Analysis and Mapping – Automated Engineering.”<https://www.fema.gov/media-library-data/1469144112748-f3c4ecd90cb927cd200b6a3e9da80d8a/Automated_Engineering_Guidance_May_2016.pdf> (Feb. 1, 2018).

- National Resources Conservation Service (NRCS). “Texas Precipitation – Average monthly and annual precipitation for the climatological period 1981-2010.” <http://www.twdb.texas.gov/mapping/gisdata.asp> (Oct. 2015).

- U. S. Army Corps of Engineers, Hydrologic Engineering Center. (2010). HEC-RAS River Analysis System, Version 4.1.0. Davis, California.

- Veilleux, A.G., Cohn, T.A., Flynn, K.M., Mason, R.R., Jr., and Hummel, P.R. (2014). “Estimating magnitude and frequency of floods using the PeakFQ 7.0 program: U.S. Geological Survey Fact Sheet 2013–3108.” < https://dx.doi.org/10.3133/fs20133108> (Feb. 1, 2018).

- Xian, G., Homer, C., Dewitz, J., Fry, J., Hossain, N., and Wickham, J. (2011). The change of impervious surface area between 2001 and 2006 in the conterminous United States. Photogrammetric Engineering and Remote Sensing, 77(8), 758-762.

- U. S. Geological Survey, Interagency Advisory Committee on Water Data (1982).  Guidelines for determining Flood Flow Frequency, Bulletin #17B of the Hydrology Subcommittee, Revised September 1981, Editorial Corrections March 1982. Reston, VA.

- Watershed Concepts, a Division of Hayes, Seay, Mattern & Mattern (2008). Watershed Information System (WISE,) Version 4.1.0.

Summary of Peak Qs

Table A-1: Summary of Hydrology

| Stream Name | Drainage Area (sq. mi.) | 1% Peak Discharge | 1%- Peak Discharge | 1%+ Peak Discharge |
| --- | --- | --- | --- | --- |
|  |  | (cfs) | (cfs) | (cfs) |
| Brushy Creek - Lake Livingston |  |  |  |  |
| BIG BRANCH | 1.600433 | 2101 | 1553 | 2841 |
| BIG SPRING CREEK | 1.434713 | 2683 | 1984 | 3629 |
| BIG SPRING CREEK | 1.907122 | 3165 | 2340 | 4281 |
| BROWNS CREEK | 1.217609 | 2294 | 1696 | 3102 |
| BROWNS CREEK | 1.968998 | 2813 | 2080 | 3805 |
| BROWNS CREEK | 2.286537 | 2954 | 2185 | 3995 |
| BROWNS CREEK | 3.689326 | 4342 | 3210 | 5871 |
| BROWNS CREEK | 5.180534 | 5168 | 3821 | 6989 |
| BRUSHY CREEK | 1.025435 | 1446 | 1069 | 1955 |
| BRUSHY CREEK | 2.218115 | 2541 | 1879 | 3437 |
| BRUSHY CREEK | 4.196977 | 3983 | 2945 | 5387 |
| BRUSHY CREEK | 5.565868 | 4855 | 3590 | 6566 |
| BRUSHY CREEK | 9.000786 | 7243 | 5356 | 9796 |
| BRUSHY CREEK | 12.186047 | 8479 | 6270 | 11467 |
| BRUSHY CREEK | 13.537093 | 9066 | 6704 | 12260 |
| BRUSHY CREEK | 14.942393 | 9486 | 7014 | 12829 |
| BRUSHY CREEK | 17.145084 | 9831 | 7269 | 13295 |
| BRUSHY CREEK | 19.805566 | 10949 | 8096 | 14807 |
| BRUSHY CREEK | 20.641428 | 10949 | 8096 | 14807 |
| BRUSHY CREEK | 27.795623 | 13234 | 9785 | 17897 |
| BRUSHY CREEK | 28.666369 | 13234 | 9785 | 17897 |
| BRUSHY CREEK | 29.439728 | 13299 | 9834 | 17986 |
| BRUSHY CREEK | 35.363758 | 14997 | 11089 | 20282 |
| BRUSHY CREEK | 39.154238 | 15506 | 11465 | 20970 |
| BRUSHY CREEK 2 | 0.309559 | 763 | 564 | 1032 |
| BRUSHY CREEK 2 | 1.026243 | 1957 | 1447 | 2647 |
| BRUSHY CREEK 2 | 2.20009 | 3508 | 2594 | 4744 |
| BRUSHY CREEK 2 | 2.811585 | 3922 | 2900 | 5304 |
| CANEY CREEK | 1.295985 | 2709 | 2003 | 3663 |
| CANEY CREEK | 2.787866 | 4145 | 3065 | 5606 |
| CANEY CREEK | 7.215174 | 8379 | 6196 | 11331 |
| CANEY CREEK | 7.605171 | 8379 | 6196 | 11331 |
| CANEY CREEK 2 | 0.451783 | 833 | 616 | 1126 |
| CANEY CREEK 2 | 0.898097 | 1401 | 1036 | 1895 |
| CANEY CREEK 2 | 1.191514 | 1675 | 1239 | 2266 |
| CANEY CREEK 2 | 1.724722 | 2231 | 1650 | 3017 |
| CANEY CREEK 2 | 2.48645 | 2827 | 2090 | 3823 |
| CEDAR LICK BRANCH | 1.411039 | 2486 | 1838 | 3361 |
| CEDAR LICK BRANCH | 1.966307 | 3285 | 2429 | 4442 |
| CEDAR LICK BRANCH | 3.368827 | 4693 | 3470 | 6347 |
| CEDAR LICK BRANCH | 4.655307 | 5575 | 4122 | 7540 |
| CLEAR CREEK | 1.089643 | 1655 | 1224 | 2239 |
| CLEAR CREEK 2 | 1.368712 | 2115 | 1564 | 2860 |
| CLEAR CREEK 2 | 2.216232 | 2866 | 2119 | 3876 |
| DOBBINS CREEK | 1.010011 | 1906 | 1409 | 2577 |
| DOBBINS CREEK | 1.731807 | 2793 | 2065 | 3777 |
| DOBBINS CREEK | 3.085812 | 4494 | 3323 | 6078 |
| DOBBINS CREEK | 5.652763 | 6945 | 5135 | 9392 |
| DOBBINS CREEK | 7.083441 | 7450 | 5509 | 10075 |
| DOBBINS CREEK | 11.801431 | 11098 | 8206 | 15008 |
| DOBBINS CREEK | 14.60898 | 12526 | 9262 | 16940 |
| DOBBINS CREEK | 15.442601 | 12526 | 9262 | 16940 |
| DOBBINS CREEK | 21.092136 | 14659 | 10839 | 19824 |
| DOBBINS CREEK | 21.924859 | 14659 | 10839 | 19824 |
| DOBBINS CREEK | 22.65634 | 14659 | 10839 | 19824 |
| DRY HUFFMAN CREEK | 1.003375 | 2119 | 1567 | 2866 |
| DRY HUFFMAN CREEK | 1.630653 | 3122 | 2308 | 4222 |
| DRY HUFFMAN CREEK | 3.049728 | 4703 | 3478 | 6361 |
| DRY HUFFMAN CREEK | 4.067911 | 5442 | 4024 | 7360 |
| HARTS CREEK | 1.809555 | 2577 | 1905 | 3485 |
| HARTS CREEK | 2.806564 | 3052 | 2257 | 4128 |
| HUFFMAN CREEK | 1.17495 | 2365 | 1749 | 3198 |
| HUFFMAN CREEK | 1.83595 | 3218 | 2379 | 4352 |
| HUFFMAN CREEK | 2.71793 | 4480 | 3313 | 6058 |
| HUFFMAN CREEK | 7.254596 | 8165 | 6037 | 11042 |
| HUFFMAN CREEK | 8.853863 | 8563 | 6332 | 11580 |
| HUFFMAN CREEK | 9.229961 | 8693 | 6428 | 11756 |
| HUFFMAN CREEK | 9.369406 | 8811 | 6515 | 11916 |
| HUFFMAN CREEK | 10.14671 | 8866 | 6556 | 11990 |
| HUFFMAN CREEK | 11.329525 | 9723 | 7189 | 13149 |
| HUFFMAN CREEK | 12.899379 | 10545 | 7797 | 14261 |
| HUFFMAN CREEK | 13.31592 | 10770 | 7964 | 14565 |
| HUFFMAN CREEK | 13.797117 | 10801 | 7986 | 14607 |
| INDIAN CREEK | 0.181592 | 563 | 416 | 761 |
| INDIAN CREEK | 0.771386 | 1672 | 1237 | 2262 |
| INDIAN CREEK | 1.267827 | 2614 | 1933 | 3536 |
| INDIAN CREEK | 2.403877 | 4501 | 3328 | 6086 |
| INDIAN CREEK | 2.894535 | 4988 | 3688 | 6746 |
| INDIAN CREEK | 3.378781 | 5082 | 3758 | 6873 |
| INDIAN CREEK | 3.556876 | 5224 | 3863 | 7065 |
| INDIAN CREEK | 4.591189 | 6174 | 4565 | 8350 |
| INDIAN CREEK | 4.849633 | 6414 | 4743 | 8674 |
| INDIAN CREEK | 5.024589 | 6534 | 4832 | 8837 |
| INDIAN CREEK | 5.317289 | 6678 | 4938 | 9031 |
| INDIAN CREEK | 7.032954 | 7473 | 5526 | 10107 |
| INDIAN CREEK | 8.083767 | 8301 | 6138 | 11226 |
| INDIAN CREEK | 10.75797 | 9755 | 7213 | 13193 |
| INDIAN CREEK | 11.702698 | 10354 | 7656 | 14002 |
| INDIAN CREEK | 12.759879 | 10354 | 7656 | 14002 |
| MILL CREEK | 0.816492 | 1782 | 1317 | 2409 |
| MILL CREEK | 1.81924 | 3083 | 2280 | 4169 |
| MILL CREEK | 2.335321 | 3607 | 2667 | 4878 |
| MILL CREEK | 2.58937 | 3637 | 2690 | 4919 |
| MILL CREEK | 5.647742 | 6204 | 4588 | 8391 |
| MILL CREEK | 6.641432 | 6479 | 4790 | 8761 |
| MILL CREEK | 7.367442 | 6584 | 4869 | 8905 |
| MILL CREEK 2 | 0.999805 | 2559 | 1892 | 3460 |
| MILL CREEK 2 | 2.3685 | 4477 | 3310 | 6054 |
| MILL CREEK 2 | 3.22167 | 5020 | 3712 | 6789 |
| MILL CREEK 2 | 4.175724 | 5898 | 4361 | 7976 |
| MILL CREEK 2 | 6.449886 | 7818 | 5781 | 10572 |
| MILL CREEK 2 | 10.765413 | 10396 | 7687 | 14060 |
| MILL CREEK 2 | 17.555706 | 14287 | 10564 | 19321 |
| MILL CREEK 2 | 18.532807 | 14287 | 10564 | 19321 |
| MILL CREEK 3 | 1.456056 | 2165 | 1601 | 2928 |
| MILL CREEK 3 | 2.592061 | 3282 | 2427 | 4438 |
| MORRIS CREEK | 2.067909 | 3596 | 2659 | 4863 |
| MORRIS CREEK | 2.553052 | 4142 | 3063 | 5602 |
| MORRIS CREEK | 4.41408 | 5453 | 4032 | 7374 |
| OPEN BOTTOM CREEK | 1.277961 | 1795 | 1327 | 2428 |
| OPEN BOTTOM CREEK | 2.290931 | 2725 | 2015 | 3685 |
| OPEN BOTTOM CREEK | 2.61708 | 3010 | 2226 | 4071 |
| OPEN BOTTOM CREEK | 4.821455 | 4598 | 3400 | 6218 |
| OPEN BOTTOM CREEK | 5.414354 | 4835 | 3575 | 6539 |
| PALMETTO CREEK | 1.35553 | 2297 | 1698 | 3106 |
| PALMETTO CREEK | 2.982955 | 3907 | 2889 | 5284 |
| PALMETTO CREEK | 4.332476 | 4848 | 3585 | 6556 |
| PALMETTO CREEK | 4.680057 | 5209 | 3851 | 7044 |
| PALMETTO CREEK | 5.01876 | 5388 | 3984 | 7286 |
| PALMETTO CREEK | 7.308974 | 7172 | 5303 | 9699 |
| PALMETTO CREEK | 12.774566 | 11051 | 8172 | 14946 |
| PALMETTO CREEK | 13.066299 | 11217 | 8294 | 15169 |
| PALMETTO CREEK | 13.159112 | 11217 | 8294 | 15169 |
| PALMETTO CREEK | 15.136988 | 11762 | 8697 | 15907 |
| PALMETTO CREEK | 18.700768 | 13955 | 10319 | 18872 |
| PALMETTO CREEK | 20.337878 | 14273 | 10554 | 19303 |
| PALMETTO CREEK | 20.527003 | 14384 | 10636 | 19452 |
| PALMETTO CREEK | 21.100386 | 14494 | 10717 | 19601 |
| PALMETTO CREEK | 22.528642 | 14998 | 11090 | 20283 |
| PALMETTO CREEK | 23.862202 | 15036 | 11118 | 20334 |
| PALMETTO CREEK | 24.139208 | 15036 | 11118 | 20334 |
| PALMETTO CREEK | 29.078785 | 17011 | 12578 | 23005 |
| PALMETTO CREEK | 30.104848 | 17057 | 12613 | 23068 |
| PALMETTO CREEK | 31.653359 | 17291 | 12786 | 23385 |
| POOLS CREEK | 1.21178 | 2265 | 1675 | 3063 |
| POOLS CREEK | 2.22233 | 3041 | 2248 | 4112 |
| POOLS CREEK | 4.712968 | 5653 | 4180 | 7645 |
| POOLS CREEK | 5.431893 | 6046 | 4471 | 8177 |
| POOLS CREEK | 6.460826 | 6735 | 4980 | 9109 |
| POOLS CREEK | 6.894495 | 6735 | 4980 | 9109 |
| POOLS CREEK | 7.19697 | 6849 | 5065 | 9263 |
| POOLS CREEK | 7.539887 | 7032 | 5200 | 9510 |
| POOLS CREEK | 8.866542 | 7032 | 5200 | 9510 |
| ROCKY CREEK | 0.916435 | 1779 | 1315 | 2406 |
| ROCKY CREEK | 1.229267 | 2224 | 1645 | 3008 |
| ROCKY CREEK | 1.842825 | 3019 | 2232 | 4083 |
| ROCKY CREEK | 2.001012 | 3023 | 2236 | 4089 |
| ROCKY CREEK | 2.575829 | 3584 | 2650 | 4847 |
| ROCKY CREEK | 4.316155 | 4735 | 3501 | 6404 |
| ROCKY CREEK | 4.624727 | 4890 | 3616 | 6613 |
| STEPHEN CREEK | 1.103453 | 2324 | 1718 | 3143 |
| STEPHEN CREEK | 2.823474 | 4126 | 3051 | 5580 |
| STEPHEN CREEK | 4.784618 | 5776 | 4271 | 7812 |
| STEPHEN CREEK | 5.63438 | 6257 | 4627 | 8462 |
| STEPHEN CREEK | 6.129387 | 6269 | 4635 | 8478 |
| STEPHEN CREEK | 8.328043 | 7803 | 5770 | 10553 |
| STEPHEN CREEK | 9.732087 | 8241 | 6093 | 11144 |
| STEPHEN CREEK | 10.603908 | 8328 | 6158 | 11262 |
| SULPHUR CREEK | 1.041308 | 2175 | 1608 | 2941 |
| SULPHUR CREEK | 2.455934 | 3590 | 2655 | 4855 |
| SULPHUR CREEK | 3.846347 | 4835 | 3575 | 6538 |
| SULPHUR CREEK | 7.06156 | 7250 | 5361 | 9805 |
| UNT 02121 in BCLL | 0.036856 | 71 | 52 | 96 |
| UNT 06865 in BCLL | 0.403538 | 618 | 457 | 836 |
| UNT 06890 in BCLL | 0.195313 | 434 | 321 | 587 |
| UNT 06891 in BCLL | 0.871463 | 984 | 728 | 1331 |
| UNT 06904 in BCLL | 1.023821 | 1557 | 1151 | 2106 |
| UNT 06904 in BCLL | 1.770008 | 2207 | 1632 | 2985 |
| UNT 06922 in BCLL | 0.184103 | 410 | 303 | 555 |
| UNT 06931 in BCLL | 0.283463 | 632 | 467 | 854 |
| UNT 06932 in BCLL | 0.109673 | 294 | 217 | 398 |
| UNT 06932 in BCLL | 0.28445 | 606 | 448 | 819 |
| UNT 06933 in BCLL | 0.103126 | 269 | 199 | 364 |
| UNT 06934 in BCLL | 0.282387 | 482 | 357 | 652 |
| UNT 06963 in BCLL | 0.608212 | 1377 | 1019 | 1863 |
| UNT 06965 in BCLL | 0.161684 | 518 | 383 | 700 |
| UNT 06966 in BCLL | 0.136665 | 441 | 326 | 597 |
| UNT 06976 in BCLL | 0.613468 | 1377 | 1018 | 1862 |
| UNT 06979 in BCLL | 0.656488 | 1532 | 1133 | 2071 |
| UNT 06981 in BCLL | 0.587552 | 1207 | 893 | 1633 |
| UNT 06981 in BCLL | 1.302352 | 2260 | 1671 | 3056 |
| UNT 06983 in BCLL | 0.290503 | 751 | 555 | 1015 |
| UNT 06986 in BCLL | 0.136755 | 392 | 290 | 530 |
| UNT 06991 in BCLL | 0.41421 | 778 | 575 | 1052 |
| UNT 06994 in BCLL | 0.347816 | 850 | 629 | 1150 |
| UNT 06994 in BCLL | 0.646379 | 1299 | 961 | 1757 |
| UNT 06996 in BCLL | 0.229692 | 551 | 408 | 746 |
| UNT 06999 in BCLL | 0.244365 | 665 | 492 | 900 |
| UNT 07003 in BCLL | 0.191277 | 361 | 267 | 488 |
| UNT 07004 in BCLL | 0.174847 | 254 | 188 | 343 |
| UNT 07006 in BCLL | 0.142314 | 512 | 379 | 693 |
| UNT 07007 in BCLL | 0.541459 | 1367 | 1011 | 1848 |
| UNT 07009 in BCLL | 0.933608 | 1223 | 904 | 1654 |
| UNT 07009 in BCLL | 1.340285 | 1636 | 1210 | 2212 |
| UNT 07013 in BCLL | 0.14348 | 246 | 182 | 332 |
| UNT 07015 in BCLL | 0.109404 | 325 | 241 | 440 |
| UNT 07019 in BCLL | 0.16775 | 480 | 355 | 650 |
| UNT 07024 in BCLL | 0.304223 | 795 | 588 | 1076 |
| UNT 07025 in BCLL | 0.294852 | 782 | 578 | 1057 |
| UNT 07027 in BCLL | 0.146171 | 476 | 352 | 643 |
| UNT 07028 in BCLL | 0.266515 | 604 | 447 | 817 |
| UNT 07035 in BCLL | 0.276738 | 766 | 567 | 1036 |
| UNT 07036 in BCLL | 0.444609 | 613 | 453 | 829 |
| UNT 07036 in BCLL | 0.629699 | 788 | 582 | 1065 |
| UNT 07038 in BCLL | 0.10483 | 227 | 168 | 307 |
| UNT 07040 in BCLL | 0.923654 | 873 | 645 | 1180 |
| UNT 07119 in BCLL | 1.285583 | 1916 | 1417 | 2592 |
| UNT 07119 in BCLL | 2.333348 | 3106 | 2296 | 4200 |
| UNT 07119 in BCLL | 2.605601 | 3470 | 2566 | 4693 |
| UNT 07119 in BCLL | 2.97587 | 3791 | 2803 | 5127 |
| UNT 07119 in BCLL | 4.140661 | 4636 | 3428 | 6269 |
| UNT 07119 in BCLL | 5.311639 | 5130 | 3793 | 6937 |
| UNT 07119 in BCLL | 6.011016 | 5567 | 4116 | 7528 |
| UNT 07125 in BCLL | 0.270729 | 585 | 433 | 791 |
| UNT 07126 in BCLL | 0.354038 | 703 | 520 | 951 |
| UNT 07127 in BCLL | 0.154421 | 321 | 237 | 434 |
| UNT 07145 in BCLL | 0.573562 | 627 | 463 | 848 |
| UNT 07155 in BCLL | 0.300501 | 610 | 451 | 825 |
| UNT 07166 in BCLL | 0.281132 | 599 | 443 | 811 |
| UNT 07167 in BCLL | 0.14644 | 371 | 274 | 501 |
| UNT 07168 in BCLL | 0.399548 | 798 | 590 | 1079 |
| UNT 07170 in BCLL | 0.36991 | 709 | 525 | 960 |
| UNT 07196 in BCLL | 0.346505 | 488 | 361 | 660 |
| UNT 07208 in BCLL | 0.157559 | 292 | 216 | 395 |
| UNT 07645 in BCLL | 0.830123 | 460 | 340 | 622 |
| UNT 07664 in BCLL | 0.316822 | 894 | 661 | 1209 |
| UNT 07729 in BCLL | 0.183386 | 610 | 451 | 824 |
| UNT 07763 in BCLL | 0.931635 | 1727 | 1277 | 2335 |
| UNT 07768 in BCLL | 1.212536 | 2301 | 1701 | 3111 |
| UNT 07779 in BCLL | 0.817658 | 1643 | 1215 | 2222 |
| UNT 07779 in BCLL | 1.009294 | 1933 | 1430 | 2615 |
| UNT 07808 in BCLL | 0.307048 | 848 | 627 | 1146 |
| UNT 07836 in BCLL | 0.55055 | 1276 | 943 | 1725 |
| UNT 07841 in BCLL | 0.302205 | 916 | 677 | 1238 |
| UNT 07871 in BCLL | 0.561546 | 1283 | 949 | 1735 |
| UNT 07874 in BCLL | 0.491868 | 1225 | 906 | 1656 |
| UNT 07875 in BCLL | 0.548663 | 1503 | 1111 | 2033 |
| UNT 07875 in BCLL | 0.930649 | 2041 | 1509 | 2761 |
| UNT 07877 in BCLL | 0.18416 | 563 | 417 | 762 |
| UNT 07878 in BCLL | 0.462724 | 1241 | 918 | 1679 |
| UNT 07879 in BCLL | 0.155048 | 597 | 442 | 808 |
| UNT 07880 in BCLL | 0.161326 | 530 | 392 | 717 |
| UNT 07881 in BCLL | 0.474382 | 1208 | 893 | 1633 |
| UNT 07881 in BCLL | 0.887963 | 1955 | 1446 | 2644 |
| UNT 07883 in BCLL | 0.349374 | 954 | 705 | 1290 |
| UNT 07884 in BCLL | 0.253422 | 789 | 583 | 1067 |
| UNT 07885 in BCLL | 0.159891 | 522 | 386 | 705 |
| UNT 07886 in BCLL | 0.143749 | 511 | 378 | 691 |
| UNT 07887 in BCLL | 0.363184 | 1003 | 742 | 1356 |
| UNT 07953 in BCLL | 0.588179 | 1552 | 1147 | 2099 |
| UNT 07959 in BCLL | 0.379505 | 859 | 635 | 1162 |
| UNT 07965 in BCLL | 1.142551 | 2553 | 1888 | 3453 |
| UNT 07965 in BCLL | 2.410917 | 4284 | 3168 | 5794 |
| UNT 07965 in BCLL | 3.339234 | 5097 | 3769 | 6893 |
| UNT 07965 in BCLL | 3.647717 | 5221 | 3861 | 7061 |
| UNT 07967 in BCLL | 0.120523 | 452 | 334 | 611 |
| UNT 07984 in BCLL | 0.218987 | 616 | 455 | 833 |
| UNT 07985 in BCLL | 0.196389 | 588 | 435 | 795 |
| UNT 07986 in BCLL | 0.181144 | 470 | 348 | 636 |
| UNT 08022 in BCLL | 1.547425 | 791 | 585 | 1069 |
| UNT 08040 in BCLL | 0.746067 | 310 | 229 | 420 |
| UNT 08043 in BCLL | 0.734619 | 593 | 438 | 801 |
| UNT 08046 in BCLL | 0.329586 | 307 | 227 | 416 |
| UNT 08048 in BCLL | 0.12689 | 257 | 190 | 348 |
| UNT 08055 in BCLL | 0.185807 | 249 | 184 | 337 |
| UNT 08070 in BCLL | 0.210468 | 895 | 662 | 1211 |
| UNT 08071 in BCLL | 0.198003 | 814 | 602 | 1101 |
| UNT 08072 in BCLL | 0.315926 | 1065 | 787 | 1440 |
| UNT 08072 in BCLL | 0.674895 | 1944 | 1438 | 2629 |
| UNT 08073 in BCLL | 0.270729 | 870 | 643 | 1176 |
| UNT 08091 in BCLL | 0.730967 | 1727 | 1277 | 2336 |
| UNT 08095 in BCLL | 0.304806 | 842 | 623 | 1139 |
| UNT 08099 in BCLL | 0.36991 | 1021 | 755 | 1380 |
| UNT 08100 in BCLL | 0.540472 | 738 | 545 | 997 |
| UNT 08100 in BCLL | 1.050186 | 1652 | 1222 | 2234 |
| UNT 08102 in BCLL | 0.322562 | 600 | 444 | 812 |
| UNT 08108 in BCLL | 0.340945 | 809 | 598 | 1095 |
| UNT 08109 in BCLL | 0.158187 | 421 | 311 | 569 |
| UNT 08113 in BCLL | 0.122945 | 252 | 187 | 341 |
| UNT 08113 in BCLL | 0.180875 | 286 | 212 | 387 |
| UNT 08113 in BCLL | 0.324714 | 535 | 395 | 723 |
| UNT 08114 in BCLL | 0.334399 | 245 | 181 | 331 |
| UNT 08115 in BCLL | 0.202128 | 139 | 102 | 187 |
| UNT 08116 in BCLL | 0.514735 | 291 | 215 | 393 |
| UNT 08117 in BCLL | 2.228428 | 1629 | 1205 | 2204 |
| UNT 08117 in BCLL | 2.714826 | 1828 | 1352 | 2472 |
| UNT 0891 in BCLL | 1.141565 | 1670 | 1235 | 2259 |
| UNT 0894 in BCLL | 1.31392 | 1686 | 1247 | 2281 |
| UNT 0894 in BCLL | 2.106739 | 2185 | 1615 | 2955 |
| UNT 0894 in BCLL | 2.957756 | 2529 | 1870 | 3420 |
| UNT 0894 in BCLL | 3.902035 | 2965 | 2192 | 4009 |
| UNT 0904 in BCLL | 1.361359 | 2192 | 1621 | 2964 |
| UNT 0904 in BCLL | 1.930886 | 2673 | 1976 | 3614 |
| UNT 0905 in BCLL | 1.282714 | 2297 | 1698 | 3106 |
| UNT 0905 in BCLL | 2.173457 | 3411 | 2522 | 4612 |
| UNT 0905 in BCLL | 3.123565 | 4432 | 3277 | 5994 |
| UNT 0905 in BCLL | 4.75664 | 6019 | 4451 | 8141 |
| UNT 0907 in BCLL | 1.408169 | 2125 | 1571 | 2874 |
| UNT 0909 in BCLL | 0.712507 | 1021 | 755 | 1380 |
| UNT 0909 in BCLL | 0.995406 | 1383 | 1023 | 1870 |
| UNT 0909 in BCLL | 1.25895 | 1758 | 1300 | 2378 |
| UNT 0912 in BCLL | 1.049558 | 1667 | 1233 | 2255 |
| UNT 0917 in BCLL | 1.075116 | 1294 | 957 | 1750 |
| UNT 0919 in BCLL | 1.000237 | 1419 | 1049 | 1919 |
| UNT 0919 in BCLL | 2.275687 | 2490 | 1841 | 3368 |
| UNT 0933 in BCLL | 0.569617 | 643 | 475 | 869 |
| UNT 0933 in BCLL | 1.598191 | 1529 | 1131 | 2068 |
| UNT 0933 in BCLL | 2.513326 | 1830 | 1353 | 2475 |
| UNT 1017 in BCLL | 0.851286 | 484 | 358 | 654 |
| UNT 1017 in BCLL | 1.788392 | 1183 | 875 | 1600 |
| UNT 1017 in BCLL | 2.077415 | 1304 | 964 | 1763 |
| UNT 1017 in BCLL | 3.827336 | 2142 | 1584 | 2897 |
| UNT 1017 in BCLL | 7.12227 | 3006 | 2223 | 4065 |
| UNT 1021 in BCLL | 1.028484 | 2323 | 1718 | 3141 |
| UNT 1021 in BCLL | 1.295358 | 2543 | 1881 | 3440 |
| UNT 1025 in BCLL | 1.826145 | 3751 | 2774 | 5073 |
| UNT 1025 in BCLL | 4.300552 | 6661 | 4925 | 9008 |
| UNT 1025 in BCLL | 4.816094 | 6661 | 4925 | 9008 |
| UNT 1025 in BCLL | 6.329093 | 7071 | 5229 | 9563 |
| UNT 1029 in BCLL | 0.909764 | 607 | 449 | 821 |
| UNT 1029 in BCLL | 1.675321 | 1109 | 820 | 1500 |
| UNT 1029 in BCLL | 3.930084 | 2088 | 1544 | 2824 |
| UNT 1031 in BCLL | 1.306388 | 1006 | 744 | 1360 |
| UNT 1031 in BCLL | 1.533804 | 1098 | 812 | 1485 |
| UNT 1033 in BCLL | 1.100583 | 2518 | 1862 | 3405 |
| UNT 1033 in BCLL | 1.952228 | 3586 | 2651 | 4849 |
| UNT 1038 in BCLL | 1.458566 | 3027 | 2238 | 4094 |
| UNT 2092 in BCLL | 0.515632 | 1155 | 854 | 1562 |
| UNT 2092 in BCLL | 1.143358 | 1834 | 1356 | 2480 |
| UNT 2093 in BCLL | 0.456536 | 879 | 650 | 1188 |
| UNT 2123 in BCLL | 1.417675 | 1687 | 1248 | 2282 |
| UNT 2123 in BCLL | 2.417284 | 2590 | 1915 | 3503 |
| UNT 2123 in BCLL | 3.432317 | 3122 | 2309 | 4223 |
| UNT 2130 in BCLL | 1.335532 | 1911 | 1413 | 2584 |
| UNT 2135 in BCLL | 0.911458 | 1458 | 1078 | 1972 |
| UNT 2142 in BCLL | 0.648979 | 1129 | 835 | 1526 |
| UNT 2152 in BCLL | 0.879803 | 878 | 649 | 1188 |
| UNT 2152 in BCLL | 1.881206 | 1454 | 1075 | 1967 |
| UNT 2152 in BCLL | 2.893817 | 1928 | 1426 | 2608 |
| UNT 2269 in BCLL | 1.057001 | 745 | 551 | 1007 |
| UNT 2269 in BCLL | 2.412262 | 1236 | 914 | 1671 |
| UNT 2269 in BCLL | 3.997808 | 1783 | 1319 | 2412 |
| UNT 2269 in BCLL | 4.996521 | 1984 | 1467 | 2683 |
| UNT 2269 in BCLL | 6.188931 | 1984 | 1467 | 2683 |
| UNT 2274 in BCLL | 0.558049 | 1484 | 1097 | 2006 |
| UNT 2275 in BCLL | 0.677047 | 552 | 408 | 746 |
| UNT 2275 in BCLL | 1.006783 | 1136 | 840 | 1537 |
| UNT 2275 in BCLL | 1.211063 | 1186 | 877 | 1604 |
| UNT 2275 in BCLL | 2.063246 | 1696 | 1254 | 2294 |
| UNT 2276 in BCLL | 0.173342 | 518 | 383 | 700 |
| UNT 2276 in BCLL | 0.507651 | 1303 | 963 | 1762 |
| UNT 2283 in BCLL | 1.658542 | 3319 | 2454 | 4488 |
| UNT 2285 in BCLL | 0.457074 | 1086 | 803 | 1469 |
| UNT 2312 in BCLL | 0.585579 | 1478 | 1093 | 1998 |
| UNT 2312 in BCLL | 1.074219 | 2308 | 1707 | 3122 |
| UNT 2312 in BCLL | 2.822526 | 4199 | 3105 | 5678 |
| UNT 2332 in BCLL | 1.082379 | 2048 | 1514 | 2769 |
| UNT 2332 in BCLL | 2.171753 | 3543 | 2620 | 4792 |
| UNT 2338 in BCLL | 1.485379 | 1074 | 794 | 1452 |
| UNT 2338 in BCLL | 2.000473 | 1205 | 891 | 1630 |
| UNT 2364 in BCLL | 1.036286 | 436 | 322 | 590 |
| UNT 2364 in BCLL | 2.004299 | 809 | 598 | 1094 |
| UNT 2371 in BCLL | 0.889936 | 2604 | 1925 | 3521 |
| UNT 2371 in BCLL | 1.112151 | 3126 | 2311 | 4227 |
| UNT 2371 in BCLL | 1.473273 | 3583 | 2649 | 4845 |
| UNT 2377 in BCLL | 0.428805 | 1195 | 883 | 1616 |
| UNT 45691 in BCLL | 0.036049 | 66 | 49 | 89 |
| UNT 7896 in BCLL | 0.076493 | 246 | 182 | 332 |
| WILLIAMS CREEK | 2.113285 | 3532 | 2612 | 4777 |
| WILLIAMS CREEK | 2.362761 | 3849 | 2846 | 5205 |
| WILLIAMS CREEK | 2.78809 | 4127 | 3052 | 5582 |
| WILLIAMS CREEK | 3.996105 | 5121 | 3787 | 6926 |
| WILLIAMS CREEK | 4.996879 | 5498 | 4065 | 7435 |
| WOLF CREEK | 1.377859 | 2533 | 1873 | 3426 |
| WOLF CREEK | 1.826324 | 2673 | 1977 | 3615 |
| WOLF CREEK | 3.176977 | 4211 | 3113 | 5694 |
| WOLF CREEK | 4.759689 | 5376 | 3975 | 7271 |
| WOLF CREEK | 5.677962 | 5909 | 4369 | 7992 |
| WOLF CREEK | 8.383731 | 7587 | 5610 | 10260 |
| WOLF CREEK | 10.548489 | 8757 | 6475 | 11843 |
| WOLF CREEK | 11.493576 | 8757 | 6475 | 11843 |
| ZWICKY CREEK | 1.118877 | 1640 | 1212 | 2217 |
| ZWICKY CREEK | 1.363421 | 1892 | 1399 | 2559 |
| Big Creek - Trinity River |  |  |  |  |
| BEAVER DAM CREEK | 1.06704 | 1338 | 989 | 1809 |
| BEAVER DAM CREEK | 1.7857 | 1914 | 1416 | 2589 |
| BEAVER DAM CREEK | 2.68568 | 2662 | 1969 | 3600 |
| BEAVER DAM CREEK | 3.10868 | 2788 | 2062 | 3771 |
| BEAVER DAM CREEK | 4.50171 | 3662 | 2708 | 4952 |
| BEAVER DAM CREEK | 4.7804 | 3691 | 2729 | 4991 |
| BEAVER DAM CREEK | 7.03878 | 4812 | 3558 | 6508 |
| BEAVER DAM CREEK | 7.26258 | 4812 | 3558 | 6508 |
| BEAVER DAM CREEK | 8.77032 | 5521 | 4082 | 7467 |
| BEAVER DAM CREEK | 11.18312 | 6509 | 4813 | 8802 |
| BEAVER DAM CREEK | 12.18156 | 6697 | 4952 | 9058 |
| BEAVER DAM CREEK | 13.16943 | 6894 | 5097 | 9323 |
| BEAVER DAM CREEK | 13.63314 | 7006 | 5181 | 9475 |
| BEAVER DAM CREEK | 16.32285 | 7887 | 5832 | 10666 |
| BEAVER DAM CREEK | 17.04061 | 8076 | 5971 | 10921 |
| BEAVER DAM CREEK | 17.46047 | 8076 | 5971 | 10921 |
| BEAVER DAM CREEK | 20.08526 | 8621 | 6374 | 11658 |
| BEAVER DAM CREEK | 22.78484 | 9223 | 6820 | 12473 |
| BEAVER DAM CREEK | 23.49131 | 9235 | 6828 | 12489 |
| BEAVER DAM CREEK | 33.48632 | 11856 | 8767 | 16034 |
| BEAVER DAM CREEK | 36.251 | 12622 | 9333 | 17070 |
| BEAVER DAM CREEK | 37.00553 | 12622 | 9333 | 17070 |
| BEAVER DAM CREEK | 37.83207 | 12622 | 9333 | 17070 |
| BEAVER DAM CREEK | 51.27312 | 15777 | 11666 | 21337 |
| BEAVER DAM CREEK | 53.45344 | 15777 | 11666 | 21337 |
| BEAVER DAM CREEK | 53.87052 | 15777 | 11666 | 21337 |
| BEAVER DAM CREEK | 67.26252 | 17151 | 12682 | 23195 |
| BEAVER DAM CREEK | 68.33772 | 17151 | 12682 | 23195 |
| BIG CREEK | 1.18435 | 904 | 668 | 1223 |
| BIG CREEK | 1.29588 | 1021 | 755 | 1381 |
| BIG CREEK | 4.40875 | 2775 | 2052 | 3753 |
| BIG CREEK | 5.29286 | 2987 | 2209 | 4040 |
| BIG CREEK | 6.43218 | 3435 | 2540 | 4646 |
| BIG CREEK | 8.85892 | 4405 | 3257 | 5957 |
| BIG CREEK | 9.6638 | 4578 | 3385 | 6191 |
| BIG CREEK | 10.53512 | 4587 | 3392 | 6203 |
| BIG CREEK | 15.60643 | 6178 | 4568 | 8355 |
| BIG CREEK | 18.06112 | 6773 | 5008 | 9160 |
| BIG CREEK | 21.61978 | 7801 | 5768 | 10550 |
| BIG CREEK | 27.43643 | 9011 | 6663 | 12187 |
| BIG CREEK | 29.71364 | 9542 | 7056 | 12905 |
| BIG CREEK | 36.17628 | 10671 | 7891 | 14431 |
| BIG CREEK | 37.21487 | 10671 | 7891 | 14431 |
| BIG CREEK | 42.32354 | 11576 | 8560 | 15656 |
| BIG CREEK | 43.116 | 11654 | 8618 | 15761 |
| BIG CREEK | 48.42755 | 12442 | 9200 | 16826 |
| BIG CREEK | 50.67396 | 12621 | 9332 | 17068 |
| BIG CREEK | 50.76005 | 12661 | 9362 | 17123 |
| BIG CREEK | 53.3733 | 12936 | 9565 | 17494 |
| BIG CREEK | 55.06834 | 13275 | 9816 | 17953 |
| BIG CREEK | 55.79454 | 13275 | 9816 | 17953 |
| BIG CREEK | 61.35844 | 13761 | 10176 | 18611 |
| BIG CREEK | 65.58094 | 14462 | 10694 | 19558 |
| BIG CREEK | 67.51899 | 14630 | 10818 | 19785 |
| BIG CREEK | 68.71428 | 14630 | 10818 | 19785 |
| BIG CREEK | 69.86661 | 14630 | 10818 | 19785 |
| BIG CREEK | 70.54482 | 14630 | 10818 | 19785 |
| BIG CREEK | 72.41307 | 14630 | 10818 | 19785 |
| BIG CREEK | 72.85257 | 14867 | 10993 | 20106 |
| BIG CREEK | 86.90583 | 16905 | 12500 | 22862 |
| BIG CREEK | 87.62978 | 16905 | 12500 | 22862 |
| BIG CREEK | 99.92578 | 18446 | 13640 | 24946 |
| BIG CREEK | 100.47899 | 18446 | 13640 | 24946 |
| BLACKHOUSE CREEK | 1.03817 | 954 | 706 | 1291 |
| BLACKHOUSE CREEK | 1.91896 | 1435 | 1061 | 1940 |
| BLACKHOUSE CREEK | 2.38986 | 1710 | 1265 | 2313 |
| BLACKHOUSE CREEK | 2.4962 | 1763 | 1304 | 2385 |
| BUSCH SLOUGH | 1.30101 | 846 | 625 | 1144 |
| BUSCH SLOUGH | 1.99366 | 1264 | 935 | 1710 |
| BUSCH SLOUGH | 2.9739 | 1706 | 1261 | 2307 |
| BUSCH SLOUGH | 3.12608 | 1765 | 1305 | 2387 |
| BUSCH SLOUGH | 5.00621 | 2577 | 1905 | 3485 |
| BUSCH SLOUGH | 5.69392 | 2780 | 2055 | 3759 |
| BUSCH SLOUGH | 6.66086 | 3023 | 2235 | 4088 |
| BUSCH SLOUGH | 7.34843 | 3023 | 2235 | 4088 |
| BUSCH SLOUGH | 8.5047 | 3163 | 2339 | 4277 |
| BUSCH SLOUGH | 10.21041 | 3537 | 2615 | 4784 |
| COBB CREEK | 1.03638 | 932 | 689 | 1260 |
| COBB CREEK | 1.75055 | 1327 | 981 | 1795 |
| COBB CREEK | 2.29093 | 1661 | 1228 | 2246 |
| COBB CREEK | 3.28915 | 2159 | 1596 | 2920 |
| COBB CREEK | 3.89213 | 2461 | 1820 | 3328 |
| COBB CREEK | 4.84551 | 2941 | 2175 | 3977 |
| COBB CREEK | 5.51054 | 3093 | 2287 | 4183 |
| COBB CREEK | 8.07498 | 4124 | 3050 | 5577 |
| COBB CREEK | 8.75821 | 4240 | 3136 | 5735 |
| COBB CREEK | 9.6312 | 4424 | 3271 | 5983 |
| COBB CREEK | 10.39039 | 4704 | 3478 | 6361 |
| COBB CREEK | 11.62522 | 4704 | 3478 | 6361 |
| DIXON CREEK | 1.00042 | 1258 | 930 | 1702 |
| DIXON CREEK | 1.98622 | 2090 | 1545 | 2826 |
| DIXON CREEK | 2.93032 | 2342 | 1731 | 3167 |
| DIXON CREEK | 6.2118 | 4115 | 3043 | 5565 |
| DIXON CREEK | 13.43854 | 6633 | 4905 | 8970 |
| GEORGIA CAMP CREEK | 1.10372 | 930 | 688 | 1258 |
| GEORGIA CAMP CREEK | 1.34773 | 1055 | 780 | 1427 |
| GEORGIA CAMP CREEK | 1.70759 | 1302 | 963 | 1761 |
| GEORGIA CAMP CREEK | 3.50648 | 2221 | 1642 | 3003 |
| GEORGIA CAMP CREEK | 4.54528 | 2656 | 1964 | 3592 |
| GEORGIA CAMP CREEK | 5.18609 | 2815 | 2082 | 3807 |
| HENDERSON CREEK | 1.13811 | 1495 | 1105 | 2022 |
| HENDERSON CREEK | 2.10826 | 737 | 545 | 997 |
| HENDERSON CREEK | 3.13451 | 1905 | 1408 | 2576 |
| HENDERSON CREEK | 4.0856 | 2244 | 1659 | 3034 |
| HENDERSON CREEK | 5.78925 | 2800 | 2071 | 3787 |
| HENDERSON CREEK | 6.59301 | 3193 | 2361 | 4318 |
| HENDERSON CREEK | 7.3122 | 3488 | 2579 | 4717 |
| HENDERSON CREEK | 8.90734 | 3837 | 2837 | 5190 |
| HENDERSON CREEK | 9.99932 | 4172 | 3085 | 5642 |
| HENDERSON CREEK | 11.2015 | 4400 | 3254 | 5951 |
| HENDERSON CREEK | 13.69943 | 5145 | 3805 | 6958 |
| HENDERSON CREEK | 15.34353 | 5298 | 3918 | 7165 |
| HENDERSON CREEK | 16.21713 | 5583 | 4128 | 7550 |
| HENDERSON CREEK | 17.16329 | 5939 | 4392 | 8032 |
| HENDERSON CREEK | 18.4538 | 6191 | 4578 | 8372 |
| HENDERSON CREEK | 18.77024 | 6262 | 4630 | 8468 |
| HENDERSON CREEK | 23.30259 | 7324 | 5416 | 9905 |
| HENDERSON CREEK | 23.62649 | 7324 | 5416 | 9905 |
| HENDERSON CREEK | 24.67417 | 7609 | 5626 | 10290 |
| HENESY BRANCH | 1.00902 | 818 | 605 | 1106 |
| HENESY BRANCH | 1.57766 | 1193 | 882 | 1614 |
| KNIGHT CREEK | 0.4073 | 568 | 420 | 768 |
| KNIGHT CREEK | 2.66533 | 2056 | 1521 | 2781 |
| KNIGHT CREEK | 4.64472 | 3102 | 2294 | 4195 |
| KNIGHT CREEK | 5.07893 | 555 | 411 | 751 |
| KNIGHT CREEK | 5.49027 | 3351 | 2478 | 4531 |
| KNIGHT CREEK | 6.47715 | 3710 | 2744 | 5018 |
| KNIGHT CREEK | 7.34538 | 3985 | 2946 | 5389 |
| KNIGHT CREEK | 7.69418 | 3985 | 2946 | 5389 |
| KNIGHT CREEK | 9.30218 | 4598 | 3400 | 6218 |
| LOST CREEK | 0.99916 | 1002 | 741 | 1355 |
| LOST CREEK | 3.04416 | 2428 | 1795 | 3284 |
| LOST CREEK | 4.28485 | 3000 | 2219 | 4058 |
| LOST CREEK | 4.56356 | 3052 | 2256 | 4127 |
| LOST CREEK | 7.36655 | 4148 | 3067 | 5609 |
| LOST CREEK | 8.02745 | 4526 | 3347 | 6121 |
| LOST CREEK | 8.81175 | 4663 | 3448 | 6306 |
| LOST CREEK | 9.98685 | 5040 | 3727 | 6816 |
| LOST CREEK | 10.62148 | 5279 | 3903 | 7139 |
| LOST CREEK | 11.15218 | 5445 | 4026 | 7364 |
| LOST CREEK | 12.43014 | 5846 | 4323 | 7906 |
| LOST CREEK | 13.02731 | 5846 | 4323 | 7906 |
| LOST CREEK | 13.09616 | 5980 | 4422 | 8087 |
| LOST CREEK | 13.52481 | 5980 | 4422 | 8087 |
| LOST CREEK | 13.88252 | 5980 | 4422 | 8087 |
| MILL CREEK | 1.02705 | 1458 | 1078 | 1972 |
| MILL CREEK | 1.86641 | 2311 | 1709 | 3126 |
| MILL CREEK | 3.90244 | 3695 | 2732 | 4998 |
| MILL CREEK | 4.36062 | 3948 | 2919 | 5339 |
| MILL CREEK | 5.28146 | 4525 | 3346 | 6119 |
| MILL CREEK | 6.07401 | 4801 | 3550 | 6493 |
| MILL CREEK | 8.02987 | 5756 | 4256 | 7785 |
| MILL CREEK | 9.61318 | 6396 | 4730 | 8650 |
| PINE CREEK | 1.24604 | 1430 | 1058 | 1934 |
| PINE CREEK | 1.78776 | 1796 | 1328 | 2429 |
| PINE CREEK | 3.10769 | 2774 | 2052 | 3752 |
| PINE CREEK | 5.23317 | 3991 | 2951 | 5397 |
| PINE CREEK | 6.02097 | 3991 | 2951 | 5397 |
| PINE CREEK | 7.00731 | 4049 | 2994 | 5476 |
| PINE CREEK | 7.19787 | 4049 | 2994 | 5476 |
| PORTER BRANCH | 1.32979 | 1279 | 946 | 1730 |
| PORTER BRANCH | 1.95142 | 1704 | 1260 | 2305 |
| UNT 001 IN BCTR | 0.43645 | 702 | 519 | 949 |
| UNT 002 IN BCTR | 1.04803 | 1211 | 895 | 1637 |
| UNT 003 IN BCTR | 1.07597 | 1226 | 907 | 1658 |
| UNT 003 IN BCTR | 1.4031 | 1438 | 1064 | 1945 |
| UNT 004 IN BCTR | 1.14946 | 1426 | 1055 | 1929 |
| UNT 004 IN BCTR | 2.05518 | 2085 | 1542 | 2820 |
| UNT 005 IN BCTR | 1.03907 | 1176 | 869 | 1590 |
| UNT 005 IN BCTR | 1.26375 | 1265 | 936 | 1711 |
| UNT 006 IN BCTR | 1.14049 | 1398 | 1033 | 1890 |
| UNT 007 IN BCTR | 1.11475 | 1374 | 1016 | 1859 |
| UNT 007 IN BCTR | 2.12701 | 2189 | 1618 | 2960 |
| UNT 008 IN BCTR | 1.06893 | 1428 | 1056 | 1931 |
| UNT 008 IN BCTR | 1.9525 | 2002 | 1480 | 2708 |
| UNT 009 IN BCTR | 1.00714 | 1456 | 1077 | 1969 |
| UNT 009 IN BCTR | 1.5304 | 1827 | 1351 | 2470 |
| UNT 009 IN BCTR | 2.42482 | 2514 | 1859 | 3400 |
| UNT 010 IN BCTR | 1.06104 | 1114 | 824 | 1507 |
| UNT 011 IN BCTR | 0.96993 | 1005 | 743 | 1359 |
| UNT 012 IN BCTR | 1.06247 | 1576 | 1166 | 2132 |
| UNT 012 IN BCTR | 1.40933 | 1769 | 1308 | 2392 |
| UNT 013 IN BCTR | 1.0084 | 1394 | 1031 | 1886 |
| UNT 013 IN BCTR | 1.4846 | 1708 | 1263 | 2310 |
| UNT 014 IN BCTR | 1.23734 | 1581 | 1169 | 2138 |
| UNT 016 IN BCTR | 1.00508 | 1439 | 1064 | 1947 |
| UNT 016 IN BCTR | 1.56178 | 1735 | 1283 | 2347 |
| UNT 016 IN BCTR | 2.51 | 2234 | 1652 | 3021 |
| UNT 018 IN BCTR | 1.68809 | 1087 | 804 | 1470 |
| UNT 018 IN BCTR | 2.22495 | 1380 | 1020 | 1866 |
| UNT 019 IN BCTR | 1.20156 | 1091 | 807 | 1475 |
| UNT 020 IN BCTR | 1.02283 | 654 | 484 | 885 |
| UNT 020 IN BCTR | 1.65474 | 960 | 710 | 1299 |
| UNT 020 IN BCTR | 2.09223 | 1270 | 939 | 1718 |
| UNT 021 IN BCTR | 0.9919 | 763 | 564 | 1032 |
| UNT 022 IN BCTR | 1.4126 | 1143 | 845 | 1545 |
| UNT 023 IN BCTR | 1.3827 | 967 | 715 | 1307 |
| UNT 024 IN BCTR | 1.17116 | 879 | 650 | 1189 |
| UNT 024 IN BCTR | 1.30208 | 1007 | 744 | 1361 |
| UNT 025 IN BCTR | 1.37024 | 522 | 386 | 706 |
| UNT 025 IN BCTR | 2.31245 | 775 | 573 | 1048 |
| UNT 025 IN BCTR | 3.5714 | 1340 | 991 | 1812 |
| UNT 026 IN BCTR | 1.00723 | 725 | 536 | 980 |
| UNT 026 IN BCTR | 1.59819 | 1119 | 827 | 1513 |
| UNT 027 IN BCTR | 1.01682 | 1057 | 782 | 1430 |
| UNT 028 IN BCTR | 0.95683 | 1051 | 777 | 1422 |
| UNT 029 IN BCTR | 0.3752 | 191 | 141 | 258 |
| UNT 029 IN BCTR | 1.57757 | 776 | 574 | 1049 |
| UNT 031 IN BCTR | 1.00714 | 934 | 690 | 1263 |
| UNT 031 IN BCTR | 1.05261 | 951 | 703 | 1286 |
| UNT 034 IN BCTR | 1.15008 | 1501 | 1110 | 2029 |
| UNT 034 IN BCTR | 3.02923 | 2948 | 2180 | 3987 |
| UNT 034 IN BCTR | 4.1273 | 3363 | 2487 | 4548 |
| UNT 034 IN BCTR | 4.46726 | 3363 | 2487 | 4548 |
| UNT 035 IN BCTR | 0.82519 | 860 | 636 | 1163 |
| UNT 035 IN BCTR | 2.07069 | 1690 | 1250 | 2286 |
| UNT 035 IN BCTR | 3.28749 | 2387 | 1765 | 3228 |
| UNT 035 IN BCTR | 5.23891 | 3365 | 2488 | 4551 |
| UNT 035 IN BCTR | 6.22722 | 3704 | 2739 | 5009 |
| UNT 035 IN BCTR | 6.81657 | 3704 | 2739 | 5009 |
| UNT 036 IN BCTR | 0.92796 | 975 | 721 | 1319 |
| UNT 037 IN BCTR | 0.79097 | 754 | 557 | 1019 |
| UNT 037 IN BCTR | 1.67589 | 1222 | 904 | 1653 |
| UNT 037 IN BCTR | 2.0183 | 1292 | 955 | 1747 |
| UNT 037 IN BCTR | 3.43131 | 1967 | 1455 | 2660 |
| UNT 037 IN BCTR | 8.69925 | 4097 | 3030 | 5541 |
| UNT 037 IN BCTR | 12.74557 | 5565 | 4115 | 7526 |
| UNT 037 IN BCTR | 13.88669 | 5971 | 4415 | 8075 |
| UNT 038 IN BCTR | 1.15636 | 961 | 711 | 1300 |
| UNT 038 IN BCTR | 1.97384 | 1517 | 1122 | 2052 |
| UNT 038 IN BCTR | 2.96941 | 2034 | 1504 | 2751 |
| UNT 038 IN BCTR | 3.93967 | 2342 | 1732 | 3167 |
| UNT 039 IN BCTR | 1.81529 | 1516 | 1121 | 2050 |
| UNT 039 IN BCTR | 2.03984 | 1516 | 1121 | 2050 |
| UNT 039 IN BCTR | 2.96466 | 1901 | 1406 | 2571 |
| UNT 040 IN BCTR | 1.03073 | 950 | 703 | 1285 |
| UNT 040 IN BCTR | 3.88723 | 2555 | 1889 | 3456 |
| UNT 040 IN BCTR | 3.9318 | 2555 | 1889 | 3456 |
| UNT 040 IN BCTR | 4.96209 | 2583 | 1910 | 3493 |
| UNT 040 IN BCTR | 7.92856 | 3538 | 2616 | 4785 |
| UNT 040 IN BCTR | 8.83186 | 3656 | 2703 | 4945 |
| UNT 040 IN BCTR | 9.82339 | 3656 | 2703 | 4945 |
| UNT 040 IN BCTR | 12.51185 | 4101 | 3033 | 5547 |
| UNT 040 IN BCTR | 13.58421 | 4101 | 3033 | 5547 |
| UNT 040 IN BCTR | 16.56351 | 4391 | 3247 | 5939 |
| UNT 045 IN BCTR | 0.93926 | 796 | 589 | 1077 |
| UNT 045 IN BCTR | 1.33388 | 1147 | 848 | 1551 |
| UNT 045 IN BCTR | 1.95371 | 1352 | 1000 | 1829 |
| UNT 045 IN BCTR | 4.12793 | 2479 | 1833 | 3353 |
| UNT 045 IN BCTR | 4.56581 | 2600 | 1922 | 3516 |
| UNT 045 IN BCTR | 6.45607 | 3726 | 2755 | 5039 |
| UNT 046 IN BCTR | 1.14031 | 1101 | 814 | 1489 |
| UNT 046 IN BCTR | 1.25097 | 1112 | 822 | 1503 |
| UNT 047 IN BCTR | 1.0147 | 773 | 572 | 1045 |
| UNT 051 IN BCTR | 1.0058 | 719 | 531 | 972 |
| UNT 051 IN BCTR | 2.00496 | 1283 | 948 | 1735 |
| UNT 051 IN BCTR | 2.14997 | 1312 | 970 | 1774 |
| UNT 051 IN BCTR | 2.20691 | 1322 | 978 | 1788 |
| UNT 051 IN BCTR | 2.29811 | 1393 | 1030 | 1884 |
| UNT 052 IN BCTR | 1.68853 | 1669 | 1234 | 2257 |
| UNT 052 IN BCTR | 2.27599 | 1982 | 1465 | 2680 |
| UNT 053 IN BCTR | 1.27318 | 1271 | 940 | 1720 |
| UNT 053 IN BCTR | 2.15566 | 1602 | 1185 | 2167 |
| UNT 054 IN BCTR | 1.04938 | 1106 | 818 | 1496 |
| UNT 054 IN BCTR | 1.65478 | 1525 | 1127 | 2062 |
| UNT 054 IN BCTR | 4.64428 | 3220 | 2381 | 4355 |
| UNT 055 IN BCTR | 1.612 | 1791 | 1325 | 2423 |
| UNT 055 IN BCTR | 1.99742 | 1899 | 1405 | 2569 |
| UNT 056 IN BCTR | 1.18918 | 992 | 734 | 1342 |
| UNT 056 IN BCTR | 2.31604 | 1664 | 1230 | 2250 |
| UNT 056 IN BCTR | 3.47023 | 2159 | 1596 | 2920 |
| UNT 056 IN BCTR | 3.47744 | 2159 | 1596 | 2920 |
| UNT 056 IN BCTR | 4.65086 | 2628 | 1943 | 3554 |
| UNT 056 IN BCTR | 4.89925 | 2746 | 2031 | 3714 |
| UNT 057 IN BCTR | 1.03357 | 822 | 608 | 1112 |
| UNT 058 IN BCTR | 1.33669 | 1121 | 829 | 1516 |
| UNT 058 IN BCTR | 2.28761 | 1760 | 1301 | 2380 |
| UNT 058 IN BCTR | 2.96045 | 2049 | 1515 | 2771 |
| UNT 058 IN BCTR | 3.50543 | 2242 | 1657 | 3031 |
| UNT 059 IN BCTR | 0.91661 | 710 | 525 | 961 |
| UNT 059 IN BCTR | 1.16526 | 869 | 642 | 1175 |
| UNT 059 IN BCTR | 2.10898 | 1441 | 1065 | 1948 |
| UNT 060 IN BCTR | 1.53551 | 1422 | 1052 | 1924 |
| UNT 060 IN BCTR | 2.60353 | 2064 | 1526 | 2791 |
| UNT 060 IN BCTR | 2.86889 | 2064 | 1526 | 2791 |
| UNT 061 IN BCTR | 1.1334 | 1051 | 777 | 1422 |
| UNT 061 IN BCTR | 2.23887 | 1736 | 1283 | 2347 |
| UNT 061 IN BCTR | 5.33231 | 3318 | 2454 | 4487 |
| UNT 061 IN BCTR | 5.96315 | 3587 | 2652 | 4851 |
| UNT 062 IN BCTR | 1.26657 | 1222 | 904 | 1653 |
| UNT 062 IN BCTR | 1.56914 | 1453 | 1074 | 1965 |
| UNT 062 IN BCTR | 3.11289 | 2433 | 1799 | 3291 |
| UNT 062 IN BCTR | 3.96109 | 2832 | 2094 | 3830 |
| UNT 063 IN BCTR | 1.15555 | 948 | 701 | 1283 |
| UNT 064 IN BCTR | 0.9094 | 965 | 713 | 1304 |
| UNT 065 IN BCTR | 0.9886 | 526 | 389 | 711 |
| UNT 065 IN BCTR | 1.48489 | 755 | 558 | 1021 |
| UNT 066 IN BCTR | 1.22078 | 1073 | 793 | 1450 |
| UNT 067 IN BCTR | 1.07503 | 846 | 626 | 1144 |
| UNT 067 IN BCTR | 2.2032 | 1498 | 1107 | 2025 |
| UNT 067 IN BCTR | 2.80034 | 1797 | 1329 | 2430 |
| UNT 067 IN BCTR | 3.71074 | 2088 | 1544 | 2823 |
| UNT 067 IN BCTR | 3.89451 | 2148 | 1588 | 2905 |
| UNT 068 IN BCTR | 0.80465 | 797 | 589 | 1078 |
| UNT 069 IN BCTR | 1.00687 | 900 | 666 | 1217 |
| UNT 069 IN BCTR | 1.53515 | 1176 | 870 | 1591 |
| UNT 070 IN BCTR | 0.93845 | 762 | 564 | 1031 |
| UNT 070 IN BCTR | 2.06471 | 1341 | 992 | 1814 |
| UNT 070 IN BCTR | 2.451 | 1573 | 1163 | 2127 |
| UNT 070 IN BCTR | 2.75464 | 1649 | 1219 | 2230 |
| UNT 070 IN BCTR | 4.06264 | 2218 | 1640 | 2999 |
| UNT 071 IN BCTR | 0.23567 | 244 | 180 | 330 |
| UNT 071 IN BCTR | 0.66817 | 654 | 483 | 884 |
| UNT 072 IN BCTR | 0.33835 | 366 | 271 | 495 |
| UNT 072 IN BCTR | 0.38058 | 406 | 300 | 549 |
| UNT 073 IN BCTR | 0.37852 | 271 | 201 | 367 |
| UNT 074 IN BCTR | 0.14608 | 176 | 130 | 238 |
| UNT 075 IN BCTR | 1.22504 | 360 | 266 | 487 |
| UNT 075 IN BCTR | 1.92957 | 403 | 298 | 545 |
| UNT 075 IN BCTR | 2.13071 | 636 | 470 | 860 |
| UNT 075 IN BCTR | 3.09598 | 843 | 623 | 1140 |
| UNT 075 IN BCTR | 3.47961 | 843 | 623 | 1140 |
| UNT 075 IN BCTR | 6.19776 | 1249 | 924 | 1689 |
| UNT 075 IN BCTR | 9.44442 | 1704 | 1260 | 2305 |
| UNT 075 IN BCTR | 12.27923 | 1923 | 1422 | 2600 |
| UNT 076 IN BCTR | 0.2171 | 274 | 203 | 371 |
| UNT 077 IN BCTR | 0.17935 | 269 | 199 | 364 |
| UNT 078 IN BCTR | 0.38551 | 567 | 419 | 767 |
| UNT 080 IN BCTR | 0.99584 | 930 | 688 | 1258 |
| UNT 081 IN BCTR | 1.03046 | 920 | 680 | 1244 |
| UNT 082 IN BCTR | 1.09368 | 1080 | 799 | 1461 |
| UNT 082 IN BCTR | 1.94596 | 1515 | 1120 | 2049 |
| UNT 083 IN BCTR | 1.17653 | 1038 | 767 | 1404 |
| UNT 084 IN BCTR | 1.01687 | 798 | 590 | 1080 |
| UNT 084 IN BCTR | 1.38167 | 994 | 735 | 1345 |
| UNT 084 IN BCTR | 2.13265 | 1444 | 1068 | 1952 |
| UNT 084 IN BCTR | 4.36503 | 2538 | 1876 | 3432 |
| UNT 084 IN BCTR | 10.35748 | 4962 | 3669 | 6710 |
| UNT 085 IN BCTR | 0.90258 | 895 | 662 | 1211 |
| UNT 087 IN BCTR | 0.69184 | 719 | 531 | 972 |
| UNT 088 IN BCTR | 0.57679 | 612 | 452 | 827 |
| UNT 089 IN BCTR | 0.83537 | 665 | 492 | 899 |
| UNT 090 IN BCTR | 0.88061 | 972 | 719 | 1314 |
| UNT 091 IN BCTR | 0.21155 | 224 | 165 | 303 |
| UNT 092 IN BCTR | 0.9267 | 835 | 617 | 1129 |
| UNT 093 IN BCTR | 0.71256 | 674 | 498 | 912 |
| UNT 094 IN BCTR | 0.4933 | 636 | 470 | 860 |
| UNT 095 IN BCTR | 0.51743 | 600 | 444 | 811 |
| Bedias Creek |  |  |  |  |
| BEDIAS CREEK | 1.38629 | 1284 | 950 | 1737 |
| BEDIAS CREEK | 6.22326 | 4921 | 3639 | 6655 |
| BEDIAS CREEK | 8.49367 | 4921 | 3639 | 6655 |
| BEDIAS CREEK | 34.46093 | 13095 | 9683 | 17709 |
| BEDIAS CREEK | 49.22537 | 18008 | 13316 | 24354 |
| BEDIAS CREEK | 96.17135 | 27157 | 20081 | 36726 |
| BEDIAS CREEK | 112.01579 | 27157 | 20081 | 36726 |
| BEDIAS CREEK | 150.1294 | 30371 | 22457 | 41073 |
| BEDIAS CREEK | 163.96099 | 32272 | 23863 | 43644 |
| BEDIAS CREEK | 170.43688 | 32272 | 23863 | 43644 |
| BEDIAS CREEK | 176.28761 | 32272 | 23863 | 43644 |
| BEDIAS CREEK | 313.88195 | 50547 | 37376 | 68359 |
| BEDIAS CREEK | 329.80827 | 50939 | 37666 | 68889 |
| BEDIAS CREEK | 330.69737 | 50939 | 37666 | 68889 |
| BEDIAS CREEK | 360.17647 | 53805 | 39785 | 72765 |
| BEDIAS CREEK | 363.18871 | 53805 | 39785 | 72765 |
| BEDIAS CREEK | 373.41105 | 53805 | 39785 | 72765 |
| BEDIAS CREEK | 444.24411 | 55326 | 40910 | 74822 |
| BEDIAS CREEK | 447.52757 | 55619 | 41126 | 75218 |
| BEDIAS CREEK | 585.75429 | 67788 | 50125 | 91676 |
| BEDIAS CREEK | 592.33376 | 68334 | 50529 | 92414 |
| BEDIAS CREEK | 599.68793 | 68936 | 50974 | 93228 |
| BEDIAS CREEK | 604.27858 | 69315 | 51254 | 93740 |
| Boggy Creek |  |  |  |  |
| ADKISSON BRANCH | 0.373856 | 422 | 312 | 571 |
| ADKISSON BRANCH | 0.486308 | 496 | 367 | 671 |
| ADKISSON BRANCH | 1.311932 | 1015 | 750 | 1373 |
| ADKISSON BRANCH | 1.748301 | 1341 | 992 | 1814 |
| BOGGY CREEK | 1.23958 | 2792 | 2064 | 3776 |
| BOGGY CREEK | 2.203543 | 4662 | 3447 | 6305 |
| BOGGY CREEK | 3.965884 | 7190 | 5317 | 9724 |
| BOGGY CREEK | 5.066378 | 8637 | 6387 | 11681 |
| BOGGY CREEK | 8.01669 | 11922 | 8816 | 16123 |
| BOGGY CREEK | 9.102118 | 12390 | 9161 | 16756 |
| BOGGY CREEK | 11.015428 | 14455 | 10689 | 19549 |
| BOGGY CREEK | 12.304329 | 15242 | 11271 | 20614 |
| BOGGY CREEK | 14.397437 | 16947 | 12531 | 22918 |
| BOGGY CREEK | 18.093219 | 19161 | 14169 | 25914 |
| BOGGY CREEK | 18.961275 | 19746 | 14601 | 26704 |
| BOGGY CREEK | 19.881342 | 20146 | 14897 | 27246 |
| BOGGY CREEK | 20.511758 | 20146 | 14897 | 27246 |
| BOGGY CREEK | 22.414396 | 21351 | 15788 | 28875 |
| BOGGY CREEK | 23.595418 | 21574 | 15953 | 29177 |
| BOGGY CREEK | 23.758896 | 21574 | 15953 | 29177 |
| BOGGY CREEK | 48.191431 | 38823 | 28707 | 52504 |
| BOGGY CREEK | 48.938605 | 38823 | 28707 | 52504 |
| BOGGY CREEK | 51.503852 | 38823 | 28707 | 52504 |
| BOGGY CREEK | 58.965812 | 41988 | 31047 | 56784 |
| BOGGY CREEK | 59.065172 | 41988 | 31047 | 56784 |
| BOGGY CREEK | 60.968438 | 41988 | 31047 | 56784 |
| BOGGY CREEK | 61.590066 | 41988 | 31047 | 56784 |
| BOGGY CREEK | 61.654901 | 41988 | 31047 | 56784 |
| BOGGY CREEK | 67.815585 | 41988 | 31047 | 56784 |
| BOGGY CREEK | 68.842276 | 41988 | 31047 | 56784 |
| BOGGY CREEK | 72.07976 | 41988 | 31047 | 56784 |
| BOGGY CREEK | 73.283559 | 41988 | 31047 | 56784 |
| BOGGY CREEK | 91.459338 | 41988 | 31047 | 56784 |
| BOGGY CREEK | 91.508659 | 41988 | 31047 | 56784 |
| BOGGY CREEK | 92.796372 | 41988 | 31047 | 56784 |
| BOGGY CREEK | 95.592107 | 41988 | 31047 | 56784 |
| BOGGY CREEK | 97.334765 | 41988 | 31047 | 56784 |
| BOGGY CREEK | 99.105132 | 41988 | 31047 | 56784 |
| BOGGY CREEK | 99.820979 | 41988 | 31047 | 56784 |
| BOGGY CREEK | 100.135156 | 41988 | 31047 | 56784 |
| BOGGY CREEK | 100.575685 | 41988 | 31047 | 56784 |
| BOGGY CREEK | 128.552103 | 41988 | 31047 | 56784 |
| BOGGY CREEK | 129.406349 | 41988 | 31047 | 56784 |
| BOGGY CREEK | 142.001155 | 41988 | 31047 | 56784 |
| BOGGY CREEK | 142.198171 | 41988 | 31047 | 56784 |
| BOGGY CREEK | 144.068257 | 41988 | 31047 | 56784 |
| BOGGY CREEK | 144.982854 | 41988 | 31047 | 56784 |
| BOGGY CREEK | 147.706378 | 41988 | 31047 | 56784 |
| BOGGY CREEK | 149.95857 | 41988 | 31047 | 56784 |
| BOGGY CREEK | 150.548812 | 41988 | 31047 | 56784 |
| BUTLER BRANCH | 1.036241 | 814 | 602 | 1101 |
| BUTLER BRANCH | 1.869279 | 1255 | 928 | 1697 |
| BUTLER BRANCH | 2.085911 | 1341 | 992 | 1814 |
| BUTLER BRANCH | 2.143494 | 1376 | 1017 | 1861 |
| BUTLER BRANCH | 3.496031 | 2055 | 1519 | 2778 |
| CANEY CREEK | 1.000237 | 954 | 705 | 1290 |
| CANEY CREEK | 1.172413 | 961 | 710 | 1299 |
| CANEY CREEK | 1.847667 | 1415 | 1046 | 1914 |
| CANEY CREEK | 2.119831 | 1542 | 1140 | 2085 |
| CANEY CREEK | 3.661975 | 2535 | 1875 | 3429 |
| CANEY CREEK | 6.099525 | 3073 | 2272 | 4155 |
| CANEY CREEK | 6.936218 | 3504 | 2591 | 4739 |
| CANEY CREEK | 7.751883 | 3609 | 2669 | 4881 |
| CANEY CREEK | 8.14143 | 3823 | 2827 | 5171 |
| CANEY CREEK | 8.641368 | 3883 | 2872 | 5252 |
| CANEY CREEK | 9.586633 | 4080 | 3017 | 5518 |
| CANEY CREEK | 9.790913 | 4314 | 3190 | 5834 |
| CANEY CREEK | 10.439668 | 4314 | 3190 | 5834 |
| CANEY CREEK | 10.642558 | 4336 | 3206 | 5864 |
| CANEY CREEK | 11.155097 | 4355 | 3220 | 5889 |
| CANEY CREEK | 12.518493 | 5049 | 3733 | 6828 |
| CEDAR CREEK | 1.78337 | 3837 | 2837 | 5189 |
| CEDAR CREEK | 2.099565 | 3995 | 2954 | 5403 |
| CEDAR CREEK | 3.56109 | 6287 | 4649 | 8503 |
| CEDAR CREEK | 4.536397 | 7635 | 5646 | 10326 |
| CEDAR CREEK | 4.946033 | 7916 | 5854 | 10706 |
| CEDAR CREEK | 5.525425 | 8217 | 6076 | 11113 |
| CEDAR CREEK | 6.155034 | 8459 | 6255 | 11440 |
| CEDAR CREEK | 7.159037 | 9230 | 6825 | 12483 |
| CONVICT BRANCH | 1.003913 | 885 | 655 | 1197 |
| CONVICT BRANCH | 1.390772 | 1137 | 841 | 1538 |
| JONES BRANCH | 0.577329 | 582 | 431 | 787 |
| LANGFORD BRANCH | 1.50448 | 1556 | 1151 | 2105 |
| LANGFORD BRANCH | 1.723826 | 1615 | 1194 | 2184 |
| LANGFORD BRANCH | 5.250032 | 3189 | 2358 | 4313 |
| LEONA BRANCH | 0.395916 | 444 | 329 | 601 |
| LEONA BRANCH | 0.786206 | 764 | 565 | 1033 |
| LEONA BRANCH | 2.261249 | 1790 | 1324 | 2421 |
| LEONA BRANCH | 2.457458 | 1831 | 1354 | 2477 |
| LEONA BRANCH | 2.722538 | 1971 | 1458 | 2666 |
| LEONA BRANCH TRIBUTARY NO. 1 | 0.358933 | 397 | 293 | 537 |
| LEONA BRANCH TRIBUTARY NO. 2 | 0.247593 | 351 | 260 | 475 |
| LITTLE SPRING BRANCH | 1.109999 | 808 | 598 | 1093 |
| LITTLE SPRING BRANCH | 1.869368 | 1232 | 911 | 1666 |
| MCDANIEL BRANCH | 1.475246 | 4422 | 3270 | 5980 |
| MCDANIEL BRANCH | 2.161351 | 5025 | 3715 | 6795 |
| MCDANIEL BRANCH | 3.43976 | 7518 | 5559 | 10167 |
| MCDANIEL BRANCH | 4.544827 | 9001 | 6655 | 12172 |
| MCDANIEL BRANCH | 5.73105 | 10104 | 7471 | 13664 |
| MCDANIEL BRANCH | 6.7164 | 10676 | 7894 | 14438 |
| MCDANIEL BRANCH | 7.653058 | 10875 | 8041 | 14707 |
| MUSTANG CREEK | 1.302621 | 2177 | 1610 | 2944 |
| MUSTANG CREEK | 1.571826 | 2322 | 1717 | 3141 |
| MUSTANG CREEK | 2.174981 | 3030 | 2241 | 4098 |
| MUSTANG CREEK | 2.734644 | 3649 | 2698 | 4934 |
| MUSTANG CREEK | 3.565895 | 4528 | 3348 | 6124 |
| MUSTANG CREEK | 3.845165 | 4781 | 3536 | 6466 |
| MUSTANG CREEK | 4.310864 | 5148 | 3807 | 6962 |
| MUSTANG CREEK | 5.844579 | 5938 | 4391 | 8031 |
| MUSTANG CREEK | 7.503031 | 7296 | 5395 | 9867 |
| MUSTANG CREEK | 8.054802 | 7296 | 5395 | 9867 |
| MUSTANG CREEK | 8.968503 | 7296 | 5395 | 9867 |
| MUSTANG CREEK | 10.60902 | 7468 | 5522 | 10099 |
| MUSTANG CREEK | 10.879211 | 7476 | 5528 | 10110 |
| MUSTANG CREEK | 11.171193 | 7476 | 5528 | 10110 |
| MUSTANG CREEK | 13.264834 | 7968 | 5892 | 10775 |
| MUSTANG CREEK | 13.450467 | 7968 | 5892 | 10775 |
| MUSTANG CREEK | 16.89151 | 8379 | 6196 | 11331 |
| MUSTANG CREEK | 17.463081 | 8550 | 6322 | 11563 |
| MUSTANG CREEK | 17.604637 | 8550 | 6322 | 11563 |
| NIXON BRANCH | 1.146766 | 1910 | 1413 | 2584 |
| PINE BRANCH | 1.001941 | 808 | 597 | 1092 |
| PINE BRANCH | 2.000115 | 1354 | 1001 | 1831 |
| PINE BRANCH | 2.544711 | 1514 | 1119 | 2047 |
| PINE BRANCH | 3.135716 | 1885 | 1394 | 2550 |
| PINE BRANCH | 4.044708 | 2127 | 1572 | 2876 |
| PINE BRANCH | 4.381977 | 2294 | 1697 | 3103 |
| PINE BRANCH | 4.917133 | 2508 | 1854 | 3392 |
| ROBESON BRANCH | 0.973424 | 1016 | 751 | 1374 |
| ROCKY BRANCH | 1.001941 | 1104 | 817 | 1493 |
| ROCKY BRANCH | 1.530038 | 1416 | 1047 | 1915 |
| SPRING BRANCH | 1.233213 | 1017 | 752 | 1376 |
| SPRING BRANCH | 1.999399 | 1352 | 1000 | 1828 |
| SPRING BRANCH | 2.98699 | 1762 | 1303 | 2383 |
| SPRING BRANCH | 4.03601 | 2167 | 1602 | 2930 |
| SPRING BRANCH | 4.203972 | 2205 | 1630 | 2982 |
| SPRING CREEK | 1.613705 | 4132 | 3056 | 5588 |
| SPRING CREEK | 2.762174 | 6003 | 4439 | 8118 |
| SPRING CREEK | 4.873262 | 8578 | 6343 | 11600 |
| SPRING CREEK | 5.248015 | 8889 | 6573 | 12021 |
| SPRING CREEK | 8.316564 | 12415 | 9180 | 16790 |
| SPRING CREEK | 10.578082 | 14402 | 10650 | 19477 |
| SPRING CREEK | 11.420087 | 14941 | 11048 | 20206 |
| SPRING CREEK | 12.379208 | 15797 | 11681 | 21363 |
| SPRING CREEK | 13.437195 | 15797 | 11681 | 21363 |
| SPRING CREEK | 21.49675 | 22480 | 16623 | 30402 |
| SPRING CREEK | 22.376822 | 23003 | 17009 | 31109 |
| SPRING CREEK | 23.336346 | 23003 | 17009 | 31109 |
| SPRING CREEK | 23.482786 | 23003 | 17009 | 31109 |
| TWOMILE CREEK | 1.03521 | 944 | 698 | 1277 |
| TWOMILE CREEK | 1.993299 | 1589 | 1175 | 2149 |
| TWOMILE CREEK | 2.392679 | 1850 | 1368 | 2501 |
| TWOMILE CREEK | 3.846795 | 2715 | 2008 | 3672 |
| TWOMILE CREEK | 4.103356 | 2734 | 2021 | 3697 |
| TWOMILE CREEK | 4.196439 | 2813 | 2080 | 3805 |
| TWOMILE CREEK | 4.638806 | 2961 | 2189 | 4004 |
| TWOMILE CREEK | 5.276845 | 3232 | 2390 | 4371 |
| TWOMILE CREEK | 6.237625 | 3549 | 2624 | 4799 |
| TWOMILE CREEK | 10.489931 | 4809 | 3556 | 6504 |
| TWOMILE CREEK | 11.425871 | 4932 | 3647 | 6669 |
| TWOMILE CREEK | 11.751481 | 5022 | 3713 | 6791 |
| TWOMILE CREEK | 12.99958 | 5026 | 3717 | 6798 |
| TWOMILE CREEK | 13.986724 | 5284 | 3907 | 7145 |
| TWOMILE CREEK | 14.123927 | 5406 | 3997 | 7311 |
| TWOMILE CREEK | 19.080039 | 6841 | 5058 | 9252 |
| TWOMILE CREEK | 19.622539 | 6841 | 5058 | 9252 |
| TWOMILE CREEK | 23.267161 | 7770 | 5745 | 10508 |
| TWOMILE CREEK | 24.100468 | 7927 | 5862 | 10721 |
| TWOMILE CREEK | 24.204626 | 7929 | 5863 | 10723 |
| TWOMILE CREEK | 25.572397 | 8004 | 5919 | 10825 |
| TWOMILE CREEK | 25.843754 | 8082 | 5976 | 10930 |
| TWOMILE CREEK | 27.414873 | 8309 | 6144 | 11237 |
| TWOMILE CREEK | 27.732143 | 8309 | 6144 | 11237 |
| UNT 001 IN BOCR | 0.079721 | 83 | 62 | 113 |
| UNT 002 IN BOCR | 0.958583 | 2197 | 1625 | 2971 |
| UNT 003 IN BOCR | 1.081931 | 2283 | 1688 | 3088 |
| UNT 004 IN BOCR | 1.004093 | 2242 | 1657 | 3031 |
| UNT 004 IN BOCR | 1.671276 | 3025 | 2237 | 4091 |
| UNT 004 IN BOCR | 1.730641 | 3101 | 2293 | 4194 |
| UNT 005 IN BOCR | 1.360821 | 3462 | 2560 | 4683 |
| UNT 005 IN BOCR | 1.911785 | 3876 | 2866 | 5241 |
| UNT 006 IN BOCR | 1.109102 | 2540 | 1879 | 3436 |
| UNT 007 IN BOCR | 1.071708 | 2706 | 2001 | 3660 |
| UNT 007 IN BOCR | 1.295447 | 3057 | 2261 | 4135 |
| UNT 009 IN BOCR | 1.629135 | 3309 | 2446 | 4474 |
| UNT 010 IN BOCR | 0.949032 | 2289 | 1693 | 3096 |
| UNT 012 IN BOCR | 1.133494 | 2779 | 2055 | 3759 |
| UNT 013 IN BOCR | 1.251686 | 3036 | 2245 | 4106 |
| UNT 013 IN BOCR | 2.276269 | 4919 | 3637 | 6653 |
| UNT 014 IN BOCR | 0.90832 | 2497 | 1846 | 3376 |
| UNT 015 IN BOCR | 1.301994 | 3166 | 2341 | 4282 |
| UNT 01639 IN BOCR | 0.635707 | 467 | 345 | 632 |
| UNT 017 IN BOCR | 0.628802 | 1423 | 1052 | 1925 |
| UNT 018 IN BOCR | 0.360853 | 1138 | 842 | 1539 |
| UNT 019 IN BOCR | 1.140175 | 2870 | 2122 | 3881 |
| UNT 019 IN BOCR | 2.379889 | 4416 | 3265 | 5972 |
| UNT 020 IN BOCR | 0.555313 | 1532 | 1133 | 2071 |
| UNT 022 IN BOCR | 1.206579 | 2394 | 1771 | 3238 |
| UNT 023 IN BOCR | 0.954771 | 1983 | 1467 | 2682 |
| UNT 024 IN BOCR | 0.815058 | 1933 | 1429 | 2614 |
| UNT 025 IN BOCR | 0.8807 | 993 | 734 | 1342 |
| UNT 025 IN BOCR | 1.762655 | 1715 | 1268 | 2319 |
| UNT 026 IN BOCR | 0.571858 | 818 | 605 | 1106 |
| UNT 026 IN BOCR | 0.673909 | 880 | 651 | 1190 |
| UNT 027 IN BOCR | 0.356369 | 531 | 393 | 719 |
| UNT 028 IN BOCR | 1.001044 | 1489 | 1101 | 2014 |
| UNT 028 IN BOCR | 1.691991 | 1622 | 1199 | 2193 |
| UNT 028 IN BOCR | 2.638243 | 2311 | 1709 | 3125 |
| UNT 028 IN BOCR | 3.198265 | 2484 | 1837 | 3359 |
| UNT 028 IN BOCR | 3.333405 | 2484 | 1837 | 3359 |
| UNT 029 IN BOCR | 0.273599 | 364 | 269 | 492 |
| UNT 031 IN BOCR | 1.287018 | 1231 | 910 | 1664 |
| UNT 031 IN BOCR | 1.452289 | 1238 | 915 | 1674 |
| UNT 031 IN BOCR | 2.591164 | 1929 | 1427 | 2609 |
| UNT 032 IN BOCR | 0.907692 | 827 | 611 | 1118 |
| UNT 033 IN BOCR | 1.127845 | 962 | 711 | 1301 |
| UNT 035 IN BOCR | 0.792639 | 1782 | 1318 | 2410 |
| UNT 037 IN BOCR | 1.01584 | 2075 | 1535 | 2807 |
| UNT 037 IN BOCR | 1.435968 | 2075 | 1535 | 2807 |
| UNT 038 IN BOCR | 0.485798 | 649 | 480 | 877 |
| UNT 038 IN BOCR | 0.768247 | 787 | 582 | 1064 |
| UNT 038 IN BOCR | 1.036376 | 858 | 635 | 1161 |
| UNT 038 IN BOCR | 1.444129 | 1180 | 873 | 1596 |
| UNT 039 IN BOCR | 0.17397 | 164 | 121 | 222 |
| UNT 044 IN BOCR | 0.402014 | 501 | 370 | 677 |
| UNT 047 IN BOCR | 0.952261 | 863 | 638 | 1167 |
| UNT 048 IN BOCR | 0.192488 | 225 | 166 | 304 |
| UNT 049 IN BOCR | 0.481287 | 501 | 371 | 678 |
| UNT 049 IN BOCR | 0.954054 | 882 | 652 | 1193 |
| UNT 050 IN BOCR | 0.442099 | 440 | 326 | 596 |
| UNT 052 IN BOCR | 1.232452 | 825 | 610 | 1115 |
| UNT 052 IN BOCR | 1.416901 | 1015 | 750 | 1372 |
| UNT 053 IN BOCR | 0.118999 | 143 | 105 | 193 |
| UNT 054 IN BOCR | 0.226788 | 261 | 193 | 352 |
| UNT 055 IN BOCR | 0.562263 | 508 | 376 | 687 |
| UNT 057 IN BOCR | 0.444161 | 511 | 378 | 691 |
| UNT 058 IN BOCR | 0.231362 | 327 | 242 | 442 |
| UNT 059 IN BOCR | 0.576253 | 523 | 387 | 707 |
| UNT 061 IN BOCR | 0.375291 | 357 | 264 | 483 |
| UNT 062 IN BOCR | 0.52684 | 451 | 333 | 609 |
| UNT 063 IN BOCR | 0.479672 | 497 | 367 | 672 |
| UNT 064 IN BOCR | 0.155855 | 189 | 140 | 255 |
| UNT 065 IN BOCR | 0.358521 | 405 | 300 | 548 |
| UNT 068 IN BOCR | 0.532054 | 472 | 349 | 639 |
| UNT 069 IN BOCR | 0.805867 | 663 | 490 | 896 |
| UNT 070 IN BOCR | 0.413761 | 390 | 289 | 528 |
| UNT 071 IN BOCR | 0.214593 | 322 | 238 | 435 |
| UNT 076 IN BOCR | 0.820282 | 860 | 636 | 1164 |
| UNT 077 IN BOCR | 0.30319 | 367 | 271 | 496 |
| UNT 078 IN BOCR | 0.482003 | 511 | 378 | 691 |
| UNT 078 IN BOCR | 0.496799 | 522 | 386 | 706 |
| UNT 079 IN BOCR | 0.814878 | 735 | 544 | 994 |
| UNT 080 IN BOCR | 0.556883 | 607 | 449 | 821 |
| UNT 080 IN BOCR | 0.612033 | 623 | 461 | 842 |
| UNT 081 IN BOCR | 0.120673 | 213 | 157 | 288 |
| UNT 082 IN BOCR | 0.124469 | 191 | 142 | 259 |
| UNT 083 IN BOCR | 0.65297 | 649 | 480 | 878 |
| UNT 084 IN BOCR | 0.925179 | 957 | 708 | 1294 |
| UNT 085 IN BOCR | 0.95504 | 855 | 632 | 1156 |
| UNT 085 IN BOCR | 1.741312 | 1362 | 1007 | 1842 |
| UNT 086 IN BOCR | 0.959076 | 720 | 533 | 974 |
| UNT 086 IN BOCR | 1.287108 | 957 | 707 | 1294 |
| UNT 087 IN BOCR | 0.178723 | 315 | 233 | 426 |
| UNT 088 IN BOCR | 0.389549 | 501 | 370 | 677 |
| UNT 089 IN BOCR | 0.220422 | 268 | 198 | 363 |
| UNT 090 IN BOCR | 0.659381 | 683 | 505 | 924 |
| UNT 091 IN BOCR | 0.189304 | 266 | 197 | 360 |
| UNT 092 IN BOCR | 0.035001 | 116 | 86 | 157 |
| YELLOW BRANCH | 1.157527 | 2807 | 2076 | 3797 |
| YELLOW BRANCH | 2.201884 | 4183 | 3093 | 5657 |
| YELLOW BRANCH | 2.47889 | 4769 | 3526 | 6449 |
| Caney Creek - Bedias Creek |  |  |  |  |
| BIG TURKEY CREEK | 0.97809 | 1062 | 785 | 1436 |
| BIG TURKEY CREEK | 1.44547 | 1310 | 969 | 1772 |
| BIG TURKEY CREEK | 2.58758 | 2136 | 1579 | 2889 |
| BIG TURKEY CREEK | 3.84464 | 2816 | 2082 | 3808 |
| BIG TURKEY CREEK | 4.54922 | 2934 | 2169 | 3968 |
| BIG TURKEY CREEK | 6.96014 | 3937 | 2911 | 5324 |
| BIG TURKEY CREEK | 7.09842 | 4115 | 3042 | 5565 |
| BISHOP CREEK | 0.95943 | 1188 | 878 | 1607 |
| BISHOP CREEK | 1.56779 | 1661 | 1228 | 2247 |
| BISHOP CREEK | 2.09616 | 2037 | 1506 | 2754 |
| BISHOP CREEK | 2.44948 | 2270 | 1679 | 3070 |
| BONEY BRANCH | 0.47429 | 784 | 579 | 1060 |
| BONEY BRANCH | 2.03437 | 2476 | 1831 | 3349 |
| BONEY BRANCH | 2.39137 | 2764 | 2043 | 3737 |
| BONEY BRANCH | 2.96161 | 2903 | 2146 | 3926 |
| BONEY BRANCH | 4.4973 | 4051 | 2996 | 5479 |
| BOWMAN CREEK | 1.05413 | 1206 | 892 | 1631 |
| BOWMAN CREEK | 2.00245 | 1969 | 1456 | 2663 |
| BOWMAN CREEK | 2.96968 | 2599 | 1922 | 3515 |
| BOWMAN CREEK | 3.89908 | 3038 | 2246 | 4108 |
| BOWMAN CREEK | 4.20711 | 3326 | 2459 | 4498 |
| BOWMAN CREEK | 4.81179 | 3433 | 2538 | 4643 |
| BOWMAN CREEK | 5.80171 | 3970 | 2935 | 5369 |
| BOWMAN CREEK | 6.74223 | 4347 | 3214 | 5879 |
| BOWMAN CREEK | 7.74103 | 4901 | 3624 | 6628 |
| BOWMAN CREEK | 8.87425 | 5208 | 3851 | 7044 |
| BRUSHY CREEK 1 | 1.06462 | 1456 | 1077 | 1970 |
| BRUSHY CREEK 1 | 1.91663 | 2147 | 1588 | 2903 |
| BRUSHY CREEK 1 | 2.32017 | 2451 | 1812 | 3314 |
| BRUSHY CREEK 1 | 2.74711 | 2690 | 1989 | 3638 |
| BRUSHY CREEK 2 | 0.70269 | 1558 | 1152 | 2108 |
| BRUSHY CREEK 2 | 1.98083 | 3105 | 2296 | 4199 |
| BRUSHY CREEK 2 | 2.67402 | 3982 | 2944 | 5385 |
| BRUSHY CREEK 2 | 3.00322 | 4336 | 3206 | 5863 |
| BRUSHY CREEK 2 | 4.29948 | 5434 | 4018 | 7348 |
| BRUSHY CREEK 2 | 7.29812 | 6421 | 4748 | 8683 |
| BRUSHY CREEK 2 | 8.63617 | 6985 | 5165 | 9446 |
| BRUSHY CREEK 2 | 9.65192 | 7342 | 5429 | 9929 |
| BRUSHY CREEK 2 | 10.52975 | 7606 | 5624 | 10286 |
| BRUSHY CREEK 2 | 11.50084 | 7741 | 5724 | 10469 |
| BRUSHY CREEK 2 | 12.16834 | 8036 | 5942 | 10867 |
| BRUSHY CREEK 2 | 13.40258 | 8116 | 6001 | 10976 |
| CANE BRANCH | 1.28648 | 1769 | 1308 | 2393 |
| CANE BRANCH | 2.57834 | 2945 | 2178 | 3983 |
| CANEY CREEK | 1.01064 | 1925 | 1423 | 2603 |
| CANEY CREEK | 1.93932 | 3128 | 2313 | 4230 |
| CANEY CREEK | 2.22717 | 3395 | 2510 | 4591 |
| CANEY CREEK | 4.58473 | 6007 | 4442 | 8123 |
| CANEY CREEK | 5.06288 | 6257 | 4627 | 8463 |
| CANEY CREEK | 5.46104 | 6504 | 4810 | 8796 |
| CANEY CREEK | 5.7515 | 6701 | 4955 | 9063 |
| CANEY CREEK | 10.95911 | 11361 | 8400 | 15364 |
| CANEY CREEK | 11.44336 | 11540 | 8533 | 15607 |
| CANEY CREEK | 12.25411 | 11666 | 8626 | 15777 |
| CANEY CREEK | 13.8462 | 12424 | 9187 | 16802 |
| CANEY CREEK | 19.35307 | 16053 | 11870 | 21710 |
| CANEY CREEK | 21.55405 | 16820 | 12437 | 22747 |
| CANEY CREEK | 22.86761 | 17213 | 12728 | 23279 |
| CANEY CREEK | 44.65267 | 28745 | 21255 | 38874 |
| CANEY CREEK | 64.08241 | 33274 | 24604 | 44999 |
| CANEY CREEK | 65.58196 | 33349 | 24660 | 45101 |
| CANEY CREEK | 66.42212 | 33670 | 24897 | 45535 |
| CANEY CREEK | 66.74585 | 33670 | 24897 | 45535 |
| CANEY CREEK | 72.05471 | 33670 | 24897 | 45535 |
| CANEY CREEK | 72.846 | 33670 | 24897 | 45535 |
| CANEY CREEK | 87.19223 | 34778 | 25716 | 47033 |
| CANEY CREEK | 94.90708 | 34778 | 25716 | 47033 |
| CANEY CREEK | 96.84541 | 34778 | 25716 | 47033 |
| CANEY CREEK | 103.03819 | 35123 | 25971 | 47500 |
| CANEY CREEK | 110.43146 | 36195 | 26764 | 48949 |
| CANEY CREEK | 110.57019 | 36195 | 26764 | 48949 |
| CANEY CREEK | 112.63998 | 36195 | 26764 | 48949 |
| CANEY CREEK | 113.00102 | 36195 | 26764 | 48949 |
| CANEY CREEK | 114.00071 | 36195 | 26764 | 48949 |
| CANEY CREEK | 114.59723 | 36195 | 26764 | 48949 |
| CANEY CREEK | 115.64177 | 36195 | 26764 | 48949 |
| CANEY CREEK | 115.81493 | 36195 | 26764 | 48949 |
| CANEY CREEK | 120.98677 | 36195 | 26764 | 48949 |
| CANEY CREEK | 123.90722 | 36547 | 27024 | 49425 |
| CANEY CREEK | 126.78498 | 36935 | 27311 | 49951 |
| CANEY CREEK | 128.49043 | 37074 | 27413 | 50138 |
| CANEY CREEK | 129.59944 | 37074 | 27413 | 50138 |
| CANEY CREEK | 130.6907 | 37074 | 27413 | 50138 |
| CANEY CREEK | 131.32344 | 37074 | 27413 | 50138 |
| CANEY CREEK | 132.99768 | 37074 | 27413 | 50138 |
| CANEY CREEK | 134.02435 | 37074 | 27413 | 50138 |
| CANEY CREEK | 134.75244 | 37074 | 27413 | 50138 |
| CANEY CREEK | 135.28907 | 37074 | 27413 | 50138 |
| CANEY CREEK | 136.38301 | 37074 | 27413 | 50138 |
| CANEY CREEK | 136.71355 | 37074 | 27413 | 50138 |
| CEDAR CREEK | 0.76502 | 897 | 663 | 1213 |
| CEDAR CREEK | 1.63477 | 1601 | 1184 | 2165 |
| CEDAR CREEK | 1.97626 | 1874 | 1385 | 2534 |
| CHOPIN BRANCH | 1.00015 | 818 | 605 | 1106 |
| CHOPIN BRANCH | 1.63567 | 1203 | 890 | 1627 |
| CHOPIN BRANCH | 2.55628 | 1684 | 1245 | 2277 |
| CHOPIN BRANCH | 3.13271 | 1684 | 1245 | 2277 |
| CHOPIN BRANCH | 4.58688 | 2205 | 1630 | 2982 |
| COON BRANCH | 1.22236 | 1521 | 1125 | 2057 |
| COON BRANCH | 1.53318 | 1729 | 1278 | 2338 |
| COPELAND BRANCH | 0.9682 | 1644 | 1215 | 2223 |
| COPELAND BRANCH | 1.39526 | 2105 | 1556 | 2847 |
| COPELAND BRANCH | 2.02325 | 2774 | 2051 | 3752 |
| COPELAND BRANCH | 3.15845 | 3982 | 2944 | 5385 |
| COPELAND BRANCH | 3.55903 | 4233 | 3130 | 5724 |
| DEADMAN HOLLOW BRANCH | 1.00015 | 1577 | 1166 | 2132 |
| DEADMAN HOLLOW BRANCH | 1.4227 | 2181 | 1613 | 2949 |
| EAST CANEY CREEK | 1.35634 | 2844 | 2103 | 3846 |
| EAST CANEY CREEK | 1.54752 | 3034 | 2243 | 4103 |
| EAST CANEY CREEK | 3.37475 | 5551 | 4105 | 7507 |
| EAST CANEY CREEK | 4.37041 | 6347 | 4693 | 8584 |
| EAST CANEY CREEK | 4.43838 | 6347 | 4693 | 8584 |
| EAST CANEY CREEK | 8.09928 | 10059 | 7438 | 13603 |
| EAST CANEY CREEK | 13.67188 | 13746 | 10164 | 18589 |
| EAST CANEY CREEK | 14.41555 | 13780 | 10189 | 18635 |
| EAST CANEY CREEK | 15.13232 | 13935 | 10304 | 18845 |
| EAST CANEY CREEK | 19.64442 | 16412 | 12136 | 22195 |
| EAST CANEY CREEK | 20.46952 | 16649 | 12311 | 22516 |
| EAST CANEY CREEK | 21.38223 | 16649 | 12311 | 22516 |
| EAST CANEY CREEK | 21.70354 | 16769 | 12400 | 22679 |
| FERRY BRANCH | 1.11054 | 1511 | 1117 | 2043 |
| FERRY BRANCH | 1.82328 | 2057 | 1521 | 2782 |
| FERRY BRANCH | 2.81876 | 2861 | 2115 | 3869 |
| FERRY BRANCH | 3.74986 | 3408 | 2520 | 4609 |
| FERRY BRANCH | 4.58778 | 3617 | 2675 | 4892 |
| FERRY BRANCH | 6.18481 | 4639 | 3430 | 6274 |
| FOARDS BRANCH | 1.10031 | 1133 | 838 | 1532 |
| FOARDS BRANCH | 2.3251 | 1989 | 1470 | 2689 |
| FOARDS BRANCH | 2.92377 | 2284 | 1689 | 3088 |
| GREENBRIER CREEK | 1.05942 | 906 | 670 | 1225 |
| GREENBRIER CREEK | 2.022 | 1485 | 1098 | 2008 |
| GREENBRIER CREEK | 2.99147 | 1890 | 1397 | 2556 |
| GREENBRIER CREEK | 3.54073 | 2079 | 1537 | 2812 |
| GREENBRIER CREEK | 5.85337 | 3073 | 2273 | 4157 |
| GREENBRIER CREEK | 7.27014 | 3621 | 2677 | 4896 |
| HOLLIS BRANCH | 1.07682 | 2185 | 1615 | 2955 |
| HOLLIS BRANCH | 2.07867 | 3479 | 2572 | 4705 |
| HOLLIS BRANCH | 3.07729 | 4362 | 3225 | 5899 |
| HOLLIS BRANCH | 3.08877 | 4362 | 3225 | 5899 |
| HOLLIS BRANCH | 3.88168 | 5141 | 3801 | 6952 |
| HOLLIS BRANCH | 4.87815 | 5993 | 4431 | 8105 |
| HOLLIS BRANCH | 4.89115 | 6018 | 4450 | 8139 |
| IRON CREEK | 0.56495 | 1151 | 851 | 1557 |
| IRON CREEK | 1.00167 | 1608 | 1189 | 2175 |
| IRON CREEK | 1.75432 | 2317 | 1713 | 3134 |
| IRON CREEK | 3.08474 | 3264 | 2414 | 4414 |
| IRON CREEK | 4.19626 | 3611 | 2670 | 4884 |
| IRON CREEK | 5.50363 | 3940 | 2913 | 5329 |
| IRON CREEK | 7.05089 | 4412 | 3263 | 5967 |
| IRON CREEK | 7.98593 | 4790 | 3542 | 6478 |
| IRON CREEK | 8.54237 | 4978 | 3681 | 6733 |
| IRON CREEK | 8.84215 | 4978 | 3681 | 6733 |
| IRON CREEK | 12.23169 | 6092 | 4505 | 8239 |
| IRON CREEK | 13.19319 | 6104 | 4513 | 8254 |
| IRON CREEK | 13.5466 | 6174 | 4566 | 8350 |
| IRON CREEK | 16.01446 | 6892 | 5096 | 9321 |
| IRON CREEK | 16.70092 | 7056 | 5218 | 9543 |
| IRON CREEK | 17.78859 | 7369 | 5449 | 9966 |
| IRON CREEK | 19.43736 | 7662 | 5665 | 10362 |
| IRON CREEK | 20.43015 | 7700 | 5694 | 10414 |
| IRON CREEK | 21.20728 | 8021 | 5931 | 10847 |
| IRON CREEK | 22.14169 | 8216 | 6075 | 11111 |
| IRON CREEK | 22.8868 | 8216 | 6075 | 11111 |
| IRON CREEK | 29.12488 | 9824 | 7264 | 13286 |
| IRON CREEK | 30.05858 | 10003 | 7397 | 13528 |
| IRON CREEK | 31.25879 | 10035 | 7420 | 13571 |
| IRON CREEK | 32.07142 | 10035 | 7420 | 13571 |
| IRON CREEK | 32.93688 | 10115 | 7479 | 13679 |
| IRON CREEK | 34.35769 | 10705 | 7916 | 14477 |
| IRON CREEK | 35.26951 | 10909 | 8067 | 14754 |
| IRON CREEK | 35.46787 | 10909 | 8067 | 14754 |
| IRON CREEK | 35.93095 | 10909 | 8067 | 14754 |
| TEHUACANA CREEK | 0.92096 | 996 | 737 | 1348 |
| TEHUACANA CREEK | 1.91304 | 1633 | 1208 | 2209 |
| TEHUACANA CREEK | 3.84321 | 2908 | 2150 | 3933 |
| TEHUACANA CREEK | 5.63008 | 3571 | 2641 | 4830 |
| TEHUACANA CREEK | 6.17082 | 3876 | 2866 | 5242 |
| TEHUACANA CREEK | 7.12864 | 4442 | 3285 | 6007 |
| TEHUACANA CREEK | 8.16277 | 4643 | 3433 | 6279 |
| TEHUACANA CREEK | 9.5617 | 5001 | 3698 | 6764 |
| TEHUACANA CREEK | 13.04442 | 5892 | 4356 | 7968 |
| TEHUACANA CREEK | 14.48855 | 6394 | 4728 | 8648 |
| TEHUACANA CREEK | 23.2104 | 9255 | 6843 | 12516 |
| TEHUACANA CREEK | 24.66681 | 9255 | 6843 | 12516 |
| TEHUACANA CREEK | 26.20519 | 9255 | 6843 | 12516 |
| TEHUACANA CREEK | 28.18046 | 9822 | 7263 | 13284 |
| TEHUACANA CREEK | 30.01486 | 10245 | 7576 | 13855 |
| TEHUACANA CREEK | 30.76114 | 10245 | 7576 | 13855 |
| TEHUACANA CREEK | 32.25571 | 10327 | 7636 | 13966 |
| TEHUACANA CREEK | 34.52332 | 10713 | 7921 | 14488 |
| TEHUACANA CREEK | 35.74793 | 10943 | 8092 | 14799 |
| TEHUACANA CREEK | 36.88061 | 10943 | 8092 | 14799 |
| MIDDLE BRANCH RILEY CREEK | 1.19698 | 2003 | 1481 | 2709 |
| MIDDLE BRANCH RILEY CREEK | 1.60743 | 2534 | 1874 | 3427 |
| MIDDLE BRANCH RILEY CREEK | 2.1019 | 2931 | 2167 | 3963 |
| MIDDLE BRANCH RILEY CREEK | 2.74881 | 3461 | 2559 | 4681 |
| MUSTANG CREEK | 1.17062 | 966 | 714 | 1306 |
| MUSTANG CREEK | 3.51482 | 2579 | 1907 | 3488 |
| MUSTANG CREEK | 4.38763 | 3156 | 2334 | 4268 |
| MUSTANG CREEK | 6.29717 | 4218 | 3119 | 5705 |
| MUSTANG CREEK | 7.22845 | 4562 | 3373 | 6170 |
| NEELEY CREEK | 0.46497 | 589 | 436 | 797 |
| NEELEY CREEK | 1.02005 | 1161 | 858 | 1570 |
| NEELEY CREEK | 1.72741 | 1601 | 1184 | 2166 |
| NEELEY CREEK | 2.54534 | 2216 | 1639 | 2997 |
| NEELEY CREEK | 4.43264 | 2982 | 2205 | 4033 |
| NORTH BEDIAS CREEK | 1.04355 | 1179 | 872 | 1594 |
| NORTH BEDIAS CREEK | 2.28421 | 1739 | 1286 | 2352 |
| NORTH BEDIAS CREEK | 3.32569 | 2350 | 1738 | 3178 |
| NORTH BEDIAS CREEK | 6.88858 | 4061 | 3003 | 5492 |
| NORTH BEDIAS CREEK | 8.83785 | 4776 | 3532 | 6459 |
| NORTH BEDIAS CREEK | 13.28197 | 6558 | 4849 | 8869 |
| NORTH BEDIAS CREEK | 14.17702 | 6576 | 4863 | 8893 |
| NORTH BEDIAS CREEK | 14.6939 | 6630 | 4902 | 8966 |
| NORTH BEDIAS CREEK | 16.57583 | 7197 | 5322 | 9733 |
| NORTH BEDIAS CREEK | 17.48603 | 7197 | 5322 | 9733 |
| NORTH BEDIAS CREEK | 17.73577 | 7197 | 5322 | 9733 |
| PANKY CREEK | 1.11421 | 1416 | 1047 | 1915 |
| PANKY CREEK | 4.63621 | 3584 | 2650 | 4847 |
| PANKY CREEK | 5.59349 | 4126 | 3051 | 5580 |
| PANKY CREEK | 6.48289 | 4480 | 3313 | 6058 |
| PANKY CREEK | 7.38385 | 4776 | 3532 | 6460 |
| PANKY CREEK | 8.1773 | 4981 | 3683 | 6736 |
| PANKY CREEK | 9.17718 | 5426 | 4012 | 7338 |
| PANKY CREEK | 9.19215 | 5440 | 4022 | 7356 |
| POND BRANCH | 1.0891 | 2054 | 1519 | 2778 |
| POND BRANCH | 2.20726 | 3436 | 2540 | 4646 |
| POND BRANCH | 4.53326 | 5472 | 4046 | 7400 |
| POND BRANCH | 5.27362 | 5861 | 4334 | 7926 |
| POND BRANCH | 5.71733 | 6196 | 4582 | 8379 |
| POOLES BRANCH | 1.39768 | 1332 | 985 | 1801 |
| POOLES BRANCH | 2.39495 | 1856 | 1373 | 2510 |
| POOLES BRANCH | 3.27691 | 2427 | 1794 | 3282 |
| POOLES BRANCH | 4.37785 | 2915 | 2155 | 3942 |
| POOLES BRANCH | 5.49278 | 3353 | 2480 | 4535 |
| REED BRANCH | 1.01494 | 1270 | 939 | 1717 |
| REED BRANCH | 1.87233 | 2068 | 1529 | 2797 |
| REED BRANCH | 2.4771 | 2384 | 1763 | 3225 |
| RILEY CREEK | 1.03987 | 2226 | 1646 | 3011 |
| RILEY CREEK | 1.65729 | 2905 | 2148 | 3928 |
| RILEY CREEK | 2.517 | 3340 | 2470 | 4517 |
| RILEY CREEK | 3.03739 | 3761 | 2781 | 5087 |
| RILEY CREEK | 3.94849 | 3967 | 2933 | 5364 |
| RILEY CREEK | 4.89456 | 4083 | 3019 | 5522 |
| RILEY CREEK | 8.57985 | 6580 | 4866 | 8899 |
| RILEY CREEK | 9.44728 | 6580 | 4866 | 8899 |
| RILEY CREEK | 15.61496 | 10921 | 8075 | 14769 |
| RILEY CREEK | 18.88263 | 11409 | 8436 | 15429 |
| ROCKY BRANCH | 1.07117 | 2164 | 1600 | 2926 |
| ROCKY BRANCH | 1.86067 | 3096 | 2289 | 4187 |
| ROCKY BRANCH | 2.64264 | 4034 | 2983 | 5455 |
| ROCKY BRANCH | 4.74947 | 5979 | 4421 | 8086 |
| ROCKY BRANCH | 5.38194 | 6676 | 4936 | 9028 |
| ROCKY CREEK | 1.01782 | 1318 | 975 | 1783 |
| ROCKY CREEK | 1.13654 | 1329 | 982 | 1797 |
| ROCKY CREEK | 1.39499 | 1622 | 1199 | 2194 |
| ROCKY CREEK | 5.97147 | 4735 | 3501 | 6403 |
| ROCKY CREEK | 7.60732 | 5302 | 3920 | 7170 |
| SALT CREEK | 1.09242 | 1965 | 1453 | 2658 |
| SALT CREEK | 3.92096 | 5356 | 3960 | 7243 |
| SALT CREEK | 5.33522 | 6372 | 4712 | 8617 |
| SAND BRANCH | 1.36656 | 1132 | 837 | 1530 |
| SAND BRANCH | 2.53332 | 1584 | 1172 | 2143 |
| SIMES CREEK | 0.78484 | 998 | 738 | 1350 |
| SIMES CREEK | 1.2464 | 1469 | 1087 | 1987 |
| SIMES CREEK | 1.99438 | 2000 | 1479 | 2705 |
| SIMES CREEK | 2.63753 | 2353 | 1740 | 3182 |
| SIMES CREEK | 3.73712 | 3129 | 2314 | 4231 |
| SIMES CREEK | 7.1541 | 5084 | 3759 | 6875 |
| SIMES CREEK | 8.11255 | 5429 | 4014 | 7342 |
| SIMES CREEK | 9.93619 | 6271 | 4637 | 8481 |
| SIMES CREEK | 9.95762 | 6271 | 4637 | 8481 |
| SIMES CREEK | 10.53235 | 6320 | 4673 | 8547 |
| SIMES CREEK | 10.64749 | 6458 | 4775 | 8733 |
| TOWN BRANCH | 1.08515 | 1144 | 846 | 1546 |
| TOWN BRANCH | 1.5417 | 1734 | 1282 | 2345 |
| TOWN BRANCH | 1.95097 | 2066 | 1528 | 2795 |
| TOWN BRANCH | 2.82952 | 2801 | 2071 | 3788 |
| TOWN BRANCH | 3.36766 | 2831 | 2093 | 3828 |
| TOWN BRANCH | 4.69243 | 3662 | 2708 | 4953 |
| TOWN BRANCH | 4.90433 | 3708 | 2742 | 5014 |
| TWISTED CREEK | 1.15044 | 1559 | 1153 | 2108 |
| TWISTED CREEK | 1.44 | 1880 | 1390 | 2542 |
| UNT 001 in CCBC | 0.57087 | 558 | 413 | 755 |
| UNT 002 in CCBC | 0.86294 | 895 | 662 | 1211 |
| UNT 003 in CCBC | 0.482 | 577 | 427 | 781 |
| UNT 003 in CCBC | 1.0909 | 1163 | 860 | 1572 |
| UNT 003 in CCBC | 2.57691 | 2196 | 1624 | 2970 |
| UNT 004 in CCBC | 0.21692 | 408 | 302 | 552 |
| UNT 005 in CCBC | 0.73839 | 797 | 590 | 1078 |
| UNT 006 in CCBC | 0.70243 | 732 | 541 | 990 |
| UNT 007 in CCBC | 0.38758 | 424 | 314 | 574 |
| UNT 008 in CCBC | 0.59293 | 712 | 526 | 963 |
| UNT 008 in CCBC | 1.08301 | 1128 | 834 | 1525 |
| UNT 009 in CCBC | 0.30759 | 431 | 319 | 583 |
| UNT 010 in CCBC | 0.47967 | 623 | 460 | 842 |
| UNT 011 in CCBC | 0.42273 | 568 | 420 | 768 |
| UNT 011 in CCBC | 0.66189 | 795 | 588 | 1075 |
| UNT 011 in CCBC | 1.16129 | 1224 | 905 | 1656 |
| UNT 011 in CCBC | 1.86829 | 1809 | 1338 | 2446 |
| UNT 012 in CCBC | 0.35 | 440 | 325 | 595 |
| UNT 012 in CCBC | 0.44022 | 569 | 421 | 770 |
| UNT 013 in CCBC | 0.27476 | 381 | 282 | 515 |
| UNT 013 in CCBC | 0.53895 | 652 | 482 | 882 |
| UNT 014 in CCBC | 0.54998 | 649 | 480 | 877 |
| UNT 015 in CCBC | 1.20236 | 1232 | 911 | 1666 |
| UNT 015 in CCBC | 1.90075 | 1708 | 1263 | 2309 |
| UNT 015 in CCBC | 2.66228 | 2253 | 1666 | 3046 |
| UNT 016 in CCBC | 0.95199 | 1121 | 829 | 1516 |
| UNT 016 in CCBC | 1.82525 | 1760 | 1301 | 2380 |
| UNT 017 in CCBC | 0.24167 | 362 | 267 | 489 |
| UNT 018 in CCBC | 0.67023 | 820 | 606 | 1109 |
| UNT 018 in CCBC | 1.1004 | 1126 | 833 | 1523 |
| UNT 019 in CCBC | 0.78932 | 788 | 583 | 1066 |
| UNT 019 in CCBC | 1.33751 | 1210 | 895 | 1636 |
| UNT 020 in CCBC | 0.19199 | 262 | 194 | 354 |
| UNT 021 in CCBC | 1.0145 | 1030 | 761 | 1393 |
| UNT 021 in CCBC | 1.96891 | 1657 | 1225 | 2240 |
| UNT 021 in CCBC | 2.85965 | 2047 | 1514 | 2769 |
| UNT 021 in CCBC | 3.28834 | 2355 | 1742 | 3185 |
| UNT 022 in CCBC | 1.00069 | 966 | 714 | 1307 |
| UNT 022 in CCBC | 1.42673 | 1299 | 961 | 1757 |
| UNT 023 in CCBC | 0.40982 | 619 | 458 | 837 |
| UNT 024 in CCBC | 0.23558 | 420 | 311 | 568 |
| UNT 025 in CCBC | 0.24472 | 395 | 292 | 535 |
| UNT 026 in CCBC | 0.27315 | 554 | 410 | 750 |
| UNT 027 in CCBC | 0.92087 | 1364 | 1008 | 1844 |
| UNT 027 in CCBC | 1.45803 | 1815 | 1342 | 2455 |
| UNT 028 in CCBC | 0.15899 | 368 | 272 | 497 |
| UNT 029 in CCBC | 1.37086 | 1263 | 934 | 1709 |
| UNT 029 in CCBC | 1.92882 | 1583 | 1170 | 2140 |
| UNT 030 in CCBC | 0.34068 | 612 | 452 | 827 |
| UNT 031 in CCBC | 0.2675 | 510 | 377 | 690 |
| UNT 032 in CCBC | 1.0014 | 1225 | 906 | 1657 |
| UNT 032 in CCBC | 1.97196 | 1969 | 1456 | 2663 |
| UNT 032 in CCBC | 2.55458 | 2369 | 1752 | 3204 |
| UNT 035 in CCBC | 1.05261 | 1194 | 883 | 1615 |
| UNT 035 in CCBC | 1.82686 | 1692 | 1251 | 2288 |
| UNT 035 in CCBC | 2.70962 | 2474 | 1830 | 3346 |
| UNT 035 in CCBC | 4.31176 | 3582 | 2649 | 4844 |
| UNT 036 in CCBC | 0.34426 | 571 | 422 | 772 |
| UNT 037 in CCBC | 0.52352 | 605 | 447 | 818 |
| UNT 037 in CCBC | 1.13412 | 1196 | 884 | 1617 |
| UNT 037 in CCBC | 1.49542 | 1476 | 1092 | 1996 |
| UNT 038 in CCBC | 0.28714 | 486 | 359 | 657 |
| UNT 038 in CCBC | 0.53841 | 855 | 632 | 1156 |
| UNT 039 in CCBC | 0.36318 | 456 | 337 | 616 |
| UNT 040 in CCBC | 0.27844 | 502 | 371 | 679 |
| UNT 041 in CCBC | 0.20858 | 384 | 284 | 519 |
| UNT 042 in CCBC | 1.22882 | 1407 | 1041 | 1903 |
| UNT 043 in CCBC | 1.19385 | 1098 | 812 | 1486 |
| UNT 043 in CCBC | 1.55811 | 1480 | 1094 | 2001 |
| UNT 044 in CCBC | 0.95755 | 1079 | 798 | 1460 |
| UNT 045 in CCBC | 1.2472 | 1170 | 865 | 1582 |
| UNT 046 in CCBC | 1.16887 | 3602 | 2663 | 4871 |
| UNT 046 in CCBC | 2.39706 | 4243 | 3137 | 5738 |
| UNT 046 in CCBC | 6.11203 | 1129 | 835 | 1526 |
| UNT 046 in CCBC | 7.43923 | 1576 | 1165 | 2131 |
| UNT 047 in CCBC | 1.00185 | 1970 | 1457 | 2665 |
| UNT 047 in CCBC | 1.63989 | 1064 | 787 | 1439 |
| UNT 047 in CCBC | 2.42849 | 1789 | 1323 | 2420 |
| UNT 049 in CCBC | 0.94141 | 1066 | 788 | 1442 |
| UNT 049 in CCBC | 1.20568 | 1142 | 844 | 1545 |
| UNT 050 in CCBC | 0.97647 | 897 | 663 | 1213 |
| UNT 051 in CCBC | 0.90303 | 809 | 598 | 1095 |
| UNT 051 in CCBC | 1.71028 | 1398 | 1033 | 1890 |
| UNT 052 in CCBC | 1.24917 | 1617 | 1196 | 2187 |
| UNT 052 in CCBC | 1.52412 | 1661 | 1228 | 2246 |
| UNT 053 in CCBC | 1.30258 | 1396 | 1032 | 1887 |
| UNT 054 in CCBC | 0.92939 | 1029 | 761 | 1391 |
| UNT 054 in CCBC | 1.47121 | 1463 | 1082 | 1979 |
| UNT 054 in CCBC | 2.16314 | 1941 | 1435 | 2625 |
| UNT 055 in CCBC | 0.35673 | 478 | 353 | 646 |
| UNT 056 in CCBC | 0.43869 | 626 | 463 | 846 |
| UNT 057 in CCBC | 0.35045 | 519 | 384 | 702 |
| UNT 058 in CCBC | 1.17349 | 1506 | 1114 | 2037 |
| UNT 059 in CCBC | 1.10587 | 1058 | 782 | 1430 |
| UNT 060 in CCBC | 0.96311 | 1055 | 780 | 1427 |
| UNT 061 in CCBC | 0.92724 | 838 | 620 | 1134 |
| UNT 061 in CCBC | 1.31688 | 1117 | 826 | 1510 |
| UNT 061 in CCBC | 1.36235 | 1146 | 848 | 1550 |
| UNT 061 in CCBC | 1.89627 | 1492 | 1103 | 2018 |
| UNT 061 in CCBC | 2.31703 | 1765 | 1305 | 2387 |
| UNT 061 in CCBC | 2.34796 | 1765 | 1305 | 2387 |
| UNT 061 in CCBC | 2.66192 | 1864 | 1378 | 2520 |
| UNT 062 in CCBC | 1.4696 | 1407 | 1040 | 1902 |
| UNT 063 in CCBC | 1.02275 | 1103 | 815 | 1491 |
| UNT 064 in CCBC | 1.06857 | 1035 | 765 | 1399 |
| UNT 065 in CCBC | 0.70225 | 875 | 647 | 1183 |
| UNT 066 in CCBC | 0.16303 | 270 | 200 | 365 |
| UNT 067 in CCBC | 1.02938 | 2077 | 1536 | 2809 |
| UNT 067 in CCBC | 1.36638 | 2387 | 1765 | 3229 |
| UNT 068 in CCBC | 0.28481 | 883 | 653 | 1194 |
| UNT 069 in CCBC | 0.18383 | 511 | 378 | 691 |
| UNT 070 in CCBC | 0.36632 | 932 | 689 | 1261 |
| UNT 071 in CCBC | 0.1181 | 332 | 246 | 449 |
| UNT 072 in CCBC | 0.85003 | 1631 | 1206 | 2206 |
| UNT 073 in CCBC | 1.28944 | 2595 | 1919 | 3509 |
| UNT 073 in CCBC | 1.82507 | 3133 | 2317 | 4237 |
| UNT 074 in CCBC | 0.49124 | 994 | 735 | 1345 |
| UNT 074 in CCBC | 0.5672 | 1098 | 812 | 1485 |
| UNT 075 in CCBC | 0.76 | 1377 | 1018 | 1863 |
| UNT 075 in CCBC | 1.51981 | 2498 | 1847 | 3378 |
| UNT 076 in CCBC | 0.48236 | 1137 | 840 | 1537 |
| UNT 077 in CCBC | 0.53599 | 1254 | 927 | 1695 |
| UNT 077 in CCBC | 1.02485 | 2076 | 1535 | 2808 |
| UNT 078 in CCBC | 0.29799 | 774 | 573 | 1047 |
| UNT 079 in CCBC | 1.22819 | 3302 | 2442 | 4466 |
| UNT 080 in CCBC | 1.09108 | 2305 | 1705 | 3118 |
| UNT 080 in CCBC | 2.06934 | 3413 | 2524 | 4615 |
| UNT 080 in CCBC | 3.06573 | 4474 | 3309 | 6051 |
| UNT 081 in CCBC | 1.00454 | 2366 | 1749 | 3199 |
| UNT 081 in CCBC | 1.1439 | 2437 | 1802 | 3296 |
| UNT 082 in CCBC | 0.37126 | 738 | 546 | 998 |
| UNT 083 in CCBC | 1.10354 | 1826 | 1350 | 2470 |
| UNT 083 in CCBC | 1.13071 | 1826 | 1350 | 2470 |
| UNT 084 in CCBC | 1.00382 | 902 | 667 | 1220 |
| UNT 084 in CCBC | 1.43077 | 1132 | 837 | 1530 |
| UNT 085 in CCBC | 0.67382 | 1537 | 1137 | 2079 |
| UNT 086 in CCBC | 0.31225 | 581 | 429 | 785 |
| UNT 086 in CCBC | 0.43448 | 847 | 626 | 1146 |
| UNT 086 in CCBC | 0.65831 | 1172 | 866 | 1585 |
| UNT 087 in CCBC | 0.12178 | 291 | 215 | 394 |
| UNT 088 in CCBC | 0.26831 | 647 | 479 | 876 |
| UNT 089 in CCBC | 0.45134 | 1067 | 789 | 1443 |
| UNT 089 in CCBC | 0.63033 | 1302 | 963 | 1761 |
| UNT 090 in CCBC | 1.19779 | 1347 | 996 | 1821 |
| UNT 090 in CCBC | 1.98622 | 1974 | 1459 | 2669 |
| UNT 090 in CCBC | 2.14826 | 2092 | 1547 | 2829 |
| UNT 091 in CCBC | 1.41068 | 1169 | 865 | 1581 |
| UNT 092 in CCBC | 1.39283 | 1476 | 1091 | 1996 |
| UNT 093 in CCBC | 1.34845 | 1197 | 885 | 1619 |
| UNT 094 in CCBC | 1.08893 | 1276 | 944 | 1726 |
| UNT 094 in CCBC | 1.57317 | 1700 | 1257 | 2299 |
| UNT 094 in CCBC | 1.66993 | 1883 | 1392 | 2546 |
| UNT 094 in CCBC | 1.86291 | 1999 | 1478 | 2703 |
| UNT 095 in CCBC | 0.32525 | 493 | 365 | 667 |
| UNT 096 in CCBC | 1.50376 | 1597 | 1181 | 2159 |
| UNT 097 in CCBC | 0.3657 | 753 | 557 | 1019 |
| UNT 098 in CCBC | 1.16237 | 1418 | 1049 | 1918 |
| UNT 099 in CCBC | 1.0231 | 1219 | 902 | 1649 |
| UNT 099 in CCBC | 2.07598 | 1998 | 1477 | 2702 |
| UNT 100 in CCBC | 1.43059 | 1248 | 923 | 1688 |
| UNT 101 in CCBC | 0.44613 | 570 | 422 | 771 |
| UNT 102 in CCBC | 0.97432 | 977 | 722 | 1321 |
| UNT 102 in CCBC | 1.34127 | 1246 | 921 | 1685 |
| UNT 103 in CCBC | 0.31906 | 463 | 342 | 626 |
| UNT 104 in CCBC | 0.31468 | 522 | 386 | 706 |
| UNT 106 in CCBC | 0.13254 | 289 | 214 | 391 |
| UNT 107 in CCBC | 0.15039 | 289 | 214 | 391 |
| UNT 107 in CCBC | 0.3005 | 516 | 381 | 697 |
| UNT 107 in CCBC | 0.60819 | 839 | 620 | 1135 |
| UNT 107 in CCBC | 1.27213 | 1277 | 945 | 1728 |
| UNT 108 in CCBC | 0.10716 | 158 | 117 | 214 |
| UNT 108 in CCBC | 0.85846 | 880 | 651 | 1190 |
| UNT 109 in CCBC | 0.34973 | 424 | 313 | 573 |
| UNT 110 in CCBC | 0.27378 | 322 | 238 | 435 |
| UNT 111 in CCBC | 0.31835 | 397 | 294 | 537 |
| UNT 112 in CCBC | 0.63885 | 823 | 608 | 1113 |
| UNT 113 in CCBC | 0.57383 | 739 | 546 | 999 |
| UNT 113 in CCBC | 1.08094 | 1204 | 890 | 1629 |
| UNT 114 in CCBC | 0.37287 | 549 | 406 | 743 |
| UNT 115 in CCBC | 0.29674 | 460 | 340 | 622 |
| UNT 116 in CCBC | 0.54827 | 819 | 606 | 1108 |
| UNT 116 in CCBC | 0.7947 | 1113 | 823 | 1505 |
| UNT 119 in CCBC | 0.238 | 384 | 284 | 520 |
| UNT 120 in CCBC | 0.26418 | 463 | 342 | 626 |
| UNT 120 in CCBC | 0.5932 | 962 | 712 | 1301 |
| UNT 120 in CCBC | 0.92043 | 1332 | 985 | 1801 |
| UNT 121 in CCBC | 0.04555 | 82 | 61 | 111 |
| UNT 122 in CCBC | 0.05631 | 105 | 78 | 142 |
| UNT 123 in CCBC | 0.16913 | 278 | 206 | 377 |
| UNT 124 in CCBC | 0.1225 | 214 | 159 | 290 |
| UNT 125 in CCBC | 0.23872 | 468 | 346 | 633 |
| UNT 126 in CCBC | 1.39104 | 1506 | 1114 | 2037 |
| UNT 127 in CCBC | 0.38166 | 561 | 414 | 758 |
| UNT 128 in CCBC | 0.45546 | 538 | 398 | 728 |
| UNT 129 in CCBC | 0.08985 | 125 | 93 | 169 |
| UNT 130 in CCBC | 0.27988 | 458 | 338 | 619 |
| UNT 131 in CCBC | 0.15728 | 313 | 232 | 424 |
| WREN BRANCH | 0.87729 | 1208 | 893 | 1634 |
| WREN BRANCH | 1.48493 | 1892 | 1399 | 2559 |
| WREN BRANCH | 1.82848 | 2130 | 1575 | 2880 |
| WREN BRANCH | 1.8865 | 2130 | 1575 | 2880 |
| Tehuacana Creek |  |  |  |  |
| BAIRD BRANCH | 0.922219 | 2317 | 1713 | 3134 |
| BAIRD BRANCH | 1.591017 | 3681 | 2722 | 4978 |
| BEAR CREEK | 1.311141 | 1417 | 1047 | 1916 |
| BEAR CREEK | 2.303934 | 2126 | 1572 | 2876 |
| BEAR CREEK | 2.629993 | 2434 | 1800 | 3292 |
| BEAR CREEK | 3.446485 | 2988 | 2210 | 4042 |
| BEAR CREEK | 4.741753 | 4664 | 3449 | 6308 |
| BEAR CREEK | 6.39357 | 5918 | 4376 | 8003 |
| BEAR CREEK | 7.128368 | 6450 | 4769 | 8723 |
| BEAR CREEK | 7.930154 | 7313 | 5408 | 9890 |
| BEAR CREEK | 9.119246 | 8097 | 5987 | 10951 |
| BEAR CREEK | 13.849342 | 11506 | 8508 | 15561 |
| BRANCHIDGE CREEK | 0.995042 | 1767 | 1306 | 2389 |
| BRANCHIDGE CREEK | 3.716721 | 5846 | 4323 | 7906 |
| BRANCHIDGE CREEK | 4.57702 | 6900 | 5102 | 9332 |
| BRANCHIDGE CREEK | 4.655755 | 6915 | 5113 | 9352 |
| CANEY CREEK | 0.944279 | 2671 | 1975 | 3612 |
| CANEY CREEK | 1.208283 | 2739 | 2026 | 3705 |
| CANEY CREEK | 1.874301 | 3962 | 2930 | 5358 |
| CANEY CREEK | 2.286179 | 4206 | 3110 | 5688 |
| CEDAR CREEK | 1.389875 | 3271 | 2419 | 4424 |
| CEDAR CREEK | 1.796283 | 3869 | 2861 | 5233 |
| CEDAR CREEK | 2.273714 | 4557 | 3369 | 6162 |
| CEDAR CREEK | 2.595737 | 5140 | 3801 | 6951 |
| CHARLES CREEK | 1.380011 | 2776 | 2053 | 3754 |
| CHARLES CREEK | 2.225468 | 3939 | 2913 | 5328 |
| CHARLES CREEK | 2.591074 | 4521 | 3343 | 6114 |
| CHARLES CREEK | 3.558848 | 5379 | 3978 | 7275 |
| CHARLES CREEK | 4.341175 | 6218 | 4598 | 8409 |
| CHARLES CREEK | 5.313612 | 7187 | 5314 | 9720 |
| CHARLES CREEK | 5.841799 | 7771 | 5746 | 10509 |
| CHARLES CREEK | 7.422413 | 8769 | 6484 | 11859 |
| TEHUACANA CREEK | 0.243199 | 457 | 338 | 618 |
| TEHUACANA CREEK | 0.454698 | 710 | 525 | 960 |
| TEHUACANA CREEK | 1.495175 | 1796 | 1328 | 2429 |
| TEHUACANA CREEK | 2.218181 | 2482 | 1835 | 3357 |
| TEHUACANA CREEK | 2.391368 | 2712 | 2005 | 3668 |
| TEHUACANA CREEK | 2.907158 | 2846 | 2104 | 3848 |
| TEHUACANA CREEK | 3.03685 | 2988 | 2209 | 4041 |
| TEHUACANA CREEK | 5.321306 | 4618 | 3415 | 6246 |
| TEHUACANA CREEK | 10.137867 | 7090 | 5243 | 9589 |
| TEHUACANA CREEK | 11.883214 | 8332 | 6161 | 11268 |
| TEHUACANA CREEK | 12.777993 | 9058 | 6698 | 12250 |
| TEHUACANA CREEK | 13.882163 | 9715 | 7184 | 13138 |
| TEHUACANA CREEK | 14.963915 | 10202 | 7544 | 13797 |
| TEHUACANA CREEK | 15.620158 | 10622 | 7855 | 14365 |
| TEHUACANA CREEK | 16.619946 | 11114 | 8218 | 15030 |
| TEHUACANA CREEK | 17.27045 | 11643 | 8609 | 15746 |
| TEHUACANA CREEK | 41.933136 | 25042 | 18517 | 33866 |
| TEHUACANA CREEK | 43.557378 | 25400 | 18782 | 34351 |
| TEHUACANA CREEK | 44.460496 | 25693 | 18998 | 34746 |
| TEHUACANA CREEK | 55.982356 | 31406 | 23223 | 42473 |
| TEHUACANA CREEK | 57.418333 | 32255 | 23851 | 43621 |
| TEHUACANA CREEK | 57.786046 | 32402 | 23959 | 43820 |
| TEHUACANA CREEK | 58.039647 | 32507 | 24037 | 43962 |
| TEHUACANA CREEK | 61.08932 | 33131 | 24498 | 44805 |
| TEHUACANA CREEK | 61.56424 | 33555 | 24811 | 45379 |
| TEHUACANA CREEK | 63.415494 | 34083 | 25202 | 46094 |
| TEHUACANA CREEK | 65.809103 | 34886 | 25796 | 47180 |
| TEHUACANA CREEK | 65.88712 | 34886 | 25796 | 47180 |
| TEHUACANA CREEK | 67.308382 | 34886 | 25796 | 47180 |
| TEHUACANA CREEK | 69.002884 | 34911 | 25814 | 47213 |
| TEHUACANA CREEK | 69.881073 | 34911 | 25814 | 47213 |
| TEHUACANA CREEK | 70.174759 | 34911 | 25814 | 47213 |
| TEHUACANA CREEK | 72.540659 | 35046 | 25914 | 47396 |
| TEHUACANA CREEK | 113.616725 | 47526 | 35143 | 64274 |
| TEHUACANA CREEK | 113.965023 | 47526 | 35143 | 64274 |
| TEHUACANA CREEK | 114.563336 | 47526 | 35143 | 64274 |
| TEHUACANA CREEK | 116.305545 | 47526 | 35143 | 64274 |
| TEHUACANA CREEK | 117.063659 | 47526 | 35143 | 64274 |
| TEHUACANA CREEK | 118.063626 | 47526 | 35143 | 64274 |
| TEHUACANA CREEK | 118.968628 | 47526 | 35143 | 64274 |
| TEHUACANA CREEK | 119.157843 | 47526 | 35143 | 64274 |
| TEHUACANA CREEK | 119.757231 | 47526 | 35143 | 64274 |
| TEHUACANA CREEK | 136.395023 | 50753 | 37528 | 68637 |
| TEHUACANA CREEK | 137.715041 | 50753 | 37528 | 68637 |
| TEHUACANA CREEK | 140.517165 | 50753 | 37528 | 68637 |
| TEHUACANA CREEK | 146.233822 | 51799 | 38302 | 70052 |
| TEHUACANA CREEK | 148.778445 | 51799 | 38302 | 70052 |
| MAGNOLIA CREEK | 0.917386 | 2316 | 1712 | 3132 |
| MAGNOLIA CREEK | 1.241284 | 2905 | 2148 | 3928 |
| MAGNOLIA CREEK | 1.546 | 3328 | 2461 | 4501 |
| MAGNOLIA CREEK | 2.126288 | 4253 | 3145 | 5752 |
| MAINER BRANCH | 1.020772 | 3117 | 2305 | 4216 |
| MAINER BRANCH | 1.678181 | 4217 | 3118 | 5704 |
| MANNING BRANCH | 0.90536 | 3123 | 2309 | 4223 |
| MANTON CREEK | 0.948853 | 1724 | 1275 | 2331 |
| MANTON CREEK | 1.996079 | 3341 | 2470 | 4518 |
| MANTON CREEK | 2.843599 | 4861 | 3594 | 6574 |
| MANTON CREEK | 3.133788 | 5343 | 3951 | 7226 |
| MANTON CREEK | 5.263215 | 8161 | 6034 | 11036 |
| N MAGNOLIA CREEK | 1.087311 | 1501 | 1110 | 2030 |
| N MAGNOLIA CREEK | 1.949897 | 2594 | 1918 | 3508 |
| N MAGNOLIA CREEK | 2.743612 | 3785 | 2799 | 5118 |
| N MAGNOLIA CREEK | 3.55293 | 4685 | 3464 | 6336 |
| N MAGNOLIA CREEK | 7.886213 | 8529 | 6306 | 11534 |
| N MAGNOLIA CREEK | 8.864748 | 9413 | 6960 | 12730 |
| N MAGNOLIA CREEK | 9.399571 | 9702 | 7174 | 13121 |
| N MAGNOLIA CREEK | 10.787742 | 11030 | 8156 | 14917 |
| ROBINETT BRANCH | 0.403056 | 1336 | 988 | 1806 |
| ROBINETT BRANCH | 0.42802 | 1418 | 1048 | 1917 |
| ROBINETT BRANCH | 1.230254 | 3011 | 2226 | 4072 |
| ROCKY CREEK | 1.264689 | 1553 | 1148 | 2101 |
| ROCKY CREEK | 1.569405 | 1815 | 1342 | 2455 |
| ROCKY CREEK | 2.076428 | 2166 | 1601 | 2929 |
| ROCKY CREEK | 2.663994 | 2441 | 1805 | 3301 |
| ROCKY CREEK | 4.536128 | 3682 | 2723 | 4979 |
| ROCKY CREEK | 6.926151 | 5316 | 3931 | 7189 |
| ROCKY CREEK | 10.090339 | 8410 | 6219 | 11374 |
| ROCKY CREEK | 12.392121 | 10667 | 7887 | 14425 |
| ROCKY CREEK | 13.641834 | 11579 | 8562 | 15659 |
| ROCKY CREEK | 13.978474 | 11775 | 8707 | 15924 |
| ROCKY CREEK | 14.980684 | 12744 | 9423 | 17235 |
| ROCKY CREEK | 17.139165 | 14244 | 10532 | 19263 |
| ROCKY CREEK | 18.121828 | 14598 | 10794 | 19742 |
| ROCKY CREEK | 18.963794 | 15025 | 11110 | 20319 |
| ROCKY CREEK | 19.663329 | 15400 | 11387 | 20827 |
| ROCKY CREEK | 20.695144 | 15635 | 11561 | 21144 |
| ROCKY CREEK | 21.048464 | 15635 | 11561 | 21144 |
| ROCKY CREEK | 35.083075 | 21977 | 16250 | 29721 |
| ROCKY CREEK | 35.472265 | 22091 | 16335 | 29876 |
| ROCKY CREEK | 38.02729 | 22347 | 16524 | 30221 |
| ROCKY CREEK | 38.994347 | 23041 | 17038 | 31161 |
| ROCKY CREEK | 39.902481 | 23530 | 17399 | 31822 |
| ROCKY CREEK | 40.614149 | 23530 | 17399 | 31822 |
| ROCKY CREEK | 41.059656 | 23530 | 17399 | 31822 |
| SANDY CREEK | 1.384674 | 3249 | 2403 | 4394 |
| SANDY CREEK | 2.228428 | 4648 | 3437 | 6286 |
| SANDY CREEK | 3.182661 | 6211 | 4593 | 8400 |
| SANDY CREEK | 3.504057 | 6457 | 4774 | 8732 |
| SANDY CREEK | 3.87585 | 7010 | 5184 | 9481 |
| SANDY CREEK | 4.180118 | 7347 | 5433 | 9937 |
| SANDY CREEK | 4.372292 | 7455 | 5512 | 10082 |
| SANDY CREEK | 6.155124 | 9736 | 7199 | 13166 |
| SANDY CREEK | 12.089932 | 14641 | 10826 | 19800 |
| SANDY CREEK | 13.024026 | 15201 | 11240 | 20558 |
| SANDY CREEK | 13.404075 | 15597 | 11533 | 21094 |
| SANDY CREEK | 14.73892 | 16269 | 12030 | 22002 |
| SANDY CREEK | 15.338398 | 16477 | 12184 | 22284 |
| SANDY CREEK | 15.61388 | 16477 | 12184 | 22284 |
| SANK BRANCH | 0.544687 | 1721 | 1272 | 2327 |
| SANK BRANCH | 0.628802 | 1839 | 1360 | 2487 |
| STEAM MILL CREEK | 0.9415 | 2917 | 2157 | 3945 |
| STEAM MILL CREEK | 1.215457 | 3368 | 2491 | 4555 |
| STEAM MILL CREEK | 3.137465 | 7137 | 5277 | 9652 |
| STEAM MILL CREEK | 3.490157 | 7221 | 5340 | 9766 |
| STEAM MILL CREEK | 4.906666 | 9012 | 6663 | 12187 |
| UNT 001 IN KICR | 0.344442 | 928 | 686 | 1255 |
| UNT 001 IN KICR | 0.935043 | 2173 | 1607 | 2939 |
| UNT 001 IN KICR | 2.287434 | 3324 | 2458 | 4495 |
| UNT 002 IN KICR | 0.271178 | 822 | 608 | 1112 |
| UNT 002 IN KICR | 0.458687 | 1229 | 909 | 1662 |
| UNT 004 IN KICR | 1.133763 | 1542 | 1140 | 2085 |
| UNT 004 IN KICR | 1.336429 | 1752 | 1295 | 2369 |
| UNT 005 IN KICR | 1.04068 | 1729 | 1279 | 2338 |
| UNT 005 IN KICR | 1.303518 | 1966 | 1453 | 2658 |
| UNT 007 IN KICR | 0.918363 | 1231 | 910 | 1665 |
| UNT 007 IN KICR | 1.295627 | 1688 | 1248 | 2283 |
| UNT 008 IN KICR | 1.053862 | 1812 | 1340 | 2450 |
| UNT 008 IN KICR | 1.25052 | 1979 | 1463 | 2676 |
| UNT 010 IN KICR | 0.976114 | 2868 | 2120 | 3878 |
| UNT 010 IN KICR | 1.159948 | 3263 | 2412 | 4412 |
| UNT 011 IN KICR | 1.022656 | 2095 | 1549 | 2833 |
| UNT 012 IN KICR | 1.178601 | 3082 | 2279 | 4168 |
| UNT 012 IN KICR | 1.831705 | 4053 | 2997 | 5481 |
| UNT 013 IN KICR | 0.66578 | 1037 | 767 | 1402 |
| UNT 015 IN KICR | 0.980545 | 1153 | 853 | 1559 |
| UNT 015 IN KICR | 1.380548 | 1604 | 1186 | 2169 |
| UNT 016 IN KICR | 0.412954 | 522 | 386 | 705 |
| UNT 016 IN KICR | 0.545255 | 689 | 509 | 932 |
| UNT 016 IN KICR | 1.164432 | 1193 | 882 | 1613 |
| UNT 016 IN KICR | 2.09257 | 1939 | 1434 | 2622 |
| UNT 016 IN KICR | 3.10563 | 2697 | 1994 | 3647 |
| UNT 016 IN KICR | 4.104962 | 3178 | 2350 | 4298 |
| UNT 017 IN KICR | 1.014316 | 1772 | 1310 | 2396 |
| UNT 018 IN KICR | 0.911103 | 2497 | 1846 | 3377 |
| UNT 020 IN KICR | 1.020234 | 2053 | 1518 | 2776 |
| UNT 020 IN KICR | 1.985946 | 3562 | 2634 | 4817 |
| UNT 020 IN KICR | 2.655102 | 4470 | 3305 | 6045 |
| UNT 020 IN KICR | 3.379857 | 5384 | 3981 | 7282 |
| UNT 021 IN KICR | 0.945625 | 2870 | 2122 | 3881 |
| UNT 021 IN KICR | 1.382881 | 3272 | 2419 | 4425 |
| UNT 022 IN KICR | 0.924999 | 2437 | 1802 | 3296 |
| UNT 022 IN KICR | 1.264103 | 3230 | 2388 | 4368 |
| UNT 023 IN KICR | 0.352334 | 1246 | 921 | 1685 |
| UNT 025 IN KICR | 0.329886 | 1130 | 836 | 1529 |
| UNT 025 IN KICR | 0.382465 | 1455 | 1076 | 1968 |
| UNT 026 IN KICR | 1.304415 | 3592 | 2656 | 4857 |
| UNT 026 IN KICR | 1.688673 | 3963 | 2930 | 5360 |
| UNT 027 IN KICR | 0.801839 | 2081 | 1539 | 2814 |
| UNT 027 IN KICR | 1.868651 | 4103 | 3034 | 5549 |
| UNT 028 IN KICR | 0.677855 | 2029 | 1501 | 2744 |
| UNT 029 IN KICR | 1.149546 | 2693 | 1992 | 3643 |
| UNT 029 IN KICR | 1.368981 | 2973 | 2199 | 4021 |
| UNT 031 IN KICR | 0.650773 | 1711 | 1265 | 2313 |
| UNT 033 IN KICR | 0.569168 | 1678 | 1241 | 2270 |
| UNT 035 IN KICR | 0.924362 | 1190 | 880 | 1609 |
| UNT 036 IN KICR | 0.946252 | 1074 | 794 | 1452 |
| UNT 036 IN KICR | 1.745348 | 1679 | 1242 | 2271 |
| UNT 036 IN KICR | 2.170229 | 2223 | 1644 | 3006 |
| UNT 037 IN KICR | 1.015302 | 3145 | 2325 | 4253 |
| UNT 037 IN KICR | 1.793331 | 4912 | 3632 | 6643 |
| UNT 037 IN KICR | 2.642996 | 5988 | 4428 | 8098 |
| UNT 037 IN KICR | 3.131367 | 6229 | 4606 | 8424 |
| UNT 038 IN KICR | 0.400734 | 760 | 562 | 1028 |
| UNT 038 IN KICR | 0.424694 | 832 | 615 | 1125 |
| UNT 039 IN KICR | 0.940603 | 2687 | 1987 | 3634 |
| UNT 039 IN KICR | 1.3025 | 3427 | 2534 | 4635 |
| UNT 040 IN KICR | 0.979163 | 2519 | 1862 | 3406 |
| UNT 041 IN KICR | 1.154837 | 2750 | 2034 | 3719 |
| UNT 042 IN KICR | 0.26676 | 918 | 679 | 1241 |
| UNT 043 IN KICR | 0.613175 | 1541 | 1139 | 2084 |
| UNT 044 IN KICR | 0.238115 | 919 | 680 | 1243 |
| UNT 046 IN KICR | 1.066058 | 2000 | 1479 | 2705 |
| UNT 047 IN KICR | 1.392207 | 2133 | 1577 | 2885 |
| UNT 047 IN KICR | 1.616305 | 2446 | 1809 | 3308 |
| UNT 048 IN KICR | 0.247234 | 889 | 658 | 1203 |
| UNT 049 IN KICR | 0.734081 | 2265 | 1675 | 3064 |
| UNT 050 IN KICR | 0.185538 | 511 | 378 | 691 |
| UNT 051 IN KICR | 1.266931 | 2029 | 1501 | 2745 |
| UNT 051 IN KICR | 1.68679 | 2703 | 1998 | 3655 |
| UNT 051 IN KICR | 2.604517 | 4552 | 3366 | 6157 |
| UNT 052 IN KICR | 0.449542 | 1700 | 1257 | 2299 |
| UNT 052 IN KICR | 0.88895 | 3144 | 2325 | 4252 |
| UNT 053 IN KICR | 0.422818 | 1567 | 1159 | 2119 |
| UNT 055 IN KICR | 0.284694 | 915 | 676 | 1237 |
| UNT 057 IN KICR | 0.329915 | 1054 | 779 | 1425 |
| UNT 057 IN KICR | 0.351078 | 1129 | 835 | 1527 |
| UNT 059 IN KICR | 0.284906 | 676 | 500 | 914 |
| UNT 060 IN KICR | 0.098959 | 325 | 241 | 440 |
| UNT 060 IN KICR | 0.123474 | 453 | 335 | 613 |
| UNT 061 IN KICR | 0.199438 | 738 | 546 | 999 |
| UNT 062 IN KICR | 0.596609 | 1745 | 1291 | 2361 |
| UNT 065 IN KICR | 0.648082 | 1956 | 1446 | 2645 |
| UNT 066 IN KICR | 0.904284 | 2536 | 1875 | 3429 |
| UNT 067 IN KICR | 0.387486 | 1118 | 827 | 1512 |
| UNT 068 IN KICR | 0.094237 | 482 | 356 | 652 |
| UNT 068 IN KICR | 0.541997 | 1971 | 1458 | 2666 |
| UNT 069 IN KICR | 0.38085 | 993 | 734 | 1342 |
| UNT 071 IN KICR | 0.844946 | 2721 | 2012 | 3680 |
| UNT 071 IN KICR | 1.423145 | 4219 | 3120 | 5706 |
| UNT 07189 IN KICR | 0.153345 | 635 | 470 | 859 |
| UNT 073 IN KICR | 1.06561 | 2633 | 1947 | 3561 |
| UNT 073 IN KICR | 1.577296 | 3686 | 2725 | 4985 |
| UNT 07397 IN KICR | 0.757145 | 1764 | 1304 | 2385 |
| UNT 074 IN KICR | 0.50614 | 1767 | 1307 | 2390 |
| UNT 07482 IN KICR | 0.536078 | 801 | 592 | 1083 |
| UNT 075 IN KICR | 0.378923 | 1354 | 1001 | 1831 |
| UNT 07554 IN KICR | 0.196747 | 713 | 527 | 964 |
| UNT 07557 IN KICR | 0.52046 | 2006 | 1484 | 2713 |
| UNT 076 IN KICR | 0.876052 | 2508 | 1855 | 3392 |
| UNT 077 IN KICR | 0.356235 | 1035 | 765 | 1400 |
| UNT 078 IN KICR | 0.182579 | 768 | 568 | 1039 |
| UNT 079 IN KICR | 0.898724 | 2364 | 1748 | 3198 |
| UNT 081 IN KICR | 0.236115 | 529 | 391 | 715 |
| UNT 081 IN KICR | 0.881327 | 1428 | 1056 | 1931 |
| UNT 081 IN KICR | 1.477578 | 1964 | 1452 | 2656 |
| UNT 0980 IN KICR | 1.013419 | 3517 | 2600 | 4756 |
| UNT 0980 IN KICR | 2.001101 | 4818 | 3563 | 6516 |
| UNT 0980 IN KICR | 2.486333 | 5006 | 3702 | 6770 |
| UNT 0980 IN KICR | 4.042915 | 6552 | 4845 | 8861 |
| UNT 0980 IN KICR | 5.711545 | 8682 | 6420 | 11741 |
| UNT 11455 IN KICR | 0.053542 | 295 | 218 | 399 |
| UNT 11455 IN KICR | 0.060515 | 327 | 242 | 442 |
| UNT 2263 IN KICR | 1.146318 | 2350 | 1737 | 3177 |
| UPPER TEHUACANA CREEK | 1.606351 | 2083 | 1540 | 2817 |
| UPPER TEHUACANA CREEK | 1.747859 | 2140 | 1583 | 2895 |
| UPPER TEHUACANA CREEK | 3.796577 | 4017 | 2970 | 5432 |
| UPPER TEHUACANA CREEK | 4.702207 | 4606 | 3406 | 6230 |
| UPPER TEHUACANA CREEK | 6.62278 | 6277 | 4641 | 8488 |
| UPPER TEHUACANA CREEK | 14.484153 | 14205 | 10504 | 19210 |
| UPPER TEHUACANA CREEK | 16.728543 | 15262 | 11285 | 20640 |
| UPPER TEHUACANA CREEK | 17.180595 | 15604 | 11538 | 21103 |
| UPPER TEHUACANA CREEK | 22.27818 | 19953 | 14754 | 26984 |
| UPPER TEHUACANA CREEK | 24.487506 | 21658 | 16014 | 29289 |
| Lower Keechi Creek |  |  |  |  |
| BAIN BRANCH | 0.230017 | 442 | 327 | 598 |
| BAIN BRANCH | 0.258533 | 515 | 381 | 696 |
| BAIN BRANCH | 0.948943 | 1331 | 984 | 1800 |
| BAIN BRANCH | 1.686879 | 2068 | 1529 | 2797 |
| BEAR BRANCH | 0.757966 | 659 | 488 | 892 |
| BEAR BRANCH | 1.37741 | 1054 | 780 | 1426 |
| BEAR BRANCH | 2.372087 | 1779 | 1316 | 2406 |
| BEAR BRANCH | 2.798313 | 1973 | 1459 | 2668 |
| BEAR BRANCH | 3.014879 | 2069 | 1530 | 2798 |
| BEAVER CREEK | 0.820438 | 2643 | 1954 | 3575 |
| BEAVER CREEK | 1.710105 | 4650 | 3438 | 6289 |
| BEAVER CREEK | 2.461942 | 6105 | 4514 | 8256 |
| BEAVER CREEK | 3.139169 | 7367 | 5448 | 9964 |
| BEAVER CREEK | 3.384341 | 7495 | 5542 | 10135 |
| BEAVER CREEK | 4.514337 | 9251 | 6841 | 12511 |
| BEAVER CREEK | 5.063239 | 9785 | 7236 | 13233 |
| BEAVER CREEK | 6.193863 | 11143 | 8240 | 15070 |
| BEAVER CREEK | 6.710751 | 11264 | 8329 | 15233 |
| BEAVER CREEK | 10.345106 | 15070 | 11143 | 20380 |
| BEAVER CREEK | 11.141604 | 15971 | 11810 | 21599 |
| BEAVER CREEK | 12.097807 | 15971 | 11810 | 21599 |
| BEAVER CREEK | 12.291741 | 15971 | 11810 | 21599 |
| BEAVER CREEK | 13.370322 | 15971 | 11810 | 21599 |
| BEAVER CREEK | 13.862883 | 15971 | 11810 | 21599 |
| BEAVER CREEK | 14.67256 | 15971 | 11810 | 21599 |
| BEAVER CREEK | 14.902128 | 15971 | 11810 | 21599 |
| BEAVER CREEK | 14.913876 | 15971 | 11810 | 21599 |
| BEAVER CREEK | 19.534747 | 15971 | 11810 | 21599 |
| BEAVER CREEK | 19.859461 | 15971 | 11810 | 21599 |
| BEAVER CREEK | 21.013311 | 15971 | 11810 | 21599 |
| BEAVER CREEK | 22.069521 | 15971 | 11810 | 21599 |
| BOB BRANCH | 1.185775 | 1032 | 763 | 1396 |
| BOB BRANCH | 2.10916 | 1462 | 1081 | 1978 |
| CATTAIL BRANCH | 0.454294 | 672 | 497 | 908 |
| CATTAIL BRANCH | 0.66817 | 928 | 686 | 1255 |
| CLAPPS CREEK | 1.092512 | 1280 | 947 | 1731 |
| CLAPPS CREEK | 1.926402 | 1801 | 1332 | 2436 |
| CLAPPS CREEK | 2.569104 | 2159 | 1596 | 2920 |
| CLAPPS CREEK | 3.546922 | 2788 | 2061 | 3770 |
| CLAPPS CREEK | 4.611904 | 3326 | 2459 | 4498 |
| CLAPPS CREEK | 8.203842 | 4977 | 3680 | 6731 |
| CLAPPS CREEK | 9.138168 | 5292 | 3913 | 7157 |
| CLAPPS CREEK | 9.474629 | 5522 | 4083 | 7468 |
| CLAPPS CREEK | 10.113969 | 5522 | 4083 | 7468 |
| CLAPPS CREEK | 11.141063 | 5561 | 4112 | 7521 |
| CLAPPS CREEK | 13.429931 | 6414 | 4743 | 8674 |
| CLAPPS CREEK | 13.596369 | 6414 | 4743 | 8674 |
| CLAPPS CREEK | 15.897164 | 6924 | 5120 | 9363 |
| CLAPPS CREEK | 19.94815 | 7783 | 5755 | 10526 |
| CLAPPS CREEK | 22.053812 | 8020 | 5931 | 10847 |
| CLAPPS CREEK | 22.955406 | 8236 | 6090 | 11138 |
| CLAPPS CREEK | 24.115444 | 8418 | 6225 | 11385 |
| CLAPPS CREEK | 24.578258 | 8418 | 6225 | 11385 |
| ELY BRANCH | 1.004182 | 3314 | 2450 | 4481 |
| ELY BRANCH | 1.227563 | 3668 | 2712 | 4960 |
| ELY BRANCH | 1.679078 | 4201 | 3106 | 5681 |
| FOREHAND BRANCH | 1.000147 | 1148 | 849 | 1553 |
| FOREHAND BRANCH | 2.052933 | 2062 | 1524 | 2788 |
| FOREHAND BRANCH | 2.846021 | 2652 | 1961 | 3587 |
| FOREHAND BRANCH | 3.447382 | 2953 | 2184 | 3994 |
| FOREHAND BRANCH | 4.447171 | 3630 | 2684 | 4909 |
| FOREHAND BRANCH | 4.879584 | 3700 | 2736 | 5004 |
| FOREHAND BRANCH | 6.932876 | 4382 | 3240 | 5926 |
| FOREHAND BRANCH | 7.388785 | 4591 | 3395 | 6209 |
| GREER BRANCH | 1.21456 | 3772 | 2789 | 5101 |
| GREER BRANCH | 2.029887 | 5358 | 3962 | 7246 |
| GREER BRANCH | 2.771949 | 6336 | 4685 | 8568 |
| GREER BRANCH | 3.028599 | 6412 | 4742 | 8672 |
| HALE BRANCH | 1.032789 | 1095 | 810 | 1481 |
| HALE BRANCH | 1.531921 | 1362 | 1007 | 1842 |
| HALE BRANCH | 2.350655 | 2067 | 1528 | 2795 |
| HALEY BRANCH | 1.063996 | 855 | 632 | 1156 |
| HALEY BRANCH | 2.093377 | 1457 | 1077 | 1970 |
| HENRY THOMAS BRANCH | 1.393305 | 1349 | 997 | 1824 |
| HENRY THOMAS BRANCH | 2.072647 | 1685 | 1246 | 2279 |
| HOGPEN BRANCH | 1.400278 | 3698 | 2735 | 5001 |
| HOGPEN BRANCH | 1.86354 | 4032 | 2982 | 5453 |
| HOGPEN BRANCH | 2.807819 | 5590 | 4134 | 7560 |
| HOGPEN BRANCH | 3.115405 | 5841 | 4319 | 7899 |
| LIME BRANCH | 0.956655 | 2567 | 1898 | 3472 |
| LITTLE BEAVER CREEK | 0.18213 | 280 | 207 | 379 |
| LITTLE BEAVER CREEK | 0.432324 | 585 | 433 | 791 |
| LITTLE BEAVER CREEK | 2.282053 | 2560 | 1893 | 3463 |
| LITTLE BEAVER CREEK | 2.588081 | 2681 | 1982 | 3626 |
| LITTLE BEAVER CREEK | 2.617226 | 2788 | 2062 | 3770 |
| LITTLE BEAVER CREEK | 3.561856 | 3284 | 2428 | 4441 |
| LITTLE BEAVER CREEK | 3.835194 | 3341 | 2470 | 4518 |
| LITTLE BEAVER CREEK | 4.522677 | 3696 | 2733 | 4998 |
| LITTLE KEECHI CREEK | 1.009832 | 2508 | 1855 | 3392 |
| LITTLE KEECHI CREEK | 1.894208 | 3650 | 2699 | 4937 |
| LITTLE KEECHI CREEK | 2.573946 | 4557 | 3370 | 6163 |
| LITTLE KEECHI CREEK | 3.466752 | 5793 | 4284 | 7834 |
| LITTLE KEECHI CREEK | 4.360365 | 6652 | 4919 | 8996 |
| LITTLE KEECHI CREEK | 5.464715 | 7799 | 5767 | 10548 |
| LITTLE KEECHI CREEK | 13.854459 | 16337 | 12080 | 22094 |
| LITTLE KEECHI CREEK | 15.363328 | 16841 | 12453 | 22776 |
| LITTLE KEECHI CREEK | 17.241577 | 18395 | 13602 | 24877 |
| LITTLE KEECHI CREEK | 17.645292 | 18395 | 13602 | 24877 |
| LOWER KEECHI CREEK | 0.961228 | 2138 | 1581 | 2891 |
| LOWER KEECHI CREEK | 1.455607 | 3097 | 2290 | 4188 |
| LOWER KEECHI CREEK | 2.535296 | 4196 | 3102 | 5674 |
| LOWER KEECHI CREEK | 5.854802 | 8046 | 5950 | 10882 |
| LOWER KEECHI CREEK | 9.909195 | 11846 | 8760 | 16021 |
| LOWER KEECHI CREEK | 10.696991 | 12069 | 8924 | 16322 |
| LOWER KEECHI CREEK | 11.420042 | 12683 | 9378 | 17152 |
| LOWER KEECHI CREEK | 12.207928 | 12966 | 9587 | 17534 |
| LOWER KEECHI CREEK | 12.370509 | 13059 | 9656 | 17660 |
| LOWER KEECHI CREEK | 21.953466 | 19748 | 14602 | 26707 |
| LOWER KEECHI CREEK | 23.375445 | 19748 | 14602 | 26707 |
| LOWER KEECHI CREEK | 41.592416 | 30067 | 22233 | 40663 |
| LOWER KEECHI CREEK | 42.361111 | 30067 | 22233 | 40663 |
| LOWER KEECHI CREEK | 44.667198 | 30067 | 22233 | 40663 |
| LOWER KEECHI CREEK | 45.533191 | 30067 | 22233 | 40663 |
| LOWER KEECHI CREEK | 46.465902 | 30381 | 22464 | 41086 |
| LOWER KEECHI CREEK | 55.213714 | 34364 | 25410 | 46473 |
| LOWER KEECHI CREEK | 57.092773 | 35449 | 26213 | 47941 |
| LOWER KEECHI CREEK | 58.228503 | 35449 | 26213 | 47941 |
| LOWER KEECHI CREEK | 61.085733 | 35449 | 26213 | 47941 |
| LOWER KEECHI CREEK | 63.04926 | 35449 | 26213 | 47941 |
| LOWER KEECHI CREEK | 65.761676 | 35449 | 26213 | 47941 |
| LOWER KEECHI CREEK | 67.135764 | 35449 | 26213 | 47941 |
| LOWER KEECHI CREEK | 85.13765 | 35449 | 26213 | 47941 |
| LOWER KEECHI CREEK | 85.8767 | 35449 | 26213 | 47941 |
| LOWER KEECHI CREEK | 93.269252 | 35449 | 26213 | 47941 |
| LOWER KEECHI CREEK | 95.899465 | 35449 | 26213 | 47941 |
| LOWER KEECHI CREEK | 96.91078 | 35449 | 26213 | 47941 |
| LOWER KEECHI CREEK | 119.16853 | 35449 | 26213 | 47941 |
| LOWER KEECHI CREEK | 119.765766 | 35449 | 26213 | 47941 |
| LOWER KEECHI CREEK | 122.674191 | 35449 | 26213 | 47941 |
| LOWER KEECHI CREEK | 123.483689 | 35449 | 26213 | 47941 |
| LOWER KEECHI CREEK | 124.621487 | 35449 | 26213 | 47941 |
| LOWER KEECHI CREEK | 132.951503 | 35449 | 26213 | 47941 |
| LOWER KEECHI CREEK | 137.216234 | 35449 | 26213 | 47941 |
| LOWER KEECHI CREEK | 138.476708 | 35449 | 26213 | 47941 |
| LOWER KEECHI CREEK | 139.932764 | 35449 | 26213 | 47941 |
| LOWER KEECHI CREEK | 140.717136 | 35449 | 26213 | 47941 |
| LOWER KEECHI CREEK | 142.174643 | 35449 | 26213 | 47941 |
| LOWER KEECHI CREEK | 145.126773 | 35449 | 26213 | 47941 |
| LOWER KEECHI CREEK | 145.40546 | 35449 | 26213 | 47941 |
| LOWER KEECHI CREEK | 149.362555 | 35449 | 26213 | 47941 |
| LOWER KEECHI CREEK | 150.567912 | 35449 | 26213 | 47941 |
| LOWER KEECHI CREEK | 151.686397 | 35449 | 26213 | 47941 |
| LOWER KEECHI CREEK | 151.921526 | 35449 | 26213 | 47941 |
| LOWER KEECHI CREEK | 152.862846 | 35449 | 26213 | 47941 |
| LOWER KEECHI CREEK | 153.444991 | 35449 | 26213 | 47941 |
| LOWER KEECHI CREEK | 154.571696 | 35449 | 26213 | 47941 |
| LOWER KEECHI CREEK | 155.479208 | 35449 | 26213 | 47941 |
| LOWER KEECHI CREEK | 156.805325 | 35449 | 26213 | 47941 |
| LOWER KEECHI CREEK | 181.413175 | 35449 | 26213 | 47941 |
| LOWER KEECHI CREEK | 183.398942 | 35449 | 26213 | 47941 |
| MILL BRANCH | 1.05458 | 1540 | 1139 | 2083 |
| MILL BRANCH | 1.597204 | 1723 | 1274 | 2331 |
| NORTH FORK LOWER KEECHI CREEK | 1.044447 | 2527 | 1869 | 3417 |
| NORTH FORK LOWER KEECHI CREEK | 1.799242 | 3662 | 2708 | 4953 |
| NORTH FORK LOWER KEECHI CREEK | 2.183949 | 4005 | 2961 | 5416 |
| NORTH FORK LOWER KEECHI CREEK | 5.225999 | 7994 | 5911 | 10811 |
| NORTH FORK LOWER KEECHI CREEK | 6.302281 | 8476 | 6268 | 11463 |
| NORTH FORK LOWER KEECHI CREEK | 9.037194 | 10770 | 7964 | 14565 |
| PARKS BRANCH | 1.400995 | 1763 | 1303 | 2384 |
| PARKS BRANCH | 2.389753 | 2307 | 1706 | 3121 |
| PARKS BRANCH | 6.024646 | 4312 | 3188 | 5831 |
| PARKS BRANCH | 7.294626 | 5017 | 3710 | 6785 |
| PINE BRANCH | 1.091698 | 1072 | 793 | 1450 |
| PINE BRANCH | 2.182625 | 1634 | 1208 | 2210 |
| PINE BRANCH | 2.24734 | 1835 | 1357 | 2481 |
| PINE BRANCH 2 | 0.483887 | 552 | 408 | 747 |
| REED CREEK | 0.93596 | 2983 | 2206 | 4034 |
| REED CREEK | 2.098668 | 5398 | 3992 | 7301 |
| REED CREEK | 3.044023 | 7305 | 5402 | 9879 |
| REED CREEK | 3.662782 | 8206 | 6068 | 11097 |
| REED CREEK | 4.775471 | 9564 | 7072 | 12934 |
| REED CREEK | 5.212907 | 9564 | 7072 | 12934 |
| REED CREEK | 7.134825 | 11846 | 8759 | 16020 |
| SMITH BRANCH | 1.103363 | 2135 | 1579 | 2887 |
| SMITH BRANCH | 1.802829 | 3034 | 2243 | 4103 |
| SMITH BRANCH | 3.745014 | 6923 | 5119 | 9363 |
| SMITH BRANCH | 4.415067 | 8022 | 5932 | 10849 |
| SMITH BRANCH | 5.287068 | 8022 | 5932 | 10849 |
| SMITH BRANCH | 6.331246 | 9256 | 6844 | 12518 |
| SMITH BRANCH | 8.084485 | 10697 | 7910 | 14466 |
| SPRING BRANCH | 1.026243 | 1055 | 780 | 1427 |
| SPRING BRANCH | 2.014283 | 1822 | 1347 | 2464 |
| SPRING BRANCH | 2.736258 | 2288 | 1692 | 3095 |
| SPRING BRANCH | 2.823293 | 2297 | 1698 | 3106 |
| SPRING BRANCH | 3.804787 | 2709 | 2003 | 3663 |
| TURKEY LAKE BRANCH | 1.315893 | 1470 | 1087 | 1988 |
| TURKEY LAKE BRANCH | 2.133731 | 1951 | 1443 | 2638 |
| TURKEY LAKE BRANCH | 2.442662 | 1951 | 1443 | 2638 |
| UNT 00800 in LKCR | 1.141744 | 2992 | 2213 | 4047 |
| UNT 00800 in LKCR | 1.447178 | 3201 | 2367 | 4328 |
| UNT 00800 in LKCR | 2.059838 | 4414 | 3264 | 5970 |
| UNT 00826 in LKCR | 0.807076 | 2121 | 1569 | 2869 |
| UNT 00923 in LKCR | 1.02418 | 2432 | 1798 | 3289 |
| UNT 01066 in LKCR | 0.152896 | 661 | 489 | 894 |
| UNT 01069 in LKCR | 0.30095 | 1097 | 811 | 1484 |
| UNT 01071 in LKCR | 0.58791 | 1878 | 1388 | 2539 |
| UNT 01074 in LKCR | 0.988131 | 2697 | 1994 | 3647 |
| UNT 01076 in LKCR | 0.116623 | 439 | 325 | 594 |
| UNT 01096 in LKCR | 0.628623 | 1484 | 1097 | 2007 |
| UNT 01104 in LKCR | 0.296376 | 466 | 344 | 630 |
| UNT 01104 in LKCR | 0.483663 | 728 | 538 | 984 |
| UNT 01105 in LKCR | 0.560021 | 724 | 535 | 979 |
| UNT 01110 in LKCR | 0.528366 | 842 | 622 | 1138 |
| UNT 01110 in LKCR | 0.572397 | 942 | 697 | 1274 |
| UNT 01112 in LKCR | 0.20697 | 327 | 242 | 443 |
| UNT 01119 in LKCR | 0.027889 | 59 | 44 | 80 |
| UNT 01119 in LKCR | 0.028158 | 59 | 44 | 80 |
| UNT 01119 in LKCR | 0.304032 | 494 | 365 | 668 |
| UNT 01119 in LKCR | 0.56678 | 740 | 547 | 1001 |
| UNT 01119 in LKCR | 0.622917 | 776 | 574 | 1049 |
| UNT 01120 in LKCR | 0.295838 | 459 | 339 | 621 |
| UNT 01125 in LKCR | 0.138205 | 222 | 164 | 301 |
| UNT 01131 in LKCR | 0.277365 | 444 | 328 | 600 |
| UNT 0191 in LKCR | 0.879713 | 2442 | 1806 | 3302 |
| UNT 0191 in LKCR | 1.585188 | 3507 | 2593 | 4743 |
| UNT 0196 in LKCR | 1.415791 | 3170 | 2344 | 4287 |
| UNT 0196 in LKCR | 2.493866 | 4530 | 3350 | 6126 |
| UNT 0197 in LKCR | 0.925896 | 1952 | 1443 | 2640 |
| UNT 0200 in LKCR | 1.364677 | 3866 | 2859 | 5228 |
| UNT 0203 in LKCR | 0.994049 | 2697 | 1994 | 3647 |
| UNT 0204 in LKCR | 1.008472 | 2940 | 2174 | 3976 |
| UNT 0208 in LKCR | 1.132059 | 2825 | 2089 | 3820 |
| UNT 0208 in LKCR | 1.652713 | 3479 | 2572 | 4705 |
| UNT 0210 in LKCR | 1.232 | 2594 | 1918 | 3508 |
| UNT 0210 in LKCR | 1.546701 | 2978 | 2202 | 4027 |
| UNT 0211 in LKCR | 0.982843 | 2859 | 2114 | 3866 |
| UNT 0211 in LKCR | 1.249354 | 3180 | 2351 | 4300 |
| UNT 0214 in LKCR | 0.963649 | 907 | 670 | 1226 |
| UNT 0215 in LKCR | 1.200123 | 1911 | 1413 | 2585 |
| UNT 0215 in LKCR | 1.342437 | 1975 | 1460 | 2671 |
| UNT 0215 in LKCR | 2.267078 | 2874 | 2125 | 3887 |
| UNT 0215 in LKCR | 4.080399 | 3371 | 2493 | 4559 |
| UNT 0215 in LKCR | 4.965051 | 3750 | 2773 | 5072 |
| UNT 0215 in LKCR | 5.904123 | 3935 | 2910 | 5322 |
| UNT 0215 in LKCR | 10.660135 | 5459 | 4037 | 7383 |
| UNT 0215 in LKCR | 15.620621 | 6783 | 5015 | 9173 |
| UNT 0215 in LKCR | 17.704477 | 7451 | 5509 | 10076 |
| UNT 0216 in LKCR | 0.968671 | 1042 | 770 | 1409 |
| UNT 0217 in LKCR | 1.179587 | 1309 | 968 | 1771 |
| UNT 0217 in LKCR | 2.103241 | 1993 | 1474 | 2695 |
| UNT 0217 in LKCR | 2.965109 | 2466 | 1823 | 3335 |
| UNT 0217 in LKCR | 3.574811 | 2726 | 2016 | 3687 |
| UNT 0218 in LKCR | 1.001582 | 1246 | 921 | 1685 |
| UNT 0218 in LKCR | 1.456952 | 1507 | 1114 | 2038 |
| UNT 0218 in LKCR | 3.255029 | 2708 | 2003 | 3663 |
| UNT 0218 in LKCR | 4.27495 | 3272 | 2419 | 4425 |
| UNT 0218 in LKCR | 4.607868 | 3329 | 2461 | 4502 |
| UNT 0219 in LKCR | 1.000326 | 1155 | 854 | 1562 |
| UNT 0222 in LKCR | 1.067045 | 1460 | 1080 | 1975 |
| UNT 0222 in LKCR | 1.556492 | 1772 | 1310 | 2397 |
| UNT 0222 in LKCR | 2.409751 | 2227 | 1646 | 3011 |
| UNT 0222 in LKCR | 3.369006 | 2731 | 2020 | 3694 |
| UNT 0223 in LKCR | 1.179318 | 1526 | 1129 | 2064 |
| UNT 0225 in LKCR | 1.098969 | 1572 | 1162 | 2126 |
| UNT 0225 in LKCR | 1.243615 | 1572 | 1162 | 2126 |
| UNT 0226 in LKCR | 1.163266 | 1352 | 1000 | 1829 |
| UNT 0231 in LKCR | 1.076853 | 1305 | 965 | 1765 |
| UNT 0233 in LKCR | 0.786361 | 925 | 684 | 1251 |
| UNT 0233 in LKCR | 1.008935 | 1109 | 820 | 1499 |
| UNT 0233 in LKCR | 1.119325 | 1148 | 849 | 1552 |
| UNT 0235 in LKCR | 1.513979 | 1882 | 1392 | 2545 |
| UNT 0235 in LKCR | 2.41369 | 764 | 565 | 1033 |
| UNT 0237 in LKCR | 1.447088 | 1429 | 1057 | 1933 |
| UNT 0239 in LKCR | 1.013369 | 885 | 655 | 1197 |
| UNT 0240 in LKCR | 1.107488 | 1217 | 900 | 1646 |
| UNT 0248 in LKCR | 1.007949 | 868 | 642 | 1173 |
| UNT 0248 in LKCR | 1.523222 | 1202 | 889 | 1626 |
| UNT 0249 in LKCR | 1.139054 | 869 | 642 | 1175 |
| UNT 0249 in LKCR | 2.272727 | 1498 | 1108 | 2026 |
| UNT 0249 in LKCR | 2.971566 | 1852 | 1369 | 2504 |
| UNT 0250 in LKCR | 1.040609 | 791 | 585 | 1069 |
| UNT 0250 in LKCR | 1.397049 | 906 | 670 | 1226 |
| UNT 0251 in LKCR | 1.003465 | 406 | 300 | 550 |
| UNT 1207 in LKCR | 0.587331 | 595 | 440 | 804 |
| UNT 2 TO LITTLE BEAVER CREEK | 0.158243 | 287 | 213 | 389 |
| UNT 2 TO LITTLE BEAVER CREEK | 0.238323 | 402 | 298 | 544 |
| Long King Creek |  |  |  |  |
| ALEXANDER CREEK | 1.01306 | 1643 | 1215 | 2222 |
| ALEXANDER CREEK | 1.72804 | 2177 | 1610 | 2945 |
| ALEXANDER CREEK | 2.79491 | 2823 | 2087 | 3818 |
| ALEXANDER CREEK | 4.929 | 3965 | 2932 | 5362 |
| ALEXANDER CREEK | 6.22722 | 4577 | 3385 | 6190 |
| ALEXANDER CREEK | 6.67246 | 4649 | 3438 | 6287 |
| ALEXANDER CREEK | 19.72154 | 10929 | 8081 | 14780 |
| BARNETT CREEK | 1.08498 | 1169 | 864 | 1581 |
| BARNETT CREEK | 2.81508 | 2392 | 1769 | 3235 |
| BARNETT CREEK | 3.47787 | 2590 | 1915 | 3502 |
| BARNETT CREEK | 4.10927 | 2886 | 2134 | 3903 |
| BARNETT CREEK | 5.75168 | 3600 | 2662 | 4869 |
| BARNETT CREEK | 6.98112 | 3969 | 2935 | 5368 |
| BARNETT CREEK | 12.86296 | 6318 | 4672 | 8545 |
| BARNETT CREEK | 16.9217 | 7588 | 5611 | 10262 |
| BARNETT CREEK | 18.73511 | 7923 | 5859 | 10715 |
| BARNETT CREEK | 20.0684 | 8753 | 6472 | 11838 |
| BARNETT CREEK TRIBUTARY NO1 | 0.68458 | 1029 | 761 | 1391 |
| BARNETT CREEK TRIBUTARY NO1 | 1.57999 | 1904 | 1408 | 2576 |
| BARNETT CREEK TRIBUTARY NO1 | 2.53368 | 2726 | 2016 | 3687 |
| BARNETT CREEK TRIBUTARY NO1 | 5.40956 | 4475 | 3309 | 6052 |
| CHOATES CREEK | 1.00902 | 1503 | 1112 | 2033 |
| CHOATES CREEK | 1.99886 | 2371 | 1753 | 3207 |
| CHOATES CREEK | 2.87239 | 3099 | 2292 | 4192 |
| CHOATES CREEK | 3.40801 | 3348 | 2476 | 4528 |
| CHOATES CREEK | 4.6319 | 4299 | 3179 | 5814 |
| CHOATES CREEK | 7.05474 | 5807 | 4294 | 7853 |
| CHOATES CREEK | 9.32586 | 7200 | 5324 | 9738 |
| CHOATES CREEK | 11.06959 | 7960 | 5886 | 10765 |
| CHOATES CREEK | 12.40763 | 8454 | 6251 | 11433 |
| CHOATES CREEK | 12.9638 | 8454 | 6251 | 11433 |
| CHOATES CREEK | 13.96341 | 8868 | 6558 | 11993 |
| CHOATES CREEK | 14.78501 | 9028 | 6676 | 12210 |
| CHOATES CREEK | 14.82169 | 9443 | 6983 | 12771 |
| CROOKED CREEK | 1.01979 | 1714 | 1267 | 2318 |
| CROOKED CREEK | 1.65334 | 2287 | 1691 | 3093 |
| CROOKED CREEK | 2.272 | 2887 | 2135 | 3905 |
| CROOKED CREEK | 2.66815 | 3072 | 2272 | 4155 |
| EAST TEMPE CREEK | 1.04149 | 1927 | 1425 | 2606 |
| EAST TEMPE CREEK | 1.48646 | 2556 | 1890 | 3457 |
| EAST TEMPE CREEK | 1.77431 | 2918 | 2157 | 3946 |
| EAST TEMPE CREEK | 2.4623 | 3710 | 2743 | 5017 |
| EAST TEMPE CREEK | 4.08793 | 5072 | 3751 | 6860 |
| EAST TEMPE CREEK | 5.12987 | 6189 | 4576 | 8370 |
| EAST TEMPE CREEK | 5.73589 | 6524 | 4824 | 8823 |
| EAST TEMPE CREEK | 6.66995 | 7377 | 5455 | 9976 |
| EAST TEMPE CREEK | 7.12379 | 7594 | 5615 | 10270 |
| EAST TEMPE CREEK | 7.91312 | 7778 | 5751 | 10519 |
| EAST TEMPE CREEK | 8.44139 | 8442 | 6243 | 11417 |
| EAST TEMPE CREEK | 9.13655 | 8754 | 6473 | 11839 |
| EAST TEMPE CREEK | 10.72945 | 9436 | 6978 | 12761 |
| KIBBLE CREEK | 1.1717 | 2154 | 1593 | 2913 |
| KIBBLE CREEK | 2.13642 | 2997 | 2216 | 4053 |
| KIBBLE CREEK | 2.96923 | 3515 | 2599 | 4754 |
| LIMA BRANCH | 1.05986 | 1749 | 1294 | 2366 |
| LIMA BRANCH | 1.1904 | 1931 | 1428 | 2611 |
| LONG KING CREEK | 1.10291 | 1425 | 1053 | 1927 |
| LONG KING CREEK | 1.17331 | 1492 | 1104 | 2018 |
| LONG KING CREEK | 2.48485 | 2541 | 1879 | 3437 |
| LONG KING CREEK | 6.21834 | 4999 | 3697 | 6761 |
| LONG KING CREEK | 11.61419 | 8404 | 6215 | 11366 |
| LONG KING CREEK | 12.14435 | 8703 | 6435 | 11769 |
| LONG KING CREEK | 13.51225 | 8703 | 6435 | 11769 |
| LONG KING CREEK | 14.63561 | 8703 | 6435 | 11769 |
| LONG KING CREEK | 24.75393 | 12789 | 9456 | 17295 |
| LONG KING CREEK | 25.57347 | 12789 | 9456 | 17295 |
| LONG KING CREEK | 31.541 | 14311 | 10582 | 19354 |
| LONG KING CREEK | 31.62197 | 14311 | 10582 | 19354 |
| LONG KING CREEK | 33.63222 | 14375 | 10629 | 19440 |
| LONG KING CREEK | 54.9776 | 17991 | 13304 | 24331 |
| LONG KING CREEK | 57.22504 | 18239 | 13486 | 24666 |
| LONG KING CREEK | 59.44217 | 18404 | 13608 | 24889 |
| LONG KING CREEK | 79.20496 | 22636 | 16738 | 30613 |
| LONG KING CREEK | 80.39647 | 22758 | 16828 | 30778 |
| LONG KING CREEK | 122.66333 | 30862 | 22821 | 41738 |
| LONG KING CREEK | 124.99085 | 31024 | 22940 | 41957 |
| LONG KING CREEK | 127.52238 | 31100 | 22996 | 42059 |
| LONG KING CREEK | 131.83809 | 31124 | 23014 | 42091 |
| LONG KING CREEK | 135.3643 | 31517 | 23305 | 42623 |
| LONG KING CREEK | 136.16644 | 31517 | 23305 | 42623 |
| LONG KING CREEK | 138.49266 | 31517 | 23305 | 42623 |
| LONG KING CREEK | 138.59816 | 31517 | 23305 | 42623 |
| LONG KING CREEK | 138.68937 | 31517 | 23305 | 42623 |
| LONG KING CREEK | 139.67704 | 31517 | 23305 | 42623 |
| LONG KING CREEK | 140.55371 | 31517 | 23305 | 42623 |
| LONG KING CREEK | 156.0285 | 33272 | 24603 | 44997 |
| LONG KING CREEK | 156.03801 | 33327 | 24643 | 45070 |
| LONG KING CREEK | 158.2168 | 33327 | 24643 | 45070 |
| LONG KING CREEK | 159.14211 | 33389 | 24689 | 45155 |
| LONG KING CREEK | 161.94603 | 33607 | 24850 | 45450 |
| LONG KING CREEK | 198.88309 | 41217 | 30477 | 55741 |
| LONG KING CREEK | 199.61928 | 41217 | 30477 | 55741 |
| LONG KING CREEK | 200.62607 | 41217 | 30477 | 55741 |
| LONG KING CREEK | 202.02607 | 41217 | 30477 | 55741 |
| LONG KING CREEK | 203.49522 | 41217 | 30477 | 55741 |
| LONG KING CREEK | 210.93082 | 41217 | 30477 | 55741 |
| LONG KING CREEK | 211.91724 | 41217 | 30477 | 55741 |
| LONG KING CREEK | 216.64756 | 41562 | 30732 | 56208 |
| LONG KING CREEK | 217.92046 | 41589 | 30752 | 56244 |
| LONG KING CREEK | 219.837 | 41758 | 30878 | 56473 |
| LONG KING CREEK | 220.79571 | 41758 | 30878 | 56473 |
| LONG KING CREEK | 221.89181 | 41758 | 30878 | 56473 |
| LONG TOM CREEK | 1.05557 | 1280 | 946 | 1731 |
| LONG TOM CREEK | 1.43184 | 1542 | 1140 | 2085 |
| LONG TOM CREEK | 3.51858 | 2931 | 2167 | 3963 |
| LONG TOM CREEK | 3.93459 | 3090 | 2285 | 4178 |
| LONG TOM CREEK | 4.43363 | 3299 | 2439 | 4461 |
| LONG TOM CREEK | 4.92146 | 3701 | 2737 | 5005 |
| LONG TOM CREEK | 5.23192 | 3701 | 2737 | 5005 |
| LONG TOM CREEK | 6.56871 | 4278 | 3163 | 5785 |
| LONG TOM CREEK | 15.01512 | 8114 | 5999 | 10973 |
| LONG TOM CREEK | 16.91865 | 8655 | 6400 | 11704 |
| LONG TOM CREEK | 20.61802 | 9810 | 7254 | 13267 |
| LONG TOM CREEK | 22.42121 | 10193 | 7537 | 13784 |
| LONG TOM CREEK | 23.13794 | 10277 | 7599 | 13898 |
| LONG TOM CREEK | 24.70959 | 10571 | 7817 | 14296 |
| LONG TOM CREEK | 29.60383 | 12370 | 9147 | 16728 |
| LONG TOM CREEK | 30.37594 | 12370 | 9147 | 16728 |
| LONG TOM CREEK | 32.08216 | 12773 | 9445 | 17275 |
| LONG TOM CREEK | 33.98707 | 13224 | 9778 | 17884 |
| LONG TOM CREEK | 34.62779 | 13224 | 9778 | 17884 |
| LONG TOM CREEK | 41.70801 | 14757 | 10912 | 19958 |
| LONG TOM CREEK | 42.25727 | 14795 | 10940 | 20009 |
| MEADOW CREEK | 1.07063 | 1548 | 1145 | 2094 |
| MEADOW CREEK | 1.8092 | 1914 | 1415 | 2589 |
| MEADOW CREEK | 2.24807 | 2358 | 1743 | 3188 |
| MEADOW CREEK | 4.87012 | 4300 | 3180 | 5815 |
| MEADOW CREEK | 5.80593 | 4568 | 3378 | 6178 |
| MEADOW CREEK | 7.86568 | 5767 | 4264 | 7799 |
| MILTON CREEK | 1.20434 | 1506 | 1114 | 2037 |
| MILTON CREEK | 1.75954 | 1871 | 1384 | 2531 |
| MILTON CREEK | 7.76336 | 6656 | 4921 | 9001 |
| MILTON CREEK | 8.68585 | 6763 | 5001 | 9146 |
| MILTON CREEK | 11.82611 | 8533 | 6310 | 11540 |
| MILTON CREEK | 12.82615 | 8626 | 6378 | 11666 |
| MILTON CREEK TRIBUTARY NO1 | 1.43552 | 2196 | 1624 | 2970 |
| MILTON CREEK TRIBUTARY NO1 | 2.46589 | 3213 | 2376 | 4345 |
| MILTON CREEK TRIBUTARY NO1 | 4.93563 | 5132 | 3795 | 6940 |
| MILTON CREEK TRIBUTARY NO1 | 5.97934 | 5358 | 3962 | 7246 |
| MORGAN CREEK | 1.08893 | 1707 | 1262 | 2308 |
| MORGAN CREEK | 1.95626 | 2300 | 1700 | 3110 |
| MORGAN CREEK | 5.98483 | 5129 | 3792 | 6936 |
| MORGAN CREEK | 6.11262 | 5253 | 3884 | 7104 |
| MUD CREEK | 1.35401 | 2274 | 1682 | 3076 |
| MUD CREEK | 2.35379 | 3023 | 2235 | 4088 |
| MUD CREEK | 2.98089 | 3178 | 2350 | 4297 |
| MUD CREEK | 3.01655 | 3178 | 2350 | 4297 |
| MUD CREEK | 3.85177 | 3717 | 2749 | 5027 |
| MUD CREEK | 5.38526 | 4548 | 3363 | 6150 |
| PLUM CREEK | 1.12094 | 1933 | 1429 | 2614 |
| PLUM CREEK | 2.27843 | 2800 | 2070 | 3786 |
| PLUM CREEK | 3.23071 | 3186 | 2356 | 4309 |
| PLUM CREEK | 5.20797 | 4515 | 3339 | 6107 |
| PLUM CREEK | 6.31555 | 5118 | 3785 | 6922 |
| SAND CREEK | 1.00104 | 1322 | 978 | 1788 |
| SAND CREEK | 1.58 | 1819 | 1345 | 2460 |
| SAND CREEK | 2.47961 | 2471 | 1827 | 3342 |
| SAND CREEK | 5.38006 | 4634 | 3426 | 6267 |
| SANDY CREEK | 1.21671 | 1798 | 1330 | 2432 |
| SANDY CREEK | 2.21094 | 2645 | 1956 | 3577 |
| SANDY CREEK | 3.02851 | 2961 | 2190 | 4005 |
| SANDY CREEK | 3.95589 | 3525 | 2606 | 4767 |
| SANDY CREEK | 4.5707 | 3845 | 2843 | 5199 |
| SANDY CREEK BRANCH | 0.23432 | 488 | 361 | 659 |
| SANDY CREEK BRANCH | 0.47725 | 843 | 623 | 1140 |
| SANSON CREEK | 1.01683 | 1428 | 1056 | 1931 |
| SANSON CREEK | 1.69217 | 2022 | 1495 | 2735 |
| SANSON CREEK | 2.44517 | 2623 | 1940 | 3548 |
| SANSON CREEK | 3.62046 | 3545 | 2621 | 4794 |
| SANSON CREEK | 3.90024 | 3545 | 2621 | 4794 |
| SANSON CREEK | 4.18344 | 3744 | 2769 | 5063 |
| SANSON CREEK | 4.70104 | 4050 | 2994 | 5477 |
| TEMPE CREEK | 26.30842 | 21301 | 15750 | 28807 |
| TEMPE CREEK | 26.3237 | 21301 | 15750 | 28807 |
| TEMPE CREEK | 27.25443 | 21301 | 15750 | 28807 |
| TEMPE CREEK | 27.61591 | 21301 | 15750 | 28807 |
| TEMPE CREEK | 28.19217 | 21301 | 15750 | 28807 |
| TEMPE CREEK | 28.34372 | 21301 | 15750 | 28807 |
| TEMPE CREEK | 29.2914 | 21301 | 15750 | 28807 |
| TEMPE CREEK | 31.97664 | 21764 | 16093 | 29433 |
| TEMPE CREEK | 33.42121 | 21764 | 16093 | 29433 |
| TEMPE CREEK | 35.03949 | 21764 | 16093 | 29433 |
| TEMPE CREEK | 36.83649 | 21764 | 16093 | 29433 |
| UNT 001 in LKiC | 1.22617 | 1407 | 1041 | 1903 |
| UNT 002 in LKiC | 1.24559 | 1720 | 1272 | 2326 |
| UNT 002 in LKiC | 2.16574 | 2424 | 1792 | 3278 |
| UNT 002 in LKiC | 3.35762 | 3355 | 2481 | 4537 |
| UNT 003 in LKiC | 0.66503 | 1324 | 979 | 1791 |
| UNT 003 in LKiC | 1.1083 | 1939 | 1433 | 2622 |
| UNT 004 in LKiC | 1.30262 | 1770 | 1309 | 2394 |
| UNT 005 in LKiC | 1.11708 | 1056 | 781 | 1428 |
| UNT 006 in LKiC | 1.03207 | 1157 | 855 | 1564 |
| UNT 007 in LKiC | 0.87101 | 652 | 482 | 882 |
| UNT 008 in LKiC | 0.47949 | 675 | 499 | 913 |
| UNT 009 in LKiC | 1.30482 | 1664 | 1231 | 2251 |
| UNT 010 in LKiC | 1.01207 | 1493 | 1104 | 2018 |
| UNT 011 in LKiC | 0.91244 | 1513 | 1118 | 2046 |
| UNT 012 in LKiC | 1.14533 | 1628 | 1203 | 2201 |
| UNT 012 in LKiC | 2.22475 | 2747 | 2031 | 3715 |
| UNT 013 in LKiC | 1.14192 | 1726 | 1276 | 2334 |
| UNT 013 in LKiC | 2.20431 | 2484 | 1837 | 3359 |
| UNT 013 in LKiC | 3.43604 | 3596 | 2659 | 4863 |
| UNT 014 in LKiC | 0.59204 | 672 | 497 | 909 |
| UNT 015 in LKiC | 0.42147 | 659 | 487 | 891 |
| UNT 016 in LKiC | 0.31682 | 587 | 434 | 794 |
| UNT 017 in LKiC | 0.75776 | 967 | 715 | 1308 |
| UNT 018 in LKiC | 0.12949 | 622 | 460 | 841 |
| UNT 019 in LKiC | 0.36085 | 1160 | 857 | 1568 |
| UNT 020 in LKiC | 0.7417 | 2304 | 1703 | 3115 |
| UNT 021 in LKiC | 0.28678 | 935 | 692 | 1265 |
| UNT 022 in LKiC | 0.28624 | 826 | 611 | 1117 |
| UNT 023 in LKiC | 0.56217 | 814 | 602 | 1101 |
| UNT 024 in LKiC | 0.45909 | 1144 | 846 | 1546 |
| UNT 025 in LKiC | 0.39251 | 1640 | 1213 | 2218 |
| UNT 026 in LKiC | 0.456 | 1200 | 888 | 1623 |
| UNT 027 in LKiC | 0.6879 | 907 | 671 | 1227 |
| UNT 028 in LKiC | 0.64665 | 1017 | 752 | 1375 |
| UNT 029 in LKiC | 0.90231 | 1327 | 981 | 1795 |
| UNT 030 in LKiC | 0.89666 | 758 | 561 | 1025 |
| UNT 031 in LKiC | 0.41887 | 826 | 611 | 1117 |
| UNT 032 in LKiC | 1.36737 | 899 | 665 | 1216 |
| UNT 033 in LKiC | 1.06633 | 1470 | 1087 | 1988 |
| UNT 033 in LKiC | 1.67684 | 1968 | 1456 | 2662 |
| UNT 033 in LKiC | 2.48445 | 2412 | 1784 | 3263 |
| UNT 034 in LKiC | 1.01172 | 1715 | 1268 | 2319 |
| UNT 034 in LKiC | 1.21241 | 1863 | 1377 | 2519 |
| UNT 034 in LKiC | 1.28155 | 1992 | 1473 | 2694 |
| UNT 035 in LKiC | 1.17322 | 1425 | 1054 | 1927 |
| UNT 036 in LKiC | 1.01207 | 1118 | 826 | 1512 |
| UNT 036 in LKiC | 1.67791 | 1659 | 1227 | 2244 |
| UNT 037 in LKiC | 1.00122 | 1615 | 1194 | 2184 |
| UNT 037 in LKiC | 1.59147 | 2089 | 1545 | 2826 |
| UNT 038 in LKiC | 1.3749 | 1992 | 1473 | 2694 |
| UNT 038 in LKiC | 2.75536 | 2919 | 2159 | 3948 |
| UNT 038 in LKiC | 3.65798 | 3359 | 2484 | 4543 |
| UNT 039 in LKiC | 0.86196 | 1361 | 1006 | 1840 |
| UNT 039 in LKiC | 1.52385 | 1784 | 1319 | 2412 |
| UNT 040 in LKiC | 1.13838 | 1497 | 1107 | 2024 |
| UNT 040 in LKiC | 1.86184 | 2083 | 1540 | 2817 |
| UNT 041 in LKiC | 1.31033 | 1130 | 835 | 1528 |
| UNT 041 in LKiC | 1.91465 | 1750 | 1294 | 2366 |
| UNT 042 in LKiC | 1.05897 | 1544 | 1141 | 2088 |
| UNT 042 in LKiC | 1.49955 | 1724 | 1275 | 2331 |
| UNT 043 in LKiC | 0.83555 | 1225 | 906 | 1657 |
| UNT 043 in LKiC | 1.48825 | 1803 | 1333 | 2438 |
| UNT 043 in LKiC | 1.84812 | 2118 | 1566 | 2865 |
| UNT 044 in LKiC | 0.90222 | 1674 | 1238 | 2264 |
| UNT 044 in LKiC | 1.22523 | 1885 | 1393 | 2549 |
| UNT 045 in LKiC | 1.40342 | 2166 | 1602 | 2929 |
| UNT 045 in LKiC | 1.72481 | 2183 | 1614 | 2952 |
| UNT 046 in LKiC | 0.73274 | 980 | 725 | 1326 |
| UNT 046 in LKiC | 1.53309 | 1604 | 1186 | 2169 |
| UNT 047 in LKiC | 1.11125 | 1638 | 1211 | 2215 |
| UNT 047 in LKiC | 2.00137 | 2395 | 1771 | 3239 |
| UNT 049 in LKiC | 1.29688 | 2152 | 1591 | 2910 |
| UNT 049 in LKiC | 1.59972 | 2215 | 1638 | 2995 |
| UNT 050 in LKiC | 1.15446 | 1463 | 1082 | 1978 |
| UNT 051 in LKiC | 1.09574 | 2148 | 1589 | 2905 |
| UNT 052 in LKiC | 0.83488 | 1622 | 1200 | 2194 |
| UNT 052 in LKiC | 1.52053 | 2139 | 1582 | 2893 |
| UNT 053 in LKiC | 1.00104 | 1334 | 987 | 1804 |
| UNT 053 in LKiC | 1.96864 | 2375 | 1756 | 3212 |
| UNT 053 in LKiC | 2.57179 | 2609 | 1929 | 3528 |
| UNT 054 in LKiC | 0.9884 | 1238 | 915 | 1674 |
| UNT 054 in LKiC | 1.00015 | 1240 | 917 | 1677 |
| UNT 054 in LKiC | 1.00759 | 1256 | 928 | 1698 |
| UNT 055 in LKiC | 0.9962 | 2877 | 2127 | 3891 |
| UNT 056 in LKiC | 1.02517 | 1421 | 1051 | 1921 |
| UNT 056 in LKiC | 1.18909 | 1421 | 1051 | 1921 |
| UNT 057 in LKiC | 1.09727 | 1539 | 1138 | 2081 |
| UNT 057 in LKiC | 1.86219 | 2124 | 1571 | 2873 |
| UNT 058 in LKiC | 0.70583 | 1054 | 780 | 1426 |
| UNT 059 in LKiC | 0.7026 | 969 | 717 | 1311 |
| UNT 060 in LKiC | 0.79389 | 1026 | 759 | 1388 |
| UNT 061 in LKiC | 0.70619 | 1093 | 808 | 1478 |
| UNT 062 in LKiC | 0.50263 | 761 | 563 | 1029 |
| UNT 063 in LKiC | 0.33415 | 518 | 383 | 701 |
| UNT 063 in LKiC | 0.43793 | 693 | 513 | 938 |
| UNT 064 in LKiC | 1.15887 | 1272 | 940 | 1720 |
| UNT 064 in LKiC | 1.28989 | 1349 | 997 | 1824 |
| UNT 065 in LKiC | 0.35619 | 422 | 312 | 571 |
| UNT 065 in LKiC | 0.53419 | 574 | 425 | 777 |
| UNT 066 in LKiC | 0.65124 | 1200 | 887 | 1623 |
| UNT 067 in LKiC | 0.97873 | 1642 | 1214 | 2221 |
| UNT 068 in LKiC | 1.10578 | 1414 | 1046 | 1913 |
| UNT 069 in LKiC | 0.64745 | 797 | 589 | 1077 |
| UNT 070 in LKiC | 0.85362 | 1157 | 856 | 1565 |
| UNT 070 in LKiC | 1.64854 | 9437 | 6978 | 12763 |
| UNT 071 in LKiC | 0.68118 | 1170 | 865 | 1582 |
| UNT 072 in LKiC | 0.433 | 765 | 566 | 1035 |
| UNT 073 in LKiC | 0.77461 | 807 | 596 | 1091 |
| UNT 074 in LKiC | 0.65813 | 1008 | 745 | 1363 |
| UNT 075 in LKiC | 0.63382 | 944 | 698 | 1276 |
| UNT 076 in LKiC | 0.49546 | 1244 | 920 | 1683 |
| UNT 077 in LKiC | 0.42022 | 1093 | 808 | 1478 |
| UNT 078 in LKiC | 2.10898 | 3019 | 2232 | 4083 |
| UNT 079 in LKiC | 1.05332 | 1195 | 884 | 1616 |
| UNT 079 in LKiC | 1.82328 | 2066 | 1528 | 2794 |
| UNT 079 in LKiC | 2.36303 | 2592 | 1916 | 3505 |
| UNT 080 in LKiC | 0.64665 | 812 | 600 | 1098 |
| UNT 081 in LKiC | 0.51196 | 941 | 696 | 1273 |
| UNT 081 in LKiC | 1.52313 | 2293 | 1696 | 3101 |
| UNT 082 in LKiC | 0.95943 | 1706 | 1262 | 2308 |
| UNT 083 in LKiC | 0.60522 | 1142 | 845 | 1545 |
| UNT 083 in LKiC | 1.22918 | 1850 | 1368 | 2502 |
| UNT 084 in LKiC | 0.50424 | 854 | 632 | 1155 |
| UNT 085 in LKiC | 1.20102 | 1889 | 1396 | 2554 |
| UNT 085 in LKiC | 2.00236 | 2573 | 1902 | 3480 |
| UNT 085 in LKiC | 3.52719 | 4069 | 3009 | 5503 |
| UNT 086 in LKiC | 0.47079 | 934 | 690 | 1263 |
| UNT 086 in LKiC | 0.57697 | 1052 | 778 | 1423 |
| UNT 086 in LKiC | 1.4348 | 2074 | 1533 | 2805 |
| UNT 087 in LKiC | 0.40327 | 746 | 551 | 1008 |
| UNT 087 in LKiC | 0.74143 | 1181 | 873 | 1597 |
| UNT 088 in LKiC | 0.67176 | 1056 | 781 | 1428 |
| UNT 088 in LKiC | 1.36369 | 1958 | 1448 | 2648 |
| UNT 088 in LKiC | 1.91734 | 2308 | 1707 | 3122 |
| UNT 088 in LKiC | 2.79553 | 3088 | 2284 | 4177 |
| UNT 088 in LKiC | 3.46684 | 3713 | 2746 | 5022 |
| UNT 089 in LKiC | 0.63015 | 1122 | 830 | 1518 |
| UNT 090 in LKiC | 0.39995 | 664 | 491 | 898 |
| UNT 090 in LKiC | 0.65158 | 1045 | 773 | 1413 |
| UNT 091 in LKiC | 1.01423 | 1693 | 1252 | 2290 |
| UNT 091 in LKiC | 1.86076 | 2326 | 1720 | 3146 |
| UNT 091 in LKiC | 2.51377 | 2987 | 2209 | 4040 |
| UNT 092 in LKiC | 0.64871 | 864 | 639 | 1169 |
| UNT 093 in LKiC | 0.16886 | 342 | 253 | 462 |
| UNT 094 in LKiC | 0.45833 | 572 | 423 | 774 |
| UNT 095 in LKiC | 0.37009 | 647 | 479 | 875 |
| UNT 096 in LKiC | 1.14623 | 1433 | 1060 | 1938 |
| UNT 097 in LKiC | 0.30884 | 1055 | 780 | 1427 |
| UNT 098 in LKiC | 0.11936 | 599 | 443 | 810 |
| UNT 099 in LKiC | 0.522 | 402 | 297 | 544 |
| UNT 100 in LKiC | 0.5732 | 1062 | 785 | 1436 |
| UNT 101 in LKiC | 0.52568 | 894 | 661 | 1209 |
| UNT 102 in LKiC | 0.64181 | 1113 | 823 | 1505 |
| UNT 102 in LKiC | 2.0774 | 2653 | 1961 | 3587 |
| UNT 103 in LKiC | 0.78288 | 1236 | 914 | 1671 |
| UNT 104 in LKiC | 0.44578 | 628 | 464 | 849 |
| UNT 104 in LKiC | 0.81407 | 995 | 736 | 1346 |
| UNT 104 in LKiC | 1.23537 | 1261 | 932 | 1705 |
| UNT 105 in LKiC | 0.26804 | 477 | 353 | 645 |
| UNT 106 in LKiC | 0.35368 | 625 | 462 | 845 |
| UNT 107 in LKiC | 0.54669 | 953 | 705 | 1289 |
| UNT 107 in LKiC | 0.91077 | 2079 | 1537 | 2811 |
| UNT 108 in LKiC | 0.23954 | 690 | 510 | 933 |
| WEST TEMPE CREEK | 1.00373 | 3328 | 2461 | 4501 |
| WEST TEMPE CREEK | 1.41992 | 4316 | 3191 | 5837 |
| WEST TEMPE CREEK | 1.82713 | 4862 | 3595 | 6575 |
| WEST TEMPE CREEK | 2.37424 | 5604 | 4144 | 7579 |
| WEST TEMPE CREEK | 2.72164 | 6072 | 4490 | 8212 |
| WEST TEMPE CREEK | 3.52339 | 7156 | 5292 | 9678 |
| WEST TEMPE CREEK | 3.78429 | 7794 | 5763 | 10540 |
| WEST TEMPE CREEK | 4.37148 | 8087 | 5979 | 10936 |
| WEST TEMPE CREEK | 5.40804 | 9518 | 7038 | 12872 |
| WEST TEMPE CREEK | 7.05905 | 11577 | 8560 | 15656 |
| WEST TEMPE CREEK | 7.10667 | 11577 | 8560 | 15656 |
| WEST TEMPE CREEK | 9.0719 | 13567 | 10032 | 18348 |
| WEST TEMPE CREEK | 9.5704 | 13883 | 10265 | 18775 |
| WEST TEMPE CREEK | 10.1272 | 14338 | 10602 | 19390 |
| WEST TEMPE CREEK | 10.95131 | 14733 | 10894 | 19925 |
| WEST TEMPE CREEK | 12.18165 | 15624 | 11553 | 21129 |
| WEST TEMPE CREEK | 12.94111 | 15933 | 11782 | 21548 |
| WEST TEMPE CREEK | 13.08531 | 16029 | 11852 | 21677 |
| WEST TEMPE CREEK | 13.57843 | 16138 | 11933 | 21824 |
| WEST TEMPE CREEK | 14.89056 | 16907 | 12501 | 22864 |
| WEST TEMPE CREEK TRIBUTARY | 0.9667 | 2374 | 1756 | 3211 |
| WILLS CREEK | 1.9994 | 2340 | 1730 | 3165 |
| WILLS CREEK | 2.44571 | 2572 | 1902 | 3479 |
| WILLS CREEK | 4.82614 | 3979 | 2942 | 5381 |
| WILLS CREEK | 6.39716 | 4842 | 3580 | 6548 |
| WILLS CREEK | 9.44019 | 6345 | 4692 | 8581 |
| Menard Creek - Trinity River |  |  |  |  |
| ACE EMANUEL BRANCH | 1.250072 | 1311 | 969 | 1773 |
| BEECH CREEK | 1.044536 | 1104 | 816 | 1493 |
| BEECH CREEK | 2.032129 | 1554 | 1149 | 2101 |
| BEECH CREEK | 4.042467 | 2652 | 1961 | 3586 |
| BEECH CREEK | 4.576824 | 2652 | 1961 | 3586 |
| BIG CREEK | 10.916068 | 9419 | 6964 | 12737 |
| BIG CREEK | 12.22936 | 9923 | 7337 | 13420 |
| BIG CREEK | 12.794493 | 10144 | 7501 | 13718 |
| BIG CREEK | 13.346713 | 10352 | 7655 | 14000 |
| BIG CREEK | 18.610555 | 13170 | 9738 | 17810 |
| BIG CREEK | 19.621105 | 13579 | 10041 | 18364 |
| BIG CREEK | 19.8652 | 13727 | 10150 | 18564 |
| BIG CREEK | 20.37034 | 14011 | 10360 | 18948 |
| BIG CREEK | 22.589442 | 14830 | 10966 | 20055 |
| BIG CREEK | 23.307292 | 14907 | 11023 | 20160 |
| BIG CREEK | 24.338377 | 14963 | 11064 | 20235 |
| BIG CREEK | 27.163234 | 16121 | 11920 | 21801 |
| BIG CREEK | 28.144011 | 16346 | 12087 | 22106 |
| BIG CREEK | 36.532315 | 19287 | 14262 | 26084 |
| BIG CREEK | 37.057991 | 19287 | 14262 | 26084 |
| BIG CREEK | 38.044687 | 19287 | 14262 | 26084 |
| BIG CREEK | 38.872658 | 19287 | 14262 | 26084 |
| BIG CREEK | 39.789048 | 19287 | 14262 | 26084 |
| BIG CREEK | 40.608948 | 19287 | 14262 | 26084 |
| BIG CREEK | 40.992937 | 19287 | 14262 | 26084 |
| BIG CREEK | 42.849213 | 19287 | 14262 | 26084 |
| BIG CREEK | 43.530744 | 19287 | 14262 | 26084 |
| BIG CREEK | 44.442113 | 19287 | 14262 | 26084 |
| BIG CREEK | 58.202946 | 19287 | 14262 | 26084 |
| BIG CREEK | 70.572827 | 19934 | 14740 | 26958 |
| BIG CREEK | 71.918761 | 20191 | 14930 | 27307 |
| BIG CREEK | 77.959317 | 20352 | 15049 | 27524 |
| BIG CREEK | 78.577359 | 20352 | 15049 | 27524 |
| BIG CREEK | 79.309198 | 20352 | 15049 | 27524 |
| BIG CREEK | 87.021493 | 21154 | 15642 | 28608 |
| BIG CREEK | 87.142644 | 21154 | 15642 | 28608 |
| BIG CREEK | 89.314577 | 21154 | 15642 | 28608 |
| BLACK BRANCH | 3.209922 | 2093 | 1547 | 2830 |
| BLACK BRANCH | 4.086064 | 2364 | 1748 | 3198 |
| BLACK BRANCH | 5.16224 | 2530 | 1871 | 3422 |
| BLUE BRANCH | 1.689928 | 1111 | 822 | 1503 |
| BLUE BRANCH | 2.115885 | 1250 | 924 | 1691 |
| BLUE BRANCH | 6.102395 | 2592 | 1917 | 3505 |
| BLUE BRANCH | 9.056833 | 3284 | 2428 | 4442 |
| BLUE BRANCH | 10.905845 | 3895 | 2880 | 5268 |
| BLUE BRANCH | 12.932503 | 4342 | 3211 | 5873 |
| BLUFF CREEK | 1.001761 | 879 | 650 | 1189 |
| BLUFF CREEK | 1.99034 | 1389 | 1027 | 1878 |
| BLUFF CREEK | 2.980623 | 1896 | 1402 | 2564 |
| BLUFF CREEK | 3.972879 | 2277 | 1684 | 3080 |
| BLUFF CREEK | 4.18774 | 2330 | 1723 | 3151 |
| BLUFF CREEK | 5.05741 | 2739 | 2025 | 3704 |
| BLUFF CREEK | 6.519564 | 3214 | 2377 | 4347 |
| BLUFF CREEK | 7.137425 | 3251 | 2404 | 4397 |
| BLUFF CREEK | 7.395062 | 3354 | 2480 | 4537 |
| BLUFF CREEK | 10.638523 | 4347 | 3214 | 5878 |
| BLUFF CREEK | 12.617564 | 4765 | 3523 | 6444 |
| BLUFF CREEK | 13.480329 | 4827 | 3570 | 6528 |
| BLUFF CREEK | 14.43519 | 4973 | 3677 | 6725 |
| BLUFF CREEK | 15.035206 | 4973 | 3677 | 6725 |
| BURNT OUT CREEK | 0.94419 | 684 | 506 | 925 |
| BURNT OUT CREEK | 1.44664 | 982 | 726 | 1329 |
| CAMP BRANCH | 1.250879 | 1222 | 903 | 1652 |
| CAMP BRANCH | 2.134717 | 1407 | 1040 | 1903 |
| CAMP BRANCH | 2.83777 | 1695 | 1254 | 2293 |
| COLEY CREEK | 1.003196 | 1381 | 1021 | 1868 |
| COLEY CREEK | 1.928644 | 1744 | 1290 | 2359 |
| COLEY CREEK | 2.124853 | 1755 | 1298 | 2374 |
| COLEY CREEK | 3.887777 | 2868 | 2120 | 3878 |
| COLEY CREEK | 5.118927 | 3272 | 2419 | 4425 |
| COLEY CREEK | 6.417424 | 3626 | 2681 | 4903 |
| COLEY CREEK | 7.168184 | 3733 | 2760 | 5049 |
| COLEY CREEK | 12.353112 | 5234 | 3870 | 7078 |
| COLEY CREEK | 12.813863 | 5234 | 3870 | 7078 |
| COLEY CREEK | 13.077956 | 5234 | 3870 | 7078 |
| COLEY CREEK | 13.25919 | 5234 | 3870 | 7078 |
| COLEY CREEK | 13.580586 | 5234 | 3870 | 7078 |
| COLEY CREEK | 13.747471 | 5234 | 3870 | 7078 |
| COPELAND CREEK | 1.042384 | 958 | 708 | 1295 |
| COPELAND CREEK | 3.549881 | 2037 | 1506 | 2755 |
| COPELAND CREEK | 4.537563 | 2515 | 1860 | 3401 |
| COPELAND CREEK | 5.171746 | 2673 | 1977 | 3615 |
| COPELAND CREEK | 7.22719 | 3458 | 2557 | 4676 |
| COPELAND CREEK | 13.080288 | 5478 | 4050 | 7408 |
| COPELAND CREEK | 13.916868 | 5693 | 4210 | 7699 |
| COPELAND CREEK | 14.936205 | 5710 | 4222 | 7722 |
| DAVISO BAYOU | 1.210704 | 292 | 216 | 395 |
| DAVISO BAYOU | 2.576905 | 630 | 466 | 852 |
| DAVISO BAYOU | 3.45079 | 768 | 568 | 1038 |
| DAVISO BAYOU | 4.346914 | 837 | 619 | 1132 |
| DAVISO BAYOU | 4.99114 | 837 | 619 | 1132 |
| DAVISO BAYOU | 9.866689 | 1246 | 922 | 1685 |
| DOUBLELAKE BRANCH | 1.224963 | 1958 | 1448 | 2648 |
| DOUBLELAKE BRANCH | 1.954291 | 2901 | 2145 | 3924 |
| DOUBLELAKE BRANCH | 2.963674 | 3839 | 2839 | 5192 |
| DOUBLELAKE BRANCH | 3.855417 | 4459 | 3297 | 6030 |
| DRY BRANCH | 1.278409 | 671 | 496 | 908 |
| DRY BRANCH | 1.63164 | 847 | 626 | 1145 |
| DRY BRANCH | 2.630262 | 1241 | 918 | 1679 |
| DRY BRANCH | 3.627091 | 1593 | 1178 | 2154 |
| DRY BRANCH | 4.535859 | 1850 | 1368 | 2502 |
| DRY BRANCH | 5.49637 | 2134 | 1578 | 2886 |
| DRY BRANCH | 6.438497 | 2380 | 1760 | 3219 |
| DRY BRANCH | 7.098417 | 2477 | 1832 | 3350 |
| DRY BRANCH | 12.25698 | 3785 | 2799 | 5119 |
| DRY BRANCH | 12.883541 | 3918 | 2897 | 5299 |
| DRY BRANCH | 13.522387 | 4001 | 2959 | 5411 |
| DRY BRANCH | 14.510517 | 4213 | 3115 | 5697 |
| DRY BRANCH | 15.643008 | 4312 | 3188 | 5831 |
| DRY BRANCH | 15.827263 | 4312 | 3188 | 5831 |
| EAST MERNARD CREEK | 1.008846 | 906 | 670 | 1226 |
| EAST MERNARD CREEK | 2.056431 | 1431 | 1058 | 1935 |
| FISH BRANCH | 1.054939 | 1375 | 1016 | 1859 |
| FISH BRANCH | 1.867485 | 1651 | 1221 | 2233 |
| FISH BRANCH | 2.42652 | 1720 | 1272 | 2327 |
| FISH BRANCH | 4.630287 | 2297 | 1699 | 3107 |
| FISH BRANCH | 5.165469 | 2505 | 1852 | 3388 |
| FISH BRANCH | 5.825029 | 2620 | 1938 | 3544 |
| FISH BRANCH | 6.336626 | 2795 | 2067 | 3780 |
| FISH BRANCH | 8.403908 | 3254 | 2406 | 4401 |
| GREEN BRIAR HOLLOW CREEK | 1.048572 | 837 | 619 | 1132 |
| GREEN BRIAR HOLLOW CREEK | 2.039569 | 1336 | 988 | 1807 |
| HENRY LAKE BRANCH | 0.583158 | 989 | 732 | 1338 |
| HENRY LAKE BRANCH | 2.255958 | 3012 | 2227 | 4073 |
| HENRY LAKE BRANCH | 2.797775 | 3480 | 2573 | 4706 |
| HENRY LAKE BRANCH | 3.259133 | 3901 | 2885 | 5276 |
| HENRY LAKE BRANCH | 3.750574 | 4275 | 3161 | 5782 |
| HENRY LAKE BRANCH | 3.977093 | 4515 | 3338 | 6106 |
| HENRY LAKE BRANCH | 6.571293 | 6323 | 4675 | 8551 |
| ISHAM BRANCH | 1.006424 | 765 | 566 | 1035 |
| ISHAM BRANCH | 1.588218 | 1078 | 797 | 1458 |
| LITTLE CREEK | 1.077178 | 1334 | 986 | 1804 |
| LITTLE CREEK | 1.942633 | 2202 | 1628 | 2978 |
| LITTLE CREEK | 2.736079 | 2920 | 2159 | 3949 |
| LITTLE CREEK | 3.731204 | 3774 | 2790 | 5103 |
| LITTLE CREEK | 4.57702 | 4350 | 3216 | 5883 |
| LITTLE CREEK | 5.214162 | 4564 | 3375 | 6173 |
| MATHIS BRANCH | 8.184203 | 3279 | 2424 | 4434 |
| MEETINGHOUSE BRANCH | 1.044357 | 533 | 394 | 721 |
| MEETINGHOUSE BRANCH | 1.890801 | 838 | 620 | 1133 |
| MEETINGHOUSE BRANCH | 2.804322 | 1196 | 884 | 1617 |
| MEETINGHOUSE BRANCH | 3.668521 | 1411 | 1043 | 1908 |
| MEETINGHOUSE BRANCH | 5.508655 | 1839 | 1360 | 2486 |
| MEETINGHOUSE BRANCH | 7.891055 | 2502 | 1850 | 3383 |
| MENARD CREEK | 1.008218 | 841 | 622 | 1137 |
| MENARD CREEK | 2.005585 | 1369 | 1012 | 1851 |
| MENARD CREEK | 2.774101 | 1598 | 1182 | 2162 |
| MENARD CREEK | 5.83032 | 2738 | 2025 | 3703 |
| MENARD CREEK | 6.60063 | 2917 | 2157 | 3945 |
| MENARD CREEK | 9.588427 | 3783 | 2797 | 5116 |
| MENARD CREEK | 10.310001 | 3837 | 2837 | 5189 |
| MENARD CREEK | 14.886952 | 4152 | 3070 | 5615 |
| MENARD CREEK | 16.696528 | 5675 | 4196 | 7674 |
| MENARD CREEK | 17.777629 | 5698 | 4213 | 7706 |
| MENARD CREEK | 18.664898 | 5742 | 4246 | 7766 |
| MENARD CREEK | 20.583588 | 5962 | 4409 | 8063 |
| MENARD CREEK | 21.116707 | 5962 | 4409 | 8063 |
| MENARD CREEK | 30.04629 | 7614 | 5630 | 10298 |
| MENARD CREEK | 32.882171 | 7889 | 5833 | 10669 |
| MENARD CREEK | 33.959445 | 8029 | 5937 | 10858 |
| MENARD CREEK | 49.348958 | 10302 | 7618 | 13932 |
| MENARD CREEK | 51.261191 | 10495 | 7761 | 14194 |
| MENARD CREEK | 53.022455 | 10655 | 7879 | 14409 |
| MENARD CREEK | 69.109777 | 12894 | 9534 | 17437 |
| MENARD CREEK | 74.864052 | 13684 | 10118 | 18506 |
| MENARD CREEK | 76.674146 | 13847 | 10239 | 18727 |
| MENARD CREEK | 80.492962 | 14165 | 10474 | 19156 |
| MENARD CREEK | 81.131808 | 14241 | 10530 | 19259 |
| MENARD CREEK | 81.902835 | 14291 | 10567 | 19326 |
| MENARD CREEK | 82.850702 | 14291 | 10567 | 19326 |
| MENARD CREEK | 83.550796 | 14354 | 10614 | 19412 |
| MENARD CREEK | 84.152875 | 14354 | 10614 | 19412 |
| MENARD CREEK | 85.107467 | 14373 | 10628 | 19438 |
| MENARD CREEK | 85.853385 | 14373 | 10628 | 19438 |
| MENARD CREEK | 86.672029 | 14373 | 10628 | 19438 |
| MENARD CREEK | 87.824624 | 14373 | 10628 | 19438 |
| MENARD CREEK | 89.695607 | 14420 | 10663 | 19501 |
| MENARD CREEK | 91.478528 | 14422 | 10664 | 19505 |
| MENARD CREEK | 93.523122 | 14529 | 10744 | 19649 |
| MENARD CREEK | 94.170756 | 14529 | 10744 | 19649 |
| MENARD CREEK | 97.948143 | 14861 | 10989 | 20098 |
| MENARD CREEK | 98.530672 | 14865 | 10992 | 20103 |
| MENARD CREEK | 101.442963 | 14950 | 11055 | 20219 |
| MENARD CREEK | 102.640037 | 15055 | 11132 | 20360 |
| MENARD CREEK | 104.305304 | 15083 | 11153 | 20398 |
| MENARD CREEK | 112.647336 | 15837 | 11711 | 21418 |
| MENARD CREEK | 114.644223 | 15930 | 11779 | 21544 |
| MENARD CREEK | 115.583749 | 16001 | 11831 | 21639 |
| MENARD CREEK | 116.621381 | 16001 | 11831 | 21639 |
| MENARD CREEK | 119.008623 | 16236 | 12005 | 21957 |
| MENARD CREEK | 149.855774 | 19029 | 14071 | 25735 |
| MENARD CREEK | 150.853163 | 19029 | 14071 | 25735 |
| MENARD CREEK | 157.279291 | 19261 | 14242 | 26048 |
| MENARD CREEK | 158.524075 | 19641 | 14523 | 26562 |
| MENARD CREEK | 159.710938 | 19653 | 14532 | 26579 |
| MENARD CREEK | 164.777932 | 19912 | 14723 | 26928 |
| MILL CREEK | 1.03521 | 574 | 424 | 776 |
| MILL CREEK | 2.001281 | 1006 | 744 | 1361 |
| MILL CREEK | 2.261159 | 1140 | 843 | 1542 |
| MILL CREEK | 4.205496 | 1864 | 1378 | 2521 |
| MILL CREEK | 5.184928 | 2192 | 1621 | 2964 |
| MILL CREEK | 13.851494 | 4638 | 3429 | 6272 |
| MILL CREEK | 14.456084 | 4691 | 3469 | 6344 |
| MILL CREEK | 28.449893 | 7879 | 5826 | 10655 |
| MILL CREEK | 28.944631 | 7931 | 5864 | 10726 |
| MILL CREEK | 29.620826 | 7931 | 5864 | 10726 |
| MILL CREEK | 30.599317 | 7931 | 5864 | 10726 |
| MILL CREEK2 | 0.557486 | 505 | 373 | 683 |
| MUSSEL SHOALS | 0.246696 | 28 | 21 | 38 |
| MUSSEL SHOALS | 2.182604 | 510 | 377 | 690 |
| MUSSEL SHOALS | 12.184971 | 1540 | 1139 | 2083 |
| MUSSEL SHOALS2 | 0.15998 | 38 | 28 | 52 |
| MUSSEL SHOALS2 | 9.390424 | 3583 | 2649 | 4846 |
| SALLY CREEK | 0.369766 | 443 | 328 | 600 |
| SALLY CREEK | 2.658689 | 1595 | 1179 | 2156 |
| SALLY CREEK | 3.395819 | 1848 | 1367 | 2499 |
| SALLY CREEK | 5.908069 | 2783 | 2057 | 3763 |
| SALLY CREEK | 6.835489 | 3220 | 2381 | 4355 |
| SECOND CREEK | 1.079599 | 969 | 716 | 1310 |
| SECOND CREEK | 2.146554 | 1588 | 1175 | 2148 |
| SECOND CREEK | 2.777688 | 1961 | 1450 | 2652 |
| SULPHUR BRANCH | 0.948315 | 1576 | 1165 | 2132 |
| TURKEY CREEK | 0.906526 | 589 | 435 | 796 |
| TURKEY CREEK | 2.365182 | 1158 | 856 | 1566 |
| TURKEY CREEK | 3.214541 | 1504 | 1112 | 2033 |
| TURKEY CREEK | 6.091634 | 2757 | 2038 | 3728 |
| TURNER CREEK | 1.000954 | 735 | 544 | 995 |
| TURNER CREEK | 1.935369 | 1304 | 964 | 1763 |
| TURNER CREEK | 5.82763 | 3326 | 2459 | 4498 |
| UNT 08800 IN MCTR | 0.242661 | 128 | 95 | 173 |
| UNT 08859 IN MCTR | 0.700991 | 648 | 479 | 876 |
| UNT 08883 IN MCTR | 0.16025 | 134 | 99 | 181 |
| UNT 08886 IN MCTR | 0.449721 | 589 | 435 | 796 |
| UNT 08897 IN MCTR | 0.169217 | 278 | 206 | 376 |
| UNT 08903 IN MCTR | 0.150744 | 305 | 225 | 412 |
| UNT 08904 IN MCTR | 0.112273 | 243 | 180 | 329 |
| UNT 09050 IN MCTR | 0.37825 | 413 | 306 | 559 |
| UNT 09264 IN MCTR | 0.44712 | 374 | 276 | 505 |
| UNT 09269 IN MCTR | 0.269743 | 289 | 214 | 391 |
| UNT 09336 IN MCTR | 0.291713 | 333 | 246 | 450 |
| UNT 09358 IN MCTR | 0.180247 | 253 | 187 | 342 |
| UNT 09360 IN MCTR | 0.159801 | 196 | 145 | 265 |
| UNT 09402 IN MCTR | 0.307496 | 687 | 508 | 929 |
| UNT 09404 IN MCTR | 0.153524 | 499 | 369 | 675 |
| UNT 09415 IN MCTR | 0.130029 | 300 | 222 | 406 |
| UNT 09426 IN MCTR | 0.174508 | 527 | 390 | 713 |
| UNT 09430 IN MCTR | 0.252077 | 627 | 464 | 848 |
| UNT 09432 IN MCTR | 0.208181 | 571 | 422 | 772 |
| UNT 09445 IN MCTR | 0.473395 | 899 | 665 | 1216 |
| UNT 09447 IN MCTR | 0.137203 | 291 | 215 | 393 |
| UNT 09448 IN MCTR | 0.502002 | 970 | 717 | 1312 |
| UNT 09455 IN MCTR | 0.100616 | 213 | 157 | 288 |
| UNT 09458 IN MCTR | 0.191995 | 425 | 314 | 575 |
| UNT 09460 IN MCTR | 0.381568 | 674 | 498 | 911 |
| UNT 09477 IN MCTR | 0.129581 | 331 | 245 | 448 |
| UNT 09488 IN MCTR | 0.545853 | 889 | 658 | 1203 |
| UNT 09494 IN MCTR | 0.43905 | 651 | 481 | 880 |
| UNT 09500 IN MCTR | 0.521461 | 300 | 222 | 406 |
| UNT 09509 IN MCTR | 0.322203 | 136 | 100 | 184 |
| UNT 09531 IN MCTR | 0.222843 | 380 | 281 | 515 |
| UNT 09531 IN MCTR | 0.293866 | 499 | 369 | 675 |
| UNT 09535 IN MCTR | 0.364261 | 201 | 149 | 272 |
| UNT 09539 IN MCTR | 0.746008 | 515 | 381 | 697 |
| UNT 09539 IN MCTR | 1.27688 | 793 | 586 | 1072 |
| UNT 09541 IN MCTR | 0.365247 | 251 | 186 | 340 |
| UNT 09560 IN MCTR | 0.169038 | 102 | 75 | 138 |
| UNT 1118 IN MCTR | 1.017992 | 820 | 606 | 1109 |
| UNT 1118 IN MCTR | 1.732524 | 1011 | 748 | 1367 |
| UNT 1119 IN MCTR | 1.277064 | 566 | 419 | 766 |
| UNT 1119 IN MCTR | 1.724095 | 646 | 478 | 874 |
| UNT 1121 IN MCTR | 0.914238 | 732 | 541 | 990 |
| UNT 1121 IN MCTR | 1.554609 | 1121 | 829 | 1516 |
| UNT 1124 IN MCTR | 1.127934 | 1030 | 761 | 1392 |
| UNT 1124 IN MCTR | 2.429479 | 1891 | 1398 | 2558 |
| UNT 1124 IN MCTR | 3.452494 | 2310 | 1708 | 3124 |
| UNT 1127 IN MCTR | 1.053773 | 1095 | 810 | 1481 |
| UNT 1127 IN MCTR | 1.561514 | 1332 | 985 | 1801 |
| UNT 1134 IN MCTR | 1.000685 | 974 | 720 | 1317 |
| UNT 1134 IN MCTR | 2.193365 | 1612 | 1192 | 2180 |
| UNT 1135 IN MCTR | 1.315198 | 925 | 684 | 1251 |
| UNT 1137 IN MCTR | 1.023194 | 828 | 612 | 1120 |
| UNT 1138 IN MCTR | 0.945804 | 707 | 522 | 955 |
| UNT 1141 IN MCTR | 1.001761 | 672 | 497 | 909 |
| UNT 1141 IN MCTR | 1.854841 | 1080 | 798 | 1460 |
| UNT 1143 IN MCTR | 1.022835 | 784 | 579 | 1060 |
| UNT 1143 IN MCTR | 1.557299 | 960 | 710 | 1298 |
| UNT 1145 IN MCTR | 1.002479 | 897 | 663 | 1213 |
| UNT 1145 IN MCTR | 1.080227 | 909 | 672 | 1230 |
| UNT 1146 IN MCTR | 1.108654 | 1030 | 762 | 1393 |
| UNT 1146 IN MCTR | 1.439197 | 1210 | 895 | 1636 |
| UNT 1146 IN MCTR | 2.421409 | 1759 | 1301 | 2379 |
| UNT 1146 IN MCTR | 3.212882 | 2001 | 1480 | 2706 |
| UNT 1147 IN MCTR | 0.544687 | 567 | 419 | 767 |
| UNT 1147 IN MCTR | 1.04615 | 969 | 717 | 1311 |
| UNT 1148 IN MCTR | 1.4548 | 722 | 534 | 977 |
| UNT 1151 IN MCTR | 1.10704 | 713 | 527 | 964 |
| UNT 1151 IN MCTR | 2.107904 | 1098 | 812 | 1485 |
| UNT 1151 IN MCTR | 2.641023 | 1266 | 936 | 1712 |
| UNT 1151 IN MCTR | 3.432765 | 1566 | 1158 | 2117 |
| UNT 1151 IN MCTR | 4.312927 | 1841 | 1362 | 2490 |
| UNT 1155 IN MCTR | 1.31392 | 727 | 538 | 984 |
| UNT 1157 IN MCTR | 1.478564 | 985 | 728 | 1332 |
| UNT 1158 IN MCTR | 1.038976 | 571 | 422 | 772 |
| UNT 1159 IN MCTR | 1.08211 | 707 | 523 | 956 |
| UNT 1160 IN MCTR | 1.000237 | 370 | 274 | 501 |
| UNT 1160 IN MCTR | 1.2976 | 496 | 366 | 670 |
| UNT 1160 IN MCTR | 2.095841 | 868 | 642 | 1174 |
| UNT 1160 IN MCTR | 2.781365 | 1110 | 821 | 1501 |
| UNT 1161 IN MCTR | 1.223797 | 658 | 487 | 890 |
| UNT 1161 IN MCTR | 2.04558 | 1026 | 759 | 1387 |
| UNT 1162 IN MCTR | 1.18156 | 457 | 338 | 618 |
| UNT 1164 IN MCTR | 1.088029 | 538 | 398 | 728 |
| UNT 1165 IN MCTR | 1.266303 | 664 | 491 | 898 |
| UNT 1166 IN MCTR | 0.987772 | 641 | 474 | 867 |
| UNT 1166 IN MCTR | 1.110627 | 702 | 519 | 950 |
| UNT 1169 IN MCTR | 1.226218 | 956 | 707 | 1292 |
| UNT 1170 IN MCTR | 1.244781 | 892 | 660 | 1206 |
| UNT 1172 IN MCTR | 0.951005 | 648 | 479 | 876 |
| UNT 1173 IN MCTR | 1.379294 | 789 | 584 | 1068 |
| UNT 1173 IN MCTR | 2.379261 | 1305 | 965 | 1765 |
| UNT 1173 IN MCTR | 3.367751 | 1623 | 1200 | 2194 |
| UNT 1175 IN MCTR | 1.001133 | 510 | 377 | 690 |
| UNT 1175 IN MCTR | 1.701766 | 879 | 650 | 1188 |
| UNT 1176 IN MCTR | 1.051979 | 595 | 440 | 804 |
| UNT 1177 IN MCTR | 1.345665 | 871 | 644 | 1178 |
| UNT 1179 IN MCTR | 0.924641 | 710 | 525 | 960 |
| UNT 1180 IN MCTR | 0.963201 | 698 | 516 | 944 |
| UNT 1187 IN MCTR | 0.938899 | 1500 | 1109 | 2028 |
| UNT 1187 IN MCTR | 1.297241 | 1996 | 1476 | 2699 |
| UNT 1190 IN MCTR | 1.270966 | 1467 | 1085 | 1984 |
| UNT 1190 IN MCTR | 1.407183 | 1608 | 1189 | 2175 |
| UNT 1190 IN MCTR | 1.519366 | 1722 | 1274 | 2329 |
| UNT 1191 IN MCTR | 1.424061 | 989 | 732 | 1338 |
| UNT 1191 IN MCTR | 1.561693 | 1010 | 747 | 1366 |
| UNT 1191 IN MCTR | 1.670738 | 1196 | 884 | 1617 |
| UNT 1191 IN MCTR | 1.956174 | 1613 | 1192 | 2181 |
| UNT 1191 IN MCTR | 2.727649 | 2316 | 1712 | 3132 |
| UNT 1194 IN MCTR | 1.000147 | 1751 | 1295 | 2368 |
| UNT 1194 IN MCTR | 1.49901 | 2179 | 1611 | 2947 |
| UNT 1195 IN MCTR | 1.169723 | 496 | 367 | 670 |
| UNT 1197 IN MCTR | 0.224905 | 350 | 259 | 473 |
| UNT 1197 IN MCTR | 0.968517 | 1216 | 899 | 1645 |
| UNT 1197 IN MCTR | 1.426911 | 1709 | 1264 | 2312 |
| UNT 1200 IN MCTR | 1.030816 | 550 | 407 | 744 |
| UNT 1200 IN MCTR | 1.705263 | 832 | 615 | 1125 |
| UNT 1200 IN MCTR | 2.395493 | 1102 | 815 | 1491 |
| UNT 1200 IN MCTR | 4.42242 | 1748 | 1292 | 2364 |
| UNT 1201 IN MCTR | 1.172592 | 569 | 420 | 769 |
| UNT 1201 IN MCTR | 1.445474 | 711 | 526 | 962 |
| UNT 1203 IN MCTR | 1.000416 | 296 | 219 | 401 |
| UNT 1203 IN MCTR | 1.865064 | 526 | 389 | 712 |
| UNT 1203 IN MCTR | 4.559175 | 1083 | 801 | 1465 |
| UNT 1205 IN MCTR | 1.21474 | 165 | 122 | 223 |
| UNT 1205 IN MCTR | 1.586802 | 543 | 402 | 735 |
| UNT 1210 IN MCTR | 1.328986 | 766 | 566 | 1035 |
| UNT 1210 IN MCTR | 2.477635 | 1258 | 930 | 1701 |
| UNT 1210 IN MCTR | 4.425469 | 2019 | 1493 | 2731 |
| UNT 1211 IN MCTR | 1.325578 | 851 | 629 | 1151 |
| UNT 1211 IN MCTR | 1.819061 | 980 | 724 | 1325 |
| UNT 1213 IN MCTR | 1.320018 | 490 | 362 | 662 |
| UNT 1213 IN MCTR | 1.857531 | 663 | 490 | 897 |
| UNT 2605 IN MCTR | 0.83909 | 752 | 556 | 1016 |
| UNT 2658 IN MCTR | 0.885632 | 389 | 287 | 526 |
| UNT 2658 IN MCTR | 1.505825 | 780 | 576 | 1054 |
| UNT 2658 IN MCTR | 1.785971 | 946 | 700 | 1280 |
| UNT 2711 IN MCTR | 0.56594 | 807 | 597 | 1091 |
| UNT 2711 IN MCTR | 0.633017 | 922 | 682 | 1247 |
| UNT 2712 IN MCTR | 0.628713 | 838 | 620 | 1134 |
| UNT 2712 IN MCTR | 0.88904 | 1199 | 886 | 1621 |
| UNT 2714 IN MCTR | 0.476982 | 1013 | 749 | 1370 |
| UNT 2730 IN MCTR | 0.454538 | 591 | 437 | 799 |
| UNT 2731 IN MCTR | 0.167962 | 202 | 150 | 274 |
| UNT 2731 IN MCTR | 1.168198 | 473 | 350 | 639 |
| UNT 2731 IN MCTR | 1.218327 | 494 | 365 | 668 |
| UNT 2734 IN MCTR | 0.551771 | 440 | 325 | 595 |
| UNT 327 IN MCTR | 1.372747 | 706 | 522 | 955 |
| UNT 998 IN MCTR | 0.907637 | 144 | 106 | 194 |
| UNT 998 IN MCTR | 1.010845 | 350 | 259 | 474 |
| UNT 999 IN MCTR | 0.32782 | 193 | 143 | 262 |
| UNT 9996 IN MCTR | 0.066091 | 47 | 35 | 63 |
| UNT 9997 IN MCTR | 0.668977 | 1088 | 805 | 1472 |
| WATERHOLE CREEK | 1.494616 | 700 | 518 | 947 |
| WATERHOLE CREEK | 2.546506 | 1230 | 910 | 1664 |
| WATERHOLE CREEK | 3.093255 | 1522 | 1125 | 2058 |
| WATERHOLE CREEK | 4.007045 | 1832 | 1354 | 2477 |
| WATERHOLE CREEK | 4.435244 | 1951 | 1443 | 2639 |
| WATERHOLE CREEK | 4.747493 | 2042 | 1510 | 2762 |
| WATERHOLE CREEK | 6.049397 | 2491 | 1842 | 3369 |
| WATERHOLE CREEK | 7.812052 | 2982 | 2205 | 4033 |
| WEST SPRING BRANCH | 1.326385 | 855 | 632 | 1156 |
| WILLIAMS CREEK | 1.143986 | 841 | 622 | 1137 |
| WILLIAMS CREEK | 2.135704 | 1287 | 951 | 1740 |
| WILLIAMS CREEK | 3.094152 | 1689 | 1249 | 2284 |
| WILLIAMS CREEK | 4.01951 | 2104 | 1556 | 2846 |
| WILLIAMS CREEK | 5.911835 | 2768 | 2047 | 3743 |
| WILLIAMS CREEK | 6.234493 | 2995 | 2215 | 4051 |
| WILLIAMSON BRANCH | 0.232438 | 280 | 207 | 378 |
| WILLIAMSON BRANCH | 0.482452 | 554 | 410 | 749 |
| YATES CREEK | 0.420307 | 809 | 598 | 1094 |
| YATES CREEK | 0.630237 | 1146 | 847 | 1549 |
| YATES CREEK | 1.592631 | 2493 | 1844 | 3372 |
| YATES CREEK | 2.578699 | 3794 | 2806 | 5131 |
| YATES CREEK | 4.648133 | 5463 | 4039 | 7388 |
| YATES CREEK | 6.242019 | 6408 | 4738 | 8666 |
| YATES CREEK | 6.444057 | 6408 | 4738 | 8666 |
| YATES CREEK | 7.362241 | 6828 | 5049 | 9233 |
| YELLOW BRANCH | 1.020234 | 973 | 720 | 1316 |
| YELLOW BRANCH | 2.041186 | 1537 | 1137 | 2079 |
| YELLOW BRANCH | 5.023872 | 2633 | 1947 | 3560 |
| YELLOW BRANCH | 5.696256 | 2728 | 2017 | 3689 |
| Nelson Creek - Lake Livingston |  |  |  |  |
| BALDWIN CREEK | 1.332842 | 1881 | 1391 | 2543 |
| BALDWIN CREEK | 1.569315 | 2133 | 1577 | 2885 |
| BALDWIN CREEK | 2.598696 | 3130 | 2314 | 4233 |
| BALDWIN CREEK | 3.371965 | 3801 | 2811 | 5140 |
| BALDWIN CREEK | 3.719188 | 3909 | 2891 | 5287 |
| BALDWIN CREEK | 4.005879 | 4150 | 3069 | 5613 |
| BALDWIN CREEK | 5.318275 | 5193 | 3840 | 7023 |
| BALDWIN CREEK | 5.888609 | 5419 | 4007 | 7329 |
| BALDWIN CREEK | 6.56467 | 5980 | 4422 | 8088 |
| BALDWIN CREEK | 7.34332 | 6357 | 4700 | 8597 |
| BALDWIN CREEK | 9.186862 | 7431 | 5495 | 10049 |
| BETHY CREEK | 1.415611 | 2270 | 1679 | 3070 |
| BETHY CREEK | 1.67818 | 2537 | 1876 | 3430 |
| BETHY CREEK | 2.031321 | 2828 | 2091 | 3824 |
| BETHY CREEK | 2.449207 | 3158 | 2335 | 4271 |
| BETHY CREEK | 3.197367 | 3917 | 2896 | 5297 |
| BETHY CREEK | 3.744235 | 4184 | 3094 | 5658 |
| BETHY CREEK | 3.98304 | 4211 | 3114 | 5695 |
| BETHY CREEK | 6.749698 | 6266 | 4633 | 8474 |
| BIG SULPHUR BRANCH | 0.99369 | 1759 | 1300 | 2378 |
| BIG SULPHUR BRANCH | 2.226903 | 2821 | 2086 | 3815 |
| BURGE BRANCH | 0.725562 | 1074 | 795 | 1453 |
| BURT CREEK | 0.65696 | 1054 | 779 | 1425 |
| BURT CREEK | 1.622582 | 2283 | 1688 | 3088 |
| BURT CREEK | 2.114899 | 2825 | 2089 | 3821 |
| CANEY CREEK | 0.930738 | 1606 | 1188 | 2172 |
| CANEY CREEK | 2.00424 | 2616 | 1934 | 3538 |
| CANEY CREEK | 5.836328 | 6245 | 4618 | 8446 |
| CANEY CREEK | 6.065897 | 6245 | 4618 | 8446 |
| CANEY CREEK | 6.954398 | 6439 | 4761 | 8708 |
| CANEY CREEK | 7.616291 | 6742 | 4986 | 9118 |
| CANEY CREEK | 8.96913 | 7493 | 5541 | 10134 |
| CANEY CREEK | 10.341027 | 8232 | 6087 | 11132 |
| CANEY CREEK | 13.159323 | 9836 | 7273 | 13302 |
| CANEY CREEK | 14.6712 | 10789 | 7978 | 14591 |
| CANEY CREEK | 14.905178 | 10859 | 8029 | 14685 |
| CANEY CREEK | 16.047101 | 11461 | 8474 | 15499 |
| CANEY CREEK | 17.142393 | 11461 | 8474 | 15499 |
| CANEY CREEK | 20.036749 | 12688 | 9382 | 17160 |
| CANEY CREEK | 20.256184 | 12688 | 9382 | 17160 |
| CANEY CREEK | 20.845798 | 12688 | 9382 | 17160 |
| CANEY CREEK | 21.17652 | 12777 | 9447 | 17279 |
| CANEY CREEK | 21.775461 | 12985 | 9601 | 17560 |
| CANEY CREEK | 21.875449 | 12996 | 9610 | 17576 |
| CANEY CREEK | 22.291183 | 13031 | 9636 | 17623 |
| CANEY CREEK | 22.569087 | 13166 | 9736 | 17806 |
| CANEY CREEK | 23.470412 | 13166 | 9736 | 17806 |
| CANEY CREEK | 23.690474 | 13166 | 9736 | 17806 |
| CANEY CREEK | 24.286994 | 13306 | 9839 | 17994 |
| CANEY CREEK | 28.010396 | 14528 | 10742 | 19647 |
| CANEY CREEK | 28.465408 | 14684 | 10858 | 19858 |
| CANEY CREEK | 28.68287 | 14684 | 10858 | 19858 |
| CEDAR CREEK | 1.106412 | 1666 | 1232 | 2253 |
| CEDAR CREEK | 1.617471 | 2107 | 1558 | 2850 |
| CEDAR CREEK | 2.226276 | 2585 | 1912 | 3497 |
| CHALK CREEK | 0.943383 | 944 | 698 | 1276 |
| CHALK CREEK | 1.244871 | 1234 | 913 | 1669 |
| CHALK CREEK | 2.707652 | 2509 | 1855 | 3393 |
| CHALK CREEK | 4.823806 | 3992 | 2952 | 5399 |
| CHALK CREEK | 7.086311 | 5505 | 4070 | 7445 |
| CHALK CREEK | 9.234031 | 6854 | 5068 | 9269 |
| CHALK CREEK | 9.267121 | 6854 | 5068 | 9269 |
| CHALK CREEK | 10.235702 | 7037 | 5203 | 9516 |
| CHALK CREEK | 12.420368 | 7733 | 5718 | 10458 |
| CHALK CREEK | 16.333972 | 9583 | 7086 | 12959 |
| CRABB CREEK | 1.01315 | 1706 | 1262 | 2307 |
| CRABB CREEK | 1.299662 | 1990 | 1472 | 2691 |
| CRABB CREEK | 2.705858 | 3351 | 2478 | 4532 |
| CRABB CREEK | 3.383533 | 3886 | 2873 | 5255 |
| CRABB CREEK | 4.994458 | 4900 | 3624 | 6627 |
| CRABB CREEK | 5.6759 | 5366 | 3968 | 7257 |
| CRABB CREEK | 6.055584 | 5522 | 4083 | 7468 |
| CRABB CREEK | 6.903463 | 6055 | 4477 | 8189 |
| DEANS CREEK | 1.242001 | 2280 | 1686 | 3083 |
| DEANS CREEK | 2.4597 | 3805 | 2814 | 5146 |
| DILLARD CREEK | 1.136453 | 1010 | 747 | 1366 |
| DILLARD CREEK | 1.944875 | 1491 | 1102 | 2016 |
| DILLARD CREEK | 2.944305 | 1973 | 1459 | 2668 |
| DILLARD CREEK | 4.048475 | 2534 | 1874 | 3427 |
| DILLARD CREEK | 6.804462 | 3857 | 2852 | 5215 |
| DILLARD CREEK | 8.014 | 4282 | 3167 | 5791 |
| DILLARD CREEK | 8.539855 | 4477 | 3311 | 6055 |
| DILLARD CREEK | 10.129168 | 5133 | 3795 | 6942 |
| DILLARD CREEK | 11.10878 | 5845 | 4322 | 7904 |
| DILLARD CREEK | 11.795871 | 6226 | 4603 | 8419 |
| DILLARD CREEK | 12.340108 | 6395 | 4728 | 8648 |
| DILLARD CREEK | 12.349255 | 6395 | 4728 | 8648 |
| DILLARD CREEK | 12.967835 | 6637 | 4908 | 8976 |
| DILLARD CREEK | 14.180064 | 6951 | 5140 | 9400 |
| DILLARD CREEK | 15.005255 | 7070 | 5228 | 9561 |
| EAST CAROLINA CREEK | 1.154209 | 2037 | 1506 | 2754 |
| EAST CAROLINA CREEK | 2.851401 | 4218 | 3119 | 5705 |
| EAST CAROLINA CREEK | 5.978105 | 7277 | 5381 | 9841 |
| EAST CAROLINA CREEK | 6.630581 | 7589 | 5612 | 10263 |
| EAST CAROLINA CREEK | 7.271131 | 8272 | 6117 | 11188 |
| EAST CAROLINA CREEK | 8.212362 | 8345 | 6171 | 11286 |
| EAST CAROLINA CREEK | 10.035727 | 9282 | 6864 | 12553 |
| EAST CAROLINA CREEK | 11.13066 | 10042 | 7425 | 13580 |
| EAST CAROLINA CREEK | 11.814344 | 10360 | 7660 | 14010 |
| EAST CAROLINA CREEK | 12.153675 | 10578 | 7822 | 14305 |
| EAST CAROLINA CREEK | 12.567167 | 10600 | 7838 | 14335 |
| EAST CAROLINA CREEK | 13.750072 | 11180 | 8267 | 15119 |
| EAST CAROLINA CREEK | 14.106641 | 11272 | 8335 | 15244 |
| EAST CAROLINA CREEK | 14.823571 | 11598 | 8576 | 15686 |
| EAST CAROLINA CREEK | 15.021127 | 11598 | 8576 | 15686 |
| EAST FORK HARMON CREEK | 0.934864 | 1650 | 1220 | 2232 |
| EAST FORK HARMON CREEK | 1.404582 | 2224 | 1645 | 3008 |
| EAST FORK HARMON CREEK | 1.740595 | 2420 | 1789 | 3273 |
| EAST FORK TANYARD BRANCH | 1.113855 | 1806 | 1335 | 2442 |
| EVE BRANCH | 0.731391 | 1119 | 827 | 1513 |
| EVE BRANCH | 0.819452 | 1201 | 888 | 1624 |
| FORD BRANCH | 1.031982 | 1587 | 1173 | 2146 |
| HADLEY CREEK | 0.653194 | 1166 | 862 | 1577 |
| HADLEY CREEK | 1.263344 | 2098 | 1551 | 2837 |
| HADLEY CREEK | 1.813859 | 2815 | 2081 | 3807 |
| HADLEY CREEK | 2.294249 | 3154 | 2332 | 4266 |
| HADLEY CREEK | 2.627213 | 3568 | 2638 | 4825 |
| HADLEY CREEK | 3.463793 | 3932 | 2907 | 5317 |
| HADLEY CREEK | 4.101832 | 4326 | 3199 | 5851 |
| HADLEY CREEK | 4.695033 | 4647 | 3436 | 6284 |
| HADLEY CREEK | 5.001453 | 4826 | 3569 | 6527 |
| HADLEY CREEK | 8.368486 | 7230 | 5346 | 9777 |
| HADLEY CREEK | 12.922011 | 10452 | 7728 | 14135 |
| HADLEY CREEK | 13.625065 | 10452 | 7728 | 14135 |
| HADLEY CREEK | 14.549346 | 10666 | 7887 | 14425 |
| HADLEY CREEK | 15.718441 | 10666 | 7887 | 14425 |
| HARMON CREEK | 1.299662 | 2475 | 1830 | 3348 |
| HARMON CREEK | 3.063303 | 4065 | 3006 | 5498 |
| HARMON CREEK | 3.999871 | 4513 | 3337 | 6103 |
| HARMON CREEK | 7.323232 | 7330 | 5420 | 9913 |
| HARMON CREEK | 8.016152 | 7883 | 5829 | 10660 |
| HARMON CREEK | 10.834463 | 9413 | 6960 | 12730 |
| HARMON CREEK | 11.382648 | 9689 | 7165 | 13104 |
| HARMON CREEK | 20.192694 | 16946 | 12531 | 22918 |
| HARMON CREEK | 20.80652 | 16946 | 12531 | 22918 |
| HARMON CREEK | 21.412097 | 17037 | 12598 | 23040 |
| HARMON CREEK | 22.853984 | 17309 | 12799 | 23408 |
| HARMON CREEK | 23.346031 | 17309 | 12799 | 23408 |
| HARMON CREEK | 23.763111 | 17309 | 12799 | 23408 |
| HARMON CREEK | 40.990875 | 26147 | 19334 | 35361 |
| HARMON CREEK | 41.510542 | 26147 | 19334 | 35361 |
| HARMON CREEK | 43.750448 | 26952 | 19929 | 36450 |
| HARMON CREEK | 43.905138 | 26952 | 19929 | 36450 |
| HARMON CREEK | 44.339973 | 26952 | 19929 | 36450 |
| HARMON CREEK | 44.567299 | 26952 | 19929 | 36450 |
| HARMON CREEK | 45.240581 | 26952 | 19929 | 36450 |
| HARMON CREEK | 47.528373 | 27141 | 20069 | 36705 |
| HARMON CREEK | 47.968768 | 27141 | 20069 | 36705 |
| HARMON CREEK | 49.542567 | 27427 | 20281 | 37092 |
| HARMON CREEK | 50.26786 | 27448 | 20296 | 37120 |
| HARMON CREEK | 50.501822 | 27448 | 20296 | 37120 |
| HARMON CREEK | 51.180125 | 27448 | 20296 | 37120 |
| HARMON CREEK | 52.217667 | 27524 | 20352 | 37223 |
| HARMON CREEK | 53.214807 | 27524 | 20352 | 37223 |
| HARMON CREEK | 53.843842 | 27524 | 20352 | 37223 |
| HARMON CREEK | 54.20063 | 27545 | 20368 | 37252 |
| HARMON CREEK | 54.548433 | 27545 | 20368 | 37252 |
| HARMON CREEK | 54.765763 | 27545 | 20368 | 37252 |
| HARMON CREEK | 56.13842 | 27545 | 20368 | 37252 |
| HARMON CREEK | 84.855277 | 36244 | 26800 | 49016 |
| HARMON CREEK | 85.821438 | 36244 | 26800 | 49016 |
| HARMON CREEK | 86.673441 | 36244 | 26800 | 49016 |
| HARMON CREEK | 100.055779 | 37792 | 27944 | 51109 |
| HORSE BRANCH | 0.445417 | 1157 | 855 | 1565 |
| HORSE BRANCH | 1.109282 | 2463 | 1821 | 3330 |
| HORSE BRANCH | 1.304863 | 2637 | 1950 | 3566 |
| HORSE BRANCH | 1.39956 | 2746 | 2030 | 3713 |
| JENKINS BRANCH | 0.9276 | 1022 | 756 | 1382 |
| JENKINS BRANCH | 1.273564 | 1257 | 929 | 1700 |
| JENKINS BRANCH | 1.436683 | 1421 | 1051 | 1922 |
| JENKINS BRANCH | 1.58671 | 1579 | 1167 | 2135 |
| JENKINS BRANCH | 2.105301 | 1958 | 1448 | 2648 |
| LANDMAN CREEK | 1.340285 | 1298 | 960 | 1756 |
| LANDMAN CREEK | 2.29129 | 1809 | 1338 | 2447 |
| LANDMAN CREEK | 2.70828 | 1945 | 1438 | 2630 |
| LITTLE SCHOOL BRANCH | 0.281759 | 494 | 365 | 668 |
| LITTLE SCHOOL BRANCH | 0.571589 | 950 | 702 | 1285 |
| LITTLE SCHOOL BRANCH | 0.953785 | 1471 | 1088 | 1990 |
| LITTLE SCHOOL BRANCH | 1.767766 | 2191 | 1620 | 2964 |
| NELSON CREEK | 0.640012 | 942 | 697 | 1274 |
| NELSON CREEK | 0.944279 | 1236 | 914 | 1672 |
| NELSON CREEK | 2.08432 | 2408 | 1780 | 3256 |
| NELSON CREEK | 3.02277 | 3238 | 2394 | 4379 |
| NELSON CREEK | 3.792183 | 3583 | 2649 | 4845 |
| NELSON CREEK | 8.444979 | 7972 | 5894 | 10781 |
| NELSON CREEK | 9.623311 | 8535 | 6311 | 11543 |
| NELSON CREEK | 11.106089 | 9325 | 6895 | 12611 |
| NELSON CREEK | 12.905879 | 10447 | 7725 | 14129 |
| NELSON CREEK | 17.075034 | 12316 | 9107 | 16656 |
| NELSON CREEK | 18.361088 | 12729 | 9412 | 17214 |
| NELSON CREEK | 20.726889 | 13704 | 10133 | 18533 |
| NELSON CREEK | 22.768972 | 14832 | 10967 | 20058 |
| NELSON CREEK | 23.701235 | 14904 | 11020 | 20156 |
| NELSON CREEK | 24.279281 | 15268 | 11290 | 20648 |
| NELSON CREEK | 25.688795 | 15741 | 11640 | 21288 |
| NELSON CREEK | 25.965622 | 15741 | 11640 | 21288 |
| NELSON CREEK | 29.326288 | 16981 | 12556 | 22965 |
| NELSON CREEK | 31.061682 | 17519 | 12954 | 23692 |
| NELSON CREEK | 38.099299 | 20316 | 15022 | 27475 |
| NELSON CREEK | 39.920781 | 20675 | 15288 | 27960 |
| NELSON CREEK | 40.305756 | 20735 | 15332 | 28041 |
| NELSON CREEK | 40.583929 | 20735 | 15332 | 28041 |
| NELSON CREEK | 41.517178 | 20740 | 15336 | 28049 |
| NELSON CREEK | 44.026558 | 21426 | 15843 | 28976 |
| NELSON CREEK | 59.777462 | 26236 | 19400 | 35482 |
| NELSON CREEK | 61.012827 | 26446 | 19555 | 35766 |
| NELSON CREEK | 61.839184 | 27359 | 20230 | 37000 |
| NELSON CREEK | 62.569767 | 27369 | 20238 | 37013 |
| NELSON CREEK | 64.595619 | 27941 | 20661 | 37787 |
| NELSON CREEK | 66.462207 | 28057 | 20746 | 37944 |
| NELSON CREEK | 75.652028 | 30849 | 22811 | 41719 |
| NELSON CREEK | 76.839238 | 30849 | 22811 | 41719 |
| NELSON CREEK | 77.71922 | 30849 | 22811 | 41719 |
| NELSON CREEK | 79.868733 | 30849 | 22811 | 41719 |
| NELSON CREEK | 80.985817 | 30984 | 22910 | 41902 |
| NELSON CREEK | 81.566464 | 30984 | 22910 | 41902 |
| NELSON CREEK | 84.82499 | 31292 | 23138 | 42319 |
| NELSON CREEK | 85.42115 | 31292 | 23138 | 42319 |
| NELSON CREEK | 86.813537 | 31292 | 23138 | 42319 |
| NELSON CREEK | 88.83625 | 31292 | 23138 | 42319 |
| NELSON CREEK | 89.831913 | 31292 | 23138 | 42319 |
| NELSON CREEK | 91.217932 | 31292 | 23138 | 42319 |
| NEWTON BRANCH | 1.13921 | 1660 | 1227 | 2244 |
| NEWTON BRANCH | 1.96012 | 2222 | 1643 | 3005 |
| NORTH GUM BRANCH | 1.102646 | 1966 | 1454 | 2659 |
| NORTH GUM BRANCH | 1.381715 | 2361 | 1746 | 3194 |
| NORTH GUM BRANCH | 1.677643 | 2761 | 2041 | 3733 |
| NORTH GUM BRANCH | 3.226871 | 4124 | 3050 | 5577 |
| NORTH ROCKY CREEK | 0.905898 | 1367 | 1011 | 1848 |
| NORTH ROCKY CREEK | 2.050871 | 2378 | 1758 | 3216 |
| NORTH ROCKY CREEK | 2.662276 | 2848 | 2106 | 3852 |
| NORTH ROCKY CREEK | 3.658747 | 3719 | 2750 | 5029 |
| NORTH TURKEY CREEK | 0.686105 | 1018 | 753 | 1377 |
| NORTH TURKEY CREEK | 0.815416 | 1147 | 848 | 1551 |
| NORTH TURKEY CREEK | 1.068659 | 1463 | 1082 | 1979 |
| NORTH TURKEY CREEK | 1.415881 | 1823 | 1348 | 2466 |
| NORTH TURKEY CREEK | 1.658094 | 2019 | 1493 | 2730 |
| NORTH TURKEY CREEK | 2.060825 | 2424 | 1792 | 3278 |
| PARKER CREEK | 0.412775 | 952 | 704 | 1287 |
| PARKER CREEK | 1.266751 | 2320 | 1715 | 3137 |
| PARKER CREEK | 1.598101 | 2591 | 1916 | 3504 |
| PARKER CREEK | 9.086694 | 9887 | 7311 | 13371 |
| PARKER CREEK | 10.247808 | 9887 | 7311 | 13371 |
| PARKER CREEK | 11.708527 | 10732 | 7936 | 14514 |
| PARKER CREEK | 12.706701 | 11160 | 8252 | 15093 |
| PARKER CREEK | 14.453125 | 12081 | 8933 | 16338 |
| PARKER CREEK | 15.204513 | 12261 | 9066 | 16582 |
| PARKER CREEK | 16.371725 | 12867 | 9514 | 17401 |
| PARKER CREEK | 16.62434 | 13051 | 9651 | 17650 |
| PINE CREEK | 0.943293 | 1667 | 1232 | 2254 |
| PINE CREEK | 1.753149 | 2612 | 1931 | 3532 |
| PINE CREEK | 2.107815 | 2918 | 2158 | 3946 |
| PINE CREEK | 2.493777 | 3247 | 2401 | 4392 |
| PINE CREEK | 2.776702 | 3496 | 2585 | 4728 |
| PINE CREEK | 3.07245 | 3773 | 2790 | 5102 |
| PINE CREEK | 3.43689 | 4146 | 3066 | 5607 |
| PINE CREEK | 3.620366 | 4249 | 3142 | 5746 |
| PINE CREEK | 4.301986 | 4719 | 3489 | 6382 |
| PINE CREEK | 4.620064 | 4961 | 3668 | 6709 |
| POLECAT BRANCH | 1.081572 | 1102 | 815 | 1491 |
| POSSUM BRANCH | 0.895406 | 1483 | 1096 | 2005 |
| POST OAK BRANCH | 0.911548 | 1222 | 904 | 1653 |
| POST OAK BRANCH | 1.671993 | 1957 | 1447 | 2647 |
| RED OAK CREEK | 0.911279 | 1830 | 1353 | 2474 |
| RED OAK CREEK | 1.478743 | 2567 | 1898 | 3472 |
| RED OAK CREEK | 1.786867 | 3011 | 2227 | 4072 |
| RED OAK CREEK | 2.553231 | 3880 | 2869 | 5247 |
| RED OAK CREEK | 2.838398 | 4056 | 2999 | 5486 |
| ROBINS CREEK | 1.013688 | 1583 | 1170 | 2140 |
| ROBINS CREEK | 1.495692 | 2145 | 1586 | 2901 |
| ROBINS CREEK | 3.615433 | 4590 | 3394 | 6208 |
| ROCKY BRANCH | 0.48048 | 748 | 553 | 1012 |
| ROCKY BRANCH | 0.832813 | 1137 | 841 | 1537 |
| ROCKY BRANCH | 1.114842 | 1405 | 1039 | 1901 |
| ROCKY BRANCH | 1.851254 | 2249 | 1663 | 3041 |
| ROCKY BRANCH | 2.002626 | 2344 | 1733 | 3170 |
| ROCKY BRANCH | 2.411544 | 2680 | 1982 | 3625 |
| ROCKY BRANCH | 2.766636 | 2896 | 2142 | 3917 |
| ROCKY BRANCH | 3.750014 | 3487 | 2578 | 4715 |
| SHOEMAKE BRANCH | 0.401565 | 763 | 564 | 1032 |
| SHOEMAKE BRANCH | 0.556883 | 994 | 735 | 1344 |
| SHOEMAKE BRANCH | 1.054117 | 1658 | 1226 | 2242 |
| SHOEMAKE BRANCH | 1.893506 | 2154 | 1593 | 2913 |
| SIXMILE BRANCH | 1.10139 | 1933 | 1429 | 2614 |
| SIXMILE BRANCH | 2.606857 | 3386 | 2504 | 4580 |
| SIXMILE BRANCH | 3.036222 | 3700 | 2736 | 5004 |
| SIXMILE BRANCH | 3.321478 | 4053 | 2997 | 5482 |
| SIXMILE BRANCH | 3.841415 | 4415 | 3265 | 5971 |
| SIXMILE BRANCH | 4.198591 | 4609 | 3408 | 6233 |
| SPRING BRANCH | 0.814699 | 1604 | 1186 | 2170 |
| SPRING CREEK | 0.942934 | 1685 | 1246 | 2279 |
| SPRING CREEK | 1.658452 | 2500 | 1848 | 3381 |
| SPRING CREEK | 2.011773 | 2982 | 2205 | 4032 |
| SUTTON CREEK | 0.971272 | 1440 | 1065 | 1948 |
| SUTTON CREEK | 3.391156 | 4265 | 3154 | 5768 |
| SUTTON CREEK | 3.966153 | 4604 | 3405 | 6227 |
| SUTTON CREEK | 4.479274 | 5038 | 3725 | 6813 |
| SUTTON CREEK | 8.906716 | 8161 | 6035 | 11037 |
| SUTTON CREEK | 9.388093 | 8256 | 6104 | 11165 |
| TANYARD BRANCH | 0.425867 | 967 | 715 | 1308 |
| TANYARD BRANCH | 1.319211 | 2467 | 1824 | 3336 |
| TANYARD BRANCH | 1.663833 | 2744 | 2029 | 3710 |
| TANYARD BRANCH | 2.926818 | 3971 | 2937 | 5371 |
| TANYARD BRANCH | 3.750933 | 4941 | 3653 | 6682 |
| TANYARD BRANCH | 5.262766 | 6334 | 4683 | 8565 |
| TANYARD BRANCH | 6.291878 | 6995 | 5173 | 9461 |
| TANYARD BRANCH | 6.867234 | 7180 | 5309 | 9710 |
| TANYARD BRANCH | 8.72898 | 8761 | 6478 | 11848 |
| THOMPSON BRANCH NORTH | 0.916211 | 1048 | 775 | 1417 |
| THOMPSON BRANCH NORTH | 1.590658 | 1513 | 1119 | 2046 |
| THOMPSON BRANCH NORTH | 2.306624 | 2148 | 1588 | 2905 |
| THOMPSON BRANCH NORTH | 3.347843 | 2890 | 2137 | 3908 |
| THOMPSON BRANCH SOUTH | 1.238952 | 1862 | 1377 | 2518 |
| TOWN BRANCH | 0.700273 | 1517 | 1121 | 2051 |
| TOWN BRANCH | 1.052697 | 2132 | 1576 | 2883 |
| TOWN BRANCH | 1.087849 | 2213 | 1636 | 2993 |
| TOWN BRANCH | 2.371998 | 4170 | 3084 | 5640 |
| TOWN BRANCH | 2.591791 | 4548 | 3363 | 6150 |
| TOWN BRANCH | 2.747378 | 4713 | 3485 | 6374 |
| TOWN BRANCH | 2.897046 | 4825 | 3567 | 6525 |
| TOWN BRANCH | 3.369365 | 5387 | 3983 | 7285 |
| TOWN BRANCH | 3.649779 | 5514 | 4077 | 7457 |
| TOWN BRANCH | 3.956737 | 5766 | 4264 | 7798 |
| TOWN BRANCH | 3.973327 | 5766 | 4264 | 7798 |
| TOWN BRANCH | 4.313196 | 5900 | 4363 | 7979 |
| TOWN BRANCH | 5.748357 | 7454 | 5512 | 10080 |
| TOWN BRANCH | 6.397964 | 7784 | 5756 | 10527 |
| TRIBUTARY 2 | 1.006066 | 1874 | 1386 | 2535 |
| TRIBUTARY 2 | 1.818523 | 2820 | 2085 | 3813 |
| TRIBUTARY 7 | 1.034941 | 1694 | 1253 | 2291 |
| TRIBUTARY 7 | 2.31613 | 2762 | 2042 | 3735 |
| TRIBUTARY 7 | 3.106886 | 3472 | 2567 | 4696 |
| TRIBUTARY 7 | 3.331253 | 3704 | 2739 | 5010 |
| TRIBUTARY 8 | 0.826715 | 1250 | 925 | 1691 |
| TRIBUTARY 8 | 1.182905 | 1594 | 1179 | 2156 |
| TRIBUTARY 8 | 1.744182 | 2106 | 1557 | 2848 |
| TRIBUTARY A | 0.484784 | 977 | 723 | 1322 |
| TRIBUTARY A | 1.196805 | 2160 | 1597 | 2921 |
| TRIBUTARY B | 0.209212 | 546 | 403 | 738 |
| TRIBUTARY B | 0.542445 | 1431 | 1058 | 1935 |
| TURKEY CREEK | 1.189113 | 1338 | 989 | 1810 |
| TURKEY CREEK | 1.323516 | 1423 | 1053 | 1925 |
| TURKEY CREEK | 1.999487 | 1939 | 1434 | 2622 |
| TURKEY CREEK | 3.238887 | 2828 | 2091 | 3825 |
| TURKEY CREEK | 4.195273 | 3606 | 2667 | 4877 |
| TURKEY CREEK | 4.793317 | 3785 | 2799 | 5118 |
| TURKEY CREEK | 6.73687 | 4549 | 3364 | 6153 |
| TURKEY CREEK | 7.508435 | 4669 | 3453 | 6315 |
| TURKEY CREEK | 8.806931 | 5206 | 3850 | 7041 |
| TURNER CREEK | 0.954323 | 1846 | 1365 | 2496 |
| TURNER CREEK | 2.257303 | 3406 | 2519 | 4606 |
| TURNER CREEK | 3.159973 | 3664 | 2709 | 4955 |
| TURNER CREEK | 3.863385 | 4214 | 3116 | 5699 |
| UNNAMED TRIB 1 OF TOWN BRANCH | 0.127787 | 351 | 259 | 474 |
| UNNAMED TRIB 1 OF TOWN BRANCH | 0.206074 | 514 | 380 | 695 |
| UNNAMED TRIB TO UNNAMED TRIB 1 | 0.04734 | 130 | 96 | 176 |
| UNT 001 IN NCLL | 1.304594 | 2047 | 1514 | 2769 |
| UNT 001 IN NCLL | 2.03177 | 2571 | 1901 | 3476 |
| UNT 001 IN NCLL | 2.85445 | 3069 | 2269 | 4150 |
| UNT 001 IN NCLL | 3.568444 | 3446 | 2548 | 4660 |
| UNT 001 IN NCLL | 4.85322 | 4340 | 3209 | 5869 |
| UNT 001 IN NCLL | 6.1423 | 5133 | 3795 | 6941 |
| UNT 002 IN NCLL | 0.466939 | 980 | 724 | 1325 |
| UNT 002 IN NCLL | 0.873077 | 1562 | 1155 | 2112 |
| UNT 002 IN NCLL | 1.552367 | 2276 | 1683 | 3078 |
| UNT 003 IN NCLL | 0.175763 | 484 | 358 | 655 |
| UNT 004 IN NCLL | 0.260955 | 457 | 338 | 618 |
| UNT 007 IN NCLL | 0.639205 | 1328 | 982 | 1796 |
| UNT 007 IN NCLL | 0.749774 | 1487 | 1099 | 2011 |
| UNT 007 IN NCLL | 0.815327 | 1543 | 1141 | 2087 |
| UNT 007 IN NCLL | 0.890833 | 1638 | 1211 | 2215 |
| UNT 007 IN NCLL | 1.119056 | 1767 | 1306 | 2389 |
| UNT 009 IN NCLL | 0.612721 | 757 | 560 | 1024 |
| UNT 009 IN NCLL | 1.082559 | 1317 | 974 | 1781 |
| UNT 010 IN NCLL | 1.099776 | 1543 | 1141 | 2087 |
| UNT 010 IN NCLL | 1.680692 | 1881 | 1391 | 2544 |
| UNT 010 IN NCLL | 1.798077 | 2019 | 1493 | 2731 |
| UNT 011 IN NCLL | 0.110839 | 296 | 219 | 401 |
| UNT 012 IN NCLL | 0.90258 | 1765 | 1305 | 2387 |
| UNT 012 IN NCLL | 1.997245 | 2797 | 2069 | 3783 |
| UNT 014 IN NCLL | 0.12133 | 221 | 163 | 298 |
| UNT 015 IN NCLL | 0.233873 | 411 | 304 | 556 |
| UNT 016 IN NCLL | 0.172983 | 312 | 231 | 422 |
| UNT 017 IN NCLL | 0.146978 | 279 | 206 | 377 |
| UNT 018 IN NCLL | 0.223201 | 594 | 439 | 803 |
| UNT 019 IN NCLL | 0.275668 | 670 | 495 | 906 |
| UNT 020 IN NCLL | 1.037811 | 2134 | 1578 | 2886 |
| UNT 020 IN NCLL | 1.12919 | 2239 | 1655 | 3027 |
| UNT 020 IN NCLL | 1.256528 | 2239 | 1655 | 3027 |
| UNT 022 IN NCLL | 0.60181 | 1017 | 752 | 1376 |
| UNT 022 IN NCLL | 1.28926 | 1975 | 1460 | 2671 |
| UNT 022 IN NCLL | 1.318225 | 1976 | 1461 | 2672 |
| UNT 022 IN NCLL | 1.452469 | 2136 | 1579 | 2888 |
| UNT 022 IN NCLL | 1.987202 | 2414 | 1785 | 3265 |
| UNT 023 IN NCLL | 0.113798 | 225 | 167 | 305 |
| UNT 024 IN NCLL | 0.116309 | 218 | 161 | 295 |
| UNT 025 IN NCLL | 0.636694 | 1011 | 747 | 1367 |
| UNT 027 IN NCLL | 0.900339 | 1383 | 1023 | 1870 |
| UNT 027 IN NCLL | 1.406824 | 1942 | 1436 | 2626 |
| UNT 027 IN NCLL | 2.808716 | 2852 | 2109 | 3857 |
| UNT 028 IN NCLL | 0.478865 | 709 | 524 | 958 |
| UNT 029 IN NCLL | 0.770937 | 999 | 739 | 1352 |
| UNT 031 IN NCLL | 0.103395 | 263 | 195 | 356 |
| UNT 032 IN NCLL | 0.369282 | 653 | 483 | 884 |
| UNT 035 IN NCLL | 0.33117 | 906 | 670 | 1226 |
| UNT 037 IN NCLL | 0.366054 | 817 | 604 | 1104 |
| UNT 039 IN NCLL | 0.139176 | 379 | 280 | 513 |
| UNT 042 IN NCLL | 0.420577 | 1242 | 918 | 1679 |
| UNT 047 IN NCLL | 0.352782 | 913 | 675 | 1235 |
| UNT 047 IN NCLL | 0.580109 | 1457 | 1078 | 1971 |
| UNT 047 IN NCLL | 1.178152 | 2709 | 2003 | 3663 |
| UNT 048 IN NCLL | 0.225892 | 707 | 523 | 956 |
| UNT 049 IN NCLL | 0.567016 | 1445 | 1068 | 1954 |
| UNT 050 IN NCLL | 0.312428 | 669 | 495 | 905 |
| UNT 051 IN NCLL | 0.622704 | 1366 | 1010 | 1848 |
| UNT 055 IN NCLL | 0.218269 | 531 | 392 | 718 |
| UNT 056 IN NCLL | 0.14079 | 403 | 298 | 546 |
| UNT 057 IN NCLL | 0.278441 | 691 | 511 | 935 |
| UNT 057 IN NCLL | 0.43887 | 982 | 726 | 1328 |
| UNT 058 IN NCLL | 0.131553 | 353 | 261 | 478 |
| UNT 059 IN NCLL | 0.047887 | 159 | 118 | 216 |
| UNT 061 IN NCLL | 0.220516 | 553 | 409 | 748 |
| UNT 061 IN NCLL | 0.245082 | 637 | 471 | 861 |
| UNT 063 IN NCLL | 0.114874 | 363 | 268 | 491 |
| UNT 065 IN NCLL | 0.10205 | 339 | 251 | 459 |
| UNT 066 IN NCLL | 0.443533 | 1015 | 750 | 1372 |
| UNT 068 IN NCLL | 1.240566 | 2361 | 1745 | 3192 |
| UNT 069 IN NCLL | 0.368117 | 938 | 694 | 1269 |
| UNT 071 IN NCLL | 0.277096 | 551 | 408 | 746 |
| UNT 072 IN NCLL | 0.887605 | 1358 | 1004 | 1837 |
| UNT 072 IN NCLL | 1.158693 | 1730 | 1279 | 2339 |
| UNT 073 IN NCLL | 0.197669 | 455 | 336 | 615 |
| UNT 074 IN NCLL | 0.324803 | 668 | 494 | 903 |
| UNT 075 IN NCLL | 0.596699 | 948 | 701 | 1282 |
| UNT 075 IN NCLL | 0.973155 | 1359 | 1005 | 1838 |
| UNT 075 IN NCLL | 1.322798 | 1768 | 1307 | 2391 |
| UNT 075 IN NCLL | 2.16422 | 2733 | 2021 | 3697 |
| UNT 076 IN NCLL | 0.199169 | 301 | 223 | 407 |
| UNT 077 IN NCLL | 0.329467 | 624 | 461 | 843 |
| UNT 078 IN NCLL | 0.689602 | 1157 | 855 | 1564 |
| UNT 079 IN NCLL | 0.144736 | 330 | 244 | 447 |
| UNT 080 IN NCLL | 0.182579 | 391 | 289 | 529 |
| UNT 081 IN NCLL | 0.190201 | 434 | 321 | 587 |
| UNT 082 IN NCLL | 0.648172 | 1184 | 875 | 1601 |
| UNT 084 IN NCLL | 0.330812 | 658 | 487 | 890 |
| UNT 085 IN NCLL | 0.420666 | 705 | 522 | 954 |
| UNT 086 IN NCLL | 1.272491 | 1757 | 1299 | 2376 |
| UNT 086 IN NCLL | 1.508964 | 2028 | 1500 | 2743 |
| UNT 087 IN NCLL | 0.174418 | 454 | 335 | 613 |
| UNT 088 IN NCLL | 0.138189 | 332 | 245 | 449 |
| UNT 089 IN NCLL | 0.24849 | 560 | 414 | 758 |
| UNT 089 IN NCLL | 0.468553 | 909 | 672 | 1229 |
| UNT 090 IN NCLL | 0.15191 | 364 | 269 | 493 |
| UNT 091 IN NCLL | 0.546391 | 1075 | 795 | 1454 |
| UNT 091 IN NCLL | 0.964994 | 1724 | 1275 | 2332 |
| UNT 092 IN NCLL | 0.401386 | 743 | 549 | 1004 |
| UNT 093 IN NCLL | 0.246158 | 501 | 370 | 677 |
| UNT 093 IN NCLL | 0.434925 | 813 | 601 | 1099 |
| UNT 093 IN NCLL | 0.738923 | 1321 | 977 | 1786 |
| UNT 094 IN NCLL | 0.108148 | 286 | 212 | 387 |
| UNT 095 IN NCLL | 0.167693 | 348 | 257 | 471 |
| UNT 096 IN NCLL | 0.446546 | 897 | 663 | 1213 |
| UNT 097 IN NCLL | 0.355443 | 634 | 469 | 858 |
| UNT 098 IN NCLL | 0.258486 | 454 | 336 | 614 |
| UNT 099 IN NCLL | 0.111693 | 257 | 190 | 348 |
| UNT 100 IN NCLL | 0.7869 | 1019 | 753 | 1377 |
| UNT 102 IN NCLL | 0.389818 | 828 | 612 | 1119 |
| UNT 105 IN NCLL | 0.153524 | 451 | 334 | 610 |
| UNT 106 IN NCLL | 0.18491 | 530 | 392 | 717 |
| UNT 107 IN NCLL | 0.359956 | 791 | 585 | 1070 |
| UNT 108 IN NCLL | 0.624049 | 1195 | 884 | 1617 |
| UNT 109 IN NCLL | 1.001582 | 1936 | 1431 | 2618 |
| UNT 109 IN NCLL | 1.328717 | 2096 | 1550 | 2834 |
| UNT 110 IN NCLL | 0.742421 | 1288 | 952 | 1741 |
| UNT 110 IN NCLL | 1.490401 | 2008 | 1485 | 2716 |
| UNT 111 IN NCLL | 0.334399 | 325 | 241 | 440 |
| UNT 112 IN NCLL | 1.034403 | 1972 | 1459 | 2668 |
| UNT 112 IN NCLL | 1.843721 | 2742 | 2027 | 3708 |
| UNT 112 IN NCLL | 2.378741 | 3218 | 2379 | 4351 |
| UNT 112 IN NCLL | 2.567579 | 3424 | 2532 | 4631 |
| UNT 112 IN NCLL | 2.765186 | 3692 | 2730 | 4993 |
| UNT 113 IN NCLL | 0.17935 | 454 | 336 | 614 |
| UNT 114 IN NCLL | 0.16077 | 346 | 256 | 467 |
| UNT 115 IN NCLL | 0.196738 | 423 | 313 | 572 |
| UNT 116 IN NCLL | 0.169486 | 336 | 248 | 454 |
| UNT 117 IN NCLL | 0.589614 | 907 | 670 | 1226 |
| UNT 117 IN NCLL | 1.001313 | 1391 | 1029 | 1881 |
| UNT 117 IN NCLL | 1.129638 | 1579 | 1168 | 2135 |
| UNT 118 IN NCLL | 0.311711 | 617 | 456 | 835 |
| UNT 119 IN NCLL | 0.11864 | 252 | 186 | 341 |
| UNT 120 IN NCLL | 0.187421 | 427 | 316 | 577 |
| UNT 122 IN NCLL | 0.305344 | 737 | 545 | 997 |
| UNT 123 IN NCLL | 0.515004 | 941 | 696 | 1272 |
| UNT 124 IN NCLL | 0.164913 | 382 | 283 | 517 |
| UNT 125 IN NCLL | 0.158097 | 418 | 309 | 565 |
| UNT 126 IN NCLL | 0.575266 | 883 | 653 | 1194 |
| UNT 127 IN NCLL | 0.236115 | 594 | 439 | 803 |
| UNT 128 IN NCLL | 0.168051 | 386 | 286 | 522 |
| UNT 129 IN NCLL | 0.095953 | 228 | 168 | 308 |
| UNT 130 IN NCLL | 0.258264 | 506 | 374 | 685 |
| UNT 131 IN NCLL | 0.163478 | 370 | 273 | 500 |
| UNT 132 IN NCLL | 0.101871 | 267 | 198 | 361 |
| UNT 133 IN NCLL | 0.089406 | 237 | 175 | 320 |
| UNT 134 IN NCLL | 0.309828 | 645 | 477 | 872 |
| UNT 136 IN NCLL | 0.439139 | 923 | 683 | 1249 |
| UNT 137 IN NCLL | 0.909844 | 1399 | 1034 | 1892 |
| UNT 138 IN NCLL | 0.131015 | 323 | 239 | 437 |
| UNT 139 IN NCLL | 0.05793 | 146 | 108 | 198 |
| UNT 140 IN NCLL | 0.628175 | 1013 | 749 | 1369 |
| UNT 141 IN NCLL | 0.198092 | 490 | 362 | 663 |
| UNT 141 IN NCLL | 0.340138 | 786 | 581 | 1063 |
| UNT 141 IN NCLL | 0.679738 | 1329 | 983 | 1798 |
| UNT 142 IN NCLL | 0.132899 | 359 | 265 | 485 |
| UNT 143 IN NCLL | 0.27593 | 621 | 459 | 840 |
| UNT 145 IN NCLL | 0.24302 | 463 | 342 | 626 |
| UNT 146 IN NCLL | 0.605666 | 972 | 718 | 1314 |
| UNT 147 IN NCLL | 0.955309 | 1344 | 994 | 1818 |
| UNT 148 IN NCLL | 0.996942 | 1283 | 949 | 1735 |
| UNT 149 IN NCLL | 0.691126 | 878 | 649 | 1187 |
| UNT 149 IN NCLL | 1.234827 | 1398 | 1033 | 1890 |
| UNT 150 IN NCLL | 0.436359 | 684 | 506 | 925 |
| UNT 152 IN NCLL | 0.212171 | 508 | 375 | 686 |
| UNT 153 IN NCLL | 0.24302 | 603 | 446 | 816 |
| UNT 154 IN NCLL | 0.195851 | 479 | 355 | 648 |
| UNT 155 IN NCLL | 0.205087 | 473 | 350 | 640 |
| UNT 156 IN NCLL | 0.360135 | 695 | 514 | 940 |
| UNT 157 IN NCLL | 0.159353 | 352 | 260 | 476 |
| UNT 158 IN NCLL | 0.613557 | 1140 | 843 | 1541 |
| UNT 159 IN NCLL | 0.166258 | 330 | 244 | 447 |
| UNT 160 IN NCLL | 0.344622 | 521 | 385 | 705 |
| UNT 160 IN NCLL | 1.057629 | 1252 | 926 | 1693 |
| UNT 161 IN NCLL | 0.323369 | 444 | 328 | 600 |
| UNT 163 IN NCLL | 0.142404 | 258 | 191 | 349 |
| UNT 164 IN NCLL | 0.833172 | 1356 | 1003 | 1834 |
| UNT 165 IN NCLL | 0.490702 | 659 | 488 | 892 |
| UNT 166 IN NCLL | 0.577777 | 1113 | 823 | 1505 |
| UNT 166 IN NCLL | 0.987593 | 1734 | 1282 | 2345 |
| UNT 167 IN NCLL | 0.184641 | 327 | 242 | 442 |
| UNT 167 IN NCLL | 0.391791 | 600 | 444 | 812 |
| UNT 168 IN NCLL | 0.16572 | 260 | 193 | 352 |
| UNT 169 IN NCLL | 1.196984 | 1369 | 1012 | 1851 |
| UNT 170 IN NCLL | 0.666026 | 1307 | 967 | 1768 |
| UNT 170 IN NCLL | 1.325139 | 2320 | 1715 | 3137 |
| UNT 170 IN NCLL | 1.634787 | 2753 | 2036 | 3723 |
| UNT 171 IN NCLL | 0.600017 | 1243 | 919 | 1681 |
| UNT 172 IN NCLL | 0.151445 | 230 | 170 | 311 |
| UNT 172 IN NCLL | 0.211723 | 343 | 253 | 464 |
| UNT 174 IN NCLL | 0.137203 | 205 | 152 | 278 |
| UNT 175 IN NCLL | 0.210557 | 359 | 265 | 485 |
| UNT 176 IN NCLL | 0.211992 | 426 | 315 | 576 |
| UNT 177 IN NCLL | 0.30077 | 563 | 417 | 762 |
| UNT 178 IN NCLL | 0.258332 | 498 | 368 | 673 |
| UNT 179 IN NCLL | 0.520295 | 772 | 571 | 1045 |
| UNT 180 IN NCLL | 0.509667 | 874 | 646 | 1182 |
| UNT 180 IN NCLL | 0.856689 | 1324 | 979 | 1790 |
| UNT 180 IN NCLL | 1.215479 | 1646 | 1217 | 2226 |
| UNT 181 IN NCLL | 0.310703 | 599 | 443 | 810 |
| UNT 182 IN NCLL | 0.252884 | 473 | 350 | 640 |
| UNT 183 IN NCLL | 1.540252 | 2237 | 1654 | 3025 |
| UNT 183 IN NCLL | 2.36347 | 3189 | 2358 | 4313 |
| UNT 184 IN NCLL | 0.754347 | 1313 | 971 | 1775 |
| UNT 185 IN NCLL | 1.036286 | 1958 | 1448 | 2648 |
| UNT 185 IN NCLL | 1.725081 | 2877 | 2127 | 3891 |
| UNT 186 IN NCLL | 0.608198 | 1266 | 936 | 1712 |
| UNT 187 IN NCLL | 0.560649 | 1046 | 774 | 1415 |
| UNT 188 IN NCLL | 0.567016 | 896 | 662 | 1211 |
| UNT 190 IN NCLL | 0.225892 | 509 | 377 | 689 |
| UNT 191 IN NCLL | 0.284091 | 459 | 340 | 621 |
| UNT 192 IN NCLL | 0.655794 | 1379 | 1019 | 1865 |
| UNT 194 IN NCLL | 0.282208 | 482 | 356 | 651 |
| UNT 196 IN NCLL | 0.887467 | 1103 | 816 | 1492 |
| UNT 196 IN NCLL | 0.953964 | 1373 | 1015 | 1857 |
| UNT 197 IN NCLL | 0.425154 | 739 | 547 | 1000 |
| UNT 197 IN NCLL | 0.59078 | 978 | 723 | 1322 |
| UNT 198 IN NCLL | 0.408739 | 666 | 493 | 901 |
| UNT 199 IN NCLL | 0.201052 | 443 | 328 | 599 |
| UNT 200 IN NCLL | 0.189035 | 484 | 358 | 654 |
| UNT 201 IN NCLL | 0.680365 | 1226 | 906 | 1658 |
| UNT 203 IN NCLL | 0.302385 | 646 | 478 | 874 |
| UNT 207 IN NCLL | 0.549978 | 1034 | 765 | 1399 |
| UNT 208 IN NCLL | 0.350989 | 654 | 483 | 884 |
| UNT 209 IN NCLL | 0.41134 | 809 | 599 | 1095 |
| UNT 210 IN NCLL | 0.282746 | 776 | 574 | 1049 |
| UNT 211 IN NCLL | 0.347402 | 761 | 563 | 1029 |
| UNT 212 IN NCLL | 0.250373 | 520 | 385 | 704 |
| UNT 213 IN NCLL | 0.367309 | 715 | 529 | 968 |
| UNT 214 IN NCLL | 0.219614 | 353 | 261 | 477 |
| UNT 216 IN NCLL | 0.647993 | 901 | 666 | 1218 |
| UNT 217 IN NCLL | 0.200334 | 414 | 306 | 559 |
| UNT 220 IN NCLL | 0.487743 | 858 | 635 | 1161 |
| UNT 221 IN NCLL | 0.3865 | 829 | 613 | 1121 |
| UNT 222 IN NCLL | 0.744752 | 1268 | 938 | 1715 |
| UNT 223 IN NCLL | 1.00221 | 1633 | 1208 | 2209 |
| UNT 224 IN NCLL | 0.635169 | 981 | 725 | 1326 |
| UNT 225 IN NCLL | 0.81999 | 1112 | 822 | 1504 |
| UNT 225 IN NCLL | 1.489863 | 1757 | 1299 | 2375 |
| UNT 225 IN NCLL | 2.005137 | 2354 | 1741 | 3184 |
| UNT 226 IN NCLL | 0.166706 | 360 | 266 | 486 |
| UNT 227 IN NCLL | 0.448824 | 734 | 543 | 993 |
| UNT 228 IN NCLL | 1.271235 | 2028 | 1499 | 2742 |
| UNT 230 IN NCLL | 0.13523 | 359 | 266 | 486 |
| UNT 231 IN NCLL | 0.377801 | 706 | 522 | 955 |
| UNT 231 IN NCLL | 0.620462 | 1023 | 757 | 1384 |
| UNT 232 IN NCLL | 0.165271 | 280 | 207 | 379 |
| UNT 233 IN NCLL | 0.531146 | 946 | 700 | 1280 |
| UNT 234 IN NCLL | 0.184731 | 364 | 269 | 492 |
| UNT 235 IN NCLL | 0.216117 | 383 | 283 | 518 |
| UNT 236 IN NCLL | 0.616786 | 993 | 734 | 1342 |
| UNT 236 IN NCLL | 1.282086 | 1768 | 1307 | 2391 |
| UNT 237 IN NCLL | 0.568002 | 916 | 678 | 1239 |
| UNT 238 IN NCLL | 0.321754 | 644 | 476 | 871 |
| UNT 239 IN NCLL | 0.355831 | 539 | 399 | 729 |
| UNT 240 IN NCLL | 0.296858 | 584 | 432 | 789 |
| UNT 241 IN NCLL | 0.28705 | 504 | 372 | 681 |
| UNT 243 IN NCLL | 0.249925 | 391 | 289 | 529 |
| UNT 244 IN NCLL | 0.249566 | 484 | 358 | 655 |
| UNT 245 IN NCLL | 1.038349 | 1634 | 1208 | 2210 |
| UNT 246 IN NCLL | 0.850659 | 1263 | 934 | 1709 |
| UNT 247 IN NCLL | 0.951453 | 1478 | 1093 | 1999 |
| UNT 247 IN NCLL | 1.146497 | 1756 | 1299 | 2375 |
| UNT 248 IN NCLL | 0.191905 | 416 | 308 | 563 |
| UNT 249 IN NCLL | 0.743945 | 1256 | 928 | 1698 |
| UNT 249 IN NCLL | 1.103543 | 1595 | 1179 | 2157 |
| UNT 250 IN NCLL | 0.263735 | 533 | 394 | 720 |
| UNT 251 IN NCLL | 0.443533 | 765 | 566 | 1034 |
| UNT 252 IN NCLL | 0.555448 | 964 | 713 | 1304 |
| UNT 252 IN NCLL | 1.084621 | 1551 | 1147 | 2097 |
| UNT 252 IN NCLL | 1.225142 | 1713 | 1267 | 2317 |
| UNT 252 IN NCLL | 1.928733 | 2302 | 1702 | 3113 |
| UNT 252 IN NCLL | 3.208936 | 3398 | 2512 | 4595 |
| UNT 253 IN NCLL | 0.417169 | 702 | 519 | 949 |
| UNT 254 IN NCLL | 0.132271 | 287 | 212 | 388 |
| UNT 255 IN NCLL | 0.289561 | 460 | 340 | 622 |
| UNT 256 IN NCLL | 0.64463 | 1130 | 836 | 1529 |
| UNT 257 IN NCLL | 0.376725 | 684 | 505 | 924 |
| UNT 258 IN NCLL | 1.285314 | 1165 | 862 | 1576 |
| UNT 259 IN NCLL | 0.61293 | 1106 | 818 | 1495 |
| UNT 260 IN NCLL | 1.219044 | 1958 | 1448 | 2649 |
| UNT 260 IN NCLL | 2.751055 | 3591 | 2656 | 4857 |
| UNT 260 IN NCLL | 4.540791 | 4757 | 3517 | 6433 |
| UNT 261 IN NCLL | 1.174296 | 2168 | 1603 | 2932 |
| UNT 262 IN NCLL | 0.409546 | 769 | 569 | 1041 |
| UNT 263 IN NCLL | 0.276917 | 524 | 387 | 708 |
| UNT 264 IN NCLL | 0.338613 | 602 | 445 | 815 |
| UNT 265 IN NCLL | 0.369013 | 755 | 558 | 1020 |
| UNT 266 IN NCLL | 0.296914 | 702 | 519 | 949 |
| UNT 267 IN NCLL | 0.38937 | 766 | 567 | 1037 |
| UNT 267 IN NCLL | 0.647903 | 1196 | 884 | 1618 |
| UNT 268 IN NCLL | 0.143122 | 335 | 248 | 453 |
| UNT 269 IN NCLL | 0.219614 | 557 | 412 | 753 |
| UNT 269 IN NCLL | 0.652835 | 1258 | 930 | 1701 |
| UNT 270 IN NCLL | 0.18482 | 425 | 315 | 575 |
| UNT 271 IN NCLL | 0.250732 | 514 | 380 | 696 |
| UNT 272 IN NCLL | 0.334219 | 762 | 564 | 1031 |
| UNT 272 IN NCLL | 0.585758 | 1195 | 884 | 1617 |
| UNT 273 IN NCLL | 0.216835 | 530 | 392 | 717 |
| UNT 274 IN NCLL | 0.108238 | 350 | 259 | 473 |
| UNT 275 IN NCLL | 0.407753 | 447 | 331 | 605 |
| UNT 278 IN NCLL | 0.057661 | 153 | 113 | 206 |
| UNT 279 IN NCLL | 0.086626 | 222 | 164 | 301 |
| UNT 280 IN NCLL | 0.16137 | 442 | 327 | 598 |
| UNT 281 IN NCLL | 0.133257 | 376 | 278 | 509 |
| UNT 282 IN NCLL | 0.24529 | 505 | 373 | 683 |
| UNT 283 IN NCLL | 0.05246 | 161 | 119 | 218 |
| UNT 284 IN NCLL | 0.09409 | 256 | 189 | 346 |
| UNT 285 IN NCLL | 0.040533 | 131 | 97 | 178 |
| UNT 286 IN NCLL | 0.064207 | 203 | 150 | 274 |
| UNT 287 IN NCLL | 0.129312 | 320 | 237 | 433 |
| UNT 288 IN NCLL | 0.09927 | 250 | 185 | 338 |
| UNT 288 IN NCLL | 0.17657 | 424 | 313 | 573 |
| UNT 289 IN NCLL | 0.285077 | 558 | 412 | 754 |
| UNT 290 IN NCLL | 0.028337 | 75 | 55 | 101 |
| UNT 291 IN NCLL | 0.110121 | 310 | 230 | 420 |
| UNT 291 IN NCLL | 0.291936 | 718 | 531 | 972 |
| UNT 291 IN NCLL | 0.352058 | 881 | 651 | 1191 |
| UNT 291 IN NCLL | 0.469773 | 1181 | 873 | 1597 |
| UNT 291 IN NCLL | 0.605907 | 1286 | 951 | 1739 |
| UNT 292 IN NCLL | 0.11686 | 282 | 209 | 381 |
| UNT 293 IN NCLL | 0.05237 | 188 | 139 | 254 |
| UNT 294 IN NCLL | 0.397351 | 569 | 421 | 769 |
| UNT 295 IN NCLL | 0.317002 | 794 | 587 | 1073 |
| UNT 296 IN NCLL | 0.423267 | 1012 | 748 | 1369 |
| UNT 297 IN NCLL | 0.05246 | 177 | 131 | 240 |
| UNT 298 IN NCLL | 0.480031 | 866 | 640 | 1171 |
| UNT 299 IN NCLL | 0.682249 | 1335 | 987 | 1806 |
| UNT 300 IN NCLL | 0.740313 | 1348 | 997 | 1823 |
| UNT 301 IN NCLL | 0.102857 | 236 | 175 | 320 |
| UNT 302 IN NCLL | 0.610598 | 1083 | 801 | 1464 |
| UNT 302 IN NCLL | 2.403653 | 3229 | 2388 | 4367 |
| UNT 303 IN NCLL | 0.588897 | 1198 | 886 | 1620 |
| UNT 305 IN NCLL | 0.367399 | 600 | 444 | 811 |
| UNT 306 IN NCLL | 0.352334 | 656 | 485 | 887 |
| UNT 307 IN NCLL | 1.62043 | 2501 | 1849 | 3382 |
| UNT 307 IN NCLL | 1.681768 | 2503 | 1851 | 3384 |
| UNT 307 IN NCLL | 2.652771 | 3534 | 2613 | 4780 |
| UNT 307 IN NCLL | 3.204542 | 3673 | 2716 | 4967 |
| UNT 307 IN NCLL | 3.688429 | 4103 | 3034 | 5549 |
| UNT 307 IN NCLL | 3.82626 | 4260 | 3150 | 5761 |
| UNT 307 IN NCLL | 3.959248 | 4286 | 3170 | 5797 |
| UNT 307 IN NCLL | 4.257956 | 4497 | 3325 | 6082 |
| UNT 308 IN NCLL | 0.247414 | 593 | 439 | 802 |
| UNT 310 IN NCLL | 0.965981 | 1749 | 1293 | 2365 |
| UNT 311 IN NCLL | 0.376905 | 883 | 653 | 1194 |
| UNT 312 IN NCLL | 0.246696 | 550 | 406 | 743 |
| UNT 313 IN NCLL | 0.134154 | 256 | 189 | 346 |
| UNT 314 IN NCLL | 0.194505 | 431 | 319 | 583 |
| UNT 315 IN NCLL | 0.125814 | 285 | 211 | 386 |
| UNT 316 IN NCLL | 0.125276 | 271 | 201 | 367 |
| UNT 317 IN NCLL | 0.0634 | 163 | 121 | 221 |
| UNT 318 IN NCLL | 0.330005 | 601 | 444 | 812 |
| UNT 319 IN NCLL | 0.332426 | 727 | 538 | 984 |
| UNT 320 IN NCLL | 0.188318 | 609 | 450 | 823 |
| UNT 321 IN NCLL | 0.353141 | 900 | 666 | 1217 |
| UNT 322 IN NCLL | 0.107521 | 264 | 195 | 356 |
| UNT 323 IN NCLL | 0.080811 | 239 | 176 | 323 |
| UNT 324 IN NCLL | 0.445058 | 1127 | 833 | 1524 |
| UNT 325 IN NCLL | 0.408291 | 747 | 552 | 1010 |
| UNT 327 IN NCLL | 0.984006 | 1943 | 1436 | 2627 |
| UNT 329 IN NCLL | 0.986337 | 1809 | 1338 | 2447 |
| UNT 330 IN NCLL | 0.638128 | 1284 | 949 | 1736 |
| UNT 331 IN NCLL | 0.251987 | 637 | 471 | 862 |
| UNT 332 IN NCLL | 1.372837 | 2454 | 1814 | 3318 |
| UNT 333 IN NCLL | 1.085966 | 1531 | 1132 | 2070 |
| UNT 334 IN NCLL | 0.26508 | 674 | 498 | 912 |
| UNT 335 IN NCLL | 0.195223 | 321 | 237 | 434 |
| UNT 336 IN NCLL | 1.133315 | 1917 | 1417 | 2592 |
| UNT 337 IN NCLL | 0.275572 | 636 | 470 | 860 |
| UNT 338 IN NCLL | 0.591318 | 958 | 708 | 1296 |
| UNT 339 IN NCLL | 0.232797 | 459 | 339 | 620 |
| UNT 340 IN NCLL | 0.362646 | 582 | 430 | 787 |
| UNT 343 IN NCLL | 0.196389 | 506 | 374 | 684 |
| UNT 344 IN NCLL | 0.901325 | 1051 | 777 | 1421 |
| UNT 344 IN NCLL | 1.437762 | 1644 | 1216 | 2223 |
| UNT 345 IN NCLL | 0.963649 | 1283 | 949 | 1736 |
| UNT 346 IN NCLL | 0.582889 | 982 | 726 | 1329 |
| UNT 347 IN NCLL | 0.133168 | 318 | 235 | 430 |
| UNT 348 IN NCLL | 0.499042 | 829 | 613 | 1120 |
| UNT 348 IN NCLL | 0.544418 | 895 | 662 | 1211 |
| UNT 349 IN NCLL | 0.685387 | 951 | 703 | 1286 |
| UNT 349 IN NCLL | 0.901863 | 1240 | 917 | 1676 |
| UNT 350 IN NCLL | 0.258982 | 574 | 424 | 776 |
| UNT 353 IN NCLL | 0.29826 | 545 | 403 | 736 |
| UNT 355 IN NCLL | 0.231454 | 407 | 301 | 551 |
| UNT 356 IN NCLL | 0.122496 | 265 | 196 | 359 |
| UNT 357 IN NCLL | 0.253063 | 455 | 337 | 615 |
| UNT 359 IN NCLL | 0.569796 | 951 | 703 | 1286 |
| UNT 360 IN NCLL | 0.196568 | 496 | 366 | 670 |
| UNT 360 IN NCLL | 0.416989 | 983 | 727 | 1329 |
| UNT 360 IN NCLL | 0.609074 | 1385 | 1024 | 1873 |
| UNT 361 IN NCLL | 0.176481 | 497 | 368 | 673 |
| UNT 362 IN NCLL | 0.19047 | 455 | 336 | 615 |
| UNT 363 IN NCLL | 0.100705 | 282 | 209 | 382 |
| UNT 363 IN NCLL | 0.207598 | 557 | 412 | 754 |
| UNT 363 IN NCLL | 0.341483 | 769 | 568 | 1040 |
| UNT 364 IN NCLL | 0.090572 | 282 | 208 | 381 |
| UNT 365 IN NCLL | 0.058468 | 190 | 140 | 256 |
| UNT 366 IN NCLL | 0.078933 | 136 | 100 | 183 |
| UNT 367 IN NCLL | 0.201231 | 375 | 277 | 507 |
| UNT 367 IN NCLL | 0.514735 | 839 | 620 | 1134 |
| UNT 368 IN NCLL | 0.633824 | 1339 | 990 | 1811 |
| UNT 369 IN NCLL | 0.22634 | 731 | 540 | 988 |
| UNT 370 IN NCLL | 0.057557 | 136 | 101 | 184 |
| UNT 371 IN NCLL | 0.290637 | 725 | 536 | 980 |
| UNT 380 IN NCLL | 0.421115 | 662 | 489 | 895 |
| UNT 381 IN NCLL | 0.06475 | 175 | 129 | 236 |
| VILLAGE CREEK | 0.536257 | 861 | 637 | 1164 |
| VILLAGE CREEK | 1.196625 | 1696 | 1254 | 2294 |
| VILLAGE CREEK | 3.454825 | 3612 | 2671 | 4885 |
| VILLAGE CREEK | 7.035106 | 5659 | 4184 | 7653 |
| WAYNE CREEK | 0.972168 | 1879 | 1390 | 2542 |
| WAYNE CREEK | 1.664192 | 2552 | 1887 | 3451 |
| WAYNE CREEK | 2.7055 | 3673 | 2716 | 4967 |
| WAYNE CREEK | 3.03111 | 3935 | 2909 | 5321 |
| WAYNE CREEK | 3.121772 | 3935 | 2909 | 5321 |
| WEST CAROLINA CREEK | 1.065789 | 1769 | 1308 | 2392 |
| WEST CAROLINA CREEK | 2.160454 | 3079 | 2277 | 4165 |
| WEST CAROLINA CREEK | 2.595199 | 3499 | 2588 | 4732 |
| WEST CAROLINA CREEK | 2.973987 | 3897 | 2882 | 5270 |
| WEST CAROLINA CREEK | 3.111728 | 3973 | 2938 | 5373 |
| WEST CAROLINA CREEK | 3.380036 | 4298 | 3178 | 5812 |
| WEST CAROLINA CREEK | 3.544142 | 4372 | 3233 | 5913 |
| WEST CAROLINA CREEK | 3.746987 | 4452 | 3292 | 6021 |
| WEST CAROLINA CREEK | 3.899704 | 4742 | 3507 | 6414 |
| WEST CAROLINA CREEK | 4.442059 | 5103 | 3773 | 6901 |
| WEST CAROLINA CREEK | 4.997328 | 5551 | 4105 | 7507 |
| WEST CAROLINA CREEK | 14.392325 | 11540 | 8533 | 15607 |
| WEST CAROLINA CREEK | 14.597054 | 11632 | 8601 | 15730 |
| WEST CAROLINA CREEK | 15.134746 | 11679 | 8636 | 15794 |
| WEST CAROLINA CREEK | 15.255987 | 11795 | 8722 | 15951 |
| WEST CAROLINA CREEK | 15.385621 | 11795 | 8722 | 15951 |
| WEST CAROLINA CREEK | 15.40413 | 11795 | 8722 | 15951 |
| WEST CAROLINA CREEK | 16.418087 | 11795 | 8722 | 15951 |
| WEST CAROLINA CREEK | 17.020973 | 11795 | 8722 | 15951 |
| WEST CAROLINA CREEK | 32.113931 | 18646 | 13788 | 25217 |
| WEST CAROLINA CREEK | 32.643283 | 18646 | 13788 | 25217 |
| WEST CAROLINA CREEK | 34.1596 | 18646 | 13788 | 25217 |
| WHITES CREEK | 4.727136 | 3678 | 2720 | 4975 |
| WHITES CREEK | 5.158384 | 4058 | 3000 | 5487 |
| WHITES CREEK | 7.362059 | 5144 | 3803 | 6956 |
| WHITES CREEK | 7.600236 | 5388 | 3984 | 7286 |
| WHITES CREEK | 8.641455 | 5778 | 4272 | 7814 |
| WHITES CREEK | 9.439922 | 6051 | 4475 | 8184 |
| WHITES CREEK | 10.600947 | 6444 | 4765 | 8715 |
| WHITES CREEK | 26.033504 | 11555 | 8544 | 15626 |
| WHITES CREEK | 27.047281 | 11681 | 8637 | 15797 |
| WHITES CREEK | 28.050926 | 11681 | 8637 | 15797 |
| WHITES CREEK | 28.4142 | 11738 | 8680 | 15874 |
| WOLF BRANCH | 0.740358 | 1035 | 765 | 1399 |
| South Bedias Creek - Bedias Creek |  |  |  |  |
| BEAR CREEK | 0.34368 | 395 | 292 | 534 |
| BEAR CREEK | 1.093768 | 1538 | 1137 | 2080 |
| BEAR CREEK | 1.338253 | 1817 | 1344 | 2458 |
| BEAR CREEK | 2.042935 | 2552 | 1887 | 3452 |
| BEAR CREEK | 2.198387 | 2717 | 2009 | 3675 |
| BEAR CREEK | 6.675733 | 6247 | 4619 | 8448 |
| BEAR CREEK | 6.899741 | 6247 | 4619 | 8448 |
| BEAR CREEK | 8.12708 | 7051 | 5213 | 9535 |
| BEAR CREEK | 8.559584 | 7051 | 5213 | 9535 |
| BEAR CREEK | 8.839245 | 7232 | 5348 | 9781 |
| BLACK OAK BRANCH | 1.000416 | 1060 | 784 | 1433 |
| BLACK OAK BRANCH | 2.166552 | 2039 | 1507 | 2757 |
| BLACK OAK BRANCH | 3.097918 | 2576 | 1905 | 3484 |
| BLACK OAK BRANCH | 4.055659 | 3093 | 2287 | 4183 |
| BLACK OAK BRANCH | 5.110464 | 3529 | 2610 | 4773 |
| BLACK OAK BRANCH | 5.407468 | 3627 | 2682 | 4905 |
| BLACK OAK BRANCH | 6.156843 | 4098 | 3030 | 5541 |
| BLACK OAK BRANCH | 10.178955 | 5932 | 4387 | 8023 |
| CAMP BRANCH | 0.943741 | 700 | 517 | 946 |
| CAMP BRANCH | 1.72822 | 1133 | 838 | 1532 |
| CAMP BRANCH | 2.525611 | 1536 | 1136 | 2077 |
| CEDAR BRANCH | 1.993838 | 3350 | 2477 | 4530 |
| CEDAR BRANCH | 2.156777 | 3456 | 2555 | 4673 |
| CEDAR BRANCH | 2.978247 | 4143 | 3063 | 5603 |
| CEDAR BRANCH | 3.142128 | 4221 | 3121 | 5708 |
| COLLARD CREEK | 1.005707 | 1143 | 845 | 1546 |
| COLLARD CREEK | 1.975723 | 1959 | 1448 | 2649 |
| COLLARD CREEK | 2.851042 | 2562 | 1894 | 3464 |
| COLLARD CREEK | 3.433841 | 2824 | 2088 | 3820 |
| COLLARD CREEK | 4.984549 | 3838 | 2838 | 5190 |
| COLLARD CREEK | 5.442699 | 3903 | 2886 | 5278 |
| COLLARD CREEK | 6.510551 | 4309 | 3186 | 5827 |
| COLLARD CREEK | 7.289289 | 4671 | 3454 | 6317 |
| COLLARD CREEK | 7.380031 | 4671 | 3454 | 6317 |
| COLLARD CREEK | 8.129098 | 4857 | 3591 | 6568 |
| COLLARD CREEK | 11.179351 | 6273 | 4639 | 8484 |
| COLLARD CREEK | 11.569169 | 6436 | 4759 | 8704 |
| COLLARD CREEK | 12.301994 | 6602 | 4882 | 8929 |
| COLLARD CREEK | 13.222241 | 6848 | 5063 | 9261 |
| HARBUCK BRANCH | 1.019427 | 677 | 501 | 916 |
| HARBUCK BRANCH | 1.832243 | 1031 | 762 | 1394 |
| HARBUCK BRANCH | 2.304472 | 1183 | 875 | 1600 |
| HARBUCK BRANCH | 2.75132 | 1352 | 1000 | 1829 |
| KELSO BRANCH | 0.53453 | 846 | 626 | 1144 |
| KELSO BRANCH | 1.114483 | 1431 | 1058 | 1935 |
| LARRISON CREEK | 0.888591 | 658 | 487 | 890 |
| LARRISON CREEK | 1.627425 | 1093 | 808 | 1478 |
| LARRISON CREEK | 3.103478 | 1756 | 1299 | 2375 |
| LARRISON CREEK | 4.956455 | 2675 | 1978 | 3617 |
| LARRISON CREEK | 5.284109 | 2675 | 1978 | 3617 |
| LARRISON CREEK | 7.305208 | 3361 | 2486 | 4546 |
| LARRISON CREEK | 8.45398 | 3699 | 2735 | 5003 |
| LARRISON CREEK | 9.258602 | 3699 | 2735 | 5003 |
| LARRISON CREEK | 11.15059 | 4340 | 3209 | 5869 |
| LARRISON CREEK | 12.065888 | 4541 | 3358 | 6141 |
| LARRISON CREEK | 13.059393 | 4544 | 3360 | 6145 |
| LARRISON CREEK | 14.202645 | 4739 | 3505 | 6410 |
| LARRISON CREEK | 14.751792 | 4739 | 3505 | 6410 |
| LARRISON CREEK | 28.444168 | 7760 | 5738 | 10494 |
| LARRISON CREEK | 28.542362 | 7836 | 5794 | 10597 |
| LARRISON CREEK | 31.445771 | 8128 | 6010 | 10992 |
| LARRISON CREEK | 35.486328 | 9139 | 6758 | 12359 |
| LARRISON CREEK | 41.729434 | 9713 | 7182 | 13136 |
| LARRISON CREEK | 44.025208 | 10378 | 7674 | 14035 |
| LARRISON CREEK | 45.114671 | 10463 | 7737 | 14150 |
| LARRISON CREEK | 45.423871 | 10463 | 7737 | 14150 |
| LARRISON CREEK | 48.328356 | 10872 | 8039 | 14702 |
| LARRISON CREEK | 52.08383 | 11819 | 8740 | 15984 |
| LARRISON CREEK | 52.452431 | 11819 | 8740 | 15984 |
| LARRISON CREEK | 54.755917 | 11848 | 8761 | 16023 |
| LARRISON CREEK | 55.577952 | 11905 | 8803 | 16100 |
| LARRISON CREEK | 62.625792 | 13355 | 9875 | 18061 |
| LARRISON CREEK | 64.623568 | 14038 | 10381 | 18985 |
| LARRISON CREEK | 64.732804 | 14038 | 10381 | 18985 |
| LARRISON CREEK | 65.323702 | 14174 | 10481 | 19168 |
| LARRISON CREEK | 65.407827 | 14174 | 10481 | 19168 |
| LARRISON CREEK | 65.649685 | 14234 | 10525 | 19249 |
| LARRISON CREEK | 66.036929 | 14295 | 10570 | 19333 |
| LARRISON CREEK | 66.458043 | 14398 | 10647 | 19472 |
| LARRISON CREEK | 67.142716 | 14398 | 10647 | 19472 |
| LARRISON CREEK | 69.590127 | 14794 | 10939 | 20007 |
| MANNING BRANCH | 0.728013 | 1536 | 1136 | 2077 |
| MANNING BRANCH | 1.603152 | 2854 | 2110 | 3859 |
| MANNING BRANCH | 3.081178 | 4776 | 3532 | 6460 |
| MANNING BRANCH | 3.82077 | 4913 | 3632 | 6644 |
| MANNING BRANCH | 4.168579 | 5164 | 3818 | 6983 |
| MOORING CREEK | 0.919888 | 1228 | 908 | 1661 |
| MOORING CREEK | 0.984146 | 1228 | 908 | 1661 |
| MOORING CREEK | 1.243389 | 1537 | 1137 | 2079 |
| MOORING CREEK | 1.697535 | 1904 | 1408 | 2575 |
| MOORING CREEK | 1.900261 | 2039 | 1508 | 2757 |
| MOORING CREEK | 3.049987 | 2726 | 2016 | 3687 |
| MOORING CREEK | 3.870006 | 3327 | 2460 | 4499 |
| MOORING CREEK | 4.405203 | 3419 | 2528 | 4623 |
| MOORING CREEK | 4.882812 | 3698 | 2734 | 5001 |
| MOORING CREEK | 5.15668 | 3918 | 2897 | 5299 |
| MOORING CREEK | 8.223839 | 5713 | 4225 | 7727 |
| MOORING CREEK | 8.308517 | 5713 | 4225 | 7727 |
| MOORING CREEK | 9.122475 | 5713 | 4225 | 7727 |
| MOORING CREEK | 9.756119 | 5959 | 4407 | 8059 |
| MOORING CREEK | 10.52616 | 6190 | 4577 | 8372 |
| MOORING CREEK | 10.850784 | 6293 | 4653 | 8510 |
| MOORING CREEK | 10.966196 | 6293 | 4653 | 8510 |
| MOORING CREEK | 12.013602 | 6615 | 4892 | 8946 |
| MYRTLE CREEK | 1.024539 | 782 | 578 | 1057 |
| MYRTLE CREEK | 1.867037 | 1189 | 879 | 1608 |
| MYRTLE CREEK | 2.646135 | 1566 | 1158 | 2118 |
| MYRTLE CREEK | 3.976197 | 2048 | 1514 | 2770 |
| MYRTLE CREEK | 4.895188 | 2374 | 1755 | 3211 |
| MYRTLE CREEK | 5.847448 | 2654 | 1962 | 3589 |
| MYRTLE CREEK | 6.513017 | 2813 | 2080 | 3805 |
| MYRTLE CREEK | 9.209549 | 3624 | 2680 | 4901 |
| MYRTLE CREEK | 12.756702 | 4490 | 3320 | 6072 |
| NIXON BRANCH | 1.735436 | 2001 | 1480 | 2707 |
| NIXON BRANCH | 2.710025 | 2440 | 1804 | 3300 |
| NORTH CEDAR CREEK | 0.805373 | 1551 | 1147 | 2098 |
| NORTH CEDAR CREEK | 1.069857 | 1936 | 1432 | 2618 |
| NORTH CEDAR CREEK | 1.459047 | 2542 | 1880 | 3438 |
| NORTH CEDAR CREEK | 1.723728 | 2542 | 1880 | 3438 |
| NORTH CEDAR CREEK | 3.51539 | 3752 | 2774 | 5074 |
| NORTH CEDAR CREEK | 3.588 | 3752 | 2774 | 5074 |
| NORTH CEDAR CREEK | 4.017574 | 4216 | 3117 | 5701 |
| NORTH CEDAR CREEK | 4.677601 | 4413 | 3263 | 5968 |
| NORTH CEDAR CREEK | 5.173307 | 4664 | 3449 | 6308 |
| NORTH PINE BRANCH | 1.155106 | 819 | 606 | 1108 |
| NORTH PINE BRANCH | 1.672621 | 1223 | 904 | 1654 |
| NORTH PINE BRANCH | 2.495839 | 1766 | 1306 | 2389 |
| NORTH PINE BRANCH | 3.381023 | 2238 | 1655 | 3027 |
| NORTH PINE BRANCH | 4.170971 | 2719 | 2011 | 3678 |
| NORTH PINE BRANCH | 6.123737 | 4000 | 2958 | 5410 |
| NORTH SAND BRANCH | 0.992883 | 1367 | 1011 | 1849 |
| PERSON BRANCH | 1.188472 | 756 | 559 | 1023 |
| PERSON BRANCH | 2.042262 | 1190 | 880 | 1609 |
| PERSON BRANCH | 3.318609 | 1711 | 1265 | 2314 |
| PERSON BRANCH | 4.447709 | 1968 | 1455 | 2662 |
| PINE BRANCH | 0.881327 | 1163 | 860 | 1572 |
| PINE BRANCH | 1.11043 | 1321 | 977 | 1787 |
| PINE BRANCH | 1.134046 | 1336 | 988 | 1807 |
| PINE BRANCH | 2.818939 | 3600 | 2662 | 4869 |
| PINE BRANCH | 2.848404 | 3988 | 2949 | 5393 |
| PINE BRANCH | 3.098815 | 4216 | 3118 | 5702 |
| PINE BRANCH | 3.854418 | 4307 | 3185 | 5825 |
| PINE CREEK | 1.291412 | 1472 | 1089 | 1991 |
| PINE CREEK | 1.689121 | 1658 | 1226 | 2242 |
| PINE CREEK | 2.581804 | 2356 | 1742 | 3186 |
| PINE CREEK | 2.92359 | 2521 | 1864 | 3410 |
| PINE CREEK | 5.834714 | 4363 | 3226 | 5900 |
| PINE CREEK | 9.487542 | 6627 | 4900 | 8962 |
| PINE CREEK | 10.392723 | 7045 | 5210 | 9528 |
| PINE CREEK | 11.142422 | 7045 | 5210 | 9528 |
| PINE CREEK | 11.842143 | 7209 | 5331 | 9750 |
| PINE CREEK | 12.980031 | 7209 | 5331 | 9750 |
| PINE CREEK | 14.602883 | 7569 | 5597 | 10236 |
| PINE CREEK | 14.953602 | 7937 | 5869 | 10734 |
| PINE CREEK | 15.232671 | 7937 | 5869 | 10734 |
| PINE CREEK | 15.237872 | 7937 | 5869 | 10734 |
| PINE CREEK | 15.383299 | 8013 | 5925 | 10837 |
| PINE CREEK | 16.001429 | 8089 | 5981 | 10939 |
| PINE CREEK | 16.251533 | 8203 | 6066 | 11094 |
| PINE CREEK | 22.475259 | 10002 | 7396 | 13526 |
| POLLARD BRANCH | 1.01064 | 1231 | 910 | 1664 |
| POLLARD BRANCH | 1.932949 | 2060 | 1523 | 2785 |
| POLLARD BRANCH | 3.149942 | 2915 | 2155 | 3942 |
| POLLARD BRANCH | 3.369368 | 3008 | 2224 | 4068 |
| POOLES CREEK | 0.903926 | 745 | 551 | 1007 |
| POOLES CREEK | 1.66399 | 1234 | 912 | 1669 |
| POOLES CREEK | 2.336343 | 1648 | 1218 | 2229 |
| POOLES CREEK | 3.184581 | 2356 | 1742 | 3187 |
| POOLES CREEK | 3.908528 | 2725 | 2015 | 3685 |
| POOLES CREEK | 4.869815 | 3217 | 2379 | 4351 |
| POOLES CREEK | 5.18866 | 3480 | 2573 | 4706 |
| POOLES CREEK | 7.65925 | 4817 | 3562 | 6514 |
| POOLES CREEK | 7.977149 | 4838 | 3577 | 6543 |
| POOLES CREEK | 10.414699 | 6210 | 4592 | 8398 |
| POOLES CREEK | 11.16537 | 6391 | 4726 | 8643 |
| POOLES CREEK | 12.885429 | 6866 | 5077 | 9286 |
| POOLES CREEK | 15.871253 | 7996 | 5913 | 10814 |
| POOLES CREEK | 17.053979 | 8187 | 6054 | 11073 |
| POOLES CREEK | 18.239754 | 8952 | 6620 | 12107 |
| POOLES CREEK | 20.311074 | 9281 | 6863 | 12552 |
| POOLES CREEK | 21.60679 | 9529 | 7046 | 12887 |
| POOLES CREEK | 22.81579 | 10057 | 7437 | 13601 |
| POOLES CREEK | 23.558929 | 10082 | 7455 | 13635 |
| POOLES CREEK | 24.257946 | 10300 | 7616 | 13930 |
| POOLES CREEK | 26.197441 | 10575 | 7819 | 14301 |
| POOLES CREEK | 26.221205 | 10575 | 7819 | 14301 |
| POOLES CREEK | 27.533421 | 10832 | 8010 | 14649 |
| ROCKY BRANCH | 0.894275 | 1029 | 761 | 1392 |
| SALTY CREEK | 1.003161 | 1319 | 975 | 1784 |
| SALTY CREEK | 1.603267 | 1888 | 1396 | 2553 |
| SAND BRANCH | 0.444251 | 635 | 470 | 859 |
| SAND BRANCH | 0.843349 | 1101 | 814 | 1489 |
| SAND BRANCH | 1.247381 | 1381 | 1022 | 1868 |
| SAND BRANCH | 2.486513 | 2262 | 1672 | 3058 |
| SAND BRANCH | 3.779629 | 3070 | 2270 | 4152 |
| SAND BRANCH | 3.978061 | 3127 | 2312 | 4228 |
| SAND BRANCH | 4.254324 | 3258 | 2409 | 4406 |
| SAND BRANCH | 4.561219 | 3416 | 2526 | 4619 |
| SAND BRANCH | 4.700056 | 3461 | 2559 | 4681 |
| SAND BRANCH | 5.878835 | 4185 | 3095 | 5660 |
| SCHOOLHOUSE BRANCH | 1.128338 | 2141 | 1583 | 2895 |
| SCHOOLHOUSE BRANCH | 1.521042 | 2541 | 1879 | 3436 |
| SCHOOLHOUSE BRANCH | 2.189336 | 3162 | 2338 | 4276 |
| SOUTH BEDIAS CREEK | 0.621628 | 902 | 667 | 1220 |
| SOUTH BEDIAS CREEK | 1.222272 | 1613 | 1193 | 2181 |
| SOUTH BEDIAS CREEK | 1.589268 | 1964 | 1452 | 2656 |
| SOUTH BEDIAS CREEK | 1.831974 | 2064 | 1526 | 2792 |
| SOUTH BEDIAS CREEK | 2.739666 | 2844 | 2103 | 3846 |
| SOUTH BEDIAS CREEK | 3.991654 | 3609 | 2669 | 4881 |
| SOUTH BEDIAS CREEK | 5.919675 | 5591 | 4134 | 7562 |
| SOUTH BEDIAS CREEK | 5.969983 | 204 | 151 | 276 |
| SOUTH BEDIAS CREEK | 6.289742 | 5591 | 4134 | 7562 |
| SOUTH BEDIAS CREEK | 7.311906 | 6109 | 4517 | 8262 |
| SOUTH BEDIAS CREEK | 11.420284 | 8326 | 6156 | 11260 |
| SOUTH BEDIAS CREEK | 14.057359 | 9148 | 6764 | 12371 |
| SOUTH BEDIAS CREEK | 26.245648 | 12658 | 9360 | 17119 |
| SOUTH BEDIAS CREEK | 26.525166 | 12689 | 9383 | 17160 |
| SOUTH BEDIAS CREEK | 26.892657 | 12689 | 9383 | 17160 |
| SOUTH BEDIAS CREEK | 27.365292 | 12830 | 9487 | 17352 |
| SOUTH BEDIAS CREEK | 27.395335 | 12830 | 9487 | 17352 |
| SOUTH BEDIAS CREEK | 27.558017 | 12957 | 9581 | 17523 |
| SOUTH BEDIAS CREEK | 27.702963 | 12985 | 9602 | 17561 |
| SOUTH BEDIAS CREEK | 28.241549 | 12985 | 9602 | 17561 |
| SOUTH BEDIAS CREEK | 28.614235 | 12999 | 9612 | 17579 |
| SOUTH BEDIAS CREEK | 33.470749 | 14082 | 10412 | 19044 |
| SOUTH BEDIAS CREEK | 35.445844 | 14082 | 10412 | 19044 |
| SOUTH BEDIAS CREEK | 57.934913 | 19980 | 14774 | 27021 |
| SOUTH BEDIAS CREEK | 59.45957 | 19987 | 14779 | 27030 |
| SOUTH BEDIAS CREEK | 60.668212 | 20257 | 14979 | 27396 |
| SOUTH BEDIAS CREEK | 61.179958 | 20257 | 14979 | 27396 |
| SOUTH BEDIAS CREEK | 61.493223 | 20257 | 14979 | 27396 |
| SOUTH BEDIAS CREEK | 64.084835 | 21119 | 15616 | 28562 |
| SOUTH BEDIAS CREEK | 64.327586 | 21119 | 15616 | 28562 |
| SOUTH BEDIAS CREEK | 64.714624 | 21119 | 15616 | 28562 |
| SOUTH BEDIAS CREEK | 65.291274 | 21119 | 15616 | 28562 |
| SOUTH BEDIAS CREEK | 72.169295 | 21532 | 15922 | 29120 |
| SOUTH BEDIAS CREEK | 73.729441 | 22346 | 16524 | 30221 |
| SOUTH BEDIAS CREEK | 74.458348 | 22346 | 16524 | 30221 |
| SOUTH BEDIAS CREEK | 75.552139 | 22489 | 16629 | 30413 |
| SOUTH BEDIAS CREEK | 75.578145 | 22489 | 16629 | 30413 |
| SOUTH BEDIAS CREEK | 75.89972 | 22489 | 16629 | 30413 |
| SOUTH BEDIAS CREEK | 76.282066 | 22489 | 16629 | 30413 |
| SOUTH BEDIAS CREEK | 80.280582 | 22967 | 16983 | 31060 |
| SOUTH BEDIAS CREEK | 81.281502 | 22967 | 16983 | 31060 |
| SOUTH BEDIAS CREEK | 81.631386 | 22967 | 16983 | 31060 |
| SOUTH BEDIAS CREEK | 82.144866 | 22967 | 16983 | 31060 |
| SOUTH BEDIAS CREEK | 82.617248 | 22967 | 16983 | 31060 |
| SOUTH BEDIAS CREEK | 83.625824 | 22967 | 16983 | 31060 |
| SOUTH BEDIAS CREEK | 93.720432 | 24557 | 18158 | 33211 |
| SOUTH BEDIAS CREEK | 100.736265 | 25348 | 18743 | 34281 |
| SOUTH BEDIAS CREEK | 101.67077 | 25628 | 18950 | 34659 |
| SOUTH BEDIAS CREEK | 103.08822 | 26131 | 19322 | 35339 |
| SOUTH BEDIAS CREEK | 103.74509 | 26264 | 19421 | 35519 |
| SOUTH BEDIAS CREEK | 104.7169 | 26264 | 19421 | 35519 |
| SOUTH BEDIAS CREEK | 109.273368 | 26729 | 19764 | 36148 |
| SOUTH BEDIAS CREEK | 110.192897 | 26729 | 19764 | 36148 |
| SOUTH BEDIAS CREEK | 110.554108 | 27217 | 20125 | 36808 |
| SOUTH BEDIAS CREEK | 110.864115 | 27217 | 20125 | 36808 |
| SOUTH BEDIAS CREEK | 110.919086 | 27217 | 20125 | 36808 |
| SOUTH BEDIAS CREEK | 115.735047 | 27359 | 20231 | 37000 |
| SOUTH BEDIAS CREEK | 117.350959 | 27375 | 20242 | 37022 |
| SOUTH BEDIAS CREEK | 119.83102 | 27485 | 20323 | 37170 |
| SOUTH BEDIAS CREEK | 125.319707 | 28803 | 21298 | 38953 |
| SOUTH BEDIAS CREEK | 125.582187 | 28803 | 21298 | 38953 |
| SOUTH BEDIAS CREEK | 125.750059 | 28803 | 21298 | 38953 |
| SOUTH BEDIAS CREEK | 126.1514 | 28803 | 21298 | 38953 |
| SOUTH BEDIAS CREEK | 126.590674 | 28803 | 21298 | 38953 |
| SOUTH BEDIAS CREEK | 129.867314 | 28803 | 21298 | 38953 |
| SOUTH BEDIAS CREEK | 132.405715 | 28803 | 21298 | 38953 |
| SOUTH BEDIAS CREEK | 132.593585 | 28803 | 21298 | 38953 |
| SOUTH BEDIAS CREEK | 135.141578 | 29375 | 21721 | 39726 |
| SOUTH FORK SOUTH BEDIAS CREEK | 1.139323 | 2479 | 1833 | 3353 |
| SOUTH FORK SOUTH BEDIAS CREEK | 1.320108 | 2514 | 1859 | 3400 |
| SOUTH FORK SOUTH BEDIAS CREEK | 1.801674 | 3152 | 2331 | 4263 |
| SOUTH FORK SOUTH BEDIAS CREEK | 2.790208 | 4502 | 3329 | 6088 |
| SOUTH FORK SOUTH BEDIAS CREEK | 3.279789 | 4679 | 3460 | 6328 |
| SOUTH FORK SOUTH BEDIAS CREEK | 5.472623 | 7174 | 5305 | 9703 |
| SOUTH FORK SOUTH BEDIAS CREEK | 6.698204 | 8198 | 6062 | 11087 |
| SOUTH FORK SOUTH BEDIAS CREEK | 7.007045 | 8198 | 6062 | 11087 |
| STEEP BRANCH | 0.354576 | 505 | 374 | 683 |
| STEEP BRANCH | 1.329783 | 1481 | 1095 | 2003 |
| STEEP BRANCH | 2.019295 | 2025 | 1497 | 2738 |
| STEEP BRANCH | 2.782162 | 2411 | 1783 | 3261 |
| STEEP BRANCH | 3.766167 | 3111 | 2300 | 4207 |
| SULPHUR BRANCH | 0.608446 | 1147 | 848 | 1551 |
| SULPHUR BRANCH | 0.933159 | 1550 | 1146 | 2096 |
| SULPHUR BRANCH | 1.780851 | 2554 | 1888 | 3453 |
| SULPHUR BRANCH | 2.06492 | 2736 | 2023 | 3701 |
| SULPHUR BRANCH | 2.452262 | 2904 | 2148 | 3928 |
| SULPHUR CREEK | 0.83102 | 1268 | 938 | 1715 |
| SULPHUR CREEK | 1.319929 | 1779 | 1315 | 2406 |
| SULPHUR CREEK | 2.330568 | 2989 | 2210 | 4042 |
| SULPHUR CREEK | 4.088524 | 4612 | 3410 | 6238 |
| SULPHUR CREEK | 5.344425 | 5080 | 3756 | 6870 |
| SULPHUR CREEK | 6.083258 | 5575 | 4123 | 7540 |
| TOWN BRANCH | 0.927618 | 1011 | 748 | 1367 |
| TOWN BRANCH | 1.086594 | 1159 | 857 | 1568 |
| TOWN BRANCH | 1.977741 | 1981 | 1465 | 2679 |
| TOWN BRANCH | 2.933499 | 2697 | 1994 | 3648 |
| TOWN BRANCH | 4.105659 | 3182 | 2353 | 4304 |
| TOWN BRANCH | 4.146668 | 3216 | 2378 | 4349 |
| TURKEY CREEK | 0.903836 | 1104 | 817 | 1493 |
| TURKEY CREEK | 1.640554 | 1785 | 1320 | 2413 |
| TURKEY CREEK | 1.740151 | 2710 | 2004 | 3665 |
| TURKEY CREEK | 1.753333 | 2710 | 2004 | 3665 |
| TURKEY CREEK | 1.766874 | 2953 | 2183 | 3993 |
| UNT 002 IN SBBC | 1.031892 | 697 | 515 | 942 |
| UNT 003 IN SBBC | 0.905968 | 721 | 533 | 975 |
| UNT 003 IN SBBC | 1.524694 | 1060 | 784 | 1433 |
| UNT 004 IN SBBC | 1.034224 | 721 | 533 | 975 |
| UNT 005 IN SBBC | 1.005976 | 825 | 610 | 1116 |
| UNT 005 IN SBBC | 1.606132 | 1050 | 776 | 1420 |
| UNT 007 IN SBBC | 0.918901 | 788 | 583 | 1066 |
| UNT 009 IN SBBC | 1.071977 | 893 | 660 | 1208 |
| UNT 009 IN SBBC | 1.587609 | 1058 | 782 | 1431 |
| UNT 014 IN SBBC | 1.152864 | 786 | 582 | 1064 |
| UNT 016 IN SBBC | 1.028838 | 1047 | 775 | 1417 |
| UNT 016 IN SBBC | 2.837229 | 2350 | 1738 | 3178 |
| UNT 017 IN SBBC | 1.026068 | 1317 | 974 | 1781 |
| UNT 018 IN SBBC | 1.360372 | 818 | 605 | 1106 |
| UNT 018 IN SBBC | 3.426309 | 1966 | 1454 | 2659 |
| UNT 019 IN SBBC | 1.104888 | 694 | 513 | 938 |
| UNT 020 IN SBBC | 1.022117 | 1501 | 1110 | 2030 |
| UNT 020 IN SBBC | 1.534701 | 2019 | 1493 | 2730 |
| UNT 022 IN SBBC | 1.000506 | 1396 | 1032 | 1888 |
| UNT 022 IN SBBC | 1.932858 | 2213 | 1637 | 2993 |
| UNT 023 IN SBBC | 1.266303 | 1463 | 1082 | 1978 |
| UNT 024 IN SBBC | 0.223192 | 470 | 347 | 635 |
| UNT 024 IN SBBC | 0.376166 | 678 | 501 | 916 |
| UNT 024 IN SBBC | 0.527717 | 947 | 701 | 1281 |
| UNT 025 IN SBBC | 0.127339 | 234 | 173 | 316 |
| UNT 026 IN SBBC | 0.242662 | 382 | 283 | 517 |
| UNT 027 IN SBBC | 0.414568 | 516 | 382 | 698 |
| UNT 028 IN SBBC | 0.272913 | 339 | 250 | 458 |
| UNT 028 IN SBBC | 1.126141 | 1102 | 815 | 1490 |
| UNT 028 IN SBBC | 2.424132 | 1881 | 1391 | 2545 |
| UNT 029 IN SBBC | 0.231631 | 360 | 266 | 487 |
| UNT 030 IN SBBC | 0.461648 | 471 | 348 | 637 |
| UNT 030 IN SBBC | 0.943562 | 918 | 678 | 1241 |
| UNT 031 IN SBBC | 0.223022 | 433 | 321 | 586 |
| UNT 031 IN SBBC | 0.365608 | 684 | 506 | 925 |
| UNT 031 IN SBBC | 1.331317 | 2547 | 1884 | 3445 |
| UNT 032 IN SBBC | 0.135409 | 363 | 268 | 490 |
| UNT 033 IN SBBC | 0.341931 | 836 | 618 | 1131 |
| UNT 035 IN SBBC | 0.226968 | 366 | 270 | 495 |
| UNT 036 IN SBBC | 0.398965 | 670 | 496 | 906 |
| UNT 038 IN SBBC | 1.015622 | 1302 | 963 | 1760 |
| UNT 040 IN SBBC | 0.365919 | 934 | 691 | 1264 |
| UNT 040 IN SBBC | 0.574892 | 1425 | 1054 | 1928 |
| UNT 041 IN SBBC | 0.181099 | 590 | 436 | 797 |
| UNT 042 IN SBBC | 0.786989 | 1753 | 1296 | 2371 |
| UNT 043 IN SBBC | 0.150027 | 396 | 293 | 535 |
| UNT 044 IN SBBC | 0.24302 | 419 | 310 | 567 |
| UNT 045 IN SBBC | 0.635079 | 882 | 652 | 1193 |
| UNT 045 IN SBBC | 1.235634 | 1929 | 1426 | 2608 |
| UNT 045 IN SBBC | 5.406455 | 6472 | 4785 | 8752 |
| UNT 046 IN SBBC | 0.540383 | 929 | 687 | 1257 |
| UNT 048 IN SBBC | 0.526931 | 713 | 527 | 964 |
| UNT 049 IN SBBC | 0.131598 | 281 | 208 | 380 |
| UNT 050 IN SBBC | 0.159308 | 335 | 248 | 453 |
| UNT 051 IN SBBC | 0.882631 | 1107 | 818 | 1497 |
| UNT 054 IN SBBC | 0.56325 | 827 | 611 | 1118 |
| UNT 054 IN SBBC | 0.930858 | 1189 | 879 | 1609 |
| UNT 056 IN SBBC | 0.221498 | 355 | 263 | 480 |
| UNT 057 IN SBBC | 0.312115 | 519 | 384 | 702 |
| UNT 057 IN SBBC | 0.384841 | 584 | 432 | 790 |
| UNT 058 IN SBBC | 0.115233 | 143 | 106 | 193 |
| UNT 059 IN SBBC | 0.273061 | 428 | 317 | 579 |
| UNT 059 IN SBBC | 0.445596 | 664 | 491 | 898 |
| UNT 059 IN SBBC | 0.90007 | 1665 | 1231 | 2252 |
| UNT 059 IN SBBC | 0.940961 | 1665 | 1231 | 2252 |
| UNT 059 IN SBBC | 1.366918 | 2271 | 1680 | 3072 |
| UNT 059 IN SBBC | 1.462423 | 2304 | 1704 | 3116 |
| UNT 060 IN SBBC | 0.170742 | 297 | 220 | 402 |
| UNT 061 IN SBBC | 0.410712 | 823 | 608 | 1113 |
| UNT 061 IN SBBC | 0.43423 | 866 | 640 | 1171 |
| UNT 062 IN SBBC | 0.022688 | 31 | 23 | 42 |
| UNT 063 IN SBBC | 0.209481 | 389 | 287 | 526 |
| UNT 066 IN SBBC | 0.082232 | 133 | 98 | 180 |
| UNT 066 IN SBBC | 0.108866 | 173 | 128 | 234 |
| UNT 066 IN SBBC | 0.35601 | 487 | 360 | 659 |
| UNT 067 IN SBBC | 0.056854 | 117 | 87 | 159 |
| UNT 067 IN SBBC | 0.096311 | 188 | 139 | 254 |
| UNT 068 IN SBBC | 0.138417 | 235 | 173 | 317 |
| UNT 068 IN SBBC | 0.235345 | 346 | 256 | 469 |
| UNT 069 IN SBBC | 0.094955 | 140 | 104 | 190 |
| UNT 070 IN SBBC | 0.122855 | 235 | 174 | 317 |
| UNT 071 IN SBBC | 0.352465 | 554 | 409 | 749 |
| UNT 071 IN SBBC | 0.626782 | 922 | 682 | 1247 |
| UNT 071 IN SBBC | 1.024581 | 1280 | 946 | 1731 |
| UNT 071 IN SBBC | 1.059644 | 1315 | 972 | 1778 |
| UNT 071 IN SBBC | 1.637062 | 1847 | 1366 | 2499 |
| UNT 071 IN SBBC | 1.783457 | 1962 | 1451 | 2653 |
| UNT 071 IN SBBC | 2.220444 | 2313 | 1710 | 3128 |
| UNT 072 IN SBBC | 0.257368 | 397 | 294 | 537 |
| UNT 073 IN SBBC | 0.150878 | 225 | 166 | 304 |
| UNT 074 IN SBBC | 0.428199 | 675 | 499 | 913 |
| UNT 075 IN SBBC | 0.11187 | 170 | 125 | 229 |
| UNT 076 IN SBBC | 0.343042 | 462 | 342 | 625 |
| UNT 078 IN SBBC | 0.258633 | 482 | 356 | 652 |
| UNT 079 IN SBBC | 0.140746 | 278 | 206 | 376 |
| UNT 081 IN SBBC | 0.859805 | 1156 | 855 | 1564 |
| UNT 083 IN SBBC | 0.656646 | 1012 | 748 | 1369 |
| UNT 085 IN SBBC | 0.364725 | 518 | 383 | 701 |
| UNT 086 IN SBBC | 0.459944 | 640 | 473 | 866 |
| UNT 087 IN SBBC | 0.257637 | 501 | 370 | 677 |
| UNT 088 IN SBBC | 0.744752 | 1415 | 1046 | 1914 |
| UNT 088 IN SBBC | 1.468072 | 2118 | 1566 | 2864 |
| UNT 088 IN SBBC | 2.506152 | 3259 | 2410 | 4407 |
| UNT 088 IN SBBC | 2.685681 | 3405 | 2517 | 4604 |
| UNT 089 IN SBBC | 0.665659 | 1083 | 801 | 1465 |
| UNT 090 IN SBBC | 0.283643 | 400 | 296 | 541 |
| UNT 092 IN SBBC | 0.373676 | 528 | 391 | 714 |
| UNT 092 IN SBBC | 0.693072 | 897 | 663 | 1213 |
| UNT 093 IN SBBC | 0.318167 | 452 | 335 | 612 |
| UNT 094 IN SBBC | 0.605548 | 1939 | 1434 | 2622 |
| UNT 095 IN SBBC | 0.52182 | 913 | 675 | 1234 |
| UNT 096 IN SBBC | 0.255933 | 480 | 355 | 650 |
| UNT 097 IN SBBC | 0.397208 | 529 | 391 | 715 |
| UNT 097 IN SBBC | 0.732825 | 921 | 681 | 1245 |
| UNT 098 IN SBBC | 0.31193 | 464 | 343 | 628 |
| UNT 100 IN SBBC | 0.19612 | 291 | 215 | 394 |
| UNT 100 IN SBBC | 0.261672 | 363 | 268 | 491 |
| UNT 101 IN SBBC | 0.184641 | 264 | 195 | 357 |
| UNT 102 IN SBBC | 0.44699 | 655 | 484 | 886 |
| UNT 103 IN SBBC | 0.161366 | 275 | 204 | 372 |
| UNT 104 IN SBBC | 0.114696 | 203 | 150 | 274 |
| UNT 105 IN SBBC | 0.11839 | 155 | 114 | 209 |
| UNT 105 IN SBBC | 0.321128 | 431 | 319 | 583 |
| UNT 105 IN SBBC | 0.328121 | 431 | 319 | 583 |
| UNT 105 IN SBBC | 0.477069 | 635 | 470 | 859 |
| UNT 106 IN SBBC | 0.062189 | 122 | 91 | 166 |
| UNT 106 IN SBBC | 0.104245 | 193 | 143 | 261 |
| UNT 107 IN SBBC | 0.206701 | 339 | 251 | 458 |
| UNT 109 IN SBBC | 0.544732 | 712 | 527 | 963 |
| UNT 109 IN SBBC | 0.749998 | 842 | 622 | 1138 |
| UNT 111 IN SBBC | 0.468463 | 532 | 393 | 719 |
| UNT 111 IN SBBC | 0.830302 | 975 | 721 | 1318 |
| UNT 112 IN SBBC | 0.298977 | 429 | 318 | 581 |
| UNT 113 IN SBBC | 0.110031 | 238 | 176 | 322 |
| UNT 113 IN SBBC | 0.153075 | 248 | 184 | 336 |
| UNT 114 IN SBBC | 0.693095 | 1038 | 767 | 1403 |
| UNT 114 IN SBBC | 0.7426 | 1038 | 767 | 1403 |
| UNT 114 IN SBBC | 1.279037 | 1545 | 1142 | 2089 |
| UNT 116 IN SBBC | 0.309144 | 500 | 370 | 677 |
| UNT 120 IN SBBC | 0.260058 | 417 | 308 | 563 |
| UNT 120 IN SBBC | 0.596519 | 873 | 646 | 1181 |
| UNT 120 IN SBBC | 1.202903 | 1613 | 1193 | 2181 |
| UNT 120 IN SBBC | 2.288383 | 2675 | 1978 | 3618 |
| UNT 120 IN SBBC | 2.526956 | 2675 | 1978 | 3618 |
| UNT 121 IN SBBC | 0.240598 | 385 | 285 | 520 |
| UNT 122 IN SBBC | 0.238984 | 402 | 297 | 544 |
| UNT 122 IN SBBC | 0.493133 | 773 | 571 | 1045 |
| UNT 123 IN SBBC | 0.199497 | 373 | 276 | 504 |
| UNT 124 IN SBBC | 0.225356 | 374 | 277 | 506 |
| UNT 124 IN SBBC | 0.546839 | 775 | 573 | 1049 |
| UNT 124 IN SBBC | 0.793567 | 1043 | 771 | 1411 |
| UNT 125 IN SBBC | 0.150004 | 296 | 219 | 400 |
| UNT 126 IN SBBC | 0.199707 | 299 | 221 | 404 |
| UNT 127 IN SBBC | 1.046778 | 1392 | 1029 | 1883 |
| UNT 127 IN SBBC | 1.533515 | 1757 | 1299 | 2375 |
| UNT 127 IN SBBC | 2.004061 | 2037 | 1506 | 2755 |
| UNT 127 IN SBBC | 2.892052 | 2674 | 1977 | 3616 |
| UNT 127 IN SBBC | 3.063931 | 2706 | 2001 | 3660 |
| UNT 127 IN SBBC | 3.635431 | 3055 | 2259 | 4132 |
| UNT 129 IN SBBC | 0.212889 | 371 | 274 | 502 |
| UNT 130 IN SBBC | 0.74816 | 896 | 662 | 1212 |
| UNT 130 IN SBBC | 0.891999 | 1057 | 781 | 1429 |
| UNT 132 IN SBBC | 0.470795 | 640 | 474 | 866 |
| UNT 134 IN SBBC | 0.576328 | 883 | 653 | 1194 |
| UNT 135 IN SBBC | 0.193339 | 397 | 293 | 536 |
| UNT 136 IN SBBC | 0.91388 | 1303 | 963 | 1762 |
| UNT 136 IN SBBC | 1.174207 | 1597 | 1181 | 2159 |
| UNT 136 IN SBBC | 1.395704 | 1817 | 1344 | 2457 |
| UNT 136 IN SBBC | 1.607607 | 1958 | 1447 | 2647 |
| UNT 137 IN SBBC | 0.22374 | 314 | 232 | 424 |
| UNT 138 IN SBBC | 0.166796 | 234 | 173 | 317 |
| UNT 139 IN SBBC | 0.135499 | 154 | 114 | 208 |
| UNT 140 IN SBBC | 0.329915 | 430 | 318 | 581 |
| UNT 141 IN SBBC | 0.171011 | 217 | 160 | 293 |
| UNT 142 IN SBBC | 0.127428 | 233 | 172 | 315 |
| UNT 143 IN SBBC | 0.127904 | 176 | 130 | 238 |
| UNT 144 IN SBBC | 0.342492 | 443 | 327 | 599 |
| UNT 144 IN SBBC | 0.48335 | 613 | 453 | 829 |
| UNT 144 IN SBBC | 0.49958 | 629 | 465 | 851 |
| UNT 146 IN SBBC | 0.332426 | 550 | 407 | 744 |
| UNT 147 IN SBBC | 0.254588 | 422 | 312 | 570 |
| UNT 147 IN SBBC | 0.44869 | 684 | 506 | 925 |
| UNT 147 IN SBBC | 0.52998 | 740 | 547 | 1001 |
| UNT 148 IN SBBC | 0.068422 | 132 | 98 | 179 |
| UNT 150 IN SBBC | 0.232897 | 454 | 335 | 614 |
| UNT 152 IN SBBC | 0.124783 | 356 | 263 | 481 |
| UNT 153 IN SBBC | 0.228636 | 502 | 371 | 679 |
| UNT 154 IN SBBC | 0.187511 | 525 | 388 | 710 |
| UNT 155 IN SBBC | 0.429096 | 822 | 608 | 1112 |
| UNT 155 IN SBBC | 0.858504 | 1352 | 1000 | 1829 |
| UNT 156 IN SBBC | 0.187825 | 396 | 293 | 536 |
| UNT 157 IN SBBC | 0.752549 | 810 | 599 | 1096 |
| UNT 157 IN SBBC | 0.823573 | 874 | 647 | 1183 |
| UNT 157 IN SBBC | 1.492458 | 1304 | 964 | 1763 |
| UNT 158 IN SBBC | 0.068288 | 82 | 61 | 111 |
| UNT 159 IN SBBC | 0.296275 | 474 | 351 | 641 |
| UNT 159 IN SBBC | 0.398765 | 611 | 452 | 826 |
| UNT 160 IN SBBC | 0.09404 | 224 | 166 | 303 |
| UNT 161 IN SBBC | 0.702917 | 986 | 729 | 1333 |
| UNT 161 IN SBBC | 1.200033 | 1660 | 1228 | 2246 |
| UNT 162 IN SBBC | 0.393053 | 631 | 467 | 854 |
| UNT 163 IN SBBC | 0.455998 | 660 | 488 | 893 |
| UNT 164 IN SBBC | 0.287004 | 422 | 312 | 571 |
| UNT 165 IN SBBC | 0.418155 | 615 | 455 | 832 |
| UNT 165 IN SBBC | 0.693054 | 917 | 678 | 1241 |
| UNT 165 IN SBBC | 0.890205 | 1135 | 839 | 1535 |
| UNT 165 IN SBBC | 0.921374 | 1135 | 839 | 1535 |
| UNT 165 IN SBBC | 1.73629 | 1832 | 1355 | 2478 |
| UNT 165 IN SBBC | 2.06782 | 2132 | 1576 | 2883 |
| UNT 165 IN SBBC | 2.583183 | 2409 | 1781 | 3257 |
| UNT 166 IN SBBC | 0.202397 | 331 | 245 | 448 |
| UNT 167 IN SBBC | 0.124155 | 320 | 236 | 432 |
| UNT 168 IN SBBC | 0.335693 | 577 | 426 | 780 |
| UNT 169 IN SBBC | 0.146171 | 312 | 231 | 422 |
| UNT 170 IN SBBC | 0.239881 | 432 | 319 | 584 |
| UNT 171 IN SBBC | 0.126173 | 187 | 138 | 253 |
| UNT 172 IN SBBC | 0.354486 | 458 | 339 | 620 |
| UNT 173 IN SBBC | 0.540474 | 835 | 617 | 1129 |
| UNT 175 IN SBBC | 0.421563 | 811 | 600 | 1097 |
| UNT 176 IN SBBC | 1.000506 | 1431 | 1058 | 1936 |
| UNT 178 IN SBBC | 0.714442 | 827 | 611 | 1118 |
| UNT 179 IN SBBC | 0.268577 | 444 | 328 | 601 |
| UNT 179 IN SBBC | 0.482811 | 683 | 505 | 924 |
| UNT 179 IN SBBC | 0.731839 | 935 | 691 | 1265 |
| UNT 180 IN SBBC | 0.122317 | 268 | 198 | 363 |
| UNT 181 IN SBBC | 0.126083 | 198 | 146 | 268 |
| UNT 182 IN SBBC | 0.580264 | 962 | 711 | 1301 |
| UNT 182 IN SBBC | 1.32304 | 1858 | 1374 | 2513 |
| UNT 182 IN SBBC | 1.540687 | 2090 | 1545 | 2826 |
| UNT 183 IN SBBC | 0.231032 | 371 | 275 | 502 |
| UNT 183 IN SBBC | 0.460213 | 690 | 510 | 933 |
| UNT 184 IN SBBC | 0.227036 | 479 | 354 | 647 |
| UNT 185 IN SBBC | 0.204464 | 461 | 341 | 623 |
| UNT 186 IN SBBC | 0.37566 | 646 | 478 | 874 |
| UNT 186 IN SBBC | 0.544159 | 906 | 670 | 1225 |
| UNT 186 IN SBBC | 0.662889 | 1073 | 793 | 1451 |
| UNT 187 IN SBBC | 0.147964 | 408 | 302 | 551 |
| UNT 188 IN SBBC | 0.101692 | 311 | 230 | 420 |
| UNT 189 IN SBBC | 0.876424 | 1009 | 746 | 1365 |
| UNT 190 IN SBBC | 0.316553 | 584 | 432 | 789 |
| UNT 191 IN SBBC | 0.374096 | 514 | 380 | 695 |
| UNT 193 IN SBBC | 0.958886 | 1061 | 785 | 1435 |
| UNT 194 IN SBBC | 0.568451 | 785 | 581 | 1062 |
| UNT 195 IN SBBC | 0.329181 | 488 | 361 | 660 |
| UNT 196 IN SBBC | 0.696059 | 913 | 675 | 1235 |
| UNT 197 IN SBBC | 0.305972 | 389 | 288 | 527 |
| UNT 197 IN SBBC | 0.651131 | 812 | 600 | 1098 |
| UNT 197 IN SBBC | 0.926255 | 1043 | 771 | 1411 |
| UNT 198 IN SBBC | 0.282208 | 392 | 290 | 530 |
| UNT 199 IN SBBC | 0.183924 | 345 | 255 | 467 |
| UNT 200 IN SBBC | 0.145335 | 304 | 225 | 411 |
| UNT 201 IN SBBC | 0.388204 | 565 | 418 | 764 |
| UNT 202 IN SBBC | 0.234142 | 469 | 347 | 635 |
| UNT 202 IN SBBC | 0.411402 | 800 | 592 | 1082 |
| UNT 203 IN SBBC | 0.163361 | 307 | 227 | 416 |
| UNT 204 IN SBBC | 0.777573 | 1010 | 747 | 1365 |
| UNT 205 IN SBBC | 0.478552 | 772 | 571 | 1044 |
| UNT 205 IN SBBC | 0.916525 | 1409 | 1042 | 1906 |
| UNT 205 IN SBBC | 1.624869 | 2122 | 1569 | 2870 |
| UNT 205 IN SBBC | 3.564534 | 4048 | 2993 | 5474 |
| UNT 205 IN SBBC | 4.286428 | 4519 | 3342 | 6112 |
| UNT 206 IN SBBC | 0.431158 | 1019 | 753 | 1378 |
| UNT 207 IN SBBC | 0.139355 | 449 | 332 | 607 |
| UNT 208 IN SBBC | 1.378576 | 2974 | 2199 | 4022 |
| UNT 208 IN SBBC | 1.584051 | 2974 | 2199 | 4022 |
| UNT 208 IN SBBC | 1.802291 | 2974 | 2199 | 4022 |
| UNT 209 IN SBBC | 0.446771 | 700 | 518 | 947 |
| UNT 211 IN SBBC | 0.2714 | 501 | 370 | 677 |
| UNT 212 IN SBBC | 0.239874 | 348 | 257 | 471 |
| UNT 213 IN SBBC | 0.537572 | 757 | 559 | 1023 |
| UNT 214 IN SBBC | 0.206701 | 496 | 367 | 671 |
| UNT 215 IN SBBC | 1.002165 | 1258 | 931 | 1702 |
| UNT 215 IN SBBC | 1.095517 | 1321 | 977 | 1786 |
| UNT 216 IN SBBC | 0.156124 | 340 | 251 | 460 |
| UNT 217 IN SBBC | 0.194021 | 416 | 307 | 562 |
| UNT 219 IN SBBC | 0.421214 | 754 | 558 | 1020 |
| UNT 220 IN SBBC | 0.463531 | 1033 | 764 | 1398 |
| UNT 220 IN SBBC | 0.978221 | 1786 | 1321 | 2416 |
| UNT 221 IN SBBC | 0.149802 | 229 | 170 | 310 |
| UNT 222 IN SBBC | 0.17267 | 347 | 256 | 469 |
| UNT 224 IN SBBC | 0.213103 | 494 | 365 | 668 |
| UNT 225 IN SBBC | 0.361339 | 522 | 386 | 706 |
| UNT 225 IN SBBC | 1.056274 | 1264 | 935 | 1710 |
| UNT 226 IN SBBC | 0.354073 | 589 | 435 | 796 |
| UNT 227 IN SBBC | 0.228492 | 368 | 272 | 498 |
| UNT 228 IN SBBC | 0.914597 | 962 | 712 | 1301 |
| UNT 229 IN SBBC | 0.528456 | 855 | 632 | 1156 |
| UNT 229 IN SBBC | 0.866706 | 1185 | 876 | 1602 |
| UNT 229 IN SBBC | 1.400321 | 1845 | 1365 | 2496 |
| UNT 230 IN SBBC | 0.196747 | 356 | 263 | 481 |
| UNT 231 IN SBBC | 0.532894 | 779 | 576 | 1054 |
| UNT 232 IN SBBC | 0.397709 | 655 | 485 | 886 |
| UNT 233 IN SBBC | 0.675254 | 895 | 661 | 1210 |
| UNT 234 IN SBBC | 1.404717 | 2320 | 1716 | 3138 |
| UNT 234 IN SBBC | 1.704259 | 2525 | 1867 | 3415 |
| UNT 235 IN SBBC | 0.34368 | 891 | 659 | 1205 |
| UNT 235 IN SBBC | 0.462693 | 1160 | 858 | 1569 |
| UNT 236 IN SBBC | 0.113812 | 363 | 269 | 492 |
| UNT 238 IN SBBC | 0.43975 | 822 | 608 | 1111 |
| UNT 239 IN SBBC | 0.217731 | 379 | 281 | 513 |
| UNT 240 IN SBBC | 0.252884 | 479 | 354 | 647 |
| UNT 241 IN SBBC | 0.136172 | 234 | 173 | 316 |
| UNT 242 IN SBBC | 0.914687 | 1243 | 919 | 1681 |
| UNT 243 IN SBBC | 0.292431 | 641 | 474 | 866 |
| UNT 244 IN SBBC | 1.0188 | 1244 | 920 | 1682 |
| UNT 244 IN SBBC | 2.257034 | 2233 | 1651 | 3020 |
| UNT 244 IN SBBC | 4.339068 | 3268 | 2416 | 4420 |
| UNT 244 IN SBBC | 4.450983 | 3268 | 2416 | 4420 |
| UNT 245 IN SBBC | 1.105472 | 1195 | 884 | 1616 |
| UNT 245 IN SBBC | 1.718312 | 1654 | 1223 | 2237 |
| UNT 246 IN SBBC | 0.18778 | 283 | 209 | 382 |
| UNT 247 IN SBBC | 0.531953 | 810 | 599 | 1095 |
| UNT 247 IN SBBC | 1.174386 | 1472 | 1089 | 1991 |
| UNT 248 IN SBBC | 0.262748 | 434 | 321 | 587 |
| UNT 250 IN SBBC | 0.130298 | 308 | 227 | 416 |
| UNT 251 IN SBBC | 0.57645 | 1094 | 809 | 1480 |
| UNT 252 IN SBBC | 0.227237 | 409 | 302 | 552 |
| UNT 253 IN SBBC | 0.201321 | 396 | 293 | 536 |
| UNT 254 IN SBBC | 0.326104 | 571 | 423 | 773 |
| UNT 254 IN SBBC | 0.355786 | 622 | 460 | 841 |
| UNT 255 IN SBBC | 0.230241 | 490 | 362 | 662 |
| UNT 255 IN SBBC | 0.29113 | 538 | 397 | 727 |
| UNT 257 IN SBBC | 0.420846 | 886 | 655 | 1198 |
| UNT 258 IN SBBC | 0.552915 | 809 | 598 | 1094 |
| UNT 258 IN SBBC | 1.240992 | 1508 | 1115 | 2040 |
| UNT 258 IN SBBC | 1.775355 | 1929 | 1426 | 2609 |
| UNT 258 IN SBBC | 1.878623 | 1955 | 1445 | 2643 |
| UNT 258 IN SBBC | 2.534814 | 2409 | 1781 | 3258 |
| UNT 259 IN SBBC | 0.350002 | 554 | 409 | 749 |
| UNT 260 IN SBBC | 0.34776 | 643 | 475 | 869 |
| UNT 261 IN SBBC | 0.291534 | 536 | 396 | 725 |
| UNT 261 IN SBBC | 0.561726 | 944 | 698 | 1277 |
| UNT 261 IN SBBC | 0.598862 | 996 | 736 | 1347 |
| UNT 262 IN SBBC | 0.141137 | 289 | 214 | 391 |
| UNT 263 IN SBBC | 0.142314 | 298 | 221 | 403 |
| UNT 264 IN SBBC | 0.70386 | 1238 | 915 | 1674 |
| UNT 264 IN SBBC | 1.33424 | 2009 | 1485 | 2717 |
| UNT 264 IN SBBC | 1.893723 | 2600 | 1922 | 3516 |
| UNT 265 IN SBBC | 0.362616 | 654 | 483 | 884 |
| UNT 266 IN SBBC | 0.389101 | 704 | 521 | 953 |
| UNT 267 IN SBBC | 0.41412 | 661 | 489 | 894 |
| UNT 267 IN SBBC | 1.383329 | 1766 | 1306 | 2389 |
| UNT 268 IN SBBC | 0.326507 | 609 | 451 | 824 |
| UNT 269 IN SBBC | 1.001582 | 1122 | 830 | 1517 |
| UNT 269 IN SBBC | 2.108891 | 1684 | 1246 | 2278 |
| UNT 270 IN SBBC | 0.582419 | 1297 | 719 | 1753 |
| UNT 270 IN SBBC | 0.679958 | 1297 | 959 | 1753 |
| UNT 270 IN SBBC | 0.680209 | 1821 | 1347 | 2463 |
| UNT 270 IN SBBC | 1.311133 | 1821 | 1347 | 2463 |
| UNT 271 IN SBBC | 0.089889 | 218 | 161 | 295 |
| UNT 271 IN SBBC | 0.203652 | 416 | 307 | 562 |
| UNT 272 IN SBBC | 0.67902 | 837 | 619 | 1132 |
| UNT 273 IN SBBC | 0.428647 | 627 | 464 | 849 |
| UNT 274 IN SBBC | 0.89155 | 1823 | 1348 | 2465 |
| UNT 275 IN SBBC | 0.859208 | 494 | 365 | 668 |
| UNT 276 IN SBBC | 0.744752 | 1115 | 824 | 1508 |
| UNT 277 IN SBBC | 0.411878 | 816 | 603 | 1104 |
| UNT 278 IN SBBC | 0.241944 | 569 | 420 | 769 |
| UNT 279 IN SBBC | 0.386769 | 548 | 405 | 741 |
| UNT 279 IN SBBC | 0.956475 | 1282 | 948 | 1734 |
| UNT 280 IN SBBC | 0.899173 | 977 | 722 | 1321 |
| UNT 281 IN SBBC | 0.346236 | 657 | 486 | 888 |
| UNT 283 IN SBBC | 0.193287 | 389 | 288 | 526 |
| UNT 284 IN SBBC | 1.047765 | 1392 | 1030 | 1883 |
| UNT 284 IN SBBC | 1.758186 | 2058 | 1522 | 2783 |
| UNT 285 IN SBBC | 0.512314 | 917 | 678 | 1240 |
| UNT 286 IN SBBC | 0.423028 | 614 | 454 | 830 |
| UNT 287 IN SBBC | 0.304447 | 496 | 367 | 671 |
| UNT 288 IN SBBC | 0.229387 | 292 | 216 | 395 |
| UNT 289 IN SBBC | 1.283879 | 1575 | 1164 | 2129 |
| UNT 289 IN SBBC | 1.50995 | 1776 | 1313 | 2401 |
| UNT 289 IN SBBC | 1.749293 | 2063 | 1526 | 2791 |
| UNT 289 IN SBBC | 2.308508 | 2483 | 1836 | 3358 |
| UNT 290 IN SBBC | 0.188408 | 499 | 369 | 675 |
| UNT 291 IN SBBC | 0.10223 | 326 | 241 | 441 |
| UNT 292 IN SBBC | 0.187959 | 502 | 371 | 679 |
| UNT 294 IN SBBC | 1.340913 | 1806 | 1335 | 2442 |
| UNT 294 IN SBBC | 1.798313 | 2131 | 1576 | 2882 |
| UNT 295 IN SBBC | 1.242332 | 1598 | 1182 | 2161 |
| UNT 297 IN SBBC | 1.120401 | 1354 | 1001 | 1831 |
| UNT 297 IN SBBC | 1.456056 | 1608 | 1189 | 2175 |
| UNT 298 IN SBBC | 0.976652 | 1134 | 839 | 1534 |
| UNT 298 IN SBBC | 1.780321 | 1785 | 1320 | 2414 |
| UNT 300 IN SBBC | 1.175103 | 1404 | 1038 | 1899 |
| UNT 304 IN SBBC | 0.315388 | 483 | 357 | 653 |
| UNT 307 IN SBBC | 0.185717 | 361 | 267 | 488 |
| UNT 308 IN SBBC | 0.540741 | 943 | 697 | 1275 |
| UNT 309 IN SBBC | 0.601473 | 767 | 567 | 1037 |
| UNT 309 IN SBBC | 0.899442 | 994 | 735 | 1344 |
| UNT 311 IN SBBC | 0.201769 | 461 | 341 | 623 |
| UNT 311 IN SBBC | 0.358252 | 739 | 546 | 999 |
| UNT 311 IN SBBC | 1.053773 | 1390 | 1028 | 1880 |
| UNT 311 IN SBBC | 1.346069 | 1590 | 1176 | 2150 |
| UNT 311 IN SBBC | 1.590041 | 1685 | 1246 | 2279 |
| UNT 311 IN SBBC | 1.790376 | 1847 | 1366 | 2498 |
| UNT 312 IN SBBC | 0.133616 | 302 | 224 | 409 |
| UNT 313 IN SBBC | 0.356459 | 613 | 453 | 829 |
| UNT 314 IN SBBC | 0.168096 | 358 | 265 | 485 |
| UNT 315 IN SBBC | 0.216442 | 429 | 317 | 580 |
| UNT 316 IN SBBC | 0.184462 | 312 | 231 | 422 |
| UNT 317 IN SBBC | 0.336307 | 554 | 410 | 750 |
| UNT 317 IN SBBC | 0.709714 | 950 | 703 | 1285 |
| UNT 317 IN SBBC | 0.893996 | 1145 | 847 | 1549 |
| UNT 318 IN SBBC | 0.177736 | 304 | 225 | 411 |
| UNT 321 IN SBBC | 0.551951 | 758 | 560 | 1025 |
| UNT 321 IN SBBC | 0.860674 | 1010 | 747 | 1366 |
| UNT 326 IN SBBC | 0.219614 | 301 | 222 | 407 |
| UNT 327 IN SBBC | 0.079076 | 124 | 92 | 167 |
| UNT 328 IN SBBC | 0.034615 | 85 | 63 | 114 |
| UNT 329 IN SBBC | 0.101692 | 169 | 125 | 229 |
| UNT 330 IN SBBC | 0.05255 | 68 | 51 | 93 |
| UNT 8507 IN SBBC | 0.071859 | 164 | 121 | 221 |
| UNT 900 IN SBBC | 0.089168 | 214 | 158 | 290 |
| UNT 901 IN SBBC | 0.058805 | 122 | 90 | 165 |
| UNT 902 IN SBBC | 0.086716 | 105 | 78 | 142 |
| UNT 903 IN SBBC | 0.153526 | 391 | 289 | 529 |
| UNT 904 IN SBBC | 0.190834 | 324 | 240 | 439 |
| UNT 905 IN SBBC | 0.173252 | 335 | 248 | 453 |
| UNT 909 IN SBBC | 0.123214 | 271 | 200 | 366 |
| UNT 910 IN SBBC | 0.241351 | 417 | 308 | 564 |
| UNT 912 IN SBBC | 0.092186 | 208 | 154 | 281 |
| UNT 913 IN SBBC | 0.07715 | 179 | 132 | 241 |
| UNT 914 IN SBBC | 0.084174 | 226 | 167 | 305 |
| WESTMORELAND BRANCH | 1.796788 | 1144 | 846 | 1547 |
| WESTMORELAND BRANCH | 3.655469 | 1928 | 1426 | 2608 |
| WHITE OAK BRANCH | 0.423074 | 1011 | 748 | 1368 |
| WHITE OAK BRANCH | 1.281566 | 2287 | 1691 | 3093 |
| WHITE OAK BRANCH | 3.157462 | 4025 | 2976 | 5443 |
| WHITE OAK BRANCH | 3.606449 | 4609 | 3408 | 6233 |
| WHITE OAK BRANCH | 3.827857 | 4782 | 3536 | 6467 |
| WHITE OAK BRANCH | 4.291792 | 4962 | 3669 | 6710 |
| WHITE OAK BRANCH | 4.540685 | 5066 | 3746 | 6852 |
| WILLOW BRANCH | 1.065344 | 1464 | 1083 | 1980 |
| Trinity River |  |  |  |  |
| TRINITY RIVER | 29.44022 | 188090 | 139080 | 254369 |
| TRINITY RIVER | 99.23102 | 188090 | 139080 | 254369 |
| TRINITY RIVER | 131.79655 | 188090 | 139080 | 254369 |
| TRINITY RIVER | 176.45656 | 188090 | 139080 | 254369 |
| TRINITY RIVER | 364.85466 | 215708 | 159502 | 291721 |
| TRINITY RIVER | 516.46517 | 230106 | 170149 | 311192 |
| TRINITY RIVER | 545.79035 | 230106 | 170149 | 311192 |
| TRINITY RIVER | 648.44279 | 230106 | 170149 | 311192 |
| TRINITY RIVER | 762.43578 | 230106 | 170149 | 311192 |
| TRINITY RIVER | 1369.18213 | 255074 | 188610 | 344958 |
| TRINITY RIVER | 1425.65843 | 256110 | 189377 | 346359 |
| TRINITY RIVER | 1516.7933 | 260370 | 192527 | 352120 |
| TRINITY RIVER | 1693.43997 | 267602 | 197874 | 361900 |
| TRINITY RIVER | 1761.805 | 270840 | 200268 | 366279 |
| TRINITY RIVER | 2272.05851 | 270840 | 200268 | 366279 |
| TRINITY RIVER | 2389.43324 | 270840 | 200268 | 366279 |
| TRINITY RIVER | 2550.9761 | 274081 | 202665 | 370662 |
| TRINITY RIVER | 2616.83461 | 276631 | 204551 | 374112 |
| TRINITY RIVER | 2653.32837 | 278036 | 205590 | 376012 |
| TRINITY RIVER | 2685.83499 | 279382 | 206585 | 377831 |
| TRINITY RIVER | 2719.25674 | 280155 | 207156 | 378877 |
| TRINITY RIVER | 2945.36476 | 280155 | 207156 | 378877 |
| TRINITY RIVER | 2977.63661 | 280155 | 207156 | 378877 |
| TRINITY RIVER | 3160.03358 | 280155 | 207156 | 378877 |
| TRINITY RIVER | 3256.64285 | 280155 | 207156 | 378877 |
| Wright Creek - Trinity River |  |  |  |  |
| BLACK CREEK | 1.10265 | 1861 | 1376 | 2517 |
| BLACK CREEK | 2.27452 | 2966 | 2193 | 4011 |
| BLACK CREEK | 2.81316 | 3183 | 2354 | 4305 |
| BLACK CREEK | 3.37555 | 3562 | 2634 | 4818 |
| BLACK CREEK | 6.3603 | 5962 | 4408 | 8063 |
| BLACK CREEK | 6.72618 | 6023 | 4454 | 8146 |
| CEDAR CREEK | 0.92132 | 937 | 693 | 1267 |
| CEDAR CREEK | 1.76517 | 1474 | 1090 | 1993 |
| CEDAR CREEK | 2.87283 | 2137 | 1580 | 2890 |
| CEDAR CREEK | 3.52388 | 2261 | 1672 | 3057 |
| CEDAR CREEK | 4.52169 | 2636 | 1949 | 3565 |
| CEDAR CREEK | 4.88308 | 2636 | 1949 | 3565 |
| CEDAR LAKE BRANCH | 1.13466 | 854 | 631 | 1154 |
| CEDAR LAKE BRANCH | 1.43722 | 981 | 725 | 1326 |
| CEDAR LAKE BRANCH | 2.10898 | 1317 | 974 | 1782 |
| CEDAR LAKE BRANCH | 3.01856 | 1764 | 1304 | 2385 |
| CEDAR LAKE BRANCH | 4.00498 | 1953 | 1444 | 2642 |
| CEDAR LAKE BRANCH | 4.68149 | 2205 | 1631 | 2983 |
| CEDAR LAKE BRANCH | 5.12314 | 2272 | 1680 | 3073 |
| CEDAR LAKE BRANCH | 5.26304 | 2291 | 1694 | 3098 |
| CRABS BRANCH | 1.00015 | 811 | 599 | 1096 |
| CRABS BRANCH | 1.71154 | 1148 | 849 | 1552 |
| CRABS BRANCH | 2.08109 | 1311 | 970 | 1773 |
| EAST SULPHUR BRANCH | 1.00006 | 1443 | 1067 | 1952 |
| EAST SULPHUR BRANCH | 1.97868 | 2628 | 1943 | 3554 |
| EAST SULPHUR BRANCH | 2.09786 | 2642 | 1954 | 3574 |
| EAST SULPHUR BRANCH | 2.55574 | 3046 | 2252 | 4119 |
| EAST SULPHUR BRANCH | 2.79033 | 3218 | 2379 | 4352 |
| EAST SULPHUR BRANCH | 3.0182 | 3266 | 2415 | 4417 |
| GARDNER CREEK | 0.68808 | 610 | 451 | 825 |
| GARDNER CREEK | 1.89295 | 1389 | 1027 | 1878 |
| GARDNER CREEK | 2.29649 | 1528 | 1130 | 2066 |
| GARDNER CREEK | 2.95471 | 1799 | 1330 | 2433 |
| GARDNER CREEK | 4.25612 | 2391 | 1768 | 3234 |
| GARDNER CREEK | 6.04985 | 3057 | 2261 | 4135 |
| GARDNER CREEK | 6.7928 | 3360 | 2484 | 4544 |
| GIN CREEK | 0.33395 | 305 | 225 | 412 |
| GIN CREEK | 0.73991 | 633 | 468 | 857 |
| GIN CREEK | 1.00687 | 759 | 561 | 1026 |
| GIN CREEK | 1.09547 | 871 | 644 | 1177 |
| GIN CREEK | 1.58061 | 1122 | 830 | 1517 |
| GIN CREEK | 1.87035 | 1289 | 953 | 1743 |
| GIN CREEK | 2.5031 | 1611 | 1192 | 2179 |
| GIN CREEK | 6.33842 | 3328 | 2461 | 4500 |
| HACKETT BRANCH | 0.97782 | 810 | 599 | 1096 |
| HACKETT BRANCH | 1.14327 | 869 | 642 | 1175 |
| HACKETT BRANCH | 2.42051 | 1777 | 1314 | 2403 |
| HACKETT BRANCH | 3.33744 | 2161 | 1598 | 2923 |
| HACKETT BRANCH | 3.82671 | 2321 | 1716 | 3139 |
| HARPER BRANCH | 0.40174 | 460 | 340 | 622 |
| HARPER BRANCH | 0.84196 | 899 | 665 | 1216 |
| HARPER BRANCH | 1.54421 | 1398 | 1033 | 1890 |
| IVEY CREEK | 1.05566 | 892 | 660 | 1207 |
| IVEY CREEK | 1.86937 | 1374 | 1016 | 1859 |
| IVEY CREEK | 2.69779 | 1662 | 1229 | 2248 |
| IVEY CREEK | 3.22337 | 1884 | 1393 | 2548 |
| IVEY CREEK | 3.47509 | 1903 | 1407 | 2573 |
| IVEY CREEK | 3.84357 | 2044 | 1511 | 2764 |
| IVEY CREEK | 4.41157 | 2283 | 1688 | 3088 |
| KELLISONS CREEK | 0.76977 | 609 | 451 | 824 |
| KELLISONS CREEK | 1.76221 | 1132 | 837 | 1530 |
| KELLISONS CREEK | 2.72612 | 1694 | 1252 | 2290 |
| KELLISONS CREEK | 3.11711 | 1798 | 1330 | 2432 |
| KELLISONS CREEK | 5.59555 | 2825 | 2089 | 3821 |
| KELLISONS CREEK | 6.59202 | 3021 | 2234 | 4085 |
| KELLISONS CREEK | 7.59011 | 3344 | 2473 | 4523 |
| KELLISONS CREEK | 7.80515 | 3379 | 2498 | 4569 |
| KELLISONS CREEK | 12.70769 | 5014 | 3708 | 6781 |
| KELLISONS CREEK | 13.95157 | 5241 | 3875 | 7088 |
| KELLISONS CREEK | 14.46308 | 5364 | 3967 | 7255 |
| KELLISONS CREEK | 14.84411 | 5364 | 3967 | 7255 |
| KELLISONS CREEK | 17.05998 | 5646 | 4175 | 7636 |
| KELLISONS CREEK | 17.49347 | 5646 | 4175 | 7636 |
| KELLISONS CREEK | 17.89746 | 5717 | 4228 | 7732 |
| KELLISONS CREEK | 18.83125 | 5855 | 4330 | 7918 |
| KELLISONS CREEK | 19.06171 | 5855 | 4330 | 7918 |
| MELTON BRANCH | 0.97226 | 710 | 525 | 961 |
| MELTON BRANCH | 2.01608 | 1360 | 1006 | 1840 |
| MELTON BRANCH | 3.02815 | 1900 | 1405 | 2570 |
| MELTON BRANCH | 4.18837 | 2496 | 1846 | 3376 |
| MELTON BRANCH | 5.16009 | 3002 | 2220 | 4060 |
| MELTON BRANCH | 6.05227 | 3086 | 2282 | 4174 |
| NEGRO CREEK | 1.09484 | 797 | 589 | 1078 |
| NEGRO CREEK | 2.41773 | 1485 | 1098 | 2009 |
| NEGRO CREEK | 3.05218 | 1822 | 1347 | 2464 |
| NEGRO CREEK | 5.14493 | 2631 | 1945 | 3558 |
| NEGRO CREEK | 5.83023 | 2765 | 2044 | 3739 |
| NEGRO CREEK | 6.71945 | 3021 | 2234 | 4085 |
| NEGRO CREEK | 7.46851 | 3170 | 2344 | 4288 |
| NEGRO CREEK | 8.23182 | 3456 | 2555 | 4674 |
| NEGRO CREEK | 16.18933 | 5660 | 4185 | 7654 |
| NEGRO CREEK | 17.50024 | 5728 | 4235 | 7746 |
| NEGRO CREEK | 18.40735 | 5830 | 4311 | 7884 |
| NEGRO CREEK | 18.92688 | 5830 | 4311 | 7884 |
| NEGRO CREEK | 20.37931 | 5968 | 4413 | 8071 |
| SALT BRANCH | 1.19887 | 2009 | 1486 | 2717 |
| SALT BRANCH | 2.18628 | 2613 | 1932 | 3533 |
| SALT BRANCH | 2.90951 | 3124 | 2310 | 4225 |
| SALT BRANCH | 3.84097 | 3832 | 2834 | 5182 |
| SAND BRANCH | 0.97638 | 753 | 557 | 1018 |
| SAND BRANCH | 1.49399 | 1056 | 781 | 1429 |
| SAND BRANCH | 2.28439 | 1380 | 1021 | 1867 |
| SAND BRANCH | 2.99794 | 1695 | 1253 | 2292 |
| SAND BRANCH | 3.47115 | 1857 | 1373 | 2511 |
| SAND BRANCH | 5.4935 | 2762 | 2042 | 3735 |
| UNT 001 IN WCTR | 0.40013 | 519 | 383 | 701 |
| UNT 002 IN WCTR | 0.51787 | 595 | 440 | 804 |
| UNT 002 IN WCTR | 0.72314 | 821 | 607 | 1110 |
| UNT 004 IN WCTR | 0.27136 | 288 | 213 | 389 |
| UNT 004 IN WCTR | 0.82444 | 744 | 550 | 1007 |
| UNT 0225 IN WCTR | 0.33467 | 306 | 226 | 413 |
| UNT 0226 IN WCTR | 0.65696 | 574 | 425 | 777 |
| UNT 0227 IN WCTR | 0.55536 | 456 | 337 | 616 |
| UNT 0228 IN WCTR | 0.50622 | 495 | 366 | 669 |
| UNT 0229 IN WCTR | 0.72852 | 616 | 456 | 834 |
| UNT 0230 IN WCTR | 0.39466 | 377 | 279 | 510 |
| UNT 0231 IN WCTR | 0.76726 | 539 | 399 | 729 |
| UNT 0345 IN WCTR | 1.22864 | 981 | 726 | 1327 |
| UNT 0348 IN WCTR | 1.16667 | 1019 | 753 | 1378 |
| UNT 0352 IN WCTR | 0.7843 | 667 | 493 | 902 |
| UNT 0354 IN WCTR | 0.9987 | 797 | 590 | 1078 |
| UNT 0355 IN WCTR | 1.08758 | 705 | 521 | 953 |
| UNT 0355 IN WCTR | 1.32952 | 759 | 561 | 1026 |
| UNT 0362 IN WCTR | 1.00293 | 247 | 183 | 334 |
| UNT 0362 IN WCTR | 1.97877 | 415 | 307 | 562 |
| UNT 0362 IN WCTR | 2.99107 | 503 | 372 | 680 |
| UNT 0362 IN WCTR | 3.97611 | 731 | 540 | 988 |
| UNT 0362 IN WCTR | 4.00615 | 740 | 547 | 1001 |
| UNT 0362 IN WCTR | 6.44513 | 1085 | 803 | 1468 |
| UNT 0362 IN WCTR | 8.53968 | 1398 | 1034 | 1891 |
| UNT 0362 IN WCTR | 8.75032 | 1437 | 1063 | 1944 |
| UNT 0362 IN WCTR | 9.09163 | 1437 | 1063 | 1944 |
| UNT 0363 IN WCTR | 0.9936 | 841 | 622 | 1138 |
| UNT 0363 IN WCTR | 2.33559 | 1398 | 1034 | 1891 |
| UNT 0364 IN WCTR | 1.00158 | 729 | 539 | 986 |
| UNT 0364 IN WCTR | 1.01808 | 730 | 540 | 987 |
| UNT 0365 IN WCTR | 1.59577 | 933 | 690 | 1262 |
| UNT 0368 IN WCTR | 1.4774 | 2049 | 1515 | 2771 |
| UNT 0368 IN WCTR | 2.47046 | 2807 | 2076 | 3796 |
| UNT 0368 IN WCTR | 2.79329 | 2965 | 2193 | 4010 |
| UNT 0373 IN WCTR | 0.43537 | 646 | 478 | 874 |
| UNT 0373 IN WCTR | 0.56585 | 717 | 530 | 969 |
| UNT 0373 IN WCTR | 1.36737 | 1406 | 1040 | 1902 |
| UNT 0373 IN WCTR | 2.34832 | 2103 | 1555 | 2844 |
| UNT 0373 IN WCTR | 2.78881 | 2394 | 1770 | 3237 |
| UNT 0376 IN WCTR | 1.02436 | 1073 | 793 | 1451 |
| UNT 0376 IN WCTR | 1.60626 | 1557 | 1151 | 2105 |
| UNT 0376 IN WCTR | 4.42314 | 3637 | 2689 | 4918 |
| UNT 0376 IN WCTR | 5.39925 | 3988 | 2949 | 5393 |
| UNT 0376 IN WCTR | 6.10921 | 4174 | 3087 | 5645 |
| UNT 0426 IN WCTR | 1.05094 | 1578 | 1167 | 2134 |
| UNT 0427 IN WCTR | 0.21746 | 544 | 402 | 735 |
| UNT 0428 IN WCTR | 0.22589 | 588 | 435 | 795 |
| UNT 0429 IN WCTR | 0.16769 | 426 | 315 | 576 |
| UNT 0430 IN WCTR | 0.22114 | 575 | 425 | 777 |
| UNT 0431 IN WCTR | 0.64225 | 551 | 407 | 745 |
| UNT 0432 IN WCTR | 0.76771 | 641 | 474 | 867 |
| UNT 0433 IN WCTR | 0.60002 | 507 | 375 | 686 |
| UNT 0434 IN WCTR | 0.32713 | 384 | 284 | 519 |
| UNT 0435 IN WCTR | 0.13774 | 172 | 127 | 233 |
| UNT 0436 IN WCTR | 0.35206 | 381 | 282 | 516 |
| UNT 0437 IN WCTR | 0.51382 | 515 | 381 | 696 |
| UNT 0437 IN WCTR | 1.04828 | 936 | 692 | 1266 |
| UNT 0437 IN WCTR | 1.27067 | 1049 | 776 | 1418 |
| UNT 0438 IN WCTR | 0.75435 | 730 | 540 | 988 |
| UNT 0439 IN WCTR | 0.16106 | 216 | 159 | 292 |
| UNT 0439 IN WCTR | 0.50424 | 692 | 512 | 936 |
| UNT 0440 IN WCTR | 0.35942 | 358 | 265 | 485 |
| UNT 0588 IN WCTR | 0.75749 | 647 | 478 | 875 |
| UNT 0588 IN WCTR | 1.76768 | 1291 | 955 | 1746 |
| UNT 0590 IN WCTR | 1.16102 | 949 | 702 | 1284 |
| UNT 0590 IN WCTR | 1.40189 | 1144 | 846 | 1547 |
| UNT 0590 IN WCTR | 1.89977 | 1376 | 1017 | 1861 |
| UNT 0591 IN WCTR | 0.97728 | 879 | 650 | 1189 |
| UNT 0592 IN WCTR | 1.05359 | 876 | 648 | 1185 |
| UNT 0594 IN WCTR | 0.96598 | 677 | 500 | 915 |
| UNT 0594 IN WCTR | 2.06934 | 1344 | 994 | 1817 |
| UNT 0594 IN WCTR | 3.65579 | 2324 | 1718 | 3143 |
| UNT 0594 IN WCTR | 4.52851 | 2933 | 2169 | 3967 |
| UNT 0594 IN WCTR | 4.72068 | 3037 | 2246 | 4107 |
| UNT 0595 IN WCTR | 1.07924 | 636 | 470 | 860 |
| UNT 0600 IN WCTR | 0.32238 | 419 | 309 | 566 |
| UNT 0601 IN WCTR | 0.20141 | 299 | 221 | 404 |
| UNT 0610 IN WCTR | 0.14698 | 239 | 177 | 323 |
| UNT 0611 IN WCTR | 0.13111 | 198 | 147 | 268 |
| UNT 0613 IN WCTR | 0.32184 | 413 | 306 | 559 |
| UNT 0614 IN WCTR | 0.12528 | 317 | 234 | 428 |
| UNT 0616 IN WCTR | 0.46541 | 492 | 364 | 665 |
| UNT 0617 IN WCTR | 0.11936 | 232 | 172 | 314 |
| UNT 0618 IN WCTR | 0.09703 | 257 | 190 | 347 |
| UNT 0619 IN WCTR | 0.07335 | 260 | 192 | 351 |
| UNT 0620 IN WCTR | 0.06259 | 202 | 149 | 273 |
| UNT 0621 IN WCTR | 0.34794 | 338 | 250 | 456 |
| UNT 0622 IN WCTR | 0.37726 | 352 | 260 | 475 |
| UNT 0623 IN WCTR | 0.1928 | 298 | 220 | 403 |
| UNT 0624 IN WCTR | 0.12043 | 192 | 142 | 260 |
| WEST SULPHUR BRANCH | 0.95477 | 1531 | 1132 | 2070 |
| WEST SULPHUR BRANCH | 1.33383 | 1904 | 1408 | 2574 |
| WHITE OAK CREEK | 1.2594 | 1981 | 1465 | 2680 |
| WHITE OAK CREEK | 2.34985 | 3361 | 2485 | 4546 |
| WHITE OAK CREEK | 2.46714 | 3419 | 2528 | 4623 |
| WHITE OAK CREEK | 2.73733 | 3500 | 2588 | 4734 |
| WRIGHT CREEK | 1.23366 | 985 | 729 | 1333 |
| WRIGHT CREEK | 1.58079 | 1048 | 775 | 1417 |
| WRIGHT CREEK | 2.32573 | 1416 | 1047 | 1915 |
| WRIGHT CREEK | 4.12107 | 2394 | 1770 | 3237 |
| WRIGHT CREEK | 4.96379 | 2726 | 2016 | 3687 |
| WRIGHT CREEK | 5.54695 | 2824 | 2088 | 3819 |
| WRIGHT CREEK | 6.44307 | 3145 | 2326 | 4254 |
| WRIGHT CREEK | 7.09492 | 3359 | 2484 | 4543 |
| WRIGHT CREEK | 7.29391 | 3359 | 2484 | 4543 |
| WRIGHT CREEK | 13.00837 | 5073 | 3751 | 6860 |
| WRIGHT CREEK | 14.11505 | 5522 | 4083 | 7468 |
| WRIGHT CREEK | 16.11339 | 5834 | 4314 | 7889 |
| WRIGHT CREEK | 17.71748 | 6418 | 4746 | 8679 |
| WRIGHT CREEK | 22.57061 | 7980 | 5900 | 10792 |
| WRIGHT CREEK | 23.60699 | 8297 | 6135 | 11221 |
| WRIGHT CREEK | 24.51809 | 8297 | 6135 | 11221 |
| WRIGHT CREEK | 31.57561 | 10347 | 7651 | 13993 |
| WRIGHT CREEK | 33.98151 | 11221 | 8297 | 15175 |
| WRIGHT CREEK | 34.95439 | 11345 | 8389 | 15343 |
| WRIGHT CREEK | 36.04556 | 11384 | 8418 | 15396 |
| WRIGHT CREEK | 36.74664 | 11384 | 8418 | 15396 |
| YOUNGS CREEK | 1.19187 | 681 | 504 | 922 |
| YOUNGS CREEK | 1.81323 | 848 | 627 | 1146 |
| YOUNGS CREEK | 4.38709 | 2210 | 1634 | 2988 |
| YOUNGS CREEK | 5.18421 | 2377 | 1758 | 3214 |
| YOUNGS CREEK | 10.47011 | 3992 | 2952 | 5399 |
| YOUNGS CREEK | 15.99527 | 5499 | 4066 | 7436 |
| YOUNGS CREEK | 16.45503 | 5510 | 4074 | 7452 |
| YOUNGS CREEK | 23.46512 | 7190 | 5317 | 9724 |
| YOUNGS CREEK | 33.55761 | 8856 | 6549 | 11977 |
| YOUNGS CREEK | 34.43338 | 8930 | 6603 | 12077 |
| YOUNGS CREEK | 36.48712 | 8965 | 6629 | 12125 |
| White Rock Creek |  |  |  |  |
| ARNOLD CREEK | 1.02831 | 1065 | 788 | 1441 |
| ARNOLD CREEK | 1.71133 | 1479 | 1093 | 2000 |
| ARNOLD CREEK | 1.97797 | 1638 | 1211 | 2216 |
| ARNOLD CREEK | 2.19588 | 1709 | 1264 | 2312 |
| ARNOLD CREEK | 4.22398 | 2942 | 2176 | 3979 |
| ARNOLD CREEK | 4.8325 | 3122 | 2308 | 4222 |
| BEAVER CREEK | 0.75874 | 832 | 615 | 1125 |
| BEAVER CREEK | 1.56976 | 1397 | 1033 | 1890 |
| BEAVER CREEK | 2.15737 | 1670 | 1235 | 2259 |
| BEAVER CREEK | 3.03066 | 2115 | 1564 | 2861 |
| BEAVER CREEK | 3.76842 | 2375 | 1756 | 3212 |
| BEAVER CREEK | 4.3279 | 2528 | 1870 | 3419 |
| BEAVER CREEK | 4.99903 | 2732 | 2020 | 3695 |
| BEAVER CREEK | 5.31653 | 2840 | 2100 | 3841 |
| BEAVER CREEK | 5.63895 | 2949 | 2181 | 3989 |
| BEAVER CREEK | 5.92466 | 2996 | 2215 | 4052 |
| BEAVER CREEK | 6.54235 | 3200 | 2366 | 4327 |
| BIG BRANCH | 1.03566 | 1070 | 791 | 1447 |
| BIG BRANCH | 2.03706 | 1531 | 1132 | 2071 |
| BIG BRANCH | 2.79209 | 1872 | 1384 | 2532 |
| BOX CREEK | 0.98392 | 942 | 697 | 1274 |
| BOX CREEK | 1.7466 | 1474 | 1090 | 1994 |
| BOX CREEK | 2.29853 | 1709 | 1263 | 2311 |
| BOX CREEK | 3.13693 | 2183 | 1614 | 2953 |
| BOX CREEK | 3.78949 | 4769 | 3526 | 6449 |
| BOX CREEK | 4.32239 | 2753 | 2036 | 3724 |
| BOX CREEK | 5.83534 | 3430 | 2536 | 4639 |
| BOX CREEK | 6.45834 | 3572 | 2641 | 4831 |
| BOX CREEK | 6.8322 | 3661 | 2707 | 4951 |
| BOX CREEK | 7.41954 | 3710 | 2743 | 5017 |
| BOX CREEK | 8.9598 | 4437 | 3281 | 6000 |
| BOX CREEK | 9.50046 | 4577 | 3385 | 6190 |
| BOX CREEK | 10.45099 | 4629 | 3423 | 6260 |
| BOX CREEK | 11.62422 | 4867 | 3599 | 6582 |
| BOX CREEK | 11.79184 | 4769 | 3526 | 6449 |
| BOX CREEK | 12.90762 | 5050 | 3734 | 6829 |
| BROWNLEE CREEK | 1.0579 | 1065 | 788 | 1441 |
| BROWNLEE CREEK | 3.23002 | 2718 | 2010 | 3676 |
| BROWNLEE CREEK | 3.25437 | 3723 | 2753 | 5035 |
| BROWNLEE CREEK | 5.87336 | 3723 | 2753 | 5035 |
| BROWNLEE CREEK | 6.82625 | 4052 | 2996 | 5480 |
| BROWNLEE CREEK | 7.95688 | 4469 | 3304 | 6044 |
| BROWNLEE CREEK | 8.84842 | 4488 | 3319 | 6070 |
| CAMP CREEK | 1.00015 | 1115 | 824 | 1508 |
| CAMP CREEK | 1.73746 | 1580 | 1168 | 2137 |
| CAMP CREEK | 4.11864 | 2506 | 1853 | 3389 |
| CAMP CREEK | 4.81848 | 2706 | 2001 | 3659 |
| CAMP CREEK | 5.08243 | 2706 | 2001 | 3659 |
| CAMP CREEK | 6.01774 | 3027 | 2239 | 4094 |
| CANEY CREEK | 1.0032 | 1058 | 782 | 1431 |
| CANEY CREEK | 1.97169 | 1677 | 1240 | 2268 |
| CANEY CREEK | 3.81209 | 9481 | 7011 | 12823 |
| CANEY CREEK | 5.82395 | 9481 | 7011 | 12823 |
| CANEY CREEK | 11.79214 | 9481 | 7011 | 12823 |
| CANEY CREEK | 15.86461 | 9481 | 7011 | 12823 |
| CANEY CREEK | 16.31652 | 9481 | 7011 | 12823 |
| CANEY CREEK | 16.78668 | 9481 | 7011 | 12823 |
| CANEY CREEK | 21.20154 | 9481 | 7011 | 12823 |
| CANEY CREEK | 22.08903 | 9481 | 7011 | 12823 |
| CANEY CREEK | 23.87583 | 9481 | 7011 | 12823 |
| CANEY CREEK | 25.49294 | 9730 | 7194 | 13158 |
| CANEY CREEK | 26.52932 | 10059 | 7438 | 13603 |
| CANEY CREEK | 26.7657 | 10059 | 7438 | 13603 |
| CANEY CREEK | 38.56489 | 12998 | 9612 | 17579 |
| CANEY CREEK | 39.42568 | 13124 | 9705 | 17749 |
| CANEY CREEK | 40.37552 | 13433 | 9933 | 18166 |
| CANEY CREEK | 41.29541 | 13444 | 9941 | 18181 |
| CANEY CREEK | 42.0668 | 13757 | 10173 | 18605 |
| CANEY CREEK | 44.19389 | 14092 | 10420 | 19058 |
| CANEY CREEK | 65.02696 | 20034 | 14814 | 27093 |
| CANEY CREEK | 67.03891 | 20436 | 15111 | 27637 |
| CANEY CREEK | 67.9744 | 20523 | 15175 | 27754 |
| CANEY CREEK | 68.72202 | 22660 | 16756 | 30645 |
| CANEY CREEK | 71.33363 | 20436 | 15111 | 27637 |
| CANEY CREEK | 73.60788 | 20759 | 15350 | 28074 |
| CANEY CREEK | 83.31925 | 22624 | 16729 | 30597 |
| CANEY CREEK | 84.34209 | 22660 | 16756 | 30645 |
| CEDAR CREEK | 0.92948 | 910 | 673 | 1231 |
| CEDAR CREEK | 1.34656 | 1106 | 818 | 1496 |
| CEDAR CREEK | 2.31904 | 1786 | 1321 | 2416 |
| CEDAR CREEK | 4.16891 | 4766 | 3524 | 6445 |
| CEDAR CREEK | 5.0419 | 4766 | 3524 | 6445 |
| CEDAR CREEK | 5.54901 | 4766 | 3524 | 6445 |
| CEDAR CREEK | 6.6851 | 4766 | 3524 | 6445 |
| CEDAR CREEK | 9.29187 | 4528 | 3349 | 6124 |
| CEDAR CREEK | 10.09563 | 4766 | 3524 | 6445 |
| COLLINS CREEK | 0.8173 | 879 | 650 | 1188 |
| COLLINS CREEK | 1.98376 | 1706 | 1261 | 2307 |
| COLLINS CREEK | 2.49405 | 1936 | 1432 | 2618 |
| COLLINS CREEK | 3.191 | 2757 | 2039 | 3729 |
| COLLINS CREEK | 4.35606 | 2757 | 2039 | 3729 |
| COLLINS CREEK | 5.79068 | 3372 | 2494 | 4561 |
| COLLINS CREEK | 5.90583 | 3372 | 2494 | 4561 |
| COLLINS CREEK | 6.77254 | 3661 | 2707 | 4952 |
| COLLINS CREEK | 7.68387 | 3792 | 2804 | 5129 |
| DADS CREEK | 1.00167 | 775 | 573 | 1048 |
| DADS CREEK | 1.56133 | 1093 | 808 | 1478 |
| DADS CREEK | 2.33007 | 1433 | 1059 | 1938 |
| DADS CREEK | 2.87062 | 1981 | 1465 | 2679 |
| DADS CREEK | 4.4817 | 2811 | 2079 | 3802 |
| DADS CREEK | 7.12604 | 4440 | 3283 | 6004 |
| DADS CREEK | 8.10116 | 4821 | 3565 | 6520 |
| DADS CREEK | 8.68796 | 4821 | 3565 | 6520 |
| DADS CREEK | 9.09001 | 4947 | 3658 | 6690 |
| DICKEY CREEK | 0.92114 | 917 | 678 | 1240 |
| DICKEY CREEK | 1.97214 | 1604 | 1186 | 2169 |
| DICKEY CREEK | 6.44119 | 3362 | 2486 | 4547 |
| DICKEY CREEK | 7.03869 | 3567 | 2638 | 4824 |
| DICKEY CREEK | 7.96549 | 3895 | 2880 | 5267 |
| DICKEY CREEK | 9.21796 | 4251 | 3143 | 5749 |
| DICKEY CREEK | 11.58057 | 4935 | 3649 | 6674 |
| DICKEY CREEK | 12.34791 | 5118 | 3785 | 6922 |
| DICKEY CREEK | 13.07078 | 5474 | 4048 | 7404 |
| DICKEY CREEK | 13.4259 | 5505 | 4070 | 7444 |
| DICKEY CREEK | 13.95651 | 5626 | 4160 | 7608 |
| DICKEY CREEK | 14.63633 | 5767 | 4264 | 7799 |
| DICKEY CREEK | 17.10903 | 6891 | 5095 | 9319 |
| DICKEY CREEK | 18.45757 | 6714 | 4965 | 9080 |
| DICKEY CREEK | 19.47951 | 6891 | 5095 | 9319 |
| DICKEY CREEK | 21.68588 | 7277 | 5381 | 9841 |
| DICKEY CREEK | 23.15799 | 7600 | 5620 | 10278 |
| DICKEY CREEK | 24.71434 | 7823 | 5785 | 10580 |
| DICKEY CREEK | 25.27334 | 7917 | 5854 | 10707 |
| DICKEY CREEK | 26.22093 | 8141 | 6020 | 11010 |
| DICKEY CREEK | 27.41101 | 8025 | 5934 | 10852 |
| DICKEY CREEK | 27.85804 | 8082 | 5976 | 10930 |
| DUTCH SMITH CREEK | 0.68042 | 832 | 615 | 1125 |
| DUTCH SMITH CREEK | 1.80193 | 1644 | 1215 | 2223 |
| DUTCH SMITH CREEK | 2.50944 | 1967 | 1455 | 2661 |
| DUTCH SMITH CREEK | 2.59002 | 2012 | 1488 | 2721 |
| DUTCH SMITH CREEK | 2.89705 | 2095 | 1549 | 2834 |
| EAST FORK WHITE ROCK CREEK | 1.01441 | 943 | 698 | 1276 |
| EAST FORK WHITE ROCK CREEK | 2.08531 | 1554 | 1149 | 2101 |
| EAST FORK WHITE ROCK CREEK | 3.23527 | 2007 | 1484 | 2714 |
| EAST FORK WHITE ROCK CREEK | 5.06154 | 2911 | 2152 | 3937 |
| EAST FORK WHITE ROCK CREEK | 5.78647 | 3089 | 2284 | 4178 |
| EAST FORK WHITE ROCK CREEK | 11.0061 | 5088 | 3763 | 6881 |
| EAST FORK WHITE ROCK CREEK | 14.35717 | 6330 | 4681 | 8561 |
| EAST FORK WHITE ROCK CREEK | 14.36641 | 6330 | 4681 | 8561 |
| EAST FORK WHITE ROCK CREEK | 15.4728 | 6330 | 4681 | 8561 |
| EAST FORK WHITE ROCK CREEK | 18.29452 | 6741 | 4985 | 9117 |
| ELM CREEK | 1.22236 | 1022 | 756 | 1382 |
| ELM CREEK | 2.55825 | 1877 | 1388 | 2539 |
| ELM CREEK | 2.7064 | 1891 | 1398 | 2557 |
| ELM CREEK | 3.5287 | 2278 | 1685 | 3081 |
| ELM CREEK | 5.28814 | 2853 | 2109 | 3858 |
| ELM CREEK | 6.98148 | 3373 | 2494 | 4562 |
| FOUNTAIN CREEK | 0.94912 | 1033 | 764 | 1398 |
| FOUNTAIN CREEK | 1.79359 | 1612 | 1192 | 2180 |
| FOUNTAIN CREEK | 3.54432 | 2671 | 1975 | 3612 |
| FOUNTAIN CREEK | 5.41674 | 3687 | 2726 | 4986 |
| FOUNTAIN CREEK | 7.06793 | 4632 | 3425 | 6264 |
| FOUNTAIN CREEK | 8.05354 | 4881 | 3609 | 6600 |
| FOUNTAIN CREEK | 11.93783 | 7465 | 5520 | 10095 |
| FOUNTAIN CREEK | 12.68984 | 7523 | 5563 | 10174 |
| FOUNTAIN CREEK | 16.52023 | 9870 | 7298 | 13348 |
| FOUNTAIN CREEK | 19.71051 | 11333 | 8380 | 15326 |
| FOUNTAIN CREEK | 20.77863 | 11333 | 8380 | 15326 |
| GAIL CREEK | 0.57749 | 512 | 378 | 692 |
| GAIL CREEK | 0.64566 | 608 | 450 | 822 |
| GAIL CREEK | 1.76813 | 1485 | 1098 | 2009 |
| GAIL CREEK | 6.88123 | 4150 | 3069 | 5613 |
| GAIL CREEK | 7.01816 | 4150 | 3069 | 5613 |
| GAIL CREEK | 10.0865 | 5343 | 3951 | 7226 |
| GAIL CREEK | 14.82656 | 6594 | 4876 | 8917 |
| GAIL CREEK | 21.72928 | 8321 | 6153 | 11253 |
| GAIL CREEK | 22.97765 | 8397 | 6209 | 11356 |
| GAIL CREEK | 26.46502 | 9310 | 6884 | 12591 |
| GAIL CREEK | 26.75853 | 9325 | 6895 | 12611 |
| GAIL CREEK | 28.48227 | 9325 | 6895 | 12611 |
| GAIL CREEK | 28.95611 | 9325 | 6895 | 12611 |
| GAIL CREEK | 41.0414 | 11639 | 8606 | 15740 |
| GAIL CREEK | 41.63349 | 11639 | 8606 | 15740 |
| GAIL CREEK | 44.92815 | 12167 | 8997 | 16455 |
| GAIL CREEK | 45.47383 | 12262 | 9067 | 16583 |
| GAIL CREEK | 45.501 | 12262 | 9067 | 16583 |
| GAIL CREEK | 47.48354 | 12390 | 9162 | 16757 |
| GAIL CREEK | 48.19402 | 12390 | 9162 | 16757 |
| GAIL CREEK | 48.85772 | 12390 | 9162 | 16757 |
| GAIL CREEK | 62.15009 | 14306 | 10578 | 19347 |
| GAIL CREEK | 63.36698 | 14306 | 10578 | 19347 |
| GAIL CREEK | 72.75588 | 15691 | 11603 | 21221 |
| GAIL CREEK | 73.11646 | 15691 | 11603 | 21221 |
| GAIL CREEK | 75.71928 | 15880 | 11742 | 21476 |
| GAIL CREEK | 79.38933 | 16391 | 12120 | 22167 |
| GAIL CREEK | 82.42986 | 16333 | 12077 | 22088 |
| GAIL CREEK | 83.09363 | 16391 | 12120 | 22167 |
| GAIL CREEK | 83.37629 | 16348 | 12088 | 22109 |
| HARTTS CREEK | 1.09462 | 893 | 660 | 1207 |
| HARTTS CREEK | 2.13319 | 1476 | 1091 | 1996 |
| HARTTS CREEK | 2.78091 | 1671 | 1235 | 2260 |
| HARTTS CREEK | 3.3686 | 1910 | 1412 | 2582 |
| HARTTS CREEK | 3.55875 | 1966 | 1454 | 2659 |
| HARTTS CREEK | 3.68439 | 2006 | 1483 | 2713 |
| HARTTS CREEK | 4.08814 | 2172 | 1606 | 2938 |
| HARTTS CREEK | 6.27215 | 3100 | 2292 | 4193 |
| HARTTS CREEK | 8.3426 | 3918 | 2897 | 5299 |
| HARTTS CREEK | 8.60541 | 3918 | 2897 | 5299 |
| HICK BRANCH | 1.21229 | 951 | 703 | 1286 |
| HICK BRANCH | 1.287 | 956 | 707 | 1293 |
| LAKE CREEK | 0.98804 | 963 | 712 | 1303 |
| LAKE CREEK | 1.2941 | 1191 | 881 | 1611 |
| LAKE CREEK | 1.62532 | 1337 | 989 | 1808 |
| LITTLE WHITE ROCK CREEK | 16.02961 | 7000 | 5176 | 9467 |
| LITTLE WHITE ROCK CREEK | 24.65636 | 9493 | 7020 | 12839 |
| LITTLE WHITE ROCK CREEK | 26.40594 | 9493 | 7020 | 12839 |
| LITTLE WHITE ROCK CREEK | 28.17816 | 9727 | 7193 | 13155 |
| LITTLE WHITE ROCK CREEK | 29.95525 | 9943 | 7352 | 13447 |
| LITTLE WHITE ROCK CREEK | 31.9631 | 10591 | 7831 | 14323 |
| LITTLE WHITE ROCK CREEK | 35.43039 | 11033 | 8158 | 14921 |
| LITTLE WHITE ROCK CREEK | 37.04266 | 11547 | 8538 | 15616 |
| LITTLE WHITE ROCK CREEK | 37.14004 | 11549 | 8540 | 15618 |
| LITTLE WHITE ROCK CREEK | 41.85039 | 14842 | 10975 | 20072 |
| LITTLE WHITE ROCK CREEK | 45.15602 | 14842 | 10975 | 20072 |
| LITTLE WHITE ROCK CREEK | 47.62667 | 14842 | 10975 | 20072 |
| LITTLE WHITE ROCK CREEK | 49.97973 | 14842 | 10975 | 20072 |
| LITTLE WHITE ROCK CREEK | 56.18678 | 14842 | 10975 | 20072 |
| LITTLE WHITE ROCK CREEK | 58.63617 | 15104 | 11169 | 20427 |
| LITTLE WHITE ROCK CREEK | 60.89876 | 15565 | 11509 | 21049 |
| LITTLE WHITE ROCK CREEK | 62.24936 | 15817 | 11696 | 21391 |
| LITTLE WHITE ROCK CREEK | 63.9887 | 15909 | 11763 | 21514 |
| LITTLE WHITE ROCK CREEK | 65.77834 | 16047 | 11866 | 21701 |
| LITTLE WHITE ROCK CREEK | 66.6438 | 16047 | 11866 | 21701 |
| LOUISVILLE CREEK | 1.00929 | 967 | 715 | 1308 |
| LOUISVILLE CREEK | 1.94656 | 1451 | 1073 | 1962 |
| LOUISVILLE CREEK | 3.13961 | 2098 | 1552 | 2838 |
| LOUISVILLE CREEK | 4.34558 | 2764 | 2043 | 3737 |
| LOUISVILLE CREEK | 7.13304 | 3795 | 2806 | 5133 |
| LYNCH CREEK | 1.0127 | 871 | 644 | 1178 |
| LYNCH CREEK | 1.01828 | 896 | 663 | 1212 |
| LYNCH CREEK | 1.31042 | 1069 | 790 | 1446 |
| MAGNOLIA CREEK | 1.04929 | 1227 | 907 | 1660 |
| MAGNOLIA CREEK | 1.88659 | 2019 | 1493 | 2730 |
| MAGNOLIA CREEK | 2.7585 | 2800 | 2071 | 3787 |
| MAGNOLIA CREEK | 3.7165 | 3556 | 2629 | 4809 |
| OLDHAM BRANCH | 1.11332 | 936 | 692 | 1266 |
| OLDHAM BRANCH | 1.54699 | 1184 | 875 | 1601 |
| OLDHAM BRANCH | 2.33989 | 1539 | 1138 | 2082 |
| PEACHTREE CREEK | 1.31885 | 1089 | 805 | 1473 |
| PEACHTREE CREEK | 2.31891 | 1720 | 1272 | 2327 |
| PEACHTREE CREEK | 2.90637 | 2090 | 1546 | 2827 |
| PEACHTREE CREEK | 3.81932 | 2476 | 1831 | 3349 |
| PEACHTREE CREEK | 4.37021 | 2652 | 1961 | 3587 |
| PEACHTREE CREEK | 6.88119 | 3719 | 2750 | 5030 |
| PEACHTREE CREEK | 6.9837 | 3831 | 2833 | 5181 |
| RIDGEWAY CREEK | 0.81784 | 762 | 563 | 1030 |
| RIDGEWAY CREEK | 2.47027 | 1584 | 1171 | 2142 |
| RIDGEWAY CREEK | 3.89782 | 2226 | 1646 | 3010 |
| RIDGEWAY CREEK | 4.56551 | 2487 | 1839 | 3363 |
| RIDGEWAY CREEK | 5.57298 | 2934 | 2170 | 3968 |
| RIDGEWAY CREEK | 5.92278 | 3049 | 2255 | 4124 |
| RIDGEWAY CREEK | 6.02958 | 3078 | 2276 | 4162 |
| RIDGEWAY CREEK | 7.23051 | 3390 | 2507 | 4585 |
| RIDGEWAY CREEK | 7.71278 | 3497 | 2585 | 4729 |
| SALT CREEK | 1.00051 | 769 | 568 | 1040 |
| SALT CREEK | 2.16432 | 1379 | 1020 | 1865 |
| SALT CREEK | 2.99004 | 1708 | 1263 | 2310 |
| SALT CREEK | 4.08775 | 2279 | 1685 | 3082 |
| SALT CREEK | 5.32437 | 2713 | 2006 | 3669 |
| SALT CREEK | 6.3212 | 3025 | 2237 | 4091 |
| SALT CREEK | 7.35184 | 3324 | 2458 | 4495 |
| SALT CREEK | 12.94936 | 5742 | 4246 | 7766 |
| SALT CREEK | 13.85481 | 6054 | 4477 | 8188 |
| SAND BRANCH | 1.00239 | 903 | 668 | 1221 |
| SAND BRANCH | 1.15385 | 937 | 693 | 1267 |
| SAND BRANCH | 1.61209 | 1168 | 863 | 1579 |
| SAND BRANCH | 2.50158 | 1601 | 1184 | 2165 |
| SANDY CREEK | 1.13179 | 1209 | 894 | 1635 |
| SANDY CREEK | 1.55729 | 1669 | 1234 | 2257 |
| SANDY CREEK | 4.32629 | 3881 | 2870 | 5248 |
| SANDY CREEK | 5.1035 | 4373 | 3234 | 5914 |
| SANDY CREEK | 6.19754 | 5130 | 3793 | 6938 |
| SANDY CREEK | 7.09685 | 5438 | 4021 | 7354 |
| SANDY CREEK | 7.85949 | 5608 | 4147 | 7585 |
| SOOKIE CREEK | 1.12013 | 937 | 693 | 1267 |
| SOOKIE CREEK | 1.60034 | 1183 | 875 | 1600 |
| SPRING BRANCH | 0.92509 | 1019 | 754 | 1378 |
| SPRING BRANCH | 1.34638 | 1305 | 965 | 1765 |
| SPRING BRANCH | 1.63842 | 1430 | 1057 | 1933 |
| SPRING BRANCH | 2.87015 | 2344 | 1733 | 3170 |
| SPRING CREEK | 0.92348 | 755 | 558 | 1021 |
| SPRING CREEK | 1.31778 | 1013 | 749 | 1370 |
| SPRING CREEK | 2.64712 | 1874 | 1386 | 2535 |
| SPRING CREEK | 4.30692 | 3060 | 2263 | 4138 |
| SPRING CREEK | 4.83806 | 3129 | 2314 | 4232 |
| SPRING CREEK | 6.52923 | 3954 | 2923 | 5347 |
| SPRING CREEK | 8.32383 | 4342 | 3211 | 5873 |
| SPRING CREEK | 9.31079 | 4669 | 3452 | 6314 |
| SPRING CREEK | 9.85369 | 4669 | 3452 | 6314 |
| SPRING CREEK | 12.07799 | 5433 | 4017 | 7347 |
| TANTABOGUE CREEK | 1.00427 | 821 | 607 | 1111 |
| TANTABOGUE CREEK | 1.66491 | 1201 | 888 | 1624 |
| TANTABOGUE CREEK | 2.41926 | 1575 | 1165 | 2131 |
| TANTABOGUE CREEK | 3.09711 | 1847 | 1366 | 2498 |
| TANTABOGUE CREEK | 3.90105 | 2110 | 1560 | 2854 |
| TANTABOGUE CREEK | 5.35988 | 2720 | 2011 | 3679 |
| TANTABOGUE CREEK | 6.56028 | 3015 | 2229 | 4077 |
| TANTABOGUE CREEK | 7.29875 | 3258 | 2409 | 4406 |
| TANTABOGUE CREEK | 7.88926 | 3393 | 2509 | 4588 |
| TANTABOGUE CREEK | 9.01354 | 3779 | 2794 | 5110 |
| TANTABOGUE CREEK | 9.07784 | 3779 | 2794 | 5110 |
| TANTABOGUE CREEK | 11.46658 | 4359 | 3223 | 5895 |
| TANTABOGUE CREEK | 11.67777 | 4419 | 3267 | 5976 |
| TANTABOGUE CREEK | 13.22287 | 4851 | 3587 | 6560 |
| TANTABOGUE CREEK | 16.76863 | 5653 | 4180 | 7646 |
| TANTABOGUE CREEK | 18.31425 | 6012 | 4446 | 8131 |
| TANTABOGUE CREEK | 21.19123 | 6823 | 5045 | 9227 |
| TANTABOGUE CREEK | 24.96493 | 7529 | 5567 | 10182 |
| TANTABOGUE CREEK | 27.58676 | 7998 | 5914 | 10817 |
| TANTABOGUE CREEK | 30.68242 | 8509 | 6292 | 11508 |
| TANTABOGUE CREEK | 33.75706 | 9136 | 6755 | 12355 |
| TANTABOGUE CREEK | 36.07784 | 9306 | 6881 | 12585 |
| TANTABOGUE CREEK | 36.1433 | 9306 | 6881 | 12585 |
| TANTABOGUE CREEK | 39.05593 | 9792 | 7241 | 13243 |
| TANTABOGUE CREEK | 44.10563 | 10900 | 8060 | 14740 |
| TANTABOGUE CREEK | 44.80135 | 10994 | 8129 | 14868 |
| TANTABOGUE CREEK | 45.33779 | 11064 | 8181 | 14963 |
| TANTABOGUE CREEK | 46.31767 | 11064 | 8181 | 14963 |
| TANTABOGUE CREEK | 48.87119 | 11114 | 8218 | 15030 |
| TANTABOGUE CREEK | 49.68816 | 11412 | 8438 | 15433 |
| TANTABOGUE CREEK | 50.70359 | 11527 | 8524 | 15589 |
| TANTABOGUE CREEK | 57.97678 | 12528 | 9264 | 16943 |
| TANTABOGUE CREEK | 58.92115 | 12727 | 9411 | 17212 |
| TANTABOGUE CREEK | 59.62223 | 12739 | 9420 | 17228 |
| TANTABOGUE CREEK | 62.53901 | 13087 | 9677 | 17698 |
| TANTABOGUE CREEK | 67.5879 | 13791 | 10198 | 18651 |
| TANTABOGUE CREEK | 68.36933 | 13845 | 10238 | 18724 |
| TANTABOGUE CREEK | 70.62322 | 14202 | 10501 | 19206 |
| TANTABOGUE CREEK | 72.91371 | 14404 | 10651 | 19480 |
| TANTABOGUE CREEK | 73.90776 | 14451 | 10685 | 19543 |
| TANTABOGUE CREEK | 75.06089 | 14451 | 10685 | 19543 |
| TANYARD CREEK | 1.19618 | 1129 | 835 | 1527 |
| TANYARD CREEK | 2.12423 | 1789 | 1323 | 2419 |
| TANYARD CREEK | 2.37765 | 1848 | 1366 | 2499 |
| TANYARD CREEK | 3.24615 | 2237 | 1654 | 3025 |
| UNT 001 IN WRC | 0.26813 | 388 | 287 | 525 |
| UNT 002 IN WRC | 0.1867 | 239 | 177 | 324 |
| UNT 003 IN WRC | 0.19844 | 271 | 201 | 367 |
| UNT 004 IN WRC | 0.20105 | 258 | 190 | 348 |
| UNT 005 IN WRC | 0.48407 | 559 | 414 | 756 |
| UNT 005 IN WRC | 0.57053 | 633 | 468 | 857 |
| UNT 007 IN WRC | 0.48967 | 430 | 318 | 582 |
| UNT 007 IN WRC | 0.50084 | 436 | 322 | 589 |
| UNT 007 IN WRC | 1.18261 | 906 | 670 | 1226 |
| UNT 007 IN WRC | 1.20422 | 924 | 683 | 1249 |
| UNT 007 IN WRC | 1.91667 | 1353 | 1001 | 1830 |
| UNT 008 IN WRC | 0.58166 | 536 | 396 | 725 |
| UNT 008 IN WRC | 0.59631 | 562 | 416 | 760 |
| UNT 009 IN WRC | 0.48391 | 454 | 336 | 614 |
| UNT 010 IN WRC | 0.74378 | 746 | 552 | 1009 |
| UNT 010 IN WRC | 1.63523 | 1494 | 1105 | 2021 |
| UNT 010 IN WRC | 2.17032 | 1754 | 1297 | 2373 |
| UNT 010 IN WRC | 2.88438 | 2108 | 1559 | 2851 |
| UNT 010 IN WRC | 3.19257 | 2274 | 1682 | 3076 |
| UNT 010 IN WRC | 4.30736 | 2705 | 2000 | 3658 |
| UNT 011 IN WRC | 0.55123 | 580 | 429 | 784 |
| UNT 011 IN WRC | 0.8173 | 794 | 587 | 1074 |
| UNT 011 IN WRC | 0.93711 | 841 | 622 | 1137 |
| UNT 012 IN WRC | 0.15621 | 215 | 159 | 290 |
| UNT 013 IN WRC | 1.0892 | 1083 | 801 | 1465 |
| UNT 013 IN WRC | 2.22466 | 1870 | 1383 | 2529 |
| UNT 013 IN WRC | 3.17055 | 2208 | 1632 | 2986 |
| UNT 013 IN WRC | 3.48675 | 2417 | 1787 | 3269 |
| UNT 014 IN WRC | 0.30768 | 463 | 342 | 626 |
| UNT 015 IN WRC | 0.11631 | 174 | 129 | 235 |
| UNT 015 IN WRC | 0.24517 | 317 | 235 | 429 |
| UNT 016 IN WRC | 0.50765 | 555 | 410 | 751 |
| UNT 017 IN WRC | 1.09162 | 736 | 544 | 996 |
| UNT 017 IN WRC | 1.97895 | 1102 | 815 | 1490 |
| UNT 017 IN WRC | 2.18969 | 1180 | 873 | 1596 |
| UNT 019 IN WRC | 0.38066 | 394 | 291 | 532 |
| UNT 019 IN WRC | 0.66996 | 647 | 479 | 876 |
| UNT 020 IN WRC | 0.22876 | 271 | 200 | 367 |
| UNT 021 IN WRC | 0.48918 | 562 | 415 | 759 |
| UNT 023 IN WRC | 0.67077 | 665 | 491 | 899 |
| UNT 023 IN WRC | 0.68531 | 665 | 491 | 899 |
| UNT 023 IN WRC | 1.28774 | 986 | 729 | 1334 |
| UNT 024 IN WRC | 0.91334 | 748 | 553 | 1012 |
| UNT 025 IN WRC | 0.52612 | 500 | 370 | 677 |
| UNT 026 IN WRC | 0.53384 | 548 | 405 | 742 |
| UNT 026 IN WRC | 1.42068 | 1085 | 802 | 1468 |
| UNT 027 IN WRC | 0.5491 | 565 | 418 | 764 |
| UNT 028 IN WRC | 0.16784 | 199 | 147 | 269 |
| UNT 029 IN WRC | 0.25803 | 308 | 228 | 417 |
| UNT 030 IN WRC | 0.72323 | 673 | 498 | 910 |
| UNT 030 IN WRC | 1.19969 | 920 | 681 | 1245 |
| UNT 030 IN WRC | 1.41409 | 1069 | 790 | 1446 |
| UNT 030 IN WRC | 1.53596 | 1099 | 813 | 1486 |
| UNT 031 IN WRC | 0.39009 | 464 | 343 | 627 |
| UNT 031 IN WRC | 0.51838 | 525 | 388 | 710 |
| UNT 032 IN WRC | 0.22186 | 252 | 186 | 341 |
| UNT 032 IN WRC | 0.35018 | 374 | 276 | 505 |
| UNT 033 IN WRC | 0.12133 | 166 | 123 | 225 |
| UNT 034 IN WRC | 0.44654 | 398 | 294 | 538 |
| UNT 034 IN WRC | 0.61849 | 514 | 380 | 695 |
| UNT 035 IN WRC | 0.16952 | 158 | 117 | 213 |
| UNT 037 IN WRC | 0.2665 | 325 | 240 | 439 |
| UNT 037 IN WRC | 0.2819 | 348 | 257 | 470 |
| UNT 038 IN WRC | 0.064 | 77 | 57 | 104 |
| UNT 038 IN WRC | 0.08353 | 111 | 82 | 151 |
| UNT 039 IN WRC | 0.18805 | 252 | 186 | 341 |
| UNT 040 IN WRC | 0.9702 | 944 | 698 | 1277 |
| UNT 040 IN WRC | 1.84039 | 1442 | 1067 | 1951 |
| UNT 041 IN WRC | 0.60307 | 682 | 504 | 922 |
| UNT 041 IN WRC | 1.41064 | 1348 | 996 | 1822 |
| UNT 041 IN WRC | 1.71396 | 1599 | 1182 | 2162 |
| UNT 041 IN WRC | 2.06716 | 1647 | 1218 | 2227 |
| UNT 042 IN WRC | 0.77228 | 804 | 595 | 1087 |
| UNT 043 IN WRC | 0.28123 | 398 | 294 | 539 |
| UNT 043 IN WRC | 0.29041 | 415 | 307 | 561 |
| UNT 044 IN WRC | 0.16428 | 191 | 142 | 259 |
| UNT 045 IN WRC | 0.1546 | 247 | 183 | 334 |
| UNT 047 IN WRC | 0.09046 | 103 | 76 | 140 |
| UNT 047 IN WRC | 0.15452 | 209 | 154 | 282 |
| UNT 047 IN WRC | 0.45237 | 521 | 385 | 705 |
| UNT 047 IN WRC | 1.30657 | 1186 | 877 | 1603 |
| UNT 047 IN WRC | 2.56614 | 1829 | 1353 | 2474 |
| UNT 048 IN WRC | 0.01148 | 15 | 11 | 20 |
| UNT 048 IN WRC | 0.02865 | 38 | 28 | 51 |
| UNT 048 IN WRC | 0.12374 | 177 | 131 | 240 |
| UNT 049 IN WRC | 0.28917 | 413 | 305 | 558 |
| UNT 049 IN WRC | 0.31331 | 416 | 307 | 562 |
| UNT 049 IN WRC | 0.33529 | 474 | 350 | 641 |
| UNT 050 IN WRC | 1.0579 | 516 | 381 | 697 |
| UNT 050 IN WRC | 1.76597 | 740 | 548 | 1001 |
| UNT 050 IN WRC | 2.74935 | 1098 | 812 | 1485 |
| UNT 050 IN WRC | 3.10267 | 1138 | 842 | 1540 |
| UNT 051 IN WRC | 0.97459 | 855 | 632 | 1156 |
| UNT 051 IN WRC | 2.20691 | 1619 | 1198 | 2190 |
| UNT 051 IN WRC | 2.85508 | 1899 | 1404 | 2568 |
| UNT 053 IN WRC | 1.0128 | 1174 | 868 | 1587 |
| UNT 053 IN WRC | 1.03124 | 1174 | 868 | 1587 |
| UNT 053 IN WRC | 2.46881 | 2198 | 1625 | 2973 |
| UNT 054 IN WRC | 0.90778 | 1004 | 743 | 1358 |
| UNT 054 IN WRC | 1.02142 | 1084 | 801 | 1466 |
| UNT 055 IN WRC | 1.08534 | 1030 | 761 | 1392 |
| UNT 055 IN WRC | 1.84112 | 1637 | 1210 | 2214 |
| UNT 055 IN WRC | 2.22794 | 1809 | 1337 | 2446 |
| UNT 056 IN WRC | 0.18114 | 259 | 191 | 350 |
| UNT 056 IN WRC | 0.20643 | 313 | 232 | 424 |
| UNT 057 IN WRC | 1.08202 | 1092 | 807 | 1476 |
| UNT 057 IN WRC | 1.28722 | 1217 | 900 | 1646 |
| UNT 058 IN WRC | 0.28236 | 292 | 216 | 395 |
| UNT 060 IN WRC | 0.22984 | 310 | 229 | 419 |
| UNT 060 IN WRC | 0.50433 | 489 | 361 | 661 |
| UNT 061 IN WRC | 0.74107 | 641 | 474 | 867 |
| UNT 062 IN WRC | 0.82842 | 863 | 638 | 1168 |
| UNT 063 IN WRC | 1.27428 | 1129 | 835 | 1527 |
| UNT 063 IN WRC | 1.55927 | 1242 | 919 | 1680 |
| UNT 063 IN WRC | 2.11633 | 1518 | 1122 | 2053 |
| UNT 064 IN WRC | 0.271 | 364 | 269 | 492 |
| UNT 065 IN WRC | 0.92873 | 912 | 674 | 1234 |
| UNT 066 IN WRC | 0.91899 | 859 | 635 | 1162 |
| UNT 066 IN WRC | 2.69768 | 2100 | 1553 | 2840 |
| UNT 067 IN WRC | 1.22084 | 1251 | 925 | 1692 |
| UNT 068 IN WRC | 0.64171 | 615 | 455 | 831 |
| UNT 069 IN WRC | 1.0844 | 829 | 613 | 1121 |
| UNT 070 IN WRC | 0.46515 | 531 | 393 | 718 |
| UNT 070 IN WRC | 1.41373 | 1219 | 901 | 1648 |
| UNT 070 IN WRC | 1.95967 | 1435 | 1061 | 1941 |
| UNT 071 IN WRC | 0.92877 | 783 | 579 | 1059 |
| UNT 071 IN WRC | 2.06003 | 1465 | 1083 | 1981 |
| UNT 071 IN WRC | 3.80214 | 2837 | 2098 | 3837 |
| UNT 071 IN WRC | 4.38952 | 2972 | 2198 | 4019 |
| UNT 071 IN WRC | 5.21344 | 3312 | 2449 | 4479 |
| UNT 072 IN WRC | 1.20868 | 1088 | 805 | 1472 |
| UNT 073 IN WRC | 0.16653 | 224 | 165 | 302 |
| UNT 074 IN WRC | 0.28149 | 345 | 255 | 466 |
| UNT 074 IN WRC | 0.32172 | 387 | 286 | 524 |
| UNT 075 IN WRC | 0.37185 | 425 | 314 | 574 |
| UNT 075 IN WRC | 0.6974 | 682 | 505 | 923 |
| UNT 076 IN WRC | 0.96006 | 989 | 731 | 1338 |
| UNT 076 IN WRC | 1.06624 | 1046 | 773 | 1415 |
| UNT 077 IN WRC | 0.67875 | 762 | 563 | 1030 |
| UNT 077 IN WRC | 1.06848 | 1039 | 769 | 1406 |
| UNT 078 IN WRC | 0.72458 | 728 | 538 | 984 |
| UNT 079 IN WRC | 0.78744 | 842 | 623 | 1139 |
| UNT 080 IN WRC | 0.31646 | 440 | 326 | 596 |
| UNT 081 IN WRC | 0.51222 | 528 | 391 | 714 |
| UNT 081 IN WRC | 0.80041 | 737 | 545 | 997 |
| UNT 082 IN WRC | 0.97324 | 970 | 717 | 1312 |
| UNT 082 IN WRC | 1.9541 | 1480 | 1094 | 2002 |
| UNT 082 IN WRC | 2.52827 | 1893 | 1399 | 2560 |
| UNT 083 IN WRC | 0.87532 | 806 | 596 | 1090 |
| UNT 083 IN WRC | 1.43229 | 1103 | 815 | 1491 |
| UNT 083 IN WRC | 2.26496 | 1574 | 1164 | 2129 |
| UNT 084 IN WRC | 0.78054 | 767 | 567 | 1037 |
| UNT 087 IN WRC | 1.22554 | 1196 | 884 | 1617 |
| UNT 088 IN WRC | 0.99358 | 895 | 662 | 1210 |
| UNT 089 IN WRC | 0.43116 | 540 | 399 | 730 |
| UNT 090 IN WRC | 0.79445 | 675 | 499 | 912 |
| UNT 091 IN WRC | 0.30612 | 402 | 297 | 544 |
| UNT 092 IN WRC | 0.09737 | 177 | 131 | 239 |
| UNT 093 IN WRC | 0.32068 | 464 | 343 | 627 |
| UNT 095 IN WRC | 0.94786 | 875 | 647 | 1184 |
| UNT 095 IN WRC | 1.19629 | 1027 | 760 | 1389 |
| UNT 096 IN WRC | 0.84277 | 524 | 387 | 708 |
| UNT 096 IN WRC | 1.97913 | 1745 | 1290 | 2360 |
| UNT 097 IN WRC | 0.15416 | 230 | 170 | 311 |
| UNT 097 IN WRC | 0.89397 | 1023 | 757 | 1384 |
| UNT 098 IN WRC | 0.68581 | 832 | 615 | 1125 |
| UNT 099 IN WRC | 1.18604 | 989 | 731 | 1338 |
| UNT 099 IN WRC | 1.78223 | 1270 | 939 | 1718 |
| UNT 099 IN WRC | 2.47817 | 1734 | 1282 | 2345 |
| UNT 099 IN WRC | 3.39501 | 2075 | 1534 | 2806 |
| UNT 099 IN WRC | 4.28217 | 2414 | 1785 | 3265 |
| UNT 100 IN WRC | 1.00006 | 1069 | 790 | 1445 |
| UNT 100 IN WRC | 2.13998 | 1702 | 1259 | 2302 |
| UNT 101 IN WRC | 0.3282 | 356 | 263 | 481 |
| UNT 102 IN WRC | 0.3263 | 366 | 270 | 495 |
| UNT 103 IN WRC | 0.35664 | 386 | 286 | 523 |
| UNT 104 IN WRC | 1.07897 | 932 | 689 | 1261 |
| UNT 104 IN WRC | 1.46341 | 1105 | 817 | 1495 |
| UNT 105 IN WRC | 1.10803 | 1108 | 819 | 1499 |
| UNT 106 IN WRC | 0.81542 | 657 | 486 | 888 |
| UNT 106 IN WRC | 1.38992 | 956 | 707 | 1293 |
| UNT 107 IN WRC | 0.44813 | 469 | 347 | 635 |
| UNT 108 IN WRC | 0.54675 | 595 | 440 | 804 |
| UNT 110 IN WRC | 1.15663 | 1029 | 761 | 1392 |
| UNT 110 IN WRC | 2.32019 | 1535 | 1135 | 2076 |
| UNT 110 IN WRC | 3.41887 | 2061 | 1524 | 2788 |
| UNT 110 IN WRC | 3.78752 | 2277 | 1684 | 3079 |
| UNT 111 IN WRC | 0.09961 | 137 | 101 | 186 |
| UNT 114 IN WRC | 1.25429 | 1202 | 889 | 1625 |
| UNT 114 IN WRC | 1.56573 | 1332 | 985 | 1801 |
| UNT 115 IN WRC | 0.91908 | 777 | 574 | 1050 |
| UNT 115 IN WRC | 1.53315 | 1157 | 856 | 1565 |
| UNT 115 IN WRC | 3.8415 | 2401 | 1776 | 3248 |
| UNT 116 IN WRC | 1.22075 | 1077 | 796 | 1456 |
| UNT 116 IN WRC | 2.29692 | 1588 | 1174 | 2148 |
| UNT 117 IN WRC | 1.21716 | 1337 | 989 | 1808 |
| UNT 117 IN WRC | 1.88578 | 1656 | 1225 | 2240 |
| UNT 118 IN WRC | 0.912 | 844 | 624 | 1141 |
| UNT 118 IN WRC | 1.6976 | 1355 | 1002 | 1832 |
| UNT 118 IN WRC | 1.74266 | 1440 | 1065 | 1948 |
| UNT 119 IN WRC | 1.51112 | 1605 | 1187 | 2170 |
| UNT 119 IN WRC | 1.88712 | 1777 | 1314 | 2403 |
| UNT 120 IN WRC | 0.91881 | 913 | 675 | 1234 |
| UNT 120 IN WRC | 2.13561 | 1543 | 1141 | 2086 |
| UNT 120 IN WRC | 3.36219 | 2223 | 1644 | 3006 |
| UNT 120 IN WRC | 4.60227 | 2558 | 1891 | 3459 |
| UNT 120 IN WRC | 7.98396 | 3814 | 2820 | 5158 |
| UNT 121 IN WRC | 1.19429 | 1012 | 748 | 1369 |
| UNT 122 IN WRC | 1.2672 | 1092 | 808 | 1477 |
| UNT 122 IN WRC | 2.41536 | 1770 | 1309 | 2394 |
| UNT 123 IN WRC | 1.30711 | 1009 | 746 | 1364 |
| UNT 124 IN WRC | 1.1301 | 1036 | 766 | 1401 |
| UNT 125 IN WRC | 1.24433 | 1243 | 919 | 1681 |
| UNT 125 IN WRC | 1.9128 | 1519 | 1123 | 2054 |
| UNT 126 IN WRC | 1.00266 | 972 | 719 | 1315 |
| UNT 126 IN WRC | 1.63756 | 1402 | 1037 | 1896 |
| UNT 126 IN WRC | 3.21593 | 2389 | 1767 | 3231 |
| UNT 127 IN WRC | 1.30926 | 1197 | 885 | 1619 |
| UNT 128 IN WRC | 1.06817 | 970 | 717 | 1312 |
| UNT 128 IN WRC | 2.2556 | 1571 | 1162 | 2125 |
| UNT 128 IN WRC | 3.90356 | 2459 | 1819 | 3326 |
| UNT 128 IN WRC | 4.57585 | 2625 | 1941 | 3550 |
| UNT 129 IN WRC | 0.94473 | 996 | 737 | 1347 |
| UNT 129 IN WRC | 1.30388 | 1179 | 872 | 1594 |
| UNT 129 IN WRC | 1.3306 | 1181 | 873 | 1597 |
| UNT 130 IN WRC | 0.97307 | 1099 | 813 | 1487 |
| UNT 130 IN WRC | 2.00289 | 1768 | 1307 | 2391 |
| UNT 131 IN WRC | 1.03144 | 1098 | 812 | 1485 |
| UNT 131 IN WRC | 1.28989 | 1194 | 883 | 1615 |
| UNT 132 IN WRC | 1.01728 | 1037 | 767 | 1403 |
| UNT 132 IN WRC | 1.96971 | 1566 | 1158 | 2118 |
| UNT 132 IN WRC | 2.3488 | 1804 | 1334 | 2439 |
| UNT 133 IN WRC | 0.94607 | 1006 | 744 | 1361 |
| UNT 133 IN WRC | 2.12859 | 1887 | 1395 | 2552 |
| UNT 133 IN WRC | 3.31027 | 2603 | 1925 | 3520 |
| UNT 133 IN WRC | 4.09043 | 2866 | 2119 | 3876 |
| UNT 133 IN WRC | 5.50345 | 3502 | 2589 | 4736 |
| UNT 134 IN WRC | 1.15233 | 950 | 703 | 1285 |
| UNT 135 IN WRC | 1.00006 | 966 | 714 | 1306 |
| UNT 135 IN WRC | 1.93322 | 1485 | 1098 | 2008 |
| UNT 135 IN WRC | 2.44123 | 1892 | 1399 | 2558 |
| UNT 136 IN WRC | 1.34854 | 1193 | 882 | 1614 |
| UNT 137 IN WRC | 1.03135 | 1183 | 875 | 1600 |
| UNT 137 IN WRC | 1.34441 | 1202 | 889 | 1625 |
| UNT 138 IN WRC | 0.99593 | 1228 | 908 | 1661 |
| UNT 139 IN WRC | 0.40784 | 501 | 370 | 677 |
| UNT 140 IN WRC | 0.56684 | 619 | 458 | 837 |
| UNT 141 IN WRC | 0.34274 | 444 | 328 | 601 |
| UNT 141 IN WRC | 0.79255 | 865 | 640 | 1170 |
| UNT 142 IN WRC | 0.27593 | 399 | 295 | 540 |
| UNT 143 IN WRC | 0.65185 | 626 | 463 | 846 |
| UNT 144 IN WRC | 0.48523 | 518 | 383 | 700 |
| UNT 144 IN WRC | 0.6992 | 646 | 478 | 874 |
| UNT 145 IN WRC | 0.92186 | 893 | 660 | 1207 |
| UNT 145 IN WRC | 0.98829 | 941 | 696 | 1272 |
| UNT 146 IN WRC | 0.68243 | 565 | 417 | 764 |
| UNT 146 IN WRC | 1.43624 | 981 | 725 | 1326 |
| UNT 147 IN WRC | 0.75471 | 700 | 517 | 946 |
| UNT 147 IN WRC | 1.38988 | 1039 | 768 | 1405 |
| UNT 147 IN WRC | 1.42017 | 1039 | 768 | 1405 |
| UNT 147 IN WRC | 3.10751 | 1875 | 1387 | 2536 |
| UNT 148 IN WRC | 0.91872 | 746 | 552 | 1009 |
| UNT 148 IN WRC | 1.54149 | 1092 | 807 | 1476 |
| UNT 149 IN WRC | 1.20299 | 945 | 699 | 1278 |
| UNT 150 IN WRC | 0.90267 | 824 | 609 | 1114 |
| UNT 150 IN WRC | 1.33715 | 1026 | 759 | 1388 |
| UNT 150 IN WRC | 2.33371 | 1592 | 1177 | 2153 |
| UNT 150 IN WRC | 2.66703 | 1743 | 1289 | 2357 |
| UNT 151 IN WRC | 0.78188 | 666 | 492 | 900 |
| UNT 152 IN WRC | 1.01324 | 775 | 573 | 1048 |
| UNT 152 IN WRC | 1.83619 | 1149 | 850 | 1554 |
| UNT 152 IN WRC | 2.57072 | 1439 | 1064 | 1947 |
| UNT 153 IN WRC | 0.56782 | 500 | 370 | 676 |
| UNT 154 IN WRC | 0.69857 | 648 | 479 | 876 |
| UNT 155 IN WRC | 0.84322 | 712 | 527 | 963 |
| UNT 155 IN WRC | 2.34608 | 1703 | 1259 | 2303 |
| UNT 155 IN WRC | 2.86836 | 1918 | 1419 | 2594 |
| UNT 156 IN WRC | 0.68279 | 634 | 469 | 858 |
| UNT 157 IN WRC | 1.33715 | 1156 | 855 | 1564 |
| UNT 157 IN WRC | 1.68069 | 1337 | 988 | 1808 |
| UNT 157 IN WRC | 2.76121 | 1810 | 1339 | 2448 |
| UNT 158 IN WRC | 0.43322 | 404 | 299 | 547 |
| UNT 158 IN WRC | 1.13788 | 958 | 709 | 1296 |
| UNT 159 IN WRC | 0.70413 | 650 | 480 | 878 |
| UNT 160 IN WRC | 0.24984 | 286 | 211 | 386 |
| UNT 160 IN WRC | 0.28806 | 331 | 244 | 447 |
| UNT 160 IN WRC | 0.9087 | 897 | 663 | 1213 |
| UNT 160 IN WRC | 1.37427 | 1205 | 891 | 1629 |
| UNT 160 IN WRC | 1.74526 | 1402 | 1037 | 1896 |
| UNT 160 IN WRC | 2.44853 | 1805 | 1335 | 2441 |
| UNT 160 IN WRC | 2.87577 | 1939 | 1434 | 2623 |
| UNT 161 IN WRC | 0.11658 | 148 | 110 | 201 |
| UNT 161 IN WRC | 0.35601 | 407 | 301 | 551 |
| UNT 162 IN WRC | 0.20184 | 266 | 197 | 360 |
| UNT 163 IN WRC | 0.33924 | 382 | 283 | 517 |
| UNT 164 IN WRC | 0.23916 | 278 | 205 | 375 |
| UNT 166 IN WRC | 1.1013 | 756 | 559 | 1022 |
| UNT 167 IN WRC | 1.10551 | 876 | 648 | 1185 |
| UNT 168 IN WRC | 0.2805 | 222 | 164 | 301 |
| UNT 169 IN WRC | 0.68648 | 635 | 470 | 859 |
| UNT 170 IN WRC | 0.3743 | 417 | 308 | 564 |
| UNT 170 IN WRC | 0.52128 | 535 | 396 | 724 |
| UNT 171 IN WRC | 0.112 | 156 | 115 | 210 |
| UNT 172 IN WRC | 0.22213 | 192 | 142 | 259 |
| UNT 173 IN WRC | 0.81511 | 722 | 534 | 977 |
| UNT 174 IN WRC | 0.96724 | 873 | 646 | 1181 |
| UNT 176 IN WRC | 0.26763 | 300 | 222 | 405 |
| UNT 176 IN WRC | 0.28535 | 300 | 222 | 405 |
| UNT 177 IN WRC | 0.50973 | 469 | 347 | 635 |
| UNT 178 IN WRC | 1.00282 | 887 | 656 | 1199 |
| UNT 179 IN WRC | 0.74395 | 708 | 523 | 957 |
| UNT 180 IN WRC | 1.07099 | 1036 | 766 | 1401 |
| UNT 180 IN WRC | 1.28605 | 1252 | 925 | 1693 |
| UNT 180 IN WRC | 2.84234 | 1699 | 1256 | 2298 |
| UNT 181 IN WRC | 1.24406 | 914 | 676 | 1237 |
| UNT 181 IN WRC | 1.39956 | 984 | 728 | 1331 |
| UNT 181 IN WRC | 1.53908 | 984 | 728 | 1331 |
| UNT 182 IN WRC | 0.65355 | 642 | 474 | 868 |
| UNT 182 IN WRC | 1.18936 | 1084 | 802 | 1467 |
| UNT 182 IN WRC | 3.62826 | 2305 | 1704 | 3117 |
| UNT 182 IN WRC | 3.83487 | 2343 | 1732 | 3169 |
| UNT 183 IN WRC | 0.20432 | 265 | 196 | 359 |
| UNT 184 IN WRC | 1.1499 | 1049 | 776 | 1419 |
| UNT 185 IN WRC | 1.0014 | 782 | 578 | 1057 |
| UNT 185 IN WRC | 1.67477 | 1152 | 852 | 1558 |
| UNT 186 IN WRC | 0.92437 | 608 | 450 | 823 |
| UNT 186 IN WRC | 1.4783 | 841 | 622 | 1138 |
| UNT 187 IN WRC | 1.18075 | 1729 | 1278 | 2338 |
| UNT 188 IN WRC | 1.08928 | 2169 | 1604 | 2933 |
| UNT 188 IN WRC | 2.10925 | 3258 | 2409 | 4407 |
| UNT 188 IN WRC | 4.66768 | 4181 | 3091 | 5654 |
| UNT 189 IN WRC | 1.48699 | 1755 | 1298 | 2373 |
| UNT 189 IN WRC | 2.0946 | 2083 | 1540 | 2816 |
| UNT 190 IN WRC | 0.2718 | 509 | 376 | 688 |
| UNT 190 IN WRC | 0.91119 | 1429 | 1056 | 1932 |
| UNT 190 IN WRC | 1.95626 | 2706 | 2001 | 3659 |
| UNT 190 IN WRC | 3.16553 | 3773 | 2790 | 5102 |
| UNT 190 IN WRC | 4.04354 | 4028 | 2979 | 5448 |
| UNT 191 IN WRC | 1.22941 | 2027 | 1499 | 2741 |
| UNT 191 IN WRC | 1.69674 | 2505 | 1852 | 3387 |
| UNT 191 IN WRC | 3.03667 | 3824 | 2827 | 5171 |
| UNT 192 IN WRC | 0.68207 | 1056 | 781 | 1429 |
| UNT 192 IN WRC | 0.70312 | 1076 | 796 | 1456 |
| UNT 192 IN WRC | 1.63039 | 2116 | 1565 | 2862 |
| UNT 192 IN WRC | 3.29921 | 3627 | 2682 | 4905 |
| UNT 192 IN WRC | 5.2772 | 4900 | 3623 | 6627 |
| UNT 193 IN WRC | 0.54728 | 939 | 694 | 1269 |
| UNT 193 IN WRC | 0.61462 | 1040 | 769 | 1406 |
| UNT 194 IN WRC | 0.42835 | 622 | 460 | 841 |
| UNT 194 IN WRC | 0.49055 | 756 | 559 | 1023 |
| UNT 194 IN WRC | 0.53125 | 809 | 598 | 1095 |
| UNT 194 IN WRC | 1.01809 | 1469 | 1086 | 1987 |
| UNT 194 IN WRC | 1.29887 | 1753 | 1297 | 2371 |
| UNT 195 IN WRC | 0.49889 | 955 | 706 | 1291 |
| UNT 195 IN WRC | 0.65463 | 1106 | 818 | 1495 |
| UNT 195 IN WRC | 1.51309 | 2054 | 1518 | 2777 |
| UNT 195 IN WRC | 1.56913 | 2278 | 1684 | 3080 |
| UNT 196 IN WRC | 0.5663 | 1240 | 917 | 1677 |
| UNT 197 IN WRC | 1.00051 | 1634 | 1208 | 2210 |
| UNT 197 IN WRC | 2.022 | 2755 | 2037 | 3726 |
| UNT 197 IN WRC | 2.19238 | 2756 | 2038 | 3727 |
| UNT 197 IN WRC | 2.85819 | 3434 | 2540 | 4645 |
| UNT 197 IN WRC | 3.64547 | 3929 | 2905 | 5314 |
| UNT 198 IN WRC | 0.46441 | 909 | 672 | 1230 |
| UNT 198 IN WRC | 1.89887 | 2369 | 1752 | 3204 |
| UNT 199 IN WRC | 0.44636 | 1071 | 792 | 1448 |
| UNT 200 IN WRC | 0.99205 | 812 | 601 | 1099 |
| UNT 200 IN WRC | 1.50044 | 1150 | 850 | 1555 |
| UNT 200 IN WRC | 1.68491 | 1505 | 1113 | 2036 |
| UNT 201 IN WRC | 1.01925 | 1059 | 783 | 1432 |
| UNT 201 IN WRC | 1.43857 | 1302 | 963 | 1761 |
| UNT 202 IN WRC | 1.23367 | 1113 | 823 | 1506 |
| UNT 202 IN WRC | 1.41176 | 1261 | 932 | 1705 |
| UNT 203 IN WRC | 1.00006 | 1107 | 819 | 1497 |
| UNT 203 IN WRC | 2.12109 | 2018 | 1493 | 2730 |
| UNT 203 IN WRC | 2.42224 | 2018 | 1493 | 2730 |
| UNT 203 IN WRC | 4.70292 | 3248 | 2402 | 4393 |
| UNT 203 IN WRC | 5.89659 | 3790 | 2802 | 5125 |
| UNT 204 IN WRC | 1.18891 | 893 | 660 | 1208 |
| UNT 204 IN WRC | 1.984 | 1275 | 942 | 1724 |
| UNT 205 IN WRC | 1.01776 | 961 | 711 | 1300 |
| UNT 206 IN WRC | 1.00239 | 780 | 576 | 1054 |
| UNT 206 IN WRC | 1.99733 | 1351 | 999 | 1827 |
| UNT 206 IN WRC | 3.07908 | 1777 | 1314 | 2403 |
| UNT 207 IN WRC | 0.96876 | 952 | 704 | 1287 |
| UNT 207 IN WRC | 1.77691 | 1562 | 1155 | 2113 |
| UNT 207 IN WRC | 2.58982 | 1963 | 1452 | 2655 |
| UNT 207 IN WRC | 3.1205 | 2109 | 1560 | 2852 |
| UNT 207 IN WRC | 3.22944 | 2239 | 1656 | 3029 |
| UNT 208 IN WRC | 1.00239 | 842 | 623 | 1139 |
| UNT 208 IN WRC | 1.76567 | 1184 | 875 | 1601 |
| UNT 209 IN WRC | 1.1508 | 1084 | 801 | 1466 |
| UNT 210 IN WRC | 1.0414 | 1072 | 793 | 1450 |
| UNT 210 IN WRC | 1.61747 | 1386 | 1025 | 1874 |
| UNT 210 IN WRC | 2.43737 | 1892 | 1399 | 2559 |
| UNT 210 IN WRC | 3.39663 | 2365 | 1749 | 3199 |
| UNT 211 IN WRC | 1.00266 | 1452 | 1074 | 1964 |
| UNT 211 IN WRC | 1.69056 | 2217 | 1639 | 2998 |
| UNT 212 IN WRC | 0.91352 | 1090 | 806 | 1475 |
| UNT 212 IN WRC | 1.10758 | 1245 | 921 | 1684 |
| UNT 213 IN WRC | 1.01907 | 991 | 732 | 1340 |
| UNT 213 IN WRC | 1.56385 | 1351 | 999 | 1826 |
| UNT 214 IN WRC | 1.00616 | 1007 | 745 | 1362 |
| UNT 214 IN WRC | 1.55416 | 1538 | 1137 | 2079 |
| UNT 215 IN WRC | 1.30782 | 1649 | 1219 | 2230 |
| UNT 215 IN WRC | 2.22107 | 2525 | 1867 | 3415 |
| UNT 215 IN WRC | 3.37851 | 3614 | 2672 | 4887 |
| UNT 216 IN WRC | 1.01342 | 1356 | 1003 | 1834 |
| UNT 216 IN WRC | 1.9638 | 2279 | 1685 | 3082 |
| UNT 216 IN WRC | 2.29515 | 2613 | 1932 | 3534 |
| UNT 217 IN WRC | 1.01925 | 2311 | 1709 | 3126 |
| UNT 217 IN WRC | 1.40503 | 2547 | 1883 | 3444 |
| UNT 218 IN WRC | 1.41301 | 2145 | 1586 | 2901 |
| UNT 218 IN WRC | 1.82381 | 2402 | 1776 | 3248 |
| UNT 219 IN WRC | 0.95387 | 2291 | 1694 | 3098 |
| UNT 219 IN WRC | 1.56214 | 3031 | 2242 | 4100 |
| UNT 220 IN WRC | 1.27335 | 1217 | 900 | 1645 |
| UNT 221 IN WRC | 1.00176 | 1113 | 823 | 1506 |
| UNT 221 IN WRC | 1.72474 | 1910 | 1413 | 2584 |
| UNT 222 IN WRC | 0.95818 | 895 | 662 | 1210 |
| UNT 222 IN WRC | 1.8969 | 1441 | 1066 | 1949 |
| UNT 222 IN WRC | 2.88637 | 1968 | 1455 | 2662 |
| UNT 222 IN WRC | 3.26095 | 1979 | 1463 | 2676 |
| UNT 223 IN WRC | 0.57129 | 609 | 451 | 824 |
| UNT 224 IN WRC | 0.2189 | 235 | 174 | 318 |
| UNT 225 IN WRC | 0.50214 | 476 | 352 | 644 |
| UNT 226 IN WRC | 1.00553 | 825 | 610 | 1116 |
| UNT 226 IN WRC | 1.90542 | 1286 | 951 | 1740 |
| UNT 226 IN WRC | 2.36124 | 1504 | 1112 | 2034 |
| UNT 228 IN WRC | 0.66378 | 608 | 450 | 822 |
| UNT 229 IN WRC | 0.43221 | 463 | 343 | 627 |
| UNT 229 IN WRC | 0.5809 | 570 | 422 | 771 |
| UNT 240 IN WRC | 0.52926 | 611 | 452 | 827 |
| UNT 301 IN WRC | 0.23092 | 248 | 183 | 336 |
| UNT 302 IN WRC | 0.85983 | 1052 | 778 | 1423 |
| UNT 302 IN WRC | 0.94338 | 1115 | 825 | 1508 |
| UNT 303 IN WRC | 0.34476 | 558 | 412 | 754 |
| UNT 500 IN WRC | 0.65239 | 650 | 480 | 879 |
| UNT 501 IN WRC | 0.9285 | 1189 | 880 | 1609 |
| UNT 502 IN WRC | 0.31467 | 539 | 398 | 728 |
| UNT 505 IN WRC | 1.01728 | 1467 | 1085 | 1984 |
| UNT 510 IN WRC | 0.93307 | 807 | 597 | 1091 |
| UNT 511 IN WRC | 1.03584 | 1163 | 860 | 1573 |
| UNT 515 IN WRC | 0.91097 | 715 | 528 | 966 |
| UNT 525 IN WRC | 0.07571 | 114 | 84 | 154 |
| UNT 526 IN WRC | 0.02771 | 36 | 27 | 49 |
| UNT 530 IN WRC | 0.36704 | 425 | 315 | 575 |
| UNT 532 IN WRC | 0.37998 | 472 | 349 | 639 |
| UNT 534 IN WRC | 0.56406 | 563 | 416 | 761 |
| UNT 600 IN WRC | 0.21863 | 252 | 186 | 341 |
| UNT 601 IN WRC | 0.7756 | 863 | 638 | 1167 |
| UNT 602 IN WRC | 0.357 | 439 | 325 | 594 |
| UNT 602 IN WRC | 0.64665 | 681 | 504 | 921 |
| UNT 605 IN WRC | 0.23701 | 259 | 191 | 350 |
| UNT 606 IN WRC | 0.03192 | 23 | 17 | 32 |
| UNT 607 IN WRC | 0.0278 | 49 | 36 | 66 |
| UNT 608 IN WRC | 0.11276 | 205 | 152 | 278 |
| UNT 608 IN WTC | 0.07981 | 205 | 152 | 278 |
| UNT 608 IN WTC | 0.08251 | 205 | 152 | 278 |
| UNT 609 IN WRC | 0.13314 | 193 | 143 | 261 |
| UNT 610 IN WRC | 0.32166 | 521 | 385 | 705 |
| UNT 611 IN WRC | 0.92347 | 783 | 579 | 1059 |
| UNT 612 IN WRC | 0.92365 | 1163 | 860 | 1573 |
| UNT 613 IN WRC | 1.10254 | 1237 | 915 | 1673 |
| UNT 614 IN WRC | 0.98161 | 913 | 675 | 1235 |
| UNT 615 IN WRC | 0.90676 | 909 | 672 | 1229 |
| UNT 617 IN WRC | 0.58845 | 647 | 479 | 875 |
| UNT 618 IN WRC | 0.17248 | 240 | 177 | 324 |
| UNT 620 IN WRC | 0.08618 | 124 | 91 | 167 |
| UNT 621 IN WRC | 1.07448 | 839 | 620 | 1135 |
| UNT 621 IN WRC | 2.53554 | 1562 | 1155 | 2112 |
| UNT 621 IN WRC | 3.06997 | 1777 | 1314 | 2403 |
| UNT 621 IN WRC | 4.72238 | 2266 | 1675 | 3064 |
| UNT 621 IN WRC | 5.0123 | 2501 | 1849 | 3382 |
| WHITE OAK CREEK | 1.92954 | 1705 | 1261 | 2305 |
| WHITE OAK CREEK | 2.79401 | 2144 | 1586 | 2900 |
| WHITE OAK CREEK | 3.79326 | 2605 | 1926 | 3523 |
| WHITE OAK CREEK | 4.71252 | 3026 | 2237 | 4092 |
| WHITE OAK CREEK | 4.92756 | 3082 | 2279 | 4168 |
| WHITE OAK CREEK | 5.78037 | 3385 | 2503 | 4578 |
| WHITE OAK CREEK | 6.75236 | 3768 | 2786 | 5096 |
| WHITE OAK CREEK | 7.4616 | 3948 | 2919 | 5339 |
| WHITE OAK CREEK | 10.89221 | 5350 | 3956 | 7235 |
| WHITE ROCK CREEK | 1.09215 | 1202 | 889 | 1626 |
| WHITE ROCK CREEK | 2.06773 | 1970 | 1457 | 2664 |
| WHITE ROCK CREEK | 2.27461 | 1970 | 1457 | 2664 |
| WHITE ROCK CREEK | 3.01489 | 2290 | 1693 | 3096 |
| WHITE ROCK CREEK | 3.67973 | 2685 | 1986 | 3632 |
| WHITE ROCK CREEK | 4.41444 | 2866 | 2119 | 3875 |
| WHITE ROCK CREEK | 4.88667 | 3163 | 2339 | 4278 |
| WHITE ROCK CREEK | 7.61685 | 4305 | 3183 | 5822 |
| WHITE ROCK CREEK | 14.60428 | 7423 | 5489 | 10039 |
| WHITE ROCK CREEK | 15.90879 | 7634 | 5645 | 10324 |
| WHITE ROCK CREEK | 17.45339 | 8141 | 6020 | 11010 |
| WHITE ROCK CREEK | 17.45993 | 8141 | 6020 | 11010 |
| WHITE ROCK CREEK | 19.01813 | 8171 | 6042 | 11050 |
| WHITE ROCK CREEK | 19.52435 | 8269 | 6114 | 11182 |
| WHITE ROCK CREEK | 20.5487 | 8502 | 6287 | 11499 |
| WHITE ROCK CREEK | 21.46953 | 8738 | 6461 | 11817 |
| WHITE ROCK CREEK | 28.2674 | 9875 | 7302 | 13355 |
| WHITE ROCK CREEK | 30.99824 | 10457 | 7732 | 14142 |
| WHITE ROCK CREEK | 32.51756 | 10457 | 7732 | 14142 |
| WHITE ROCK CREEK | 34.15598 | 10730 | 7934 | 14511 |
| WHITE ROCK CREEK | 34.36486 | 10781 | 7972 | 14580 |
| WHITE ROCK CREEK | 36.40624 | 11138 | 8236 | 15062 |
| WHITE ROCK CREEK | 54.78795 | 15120 | 11180 | 20448 |
| WHITE ROCK CREEK | 56.50916 | 15120 | 11180 | 20448 |
| WHITE ROCK CREEK | 64.24166 | 16250 | 12016 | 21976 |
| WHITE ROCK CREEK | 65.05716 | 16286 | 12043 | 22025 |
| WHITE ROCK CREEK | 68.1707 | 16652 | 12313 | 22520 |
| WHITE ROCK CREEK | 70.67625 | 16844 | 12455 | 22780 |
| WHITE ROCK CREEK | 71.27516 | 16844 | 12455 | 22780 |
| WHITE ROCK CREEK | 74.70982 | 17160 | 12689 | 23207 |
| WHITE ROCK CREEK | 74.74973 | 17160 | 12689 | 23207 |
| WHITE ROCK CREEK | 83.00557 | 18310 | 13539 | 24762 |
| WHITE ROCK CREEK | 83.05049 | 18310 | 13539 | 24762 |
| WHITE ROCK CREEK | 84.80912 | 18310 | 13539 | 24762 |
| WHITE ROCK CREEK | 86.92072 | 18579 | 13738 | 25126 |
| WHITE ROCK CREEK | 114.88569 | 22638 | 16739 | 30615 |
| WHITE ROCK CREEK | 117.11002 | 22638 | 16739 | 30615 |
| WHITE ROCK CREEK | 117.93539 | 22638 | 16739 | 30615 |
| WHITE ROCK CREEK | 119.72238 | 22638 | 16739 | 30615 |
| WHITE ROCK CREEK | 120.75316 | 22638 | 16739 | 30615 |
| WHITE ROCK CREEK | 204.13053 | 31829 | 23536 | 43045 |
| WHITE ROCK CREEK | 208.9598 | 32045 | 23695 | 43337 |
| WHITE ROCK CREEK | 219.94663 | 32901 | 24328 | 44495 |
| WHITE ROCK CREEK | 220.59229 | 32901 | 24328 | 44495 |
| WHITE ROCK CREEK | 221.92821 | 32901 | 24328 | 44495 |
| WHITE ROCK CREEK | 226.59864 | 32901 | 24328 | 44495 |
| WHITE ROCK CREEK | 293.74865 | 38247 | 28281 | 51724 |
| WHITE ROCK CREEK | 369.74163 | 44108 | 32615 | 59650 |
| WHITE ROCK CREEK | 372.05498 | 44108 | 32615 | 59650 |
| WHITE ROCK CREEK | 386.48542 | 45071 | 33327 | 60953 |
| WHITE ROCK CREEK | 390.57012 | 45091 | 33342 | 60981 |
| WHITE ROCK CREEK | 391.69052 | 45176 | 33404 | 61095 |
| WHITE ROCK CREEK | 392.30901 | 45176 | 33404 | 61095 |
| WHITE ROCK CREEK | 395.36595 | 45176 | 33404 | 61095 |
| WHITE ROCK CREEK | 401.11206 | 45295 | 33492 | 61256 |
| WHITE ROCK CREEK | 401.95761 | 45295 | 33492 | 61256 |
| WHITE ROCK CREEK | 402.77697 | 45295 | 33492 | 61256 |
| WHITE ROCK CREEK | 406.43787 | 45531 | 33667 | 61575 |
| WHITE ROCK CREEK | 409.43302 | 45548 | 33680 | 61598 |
| WHITE ROCK CREEK | 410.59898 | 45548 | 33680 | 61598 |
| WHITE ROCK CREEK | 419.44014 | 46077 | 34071 | 62314 |
| WHITE ROCK CREEK | 419.9877 | 46077 | 34071 | 62314 |
| WHITE ROCK CREEK | 505.28823 | 53056 | 39231 | 71752 |
| WHITE ROCK CREEK | 505.35601 | 53056 | 39231 | 71752 |
| WHITE ROCK CREEK | 505.78093 | 53056 | 39231 | 71752 |
| WHITE ROCK CREEK | 506.74732 | 53056 | 39231 | 71752 |

BLE Map