# TWDB 2D BLE – DMF2 
H&H Technical Notebook

Description: Project Name: DMF2

Project Number: 60670387

Company/City: AECOM/Austin, TX

Client: Texas Water Development Board (TWDB)

Brief Model Description: This HEC-RAS model represents the 2D Hydrology and hydraulic model for Double Mountain Fork Brazos River 2 study area , which is located in the Double Mountain Fork Brazos  (HUC-8_12050004). The model was developed using HEC-RAS 2D version 6.1 for the 10PAC, 4PAC,2 PAC, 1 PAC, 1 PAC_Minus, 1PAC_Plus, and 0.2PAC. The model was developed for the Texas Water Development Board (TWDB) as part of the Task Order 1 (TO1) Flood Risk Base Level Engineering Project.

Model Version: HEC-RAS v6.1 (Unsteady Flow)

Study Date: December 2022

Topographic Data Source/Date: 1 meter (3.28 -ft DEM 2019)

Vertical Datum: NAVD88

# Terrain

DMF2 work area Terrain completed end of April 2022.

The 2019 LiDAR Topographic data were downloaded from TNRIS GIS Data Repository.

The terrain has a 1-meter (3.28 ft) resolution.

# Hydrology

- The study area has approximately 548 sq.mi drainage area.

- A spatially distributed precipitation Data Storage System (DSS) data was generated from the National Oceanic and Atmospheric Administration (NOAA) Atlas 14 gridded precipitation for the 10%,4%, 2%, 1%, +1%, -1%, and 0.2% annual-chance events.

- An infiltration layer containing attributes for Soil Conservation Service (SCS; now the Natural Resources Conservation Service (NRCS)) curve numbers (CN), abstraction ratio, and minimum infiltration rate was calculated in ArcGIS using the National Land Cover Database (NLCD, 2019) and NRCS soil data.

- The NRCS synthetic temporal distributions of rainfall for the 10-, 4-, 2-, 1-, and 0.2-percent annual chance 24-hour precipitation depths were developed. The 1-percent annual chance temporal distribution was used for the 1%-Plus and 1%-Minus frequencies.

# Hydraulics

- Inflow from upstream DMF1 work area was used as the inflow. .

- Outflow was used for the downstream DMF3 work area.

- The following boundary conditions were implemented.

| - Boundary Condition Name | - Slope/ Hydrograph |
| --- | --- |
| - Inflow_DMF_402 | - Flow Hydrograph |
| - BC_1 | - Normal Depth (0.0001) |
| - BC_2 | - Normal Depth (0.0001) |
| - BC_3 | - Normal Depth (0.0001) |
| - BC_4 | - Normal Depth (0.0001) |
| - BC_5 | - Normal Depth (0.0001) |
| - BC_6 | - Normal Depth (0.0001) |
| - BC_7 | - Normal Depth (0.0001) |
| - BC_8 | - Normal Depth (0.0001) |
| - Outflow_DMF_405 | - Normal Depth (0.0001) |

- The following Manning’s N values were implemented from the land cover data.

| Land Cover | Manning's N Value |
| --- | --- |
| Mixed Forest | 0.16 |
| Barren land | 0.025 |
| Open Water | 0.04 |
| Herbaceous | 0.035 |
| Developed Open Space | 0.04 |
| Shrub Scrub | 0.1 |
| Developed Low Intensity | 0.08 |
| Developed Medium Intensity | 0.1 |
| Cultivated Crops | 0.035 |
| Evergreen Forest | 0.16 |
| Developed High Intensity | 0.15 |
| Woody Wetlands | 0.12 |
| Emergent Herbaceous Wetlands | 0.07 |
| Hay Pasture | 0.03 |
| Deciduous Forest | 0.16 |
| Stream | 0.04 |

- A Manning’s N value of 0.04 was utilized for channels.

- A terrain modification was used for Lake Alan Henry to adjust the lake elevation to 2216.2. This was taken from the TWDB report “Volumetric and Sedimentation Survey of Alan Henry Resevoir” from the August 2017survey. This was used to help calibrate the model along with the CN to try and to get as close as possible to the gage data while not activating the emergency spill way on the Lake Alan Henry dam.

# Mapping

- No comments.

# Calibration

The following sources of data were utilized for calibration.

- Flow Regression Analysis based on SIR 2009-5087 (L Moment Based) with user-defined watershed

| - Basin Name | - Basin Outlet Location | - Regression Equation Results - Q100 | - Lower Limit - 1%PAC | - Upper Limit - 1% PAC | - Corresponding HEC-RAS 2D 1PAC WSEL using Profile Lines |
| --- | --- | --- | --- | --- | --- |
| - Basin 1 | - 101°1'31.021"W 32°57'28.296"N | - 2,958 | - 1,676.98 | - 8,933.74 | - 9097.51 |
| - Basin 2 | - 101°24'52.309"W 33°2'21.333"N | - 11,287 | - 6,722.43 | - 34,087.59 | - 11202.2 |
| - Basin 3 | - 101°16'58.76"W 33°5'4.346"N | - 8,289 | - 4,804.35 | - 25,033.04 | - 24717.13 |

- Gage

- CN value was increased by 10.

| Land Cover | Original | Adjusted |
| --- | --- | --- |
| Mixed Forest | 55 | 65 |
| Barren land | 80 | 90 |
| Open Water | 99 | 99 |
| Herbaceous | 85 | 95 |
| Developed Open Space | 69 | 79 |
| Shrub Scrub | 48 | 58 |
| Developed Low Intensity | 75 | 85 |
| Developed Medium Intensity | 88 | 98 |
| Cultivated Crops | 67 | 77 |
| Evergreen Forest | 70 | 80 |
| Developed High Intensity | 92 | 99 |
| Woody Wetlands | 87 | 97 |
| Emergent Herbaceous Wetlands | 93 | 99 |
| Hay Pasture | 84 | 94 |
| Deciduous Forest | 77 | 87 |

# Other Assumptions

- N/A