# TWDB 2D BLE – DMF3 
H&H Technical Notebook

Description: Project Name: DMF3

Project Number: 60670387

Company/City: AECOM/Austin, TX

Client: Texas Water Development Board (TWDB)

Brief Model Description: This HEC-RAS model represents the 2D Hydrology and hydraulic model for Double Mountain Fork Brazos River 3 study area , which is located in the Double Mountain Fork Brazos  (HUC-8_12050004). The model was developed using HEC-RAS 2D version 6.1 for the 10PAC, 4PAC,2 PAC, 1 PAC, 1 PAC_Minus, 1PAC_Plus, and 0.2PAC. The model was developed for the Texas Water Development Board (TWDB) as part of the Task Order 1 (TO1) Flood Risk Base Level Engineering Project.

Model Version: HEC-RAS v6.1 (Unsteady Flow)

Study Date: December 2022

Topographic Data Source/Date: 1 meter (3.28 -ft DEM 2019)

Vertical Datum: NAVD88

# Terrain

DMF3 workarea Terrain completed end of April 2022.

The 2019 LiDAR Topographic data were downloaded from TNRIS GIS Data Repository.

The terrain has a 1-meter (3.28 ft) resolution.

# Hydrology

- The study area has approximately 525 sq.mi drainage area.

- A spatially distributed precipitation Data Storage System (DSS) data was generated from the National Oceanic and Atmospheric Administration (NOAA) Atlas 14 gridded precipitation for the 10%,4%, 2%, 1%, +1%, -1%, and 0.2% annual-chance events.

- An infiltration layer containing attributes for Soil Conservation Service (SCS; now the Natural Resources Conservation Service (NRCS)) curve numbers (CN), abstraction ratio, and minimum infiltration rate was calculated in ArcGIS using the National Land Cover Database (NLCD, 2019) and NRCS soil data.

- The NRCS synthetic temporal distributions of rainfall for the 10-, 4-, 2-, 1-, and 0.2-percent annual chance 24-hour precipitation depths were developed. The 1-percent annual chance temporal distribution was used for the 1%-Plus and 1%-Minus frequencies.

- Stream naming: did not have NFHL data, so used NHD flowlines for naming.

# Hydraulics

- Inflows from upstream DMF2 & NFDMFB3 (Task Order 11) workareas were incorporated.

- Outflow going into DMF4 workarea was incorporated into the downstream model

- The following boundary conditions were implemented.

| - Boundary Condition Name | - Slope/ Hydrograph |
| --- | --- |
| - Inflow_DMFBR_405 | - Flow Hydrograph |
| - Inflow_NFDMFBR_405 | - Flow Hydrograph |
| - BC_1 | - Normal Depth (0.0001) |
| - BC_2 | - Normal Depth (0.0001) |
| - BC_3 | - Normal Depth (0.0001) |
| - BC_4 | - Normal Depth (0.0001) |
| - BC_5 | - Normal Depth (0.0001) |
| - BC_6 | - Normal Depth (0.0001) |
| - BC_7 | - Normal Depth (0.0001) |
| - BC_8 | - Normal Depth (0.0001) |
| - Outflow_DMFBR_406 | - Normal Depth (0.0001) |

- The following Manning’s N values were implemented from the land cover data.

| Land Cover | Manning's N Value |
| --- | --- |
| Mixed Forest | 0.16 |
| Barren land | 0.025 |
| Open Water | 0.04 |
| Herbaceous | 0.035 |
| Developed Open Space | 0.04 |
| Shrub Scrub | 0.1 |
| Developed Low Intensity | 0.08 |
| Developed Medium Intensity | 0.1 |
| Cultivated Crops | 0.035 |
| Evengreen Forest | 0.16 |
| Developed High Intensity | 0.15 |
| Woody Wetlands | 0.12 |
| Emergent Herbaceous Wetlands | 0.07 |
| Hay Pasture | 0.03 |
| Decideous Forest | 0.16 |
| Stream | 0.04 |

- A Manning’s N value of 0.04 was utilized for channels.

- A calibration Manning N region is added for stream with a value of 0.03 to get WSEL tie-in with DMFBR3.

# Mapping

- No comments.

# Calibration

The following sources of data were utilized for calibration.

- Flow Regression Analysis based on SIR 2009-5087 (L Moment Based) with user-defined watershed

| - Basin Name | - Basin Outlet Location | - Regression Equation Results - Q100 | - Lower Limit - 1%PAC | - Upper Limit - 1% PAC | - Corresponding HEC-RAS 2D 1PAC WSEL using References Lines |
| --- | --- | --- | --- | --- | --- |
| - Basin 2 | - 100°49'39.173"W 32°50'8.247"N | - 11,441 | - 5,734 | - 22,828 | - 19430 |
| - Basin 3 | - 100°41'37.223"W 32°58'12.873"N | - 7,537 | - 3,778 | - 15,039 | - 7113 |
| - Basin 4 | - 100°40'36.402"W 32°51'3.273"N | - 3,446 | - 1,727 | - 6,875 | - 2823 |
| - Basin 5 | - 100°38'37.665"W 32°55'20.826"N | - 3,192 | - 1,600 | - 6,368 | - 2553 |
| - Basin 6 | - 100°46'5.445"W 32°59'25.173"N | - 5,218 | - 2,615 | - 10,411 | - 4408 |
| - Basin 7 | - 100°42'4.182"W 32°51'25.271"N | - 3,030 | - 1,519 | - 6,046 | - 2384 |
| - Basin 8 | - 100°35'54.91"W 32°57'36.094"N | - 2,632 | - 1,319 | - 5,251 | - 1654 |
| - Basin 9 | - 100°53'24.297"W 33°5'24.132"N | - 7,025 | - 3,521 | - 14,016 | - 9960 |
| - Basin 10 | - 100°47'19.999"W 32°51'36.275"N | - 9,309 | - 4,665 | - 18,574 | - 13293 |

- CN values throughout the study area were decreased by 22.

| Land Cover | Original | Adjusted |
| --- | --- | --- |
| Mixed Forest | 55 | 38.5 |
| Barren land | 80 | 56 |
| Open Water | 99 | 69.3 |
| Herbaceous | 85 | 59.5 |
| Developed Open Space | 84 | 58.8 |
| Shrub Scrub | 73 | 51.1 |
| Developed Low Intensity | 83 | 58.1 |
| Developed Medium Intensity | 91 | 63.7 |
| Cultivated Crops | 76 | 53.2 |
| Evengreen Forest | 77 | 53.9 |
| Developed High Intensity | 94 | 65.8 |
| Woody Wetlands | 93 | 65.1 |
| Emergent Herbaceous Wetlands | 93 | 65.1 |
| Hay Pasture | 61 | 42.7 |
| Decideous Forest | 55 | 38.5 |

# Other Assumptions

- N/A