# Texas 2D BLE Analysis QA/QC Checklist

# Project Information

| HUC-8 Watershed Name & ID: | Double Mountain Fork Brazos: 12050004 |
| --- | --- |
| HUC-10 Watershed Name & ID: | Town of Whiteface-Town of Clauene: 1205000401 City of Levelland-Tahoka Lake: 1205000402 Double Lakes-Double Mountain Fork Brazos River: 1205000403 |
| Modeler: | Mafuzur Rahaman, PE |
| Reviewer: | Seyoum Asamenaw, PE, CFM, PE, D.WRE |

## Terrain and Modeling Review (QC Items 1-6)

| Modeler Initials: | MR, SA | Reviewer Initials: | AC |
| --- | --- | --- | --- |
| Submittal Date: | 2/17/23 | Review Submitted Date: | Received 3/8/23 |
| Review Response Date: |  | Backcheck Complete Date: | 8/2/2023 |

## Mapping Review (QC Item 7)

| Modeler Initials: | MR, ES, SA | Reviewer Initials: | AC/ EA/JM |
| --- | --- | --- | --- |
| Submittal Date: | 2/17/23 | Review Submitted Date: | Received 3/8/23 |
| Review Response Date: |  | Backcheck Complete Date: | 8/2/2023 |

## Completeness Review (QC Item 8)

| Modeler Initials: | MR, ES, SDS | Reviewer Initials: | AC/ EA/JM |
| --- | --- | --- | --- |
| Submittal Date: | 6/20/23 | Review Submitted Date: | 8/2/2023 |
| Review Response Date: | 08/30/23 | Backcheck Complete Date: |  |

(Modeler to populate with an “X” for items included in the QC submittal)

| Level of Study |  |  | Flood Events |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| X | 2D BLE |  | X | 10% ACE (10-year) | X | 0.2% ACE (500-year) |
|  | Enhanced BLE |  | X | 4% ACE (25-year) | X | 1% plus |
|  |  |  | X | 2% ACE (50-year) | X | 1% minus |
|  |  |  | X | 1% ACE (100-year) |  | Observed Rainfall |

Instructions

Each section includes instructions to the modeler and reviewer regarding their responsibilities in the completion of this checklist. In the Methodologies / Data Sources tables, grey italic text is added as an example response. Additional sample responses are located in the Texas 2D BLE Guidelines Quick Guide.

Reviewer to indicate if Technical Review items generally comply using a “Pass”. If the technical review observes an issue of noncompliance or an item that requires additional attention, an “Fail” should be utilized.

## Submittal Checklist

Data submitted for each milestone should be carried through each subsequent milestone. All geospatial layers and models must have same defined projection system as terrain. Modeler to populate with an “X” for items included in the QC submittal.

| Base Map Data (QC items 1-6) |  |
| --- | --- |
| X | Location Map (PDF): HUC-8 and HUC-10 boundaries and/or model domain boundary if different from HUC-10s, stream gages, dams (if applicable), mainstem streams (for reference), background map (associated shapefiles provided) |
| X | GIS Map (MXD or Map Package) with the following data included but not limited to: Stream centerlines, model breaklines, rainfall data, WSEL and Depth grids (10, 100, 500-yr), adjacent HUC mapping (available on the BLE viewer or currently studying), 2D connections for inflows/outflows, gages or points of flow comparisons (BFEs, highwater marks, etc.), profile lines, boundary condition lines, refinement regions, model domain boundary, model mesh, DFIRM mapping panels, base map data (roads, aerials, etc.), soils data, land use data, terrain. (associated shapefiles provided and also part of HECRAS model submittal) |
| Technical Report / Documentation (QC items 1-6) |  |
| X | Technical Report / Project Notebook: assumptions, special considerations, and modeling concerns |
| X | Detailed explanation and support data of variances in methodologies and approaches from recommended guidelines (Should be included in Technical Report / Project Notebook) |
| X | Internal QA/QC Documentation: comments and responses |
| X | Flow comparison tables within the technical report |
| Hydrology (QC items 1-6) |  |
| NA | Hydrologic Model (HEC-HMS) model with all associated DSS files for 7 required storm events |
| X | Hydrologic Data and Parameter Spreadsheets (rainfall calculations, curve number table, gage locations, gage analysis and flow validation) |
| X | Hydrologic modeling notebook including assumptions, notes, and special conditions (if not provided in report) |
| Hydraulics (QC items 1-6) |  |
| X | Hydraulic Model (HEC-RAS) with all associated files and 7 required storm events (remove non-final files)d |
| X | Hydraulic Data and Parameter Spreadsheets (Manning’s n-value table, infiltration calculations support data-Soils, Land Use, Curve Number table, other calculations/data used in model) |
| X | Manning’s n-value table |
| X | Precipitation data correctly linked to hydrologic model or hard coded from provided spreadsheet |
| X | Hydraulic modeling notebook including assumptions, notes, and special conditions |
| Mapping (QC item 7) |  |
| X | Flood risk database |
| X | Modeler to complete topology validation as outlines in the guidelines |
| X | Modeler to verify mapping meets deliverable requirements as outlined in guidelines |
| Completeness Review (QC item 8) See Texas 2D BLE Guidelines Chapter 4 – Deliverables |  |
| X | Technical BLE Report |
| NA | Final cleaned hydrologic model |
| X | Final cleaned hydraulic model |
| X | Flood risk database |
| X | CMNS updates |
| X | HAZUS results |
| X | Additional supplemental data |

# 1 – Model Terrain

Methodologies / Data Sources (Modeler to populate)

| Terrain Source(s): | LIDAR |
| --- | --- |
| Data Gaps: | N/A |
| Source Resolution: | 1-meter |
| Model Terrain Resolution: | 1-meter |
| Terrain Horizontal Projection: | NAD 1983 State Plane Texas North Central FIPS 4202 |
| Terrain Vertical Projection: | NAVD 88 |
| Z Values: | D_North_American_1983 |
| Terrain Buffer: | 10,000-ft |
| Other: NA |  |

## Technical Review (Reviewer to confirm with Pass/Fail)

| General Review |  |
| --- | --- |
| Pass | LiDAR source matches the latest available data at time of project initiation |
| Pass | Large reservoir or lake extents included in model domain |
| Pass | Check visible seams, gaps or holes of LiDAR sources (if more than one source was used) |

## Review Comments

- No comments

# 2 – Hydrology

## Methodologies / Data Sources (Modeler to populate)

| Software: | HEC-RAS 6.1 |
| --- | --- |
| Rainfall Source: | National Oceanic and Atmospheric Administration (NOAA) Atlas 14 Gridded Precipitation |
| Rainfall Duration: | 24 hours |
| Depth Selection: | Atlas 14 |
| Rainfall Temporal Distribution: | NRCS synthetic temporal distributions |
| Areal Reduction: | Areal Reduction was not used |
| Loss Methodology: | SCS |
| Unit Hydrograph Transform Method: | N/A |
| Inflow Hydrograph Source: | Inflows from upstream DMF2 & NFDMFB3 (Task Order 11) into DMF1. |
| Other: A spatially distributed precipitation Data Storage System (DSS) data was generated from the National Oceanic and Atmospheric Administration (NOAA) Atlas 14 gridded precipitation for the 10%,4%, 2%, 1%, +1%, -1%, and 0.2% annual-chance events. |  |

## Statistical Data (Modeler to populate)

| Stream Gage Name & ID: | Not Applicable |
| --- | --- |

## Technical Review (Reviewer to confirm with Pass/Fail)

| Excess Rainfall |  |
| --- | --- |
| Fail | Precipitation values are an aerial average of Atlas 14 values for the entire 2D area boundary for each storm event or other appropriate rainfall application |
| Pass | CN values appropriately reflect land use and soils for entire 2D area boundary |
| Pass | Losses are correctly calculated to establish the excess rainfall hyetographs by checking land use, soils and infiltration layers or other relative information |
| Pass | Total precipitation depths for 1%+ and 1%- are calculated correctly |
| Fail | Excess rainfall hyetographs are calculated for all storm events (10%, 4%, 2%, 1%, 0.2%, 1%+, and 1%-) |
| Pass | Cumulative excess rainfall values are in increasing order from 10%, 4%, 2%, 1%, to 0.2% |
| Pass | Cumulative excess rainfall values for the 1%+ and 1%- fall in order with the storm events matching the total precipitation values |
| Inflow Hydrographs |  |
| Pass | Shape and time-to-peak of any inflow hydrographs resemble past documented large storm events |
| Pass | Hydrographs reasonable when compared to the other return periods. The shapes should be similar, and hydrographs from different events should not intersect unless there is an engineering reason. Peak flows from a lower return period event should not be higher than a higher return period event |
| Pass | Contributing hydrographs from another RAS model applied at coincident locations (Provide locations in base map data) |
| General Hydrology |  |
| No | Any flow regulating dams in the model area? (Yes/No) |
| NA | Dam modeling methodologies applied and explained? (Yes/No). If yes, provide discharge rating curve calculations and gage analysis documenting regulated versus unregulated records, if applicable. |
| NA | Model aligns with documentation of dam modeling approaches. |
| Other Notes: NA |  |

## Review Comments

AECOM Comment: Modeling comments are resolved in the detailed QA/QC spreadsheet that is submitted with this checklist.  Please ignore

Comments

- No Data in RAS Mapper for 50pct

Response

- 50pct is available under results and events conditions

Back Check Comments

- Addressed

# 3 – Hydraulic Geometry

## Methodologies / Data Sources (Modeler to populate)

| Software: | HEC-RAS 6.1 |
| --- | --- |
| Loss Methodology: | SCS Curve Number |
| Soils Source: | NRCS Soil Data |
| Land Cover Source: | National Land Cover Database (NLCD, 2019) |
| Manning's Roughness Source: | National Land Cover Database (NLCD, 2019) |
| Nominal Mesh Cell Sizes: | 200-foot cells, except for 100-feet along the 1 square mile streams and 50-foot cells within community boundaries using refinement regions. |
| Other: |  |

## Technical Review (Reviewer to confirm with Pass/Fail)

| 2D Mesh |  |
| --- | --- |
| Pass | 2D Boundary delineated along watershed boundary or buffer to ensure all contributing area is included |
| Pass | Nominal cell size appropriate for model detail (generally 200 feet, minimum 50 feet for refinement areas) |
| Pass | Computation points are at default locations or changes documented |
| Pass | Mesh Cell and Face tolerances are default or changes documented |
| Pass | No mesh errors |
| Pass | Cell face relationships are calculated |
| Breaklines |  |
| Fail | Breaklines are consistent with the terrain data |
| Fail | Breaklines sufficiently detailed to capture features (thalweg, banks, berms, etc.) |
| Pass | Cells do not overlap terrain features where appropriate (i.e., unwanted leaking) |
| Pass | Breakline cell sizing and parameters are appropriate |
| Pass | 2D Area Connections placed at applicable benchmarks, gages, or dams |
| Pass | 2D Area Connections set to use normal 2D equation domain for benchmarks and gages. Appropriate modeling approach used for dams. |
| Manning’s N-Values |  |
| Pass | Manning’s n-values associated to land cover data provided |
| Pass | Land Cover classifications are sufficiently descriptive within the data table |
| Pass | Manning’s n-value classifications generally match current aerial imagery |
| Pass | Manning’s n dataset is associated with the geometry files |
| General |  |
| Pass | All model datasets each have unique descriptive names |

## Review Comments

AECOM Comment: Modeling comments are resolved in the detailed QA/QC spreadsheet that is submitted with this checklist. Please ignore

- Comments on geometry are included in shapefile

| FID | Id | Comment | Response |  |
| --- | --- | --- | --- | --- |
| 0 | 1 | Consider enforcing these breaklines to avoid leakage. | Breaklines are enforced | Leakage still visible. No further action is required. |
| 1 | 2 | Consider enforcing these breaklines and then adding a v-notch. | Breaklines are enforced and V-notches are added | Ok |
| 2 | 3 |  | Breaklines are revised and enforced | Ok |
| 3 | 4 | Consider enforcing these breaklines and then adding a v-notch. | Breaklines are enforced and V-notches are added | Ok |
| 4 | 5 | Consider enforcing these breaklines and then adding a v-notch. | Breaklines are enforced and V-notches are added | Ok |
| 5 | 6 | Consider adding breakline along high ground and adding v-notch at stream centerline | Breakline is added and enforced and V-notche is added | Ok |
| 6 | 7 | Consider simplifying to only have 1 breakline through here. | Breaklines are revised and enforced | Ok |
| 7 | 8 | Profile line Basin_1 shows 500yr with negative flow. Check other areas. | at start of the simulation water flows upstream direction until playas fill with water, as playas have comparatively lower elevation. Once the playas are filled up, the water starts to flow downstream. | Ok |

# 4 – Hydraulic Boundary Conditions

## Technical Review (Reviewer to confirm with Pass/Fail)

| Boundary Review |  |
| --- | --- |
| Pass | External boundaries connected to 2D mesh |
| Pass | Boundary conditions everywhere flow could build up at the boundary of the 2D mesh |
| Pass | Boundary conditions have a hydrograph or elevation/slope data assigned |
| Pass | Input hydrographs consistent with hydrologic data (DSS, manual entry) |
| Pass | Precipitation data is consistent with hydrologic data (DSS, manual entry) |
| Pass | Downstream boundary consistent with ground slope or known downstream WSEL |
| Pass | Flow hydrographs applied at appropriate locations |
| Pass | Energy grade slope for hydrograph distribution is reasonable |
| Pass | Internal rating curves appropriate and documented for control structures (if applicable) |

## Review Comments

AECOM Comment: Modeling comments are resolved in the detailed QA/QC spreadsheet that is submitted with this checklist. Please ignore

- No comments

# 5 – Hydraulic Model Execution

## Methodologies / Data Sources (Modeler to populate)

| Unsteady Plan Names |  |  |  |
| --- | --- | --- | --- |
| 10% ACE (10-year): | 10pct | 0.2% ACE (500-year): | 0_2pct |
| 4% ACE (25-year): | 4pct | 1% plus: | 1PLUSpct |
| 2% ACE (50-year): | 2pct | 1% minus: | 1MINpct |
| 1% ACE (100-year): | 1pct |  |  |
| Other: |  |  |  |

## Statistical Data (Modeler to populate)

| Time Step: | Controlled by Courant condition. |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Courant Value: | 0.4-1 |  | Simulation Length: | 96 | hours |
| Computation Interval: | 60 | seconds | Model Runtime: | 11.5 | hours |
| Hydrograph Output Interval: | 60 | minutes | Mapping Output Interval: | 60 | minutes |

## Technical Review (Reviewer to confirm with Pass/Fail)

| Execution Review |  |
| --- | --- |
| Pass | Appropriate 2D parameters are used (Boundary Conditions, Time Step, Simulation Length, etc.) |
| Pass | Hydrograph Output Interval and Mapping Output Interval set at a reasonable interval to capture peaks |
| Pass | Model runtime is reasonable (< 12 hours, but dependent upon model size) |

## Review Comments

AECOM Comment: Modeling comments are resolved in the detailed QA/QC spreadsheet that is submitted with this checklist.  Please ignore

- No comments

# 6 – Hydraulic Model Results

## Methodologies / Data Sources (Modeler to populate)

| Flow Validation/Gage Analysis |  |
| --- | --- |
| Software: | HEC-RAS 6.1 |
| Methodology | No USGS Gages or Effective product is available here to calibrate the model. Regression Analysis is also overlooked as it doesn't reflect the effect of playa. Validation was conducted by revising CN values throughout the study area to accommodate the appropriate flow to the downstream model |
| Removal of Outliers and Justification | NA |
| DMF1 work area is well known for playa-driven flood storage. The initial condition for the playa based on the volume of runoff was developed from a 2-year, 1-hour duration rain event. This initial condition completely fills the playa and provides a conservative estimation of potential downstream flood conditions due to little/no playa storage capacity. The end of the 2-year, 1-hour duration rain simulation WSEL was used as an initial condition to run all the events. |  |
| Other: |  |

## Technical Review (Reviewer to confirm with Pass/Fail)

| Computation Checks |  |
| --- | --- |
| Pass | Model runs with no cell errors |
| Pass | Volume balance is within tolerance (ΔV < 1% is ideal, however 2% to 3% can be acceptable for rain on mesh) |
| Results Spot Checks Check at all critical structures, major confluences, a handful of headwater tributaries, downstream end, at benchmarks, and a handful of randomly selected locations. |  |
| Pass | No visible spikes in WSEL visibly present with a single cell |
| Pass | WSEL and depth grids appear reasonable (no artificial dips or spikes) |
| Pass | Visual verification flooding is connected downstream for all significant contributing areas (> 64 acres) |
| Pass | Velocity grids appear reasonable (no unusually high velocity areas or Courant values greater than 2) |
| Fail | No crossing profiles between all events |
| Pass | Inflow and outflow hydrograph timing is reasonable as compared to the peak |
| Pass | Spills into/from adjacent HUCs appropriately accounted for |
| Fail | WSEL tie-in between model domains and HUCs along major rivers/streams where inflows are applied |
| Results Full Checks |  |
| Pass | Computational time step meets ideal Courant condition |
| Pass | Diffusion Wave: V∆T∆X ≤2 (Up to 5) (Results are unit-less) |
| Pass | Model simulation is long enough to pass the entire hydrograph through the model (minimum of 12 hours past the peak, preferably 24 hours) |
| Pass | Hydrographs at internal and external boundaries are smooth |
| Pass | Contributing areas are contained with 2D mesh or accounted for with inflow hydrographs |
| Pass | No flow trapped at 2D boundary |
| Pass | Visual verification mapping is reasonable compared to effective |
| Flow Verification |  |
| NA | Peak flow gage analysis performed on all gages in the watershed with sufficient data (No gage in this study area) |
| NA | All gages used have an appropriate period of record (10 to 20+ years) (No gage in this study area) |
| NA | All gages used have no extensive gaps in record which would impact the analysis (No gage in this study area) |
| NA | Gage analysis results validated to other sources (Regression, USGS, etc.) (No gage in this study area) |
| Pass | Results at applicable benchmarks justified (gages, HWMs, effective BFEs) |
| Pass | Peak discharges comparable to gages, effective/prior studies, regional regression equations |

## Review Comments

AECOM Comment: Modeling comments are resolved in the detailed QA/QC spreadsheet that is submitted with this checklist.  Please ignore

Comments

- Found crossing profiles and area where the 500yr has a big negative drop. Can find location in RAS QC shapefile.

- Some WSEL tie-in issues found. Can find location in Database QC.

Response

- At start of the simulation water flows upstream direction until playas fill with water, as playas have comparatively lower elevation.

Once the playas are filled up, the water starts to flow downstream.

As DMF1 work area is well known for playa-driven flood storage. This kind of scenario can happen repeatedly near to playas. Flow might looks negative, as it goes opposite direction. But there is no profile crossing in the revised model.

- All of them are overlapping model areas between DMFB1 and DMFB2. Expect to correct some tie-in issues that occur in model overlap regions during mapping.

Back Check Comments

- Ok.

- Additional comments will be added in the mapping section if found.

# 7 – Mapping Results

## Technical Review (Reviewer to confirm with Pass/Fail)

| Hydraulic Model |  |
| --- | --- |
| Pass | HEC-RAS model with results, pertinent data and required storm events |
| Flood Risk Database |  |
| Fail | All features fully populated per current FEMA guidelines |
| Pass | Mapping consistent between grids to polygons for all three required mapped events |
| Pass | 10yr smaller than 100yr smaller than 500yr |
| Pass | Holes filled in polygon, depth and WSE grids |
| Fail | Boundary interfaces align matching or identified as AOMI |

## Review Comments

Note: All Database and Mapping Comments will be included in this QC Document.

- Please look at Database QC for extra comments – See responses added to shapefile

Halff Review 7/20/2023: QC shapefile updated with responses to review comments. 1 comment not addressed yet and additional comments have been added.  AECOM: Shapefile “QC_DB.shp” reviewed and additional responses added to all remaining comments. Shapefile to be provided with resubmittal.

- Verify that domains are being used in DTL_STUD_AR – Will confirm domain’s added for final

Halff Review 7/20/2023: Type field missing D_Type domain.  AECOM: Domain has been added.

- No domains used under SHOWN_FIRM or SHOWN_INDX in WTR_LN and WTR_AR - Will confirm domain’s added for final

Halff Review 7/20/2023: Domains not yet updated. “D_TrueFalse” domain should be used for SHOWN_FIRM or SHOWN_INDX fields in WTR_LN and WTR_AR feature classes.  AECOM: Domains have been added.

- In FLD_HAZ_AR, ZONE_SUBTY should use the domain D_Zone_Subtype. Should be "" - empty string for 1% floodplain and "0.2 PCT ANNUAL CHANCE FLOOD HAZARD" domain for 0.2% floodplain. Domain values for D_Zone_Subtype should be used. DUAL_ZONE should also be using a domain. - Will confirm domain’s added for final

Halff Review 7/20/2023: Corrected. No further comment.

- No CNMS or HAZUS data for review – Will be delivered for final. Note that these were anticipated to be part of the Completeness Review submittal (per the checklist at top of document) and as part of QC Item 8 as they’re listed in that item below. Thanks.

Halff Review 7/20/2023: see Section 8 below for CNMs comments  AECOM: Responses added in Section 8 below.

Halff Review 7/20/2023

FLD_HAZ_AR

- Waterbodies that are named and/or larger than 5 acres should be merged into 1% and 0.2% floodplains AECOM:  Noted and edits made to some areas.  Note that water area basemap dataset boundaries are generalized and inaccurate to the DEM compared with new SFHA.  Review/editing of SFHA with DEM and orthophoto at these locations to show full coverage of wet areas.

- Holes that are smaller than 2 acres need to be filled in both 1% and 0.2% floodplains. See QC polygon for example. AECOM: Corrected

TENPCT_FP

- Fill holes smaller than 0.5 acres in size. See QC polygon for example. AECOM: Corrected

GRIDs

- BLE_WSE1PCT grid is larger than BLE_WSE0_2PCT grid in some locations. See QC polygon for example. AECOM: Corrected

# 8 – Completeness Review

## Technical Review (Reviewer to confirm with Pass/Fail)

| File Structure and Naming |  |
| --- | --- |
|  | Deliverables provided in TWDB/FEMA required file folder structure and naming convention |
|  | Data missing (if any) documented in report and ReadMe file within the required folder |
| Report |  |
|  | Technical 2D BLE report documents all assumptions, special considerations, etc. |
|  | Flow validation and difference discussed in report |
|  | Flow comparisons discussed in report |
| Hydrologic Model |  |
|  | HEC-HMS model with results, pertinent data and required storm events |
| Hydraulic Model |  |
|  | HEC-RAS model with results, pertinent data and required storm events |
| Flood Risk Database |  |
|  | All features fully populated per current FEMA guidelines |
| CNMS |  |
|  | Updated correctly with new centerlines |
|  | A1-A5 checks complete |
|  | CNMS Post- Processing Complete |
| HAZUS |  |
|  | Run per FEMA guidance |

## Review Comments

AECOM Comment: Modeling comments are resolved in the detailed QA/QC spreadsheet that is submitted with this checklist.  Please ignore

Halff Review 7/20/2023

FRD

WTR_AR

- One polygon has “FIRM1” instead of “BASE2” in SOURCE_CIT field. Recalculate to “BASE2” to be consistent with the rest of the features. AECOM: Corrected

- SHOWN_FIRM and SHOWN_INDEX fields missing domain D_TrueFalse. AECOM: Corrected

WTR_LN

- Line disconnected. See QC Polygon shapefile for location. – AECOM:  Line segment removed from BLE DB layers.

- SHOWN_FIRM and SHOWN_INDEX fields missing domain D_TrueFalse. AECOM: Corrected

S_AOMI_AR

- AOMI_CLASS field missing domain D_AOMI_Class. AECOM: Corrected

- AOMI_TYP field missing domain D_AOMI_Typ. AECOM: Corrected

CNMS

S_Studies_Ln Attribute table QC:

- Check field for missing Domains. Triballand, Floodway, FBS_CMPLNT missing, Model_2D, BS_Model_2D, Is_Urban all missing domain “D_TRUEFALSE”. AECOM: Will resolve with final delivery

- Double check the CID values for lines. Lines with CO_FIPS 48305 have the wrong CID values according to the  S_Pol_Ar featureclass. – AECOM:  Will resolve with final delivery

- ZoneA lines and ZoneX lines have two different BS_HYDRO_M attributes. Were these modeled differently from each other in your study? AECOM: Fixed attributes to match

- ZoneAE lines have a different date from the rest of the lines in the “BLE_DATE” field. Please make sure this date is the same as the rest of your lines for FEMA to track areas with BLE. – AECOM: Updated date

- Some of the A5_CMT fields are listed as “passes BLE comparison check” when it says Fail in A5_Compare field.  – AECOM:  Corrected A5_CMT field

S_Studies_Ln Geometry QC:

- Zone X lines that are running perpendicular to mainlines and completely contained by zone a floodplain can be deleted. Check for other areas throughout study area. Examples in QC polygon shapefile. AECOM: Corrected QC shapefile, will review for any other locations and correct.

- Delete Zone X tails that are outside the Zone A floodplain and less than 0.25 miles in length. Examples in polygon shapefile. AECOM: Corrected QC shapefile, will review for any other locations and correct.

- Zone X tributaries that are shorter than 0.25 miles in length are considered insignificant milage in CNMS and can be deleted. AECOM: Will delete where appropriate.  If line segment is in WTR_LN, these segments should still be included, per discussion with Region 6 CNMS lead.

- Zone X line is disconnected. See QC polygon shapefile for location. Exists in WTR_LN too. AECOM: Line segment removed from BLE DB layers.

- Mainline clipped and disconnected since it crosses HUC boundary. Reference location in QC Polygon. Recommend asking region 6 CNMS lead if missing segment should be added back in or left as is. AECOM: Added segment back in, per discussion with Region 6 CNMS lead.

S_Unmapped_Ln attribute table QC:

- Check for missing Domains. FEDLAND, TRIBALLAND, and DA_G_1SQMI all missing domain “D_TRUEFALSE” AECOM: Will correct for final delivery.

S_Unmapped_Ln Geometry QC:

- Lines that are shorter than 0.25 miles in length can be deleted. AECOM: Will review for these locations and update/remove as described.

- Are most of the tails in this feature class from the 0.5 sqmi drainage area resolution study lines? Were they removed from S_Studies_Ln since this area is mostly rural and only using 1sqmi drainage area resolution? If this is true, then these don’t need to be included in S_Unmapped_Ln and can be deleted. Otherwise I am not sure why some of these were moved to S_Unmapped_Ln because a lot of them still fall within the extent of the Zone A Floodplain. AECOM: Line segments are not from 0.5 sq mi drainage, but from original Zone X from scoping.  Guidance is to keep them, but put them back to S_Unmapped_Ln. No edits/changes made.

Report

- The report shows that the “Lower Tonk Creek” HUC-12 Passes BLE/Effective Zone A comparison. However, there doesn’t seem to be any Zone A lines in that HUC. So this HUC-12 should be NULL/not evaluated?  AECOM: There are A5 points in the HUC12 from Double Mountain Fork Brazos River, streamline doesn’t run through the HUC, but the width of the floodplain does intersect Lower Tonk Creek. This is how we would evaluate any other stream/watershed with this situation. No edits/changes made.

HAZUS

S_FRAC_AR

- “HAZARD_TYP” missing domain D_Hazard_Typ. Otherwise looks good. AECOM: Corrected

QA/QC Checklist Approval

This 2D Base Level Engineering Analysis QA/QC Review is in compliance with the contract requirements and all task checkpoints are complete.  The independent QC Team has reviewed the analysis, presented review comments to the Production Team Task Leader, and discussed any problems or issues.  The Task Leader, QC Manager, QC Reviewer, and Project Manager have signed the QA/QC Checklist to confirm that all comments have been received, addressed, and documented appropriately.

| QA/QC Team – [TWDB or company name] |  |  |  | Production Team – [add company name] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Seyoum Asamenaw (AECOM) |  | 02/06/2023 |
| QC Manager |  | Date |  | Task Lead |  | Date |
|  |  |  |  | Sean Sutton (AECOM) |  | 08/30/2023 |
| QC Reviewer |  | Date |  | Project Manager |  | Date |