# TWDB 2D BLE – DMF1 
H&H Technical Notebook

Description: Project Name: DMF1

Project Number: 60670387

Company/City: AECOM/Austin, TX

Client: Texas Water Development Board (TWDB)

Brief Model Description: This HEC-RAS model represents the 2D Hydrology and hydraulic model for  Double Mountain Fork Brazos River 1 study area (City of Levelland-Tahoka Lake), which is located in the Double Mountain Fork Brazos  (HUC-8_12050004). The model was developed using HEC-RAS 2D version 6.1 for the 10PAC, 4PAC,2 PAC, 1 PAC, 1 PAC_Minus, 1PAC_Plus, and 0.2PAC. The model was developed for the Texas Water Development Board (TWDB) as part of the Task Order 1 (TO1) Flood Risk Base Level Engineering Project.

Model Version: HEC-RAS v6.1 (Unsteady Flow)

Study Date: December 2022

Topographic Data Source/Date: 1 meter (3.28 -ft DEM 2019)

Vertical Datum: NAVD88

# Terrain

DMF1 workarea Terrain completed end of April 2022.

The 2019 LiDAR Topographic data were downloaded from TNRIS GIS Data Repository.

The terrain has a 1-meter (3.28 ft) resolution.

# Hydrology

- The study area has approximately 953 sq.mi drainage area.

- A spatially distributed precipitation Data Storage System (DSS) data was generated from the National Oceanic and Atmospheric Administration (NOAA) Atlas 14 gridded precipitation for the 10%,4%, 2%, 1%, +1%, -1%, and 0.2% annual-chance events.

- An infiltration layer containing attributes for Soil Conservation Service (SCS; now the Natural Resources Conservation Service (NRCS)) curve numbers (CN), abstraction ratio, and minimum infiltration rate was calculated in ArcGIS using the National Land Cover Database (NLCD, 2019) and NRCS soil data.

- The NRCS synthetic temporal distributions of rainfall for the 10-, 4-, 2-, 1-, and 0.2-percent annual chance 24-hour precipitation depths were developed. The 1-percent annual chance temporal distribution was used for the 1%-Plus and 1%-Minus frequencies.

- Stream naming: did not have NFHL data, so used NHD flowlines for naming.

# Hydraulics

- Outflow going into DMF2 was incorporated into the downstream model

- DMF1 workarea is well known for playa-driven flood storage.  The initial condition for the playa based on the volume of runoff was developed from a 2-year, 1-hour duration rain event. This initial condition completely fills the playa and provides a conservative estimation of potential downstream flood conditions due to little/no playa storage capacity. The end of the 2-year, 1-hour duration rain simulation WSEL was used as an initial condition to run all of the events (10PAC, 4PAC,2 PAC, 1 PAC, 1 PAC_Minus, 1PAC_Plus, and 0.2PAC).

- The following boundary conditions were implemented.

| - Boundary Condition Name | - Slope |
| --- | --- |
| - BC_1 | - 0.0007 |
| - BC_2 | - 0.0007 |
| - Outflow_DMF_402 | - 0.0007 |

- The following Manning’s N values were implemented from the land cover data.

| Land Cover | Manning's N Value |
| --- | --- |
| Developed Open Space | 0.04 |
| Cultivated Crops | 0.035 |
| Shrub Scrub | 0.1 |
| Streams | 0.04 |
| Herbaceous | 0.035 |
| Developed Low Intensity | 0.1 |
| Hay Pasture | 0.03 |
| Developed Medium Intensity | 0.08 |
| Emergent Herbaceous Wetlands | 0.07 |
| Developed High Intensity | 0.15 |
| Barren land | 0.025 |
| Woody Wetlands | 0.12 |
| Open Water | 0.04 |
| Decideous Forest | 0.16 |
| Evengreen Forest | 0.16 |

- A Manning’s N value of 0.04 was utilized for channels.

# Mapping

- No comments.

# Calibration

The following sources of data were utilized for calibration.

- No USGS Gages or Effective product is available here to calibrate the model. Regression Analysis is also overlooked as it doesn't reflect the effect of playa.

- CN values throughout the study area were Revised to accommodate the appropriate flow to the downstream model.

| Land Cover | Original | Adjusted |
| --- | --- | --- |
| Developed Open Space | 69 | 82 |
| Cultivated Crops | 67 | 80 |
| Shrub Scrub | 48 | 61 |
| Herbaceous | 74 | 87 |
| Developed Low Intensity | 75 | 88 |
| Hay Pasture | 74 | 87 |
| Developed Medium Intensity | 88 | 99 |
| Emergent Herbaceous Wetlands | 93 | 99 |
| Developed High Intensity | 92 | 99 |
| Barren land | 80 | 93 |
| Woody Wetlands | 93 | 99 |
| Open Water | 99 | 99 |
| Decideous Forest | 77 | 90 |
| Evengreen Forest | 70 | 83 |

# Other Assumptions

- N/A