# 2D BLE

# Model QM Checklist

# Model Data

Watershed Name:	Running Water Draw (Multiple HUC10s)

Level of Study:	2D BLE

Flood Events:	☐	10%	☐	4%		☐	2%		☐	1%		☐	0.2%

☐	1%	+	☐	1%	-	☐	Observed Rainfall

Modeler’s Name:

Reviewer’s Name:	Daniel Perry

Submitted Items:

All Items should be included in the HEC-RAS model itself to be reviewed.

☒	Hydrology model and/or spreadsheets (add file locations as needed)

File location(s):		W:\Citrix\33000s\33606\006\QAQC\2_RunningWaterDrawQC\FromFNI\Reporting and Documentation\Reporting and Documentation\Internal Reviews\Hydrology

☒	Hydraulic model and/or spreadsheets (add file locations as needed)

File location(s):		W:\Citrix\33000s\33606\006\QAQC\2_RunningWaterDrawQC\FromFNI\Engineering Models\1205000505

☐	Terrain Dataset(s)

File location(s):

☐	Projection File

Projection:

☐	2D Boundaries

☐	2D Breaklines

File location:

☐	Land Cover Data (Manning’s n-values).

File location:

☐	Hydrographs (Internal/External)

File location:

☐	Imagery

☐	Gage Analysis

File location:

☐	Gage Analysis Regional Regression:

File location:

☐	Validation Benchmarks (Gage shapefile/HWMs/BFEs):

File location:

☐	Validation Data:

File location:

☐	Descriptions of assumptions/special situations (modeling notebook)

File location: 		N/A

☐	Effective floodplains shapefile if applicable

☐	Preliminary floodplain runs

Modeler should fill out all above information as well as statistical data for each section below.

Review Items:

QAQC review shapefile name provided with spatially referenced comments:	W:\Citrix\33000s\33606\006\QAQC\2_RunningWaterDrawQC\QC_Shp\Watershed_QC.shp

# Hydrology

Modeler’s Initials:	     	Date Submitted:

Reviewer’s Initials:	DMP	Date Reviewed:	03/23/2022

Modeler’s Initials:	     	Date Responded:

Statistical Data:

Stream gages in model area:

Technical Review:

## Excess Rainfall

☐ 	Precipitation values are an aerial average of Atlas 14 values for the entire 2D area boundary for each storm event

☐	Area reduction is accounted for based on the 2D area size

☐	Composite CN is calculated for the entire 2D area boundary

☐	Losses are correctly calculated to establish the excess rainfall hyetographs

☒	Total precipitation depths for 1%+ and 1%- are calculated correctly

☒	Excess rainfall hyetographs are calculated for all storm events (10%, 4%, 2%, 1%, 0.2%, 1%+, and 1%-)

☒	Cumulative excess rainfall values are in increasing order from 10%, 4%, 2%, 1%, to 0.2%

☒	Cumulative excess rainfall values for the 1%+ and 1%- fall in order with the storm events matching the total precipitation values

## Inflow Hydrographs

☐	Shape and time-to-peak of any inflow hydrographs resemble past documented large storm events

☒	Hydrographs reasonable when compared to the other return periods. The shapes should be similar, and hydrographs from different events should not intersect unless there is an engineering reason. Peak flows from a lower return period event should not be higher than a higher return period event.

☐	Contributing hydrographs from another RAS model applied at coincident locations

## General Hydrology

☐	Any flow regulating dams in the model area:

Review Comments and Responses:

Comment for all Models: Precip seem to be an areal average but FNI did not provide details or GIS data for precip to confirm. Also, as noted in their 120505045 QC spreadsheet, they were asked to provide details on how NOAA depths were generated, which some were in the spreadsheets, but the process was not documented. It can be assumed from their Report, and reviewing NOAAs PF Map, precip was calculated from PF depths rasters obtained from NOAA and converted to an Areal Average, which they have in their spreadsheets, but that is not backed up with GIS data. Please provide data.

- NOAA depth grids now provided.

- Good

Based on the method FNI used, the precip was focused on 1 sq. mi. and the losses were calculated in RAS with Infiltration.

- This is correct.

- Good

CN is applied in RAS based off LU and Soils (Incorporated in Infiltration layers) – No response needed.

Comment for all Models: When reviewing the Running Water Draw models, the US BC seemed to be different lengths then the Inflow BC for the DS model.

- BC lines have been adjusted to be similar lengths.  Some differences between will be present between the US and DS BC lines due to the overlap and to avoid BC cell overlap errors

- Good

# Terrain Dataset:

Modeler’s Initials:	     	Date Submitted:

Reviewer’s Initials:	DMP	Date Reviewed:	03/23/2022

Modeler’s Initials:	     	Date Responded:

Statistical Data:

Terrain dataset name:

Grid resolution (cell size) (ft / m):

Should be 10 ft or smaller

Technical Review:

☒	Terrain datasets included in ‘Terrain’ folder

☒	Terrain projection file included

☒	Topographic abnormalities corrected (no drops or rises from underlying data errors)

☒	Terrain modification (added/removed features) documented

☒	Terrain datasets are associated with the proper geometry files

Review Comments and Responses:

- 1205000501 terrain shown in model description as 10m

  - NM Roosevelt Curry 2015 at 1 m. Check if Description in 1205000501 model is correct. If incorrect, include correct DEM size in report and adjust/remove Description in model.

- Updated model descriptions.

- Good

# HEC-RAS Geometry:

Modeler’s Initials:	     	Date Submitted:

Reviewer’s Initials:	DMP	Date Reviewed:	03/23/2022

Modeler’s Initials:	     	Date Responded:

Statistical Data:

Nominal Mesh Cell Size (ft):			200

Min/Max Cell Area (ft2):

Number of Cells:

Goal: less than 1M cells

Technical Review:

## 2D Mesh

☐	2D Boundary delineated along watershed boundary or buffer to ensure all contributing area is included

☐	Nominal cell size appropriate for model detail (generally 200 feet)

☒	Computation points are at default locations or changes documented

☒	Mesh Cell and Face tolerances are default or changes documented

☒	No mesh errors

☒	Cell face relationships are calculated

## Breaklines

Check at all urban areas, critical structures, major confluences, a handful of headwater tributaries, downstream end, at benchmarks, and a handful of randomly selected locations.

☒	Breaklines are all at least 200 feet long (at least matching cell size)

☒	Breaklines are spaced at least 200 ft (cell spacing) apart or breakline spacing parameters adjusted as necessary.

☐	Breaklines are consistent with the terrain data

☒	Breaklines sufficiently detailed to capture features (thalweg, banks, berms, etc.)

☐	Cells do not overlap terrain features where appropriate (i.e. unwanted leaking)

☒	Breakline cell sizing and parameters are appropriate

2D area connections using 2D equations are recommended at benchmark locations, these should all be verified.

☐	2D Area Connections placed at applicable benchmarks

☐	2D Area Connections set to use normal 2D equation domain

## Manning’s n-values

☐	Manning’s n-values associated to land cover data provided

☒	Land Cover classifications are sufficiently descriptive within the data table

☒  Manning’s n values generally match current aerial imagery

☐  Manning’s n values are within reason (based on Min/Max table)

☒  Manning’s n dataset is associated with the geometry files

## General

☐	All model datasets each have unique descriptive names

Review Comments and Responses:

- RAS Model 1205000501 has Description in model saying analysis was performed in HEC-RAS 5.0.3 and terrain developed from Q2 LiDAR

  - If correct, please included in Hydrologic Notebook and Report

  - If incorrect, please remove

- Updated model descriptions.

- Good

- RAS Model 1205000501 has Description and other models do not; Please include Descriptions in each model referencing project and connections to other models

- Model descriptions added to other models.

- Good

- Comment for all Models: 2D Connections need to be placed at applicable benchmarks (NRCS Dams, DAMS with defined spillways, and Gage locations)

- Reference lines added to Dams and gage locations, as well as locations being compared to local regression equations.

- Good

- Comment for all Models: Refer to QC shapefile for representative recommended V-Notches

- Changes made to areas located in QC shapefiles

- Good

- Comment for all Models: Refer to QC shapefile for any necessary breakline edits along terrain

- Changes made to areas located in QC shapefiles

- Comment # 3 for HUC 1205000505 wasn’t applied

- Mapping at inflow location for 120500502_03 to 1205000504 has areas where floodplain isn’t within 0.5 ft tolerance for mapping; We aren’t able to tell where the tie-in locations would be. For mapping checks, this will need to be defined.

- Breakline updated in HUC05, Boundary condition updated to fix tie-in for HUC02_03 and HUC04.

- Comment for all Models: Manning’s N Values need updating in RAS models

  - Found errors for Developed, Medium Intensity for all Models

  - Need to reference where N Values came from in Report

- Manning’s N value updated from 0.8 to 0.08 for Developed, Medium Intensity in all Models.

- Good

- Comment for all Models: Once Final, please remove any unnecessary data (Geometry Files, Flow Files, Plans Files, .hdf files, etc.)

- Unnecessary data removed.

- Good

- Comment for all Models: Make sure 2D Flow Area Name is labeled as the HUC study area

- 2D Flow Area Name updated.

- Please refer to Section 2.5.2 Naming Convention of the Texas 2D Base Level Engineering Guidelines for appropriate Nomenclature

- Updated

- Comment for all Models: Label Project based on HUC10

- Project names updated to be consistent.

- Please refer to Section 2.5.2 Naming Convention of the Texas 2D Base Level Engineering Guidelines for appropriate Nomenclature

- Updated

- Comment for all Models: Label all Geometries in a similar way

- Geometry names updated to be consistent.

- Please refer to Section 2.5.2 Naming Convention of the Texas 2D Base Level Engineering Guidelines for appropriate Nomenclature

- Updated

- Comment for 1205000501: Since this study crosses into New Mexico, the projection was the New Mexico East 3001, Transverse Mercator. FEMA guidance recommends a single projection be used for the entire study area, so we recommend staying in Texas North State Plane 4201.

- Model projection has been updated to be consistent with the other models.

- Good

- Comment for all Models: For future QC submittals, send in GIS files in Map Package used to create RAS model with shapefiles included (HUC boundaries, Breaklines (Road/Rail/Irrigation), Streams, Mesh Boundaries, BC lines, Inflow BCs, BFEs, Gage Locations, Profile Lines, etc.)

  - This will help with review of adjoining HUCs and provide the location where the inflow hydrographs were obtained

  - Export any relevant data from RAS Mapper for QC’er to review in GIS as needed

- Shapefiles added to QC submittal

- Good

- Comment for all Models: For Future QC submittals, send in Hydrologic or Hydraulic Notebook, if applicable

- No Hydraulic Notebook created, please refer to report.

- Good

# HEC-RAS 2D Internal/External Boundaries:

Modeler’s Initials:	     	Date Submitted:

Reviewer’s Initials:	DMP	Date Reviewed:	03/23/2022

Modeler’s Initials:	     	Date Responded:

Technical Review:

☒	External boundaries connected to 2D mesh

☐	Boundary conditions everywhere flow could build up at the boundary of the 2D mesh

☒	Boundary conditions have a hydrograph or elevation/slope data assigned

☒  Input hydrographs consistent with hydrologic data (DSS, manual entry)

☒  Precipitation data is consistent with hydrologic data (DSS, manual entry)

☒	Downstream boundary consistent with ground slope or known downstream WSEL

☒	Flow hydrographs applied at appropriate locations

☒	Energy grade slope for hydrograph distribution is reasonable

Review Comments and Responses:

- Comment for all models: We have been trying to keep the US BC and Inflow BC lines the shape of the largest floodplain to focus any flow in flat areas. It doesn’t seem like any of the models have similar BC lengths or locations. Please justify.

- Model perimeters were buffered to ensure clean tie-ins. BC lines have been adjusted to be the similar lengths.

- Good

- Comment for all models: ND BCs missing throughout the HUC. Review floodplain at full flood widths and add/extend BCs in areas along boundary that have flow building up at boundary. This will need to be done for most of the boundary for all models, make sure tolerance is at default or 0.

  - When reviewing floodplain, max tolerance should be 0.25 – 0.33 feet. If viewing at 0.5, that is too much removed for this area.

- BC lines added to models based on default tolerance.

- Good

- Comment for all models: Refer to Watershed QC shapefile for BC comments

- For 1205000505 and 1205000502, US Boundary_1 flow seems unnecessary

- BC lines were added to ensure no water was building up at model perimeter. No changes made.

- Good

- For future QC, compute the Rasters to allow for easier QC of WSEL comparisons at inflows and along the borders between HUCs, if applicable: Interface checks where WSELs at interface between adjoining models need to be within 0.5 feet

- Computed rasters added to QC submittal

- Good

- Inflow for each DS location (1% event) are not increasing. This does not make sense if there are no flow regulating dams. Please confirm and provide detailed explanation in the Report and Hydraulic Notebook for QC review.

  - 125000502_03 Peak Flow: 		01Jan2020 2030 		23955.99

  - 125000504 Peak Flow:			02Jan2020 1130 	 	21849.38

  - 125000505 Peak Flow: 			02Jan2020 1700	      21418.88

- Since the watershed has a long, narrow shape and there are playas, it would make since those inflows would not be increasing at every HUC10 boundary. More detail was added to report.

- Good

- Inflow for 1205000504 1% MINUS linked to 100 year

- Inflow corrected to be the 100 minus event instead of the 100 year event.

- Good

- Missing 1% Plus Run

- Couldn’t verify 1% MIN Outflow from 1205000501; 100% MIN not run

  - Inflow for 1% MIN 1205000502_03 model incorrect

- 1% Minus run should now be included

- Good

# HEC-RAS 2D Model Execution:

Modeler’s Initials:	     	Date Submitted:

Reviewer’s Initials:	DMP	Date Reviewed:	03/23/2022

Modeler’s Initials:	     	Date Responded:

Statistical Data:

HEC-RAS Unsteady Plan Names:

Simulation length (hours):

Calculation Interval (seconds):

Hydrograph Output Interval:

Mapping Output Interval:

Model Runtime:

Technical Review:

☒	Appropriate 2D parameters are used

☒	Hydrograph Output Interval and Mapping Output Interval set at a reasonable interval to capture peaks

☒	Model runtime is reasonable (< 12 hours, but dependent upon model size and run engine capabilities)

Review Comments and Responses:

- Model 1205000501: 1% MIN not run

  - 120500502_03 1% Min Inflow has data, but not sure how (12378.62 Peak Flow)

- 1% Minus run should now be included

- Good

# HEC-RAS 2D Model Results:

Modeler’s Initials:	     	Date Submitted:

Reviewer’s Initials:	DMP	Date Reviewed:	03/23/2022

Modeler’s Initials:	     	Date Responded:

Technical Review:

## Computation Checks

☒	Model runs with no cell errors

☒  Volume balance is within tolerance (ΔV < 1% is ideal, however 2% to 3% can be acceptable for rain on mesh)

## Results Full Checks

☒	Computational time step meets ideal Courant condition

Diffusion Wave: V∆T∆X ≤2 (Up to 5)

☒  Model simulation is long enough to pass the entire hydrograph through the model

(minimum of 12 hours past the peak, preferably 24 hours)

☒	Hydrographs at internal and external boundaries are smooth

☒	Contributing areas are contained within 2D mesh or accounted for with inflow hydrographs

☐	No flow trapped at 2D Boundary

☒	Mapping is reasonable compared to effective

## Results Spot Checks

Check at all critical structures, major confluences, a handful of headwater tributaries, downstream end, at benchmarks, and a handful of randomly selected locations.

☒	No visible spikes in WSEL visibly present with a single cell

☒	WSEL and depth grids appear reasonable (no artificial dips or spikes)

☐	Flooding is connected downstream for all significant contributing areas (> 64 acres)

☒	Velocity grids appear reasonable (no unusually high velocity areas)

## Flow Checks

### Gage Analysis

☒  Peak flow gage analysis performed on all gages in the watershed with sufficient data

☒ 	All gages used have an appropriate period of record (10 to 20+ years)

☐	All gages used have no extensive gaps in record which would impact the analysis

☐	Gage analysis results validated to other sources (Regression, USGS, etc.)

### 2D Flow Results

☒  Results at applicable benchmarks justified (gages, HWMs, effective BFEs)

☐	Peak discharges comparable to gages, effective studies, regional regression equations

Review Comments and Responses:

- Comment for all models: Identified some areas that are larger than 5 for the Courant; Courant values over 2 throughout the more riverine portions of all the models. It is recommended to use a Variable time step and set the Courant Max at 2. Since the models are all under 5 hours, it is reasonable to increase run time. Max time for a model is around 12 hours. Refer to QC shapefile and adjust as necessary, depending on level of acceptable tolerance.

- Method changed to Time Series of Divisors to avoid courant values over 2.

- Good

- Comment for all models: For future QC, compute the Rasters to allow for easier QC of WSEL comparisons at inflows and along the borders between HUCs, if applicable.

- Shapefiles now included in QC submittal.

- Good

- Comment for 1205000505: Plainview gage has large gaps in period of record. May be impacting results but was explained in report. Final report will need to keep explanation of data gaps. Please explain further.

  - The Plainview gage showed a 789% difference for the 100 year if only using the 2005-2020 data also but looking at all 50 years it showed a 19% difference.

- More explanation added to report, as well as a table comparing 17b and 17c methods

- Good

- Comment for 1205000505: No Effective BFE comparison given to QC’er

  - BFEs found in Plainview city limits (spot check comparison should be done for each set of stream BFEs and table added to Report)

  - Justify any differences as needed

- Comparison made between models and XS in detailed study area. XS were used instead of BFEs, since those can be adjusted and shifted. XS file included in QC submittal.

- Refer to report for comments

- Updated

- Comment for all models: Discharges for Plainview gage within 19% for the 100 year, 6% for the 500 year.

  - Friona Gage is much higher: 95% higher for 100 year, 83% for 500 year, but can be justified by dry conditions for the extent of the gage period of record

  - Back up differences in discharges in Report

  - Other ways to decrease Discharges would be to increase Manning’s N. Based on table Min/Max in report, there is room to adjust up.

- More explanation added to report.

- Refer to report for comments

- Updated

- Comment for all models: Include WSEL comparison at gages. If WSEL differences are reasonable (0.5 ft), discharges at 15-20% difference would be more tolerable.

- Rating curve comparison added to the report. Models match the rating curve relatively well and are within 0.5 ft. Models now match gage results for the 100 event.

- Refer to report for comments

- Updated

- Model perimeters consistently don’t include total extent of HUC. Can’t confirm any of those missing areas are covered by adjacent models.

- Areas added into models.

- Good

- Areas along boundaries are showing cut off mapping. Some of these areas can be preventing flooding across the HUC10 perimeter. It is recommended to buffer the Mesh perimeter to fix these floodplain differences. Representative locations are shown below, refer to QC shapefile for more examples:

- Boundaries adjusted to avoid flow build – up in DS model. BC lines added to ensure water does not build up at the boundary of the US models.

- Good

# QM Checklist Approval

This 2D Modeling QM Review is in compliance with the contract requirements and all task checkpoints are complete.  The independent QM Team has reviewed the hydraulic analysis, presented review comments to the Production Team Task Leader, and discussed any problems or issues.  The Task Leader, QM Team Manager, and Project Manager have signed the QM Checklist to confirm that all comments are received, addressed, and documented appropriately.

Task Leader																Date

QM Team Manager														Date

Project Manager															Date