| Submitted By: | FTN Associates, Ltd. | Date: | 02/09/2022 |
| --- | --- | --- | --- |
| Reviewed By: | Entech Civil Engineers, Inc. | Date: | 02/14/2022 |

Technical Review – Running Water Draw Watershed (HUC 8 # 12050005)

HUC 10 # 12050504

| Y | N | N/A | Item Description | Comments |
| --- | --- | --- | --- | --- |
| Data Completeness Check |  |  |  |  |
| ☐ | ☒ | ☐ | Hydrology model and/or spreadsheets included |  |
| ☒ | ☐ | ☐ | Terrain dataset included |  |
| ☒ | ☐ | ☐ | Projection file included |  |
| ☒ | ☐ | ☐ | 2D boundaries included |  |
| ☒ | ☐ | ☐ | 2D Breaklines included |  |
| ☒ | ☐ | ☐ | Land Cover dataset (Manning’s n-values) Included |  |
| ☒ | ☐ | ☐ | Infiltration dataset (Composite CN values) Included |  |
| ☒ | ☐ | ☐ | Regression analysis Included |  |
| ☒ | ☐ | ☐ | Gage analysis Included |  |
| ☒ | ☐ | ☐ | Preliminary floodplain runs Included in RAS-Mapper |  |
| ☒ | ☐ | ☐ | Terrain, land cover, and infiltration data have been associated with correct geometry(s) |  |
| ☒ | ☐ | ☐ | All flood events included in the HEC-RAS model (10%, 4%, 2%, 1%, 0.2%, 1%+, 1%-, and any observed rainfall events) |  |
| ☒ | ☐ | ☐ | Modeling narrative included | The narrative needs to be finalized. |
| Hydrology |  |  |  |  |
| Rainfall |  |  |  |  |
| ☒ | ☐ | ☐ | Precipitation values are an aerial average of Atlas 14 values for the entire 2D area boundary for each storm event | Please provide details/back up calculations on how the average NOAA Atlas 14 depths were generated. |
| ☒ | ☐ | ☐ | Total precipitation depths for 1%+ and 1%- are calculated correctly |  |
| ☒ | ☐ | ☐ | Rainfall hyetographs are calculated for all storm events (10%, 4%, 2%, 1%, 0.2%, 1%+, and 1%-) |  |
| ☒ | ☐ | ☐ | Cumulative excess rainfall values are in increasing order from 10%, 4%, 2%, 1%, to 0.2% |  |

| Y | N | N/A | Item Description | Comments |
| --- | --- | --- | --- | --- |
| ☒ | ☐ | ☐ | Cumulative excess rainfall values for the 1%+ and 1%- fall in order with the storm events matching the total precipitation values | The is a small drop in the 1%- rainfall hyetograph between hours 11:00 to 13:00. Did not have the backup data to confirm, please investigate and make sure the dip in the hyetograph is OK. |
| Inflow Hydrographs (if any) |  |  |  |  |
| ☒ | ☐ | ☐ | Correct DSS file paths are being used in HEC-RAS |  |
| ☒ | ☐ | ☐ | Contributing hydrographs from another HEC-RAS model applied at coincident locations |  |
| HEC-RAS Geometry |  |  |  |  |
| Terrain |  |  |  |  |
| ☒ | ☐ | ☐ | 10ft Imported Grid resolution used |  |
| ☒ | ☐ | ☐ | Topographic abnormalities corrected and no drops or rises from underlying data |  |
| Land Cover (Manning’s n) |  |  |  |  |
| ☒ | ☐ | ☐ | Land Cover classifications are sufficiently descriptive within the data table |  |
| ☒ | ☐ | ☐ | Manning’s n values generally match current Land Cover classifications | Update values to match narrative and FNI. |
| Infiltration |  |  |  |  |
| ☒ | ☐ | ☐ | CN values generally match current soils and land use data |  |
| ☒ | ☐ | ☐ | Appropriate abstraction ratio and minimum infiltration used for CN classifications |  |
| 2D Mesh |  |  |  |  |
| ☒ | ☐ | ☐ | 2D Boundary delineated along watershed boundary or buffer to ensure all contributing area is included | The 2D boundary along the watershed boundary considered no buffer. Please confirm. |
| ☒ | ☐ | ☐ | Nominal cell size appropriate for model detail? (generally 200 feet) |  |
| ☒ | ☐ | ☐ | No mesh errors |  |

| Y | N | N/A | Item Description | Comments |
| --- | --- | --- | --- | --- |
| ☐ | ☒ | ☐ | Where appropriate cells overlap terrain features (i.e., V-notch at bridges) | There are few more hydraulically significant features where add v-notches could be added. Please verify. See example below: There is a reservoir along main channel with large culverts. |
| ☐ | ☐ | ☒ | 2D Area Connections placed at applicable benchmarks and set to use normal 2D equations domains |  |
| ☒ | ☐ | ☐ | Refinement zone assigned appropriately |  |
| Breaklines |  |  |  |  |
| ☒ | ☐ | ☐ | Breaklines are spaced at least 200 ft (cell spacing) apart or breakline spacing parameters adjusted as necessary? | 75x150 spacing is used for Breaklines. |
| ☒ | ☐ | ☐ | Breaklines sufficiently detailed and consistent with the terrain data? |  |
| 2D Boundary Conditions |  |  |  |  |
| ☒ | ☐ | ☐ | Initial conditions used and set appropriately | Hot restart file used. |
| ☒ | ☐ | ☐ | Internal and external boundaries connected to 2D mesh |  |
| ☐ | ☒ | ☐ | Boundary conditions everywhere flow could build up at the boundary of the 2D mesh | There are instances where the flow buildup is happening near the streams. We think that the boundary conditions need to be extended. |
| ☒ | ☐ | ☐ | Boundary conditions have a precipitation or hydrograph data and assigned at correct locations |  |
| ☒ | ☐ | ☐ | Downstream boundary consistent with ground slope / known WSEL |  |
| 2D Model Execution |  |  |  |  |
| ☒ | ☐ | ☐ | Appropriate 2D parameters and options used |  |
| Y | N | N/A | Item Description | Comments |
| ☒ | ☐ | ☐ | Hydrograph Output Interval and Mapping Output Interval set at a reasonable interval to capture peaks |  |
| ☒ | ☐ | ☐ | Model runtime is reasonable |  |
| 2D Model Results |  |  |  |  |
| Computation Checks |  |  |  |  |
| ☒ | ☐ | ☐ | Model runs with no cell errors |  |
| ☒ | ☐ | ☐ | Volume balance is within tolerance (ΔV < 1% is ideal, however 2% to 3% can be acceptable for rain on mesh) |  |
| Full Checks |  |  |  |  |
| ☒ | ☐ | ☐ | Computational time step meets ideal Courant condition (diffusive wave ≤ 2, up to 5) | In the narrative it is stated that 30-second timestep is used. It seems that the plans were setup with a variable timestep of a max courant # of 2. Please update to be consistent with the narrative. Also, selecting a variable time step usually causes increase runtime. |
| ☒ | ☐ | ☐ | Model simulation is long enough to pass the entire hydrograph through the model (minimum of 12 hours past the peak, preferably 24 hours) |  |
| ☒ | ☐ | ☐ | Hydrographs at internal and external boundaries are smooth |  |
| ☒ | ☐ | ☐ | Contributing areas are contained within 2D mesh or accounted for with inflow hydrographs |  |
| ☐ | ☒ | ☐ | No Flow trapped at 2D Boundary | Please verify all the boundary conditions along the streams. |
| Spot Checks |  |  |  |  |
| ☒ | ☐ | ☐ | WSELs appear reasonable within a single cell (check for visible spikes) | See additional comments |
| ☒ | ☐ | ☐ | Depth grids appear reasonable (no artificial dips or spikes) | See additional comments |
| ☒ | ☐ | ☐ | Flooding is connected downstream for all significant contributing areas (> 64 acres) |  |

| Y | N | N/A | Item Description | Comments |
| --- | --- | --- | --- | --- |
| ☒ | ☒ | ☐ | Velocity grids appear reasonable (no unusually high velocity areas / hot spots) | Mostly, the velocity grid is reasonable There are some velocity hot spots in areas near breaklines. Please adjust breaklines and mesh to align with the underlying terrain and to avoid high velocities. |
| Flow Checks |  |  |  |  |
| ☐ | ☒ | ☐ | DSS files contain runs consistent with project | Please cleanup DSS output to only contain the final runs in the model. |
| ☒ | ☐ | ☐ | Peak modeled discharges comparable to gages, and regional regression equations |  |
| 2D Model Verification |  |  |  |  |
| Stream Gage Analysis |  |  |  |  |
| ☒ | ☐ | ☐ | Regression analysis performed at select locations in the watershed |  |
| ☒ | ☐ | ☐ | Stream gage analysis performed on all gages in the watershed with sufficient data |  |
| Sensitivity Analysis |  |  |  |  |
| ☒ | ☐ | ☐ | Sensitivity analysis performed to evaluate which model parameters significantly impact results |  |
| ☒ | ☐ | ☐ | Based on the sensitivity analysis results did the results of the selected computation settings resulted in acceptable outcomes |  |
| Additional Comments |  |  |  |  |
| - The flow in the overbanks is not connected to the main channel. Please verify the topo in the overbank areas along the channel. |  |  |  |  |