| Submitted By: | FTN Associates Ltd | Date: | 05/20/2022 |
| --- | --- | --- | --- |
| Reviewed By: | Entech Civil Engineers, Inc. | Date: | 06/02/2022 |

Technical Review

| Y | N | N/A | Item Description | Comments |
| --- | --- | --- | --- | --- |
| General Check |  |  |  |  |
| ☒ | ☐ | ☐ | Mapping narrative included? |  |
| ☒ | ☐ | ☐ | All flood events included in the Mapping Files (10%, 1%, and 0.2%)? |  |
| ☐ | ☐ | ☒ | Mapping is reasonable as compared to effective? | Please add documentation stating that the mapping has been compared with effective floodplains and note major discrepancies, if any. |
| ☒ | ☐ | ☐ | Projection file included? | Please use the same projection as the original terrain project for consistency. |
| Raster and Mapping Check |  |  |  |  |
| Raw Raster |  |  |  |  |
| ☒ | ☐ | ☐ | Has the Raw Raster depth grids been exported for all HEC-RAS 6.2 geometries (10%, 1%, and 0.2%)? |  |
| ☒ | ☐ | ☐ | Is the lowest elevation for all Raw Depth Raster = 0.001 ft? and Sloping Method used for Rendering? |  |
| ☒ | ☐ | ☐ | Has the Raw Raster WSEL grids been exported for all HEC-RAS 6.2 geometries (10%, 1%, and 0.2%)? |  |
| ☒ | ☐ | ☐ | Is the lowest elevation for all Raw Depth Raster files -100 ft? and Sloping Method used for Rendering? |  |
| ☒ | ☐ | ☐ | Tie ins values are within 0.5 ft between rasters? |  |
| Final Processed Raster |  |  |  |  |
| ☒ | ☐ | ☐ | Does the Raw Raster Files(s) include all the combined WSEL and depth grids for all events (1%, and 0.2%)? |  |
| ☒ | ☐ | ☐ | Has the TWDB tool been used to remove noise |  |
| ☒ | ☐ | ☐ | Has the final Depth and WSEL grids clipped to watershed boundary? |  |
| Mapping Layer(s) |  |  |  |  |
| ☐ | ☒ | ☐ | Does floodplains look smooth? | Few areas may need additional floodplain smoothening. |
| ☒ | ☐ | ☐ | Has 10-YR floodplain been merged into 100-YR FP and 100-YR FP been merged to 500-YR FP? |  |
| ☒ | ☐ | ☐ | All island in place? (no removal of islands) |  |
| ☒ | ☐ | ☐ | Manual review of the floodplains? | Please see comments in shapefile |
| ☒ | ☐ | ☐ | Final mapping deliverable includes all files with the correction naming convention? (FLD_HAZ_AR, TENPCT_FP, BLE_WSE01PCT, BLE_WSE0_2PCT, BLE_DEP01PCT, BLE_DEP0_2PCT) |  |