# Project Narrative

Running Water Draw Watershed (HUC 12050005)

Base Level Engineering Submittal

# Introduction

The Running Water Draw Watershed Phase 0 project consists of performing Base Level Engineering (BLE) modeling and analyses for the Running Water Draw Watershed (HUC 12050005). The total area for the watershed is approximately 1,514 square miles and has an estimated population of nearly 39,000 (2010 Census) within the watershed.

Running Water Draw Watershed originates in New Mexico (NM) and drains into West Texas. The watershed is located in the High Plains of Texas and New Mexico. It is a very long and narrow watershed, sloping from the northwest to the southeast. The terrain is mostly flat, and contains many depressions, known as playas, throughout the watershed. The watershed is sparcely populated, with the City of Plainview, Texas being the population center within the watershed.

# BLE Modeling Inputs and Controls

Section 2.0 discusses the fundamental components required to execute a two-dimensional (2D) BLE analysis for the Running Water Draw (HUC 12050005) watershed. Inputs including elevation data, hydrology from rain-on-grid and inflow hydrographs, and hydraulic analyses and variables are defined in the following sections. HEC-RAS 6.0 rain-on-grid utility was applied in the production of these Texas 2D BLE products.

## Topographic Data

A high-resolution Digital Elevation Model (DEM) derived from Light Detection and Ranging (LiDAR) provides a detailed representation of the Earth’s surface. DEMs are one of the fundamental components for 2D engineering analyses and are used to direct excess rainfall-runoff (hydrology) through the model area to develop water surface elevations (hydraulics) and resulting floodplain inundation boundaries. The seamless DEMs developed to support the 2D BLE modeling and analysis within the Running Water Draw watersheds were developed using the following steps:

- Available 1-meter DEMs within each watershed were gathered from the USGS National Map;

- Seamless DEMs were produced using GIS spatial analyst and 3D analyst tools and resampled to 10-foot resolution; and,

- Quality Assurance (QA) and Quality Control (QC) measures were implemented to ensure topographic data meets quality standards of the project.

Additional documentation regarding DEMs including coverage, accuracy, acquisition dates, and source contact/agency are presented in the figures, tables, and text in the sections below.

### Inventory

DEMs were gathered for the for the Running Water Draw Watershed, which included an approximate 1-mile buffer around the watershed boundary. Figure 1 depicts the datasets identified and leveraged for each watershed. Data from FEMA and USGS were queried to build the inventory with the most current and available data sources.

### Evaluation

A data coverage assessment was conducted to check for data gaps, extent, accuracy, and completeness. Additionally, a review of related documentation, reports, indexes, and metadata associated with the leveraged datasets were conducted.

Table 1 depicts the complete list of source elevation data and attributes leveraged for this BLE project.

Table 1: Source Topographic Data

| Watershed | Year | Description | Data Type | 3DEP Quality Level | RMSE | Source/Owner |
| --- | --- | --- | --- | --- | --- | --- |
| Running Water Draw | 2018 | FEMA Texas Panhandle LiDAR | Airborne LiDAR | 2 | 4.8 cm | USGS |
| Running Water Draw | 2016 | Roosevelt & Curry Counties, NM LiDAR | Airborne LiDAR | 2 | N/A | USGS |

Figure 1. Available Topographic Data Extents

#### Watershed Source Terrain Data

The source elevation data for the watershed were obtained as 1-meter DEMs derived from the the sources listed above (Table 2). Figure 1 depicts the extent of the data defined in Table 1.

USGS 2018 FEMA Texas Panhandle LiDAR

The 2018 FEMA Texas Panhandle LiDAR was acquired between February 9, 2018 to September 3, 2019. The vertical accuracy for the dataset is reported to be 4.3 cm RMSEz at a 95% confidence level, which meets project accuracy specifications of the NSSDA. These accuracies meet FEMA standards for floodplain reevaluation studies and map modernization programs designed to update the Flood Insurance Rate Maps (FIRM).

2016 Roosevelt & Curry Counties, NM Lidar

The 2016 Roosevelt & Curry Counties, NM LiDAR was acquired between September 30, 2015 and March 25, 2016. The vertical accuracy for the dataset is not listed per the US Interagency Elevation Inventory website; however, it is listed as Quality Level 2 or better, which meets project accuracy specifications of the NSSDA. These accuracies meet FEMA standards for floodplain reevaluation studies and map modernization programs designed to update the Flood Insurance Rate Maps (FIRM).

### Data Development Methodology

DEMs were processed for the watershed for the BLE modeling efforts. Tools within ArcGIS were used to mosaic and reproject the source data. The DEMs were projected horizontally to North American Datum of 1983 (NAD83), State Plane Coordinate System (SPCS) Texas North (4201) with units of US Survey Foot. The vertical datum for this project is set to North American Verticla Datum of 1988 (NAVD88) with units of US Survey Foot.

### DEM QA/QC

#### DEM Quality Assurance

Seamless DEMs developed for use on the BLE analyses were developed and independently assured to meet quality assurance (QA) standards of the project. Quality assurance during the data development process included, but was not limited to the following:

- Horizontal Projection Check (NAD83 SPCS 4201 FT)

- Vertical Datum Check (NAVD88 FT)

- Resolution Check (1-meter to 10-foot resolution)

- Seamless Data Check to develop DEM files that are consistent and seamless along source data

- Review and resolution of LiDAR Project transition edge

#### DEM Quality Control

Quality control (QC) measures were implemented following the development process of the seamless DEMs and included visual observations using hillshade, color rendering, and/or other visual aids to review and identify potential impactful anomalies within the DEM surfaces.  This QC step included, but was not limited to the following:

- Seamless Data Check to eliminate voids along the edges and between the prioritized datasets

- NoData Value Check to eliminate no null values

- Manual Elevation Check using hillshade rasters to find erroneous elevation issues

Following these QA/QC steps, the final DEM data developed for the Running Water Draw Watershed was considered of sufficient quality to meet FEMA standards and present a representative surface developed from leveraged elevation data for the purposes of this BLE project.