| Submitted By: | FTN Associates, Ltd. | Date: | 10/22/2021 |
| --- | --- | --- | --- |
| Reviewed By: | Entech Civil Engineers, Inc. | Date: | 11/03/2021 |

Technical Review – Running Water Draw Watershed (HUC 12050005)

| Y | N |  | N/A |  | Item Description |  | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Topology Check |  |  |  |  |  |  |  |
| Bare Earth DEM |  |  |  |  |  |  |  |
| ☒ | ☐ |  | ☐ |  | Terrain datasets included in ‘Terrain’ folder? |  |  |
| ☒ | ☐ |  | ☐ |  | Terrain projection file included? |  | All terrain files projections were included and were set to the right projections. NOTE: Please use the same name for all projection files for consistency. |
| ☒ | ☐ |  | ☐ |  | DEM has 10-ft cells and is in the correct state plane projection? |  |  |
| ☒ | ☐ |  | ☐ |  | Bare_Earth_DEM Included? |  |  |
| ☐ | ☒ |  | ☐ |  | DEMs Included? – GeoTIFF |  | No GeoTIFF files included. |
| ☒ | ☐ |  | ☐ |  | DEM Tile Index Included? – .SHP |  |  |
| Hydrologically Corrected DEMs |  |  |  |  |  |  |  |
| ☒ | ☐ |  | ☐ |  | Hydrologically Corrected DEMs Included? – Esri grid/GeoTIFF/ASCII grid |  |  |
| ☐ | ☒ |  | ☐ |  | HDEM Tile Index? – .SHP |  | The “GridID” in the attribute table doesn’t match tile names. The naming convention for the “Tile Index” shapefile is same for HDEM and Bare Earth. Add some initials to the Tile Index shapefile to differentiate. |
| ☐ | ☒ |  | ☐ |  | Any additional elevation data collected for use in the preparation of this Flood Risk Project? – Format as received |  | No documentation provided or record shown of any additional elevation data collected for the study area. |
| ☐ | ☒ |  | ☐ |  | DEM has 10-ft cells and is in the correct state plane projection? |  | HDEM_10_8 tile when imported in HEC-RAS shows the cell size is 9.999… Update to ensure all HDEM files have a 10-ft cell size. |
| ☒ | ☐ |  | ☐ |  | DEM vertical units are in feet NAVD88? |  |  |
| ☐ | ☒ |  | ☐ |  | DEM is buffered at least 1000 ft past HUC 8 watershed boundary? |  | The report states 1-mile buffer was selected, which is consistent with the FNI study. The 1-mile buffer is not consistent across the HU8 watershed boundary as well. See comments in: FTN_RunningWaterDraw.shp for additional comments on the buffer. |
| ☒ | ☐ |  | ☐ |  | DEM does not have sinks filled in? |  |  |
| ☒ | ☐ |  | ☐ |  | DEM has no missing (null) data, gaps, slivers, or shelving? |  |  |
| ☒ | ☐ |  | ☐ |  | DEM hillshade indicates no issues? |  |  |
| ☒ | ☐ |  | ☒ |  | Topographic abnormalities corrected? (no drops or rises from underlying data errors) |  |  |
| ☐ | ☒ |  | ☐ |  | Terrain modification documented? (post-processing (hydroconditioning, V-notches at structures, added/removed features)) |  | No documentation on terrain modification and their location was provided. |
| ☐ | ☒ |  | ☐ |  | Any contour information available? |  | No contour information provided. |
| Metadata Check |  |  |  |  |  |  |  |
| ☒ |  | ☐ |  | ☐ |  | Is the metadata file included with the submittal? |  |
| ☒ |  | ☐ |  | ☐ |  | Does the metadata pass validation checks? Has the documentation been provided? |  |
| ☒ |  | ☐ |  | ☐ |  | Is the metadata correct in structure and information? |  |
| ☒ |  | ☐ |  | ☐ |  | Terrain Metadata Included? – .XML |  |
| ☒ |  | ☐ |  | ☐ |  | Are all required spatial data sets and tables included? |  |
| ☒ |  | ☐ |  | ☐ |  | Do all the files have the same and correct state plane projection? |  |
| Stream Centerline and Watershed Check |  |  |  |  |  |  |  |
| ☒ |  | ☐ |  | ☐ |  | Stream centerlines are delineated to 1 sq mi? |  |
| ☒ |  | ☐ |  | ☐ |  | Stream centerlines are smoothed, generalized, and follow terrain? | Some of the stream centerlines in Streams_Smoothed shapefile could be extended further upstream. |
| ☐ |  | ☒ |  | ☐ |  | Stream centerlines are cut off at the caprock/first playa? | Please revisit the smoothened stream centerlines shapefile. In some instances, the smoothened does not extend far enough upstream. (see comments in: FTN_RunningWaterDraw.shp) |
| ☐ |  | ☒ |  | ☐ |  | Watershed boundary is cut / divided at correct high points locations? | Please revisit the Watershed boundary shapefile. The boundary in some locations is incorrect. (see comments in: FTN_RunningWaterDraw.shp) |
| ☒ |  | ☐ |  | ☐ |  | Stream Centerline and Watershed is in the correct state plane projection? |  |
| Completeness Check (Final, General, Spatial_Files, Supplemental_Data) |  |  |  |  |  |  |  |
| ☒ |  | ☐ |  | ☐ |  | Are the folders structured according to Guidance? |  |
| ☒ |  | ☐ |  | ☐ |  | Are all required spatial data sets and tables included? |  |
| ☒ |  | ☐ |  | ☐ |  | Do files meet MIP upload standards? |  |
| ☒ |  | ☐ |  | ☐ |  | Has documentation or BLE Report provided? | See comments in the report. |