| Submitted By: | FTN Associates, Ltd. | Date: | 10/22/2021 |
| --- | --- | --- | --- |
| Reviewed By: | Entech Civil Engineers, Inc. | Date: | 11/03/2021 |

Technical Review – Running Water Draw Watershed (HUC 12050005)

| Y | N |  | N/A |  | Item Description |  | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Topology Check |  |  |  |  |  |  |  |
| Bare Earth DEM |  |  |  |  |  |  |  |
| ☒ | ☐ |  | ☐ |  | Terrain datasets included in ‘Terrain’ folder? |  |  |
| ☒ | ☐ |  | ☐ |  | Terrain projection file included? |  | All terrain files projections were included and were set to the right projections. NOTE: Please use the same name for all projection files for consistency. Projections change for consistency. |
| ☒ | ☐ |  | ☐ |  | DEM has 10-ft cells and is in the correct state plane projection? |  |  |
| ☒ | ☐ |  | ☐ |  | Bare_Earth_DEM Included? |  |  |
| ☐ | ☒ |  | ☐ |  | DEMs Included? – GeoTIFF |  | No GeoTIFF files included. Upon review of file copy, it appears that the DEM GeoTiffs are present in the upload. If the DEM tif files do not come through, please let us know and we will work to resubmit another way. |
| ☒ | ☐ |  | ☐ |  | DEM Tile Index Included? – .SHP |  |  |
| Hydrologically Corrected DEMs |  |  |  |  |  |  |  |
| ☒ | ☐ |  | ☐ |  | Hydrologically Corrected DEMs Included? – Esri grid/GeoTIFF/ASCII grid |  |  |
| ☐ | ☒ |  | ☐ |  | HDEM Tile Index? – .SHP |  | The “GridID” in the attribute table doesn’t match tile names. Corrections made. The naming convention for the “Tile Index” shapefile is same for HDEM and Bare Earth. Add some initials to the Tile Index shapefile to differentiate. Corrections made. |
| ☐ | ☒ |  | ☐ |  | Any additional elevation data collected for use in the preparation of this Flood Risk Project? – Format as received |  | No documentation provided or record shown of any additional elevation data collected for the study area. Only the elevation data referenced in the narrative have been used for this project. If future information is required; this information will be updated as needed. |
| ☐ | ☒ |  | ☐ |  | DEM has 10-ft cells and is in the correct state plane projection? |  | HDEM_10_8 tile when imported in HEC-RAS shows the cell size is 9.999… Update to ensure all HDEM files have a 10-ft cell size. HDEM_10_8.tif corrected. |
| ☒ | ☐ |  | ☐ |  | DEM vertical units are in feet NAVD88? |  |  |
| ☐ | ☒ |  | ☐ |  | DEM is buffered at least 1000 ft past HUC 8 watershed boundary? |  | The report states 1-mile buffer was selected, which is consistent with the FNI study. The 1-mile buffer is not consistent across the HU8 watershed boundary as well. See comments in: FTN_RunningWaterDraw.shp for additional comments on the buffer. Report updated to include at least a 1000 ft buffer. Lidar Data source added to ensure 1000 boundary |
| ☒ | ☐ |  | ☐ |  | DEM does not have sinks filled in? |  |  |
| ☒ | ☐ |  | ☐ |  | DEM has no missing (null) data, gaps, slivers, or shelving? |  |  |
| ☒ | ☐ |  | ☐ |  | DEM hillshade indicates no issues? |  |  |
| ☒ | ☐ |  | ☒ |  | Topographic abnormalities corrected? (no drops or rises from underlying data errors) |  |  |
| ☐ | ☒ |  | ☐ |  | Terrain modification documented? (post-processing (hydroconditioning, V-notches at structures, added/removed features)) |  | No documentation on terrain modification and their location was provided. Text added |
| ☐ | ☒ |  | ☐ |  | Any contour information available? |  | No contour information provided. No contour information was created as LiDAR based DEM data is being used. |
| Metadata Check |  |  |  |  |  |  |  |
| ☒ |  | ☐ |  | ☐ |  | Is the metadata file included with the submittal? |  |
| ☒ |  | ☐ |  | ☐ |  | Does the metadata pass validation checks? Has the documentation been provided? |  |
| ☒ |  | ☐ |  | ☐ |  | Is the metadata correct in structure and information? |  |
| ☒ |  | ☐ |  | ☐ |  | Terrain Metadata Included? – .XML |  |
| ☒ |  | ☐ |  | ☐ |  | Are all required spatial data sets and tables included? |  |
| ☒ |  | ☐ |  | ☐ |  | Do all the files have the same and correct state plane projection? |  |
| Stream Centerline and Watershed Check |  |  |  |  |  |  |  |
| ☒ |  | ☐ |  | ☐ |  | Stream centerlines are delineated to 1 sq mi? |  |
| ☒ |  | ☐ |  | ☐ |  | Stream centerlines are smoothed, generalized, and follow terrain? | Some of the stream centerlines in Streams_Smoothed shapefile could be extended further upstream. Small stream segments have been removed. As the stream centerline is considered a work and progress, streams will be added/extended as necessary during the hydraulics process. |
| ☐ |  | ☒ |  | ☐ |  | Stream centerlines are cut off at the caprock/first playa? | Please revisit the smoothened stream centerlines shapefile. In some instances, the smoothened does not extend far enough upstream. (see comments in: FTN_RunningWaterDraw.shp) Small stream segments have been removed. As the stream centerline is considered a work and progress, streams will be added/extended as necessary during the hydraulics process. |
| ☐ |  | ☒ |  | ☐ |  | Watershed boundary is cut / divided at correct high points locations? | Please revisit the Watershed boundary shapefile. The boundary in some locations is incorrect. (see comments in: FTN_RunningWaterDraw.shp) Watershed Boundaries will be updated during hydraulics phase. |
| ☒ |  | ☐ |  | ☐ |  | Stream Centerline and Watershed is in the correct state plane projection? |  |
| Completeness Check (Final, General, Spatial_Files, Supplemental_Data) |  |  |  |  |  |  |  |
| ☒ |  | ☐ |  | ☐ |  | Are the folders structured according to Guidance? |  |
| ☒ |  | ☐ |  | ☐ |  | Are all required spatial data sets and tables included? |  |
| ☒ |  | ☐ |  | ☐ |  | Do files meet MIP upload standards? |  |
| ☒ |  | ☐ |  | ☐ |  | Has documentation or BLE Report provided? | See comments in the report. Comments in report have been responded to or corrected as requested. |