# TWDB 2D BLE – California Creek
H&H Technical Notebook

Description: Notebook for California Creek watershed BLE modeling. The California Creek study area is in Haskell Stonewall, Throckmorton, Fisher, Jones, and Shackelford counties.

# Terrain

Completed by AECOM.

# Hydrology

- Completed by AECOM.

# Hydraulics

            - Curve number was adjusted based on the TxDOT Climatic Adjustment suggestions, for Paint Creek study area, TxDOT suggested to use the regulatory CNs minus 20. After several runs for 100-year calibration at 4 locations, the final model applied -17 for SCS curve number adjustment.

- Regression equation flow calibration at 4 locations is listed in the table below.

# Mapping

# Other Assumptions

- Minor tiger roads removed include I, C, S, U, and O.

- Stream centerlines were buffered 100ft to both sides for Manning’s override regions in which a n value of 0.05 was applied.

- Breaklines were applied at a 100’x100’ grid cell size.

- CN was adjusted through several iterations of the 100-year run which was calibrated at 4 points. The model applied -17 for the SCS CN adjustment.

- Refine region with 50’x50’ cell size used in population centers.