# Texas 2D BLE Analysis QA/QC Checklist

# Project Information

| HUC-8 Watershed Name & ID: | Paint & 12060103 |
| --- | --- |
| HUC-10 Watershed Name & ID: | Upper Paint Creek & 1206010302, Lower Paint Creek & 12060304, Upper California Creek 1206010301, Lower California Creek 1206010303 |
| Modeler: | Simon X. Li Aldo Hernandez |
| Reviewer: |  |

## Terrain and Modeling Review (QC Items 1-6)

| Modeler Initials: | XL, AH | Reviewer Initials: | SD |
| --- | --- | --- | --- |
| Submittal Date: | 10/28/2022 | Review Submitted Date: |  |
| Review Response Date: | 01/20/2023 | Backcheck Complete Date: |  |

## Mapping Review (QC Item 7)

| Modeler Initials: | XL, AH, MS | Reviewer Initials: |  |
| --- | --- | --- | --- |
| Submittal Date: | 01/20/2023 | Review Submitted Date: |  |
| Review Response Date: | 03/07/2023 | Backcheck Complete Date: |  |

## Completeness Review (QC Item 8)

| Modeler Initials: | SDS | Reviewer Initials: |  |
| --- | --- | --- | --- |
| Submittal Date: | 05/09/2023 | Review Submitted Date: |  |
| Review Response Date: |  | Backcheck Complete Date: |  |

(Modeler to populate with an “X” for items included in the QC submittal)

| Level of Study |  |  | Flood Events |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| X | 2D BLE |  | X | 10% ACE (10-year) | X | 0.2% ACE (500-year) |
|  | Enhanced BLE |  | X | 4% ACE (25-year) | X | 1% plus |
|  |  |  | X | 2% ACE (50-year) | X | 1% minus |
|  |  |  | X | 1% ACE (100-year) |  | Observed Rainfall |

Instructions

Each section includes instructions to the modeler and reviewer regarding their responsibilities in the completion of this checklist. In the Methodologies / Data Sources tables, grey italic text is added as an example response. Additional sample responses are located in the Texas 2D BLE Guidelines Quick Guide.

Reviewer to indicate if Technical Review items generally comply using a “Pass”. If the technical review observes an issue of noncompliance or an item that requires additional attention, an “Fail” should be utilized.

## Submittal Checklist

Data submitted for each milestone should be carried through each subsequent milestone. All geospatial layers and models must have same defined projection system as terrain. Modeler to populate with an “X” for items included in the QC submittal.

| Base Map Data (QC items 1-6) |  |
| --- | --- |
| X | Location Map (PDF): HUC-8 and HUC-10 boundaries and/or model domain boundary if different from HUC-10s, stream gages, dams (if applicable), mainstem streams (for reference), background map |
| X | GIS Map (MXD or Map Package) with the following data included but not limited to: Stream centerlines, model breaklines, rainfall data, WSEL and Depth grids (10, 100, 500-yr), adjacent HUC mapping (available on the BLE viewer or currently studying), 2D connections for inflows/outflows, gages or points of flow comparisons (BFEs, highwater marks, etc.), profile lines, boundary condition lines, refinement regions, model domain boundary, model mesh, DFIRM mapping panels, base map data (roads, aerials, etc.), soils data, land use data, terrain |
| Technical Report / Documentation (QC items 1-6) |  |
| X | Technical Report / Project Notebook: assumptions, special considerations, and modeling concerns |
|  | Detailed explanation and support data of variances in methodologies and approaches from recommended guidelines (Should be included in Technical Report / Project Notebook) |
| X | Internal QA/QC Documentation: comments and responses |
| N/A | Flow comparison tables within the technical report |
| Hydrology (QC items 1-6) |  |
| N/A | Hydrologic Model (HEC-HMS) model with all associated DSS files for 7 required storm events |
| X | Hydrologic Data and Parameter Spreadsheets (rainfall calculations, curve number table, gage locations, gage analysis and flow validation) |
| N/A | Hydrologic modeling notebook including assumptions, notes, and special conditions (if not provided in report) |
| Hydraulics (QC items 1-6) |  |
| X | Hydraulic Model (HEC-RAS) with all associated files and 7 required storm events (remove non-final files) |
| X | Hydraulic Data and Parameter Spreadsheets (Manning’s n-value table, infiltration calculations support data-Soils, Land Use, Curve Number table, other calculations/data used in model) |
| X | Manning’s n-value table |
| X | Precipitation data correctly linked to hydrologic model or hard coded from provided spreadsheet |
| X | Hydraulic modeling notebook including assumptions, notes, and special conditions |
| Mapping (QC item 7) |  |
| X | Flood risk database |
| X | Modeler to complete topology validation as outlines in the guidelines |
| X | Modeler to verify mapping meets deliverable requirements as outlined in guidelines |
| Completeness Review (QC item 8) See Texas 2D BLE Guidelines Chapter 4 – Deliverables |  |
| X | Technical BLE Report |
| NA | Final cleaned hydrologic model |
| X | Final cleaned hydraulic model |
| X | Flood risk database |
| X | CMNS updates |
| X | HAZUS results |
| x | Additional supplemental data |

# 1 – Model Terrain

Methodologies / Data Sources (Modeler to populate)

| Terrain Source(s): | Provided by AECOM |
| --- | --- |
| Data Gaps: |  |
| Source Resolution: |  |
| Model Terrain Resolution: | 1 meter |
| Terrain Horizontal Projection: | NAD_1983_StatePlane_Texas_North_Central_FIPS_4202_Feet |
| Terrain Vertical Projection: |  |
| Z Values: | D_North_American_1983 |
| Terrain Buffer: | 10000-ft |
| Other: |  |

## Technical Review (Reviewer to confirm with Pass/Fail)

| General Review |  |
| --- | --- |
|  | LiDAR source matches the latest available data at time of project initiation |
|  | Large reservoir or lake extents included in model domain |
|  | Check visible seams, gaps or holes of LiDAR sources (if more than one source was used) |

## Review Comments

- No Comment.

# 2 – Hydrology

## Methodologies / Data Sources (Modeler to populate)

| Software: | Python Script |
| --- | --- |
| Rainfall Source: | NWS/NOAA Precipitation Frequency Data Server PF Map: Contiguous US (noaa.gov) |
| Rainfall Duration: | 5 minutes to 24 hours |
| Depth Selection: | 0.90 inches to 6.95 inches |
| Rainfall Temporal Distribution: |  |
| Areal Reduction: |  |
| Loss Methodology: | SCS Curve Number |
| Unit Hydrograph Transform Method: | SCS Unit Hydrograph |
| Inflow Hydrograph Source: | From upstream California Creek Model |
| Other: The hydrology for TO1 4 HUC-8 watersheds has been created by using a Python Script the AECOM team has developed. The DSS file was created by using the Gridded precipitation data from NOAA Atlas 14. The gridded precipitation data were temporally distributed using the rainfall distribution created for the TO1 area. A temporal distribution for the 10%, 4%, 2%, 1%, and 0.2% annual-chance events was developed for TO1 study area by downloading gridded precipitation for durations of 5 minutes to 24 hours for each frequency. |  |

## Statistical Data (Modeler to populate)

| Stream Gage Name & ID: | ## California Ck nr Stamford, TX & 08084800 |
| --- | --- |
| Stream Gage Name & ID: |  |
| Stream Gage Name & ID: |  |

## Technical Review (Reviewer to confirm with  Pass/Fail)

| Excess Rainfall |  |
| --- | --- |
|  | Precipitation values are an aerial average of Atlas 14 values for the entire 2D area boundary for each storm event or other appropriate rainfall application |
|  | CN values appropriately reflect land use and soils for entire 2D area boundary |
|  | Losses are correctly calculated to establish the excess rainfall hyetographs by checking land use, soils and infiltration layers or other relative information |
|  | Total precipitation depths for 1%+ and 1%- are calculated correctly |
|  | Excess rainfall hyetographs are calculated for all storm events (10%, 4%, 2%, 1%, 0.2%, 1%+, and 1%-) |
|  | Cumulative excess rainfall values are in increasing order from 10%, 4%, 2%, 1%, to 0.2% |
|  | Cumulative excess rainfall values for the 1%+ and 1%- fall in order with the storm events matching the total precipitation values |
| Inflow Hydrographs |  |
|  | Shape and time-to-peak of any inflow hydrographs resemble past documented large storm events |
|  | Hydrographs reasonable when compared to the other return periods. The shapes should be similar, and hydrographs from different events should not intersect unless there is an engineering reason. Peak flows from a lower return period event should not be higher than a higher return period event |
|  | Contributing hydrographs from another RAS model applied at coincident locations (Provide locations in base map data) |
| General Hydrology |  |
|  | Any flow regulating dams in the model area? (Yes/No) |
|  | Dam modeling methodologies applied and explained? (Yes/No). If yes, provide discharge rating curve calculations and gage analysis documenting regulated versus unregulated records, if applicable. |
|  | Model aligns with documentation of dam modeling approaches. |
| Other Notes: NA |  |

## Review Comments

- California Creek H&H notebook is very brief and does not provide all information about methods, approach, assumptions. Consider providing a report section or reference in information similar to Paint Creek notebook.

Response: Other. Methods, approach, and assumptions included in report.

Please submit the report with next submittal  Agreed

# 3 – Hydraulic Geometry

## Methodologies / Data Sources (Modeler to populate)

| Software: | HEC-RAS 6.1.0 |
| --- | --- |
| Loss Methodology: | SCS Curve Number |
| Soils Source: | NRCS web soil survey |
| Land Cover Source: | NLCD 2019 |
| Manning's Roughness Source: | V.T Chow, TWDB 2D BLE guidance |
| Nominal Mesh Cell Sizes: | 200 x 200 |
| Other: |  |

## Technical Review (Reviewer to confirm with Pass/Fail)

| 2D Mesh |  |
| --- | --- |
|  | 2D Boundary delineated along watershed boundary or buffer to ensure all contributing area is included |
|  | Nominal cell size appropriate for model detail (generally 200 feet, minimum 50 feet for refinement areas) |
|  | Computation points are at default locations or changes documented |
|  | Mesh Cell and Face tolerances are default or changes documented |
|  | No mesh errors |
|  | Cell face relationships are calculated |
| Breaklines |  |
|  | Breaklines are consistent with the terrain data |
|  | Breaklines sufficiently detailed to capture features (thalweg, banks, berms, etc.) |
|  | Cells do not overlap terrain features where appropriate (i.e., unwanted leaking) |
|  | Breakline cell sizing and parameters are appropriate |
|  | 2D Area Connections placed at applicable benchmarks, gages, or dams |
|  | 2D Area Connections set to use normal 2D equation domain for benchmarks and gages. Appropriate modeling approach used for dams. |
| Manning’s N-Values |  |
|  | Manning’s n-values associated to land cover data provided |
|  | Land Cover classifications are sufficiently descriptive within the data table |
|  | Manning’s n-value classifications generally match current aerial imagery |
|  | Manning’s n dataset is associated with the geometry files |
| General |  |
|  | All model datasets each have unique descriptive names |

## Review Comments

Paint Model-

- Most of the Features layers, projection file links were broken. Suggest saving all reference files, etc within the main RAS folder.

KFA Response: Agree.  Will save all reference files into the Feature folder which is within the main RAS folder.

No additional comments.

- Add breaklines for road near Haskell area-

KFA Response: Other.  The breaklines for the city of Haskell were intentionally removed. The grid cell size for the population center was set as 50’x50’, which is much smaller than breakline assigned cell size of 100’x100’. Adding breaklines would not help to improve the model results. Another issue is that the City of Haskell has previous detailed study, which will be superseded the BLE results in mapping.

The main use of breaklines is not for changing cell size but to help re-orient cells to the high ground of road avoiding “leaky cells”. Leaky cells can send artificially more water downstream adding inaccuracies in the model for downstream areas.  AECOM/KFA Response: Modeling team has reviewed the location and concluded that best path forward is to avoid this additional model change/breakline change at this time, noting that any leakage is minimal and more importantly not impacting model calibration/verification checks (to be documented with BLE report); however, the comment is noted and additional steps will be taken at early model stages in future task orders to identify, assess, and make changes (if needed) to breakline usage. Thank you for the comment.

AECOM/KFA M3 Update:  Breaklines in the area were added, based on re-review of the area and placement in appropriate locations based on the comment.  Sensitivity run was completed for 1PAC, showing very little impact to upstream or downstream flows, however, the geometry edit was incorporated into the 7 multiplan runs and all models re-run.  Mapping was assessed to determine where revised mapping was warranted, and downstream outflow data was provided to receiving HUC8 BLE modeling team.

High grounds/berm Near Stamford Area

Response: Other.  This is a minor road and per TWDB’s guidance, no need to add a breakline.

Breaklines are added to avoid “leaky cells”. They should be added when there is unintended significant flow caused by leaky cells. Refer to TWDB Guidelines Section 2.3.4. AECOM/KFA Final Response: Modeling team has reviewed the location and concluded that best path forward is to avoid this additional model change/breakline change at this time, noting that any leakage is minimal and more importantly not impacting model calibration/verification checks (to be documented with BLE report); however, the comment is noted and additional steps will be taken at early model stages in future task orders to identify, assess, and make changes (if needed) to breakline usage. Thank you for the comment.

AECOM/KFA M3 Update:  Similar to above, see final comment response.

Response: Other.  This is a minor road and per TWDB’s guidance, no need to add a breakline.

Breaklines are added to avoid “leaky cells”. They should be added when there is unintended significant flow caused by leaky cells. Refer to TWDB Guidelines Section 2.3.4. AECOM/KFA Final Response: Modeling team has reviewed the location and concluded that best path forward is to avoid this additional model change/breakline change at this time, noting that any leakage is minimal and more importantly not impacting model calibration/verification checks (to be documented with BLE report); however, the comment is noted and additional steps will be taken at early model stages in future task orders to identify, assess, and make changes (if needed) to breakline usage. Thank you for the comment.

AECOM/KFA M3 Update:  Similar to above, see final comment response.

Response: Agree.  A breakline was added to the top of the berm.

No additional comment.

California Model-

- The folder structure for RAS model is very complex and confusing. There are folders for each frequency with RAS models under each. What is the difference between each model? Follow folder structure in the BLE Guidance for these submittals.

Response: Agree. Models were the same, the files will be fixed so only one model is submitted rather than having stand-alone frequency folders.

No additional comments.

- There are parallel breaklines (#351 and 699(1), 288 and 696, 164 and 280) defined which are causing to cell faces not aligning to the breaklines and making those “leaky” across the embankment/berm.

Response: Other. The first two sets of parallel breaklines have been fixed. Breaklines 164 and 280 represent a roadway system with two roadways running parallel that clearly stand out in the terrain. The distance between the two is adequate so those have remained.

Breaklines are added so that cell faces are aligned to the high ground in order to capture that topography. If breaklines with current cell sizes are not capturing them, then either cell sizes should be reduced or manual edits should be made to align cells correctly. AECOM/KFA Final Response: Modeling team has reviewed the location and concluded that best path forward is to avoid this additional model change/breakline change at this time, noting that any leakage is minimal and more importantly not impacting model calibration/verification checks (to be documented with BLE report); however, the comment is noted and additional steps will be taken at early model stages in future task orders to identify, assess, and make changes (if needed) to breakline usage. Thank you for the comment.

AECOM/KFA M3 Response:  California model geometry revised in this area, and 1PAC model rerun to test sensitivity.  While the flow impact was very minor, the geometry edit was incorporated into the 7 multiplan runs and all models re-run.  Mapping was assessed to determine where revised mapping was warranted, and downstream outflow data was provided to Paint Creek model.

- Add “v-notches”/leaky cells near Hamlin

Response: Agree. V-notches have been added.

No additional comments.

# 4 – Hydraulic Boundary Conditions

## Technical Review (Reviewer to confirm with Pass/Fail)

| Boundary Review |  |
| --- | --- |
|  | External boundaries connected to 2D mesh |
|  | Boundary conditions everywhere flow could build up at the boundary of the 2D mesh |
|  | Boundary conditions have a hydrograph or elevation/slope data assigned |
|  | Input hydrographs consistent with hydrologic data (DSS, manual entry) |
|  | Precipitation data is consistent with hydrologic data (DSS, manual entry) |
|  | Downstream boundary consistent with ground slope or known downstream WSEL |
|  | Flow hydrographs applied at appropriate locations |
|  | Energy grade slope for hydrograph distribution is reasonable |
|  | Internal rating curves appropriate and documented for control structures (if applicable) |

## Review Comments

- Could not review and match calculations for rainfall to the actual input dss.

No response provided. Please provide additional documentation of rainfall distribution. AECOM/KFA Final Response: We will confirm and ensure that hydrologic data, support files, and methodology is submitted with the next/final submittal.

# 5 – Hydraulic Model Execution

## Methodologies / Data Sources (Modeler to populate)

| Unsteady Plan Names |  |  |  |
| --- | --- | --- | --- |
| 10% ACE (10-year): | 10PAC | 0.2% ACE (500-year): | 0.2PAC |
| 4% ACE (25-year): | 4PAC | 1% plus: | 1PAC_PLUS |
| 2% ACE (50-year): | 2PAC | 1% minus: | 1PAC_MIN |
| 1% ACE (100-year): | 1PAC |  |  |
| Other: |  |  |  |

## Statistical Data (Modeler to populate)

| Time Step: | 7.5 ~ 240 Second (Adjust Time Based on Courant) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Courant Value: | 0.4~3 |  | Simulation Length: | 96 | hours |
| Computation Interval: | 60 | seconds | Model Runtime: | 16 | hours |
| Hydrograph Output Interval: | 60 | minutes | Mapping Output Interval: | 60 | minutes |

## Technical Review (Reviewer to confirm with Pass/Fail)

| Execution Review |  |
| --- | --- |
| x | Appropriate 2D parameters are used (Boundary Conditions, Time Step, Simulation Length, etc.) |
| x | Hydrograph Output Interval and Mapping Output Interval set at a reasonable interval to capture peaks |
| x | Model runtime is reasonable (< 12 hours, but dependent upon model size) |

## Review Comments

- For California Creek model- consider making hydrograph and mapping output at the same interval.

Response: Agree. This has been fixed.

No additional comments

- Both models has runtime is in excess of 12 hours. Consider revising adaptive time step parameters. California model can be stopped after the peak has passed on the d/s end.

Response: Noted. this has remained the same per BLE guidance.

Per TWDB Guidance “The TWDB prefers lower runtimes with an end user model runtime goal of under 12 hours and ideally under 6 hours. However, the TWDB recommends the runtime be established not based solely on runtime but also such that a Courant Value less than two (2) is achieved.” AECOM/KFA Final Response: Modeling team has reviewed the location and concluded that best path forward is to avoid parameter changes to the model at this time. The team has worked under the assumption  that the model run times mentioned in the BLE guidance document is a target preference and recommendation, but not a requirement since some model and/or BLE study areas may require longer runtimes (due to a variety of factors).  We will note this in our BLE report and AECOM PM will discuss in more detail with both TWDB staff and our modeling teams going forward. Thank you for the comment.

# 6 – Hydraulic Model Results

## Methodologies / Data Sources (Modeler to populate)

| Flow Validation/Gage Analysis |  |
| --- | --- |
| Software: | PeakFQ 7.3 |
| Methodology | EMA (17C) |
| Removal of Outliers and Justification | MGBT |
| Validation Notes (describe parameter adjustments): 59 peaks available from 1963 to 2021 |  |
| Other: |  |

## Technical Review (Reviewer to confirm with Pass/Fail)

| Computation Checks |  |
| --- | --- |
|  | Model runs with no cell errors |
|  | Volume balance is within tolerance (ΔV < 1% is ideal, however 2% to 3% can be acceptable for rain on mesh) |
| Results Spot Checks Check at all critical structures, major confluences, a handful of headwater tributaries, downstream end, at benchmarks, and a handful of randomly selected locations. |  |
| x | No visible spikes in WSEL visibly present with a single cell |
| x | WSEL and depth grids appear reasonable (no artificial dips or spikes) |
| x | Visual verification flooding is connected downstream for all significant contributing areas (> 64 acres) |
| x | Velocity grids appear reasonable (no unusually high velocity areas or Courant values greater than 2) |
| x | No crossing profiles between all events |
| x | Inflow and outflow hydrograph timing is reasonable as compared to the peak |
| x | Spills into/from adjacent HUCs appropriately accounted for |
| x | WSEL tie-in between model domains and HUCs along major rivers/streams where inflows are applied |
| Results Full Checks |  |
| x | Computational time step meets ideal Courant condition |
| x | Diffusion Wave: V∆T∆X ≤2 (Up to 5) (Results are unit-less) |
| x | Model simulation is long enough to pass the entire hydrograph through the model (minimum of 12 hours past the peak, preferably 24 hours) |
| x | Hydrographs at internal and external boundaries are smooth |
| x | Contributing areas are contained with 2D mesh or accounted for with inflow hydrographs |
| x | No flow trapped at 2D boundary |
| x | Visual verification mapping is reasonable compared to effective |
| Flow Verification |  |
| x | Peak flow gage analysis performed on all gages in the watershed with sufficient data |
| x | All gages used have an appropriate period of record (10 to 20+ years) |
| x | All gages used have no extensive gaps in record which would impact the analysis |
| x | Gage analysis results validated to other sources (Regression, USGS, etc.) |
| x | Results at applicable benchmarks justified (gages, HWMs, effective BFEs) |
| x | Peak discharges comparable to gages, effective/prior studies, regional regression equations |

## Review Comments

- For Paint model, there are no profile lines defined to extract model results for regression results comparisons (Profile lines shapefile under features layer is missing too).

Response: Agree. Will add profile lines shapefile in the Feature folder.

Profile lines shapefile is empty/missing. Remove/update results layers generated for each frequency run.

- For California model, profile lines are present. Those profiles lines (to extract result flows) should be perpendicular to flow and should be enforced so that cell faces align with them. The results noted in the notebook needs to be updated to reflect current results in the model. E.g. Dry California is listed as 5,680 cfs while model shows 6,036 cfs. Check and revise all.

Response: Agree. 2D connections have been added at profile line locations to enforce cells. Values will be updated.

Revise report section and provide it for review.

Agreed

- Results comparison to the gage station is not noted in the notebook.

Response: Noted. Will add comparisons at applicable gage stations.

Provide report/report section for review.

Agreed

- Discuss and compare FEMA effective FPs with BLE results during in the report

Response: Noted.

Provide report/report section for review.

Agreed

# 7 – Mapping Results

## Technical Review (Reviewer to confirm with Pass/Fail)

| Hydraulic Model |  |
| --- | --- |
|  | HEC-RAS model with results, pertinent data and required storm events |
| Flood Risk Database |  |
|  | All features fully populated per current FEMA guidelines |
|  | Mapping consistent between grids to polygons for all three required mapped events |
|  | 10yr smaller than 100yr smaller than 500yr |
|  | Holes filled in polygon, depth and WSE grids |
|  | Boundary interfaces align matching or identified as AOMI |

## Review Comments

- FRD

- S_Pol_Ar Fields to Update:

- POL_NAME3- Update to empty string “” – No change

- ANI_FIRM- Update to empty string “” – No change

- COM_NFO_ID- Field should contain the CID (Community Identification Number) update to include only the CID. – Change made for consistency, but this field doesn’t have to equal CID value (see FIRM DB Tech Ref)

- S_HUC_Ar- Fields correctly attributed no further comments

- S_FRAC_Ar- Empty- Required feature class – Added to DB

- S_FRAS_Pt- Empty

- S_AOMI_Ar- Fields correctly attributed no further comments

- BFE_2D- Fields correctly attributed no further comments

- DTL_STUD_AR-

- CD_YN- Currently Null update according to BLE Submittal Guidance populate with PRELIM, YES or NO – This would get populated once community meeting held.

- CD_POC- Currently Null update to “NP” – NP is only used for Required fields in DB, Null is acceptable.

- CD_ADD1- Currently Null update to “NP” -NP is only used for Required fields in DB, Null is acceptable.

- CD_CTY- Currently Null update to “NP” - NP is only used for Required fields in DB, Null is acceptable.

- CD_STATE- Currently Null update to “NP” - NP is only used for Required fields in DB, Null is acceptable.

- CD_ZIP- Currently Null update to “NP” - NP is only used for Required fields in DB, Null is acceptable.

- CD_EMAIL- Currently Null update to “NP” - NP is only used for Required fields in DB, Null is acceptable.

- DTL_STUD_LN-

- CD_YN- Currently Null update according to BLE Submittal Guidance populate with PRELIM, YES or NO - This would get populated once community meeting held.

- CD_POC- Currently Null update to “NP” - NP is only used for Required fields in DB, Null is acceptable.

- CD_ADD1- Currently Null update to “NP” - NP is only used for Required fields in DB, Null is acceptable.

- CD_CTY- Currently Null update to “NP” - NP is only used for Required fields in DB, Null is acceptable.

- CD_STATE- Currently Null update to “NP” - NP is only used for Required fields in DB, Null is acceptable.

- CD_ZIP- Currently Null update to “NP” - NP is only used for Required fields in DB, Null is acceptable.

- CD_EMAIL- Currently Null update to “NP” - NP is only used for Required fields in DB, Null is acceptable.

- WTR_LN- Fields correctly attributed no further comments

- WTR_AR- Fields correctly attributed no further comments

- FLD_HAZ_AR-

- EST_AR_ID- Add leading 0’s to ID – The only requirement is that this value is unique, leading 0’s are unnecessary.  Even having sequential numbers aren’t a requirement.

- ZONE_SUBTY- 1% currently Null update to empty string “” - <Null> is acceptable.

- VEL_UNIT- currently Null update to empty string “” -<Null> is acceptable.

- AR_REVERT- currently Null update to empty string “”-<Null> is acceptable.

- AR_SUBTRV- currently Null update to empty string “”-<Null> is acceptable.

- DUAL_ZONE- currently Null update to domain value “False”- <Null> is acceptable. This field only needs a T/F designation for Zone AR zones.

*Note under GRIDs regarding edge effects along HUC 8 boundary.

- TENPCT_FP-

- ZONE_SUBTY- currently Null update to empty string “”-<Null> is acceptable.

- AR_REVERT- currently Null update to empty string “”-<Null> is acceptable.

- AR_SUBTRV- currently Null update to empty string “”-<Null> is acceptable.

- DUAL_ZONE- currently Null update to domain value “False” - <Null> is acceptable. This field only needs a T/F designation for Zone AR zones.

- GRIDs

- BLE_DEP01PCT- correctly matches boundary of Zone A floodplains

- BLE_WSE01PCT- correctly matches boundary of Zone A floodplains

- BLE_DEP0_2PCT- correctly matches boundary of Zone X floodplains

- BLE_WSE0_2PCT- correctly matches boundary of Zone X floodplains

*Note Edge effects occurring in all GRIDs along border of HUC 8 boundary this occurs in the FLD_HAZ_AR as well but also exceeds floodplain boundary in some spots. Please clean up along HUC 8 boundary, refer to QC polygon for location and spot check for other areas along boundary.  AECOM/KFA response:  Please note we are providing the QC polygon shapefile back with the final comment responses just for documentation – but we have reviewed/agreed with all comment locations (indicated by an “addressed” response to each comment noted in the shapefile). Thank you for the comments and for providing that helpful shapefile.

# 8 – Completeness Review

## Technical Review (Reviewer to confirm with Pass/Fail)

| File Structure and Naming |  |
| --- | --- |
|  | Deliverables provided in TWDB/FEMA required file folder structure and naming convention Please note: Submittal provided in eBFE format to allow for direct submittal to FEMA R6 and USGS and use on the eBFE viewer. Being a TWDB-only funded task order, it is anticipated that submittal to the MIP will remain in eBFE format, per past and similar task orders. |
|  | Data missing (if any) documented in report and ReadMe file within the required folder |
| Report |  |
|  | Technical 2D BLE report documents all assumptions, special considerations, etc. |
|  | Flow validation and difference discussed in report |
|  | Flow comparisons discussed in report |
| Hydrologic Model |  |
|  | HEC-HMS model with results, pertinent data and required storm events |
| Hydraulic Model |  |
|  | HEC-RAS model with results, pertinent data and required storm events |
| Flood Risk Database |  |
|  | All features fully populated per current FEMA guidelines |
| CNMS |  |
|  | Updated correctly with new centerlines |
|  | A1-A5 checks complete |
|  | CNMS Post- Processing Complete |
| HAZUS |  |
|  | Run per FEMA guidance |

## Review Comments

QA/QC Checklist Approval

This 2D Base Level Engineering Analysis QA/QC Review is in compliance with the contract requirements and all task checkpoints are complete.  The independent QC Team has reviewed the analysis, presented review comments to the Production Team Task Leader, and discussed any problems or issues.  The Task Leader, QC Manager, QC Reviewer, and Project Manager have signed the QA/QC Checklist to confirm that all comments have been received, addressed, and documented appropriately.

| QA/QC Team – [TWDB or company name] |  |  |  | Production Team – [add company name] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Seyoum Asemenaw, PE, CFM |  | 05/09/2023 |
| QC Manager |  | Date |  | Task Lead |  | Date |
|  |  |  |  | Sean Sutton, PE, CFM |  | 05/09/2023 |
| QC Reviewer |  | Date |  | Project Manager |  | Date |