# TWDB 2D BLE – Paint Creek H&H Technical Notebook

Description: The HUC8 Paint (12060103) includes 4 HUC10, two of them are in the Paint Creek Study Area.

- 1206010302, Upper Paint Creek

- 1206010304, Lower Paint Creek

Most of the Paint Creek study area are in Haskell County, and small portions of the study area are in Throckmorton County, Jones County, and Stonewell County. Three Population centers are located within the Paint Creek study area (applied 50’x50’ grid cell size):

- City of Haskell

- City Stamford

- Stamford Lake Marina

Lake Stamford, a reservoir formed by Stamford Dam, is located 10 miles northeast of Stamford, Texas, in southeastern Haskell County. The lake had a storage capacity in 1999 of 51,573 acre-feet (63,614,000 m^3) and drains an area of 368 square miles (950 km2). Due to sedimentation, the estimated capacity as of 2013 is 50,061 acre⋅ft (61,749,000 m3). https://en.wikipedia.org/wiki/Lake_Stamford - cite_note-Stamford_Report.PDF_-_Stamford1999_FinalReport.pdf-2.

The City of Stamford owns the lake and half the water rights to Lake Stamford, while American Electric Power (AEP) owns the other half. The city also owns, operates, and maintains associated Stamford Dam. Lake Stamford serves as the municipal water source for Stamford and the small neighboring communities of Lueders, Ericksdahl, Paint Creek, Sagerton, Avoca, and Corinth. For the lake's 60-year history, water levels have retained significant importance to these communities, providing them their sole source of water. Several notable rain events occurred in 1978 and 2005, resulting in record water levels. Recreational activities include boating, fishing, waterskiing, jet-skiing, and camping. Two marinas are on the lake: Stamford Marina on the south shore, and Anchor Marina on the north.

# Terrain

- The terrain was provided by AECOM terrain team. Raw LiDAR data from Texas Natural Resources Information System (TRNIS) TNRIS - Texas Natural Resources Information System

# Hydrology

The following data were used to develop precipitation DSS, infiltration parameters, and terrain in a. tiff format.

- Precipitation Data: NWS/NOAA Precipitation Frequency Data Server PF Map: Contiguous US (noaa.gov)

- Soils Data: USDA/NRCS Web Soil Survey

- Land Use Data: NLCD 2019 https://www.mrlc.gov/data

- The hydrology for TO1 4 HUC-8 watersheds has been created by using a Python Script the AECOM team has developed. The DSS file was created by using the Gridded precipitation data from NOAA Atlas 14. The gridded precipitation data were temporally distributed using the rainfall distribution created for the TO1 area. A temporal distribution for the 10%, 4%, 2%, 1%, and 0.2% annual-chance events was developed for TO1 study area by downloading gridded precipitation for durations of 5 minutes to 24 hours for each frequency. The following shows the 2% annual chance event precipitation for different frequencies.

| Dur minutes | Frequency | rainfall- in |
| --- | --- | --- |
| 5 | 5-min | 0.90 |
| 10 | 10-min | 1.46 |
| 15 | 15-min | 1.78 |
| 30 | 30-min | 2.42 |
| 60 | 60-min | 3.13 |
| 120 | 2-hr | 3.92 |
| 180 | 3-hr | 4.42 |
| 360 | 6-hr | 5.27 |
| 720 | 12-hr | 6.09 |
| 1440 | 24-hr | 6.95 |

- The precipitation DSS data was imported into the HEC-RAS model

- Infiltration parameters were imported to HEC-RAS, including the Abstraction Ratio, Curve Number, and minimum infiltration rate following the TWDB 2D BLE Project Modeling Guidance.

- Manning’s n values for different land use classification have been populated in the infiltration parameters shapefile used to import CNs and other infiltration parameters.

- The 100-ft buffer zones were created using the final streamlines, and then input as manning’s override regions by assigning the n values of 0.05 for stream channel.

- The grid cell size for break lines and stream centerline was set as 100’x100’.

- Lake Stamford was using 500’x500’ gride cell size with the manning’s override region of 0.04.

The Initial run was taken and found that the water surface elevation at Stamford Lake is approximately 1425.8 ft, way above the normal pool elevation, which is 1416.8 ft above NGVD 29, since the Terrain data vertical project is NAVD88, then convert the NGVD29 to NAVD88 at the place, which is 0.479 higher than NGVD29. Thus, the normal pool elevation at Stamford Lake should be 1417.28 ft above NAVD88.

- Hydraulics

The Stamford dam was modeled based on the dam data. Below data was obtained form

https://www.waterdatafortexas.org/reservoirs/individual/stamford

The datum was converted to NGVD88 using NOAA’s Online Vertical Datum Transformation tool.

https://vdatum.noaa.gov/vdatumweb/vdatumweb?a=134103420220623

The resulted vertical elevation from NGVD29 to NAVD 88 would need a 0.479-ft increase at Stamford Lake. For example, the Conservation pool elevation would change from 1416.8 ft above NGVD29 to 1417.28-ft above NAVD88.

### Stamford Dam:

Per TWDB “VOLUMETRIC SURVEY OF LAKE STAMFORD” (January 24, 2000), Stamford Dam and appurtenant structures consist of an earth fill embankment approximately 3,600 feet in length, with a maximum height of 78 feet and a crest elevation of 1436.8 feet NGVD29. The service spillway is an excavated channel cut through limestone rock located approximately 900 feet to the left of the dam. The uncontrolled spillway crest is 100 feet in length at elevation 1,416.8 feet NGVD29. The low-flow outlet works consist of a 24-inch diameter concrete pipe that is 442 feet in length bisects the embankment approximately 500 feet south of the old Paint Creek channel. The Invert elevation is 1382.8 feet NGVD29. The emergency spillway is a natural channel located at the right end of the embankment with a crest elevation of 1425.8 feet NGVD29.

The dam was modeled using a 2D Connection, all elevations or inverts were converted to NAVD88.

Calbibration: 
For simating a reaonsble water surface elevation at the Stamford Lake, the assumption made is that at beginning, the lake water surfave elevation was assumed to be equal to the conservation pool elevaion, which is 1417.28-ft NAVD88. The Lidar data shows the terrain at Stamford Lake is 1413.22-ft, mean when the LiDAR was take, the water level was lower than normal pool elevation.

Three methods were applied to the model for a reasonable water surface elevation for 100-year:

      - Update the Terrain to Normal Pool Elevation at 1417.28-ft

      - Clone current terrain and update the elevation at lake to 1417.28-ft

      - Create a storage area and assume the starting water surface elevation is 1417.28-ft

Regression equation flows calibration, 6 locations lisited in the below table.

Assumptions:

            - Curve number was adjusted based on the TxDOT Climatic Adjustment suggestions, for Paint Creek study area, TxDOT suggested to use the regulatory CNs minus 20. After several runs for 100-year calibration at 6 locations, the final model applied -18 for SCS curve number adjustment.

            - The Stamford Lake terrain was adjusted to conservation pool elevation, which is 1417.28-ft NAVD88.

            - Break lines were applied to the model with 100-x100’ grid cell size

            - Stream centerline was buffered 100-ft both side for Manning’s Override region, using n values 0.05

            - Stream centerlines were treated as breaklines, using 100’x100’ gride cell size.

            - Stamford Lake area was modeled using larger gride cell size 500’x500’.

            - Manning n values for Stamford Lake was set as 0.04 for open water area

            - Modeled 100-year water surface elevation at Stamford Lake is 1423.86-ft NAVD88.

# Mapping

# Other Assumptions