# Texas 2D BLE Analysis QA/QC Checklist

# Project Information

| HUC-8 Watershed Name & ID: | Lower Clear Fork Brazos: 12060104 |
| --- | --- |
| HUC-10 Watershed Name & ID: | Foyle Creek- Clear Fork Brazos River: 1206010401 Kings Creek – Clear Fork Brazos River: 1206010402 Bufford Creek – Clear Fork Brazos River: 1206010403 |
| Modeler: | Joel Ocanas, EIT, CFM |
| Reviewer: |  |

## Terrain and Modeling Review (QC Items 1-6)

| Modeler Initials: | JO, SA | Reviewer Initials: | AC |
| --- | --- | --- | --- |
| Submittal Date: | 2/16/2023 | Review Submitted Date: | Received 3/7/23 |
| Review Response Date: |  | Backcheck Complete Date: | 6/8/23 |

## Mapping Review (QC Item 7)

| Modeler Initials: | JO, ES, SA | Reviewer Initials: | AC |
| --- | --- | --- | --- |
| Submittal Date: | 2/16/2023 | Review Submitted Date: | Received 3/7/23 |
| Review Response Date: |  | Backcheck Complete Date: | 6/8/23 |

## Completeness Review (QC Item 8)

| Modeler Initials: | SDS | Reviewer Initials: | EA |
| --- | --- | --- | --- |
| Submittal Date: | 5/17/2023 | Review Submitted Date: | 6/8/23 |
| Review Response Date: | 07/18/2023 | Backcheck Complete Date: |  |

(Modeler to populate with an “X” for items included in the QC submittal)

| Level of Study |  |  | Flood Events |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| X | 2D BLE |  | X | 10% ACE (10-year) | X | 0.2% ACE (500-year) |
|  | Enhanced BLE |  | X | 4% ACE (25-year) | X | 1% plus |
|  |  |  | X | 2% ACE (50-year) | X | 1% minus |
|  |  |  | X | 1% ACE (100-year) |  | Observed Rainfall |

Instructions

Each section includes instructions to the modeler and reviewer regarding their responsibilities in the completion of this checklist. In the Methodologies / Data Sources tables, grey italic text is added as an example response. Additional sample responses are located in the Texas 2D BLE Guidelines Quick Guide.

Reviewer to indicate if Technical Review items generally comply using a “Pass”. If the technical review observes an issue of noncompliance or an item that requires additional attention, an “Fail” should be utilized.

## Submittal Checklist

Data submitted for each milestone should be carried through each subsequent milestone. All geospatial layers and models must have same defined projection system as terrain. Modeler to populate with an “X” for items included in the QC submittal.

| Base Map Data (QC items 1-6) |  |
| --- | --- |
| X | Location Map (PDF): HUC-8 and HUC-10 boundaries and/or model domain boundary if different from HUC-10s, stream gages, dams (if applicable), mainstem streams (for reference), background map (associated shapefiles provided) |
| X | GIS Map (MXD or Map Package) with the following data included but not limited to: Stream centerlines, model breaklines, rainfall data, WSEL and Depth grids (10, 100, 500-yr), adjacent HUC mapping (available on the BLE viewer or currently studying), 2D connections for inflows/outflows, gages or points of flow comparisons (BFEs, highwater marks, etc.), profile lines, boundary condition lines, refinement regions, model domain boundary, model mesh, DFIRM mapping panels, base map data (roads, aerials, etc.), soils data, land use data, terrain. (associated shapefiles provided and also part of HECRAS model submittal) |
| Technical Report / Documentation (QC items 1-6) |  |
| X | Technical Report / Project Notebook: assumptions, special considerations, and modeling concerns |
| X | Detailed explanation and support data of variances in methodologies and approaches from recommended guidelines (Should be included in Technical Report / Project Notebook) |
| X | Internal QA/QC Documentation: comments and responses |
| X | Flow comparison tables within the technical report |
| Hydrology (QC items 1-6) |  |
| NA | Hydrologic Model (HEC-HMS) model with all associated DSS files for 7 required storm events |
| X | Hydrologic Data and Parameter Spreadsheets (rainfall calculations, curve number table, gage locations, gage analysis and flow validation) |
| X | Hydrologic modeling notebook including assumptions, notes, and special conditions (if not provided in report) |
| Hydraulics (QC items 1-6) |  |
| X | Hydraulic Model (HEC-RAS) with all associated files and 7 required storm events (remove non-final files)d |
| X | Hydraulic Data and Parameter Spreadsheets (Manning’s n-value table, infiltration calculations support data-Soils, Land Use, Curve Number table, other calculations/data used in model) |
| X | Manning’s n-value table |
| X | Precipitation data correctly linked to hydrologic model or hard coded from provided spreadsheet |
| X | Hydraulic modeling notebook including assumptions, notes, and special conditions |
| Mapping (QC item 7) |  |
| X | Flood risk database |
| X | Modeler to complete topology validation as outlines in the guidelines |
| X | Modeler to verify mapping meets deliverable requirements as outlined in guidelines |
| Completeness Review (QC item 8) See Texas 2D BLE Guidelines Chapter 4 – Deliverables |  |
| X | Technical BLE Report |
| NA | Final cleaned hydrologic model |
| X | Final cleaned hydraulic model |
| X | Flood risk database |
| X | CMNS updates |
| X | HAZUS results |
| X | Additional supplemental data |

# 1 – Model Terrain

Methodologies / Data Sources (Modeler to populate)

| Terrain Source(s): | LIDAR |
| --- | --- |
| Data Gaps: | N/A |
| Source Resolution: | 1-meter |
| Model Terrain Resolution: | 1-meter |
| Terrain Horizontal Projection: | NAD 1983 State Plane Texas North Central FIPS 4202 |
| Terrain Vertical Projection: | NAVD 88 |
| Z Values: | D_North_American_1983 |
| Terrain Buffer: | 10,000-ft |
| Other: NA |  |

## Technical Review (Reviewer to confirm with Pass/Fail)

| General Review |  |
| --- | --- |
| Pass | LiDAR source matches the latest available data at time of project initiation |
| Pass | Large reservoir or lake extents included in model domain |
| Pass | Check visible seams, gaps or holes of LiDAR sources (if more than one source was used) |

## Review Comments

- No comments.

# 2 – Hydrology

## Methodologies / Data Sources (Modeler to populate)

| Software: | HEC-RAS 6.1 |
| --- | --- |
| Rainfall Source: | National Oceanic and Atmospheric Administration (NOAA) Atlas 14 Gridded Precipitation |
| Rainfall Duration: | 24 hours |
| Depth Selection: | Atlas 14 |
| Rainfall Temporal Distribution: | NRCS synthetic temporal distributions |
| Areal Reduction: | Areal Reduction was not used |
| Loss Methodology: | SCS |
| Unit Hydrograph Transform Method: | N/A |
| Inflow Hydrograph Source: | Inflow from Paint Creek, Gonsolus Creek, and Upper Clear Fork work areas |
| Other: A spatially distributed precipitation Data Storage System (DSS) data was generated from the National Oceanic and Atmospheric Administration (NOAA) Atlas 14 gridded precipitation for the 10%,4%, 2%, 1%, +1%, -1%, and 0.2% annual-chance events. |  |

## Statistical Data (Modeler to populate)

| Stream Gage Name & ID: | USGS 08085500, USGS 08087300 |
| --- | --- |

## Technical Review (Reviewer to confirm with Pass/Fail)

| Excess Rainfall |  |
| --- | --- |
| Fail | Precipitation values are an aerial average of Atlas 14 values for the entire 2D area boundary for each storm event or other appropriate rainfall application |
| Pass | CN values appropriately reflect land use and soils for entire 2D area boundary |
| Pass | Losses are correctly calculated to establish the excess rainfall hyetographs by checking land use, soils and infiltration layers or other relative information |
| Fail | Total precipitation depths for 1%+ and 1%- are calculated correctly |
| Pass | Excess rainfall hyetographs are calculated for all storm events (10%, 4%, 2%, 1%, 0.2%, 1%+, and 1%-) |
| Pass | Cumulative excess rainfall values are in increasing order from 10%, 4%, 2%, 1%, to 0.2% |
| Pass | Cumulative excess rainfall values for the 1%+ and 1%- fall in order with the storm events matching the total precipitation values |
| Inflow Hydrographs |  |
| Pass | Shape and time-to-peak of any inflow hydrographs resemble past documented large storm events |
| Pass | Hydrographs reasonable when compared to the other return periods. The shapes should be similar, and hydrographs from different events should not intersect unless there is an engineering reason. Peak flows from a lower return period event should not be higher than a higher return period event |
| Pass | Contributing hydrographs from another RAS model applied at coincident locations (Provide locations in base map data) |
| General Hydrology |  |
| No | Any flow regulating dams in the model area? (Yes/No) |
| NA | Dam modeling methodologies applied and explained? (Yes/No). If yes, provide discharge rating curve calculations and gage analysis documenting regulated versus unregulated records, if applicable. |
| NA | Model aligns with documentation of dam modeling approaches. |
| Other Notes: NA |  |

## Review Comments

- AECOM Provided Comments:

  - Modeling comments are resolved in the detailed QA/QC spreadsheet that is submitted with this checklist.  Please ignore

- Couldn’t verify precipitation. Can’t load the DSS files for that or any of the inflows

AECOM Response: Precipitation DSS will be provided in final deliverable, however, we are seeing on our end that these DSS files are accessible and loading.  It may be beneficial to jump on a call and screenshare to see if we can assess what’s happening and resolve.  Also note, the  Hydrologic methods, approach, and assumptions included in report and with next submittal (including supporting documents).

DSS files are now accessible.

# 3 – Hydraulic Geometry

## Methodologies / Data Sources (Modeler to populate)

| Software: | HEC-RAS 6.1 |
| --- | --- |
| Loss Methodology: | SCS Curve Number |
| Soils Source: | NRCS Soil Data |
| Land Cover Source: | National Land Cover Database (NLCD, 2019) |
| Manning's Roughness Source: | National Land Cover Database (NLCD, 2019) |
| Nominal Mesh Cell Sizes: | 200-foot cells, except for 100-feet along the 1 square mile streams and 50-foot cells within community boundaries using refinement regions. |
| Other: |  |

## Technical Review (Reviewer to confirm with Pass/Fail)

| 2D Mesh |  |
| --- | --- |
| Pass | 2D Boundary delineated along watershed boundary or buffer to ensure all contributing area is included |
| Pass | Nominal cell size appropriate for model detail (generally 200 feet, minimum 50 feet for refinement areas) |
| Pass | Computation points are at default locations or changes documented |
| Pass | Mesh Cell and Face tolerances are default or changes documented |
| Pass | No mesh errors |
| Pass | Cell face relationships are calculated |
| Breaklines |  |
| Fail | Breaklines are consistent with the terrain data |
| Fail | Breaklines sufficiently detailed to capture features (thalweg, banks, berms, etc.) |
| Pass | Cells do not overlap terrain features where appropriate (i.e., unwanted leaking) |
| Pass | Breakline cell sizing and parameters are appropriate |
| Pass | 2D Area Connections placed at applicable benchmarks, gages, or dams |
| Pass | 2D Area Connections set to use normal 2D equation domain for benchmarks and gages. Appropriate modeling approach used for dams. |
| Manning’s N-Values |  |
| Pass | Manning’s n-values associated to land cover data provided |
| Pass | Land Cover classifications are sufficiently descriptive within the data table |
| Pass | Manning’s n-value classifications generally match current aerial imagery |
| Pass | Manning’s n dataset is associated with the geometry files |
| General |  |
| Pass | All model datasets each have unique descriptive names |

## Review Comments

- AECOM Provided Comments:

  - Modeling comments are resolved in the detailed QA/QC spreadsheet that is submitted with this checklist. Please ignore

- Geometry comments in QC_RAS Features

Note: LCFB did not have a buffer in some areas but did follow the HUC boundary in areas without buffer.

Note: LCFB was submitted as a single HUC8 model and not smaller model domains.

| Review Comment | AECOM Response | Back Check Comment |
| --- | --- | --- |
| (2-1) Consider rearranging VNotch. Flow appears to pond upstream of road | Cells were rearranged and the breakline was edited so v-notches can allow flow to pass through the road. | VNotch were added. The mapping looks the same. No further comment |
| (2-2) Consider reinforcing breaklines/adjusting cells to avoid leakage | The breaklines were edited and reinforced to avoid leakage. | Addressed. Mapping leakage has been removed. |
| (2-3) Consider reenforcing/cleaning up double breaklines or adjusting cells to avoid leakage | The double breakline was removed and the remaining breakline was edited / reinforced to avoid leakage. | Double break line has been removed /cells has been adjusted. Leakage still present. No further comment |
| (2-4) Consider adding CL Breaklins along main channel. Could help with velocity spikes in the area. | Area was left as is to avoid causing issues with tie-in locations with upstream models. M3 Submittal Update: N-value refinement areas were added in various areas to assist with velocity spikes, but maintain tie-in with upstream. | Velocity spikes still present but much lower than before and in less locations. No further comment |
| (2-5) Velocity spike of over 100 for the 100yr. | The velocity spike issue will be resolved in final deliverable. | Velocity spike no longer present |
| (2-6) Some large velocity spikes in the area | The velocity spike issues will be resolved in the final deliverable. | Velocity spikes no longer present |
| (3) Note: LCFB did not have a buffer in some areas but did follow the HUC boundary in areas without buffer | The LCFB perimeter was extended to include a buffer area around the entirety of model. | Ok |
| (4) Note: LCFB was submitted as a single HUC8 model and not smaller model domains. | Since LCFB is a small HUC-8, it was modeled as a single HUC-8. | Ok |

# 4 – Hydraulic Boundary Conditions

## Technical Review (Reviewer to confirm with Pass/Fail)

| Boundary Review |  |
| --- | --- |
| Pass | External boundaries connected to 2D mesh |
| Pass | Boundary conditions everywhere flow could build up at the boundary of the 2D mesh |
| Pass | Boundary conditions have a hydrograph or elevation/slope data assigned |
| Fail | Input hydrographs consistent with hydrologic data (DSS, manual entry) |
| Fail | Precipitation data is consistent with hydrologic data (DSS, manual entry) |
| Pass | Downstream boundary consistent with ground slope or known downstream WSEL |
| Pass | Flow hydrographs applied at appropriate locations |
| Pass | Energy grade slope for hydrograph distribution is reasonable |
| Pass | Internal rating curves appropriate and documented for control structures (if applicable) |

## Review Comments

- AECOM Provided Comments:

  - Modeling comments are resolved in the detailed QA/QC spreadsheet that is submitted with this checklist. Please ignore

- Was not able to access the Input DSS files to verify inflow hydrographs/precipitation.

| Review Comment | AECOM Response | Back Check Comment |
| --- | --- | --- |
| (2) Was not able to access the Input DSS files to verify inflow hydrographs/precipitation. | All DSS files (precipitation and inflow hydrograph) will be included in final deliverable, however, we are seeing on our end that these DSS files are accessible and loading. It may be beneficial to jump on a call and screenshare to see if we can assess what’s happening and resolve. Also note, the Hydrologic methods, approach, and assumptions included in report and with next submittal (including supporting documents). | We can access DSS files. The inflow hydrographs have been verified. |

# 5 – Hydraulic Model Execution

## Methodologies / Data Sources (Modeler to populate)

| Unsteady Plan Names |  |  |  |
| --- | --- | --- | --- |
| 10% ACE (10-year): | 10pct | 0.2% ACE (500-year): | 0_2pct |
| 4% ACE (25-year): | 4pct | 1% plus: | 1PLUSpct |
| 2% ACE (50-year): | 2pct | 1% minus: | 1MINpct |
| 1% ACE (100-year): | 1pct |  |  |
| Other: |  |  |  |

## Statistical Data (Modeler to populate)

| Time Step: | Controlled by Courant condition. |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Courant Value: | 0.4-1 |  | Simulation Length: | 96 | hours |
| Computation Interval: | 60 | seconds | Model Runtime: | 4-16 | hours |
| Hydrograph Output Interval: | 60 | minutes | Mapping Output Interval: | 60 | minutes |

## Technical Review (Reviewer to confirm with Pass/Fail)

| Execution Review |  |
| --- | --- |
| Pass | Appropriate 2D parameters are used (Boundary Conditions, Time Step, Simulation Length, etc.) |
| Pass | Hydrograph Output Interval and Mapping Output Interval set at a reasonable interval to capture peaks |
| Fail | Model runtime is reasonable (< 12 hours, but dependent upon model size) |

## Review Comments

- AECOM Provided Comments:

  - Modeling comments are resolved in the detailed QA/QC spreadsheet that is submitted with this checklist.  Please ignore

- Some of the plans are run at 1 min and others at 5min computation interval. Consider running all at 1 to help with velocity spikes.

- Note: LCFB was submitted as a single HUC8 model and not smaller model domains. May be necessary to split into smaller model domains to achieve TWDB goal runtime of less than 12 hours, preferably under 6 hours.

| Review Comment | AECOM Response | Back Check Comment |
| --- | --- | --- |
| (2) Some of the plans are run at 1 min and others at 5min computation interval. Consider running all at 1 to help with velocity spikes. | All plans will be run for final deliverable using “Adjust Time Step Based on Courant” with the following settings, and the time step starting was set to 1-minute for all plans: | Noted. No further comment |
| (3) Note: LCFB was submitted as a single HUC8 model and not smaller model domains. May be necessary to split into smaller model domains to achieve TWDB goal runtime of less than 12 hours, preferably under 6 hours. | The LCFB final deliverable will be submitted as single HUC8 model at this stage. Modeling team has concluded that best path forward is to avoid splitting the model at this time due to constraints with schedule and budget, though this is a consideration the team will note moving forward. The team has worked under the assumption that the model run times mentioned in the BLE guidance document is a target preference and recommendation, but not a requirement since some model and/or BLE study areas may require longer runtimes (due to a variety of factors). We will note this in our BLE report and AECOM PM will discuss in more detail with both TWDB staff and our modeling teams going forward. Thank you for the comment | Noted. No further comment |

# 6 – Hydraulic Model Results

## Methodologies / Data Sources (Modeler to populate)

| Flow Validation/Gage Analysis |  |
| --- | --- |
| Software: | HEC-RAS 6.1 |
| Methodology | USGS Gage 08085500, USGS Gage 08087300 and Regression Analysis of 5 sub-basins |
| Removal of Outliers and Justification |  |
| Two USGS gages are located within the LCFB work area. Regression analysis of 5 other sub-basins within the basin were used to further validate flow within the area. |  |
| Other: |  |

## Technical Review (Reviewer to confirm with Pass/Fail)

| Computation Checks |  |
| --- | --- |
| Fail | Model runs with no cell errors |
| Pass | Volume balance is within tolerance (ΔV < 1% is ideal, however 2% to 3% can be acceptable for rain on mesh) |
| Results Spot Checks Check at all critical structures, major confluences, a handful of headwater tributaries, downstream end, at benchmarks, and a handful of randomly selected locations. |  |
| Pass | No visible spikes in WSEL visibly present with a single cell |
| Pass | WSEL and depth grids appear reasonable (no artificial dips or spikes) |
| Pass | Visual verification flooding is connected downstream for all significant contributing areas (> 64 acres) |
| Fail | Velocity grids appear reasonable (no unusually high velocity areas or Courant values greater than 2) |
| Pass | No crossing profiles between all events |
| Pass | Inflow and outflow hydrograph timing is reasonable as compared to the peak |
| Pass | Spills into/from adjacent HUCs appropriately accounted for |
| Pass | WSEL tie-in between model domains and HUCs along major rivers/streams where inflows are applied |
| Results Full Checks |  |
| Fail | Computational time step meets ideal Courant condition |
| Fail | Diffusion Wave: V∆T∆X ≤2 (Up to 5) (Results are unit-less) |
| Pass | Model simulation is long enough to pass the entire hydrograph through the model (minimum of 12 hours past the peak, preferably 24 hours) |
| Pass | Hydrographs at internal and external boundaries are smooth |
| Pass | Contributing areas are contained with 2D mesh or accounted for with inflow hydrographs |
| Pass | No flow trapped at 2D boundary |
| Pass | Visual verification mapping is reasonable compared to effective |
| Flow Verification |  |
| Fail | Peak flow gage analysis performed on all gages in the watershed with sufficient data |
| Pass | All gages used have an appropriate period of record (10 to 20+ years) |
| Pass | All gages used have no extensive gaps in record which would impact the analysis |
| Pass | Gage analysis results validated to other sources (Regression, USGS, etc.) |
| Pass | Results at applicable benchmarks justified (gages, HWMs, effective BFEs) |
| Pass | Peak discharges comparable to gages, effective/prior studies, regional regression equations |

## Review Comments

- AECOM Provided Comments:

  - Modeling comments are resolved in the detailed QA/QC spreadsheet that is submitted with this checklist.  Please ignore

| Review Comment | AECOM Response | Back Check Comment |
| --- | --- | --- |
| (2) Runtime Message Errors in 1pct plan. | All runtime errors will be resolved in final deliverable. | Noted. No further comment |
| (3) High velocities spike. Added comments in QC_RAS Features for the major ones but recommend looking throughout HUC. | All high velocity spikes will be resolved in final deliverable. | Most major ones have been removed. Noted. |
| (4) Courant values greater than 2 and greater than 5 in some areas found in HUC. | All plans will be run for final deliverable using “Adjust Time Step Based on Courant” with the following settings: | Noted. No further comment |
| (5) In the HH Notebook, the first gage screenshot shows a gage that does not match the number for DMF Brazos RV nr Aspermont, TX. The second gage should be labeled 08087300 based on the screenshot from USGS. Please update the table to compare results. | The HH Notebook for final deliverable updated to reflect gages in the LCFB model area which are 08085500 (Clear Fk Brazos at Ft Griffin, TX) and 08087300 (Clear Fk Brazos at Eliasville, TX). M3 submittal update: Report provided with this submittal. | Ok. |
| (6) The Peak FQ data that was submitted shows inputs for gage at Griffin TX instead of DMF Brazos RV nr Aspermont, TX. Please verify correct data was used to run gage analysis. | The HH Notebook for final deliverable updated to reflect gages in the LCFB model area which are 08085500 (Clear Fk Brazos at Ft Griffin, TX) and 08087300 (Clear Fk Brazos at Eliasville, TX). M3 submittal update: Report provided with this submittal. | Ok. |
| (7) In the notes for this review section, it states that gage 08080500 is being used for analysis even though it is outside of HUC boundary. In the HH notebook, data for gage 08087300 is shown. That one has sufficient data and is within HUC boundary. Please clarify. | The notes for this review section were updated to include the gages within the model location: 08085500 (Clear Fk Brazos at Ft Griffin, TX) and 08087300 (Clear Fk Brazos at Eliasville, TX). M3 submittal update: Report provided with this submittal. | Ok. |
| (8) Please provide some information on how the UCFB hydrograph calibration was done. Also, provide some more information of tie-in/calibration to it considering it is a 1D study. | Peak flows for UCFB were obtained from FEMA effective 1D HEC-RAS model. As the drainage area of UCFB is very similar to Paint Creek, the hydrograph for Paint Creek was used as a template for developing the hydrograph for UCFB. This hydrograph was scaled to have the peak flow of UCFB. Timing of the peak was shifted based on calibration using USGS gages within the LCFB HUC8 basin, and to find a tie-in location based on WSEL at Cross Sections within the 1D UCFB model. An excel sheet is provided to show development of the hydrograph. M3 submittal update: Report provided with this submittal. | Ok. |

# 7 – Mapping Results

## Technical Review (Reviewer to confirm with Pass/Fail)

| Hydraulic Model |  |
| --- | --- |
| Pass | HEC-RAS model with results, pertinent data and required storm events |
| Flood Risk Database |  |
| Pass | All features fully populated per current FEMA guidelines |
| Pass | Mapping consistent between grids to polygons for all three required mapped events |
| Pass | 10yr smaller than 100yr smaller than 500yr |
| Pass | Holes filled in polygon, depth and WSE grids |
| Pass | Boundary interfaces align matching or identified as AOMI |

## Review Comments

- For BFE_2D, under BFE_LN_TYP the domain is 1010 or 1020 but should read “Mapped” or “Not Mapped.”  - Corrected - Addressed

- Verify that for WTR_AR and WTR_LN the domains are being used for SHOWN_FIRM and SHOWN_INDX. – Corrected - Addressed

- Note: CNMS and HAZUS were missing/empty in FRD. Please submit for review with next milestone. Will be included on final submittal. Per the checklist at top of this document, those were understood to be part of the Completeness Review submittal (QC Item 8 below, as well). – CNMS and HAZUS now have been submitted in the final review

# 8 – Completeness Review

## Technical Review (Reviewer to confirm with Pass/Fail)

| File Structure and Naming |  |
| --- | --- |
| Pass | Deliverables provided in TWDB/FEMA required file folder structure and naming convention Please note: Submittal provided in eBFE format to allow for direct submittal to FEMA R6 and USGS and use on the eBFE viewer. Being a TWDB-only funded task order, it is anticipated that submittal to the MIP will remain in eBFE format, per past and similar task orders. |
| Pass | Data missing (if any) documented in report and ReadMe file within the required folder |
| Report |  |
| Pass | Technical 2D BLE report documents all assumptions, special considerations, etc. |
| Pass | Flow validation and difference discussed in report |
| Pass | Flow comparisons discussed in report |
| Hydrologic Model |  |
| NA | HEC-HMS model with results, pertinent data and required storm events |
| Hydraulic Model |  |
| Pass | HEC-RAS model with results, pertinent data and required storm events |
| Flood Risk Database |  |
| Pass | All features fully populated per current FEMA guidelines |
| CNMS |  |
|  | Updated correctly with new centerlines |
|  | A1-A5 checks complete |
|  | CNMS Post- Processing Complete |
| HAZUS |  |
| Pass | Run per FEMA guidance |

## Review Comments

- See LCFB_Mapping_QC.shp for mapping and FRD comments. General comments summarized below:

  - Connect spillways for 100 and 500-yr for NID structures.

Mapping updated to include requested spillways, per comment, follow-up discussion on 6/16/23 and confirmation email on 6/29/23.

  - Map ponds for areas included in WTR_AR. May need to revise extents for inclusion in mapping.

Mapping updated to include requested ponded water areas, per comment, follow-up discussion on 6/16/23 and confirmation email on 6/29/23.

  - Missing 100-yr mapping where there is 500-yr mapping, please review/revise as necessary.

Mapping of 1% floodplain has been updated, per comment, follow-up discussion on 6/16/23 and confirmation email on 6/29/23.

  - O.2% depth grid missing for majority of HUC

Agree, the grid somehow much have been corrupted during our packaging/transfer process.  W have replaced grid with a valid dataset, and we will attempt to do some verification that the dataset is good prior to compressing and resending.

  - Add AOMI areas for the tie in to the 1D Upper Clear Fork Brazos BLE study and inline dams along mainstem where WSE/Depth changes are dramatic

AOMI polygon added at the Upper Clear Fork Brazos HUC.  AOMI’s added to the dam locations.

- Report comments provided in BLE Report - Lower Clear Fork Brazos HUC-8 2D BLE.docx.

All report comments addressed and a revised LCFB BLE report is to be included with the resubmittal.

- Please note, CNMS was not reviewed at this time and will be reviewed in backcheck.

We have updated the LCFB CNMS datasets in an attempt to be consistent with the CNMS discussions for our other Task Order 1 HUCs, specifically updated after our coordination call on 6.16.23 that included TWDB, AECOM, Halff, and Michael Johnson.  We have updated all of our HUCs’ CNMS layers using a revised approach consistent with this coordination discussion. Please note: TWDB staff (Kelley Rich) emailed a request to see if a successful (Passing) review was completed for the DMFB HUC8 CNMS submittal (sent prior to this one on 6.27.23), if that same approval could stand for the others HUC’s revised CNMS submittals – to expedite final close-out of these Task Order 1 AECOM HUCs.

QA/QC Checklist Approval

This 2D Base Level Engineering Analysis QA/QC Review is in compliance with the contract requirements and all task checkpoints are complete.  The independent QC Team has reviewed the analysis, presented review comments to the Production Team Task Leader, and discussed any problems or issues.  The Task Leader, QC Manager, QC Reviewer, and Project Manager have signed the QA/QC Checklist to confirm that all comments have been received, addressed, and documented appropriately.

| QA/QC Team – [TWDB or company name] |  |  |  | Production Team – [add company name] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Seyoum Asamenaw (AECOM) |  | 05/17/2023 |
| QC Manager |  | Date |  | Task Lead |  | Date |
|  |  |  |  | Sean Sutton (AECOM) |  | 07/18/2023 |
| QC Reviewer |  | Date |  | Project Manager |  | Date |