# Texas 2D BLE Analysis QA/QC Checklist

# Project Information

| HUC-8 Watershed Name & ID: | Hubbard: 12060105 |
| --- | --- |
| HUC-10 Watershed Name & ID: | Hubbard 1206010_502_503 (model area: Hubbard Creek model) |
| Modeler: | Courtney Fournier, EIT |
| Reviewer: | Anna Castillo |

## Terrain and Modeling Review (QC Items 1-6)

| Modeler Initials: | SZ, CF, ES, SA | Reviewer Initials: | AC |
| --- | --- | --- | --- |
| Submittal Date: | 1/20/2023 | Review Submitted Date: | Received 2/13/2023 |
| Review Response Date: |  | Backcheck Complete Date: | 5/23/23 |

## Mapping Review (QC Item 7)

| Modeler Initials: | SZ, CF, ES, SA | Reviewer Initials: | AC |
| --- | --- | --- | --- |
| Submittal Date: | 1/20/2023 | Review Submitted Date: | Received 2/13/2023 |
| Review Response Date: |  | Backcheck Complete Date: | 5/23/23 |

## Completeness Review (QC Item 8)

| Modeler Initials: | SDS | Reviewer Initials: | AC |
| --- | --- | --- | --- |
| Submittal Date: | 05/09/2023 | Review Submitted Date: | 5/23/23 |
| Review Response Date: | 5/26/2023 | Backcheck Complete Date: |  |

(Modeler to populate with an “X” for items included in the QC submittal)

| Level of Study |  |  | Flood Events |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| X | 2D BLE |  | X | 10% ACE (10-year) | X | 0.2% ACE (500-year) |
|  | Enhanced BLE |  | X | 4% ACE (25-year) | X | 1% plus |
|  |  |  | X | 2% ACE (50-year) | X | 1% minus |
|  |  |  | X | 1% ACE (100-year) |  | Observed Rainfall |

Instructions

Each section includes instructions to the modeler and reviewer regarding their responsibilities in the completion of this checklist. In the Methodologies / Data Sources tables, grey italic text is added as an example response. Additional sample responses are located in the Texas 2D BLE Guidelines Quick Guide.

Reviewer to indicate if Technical Review items generally comply using a “Pass”. If the technical review observes an issue of noncompliance or an item that requires additional attention, an “Fail” should be utilized.

## Submittal Checklist

Data submitted for each milestone should be carried through each subsequent milestone. All geospatial layers and models must have same defined projection system as terrain. Modeler to populate with an “X” for items included in the QC submittal.

| Base Map Data (QC items 1-6) |  |
| --- | --- |
| X | Location Map (PDF): HUC-8 and HUC-10 boundaries and/or model domain boundary if different from HUC-10s, stream gages, dams (if applicable), mainstem streams (for reference), background map (associated shapefiles provided) |
| X | GIS Map (MXD or Map Package) with the following data included but not limited to: Stream centerlines, model breaklines, rainfall data, WSEL and Depth grids (10, 100, 500-yr), adjacent HUC mapping (available on the BLE viewer or currently studying), 2D connections for inflows/outflows, gages or points of flow comparisons (BFEs, highwater marks, etc.), profile lines, boundary condition lines, refinement regions, model domain boundary, model mesh, DFIRM mapping panels, base map data (roads, aerials, etc.), soils data, land use data, terrain. (associated shapefiles provided and also part of HECRAS model submittal) |
| Technical Report / Documentation (QC items 1-6) |  |
| X | Technical Report / Project Notebook: assumptions, special considerations, and modeling concerns |
| X | Detailed explanation and support data of variances in methodologies and approaches from recommended guidelines (Should be included in Technical Report / Project Notebook) |
| X | Internal QA/QC Documentation: comments and responses |
|  | Flow comparison tables within the technical report |
| Hydrology (QC items 1-6) |  |
| NA | Hydrologic Model (HEC-HMS) model with all associated DSS files for 7 required storm events |
| X | Hydrologic Data and Parameter Spreadsheets (rainfall calculations, curve number table, gage locations, gage analysis and flow validation) |
| X | Hydrologic modeling notebook including assumptions, notes, and special conditions (if not provided in report) |
| Hydraulics (QC items 1-6) |  |
| X | Hydraulic Model (HEC-RAS) with all associated files and 7 required storm events (remove non-final files)d |
| X | Hydraulic Data and Parameter Spreadsheets (Manning’s n-value table, infiltration calculations support data-Soils, Land Use, Curve Number table, other calculations/data used in model) |
| X | Manning’s n-value table |
| X | Precipitation data correctly linked to hydrologic model or hard coded from provided spreadsheet |
| X | Hydraulic modeling notebook including assumptions, notes, and special conditions |
| Mapping (QC item 7) |  |
| X | Flood risk database |
| X | Modeler to complete topology validation as outlines in the guidelines |
| X | Modeler to verify mapping meets deliverable requirements as outlined in guidelines |
| Completeness Review (QC item 8) See Texas 2D BLE Guidelines Chapter 4 – Deliverables |  |
| x | Technical BLE Report |
| NA | Final cleaned hydrologic model |
| x | Final cleaned hydraulic model |
| x | Flood risk database |
| x | CMNS updates |
| x | HAZUS results |
| x | Additional supplemental data |

# 1 – Model Terrain

Methodologies / Data Sources (Modeler to populate)

| Terrain Source(s): | LIDAR |
| --- | --- |
| Data Gaps: | N/A |
| Source Resolution: | 1-meter |
| Model Terrain Resolution: | 1-meter |
| Terrain Horizontal Projection: | NAD 1983 State Plane Texas North Central FIPS 4202 |
| Terrain Vertical Projection: | NAVD 88 |
| Z Values: | D_North_American_1983 |
| Terrain Buffer: | 10,000-ft |
| Other: NA |  |

## Technical Review (Reviewer to confirm with Pass/Fail)

| General Review |  |
| --- | --- |
| P | LiDAR source matches the latest available data at time of project initiation |
| P | Large reservoir or lake extents included in model domain |
| P | Check visible seams, gaps or holes of LiDAR sources (if more than one source was used) |

## Review Comments

# 2 – Hydrology

## Methodologies / Data Sources (Modeler to populate)

| Software: | HEC-RAS 6.1 |
| --- | --- |
| Rainfall Source: | National Oceanic and Atmospheric Administration (NOAA) Atlas 14 Gridded Precipitation |
| Rainfall Duration: | 24 hours |
| Depth Selection: | Atlas 14 |
| Rainfall Temporal Distribution: | NRCS synthetic temporal distributions |
| Areal Reduction: | Areal Reduction was not used |
| Loss Methodology: | SCS |
| Unit Hydrograph Transform Method: | N/A |
| Inflow Hydrograph Source: | No inflow into Hubbard Creek. |
| Other: A spatially distributed precipitation Data Storage System (DSS) data was generated from the National Oceanic and Atmospheric Administration (NOAA) Atlas 14 gridded precipitation for the 10%,4%, 2%, 1%, +1%, -1%, and 0.2% annual-chance events. |  |

## Statistical Data (Modeler to populate)

| Stream Gage Name & ID: | USGS 08086050 |
| --- | --- |
| Stream Gage Name & ID: | USGS 08086212 |

## Technical Review (Reviewer to confirm with Pass/Fail)

| Excess Rainfall |  |
| --- | --- |
| P | Precipitation values are an aerial average of Atlas 14 values for the entire 2D area boundary for each storm event or other appropriate rainfall application |
| P | CN values appropriately reflect land use and soils for entire 2D area boundary |
| P | Losses are correctly calculated to establish the excess rainfall hyetographs by checking land use, soils and infiltration layers or other relative information |
| P | Total precipitation depths for 1%+ and 1%- are calculated correctly |
| P | Excess rainfall hyetographs are calculated for all storm events (10%, 4%, 2%, 1%, 0.2%, 1%+, and 1%-) |
| P | Cumulative excess rainfall values are in increasing order from 10%, 4%, 2%, 1%, to 0.2% |
| P | Cumulative excess rainfall values for the 1%+ and 1%- fall in order with the storm events matching the total precipitation values |
| Inflow Hydrographs |  |
| NA | Shape and time-to-peak of any inflow hydrographs resemble past documented large storm events |
| NA | Hydrographs reasonable when compared to the other return periods. The shapes should be similar, and hydrographs from different events should not intersect unless there is an engineering reason. Peak flows from a lower return period event should not be higher than a higher return period event |
| NA | Contributing hydrographs from another RAS model applied at coincident locations (Provide locations in base map data) |
| General Hydrology |  |
| Yes | Any flow regulating dams in the model area? (Yes/No) |
| Yes | Dam modeling methodologies applied and explained? (Yes/No). If yes, provide discharge rating curve calculations and gage analysis documenting regulated versus unregulated records, if applicable. |
| Yes | Model aligns with documentation of dam modeling approaches. |
| Other Notes: NA |  |

## Review Comments

- Please provide a location map.

AECOM Response: A location map will be provided in the project report.

OK.

# 3 – Hydraulic Geometry

## Methodologies / Data Sources (Modeler to populate)

| Software: | HEC-RAS 6.1 |
| --- | --- |
| Loss Methodology: | SCS Curve Number |
| Soils Source: | NRCS Soil Data |
| Land Cover Source: | National Land Cover Database (NLCD, 2019) |
| Manning's Roughness Source: | National Land Cover Database (NLCD, 2019) |
| Nominal Mesh Cell Sizes: | 200-foot cells, except for 100-feet along the 1 square mile streams and 50-foot cells within community boundaries using refinement regions. |
| Other: |  |

## Technical Review (Reviewer to confirm with Pass/Fail)

| 2D Mesh |  |
| --- | --- |
| P | 2D Boundary delineated along watershed boundary or buffer to ensure all contributing area is included |
| P | Nominal cell size appropriate for model detail (generally 200 feet, minimum 50 feet for refinement areas) |
| P | Computation points are at default locations or changes documented |
| P | Mesh Cell and Face tolerances are default or changes documented |
| P | No mesh errors |
| P | Cell face relationships are calculated |
| Breaklines |  |
| P | Breaklines are consistent with the terrain data |
|  | Breaklines sufficiently detailed to capture features (thalweg, banks, berms, etc.) |
| P | Cells do not overlap terrain features where appropriate (i.e., unwanted leaking) |
| P | Breakline cell sizing and parameters are appropriate |
|  | 2D Area Connections placed at applicable benchmarks, gages, or dams |
|  | 2D Area Connections set to use normal 2D equation domain for benchmarks and gages. Appropriate modeling approach used for dams. |
| Manning’s N-Values |  |
|  | Manning’s n-values associated to land cover data provided |
| P | Land Cover classifications are sufficiently descriptive within the data table |
| P | Manning’s n-value classifications generally match current aerial imagery |
| P | Manning’s n dataset is associated with the geometry files |
| General |  |
|  | All model datasets each have unique descriptive names |

## Review Comments

- Please find QC_file under features in RAS Mapper.

AECOM Response: Noted. The QC file is provided in a shapefile that refers to adding a breakline at the following location. RAS Mapper shows the embankment is not overtopping even at the 500-year storm.

AECOM updated M3: While there appears to be some “leaking” due to corners of two cells reaching upstream and downstream sides of the embankment, it is not greatly impacting the results or warranting a re-run of the seven model profiles. A sensitivity analysis was performed for the 100-year storm event. A breakline was added on the dam embankment and run with identical computation settings as the previous model runs to directly compare results. The figures below show the mesh before and after with the resulting depths. The blue depth raster is the existing depth rasters, the gray depth raster displays the results with the new model run including the breakline on the dam embankment.

|  | Only difference that stands out from having a BL |
| --- | --- |

The flow observed just downstream of the dam is higher by about 800 cfs when there is no BL on the dam embankment. The peak flow observed is over 30,000 cfs, and therefore, the additional 800 cfs is negligible (2.4%), and slightly conservative in the without breakline configuration.

The depth was compared for the model run with a BL and without a BL. The WSE difference is less than 0.1 feet. The WSE for the model run without a BL was slightly higher, and therefore a more conservative value.

Profile lines were also checked upstream of the dam to confirm that no WSE elevations were changed due to the dam not having a BL on the embankment. The max difference between having a BL or not having a BL was less than 0.02 feet.

The peak flow at the outlet gage was also compared. The peak flow changed by 0.12%, and the elevation difference was less than 0.01 feet.  The dam is registered on NID and therefore labeled as an AOMI for further study.

OK.

- Small portion on the downstream area does not have landcover.

AECOM Response: The small area missing a landcover map is located in Gunsolus study area. The land cover map is extended to cover the missing area and will be provided with the updated Gunsolus HEC-RAS model. See below image.

OK.

- To what extent are CLs being used? Are they to the 1sqmi or further?

AECOM Response: The CLs are being used to the 1 sqmi threshold and up to 0.5 sqmi in population centers.

OK.

# 4 – Hydraulic Boundary Conditions

## Technical Review (Reviewer to confirm with Pass/Fail)

| Boundary Review |  |
| --- | --- |
| P | External boundaries connected to 2D mesh |
| P | Boundary conditions everywhere flow could build up at the boundary of the 2D mesh |
| P | Boundary conditions have a hydrograph or elevation/slope data assigned |
| P | Input hydrographs consistent with hydrologic data (DSS, manual entry) |
| P | Precipitation data is consistent with hydrologic data (DSS, manual entry) |
| P | Downstream boundary consistent with ground slope or known downstream WSEL |
| P | Flow hydrographs applied at appropriate locations |
| P | Energy grade slope for hydrograph distribution is reasonable |
| P | Internal rating curves appropriate and documented for control structures (if applicable) |

## Review Comments

# 5 – Hydraulic Model Execution

## Methodologies / Data Sources (Modeler to populate)

| Unsteady Plan Names |  |  |  |
| --- | --- | --- | --- |
| 10% ACE (10-year): | 10PAC | 0.2% ACE (500-year): | 0.2PAC |
| 4% ACE (25-year): | 4PAC | 1% plus: | 1PAC_PLUS |
| 2% ACE (50-year): | 2PAC | 1% minus: | 1PAC_MIN |
| 1% ACE (100-year): | 1PAC |  |  |
| Other: |  |  |  |

## Statistical Data (Modeler to populate)

| Time Step: | Controlled by Courant condition. |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Courant Value: | 0.4-1 |  | Simulation Length: | 96 | hours |
| Computation Interval: | 30 | seconds | Model Runtime: | 24-32 | hours |
| Hydrograph Output Interval: | 60 | minutes | Mapping Output Interval: | 60 | minutes |

## Technical Review (Reviewer to confirm with Pass/Fail)

| Execution Review |  |
| --- | --- |
| P | Appropriate 2D parameters are used (Boundary Conditions, Time Step, Simulation Length, etc.) |
| P | Hydrograph Output Interval and Mapping Output Interval set at a reasonable interval to capture peaks |
| P | Model runtime is reasonable (< 12 hours, but dependent upon model size) |

## Review Comments

# 6 – Hydraulic Model Results

## Methodologies / Data Sources (Modeler to populate)

| Flow Validation/Gage Analysis |  |
| --- | --- |
| Software: | HEC-RAS 6.1 |
| Methodology | Validation was conducted by implementing results USGS gages analysis, Regression Equations, FIRMs, and upstream HUC-12 basins (regional weighted-least-squares and PRESS-minimized). |
| Removal of Outliers and Justification |  |
| Validation Notes For calibration, a -25 CN adjustment in the upstream watershed was used. There was no CN adjustment in the lower watershed used in calibration. |  |
| Other: |  |

## Technical Review (Reviewer to confirm with Pass/Fail)

| Computation Checks |  |
| --- | --- |
| P | Model runs with no cell errors |
| P | Volume balance is within tolerance (ΔV < 1% is ideal, however 2% to 3% can be acceptable for rain on mesh) |
| Results Spot Checks Check at all critical structures, major confluences, a handful of headwater tributaries, downstream end, at benchmarks, and a handful of randomly selected locations. |  |
| P | No visible spikes in WSEL visibly present with a single cell |
| P | WSEL and depth grids appear reasonable (no artificial dips or spikes) |
| P | Visual verification flooding is connected downstream for all significant contributing areas (> 64 acres) |
| P | Velocity grids appear reasonable (no unusually high velocity areas or Courant values greater than 2) |
|  | No crossing profiles between all events |
| P | Inflow and outflow hydrograph timing is reasonable as compared to the peak |
| P | Spills into/from adjacent HUCs appropriately accounted for |
| P | WSEL tie-in between model domains and HUCs along major rivers/streams where inflows are applied |
| Results Full Checks |  |
| P | Computational time step meets ideal Courant condition |
| P | Diffusion Wave: V∆T∆X ≤2 (Up to 5) (Results are unit-less) |
| P | Model simulation is long enough to pass the entire hydrograph through the model (minimum of 12 hours past the peak, preferably 24 hours) |
| P | Hydrographs at internal and external boundaries are smooth |
| P | Contributing areas are contained with 2D mesh or accounted for with inflow hydrographs |
| P | No flow trapped at 2D boundary |
|  | Visual verification mapping is reasonable compared to effective |
| Flow Verification |  |
|  | Peak flow gage analysis performed on all gages in the watershed with sufficient data |
|  | All gages used have an appropriate period of record (10 to 20+ years) |
|  | All gages used have no extensive gaps in record which would impact the analysis |
|  | Gage analysis results validated to other sources (Regression, USGS, etc.) |
|  | Results at applicable benchmarks justified (gages, HWMs, effective BFEs) |
|  | Peak discharges comparable to gages, effective/prior studies, regional regression equations |

## Review Comments

- Could not load any mapping in RAS Mapper besides 100yr to verify that there were no crossing profiles.

AECOM Response: The mapping outputs submitted for review are mappable/viewable. It may be possible you need to generate the output map in RAS Mapper, but the plan run HDF files are all submitted with the model and so will allow for this data to be viewed and used. Please let us know if you want us to resubmit the HEC-RAS model – we may want to jump on a call with you to confirm what you are seeing versus what we see on our end before we do resubmit.

OK. Was able to see all mapping with new submittal.

# 7 – Mapping Results

## Technical Review (Reviewer to confirm with Pass/Fail)

| Hydraulic Model |  |
| --- | --- |
| P | HEC-RAS model with results, pertinent data and required storm events |
| Flood Risk Database |  |
| P | All features fully populated per current FEMA guidelines |
|  | Mapping consistent between grids to polygons for all three required mapped events |
| P | 10yr smaller than 100yr smaller than 500yr |
| P | Holes filled in polygon, depth and WSE grids |
|  | Boundary interfaces align matching or identified as AOMI |

## Review Comments

- No mapping at the downstream end of the HUC, also not in the RAS model. If backwater is being used from downstream model, it needs to be included in mapping with Hubbard HUC to go on viewer.

AECOM Response:  This was originally left out intentionally (Gunsolus model), due to issues with the tie-in with the downstream watershed (Lower Clear Fork Brazos) and its backwater.  We have been investigating including this downstream area, re-running the model, and updating all impacted mapping for the final; at request of AECOM PM.  That is the intention at this time, and so those changes will  be included on final submittal. However, we are still seeing tie-in issues/challenges, and so if the final approach ends up that we leave the model boundaries as they are with this initial submittal, we will document the approach and explanation with the BLE report and submit that with final submittal.

OK.

- Reviewer Note: All Mit_Haz, CNMS, and HAZUS were not provided.

AECOM Response: Will be included on final submittal. Per the checklist at top of this document, those were understood to be part of the Completeness Review submittal (QC Item 8 below, as well).

OK.

# 8 – Completeness Review

## Technical Review (Reviewer to confirm with Pass/Fail)

| File Structure and Naming |  |
| --- | --- |
| F | Deliverables provided in TWDB/FEMA required file folder structure and naming convention Please note: Submittal provided in eBFE format to allow for direct submittal to FEMA R6 and USGS and use on the eBFE viewer. Being a TWDB-only funded task order, it is anticipated that submittal to the MIP will remain in eBFE format, per past and similar task orders. |
| F | Data missing (if any) documented in report and ReadMe file within the required folder |
| Report |  |
|  | Technical 2D BLE report documents all assumptions, special considerations, etc. |
|  | Flow validation and difference discussed in report |
|  | Flow comparisons discussed in report |
| Hydrologic Model |  |
| F | HEC-HMS model with results, pertinent data and required storm events |
| Hydraulic Model |  |
| P | HEC-RAS model with results, pertinent data and required storm events |
| Flood Risk Database |  |
| P | All features fully populated per current FEMA guidelines |
| CNMS |  |
| P | Updated correctly with new centerlines |
| p | A1-A5 checks complete |
| P | CNMS Post- Processing Complete |
| HAZUS |  |
| P | Run per FEMA guidance |

## Review Comments

- File Structure and Naming – Under Engineering models, there should be a folder for Hydraulic Models and one for Hydrologic Models. Read me txt can go under Hydrologic models.  Addressed

- File Structure and Naming – Under Hydraulic models please put all inputs and outputs into Input Folder Addressed, but we have updated to match the Region 6 eBFE delivery guidance which calls for Input, Output, Terrain, and Land Cover folders.  We assume this is what was intended by comment.

- File Structure and Naming – Under Gunsolus models, there are several Terrain Files. Please clean to only have final terrain. Addressed.

- S_Pol_Ar – Aesthetic Comment: Using Null values throughout, which is okay. I would keep that consistent. There is one NP value under ANI_FIRM.  The use of the one NP value is to comply with interpretation of the S_Pol_Ar tech reference for this field.  This community is not mapped on any FIRM, so it gets the ANI_TF marked as ‘True’.  In the ANI_FIRM field, because it does not have another FIRM that it is mapped on, there is no FIRM to enter, so the ‘NP’ value is being used.

- Comments provided in report All comments addressed, each copied and responded to below.

- [General comment]: Check spelling throughout report. Seeing a few different versions.  Corrected throughout to correct spelling “Gunsolus”.

- [Figure 6 comment]: Would be helpful to show the gages that are being used in a different color. Adding a halo around the gage numbers would also help with clarity. Hard to read at the moment. Updated Figure 6.

- [Section 2.2.3.1 comment]: Missing part of this sentence? This text should have been deleted, it has been removed for the final.

- [Figure 7 comment]: Verify graph is for Hubbard/Update caption name. Figure title corrected – the figure is applicable to the Hubbard study.

- [Section 2.2.3.5 comment]: Remove. Corrected.

- [Figure 8 comment]:  Recommend adding the 1% +/- as those were also taken from Hubbard and applied to Gunsolus.  Agree, figure updated.

- [Section 2.2.5 comment]:  Update.  Corrected.

- [Section 4.1.2 comment]:  Is this a different HUC? Corrected.

QA/QC Checklist Approval

This 2D Base Level Engineering Analysis QA/QC Review is in compliance with the contract requirements and all task checkpoints are complete.  The independent QC Team has reviewed the analysis, presented review comments to the Production Team Task Leader, and discussed any problems or issues.  The Task Leader, QC Manager, QC Reviewer, and Project Manager have signed the QA/QC Checklist to confirm that all comments have been received, addressed, and documented appropriately.

| QA/QC Team – [TWDB or company name] |  |  |  | Production Team – AECOM |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Seyoum Asamenaw (AECOM) |  | 05/26/2023 |
| QC Manager |  | Date |  | Task Lead |  | Date |
|  |  |  |  | Sean Sutton (AECOM) |  | 05/26/2023 |
| QC Reviewer |  | Date |  | Project Manager |  | Date |