Lower Brazos Watershed, Texas

2D Base Level Engineering

Methods and Results

Contract #HSFE60-15-D-0003

Task Order #70FA6020F00000014

February 2023

Prepared for:

DHS/FEMA (Federal Emergency Management Agency)

FEMA Region VI

FRC 800 North Loop 288

Denton, TX 76209-3698

Submitted by:

Compass PTS JV

3101 Wilson Boulevard, Suite 900

Arlington, VA 22201

Document History

Document LocatIon

J:/FY2020/20-06-0103S

Revision History

| Version Number | Version Date | Summary Changes | Team/Author |
| --- | --- | --- | --- |
| 1.0 | February 2023 | Deliverable to FEMA Region VI | Compass PTS JV |

Approvals

This document requires the approval of the following persons:

| Role | Name | Phone | Title (CLIN/RMC) | Review Date | Approved Date |
| --- | --- | --- | --- | --- | --- |
| Project Manager | Daniel Zell | 512.457.7867 | Project Manager |  |  |

Client distribution

| Name | Title/Organization | Location |
| --- | --- | --- |
| FEMA's MIP | Region VI | See path above |
| Diane Howe | FEMA Region VI Risk MAP Lead | Denton, TX |

Table of Contents

01 Introduction	1

02 2D BLE Modeling Inputs and Controls	3

2.1	Topographic Data	3

2.2	Terrain Data Capture	8

2.2.1	Evaluation for Leverage	8

2.2.2	Source Data	9

2.2.3	Data Processing	11

2.3	Conclusions	12

03 2D BLE Methods	13

3.1	2D BLE	13

3.2	2D Computational Mesh and Settings	14

3.3	Model and Boundary Condition Setup	16

3.3.1	Decoupled Fluvial and Pluvial Scenarios	16

3.3.2	Coastal Downstream Boundary Conditions	18

3.4	Hydrology	20

3.4.1	Rain-on-Mesh (RoM) Precipitation for 2D Computational Mesh	22

3.4.2	1% (+/-) Annual Chance Gridded Precipitation Data	25

3.4.3	Areal Reduction Factor (ARF) Methods	26

3.4.4	Excess Precipitation for 2D Computational Mesh	27

3.4.5	Model and Boundary Condition Setup	28

3.4.6	Outflow Boundary Conditions	29

3.4.7	Inflow Hydrograph Boundary Conditions	29

3.5	Hydraulics	32

3.5.1	Roughness Coefficients	32

3.5.2	Breaklines	33

3.5.3	Refinement Regions	34

04 Floodplain Mapping	36

4.1	Model Outputs	36

4.2	Methodology	36

4.3	BLE Flood Hazard Layer	38

4.4	Additional 1-Percent Features	38

4.5	CNMS Zone A Validation Check A5	38

05 Results	40

5.1	CNMS Validation of Effective Zone A SFHA	40

06 Challenges	45

07 Recommendations	46

08 References	47

Appendix A – Exhibits – Base Level Engineering Results	49

List of Tables

Table 1:  United States Interagency Elevation Inventory Records Intersecting the Project Watershed and Buffer	4

Table 2:  Typical LiDAR-based Data Products and Formats	6

Table 3:  Prioritized Topographic Data Sources	8

Table 4:  Model Area Summary	13

Table 5:  Summary of 2D Computational Settings	15

Table 6:  HEC-RAS Boundary Condition Options	16

Table 7:  Tidal Datums for 8772471, Freeport Harbor TX	19

Table 8: USGS Peak Streamflow Gages with the Lower Brazos HUC-8 Watershed	21

Table 9: USGS Peak Streamflow Results used for this Study	21

Table 10:  Final Modeled ARF Values	27

Table 11:  Land Use Soils-CN Matrix	27

Table 12:  Summary of Boundary Conditions	28

Table 13:  Dimensionless Unit Hydrograph (DUH) Components	29

Table 14:  Fluvial Target Peak Discharges	30

Table 15:  NLCD 2019 Manning's n Roughness Matrix	33

Table 16: Communities with 2D Mesh Refinement	34

Table 17: Zone A Initial Assessment Results	41

Table 18: BLE Comparison Results	41

Table 19: Zone A Validation Results	44

List of Figures

Figure 1:  Lower Brazos Watershed – Initial Model Area Boundaries	2

Figure 2: United State Interagency Elevation Inventory Web Page with Topographic LiDAR layer visible	3

Figure 3: Project Watershed with Buffer (5,000 FT)	4

Figure 4: TNRIS DataHub 1-Meter DEM Availability (February 2020)	7

Figure 5:  HUC-8 Topographic Input Data Sources	9

Figure 6: Lower Brazos Watershed Topography	12

Figure 7:  Lower Brazos Watershed – Final Model Area Boundaries	14

Figure 8:  Pluvial Decoupled Plan Example	17

Figure 9:  Fluvial Decoupled Plan	18

Figure 10:  Tidal Datums for 8772471, Freeport Harbor TX	20

Figure 11:  1% AEP NOAA Atlas 14 Variable Precipitation for the Lower Brazos Watershed	23

Figure 12:  NRCS Nested Distribution Zones – Texas	24

Figure 13:  Spatially Varied Precipitation Example	25

Figure 14:  Inflow Hydrograph from USGS 08111500	30

Figure 15:  Transposed Target Flow Example	31

Figure 16:  Modeled Outflow vs Target Outflow Example	32

Figure 17: V-notch Breakline Approach at Bridge Crossing	34

Figure 18: 2D BLE Model Points for Mapping Workflow	36

Figure 19:  2D BLE Model Points to Water Surface Elevation TIN	37

Figure 20:  Water Surface Elevation TIN to Grid	37

Figure 21:  Water Surface Elevation Grid to Mapping Products	38

Figure 22:  Example A5 Assessment Profile	39

Figure 23:  Ranking of Lower Brazos HUC-12s	43

# Introduction

Recent innovations and efficiencies in floodplain mapping have allowed the U.S. Department of Homeland Security's Federal Emergency Management Agency (FEMA) to develop a process formerly known as First Order Approximation (FOA), now labeled Base Level Engineering (BLE), which can be used to address current program challenges, including the validation of Zone A studies and the availability of flood risk data in the early stages of a Flood Risk Project. The BLE process involves using best available data and automated techniques to produce estimates of flood hazard boundaries for multiple recurrence intervals. The Lower Brazos Watershed BLE documented here was designed to use 2-dimensional (2D) modeling efforts with enhancements and calibration to develop products intended to be transitioned into regulatory data development workflows.

As described in Title 42 of the Code of Federal Regulations, Chapter III, Section 4101(e), once every five years, FEMA must evaluate whether the information on Flood Insurance Rate Maps (FIRMs) reflects the current risks in flood prone areas. FEMA makes this determination of flood hazard data validity by examining flood study attributes and change characteristics, as specified in the Validation Checklist of the Coordinated Needs Management Strategy (CNMS) Technical Reference. The CNMS Validation Checklist provides a series of critical and secondary checks to determine the validity of flood hazard areas studied by detailed methods (e.g., Zone AE, AH, or AO). While the critical and secondary elements in CNMS provide a comprehensive method of evaluating the validity of Zone AE studies, a cost-effective approach for evaluating Zone A studies has been lacking.

In addition to the need for Zone A validation guidance, FEMA standards require flood risk data to be provided in the early stages of a Flood Risk Project. FEMA Program Standard Identification (SID) 29 requires that during Discovery, data must be identified that illustrates potential changes in flood elevation and mapping which may result from the proposed project scope. If available data does not clearly illustrate the likely changes, an analysis is required that estimates the likely changes. This data and any associated analyses should be shared, and results should be discussed with stakeholders.

An important goal of the BLE process is the scalability of the results. Scalability means that the results of a BLE should not only be used for CNMS evaluations of Zone A studies but can also be leveraged throughout the Risk MAP program. The large volume of data resulting from a BLE can be updated as needed and used for the eventual production of regulatory and non-regulatory products, outreach and risk communication, and MT-1 processing. Leveraging this data outside the Risk MAP program may also be valuable to external stakeholders.

To increase and enhance the flood risk products in Texas, FEMA Region VI contracted the Compass PTS JV to perform a BLE analysis for the Lower Brazos Watershed. This report documents the BLE process, products, and results for this watershed. Figure 1 depicts the Lower Brazos Watershed footprint.

Figure 1:  Lower Brazos Watershed – Initial Model Area Boundaries

# 2D BLE Modeling Inputs and Controls

Section 2 presents fundamental components required to execute a 2-dimensional (2D) hydraulic engineering analysis for the Lower Brazos Watershed. Inputs such as elevation data, hydrology from rain on mesh (RoM) and inflow hydrographs, and hydraulic analyses and variables are defined herein. HEC-RAS 6.3 rain-on-mesh utility was applied in the production of these Texas 2D BLE products.

## Topographic Data

Topographic Data Source Analysis

Figure 2: United State Interagency Elevation Inventory Web Page with Topographic LiDAR layer visible

Consulting the INTERAGENCY ELEVATION INVENTORY:

The project team consulted the United States Interagency Elevation Inventory for information pertaining to Light Detection and Ranging (LiDAR) surveys in the vicinity of the project area. This resource was accessed via the URL:

https://www.coast.noaa.gov/inventory. The purpose of the United States Interagency Elevation Inventory is as follows:

"The elevation inventory is designed to be a broadly applicable resource for elevation data information. Users can find out what data sets are available for their area, find more information about a particular data set, or use the information in the viewer to plan a future collection."

Ensuring Appropriate Topographic Coverage:

The Hydrologic Unit Code 8 (HUC-8) Lower Brazos Watershed boundary was buffered by 5,000 feet to ensure that data to be obtained would provide adequate coverage for the hydrologic and hydraulic analysis. See Figure 3.

Figure 3: Project Watershed with Buffer (5,000 FT)

Table 1 below provides a list of the elevation datasets that intersect with the project watershed with a 5,000-foot buffer.

Table 1:  United States Interagency Elevation Inventory Records Intersecting the Project Watershed and Buffer

| YEAR | NAME | AGENCY | TNRIS DOWNLOAD | USGS Download | ATLAS Download | NRCS Download |
| --- | --- | --- | --- | --- | --- | --- |
| 2019 | Missouri City LiDAR | TNRIS | https://data.tnris.org/collection/66d886a6-13b9-46a9-936a-bf2ce4f71aaf | N/A | N/A |  |
| 2018 | Upper Coast LiDAR | TNRIS | https://data.tnris.org/collection/b5bd2b96-8ba5-4dc6-ba88-d88133eb6643 | N/A | N/A | N/A |
| 2018 | Eastern Texas LiDAR | USGS | https://data.tnris.org/collection/13563a34-6a6d-4171-ad34-fbdfb26165ae | N/A | N/A |  |
| 2017 | Brazos, Freestone, Robertson Counties LiDAR | USGS | https://data.tnris.org/collection/b6ea8e3a-c8b7-4d97-b4d1-4eb8172eb87d | N/A | N/A |  |
| 2017 | Central Texas LiDAR | TNRIS | https://data.tnris.org/collection/0549d3ba-3f72-4710-b26c-28c65df9c70d | N/A | N/A | N/A |
| 2014 | Fort Bend County LiDAR | TNRIS | https://data.tnris.org/collection/21130f88-dac2-4c36-9d9b-1fd9dc07750e | N/A | N/A | N/A |
| 2011 | Austin, Grimes, Walker Counties LiDAR | TNRIS | https://data.tnris.org/collection/d69e74b0-9c20-4a7d-9b3d-f251eb318bfe | N/A | N/A | N/A |
| 2008 | Bastrop, Fayette, & Hays Counties LiDAR | TNRIS | https://data.tnris.org/collection/1f1d417a-26f2-41d5-9787-af392d0f09cf | N/A | N/A | N/A |
| 2008 | Houston-Galveston Area Council LiDAR | TNRIS | https://data.tnris.org/collection/4e306d96-e77f-4794-b818-eba5c85140de | N/A | N/A | N/A |
| 2006 | Coastal LiDAR | TNRIS | https://data.tnris.org/collection/8ea19b45-7a66-4e95-9833-f9e89611d106 | N/A | N/A | N/A |

Physical Access to LiDAR Survey Digital Data:

A variety of data products can result from a LiDAR survey as shown in Table 2. Data products may include various data depending on what the commissioning entity orders. The following is a general list of common LiDAR-based products with typical data formats:

Table 2:  Typical LiDAR-based Data Products and Formats

| LiDAR-based Data Product Type | Typical Formats |
| --- | --- |
| Tiled Polygons (Delivery Matrix) | SHP/.PGDB/.fGDB/.DXF/.DGN |
| Raw Point Cloud Data | LAS, ASCII |
| Classified Point Cloud Data | LAS, ASCII |
| Breaklines | SHP/.PGDB/.fGDB/.DXF |
| Bare Earth Digital Elevation Model (DEM) | Esri grid/GeoTIFF/ASCII grid |
| Contours | SHP/.PGDB/.fGDB/.DXF/.DGN |
| Triangulated-Irregular Network (TIN) | Esri ArcGIS / other 3D Mesh |
| Hydrologically Corrected DEM (HDEM) | Esri grid/GeoTIFF/ASCII grid |

The table of common LiDAR-based products is presented to draw specific attention to the manner in which the project team accessed certain LiDAR products. Notably, where Bare Earth Digital Elevation Models (DEMs) were available, the project team did not need to utilize other data product types. Consequently, the project team specifically sought and ultimately utilized only Bare Earth DEMs for all the various LiDAR projects covering the scope of work. While other LiDAR-based data products were available, ultimately, they were not needed because DEM's already existed.

Data that were available via the Texas Natural Resources Information System (TNRIS) Datahub (https://data.tnris.org/) were searched according to coverage (see Figure 4).

Figure 4: TNRIS DataHub 1-Meter DEM Availability (February 2020)

TNRIS DEM's were selected based on the geographic areas needed to cover the Lower Brazos Watershed (plus the 5,000-foot buffer) and downloaded to include the following areas:

- 2019 Missouri City LiDAR;

- 2018 Upper Coast LiDAR;

- 2018 Eastern Texas LiDAR;

- 2017 Brazos, Freestone, Robertson Counties LiDAR;

- 2017 Central Texas LiDAR;

- 2014 Fort Bend County LiDAR;

- 2011 Austin, Grimes, Walker Counties LiDAR;

- 2008 Bastrop, Fayette, & Hays Counties LiDAR;

- 2008 Houston – Galveston Area Council LiDAR; and

- 2006 Coastal LiDAR.

## Terrain Data Capture

### Evaluation for Leverage

The initial step in developing topographic data for the project watershed included gathering data; much of what was demonstrated throughout the previous Task Summary (Section 2.1). Six datasets were identified to be leveraged for the Lower Brazos HUC-8 Watershed. These datasets were assessed based on accuracy, age of acquisition, and coverage area. Table 3 lists the prioritization of these datasets while Figure 5 shows the location.

Table 3:  Prioritized Topographic Data Sources

| Priority | Source | Collection Date | Vertical Accuracy | Horizontal Accuracy | Point Spacing | Vertical Datum | Horizontal Datum |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2019 Missouri City LiDAR 1 Meter DEM | April 15, 2019 | 0.033 m | 0.008 m | 0.5 m | NAVD88 | NAD83 |
| 2 | 2018 Upper Coast LiDAR 1 Meter DEM | January 13, 2018 - March 22, 2018 | 0.04 m | 0.265 m | 0.5 m | NAVD88 | NAD83 |
| 3 | 2018 Eastern Texas LiDAR 1 Meter DEM | March 6, 2018 – March 21 2018 | 0.05 m | Not Provided | 0.63 m | NAVD88 | NAD83 |
| 4 | 2017 Central Texas LiDAR 1 Meter DEM | January 28, 2017 - March 22, 2017 | 5.496 cm | 0.189 m | 0.5 m | NAVD88 | NAD83 |
| 5 | 2017 Brazos, Freestone, Robertson Counties LiDAR 1 Meter DEM | January 7, 2017 - March 1, 2017 | 14.6 cm | 41 cm | 0.63 m | NAVD88 | NAD83 |
| 6 | 2014 Fort Bend County LiDAR 1 Meter DEM | February 12-27, 2014 | 12 cm | 50 cm | 0.25 m | NAVD88 | NAD83 |

- 2019 Missouri City LiDAR 1 Meter DEM

- 2018 Upper Coast LiDAR 1 Meter DEM

- 2018 Eastern Texas LiDAR 1 Meter DEM

- 2017 Central Texas LiDAR 1 Meter DEM

- 2017 Brazos, Freestone, Robertson Counties LiDAR 1 Meter DEM

- 2014 Fort Bend County LiDAR 1 Meter DEM

Figure 5:  HUC-8 Topographic Input Data Sources

### Source Data

#### 2019 Missouri City 1 Meter DEM

This Strategic Mapping (StratMap) Program LiDAR project covers a portion of Fort Bend County around Missouri City, Texas. This project was managed by StratMap and utilized the StratMap contracts held by the Department of Information Resources (DIR). Collection took place April 15, 2019. Data was collected and processed by the Sanborn Mapping Company, LLC with third party quality assurance/quality control performed by AECOM.

The Fundamental Vertical Accuracy (FVA) of the DEM tested at a Root Mean Square of the Vertical Error (RMSEz) of 0.033 meters at the 95% confidence level and 0.196 meters at the 95% confidence level in open terrain. Horizontal accuracy was measured at 0.008 meters.

#### 2018 Upper Coast LiDAR 1 Meter DEM

This StratMap LiDAR collection covers portions of the Texas upper coastal counties. Data acquisition and processing was performed by Fugro USA Land with third-party QA/QC being performed by AECOM. Data was collected January 13, 2018, through March 22, 2018. The project area of interest covers areas where previous LiDAR data was over a decade old. This project was managed by StratMap and utilized the StratMap contracts held by the DIR. Collection took place during the leaf-off season in Texas.

The FVA of the DEM tested at an RMSEz of 0.04 meters at the 95% confidence level and 9.8 centimeters at the 95% confidence level in open terrain. Horizontal accuracy was measured at 0.265 meters.

#### 2018 Eastern Texas Lidar 1 Meter DEM

This United States Geological Survey (USGS) LiDAR collection covers portions of 11 counties in the eastern region of Texas. Data was collected and processed by Digital Aerial Solutions, LLC. Data acquisition occurred March 6, 2018, through March 21, 2018. This project was initiated by the USGS and Natural Resources Conservation Services (NRCS) for conservation planning, design, research, floodplain mapping, wetlands identification, dam safety assessments, hydrologic modeling, and other environmental purposes.

The FVA of the DEM tested at an RMSEz of 0.05 meters at the 95% confidence level and 0.098 meters at the 95% confidence level in open terrain. Horizontal accuracy was not provided.

#### 2017 Central Texas LiDAR 1 Meter DEM

This project was managed by StratMap and utilized the StratMap contracts held by the DIR. Collection took place January 28, 2017, through March 22, 2017. Data was collected and processed by Fugro EarthData, Inc. with third party quality assurance/quality control performed by AECOM.

The FVA of the DEM tested at an RMSEz of 5.496 centimeters at the 95% confidence level and 10.7 centimeters at the 95% confidence level in open terrain. Horizontal accuracy was measured at 0.189 meters.

#### 2017 Brazos, Freestone, Robertson Counties LiDAR 1 Meter DEM

This USGS LiDAR collection covers portions of the Lower Brazos Basin across 12 Texas counties including Brazos, McLennan, Washington, Falls, Grimes, Navarro, Milam, Freestone, Limestone, Hill, Robertson, and Burleson. Data was collected and processed by Dewberry & Digital Aerial Solutions (DAS). Data acquisition occurred January 7, 2017, through March 1, 2017; approximately 5,445 square miles were covered. The project was funded by FEMA to assist with hydrologic studies and hydraulic analysis to determine the risk of flood hazards.

The FVA of the DEM tested at an RMSEz of 14.6 centimeters at the 95% confidence level: and 24.5 centimeters at the 95% confidence level in open terrain. Horizontal accuracy was measured at 41 centimeters.

#### 2014 Fort Bend County LiDAR 1 Meter DEM

This StratMap LiDAR project encompasses Fort Bend County, Texas. Fugro EarthData, Inc. captured the data from February 12-27, 2014. The data products became part of ongoing data collection by the state of Texas to support mapping and modeling of floodplains, hydrography, and terrain. This project was procured through the Council on Competitive Government's High Priority Imagery and Datasets (HPIDS) contract.

The FVA of the DEM tested at an RMSEz of 12 centimeters at the 95% confidence level and 24 centimeters at the 95% confidence level in open terrain. Horizontal accuracy was measured at 50 centimeters.

### Data Processing

The Watershed Information System (WISE) software platform was utilized to create a digital surface model for the project watershed. The following prioritization was given to the source datasets:

- 2019 Missouri City LiDAR 1 Meter DEM

- 2018 Upper Coast LiDAR 1 Meter DEM

- 2018 Eastern Texas LiDAR 1 Meter DEM

- 2017 Central Texas LiDAR 1 Meter DEM

- 2017 Brazos, Freestone, Robertson Counties LiDAR 1 Meter DEM

- 2014 Fort Bend County 1 Meter DEM

The DEMs created from the LiDAR datasets described in Section 2.2 and were compiled in priority order of collection date and vertical accuracy into a mosaic dataset using ArcMap. From this mosaic, a tile index was created for the project area, the mosaic was reprojected into the respective State Plane Zone for the project watershed (Texas South Central) and resampled to 10-foot resolution. The mosaic was clipped into 40,000-foot tiles, converted to ASCII text format, and imported into WISE Terrain Analyst (WTA). Visual inspection of the 10-foot DEMs was performed to ensure no voids and/or artifacts were present. The seamless DEM is shown in Figure 6.

Stream centerlines were generated from ArcHydro processes and manually edited to match the 10-foot DEMs as a source for horizontal alignment and vertical elevation. These stream centerlines are created for use in the hydraulic analysis.

In addition to the quantitative assessment of the source digital terrain, a qualitative visual inspection of the composite DEM was performed using a hillshade derived from the 10-foot DEM. The visual inspection indicated that no unusual or non-terrestrial features were observed in the composite DEM assuring the surface files are sufficient for hydraulics analyses and floodplain mapping activities.

All existing topographic data previously described includes LiDAR survey projects. Compass obtained each of the appropriate topographic data for the floodplain areas to be studied, including overbank areas and an adequate buffer beyond the HUC-8 watershed boundary. These data were processed and assembled for all topographic needs; namely, hydraulic analysis, floodplain boundary delineation, and/or testing of floodplain boundary standard compliance. Compass gathered and further assessed availability, currency, and accuracy information to meet FEMA Standards enumerated in FEMA Standards for Flood Risk Analysis and Mapping (FP 204-078-1 [Rev 8]); the data obtained and produced are documented per FEMA guidelines to include this detailed narrative.

All LiDAR data in DEM format were evaluated and subsequently leveraged based on the current FEMA requirements for the flood hazard study level of detail.

Compass updated the topographic maps and/or DEMs for the subject flooding sources using the existing data collected under this Topographic Data Development process. In addition, Compass addressed all concerns or questions regarding topographic data development that were identified during independent QC review. Compass adhered to the Program and Working standards effective as of the date of the proposal.

Figure 6: Lower Brazos Watershed Topography

## Conclusions

In conclusion, the best topographic data available for the Lower Brazos (TX) Watershed study were:

- 2019 Missouri City LiDAR 1 Meter DEM

- 2018 Upper Coast LiDAR 1 Meter DEM

- 2018 Eastern Texas LiDAR 1 Meter DEM

- 2017 Central Texas LiDAR 1 Meter DEM

- 2017 Brazos, Freestone, Robertson Counties LiDAR 1 Meter DEM

- 2014 Fort Bend County LiDAR 1 Meter DEM

The DEM data were leveraged and mosaicked to create the final terrain supporting hydraulic analyses, floodplain mapping, and non-regulatory product development. Anomalous data that were identified were deemed to not cause significant issues to engineering analyses. The terrain model developed for the Lower Brazos HUC-8 Watershed study represents the best available surface to support the scoped floodplain mapping activities.

# 2D BLE Methods

The following sections describe the 2D computational mesh and program setting considerations, followed by a discussion and tabulation of hydrologic and hydraulic engineering methods and model inputs.

## 2D BLE

The HEC-RAS version 6.3 RoM utility was applied to the Lower Brazos BLE products workflow. The following sections describe the methods and model inputs used.

The Lower Brazos HUC-8 Watershed covers an area of 1,642 square miles. To accurately model the area and provide a sufficient level of hydraulic and hydrologic detail, the watershed was divided into three individual 2D model areas. Model delineation started with a HUC-8 sub-basin, which was then subdivided trying to retain key fluvial validation checkpoints at USGS gages and then buffered sufficiently to facilitate an appropriate water surface elevation (WSEL) tie-in. Table 4 lists the specific model area sizes while Figure 7 displays the final individual model areas used to cover the project footprint and facilitate proper WSEL tie-ins.

Table 4:  Model Area Summary

| Model Area | Area (mi2) | No. of Cells |
| --- | --- | --- |
| 1 | 876.7 | 1,107,824 |
| 2 | 678.2 | 1,694,513 |
| 3 | 325.2 | 719,000 |

Figure 7:  Lower Brazos Watershed – Final Model Area Boundaries

## 2D Computational Mesh and Settings

The RAS 6.3 2D computational mesh was created for each model area using RAS Mapper and supplemented with ArcGIS toolsets, which reduced the need for manual edits to mesh cells within RAS 6.3 that result in error generation. Each individual model's 2D mesh nominal cell size is 200 feet, with additional refinement to include 100-foot cells within scoped stream areas and 50-foot cells within community boundaries.

The RAS 6.3 model was run with the Diffusion Wave (simplified Full Momentum) equations. An adaptive 30-second time step was used to allow the model to lower the time step, as necessary, to keep the maximum courant number under 1.5 for the duration of the simulation. The time step was allowed to reach 120 seconds for a maximum and 7.50 seconds for the minimum. Table 5 shows a summary of the methodology and settings used for this 2D BLE study.

Table 5:  Summary of 2D Computational Settings

| Model Settings | BLE Methodology Used |
| --- | --- |
| Equation | Diffusion Wave Equation (DWE) - Simplified Full Momentum |
| Initial Conditions | Start Completely Dry |
| Initial Time Step | 30 Seconds |
| Adaptive Time Step | Range from 7.50 to 120 seconds based on Maximum Courant number of 1.5 |
| Nominal Mesh | - 200-FT x 200-FT cell size - 4-sided (nominal) |
| Mesh Refinement | - 1-square mile stream corridor, 100-FT Cell Size - Community Boundaries, 50-FT cell size - Additional refinement along transportation features and dams (see section 3.5.2) |

The HEC-RAS 2D computational mesh defines the extents of the 2D flow and can affect the accuracy of the 2D calculations. A denser mesh may provide more accurate results, but it can dramatically increase computation times. The 2D mesh for the model was set as evenly spaced cells at 200 feet. The mesh was further refined by placing breaklines along roads, berms, ridges, and other high ground that can influence the flow.

The Lower Brazos Watershed was split into three model areas as shown in Figure 7. This was initially decided to limit the number potential tie-in issues from one model area to the next to create a more efficient mapping workflow. In addition, each model area was buffered to include an overlap for mapping tie-in. The 2D mesh was generated for each model area in RAS 6.3 using inputs from ArcGIS toolsets. At the coastal boundary in Model Area 3, a theta value of 0.6 was used to represent tidal influences.

## Model and Boundary Condition Setup

RAS 6.3 2D modeling has many options to define boundary and initial conditions. These are listed in Table 6 below:

Table 6:  HEC-RAS Boundary Condition Options

| Boundary Condition | Type | Used in this Study? |
| --- | --- | --- |
| External | Flow Hydrograph | Yes |
|  | Stage Hydrograph | Yes |
|  | Normal Depth | Yes |
|  | Rating Curve | No |
| Internal | Flow Hydrograph | Yes |
|  | Precipitation | No |
| Global | Precipitation | Yes |
|  | Evapotranspiration | No |
|  | Wind | Yes |

### Decoupled Fluvial and Pluvial Scenarios

To promote model efficiencies and the accuracy of the 2D BLE model outputs compared to best available hydrologic data sources, the decoupled fluvial and pluvial approach was utilized. The decoupled approach uses separated modeling plans for the pluvial (rain driven) condition and fluvial (riverine) flooding condition. In the example shown in Figure 8 (pluvial conditions) and Figure 9 (fluvial conditions) there is a single inflow location, but multiple inflow locations can be modeled with this same procedure.

Figure 8:  Pluvial Decoupled Plan Example

Figure 9:  Fluvial Decoupled Plan

The pluvial plan follows the same general approach as in headwater areas – the only water added to the model is in the form of precipitation applied to the 2D mesh. A zero-value inflow boundary condition hydrograph where the upstream basin is entering the watershed must be created so that the same model mesh can be used for the fluvial plan.

The fluvial plan is analogous to a 1D model where the flows at key points are determined and used as inputs to the hydraulic model. Therefore, decoupled fluvial plans start with a peak inflow and a hydrograph from an appropriate source, such as a USGS stream gage within the watershed, regional regression equations, or a stream gage upstream or downstream of the watershed. Peak flows at the outlet must also be calculated.  In a BLE project of several model areas, the peak value for an area's outflow will be the same as the peak value for the downstream area's inflow even though the hydrograph from the upstream model is not used as a direct input.

The procedures for how this was implemented in the Lower Brazos Watershed models can be found in Section 3.4 of this report.

### Coastal Downstream Boundary Conditions

Instances when the downstream boundary condition of a model area is within a coastal tidal reach, the tidal boundary of the model is taken as equal to the Mean Higher High Water (MHHW) level of the nearby tidal station. This can be done using a stage hydrograph (if available) from a nearby tidal gage or by using a known water-surface elevation.

A stage hydrograph was used for boundary conditions along the coast with water elevation determined by the MHHW) of the nearby tide station, accessed from National Oceanic and Atmospheric Administration (NOAA) tidal data (NOAA, 2022). This study referenced Freeport Harbor (8772471), which has a MHHW of 1.02 feet (NAVD 88). The data obtained from this station is shown in Table 7 and Figure 10 below.

(https://tidesandcurrents.noaa.gov/datums.html?datum=NAVD88&units=0&epoch=0&id=8772471&name=Freeport+Harbor&state=TX)

Table 7:  Tidal Datums for 8772471, Freeport Harbor TX

| Datum | Value | Description |
| --- | --- | --- |
| MHHW | 1.02 | Mean Higher-High Water |
| MHW | 0.88 | Mean High Water |
| MTL | 0.26 | Mean Tide Level |
| MSL | 0.26 | Mean Sea Level |
| DTL | 0.18 | Mean Diurnal Tide Level |
| MLW | -0.37 | Mean Low Water |
| MLLW | -0.65 | Mean Lower-Low Water |
| NAVD88 | 0 | North American Vertical Datum of 1988 |
| STND | -28 | Station Datum |
| GT | 1.67 | Great Diurnal Range |
| MN | 1.25 | Mean Range of Tide |
| DHQ | 0.14 | Mean Diurnal High Water Inequality |
| DLQ | 0.28 | Mean Diurnal Low Water Inequality |
| HWI |  | Greenwich High Water Interval (in hours) |
| LWI |  | Greenwich Low Water Interval (in hours) |
| Max Tide | 5.47 | Highest Observed Tide |
| Max Tide Date & Time | 9/14/2021 9:24 | Highest Observed Tide Date & Time |
| Min Tide | -2.97 | Lowest Observed Tide |
| Min Tide Date & Time | 12/21/2018 13:42 | Lowest Observed Tide Date & Time |
| HAT | 1.98 | Highest Astronomical Tide |
| HAT Date & Time | 10/27/2007 23:36 | HAT Date and Time |
| LAT | -2.07 | Lowest Astronomical Tide |
| LAT Date & Time | 1/10/2005 14:24 | LAT Date and Time |
| Station | 8772471, Freeport Harbor, TX |  |
| Status | Accepted (Mar 24, 2021) |  |
| Units | Feet |  |
| Control Station | 8771450 Galveston Pier 21, TX |  |
| T.M. | 0 |  |
| Epoch | 1983-2001 |  |
| Datum | NAVD 88 |  |

Figure 10:  Tidal Datums for 8772471, Freeport Harbor TX

## Hydrology

Precipitation frequency data (NOAA, 2018) was used for this study. This data was obtained from NOAA's Precipitation Frequency Data Server (PFDS) (http://hdsc.nws.noaa.gov/hdsc/pfds/index.html). The 10-, 4-, 2-, 1-, and 0.2-percent annual chance 24-hour precipitation depths were applied spatially in the 2D model using NOAA gridded PFDS data. A regionally appropriate areal reduction factor was applied to the rainfall based on the methodology described in Section 3.4.2 below.

USGS streamflow gage data were leveraged to estimate peak flows for downstream gages within each model area for calibration purposes. Models are compared to known USGS peak flows to ensure accurate results. USGS stream gages with less than 10 years of record were not considered for this study. Present conditions were also not considered for this study. The gages used for analysis in this study are listed in Table 8.

Table 8: USGS Peak Streamflow Gages with the Lower Brazos HUC-8 Watershed

| Gage ID | Flooding Source and Location | Published Drainage Area (mi2) | Period of Record |
| --- | --- | --- | --- |
| 08111500 | Brazos River near Hempstead, TX | 43,880 | 1899-2020 |
| 08115000 | Big Creek near Needville, TX | 42.8 | 1947-2020 |
| 08116400 | Dry Creek nr Rosenberg, TX | 8.65 | 1959-2020 |
| 08116650 | Brazos River near Rosharon, TX | 45,339 | 1913-2020 |
| 08114000 | Brazos River at Richmond, TX | 45,107 | 1884-2020 |
| 08111700 | Mill Creek near Bellville, TX | 376 | 1940-2020 |

If more than 10 years of continuous gage data were available, 1% peak discharge (Q) and WSEL in the model were compared against the 1% gage data. The 1% peak discharge gage data along with upper and lower bound values are presented in Table 9. The gage analysis method for this project follows Scientific Investigations Report 2009-5087 (SIR2009): Regression Equations for Estimation of Annual Peak-Streamflow Frequency for Undeveloped Watersheds in Texas Using an L-Moment-Based, Press-Minimized, Residual-Adjusted Approach. Inputs included a station skew on the main river while weighted skew was applied to gage analysis on small tributaries. In this case, Table 3 in the Water-Resources Investigations Report 96-4117 was used as a source for the skew and Mean Square Error (MSE) (USGS, 1996). The gage data uses the L-moment method to validate 1% flows and to estimate the upper/lower bound of the flows using the standard deviation.

The 1% WSEL for each gage was estimated using USGS rating curves, along with the upper and lower bound limits. If the rating curve did not reach peak discharge, it was extrapolated using either the power or the fitted equation. The final WSEL value was selected using the lowest coefficient of determination (R2) value for that particular rating curve.

The results of the stream discharge analysis, along with the +/- standard errors of estimate, are listed in Table 9.

Table 9: USGS Peak Streamflow Results used for this Study

| Gage ID | Flooding Source and Location | 1% Peak Q (cfs) | 1%- Standard Error Peak Q | 1%+ Standard Error Peak Q |
| --- | --- | --- | --- | --- |
| 08111500 | Brazos River near Hempstead, TX | 158,000 | 140,900 | 180,100 |
| 08114500 | Brazos River near Juliff, TX | 117,000 | 63,180 | 170,820 |
| 08116650 | Brazos River near Rosharon, TX | 112,800 | 99,380 | 130,600 |
| 08114000 | Brazos River at Richmond, TX | 133,700 | 122,100 | 147,800 |
| 08111700 | Mill Creek near Bellville, TX | 67,960 | 56,770 | 84,360 |

### Rain-on-Mesh (RoM) Precipitation for 2D Computational Mesh

HEC-RAS version 6.3 was used to apply a spatially varied 24-hour design storm hyetograph input to the pluvial model plans for each exceedance probability. The spatially varied 24-hour precipitation grids from NOAA Atlas 14 (NOAA, 2018) for the state of Texas were spliced into incremental 6-minute grids to apply a set of time-series grids for the 24-hour period following the specified rainfall distribution. These 6-minute splices were then combined using HEC-DSS Vue version 3.2.3 to create a gridded dataset allowing for incremental spatial rainfall input to the HEC-RAS model.

A regionally appropriate areal reduction factor was applied to each model area’s pluvial plans globally which was then applied to the gridded rainfall input. The areal rainfall reduction is discussed more in Section 3.4.2 of this report.

Figure 11:  1% Annual Exceedance Probability (AEP) NOAA Atlas 14 Variable Precipitation for the Lower Brazos Watershed

The NRCS Nested Distribution developed for the State of Texas Region A (Figure 12) was used for defining storm distributions of the spatially varied rainfall input. The nested distribution approach allows for smaller duration rainfall events to be nested within the greater 24-hour duration storm event for every recurrent interval. This is important to represent hydrology in smaller sub-basins within each model, where the controlling duration is often shorter and more intense than larger basins.

Figure 12:  NRCS Nested Distribution Zones – Texas

Figure 13 below shows an example spatially varied precipitation depths after the application of the areal reduction factor at the scalar (cell) level. These precipitation depths represent the total accumulated rainfall in the RAS model during the 24-hour design storm period at that physical location.

Figure 13:  Spatially Varied Precipitation Example

### 1% (+/-) Annual Chance Gridded Precipitation Data

In addition to recurrence interval precipitation estimates, NOAA Atlas 14 provides 90% confidence intervals of reported precipitation values.

For a 2D watershed model, the 1-percent-plus (upper 84-percent confidence limit) is to be based on the precipitation data since the flows are implicitly calculated in the model at every cell. The recommended approach to prepare the 1-percent-plus precipitation totals using available data from NOAA is described below. This process assumes the log of the precipitation uncertainty is normally distributed and uses the quantile function to transform the 90% confidence intervals into the 1-percent plus and minus precipitation values.

Given precipitation values (lower 90%, median, and upper 90%) from NOAA

Calculate µ (natural log of medial rainfall)

- μ=ln⁡Precipmedian

- Calculate σmax themaxnaturallogof the standard deviation of data

- σlower=μ-lnPreciplower 90%1.645

- σupper=lnPrecipupper 90%-µ1.645

- σmax =max⁡(σlower, σupper)

- Calculate the 1-percent plus and minus precipitation values:

- Precip1% Plus=expμ+ σmax* 2* erfinv2* 0.84-1

- =exp⁡μ+σmax* 0.994

- Precip1% Minus=expμ+ σmax* 2*erf2*0.16-1

- =expμ+σmax* -0.994

- *Note:  erfinv is the inverse error function also denoted as erf-1

### Areal Reduction Factor (ARF) Methods

An areal reduction function (ARF) is a description of the relationship between the point maximum observed depth of precipitation in a storm pattern and the average depth of precipitation over a larger area. Generally, ARFs decrease as storm area increase.

ARF= Average Precipitation within the AreaMaximum Precipiation within the Area

The most common ARF method is one developed by the National Weather Service (formerly known as the Weather Bureau). In practice, areal reduction methods are applied to the depth value derived from those relations. Per FEMA Guidance, the selected ARF methodology must be documented. Area reduction methods should come from NOAA Atlases, Regional Climate Centers, or State and Local Agencies responsible for flood control. For the Lower Brazos Watershed 2D BLE study, three ARFs were explored:

TP-29 developed by the National Weather Service, Miller, Frederic, & Tracey 1973

Areal-Reduction Factors for Precipitation of the 1-Day Design Storm in Texas, WRIR 99-4267

Calculation of Areal Reduction Factors Using NEXRAS Precipitation Estimates, Report 0-4642-3, Texas Transportation Institute and the Texas A&M University System, October 2005

- All three methods provide different ARFs. The methods outlined in WRIR 99-4267, and Report 0-4267 offer a more regionally site-specific methodology and were used as initial starting values for the pluvial runs. The final ARF applied to all events for each modeled area resulted from numerous iterations while trying to find a best fit match with the best available hydrologic data sources available. A summary of the final modeled ARF values is provided in Table 10.

Table 10:  Final Modeled ARF Values

| Model Area | Area (mi2) | Final ARF |
| --- | --- | --- |
| 1 | 876.7 | 0.5089716 |
| 2 | 678.2 | 0.4224236 |
| 3 | 325.2 | 0.5711097 |

### Excess Precipitation for 2D Computational Mesh

Excess precipitation and infiltration losses are determined in HEC-RAS 6.3 using the NRCS Curve Number (CN) Loss Method. HEC-RAS calculates a separate curve number for each 2D cell based on a combination of land use and soils datasets. This will vary the excess precipitation throughout the 2D model domain to represent varied geography within the larger model area. Initial curve numbers were computed by intersecting the National Land Cover Dataset (NLCD) 2019 and NRCS soils data based on the matrix presented in Table 11.

| Table 11: Land Use Soils-CN Matrix |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Land Use Grid Code | NLCD Description | Hydrologic Soil Group |  |  |  |  |  |  |
|  |  | A | B | C | D | AD | BC | CD |
| 11 | Open Water | 99 | 99 | 99 | 99 | 99 | 99 | 99 |
| 12 | Perennial Ice/Snow | 99 | 99 | 99 | 99 | 99 | 99 | 99 |
| 21 | Developed Open Space | 49 | 69 | 79 | 84 | 84 | 84 | 84 |
| 22 | Developed Low Intensity | 61 | 75 | 83 | 87 | 87 | 87 | 87 |
| 23 | Developed Medium Intensity | 81 | 88 | 91 | 93 | 93 | 93 | 93 |
| 24 | Developed High Intensity | 89 | 92 | 94 | 95 | 95 | 95 | 95 |
| 31 | Barren Land | 49 | 69 | 79 | 84 | 84 | 84 | 84 |
| 41 | Deciduous Forest | 30 | 48 | 57 | 63 | 63 | 63 | 63 |
| 42 | Evergreen Forest | 35 | 58 | 73 | 80 | 80 | 80 | 80 |
| 43 | Mixed Forest | 36 | 60 | 73 | 79 | 79 | 79 | 79 |
| 52 | Shrub Scrub | 35 | 56 | 70 | 77 | 77 | 77 | 77 |
| 71 | Herbaceous | 55 | 71 | 81 | 89 | 89 | 89 | 89 |
| 81 | Hay Pasture | 49 | 69 | 79 | 84 | 84 | 84 | 84 |
| 82 | Cultivated Crops | 63 | 73 | 80 | 83 | 83 | 83 | 83 |
| 90 | Woody Wetlands | 72 | 80 | 87 | 93 | 93 | 93 | 93 |
| 95 | Emergent Herbaceous Wetlands | 72 | 80 | 87 | 93 | 93 | 93 | 93 |

### Model and Boundary Condition Setup

Using RAS 6.3 RoM modeling requires a 2D computational mesh boundary and often requires defining inflow boundary conditions and application of RoM precipitation. A spatially-varied precipitation input was applied to each model 2D mesh as a meteorological variable. Models that have contributing drainage from upstream areas used inflow hydrograph boundary conditions. These inflow hydrographs are the results of the upstream 2D model to ensure continuity between models. The process to determine the excess precipitation to be applied is outlined in Section 3.4.4 of this report. The HEC-RAS models include a 1,000-foot buffer around each model boundary with further buffer distances at flow transition locations to increase the overlap between models. Including model boundary buffers helps to improve hydraulic routing and model tie-ins.

Table 12:  Summary of Boundary Conditions

| Hydrologic Feature | Boundary Condition Type | Note |
| --- | --- | --- |
| Riverine Inflow | Flow Hydrograph | Developed from USGS gage data |
| Riverine Outflow | Normal Depth | Energy grade slope estimated from terrain |
| Rain-on-Mesh (RoM) Precipitation Input | Meteorological Data Precipitation | Spatially varied DSS gridded dataset Developed from NOAA Atlas 14 |
| Precipitation Overflow | Normal Depth | Allows for rainfall in buffer area to exit model |
| Streamflow Loss (attenuation) | Internal Flow Hydrograph |  |

### Outflow Boundary Conditions

Outflow boundary conditions (from the computational 2D mesh) were used along model boundaries without an inflow boundary condition. Unique outflow boundaries were established for obvious riverine outflows, while the remaining boundaries were defined as continuous boundaries to allow drainage to adjacent basins to leave the model area freely. Normal depth was used for all non-unique outflow boundary conditions using approximate energy grade-line slopes estimated from the LiDAR terrain data.

### Inflow Hydrograph Boundary Conditions

For the Lower Brazos Watershed fluvial modeling, an analogy can be made with 1D modeling where the flows at key points are determined and used as inputs to the hydraulic model. Therefore, the decoupled fluvial plans start with a peak inflow and a hydrograph from an appropriate source. For this study the inflow hydrograph source is the USGS Gage 08111500 Brazos River near Hempstead, TX.

The inflow hydrograph at USGS Gage 08111500 was derived using the dimensionless unit hydrograph methodology. This method requires three essential components to create the shape and magnitude (outlined in Table 13). This inflow hydrograph serves as the basis for capturing all contributing drainage area upstream of Model Area 1.

Table 13:  Dimensionless Unit Hydrograph (DUH) Components

| Component | Source |
| --- | --- |
| Peak Discharge (QP) | Peak discharges calculated from a recent Bulletin 17C Expected Moments Algorithm (EMA) gage analysis post Hurricane Harvey (Table 14) |
| Time to Peak (TP), or Basin Lag Time (TL) | Average of observed time to peak values 4.5 days (108 hours) |
| Dimensionless Unit Hydrograph (DUH) | DUH created from 4 observed events then averaged - 3/6/20164/17/2016 - 8/23/20179/27/2017 - 3/26/20184/21/2018 - 8/31/201010/12/2010 |

Table 14:  Fluvial Target Peak Discharges

| Gage ID | Flooding Source and Location | 10% Annual Chance | 4% Annual Chance | 2% Annual Chance | 1% Annual Chance | 0.2% Annual Chance | 1% Plus | 1% Minus |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 08111500 | Brazos River near Hempstead, TX | 101,300 | 125,500 | 142,300 | 158,000 | 191,000 | 180,100 | 140,900 |
| 08114000 | Brazos River at Richmond, TX | 96,550 | 113,700 | 124,400 | 133,700 | 150,500 | 147,800 | 122,100 |
| 08116650 | Brazos River near Rosharon, TX | 86,320 | 99,340 | 106,800 | 112,800 | 122,500 | 130,600 | 99,380 |

Figure 14 depicts the final upstream inflow hydrograph calculated at USGS Gage 08111500.

Figure 14:  Inflow Hydrograph from USGS 08111500

Using the decoupled methodology, the inflow hydrograph derived via the DUH approach at USGS Gage 08111500 was routed through Model Area 1 with no additional contributing flows. The outflow hydrograph for Model Area 1 was then compared to the target discharge values for the peak values listed in Table 14. If the fluvial outflow verification location happened to not be coincident with the physical gage location, then the discharge was linearly interpolated and transposed between the known points. Figure 15 below illustrates the gage locations along the Brazos River relative to the model area’s inflow and outflow locations. The gage flow transposition methodology was repeated for each modeled event using the linear approach.

Figure 15:  Transposed Target Flow Example

It should be noted that the observed as well as the calculated discharges at the gages along the Brazos River decrease in the downstream direction due to the extremely flat topography within the model footprint resulting in a large attenuation of flow. Routing a single inflow hydrograph through the model area in almost every instance resulted in insufficient flow at the downstream target flow check point. To rectify this, an internal boundary condition was drawn along the stream centerline of the Brazos River.  An inline hydrograph was added to this internal boundary condition line to add additional volume to the system thus increasing the peak discharge at the outlet. This process was iterated numerous times for each scoped event until a reasonably close match with the downstream target flow value was achieved with a goal of 1-3% difference. An example of this process is illustrated in Figure 16 from the Model Area 1 model. The same concept was applied to all model areas for all scoped events.

Figure 16:  Modeled Outflow vs Target Outflow Example

## Hydraulics

This section describes the remaining hydraulic modeling considerations, including implementation of Manning's (n) roughness, breaklines, refinement regions, and hydraulic structures within the 2D computational mesh. Detailed modeling methodology is discussed in the following sub-sections.

### Roughness Coefficients

Manning's (n) roughness coverage was developed for the 2D computational mesh using typical roughness values for given NLCD land classifications. Two different sets of Manning's n values were used for areas within the channel and all other areas outside of the channels of the stream network. This allows for the model to accurately represent deep, swift flow within the channel and shallow overland flow outside of the channel areas. The table below shows a typical land use roughness matrix used in defining the roughness coverage for the study area for areas both inside and outside of the channel.

Table 15:  NLCD 2019 Manning's n Roughness Matrix

| NLCD Classification | Within Channel |  |  | Outside Channel | Source |
| --- | --- | --- | --- | --- | --- |
| Open Water | 0.040 |  |  | 0.040 | Chow 1959 |
| Developed, Open Space | 0.035 |  |  | 0.050 | Calenda, et al. 2005 |
| Developed, Low Intensity | 0.035 |  |  | 0.080 | Calenda, et al. 2005 |
| Developed, Medium Intensity | 0.035 |  |  | 0.100 | Calenda, et al. 2005 |
| Developed, High Intensity | 0.040 |  |  | 0.150 | Calenda, et al. 2005 |
| Barren Land | 0.030 |  |  | 0.040 | Chow 1959 |
| Deciduous Forest | 0.060 |  |  | 0.160 | Chow 1959 |
| Evergreen Forest | 0.060 |  |  | 0.160 | Chow 1959 |
| Mixed Forest | 0.060 |  |  | 0.160 | Chow 1959 |
| Scrub/Shrub | 0.040 |  |  | 0.100 | Chow 1959 |
| Grassland/Herbaceous | 0.040 |  | 0.060 |  | Chow 1959 |
| Pasture/Hay |  | 0.040 | 0.060 |  | Chow 1959 |
| Cultivated Crops |  | 0.040 | 0.060 |  | Chow 1959 |
| Woody Wetlands |  | 0.050 | 0.120 |  | Chow 1959 |
| Emergent Herbaceous Wetland |  | 0.045 | 0.070 |  | Chow 1959 |

### Breaklines

Breaklines align grid cell faces and were used within the 2D mesh area to define prominent features including road embankments and hydraulic structures. Road embankment features were delineated in GIS and imported into HEC-RAS as breaklines to contain water where appropriate. Similarly, significant bridge/culvert crossings that were not processed out of the terrain data were modeled by v-notch breaklines adjacent to the road embankment to align grid cells around the embankment and allow water to be routed through the embankment without creating artificial backwater. An example of the v-notch breakline approach is shown in Figure 17.

County, interstate, common name, state recognized, and U.S. roads were incorporated for road breaklines from the U.S. Census Bureau TIGER/Line Road layer.

All publicly owned dams, according to the 2018 National Inventory of Dams, were modeled with breaklines to provide protection. This method allows water to flow out of the spillway if it reaches that elevation. Private dams were not modeled as showing protection to lessen the burden on private dam owners to maintain their dams to sustain the regulatory products created because of this study.

Breaklines were also used along all the stream centerlines of streams draining greater than one square mile of drainage area. Each stream centerline was enforced with 100-foot cell spacing to align cells along the streamline and capture more detail along the one square mile stream network.

Figure 17: V-notch Breakline Approach at Bridge Crossing

### Refinement Regions

Refinement regions were created for each community within the model. The population center boundary was used to determine the shape of the refinement region. The regions were assigned a cell spacing of 50-feet by 50-feet. This finer cell spacing allows for more hydraulic calculations and mapping detail near areas where there are likely to be more structures. The table below shows the communities contained within Lower Brazos watershed that were modeled with refinement regions as well as the square mileage of the refinement region.

Table 16: Communities with 2D Mesh Refinement

| Community Name | Area (mi2) |
| --- | --- |
| Bailey's Prairie village | 4.84 |
| Beasley city | 0.22 |
| Bellville city | 2.67 |
| Brazoria city | 0.57 |
| Brazos Country city | 2.12 |
| Brenham city | 1.25 |
| Brookshire city | 3.49 |
| Burton city | 1.20 |
| Cumings CDP | 2.92 |
| Damon CDP | 1.12 |
| Fairchilds village | 2.00 |
| Freeport city | 3.30 |
| Greatwood CDP | 2.75 |
| Hempstead city | 5.56 |
| Holiday Lakes town | 0.21 |
| Industry city | 1.08 |
| Lake Jackson city | 5.41 |
| Missouri City | 7.08 |
| Needville city | 0.53 |
| New Territory CDP | 4.78 |
| Orchard city | 0.38 |
| Pattison city | 3.25 |
| Pecan Grove CDP | 7.35 |
| Pine Island town | 9.31 |
| Pleak village | 1.97 |
| Prairie View city | 5.98 |
| Richmond city | 4.28 |
| Rosenberg city | 22.06 |
| San Felipe town | 8.74 |
| Sealy city | 9.00 |
| Sienna Plantation CDP | 4.25 |
| Simonton city | 2.00 |
| Sugar Land city | 18.42 |
| Thompsons town | 8.32 |
| West Columbia city | 1.87 |
| Weston Lakes city | 2.74 |
| Wild Peach Village CDP | 2.36 |

# Floodplain Mapping

The following sections provide a synopsis of how raw modeled depths were translated into Special Flood Hazard Areas (SFHA). In addition to developing a new SFHA, the BLE model data was leveraged to validate the effective Zone A studies within the project footprint.

## Model Outputs

The HEC acknowledged RAS Mapper may not render WSEL values and/or delineate inundation extent appropriately for direct use in FEMA regulatory mapping products. As a result, Compass created a workflow to export model WSEL values for each 2D mesh cell to develop regulatory-ready WSEL grids and subsequent flood hazard area mapping features.

## Methodology

Maximum WSEL values were exported for each model's 2D computational mesh cell centroid and densified points along the floodplain fringe. A TIN was created from the centroids, and a 1-meter WSEL grid was sampled from the TIN to match the resolution of the input terrain DEM. Polygons were created from the WSEL grid for SFHA and non-SFHA features using the following approach, based on FEMA SID 110. See Figure 18-Figure 21 for an example of workflow.

- BLE 1% annual chance inundation converted from grid cells to SFHA (Zone A) polygons where:

  - Effective SFHA exists, and where:

  - Flooding source drains greater than 1 square mile

- BLE 0.2% annual chance inundation results converted from grid cells to Shaded X polygons where connected extents intersect BLE 1% SFHA

Figure 18: 2D BLE Model Points for Mapping Workflow

Figure 19:  2D BLE Model Points to Water Surface Elevation TIN

Figure 20:  Water Surface Elevation TIN to Grid

Figure 21:  Water Surface Elevation Grid to Mapping Products

Traditional and approved floodplain mapping approaches were used to remove unnecessary points, bends, and angles while preserving the natural shape of the polygon. Furthermore, small voids (or "holes") inside of the floodplain were aggregated with the larger surrounding polygons to complete the floodplain mapping product.

## BLE Flood Hazard Layer

FIRM database format S_FLD_HAZ_AR feature classes were developed and attributed with Zone A and Shaded Zone X features for each model. The BLE S_FLD_HAZ_AR feature class was tested for topological errors described in the FIRM Database Technical Reference and all errors were fixed for data integrity and schema agreement.

## Additional 1-Percent Features

BLE 1% annual chance inundation not mapped as BLE SFHA is retained in an additional 1% feature class that can be used at Discovery to communicate other areas of flood hazard.

## CNMS Zone A Validation Check A5

Consistent with the Validation Procedure for Zone A Studies in FEMA's Automated Engineering Guidance for Flood Risk Analysis and Mapping (May 2016), effective Zone A comparison was performed using the BLE where Zone A polygons exist in the National Flood Hazard Layer (NFHL). Effective Zone A boundaries were tested at points along the effective SFHA boundary. The test points were intersected with the BLE ground DEM and 1% Plus and Minus elevations from the BLE. DEM elevations represent the effective elevation of the boundary and while the "confidence band," created by the BLE 1% plus and 1% minus elevations, was used to determine if the effective elevation is within the "confidence band" range. Additionally, a vertical tolerance and a horizontal tolerance was applied to the A5 assessment, with the vertical tolerance that representings one half of the assumed effective topographic data source contour interval. Figure 22 graphically depicts an example of the elevation along effective SFHA compared to the upper and lower ranges of the "confidence band" from the 1% plus and minus recurrence intervals. Additionally, a horizontal tolerance applied to the A5 assessment. If the test point was within the vertical and horizontal range, it passed. If the test point was not within range, it failed. Vertical and horizontal tolerances used in the A5 check are consistent with expectations defined in FEMA's Automated Engineering Guidance.

Figure 22:  Example A5 Assessment Profile

# Results

The BLE results for this study produced a SFHA that compares favorably with the effective SFHA. These boundaries provide an estimated SFHA in areas that have not been previously studied and therefore do not currently have a SFHA mapped. These results provide context for flood risk communication as part of the Discovery process and should be verified through community work map meetings before being applied to a regulatory product.

A map showing the BLE results is included as Appendix A.

## CNMS Validation of Effective Zone A SFHA

The inventory of Zone A studies (927.7 miles) in the Lower Brazos HUC-8 Watershed were classified in CNMS with validation status of "UNVERIFIED" (263.7 miles) or "VALID" (664.0 miles), and with status type of "BEING STUDIED." The following is a summary of the results of the CNMS validation assessment for the effective Zone A studies in the study area.

A1 – SIGNIFICANT TOPOGRAPHY UPDATE CHECK

This check involves determining whether a topographic data source is available that is significantly better than what was used for the effective Zone A modeling and mapping. Portions of the Effective studies in Austin, Brazoria, and Fort Bend Counties utilized LiDAR data that meets SID 43 specification and, therefore, pass this check. All other effective Zone A studies within this watershed did not use LiDAR data. The LiDAR sources utilized in this BLE study are considered significant improvements from the effective Zone A topographic sources. Therefore, all other reaches in the watershed fail this check.

A2 – CHECK FOR SIGNIFICANT HYDROLOGY CHANGES

This check involves first determining if regression equations were used for the effective study. Next, it must be determined whether new regression equations have become available from the USGS since the date of the effective Zone A study. If newer regression equations exist for the area of interest, then an engineer must determine whether these regression equations would significantly affect the 1-percent annual chance flow.

Effective studies in Fort Bend County and a portion of those in Austin County are known to have used regression analysis. New equations have been published since the time of those effective studies, and therefore, these reaches fail this check. For the rest of the Zone A's in this watershed, the effective hydrologic methods are unknown and automatically pass this check for that reason.

A3 – CHECK FOR SIGNIFICANT DEVELOPMENT

This check involves using the National Urban Change Indicator (NUCI) dataset to assess increased urbanization in the watershed of the Zone A study. If the percentage of urban area within the HUC-12 watershed containing the effective Zone A study is 15% or more, and has increased by 50% or more since the effective analysis, the study would fail this check. Although the NUCI data provide year-to-year changes in urbanization, the NLCD also is needed to establish a baseline of urban land cover for this analysis. The check for significant development in this watershed was completed by evaluating percentage of urban change at the HUC-12 level.

Portions of Fort Bend and Austin Counties meet the urban threshold but have not experienced a significant increase in urbanization since the effective analysis. The remainder of the HUC-8 does not meet the urban threshold. For these reasons, all reaches in the watershed pass this check.

A4 – CHECK OF STUDIES BACKED BY TECHNICAL DATA

This check involves determining if the effective Zone A study is supported by modeling or sound engineering judgment. If the effective Zone A is not supported by modeling, or if the original engineering method used is unsupported or undocumented, a comparison of the BLE results and effective Zone A's is performed.

The effective studies in Austin, Brazoria, Fort Bend, and a portion of Washington Counties are supported by technical data and pass this check. The remaining reaches in this watershed fail this check as they are not model-backed studies and there is no supporting technical data for them.

All of the initial assessment results are shown in Table 17.

Table 17: Zone A Initial Assessment Results

| Assessment Check | Pass / Fail | Notes |
| --- | --- | --- |
| A1 – Topography | Pass/Fail | LiDAR used in effective study/LiDAR sources available are considered significant improvements from the effective Zone A topographic sources. |
| A2 – Hydrology | Pass/Fail | Regression equations not known to be used in effective/ New equations were published since effective analysis. |
| A3 – Development | Pass | Watershed does not meet urban threshold OR urban threshold increased less than 50% |
| A4 – Technical Data | Pass/Fail | Study is supported by technical data/Study is not model-backed. |

A5 – Comparison of BLE and Effective Zone A Floodplains

The effective Zone A comparison was performed at the full extent of the Lower Brazos Watershed. The validation of the effective Zone A boundaries using 2D flood hazard products differ from the standard 1D methods due to the lack of cross sections and their use with standard Floodplain Boundary Standards (FBS) methodology. For this 2D study, the effective A zone boundaries were compiled using the NFHL. This data was dissolved to one continuous Zone A layer, which then had points placed along its perimeter every 500 feet.

For each test point, a 75-foot buffer was created. Using this buffer, the minimum and maximum values of the DEM were extracted, as a proxy for the effective base flood elevation. The minimum value of the 1% minus raster and the maximum value of the 1% plus raster are also extracted. These 1% plus maximum and 1% minus minimum values are products of the new 2D BLE study and act as the vertical tolerance. The test point passes if the DEM maximum value is less than or equal to the 1% plus maximum value and the DEM minimum value is greater than or equal to the 1% minus minimum value. This can be visualized as a short 75-foot radius cylinder, with a height of 1% plus maximum – 1% minus minimum. This test verifies that at least one point from the ground surface (i.e., proxy Base Flood Elevation [BFE]) falls both vertically and horizontally within this range.

BLE comparison results are shown in Table 18.

Table 18: BLE Comparison Results

| Watershed Name | HUC-12 | Total FBS Points | Fail | Pass | % Pass | BLE Comparison Pass? (>85%) | Priority Score |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Lower Brazos | All Streams | 48,882 | 6,515 | 42,367 | 86.7% | Pass | N/A |
| Brazos River-Gulf of Mexico | 120701040505 | 33 | 21 | 12 | 36.4% | Fail | 76.9 |
| Brookshire Creek-Bessies Creek | 120701040308 | 181 | 61 | 120 | 66.3% | Fail | 20.2 |
| Browning Branch-Brazos River | 120701040102 | 833 | 329 | 504 | 60.5% | Fail | 31.6 |
| Bullinger Creek-Brazos River | 120701040304 | 551 | 181 | 370 | 67.2% | Fail | 26.3 |
| Coon Creek-Cottonwood Creek | 120701040404 | 1,073 | 183 | 890 | 83% | Fail | 15.3 |
| Cottonwood Creek-Brazos River | 120701040309 | 142 | 29 | 113 | 79.6% | Fail | 17.98 |
| Cow Creek-Brazos River | 120701040501 | 282 | 81 | 201 | 71.3% | Fail | 41.5 |
| Deadman Creek-Mill Creek | 120701040210 | 1,947 | 293 | 1,654 | 84.95% | Pass | 11.5 |
| Dry Branch-Bessies Creek | 120701040303 | 538 | 221 | 317 | 58.9% | Fail | 28.6 |
| Dry Creek-Piney Creek | 120701040107 | 2,110 | 221 | 1,889 | 89.5% | Pass | 6.5 |
| Dry Creek-Rabbs Bayou | 120701040408 | 645 | 3 | 642 | 99.5% | Pass | 0.4 |
| East Mill Creek | 120701040204 | 2,810 | 251 | 2,559 | 91% | Pass | 6.04 |
| Fairchilds Creek-Big Creek | 120701040406 | 1,187 | 152 | 1,035 | 87.2% | Pass | 11.5 |
| Gibbs Creek-Caney Creek | 120701040103 | 1,911 | 296 | 1,615 | 84.5% | Pass | 11.5 |
| Harris Creek-Irons Creek | 120701040301 | 1,181 | 299 | 882 | 74.7% | Fail | 20.3 |
| Headwaters Caney Creek | 120701040101 | 2,201 | 127 | 2,074 | 94.2% | Pass | 3.7 |
| Indian Creek-East Mill Creek | 120701040201 | 2,105 | 201 | 1,904 | 90.5% | Pass | 5.3 |
| Ives Creek-Brazos River | 120701040108 | 1,535 | 302 | 1,233 | 80.3% | Fail | 13.7 |
| Jackson Creek-East Mill Creek | 120701040203 | 1,870 | 188 | 1,682 | 89.95% | Pass | 6.5 |
| Lewisville Creek-Brazos River | 120701040106 | 787 | 283 | 504 | 64% | Fail | 25.7 |
| Lower Allens Creek | 120701040305 | 676 | 169 | 507 | 75% | Fail | 15.06 |
| Lower West Mill Creek | 120701040208 | 2,447 | 72 | 2,375 | 97% | Pass | 2.3 |
| Middle West Mill Creek | 120701040206 | 3,325 | 186 | 3,139 | 94.4% | Pass | 2.9 |
| Mulberry Creek-Irons Creek | 120701040302 | 563 | 164 | 399 | 70.9% | Fail | 23.3 |
| Ponds Creek-Clear Creek | 120701040104 | 1,312 | 143 | 1,169 | 89.1% | Pass | 8.8 |
| Post Oak Point Creek-Sandy Creek | 120701040207 | 2,026 | 21 | 2,005 | 98.96% | Pass | 0.7 |
| Rabbs Bayou-Brazos River | 120701040402 | 779 | 227 | 552 | 70.9% | Fail | 22.6 |
| Sandy Branch-Brazos River | 120701040401 | 309 | 78 | 231 | 74.8% | Fail | 15.6 |
| Sandy Creek-East Mill Creek | 120701040202 | 1,399 | 133 | 1,266 | 90.5% | Pass | 5.8 |
| Sandy Creek-Mill Creek | 120701040209 | 4,130 | 181 | 3,949 | 95.6% | Pass | 3.5 |
| Seabourne Creek-Big Creek | 120701040405 | 191 | 37 | 154 | 80.6% | Fail | 17.4 |
| Steep Bank Creek-Brazos River | 120701040403 | 933 | 529 | 404 | 43.3% | Fail | 34.3 |
| Threemile Creek-Clear Creek | 120701040105 | 1,499 | 227 | 1,272 | 84.9% | Pass | 12.1 |
| Upper Allens Creek | 120701040306 | 329 | 38 | 291 | 88.5% | Pass | 5.8 |
| Upper Dry Bayou | 120701040503 | 985 | 54 | 391 | 94.5% | Pass | 14.98 |
| Upper West Mill Creek | 120701040205 | 2,163 | 194 | 1,969 | 91% | Pass | 4.5 |
| Varner Creek | 120701040502 | 1,717 | 302 | 1,415 | 82.4% | Fail | 14.07 |
| Worthington Lake-Big Creek | 120701040407 | 50 | 5 | 45 | 90% | Pass | 9 |

An overall risk for each HUC-12 watershed was calculated using the National Flood Risk Percentages Dataset and its proportional area. The weighted risk was multiplied by the percentage of points in the watershed that failed the CNMS comparison to effective to determine the priority score. Figure 23 below shows the range of the Lower Brazos HUC-12 priority scores which can be used to initiate discussions during the Discovery phase. Brazos River-Gulf of Mexico HUC-12 was determined to have the highest priority score and the most need while Dry Creek-Rabbs Bayou HUC-12 has the lowest score.

Figure 2323:  Ranking of Lower Brazos HUC-12s

While results in this report are shown at a HUC-12 level, the pass/fail status of the A5 check are determined at the stream reach basis. The validation status of a reach is determined by the BLE comparison (A5) results except in the case where there are not enough sample points to complete an A5 comparison. When there are not sufficient sample points to complete a comparison for an individual reach, the validation status is then based on the results of the initial assessment checks (A1-A4).

A summary of the validation results is shown in Table 19.

Table 19: Zone A Validation Results

| Validation Status | Miles |
| --- | --- |
| VALID | 664.0 |
| UNVERIFIED | 263.7 |
| Total | 927.7 |

Flood Risk Assessment

A flood risk analysis was performed for this project.  The updated 1% annual chance and 0.2% annual chance depth grids were used to calculate the potential flood losses.  The loss results are stored in the S_FRAC_AR spatial file within the FRD geodatabase.  All results are reported in whole dollar values.

Hazus version 5.0  (SP03) was used for the basic and refined loss analysis.

The losses are reported via census blocks.  It is important to note that Hazus version 4.2 (SP03) uses dasymetric census blocks.  Dasymetric mapping removes undeveloped areas (such as areas covered by other bodies of water, wetlands, or forests) from the census blocks, changing their shape and reducing their size in these areas.  For more information on dasymetric data visit FEMA’s Media Library for the Hazus-MH Data Inventories: Dasymetric vs. Homogenous, or Hazus 3.0 Dasymetric Data Overview.

Hazus analysis was performed by county/parish within the project watershed extends for each return period to ensure proper model processing.  A summary of results per county/parish for the 1% annual chance event scenarios are shown in Table 20.

Table 20: Hazus 5.0 (SP02) Results for 1-percent-annual-chance (100 year) scenario for Lower Brazos Watershed, Texas.

| County/Parish | Full Replacement – Total Loss | Dollar Exposure (Replacement Value) - Buildings | Dollar Exposure (Replacement Value) - Contents |
| --- | --- | --- | --- |
| Austin | 2,589,982,000 | 46,181,722 | 35,749,722 |
| Brazoria | 1,668,572,000 | 90,132,181 | 75,128,181 |
| Fayette County | 19167000 | 0 | 0 |
| Fort Bend | 23,565,961,000 | 1,882,096,197 | 1,264,040,197 |
| Waller | 2,128,176,000 | 24,789,551 | 20,262,551 |
| Washington | 832,771,000 | 5,306,387 | 2,758,387 |
| Total | 30,804,629,000 | 2,048,506,038 | 1,397,939,038 |

# Challenges

The main challenges faced during the duration of the modeling effort of the Lower Brazos Watershed 2D BLE study were:

Extremely large 2D BLE models

- Large number of 2D cells drastically increased the model run times due to the sheer number of wet/dry calculations needed throughout the simulation.

- Models with more than 500,000 to 700,000 cells would not run in HEC-RAS version 6.2 due to a software bug. This required the models to initially be run with HEC-RAS version 6.1. Once version 6.3 was released the modeling team switched to take advantage of the 2D processing efficiencies introduced by the HEC

Tie-ins with adjacent areas

Finding the best hydrologic match for both the pluvial and fluvial flooding.

- Fluvial:  The extremely flat nature of the topography created a natural attenuation effect in the surrounding overbanks of the mainstream corridor. This natural loss of water to the available storage in the overbanks created a challenge for the fluvial runs when trying to match the downstream target discharges. As a result, each scoped event for all three model areas required numerous iterations with the addition on an incremental inline boundary condition drawn along the stream centerline of the Brazos River.

- Pluvial:  The large model area required the use of smaller ARFs to create reasonable model discharge results when compared to other best available data sources.

# Recommendations

This study provides significant information useful for flood identification and communication among those affected. The study is highly scalable, and stakeholder input and further analysis could enhance end products during transition to regulatory flood hazard areas. Results presented in this report and the accompanying FEMA data capture technical reference digital data should be presented and further evaluated through Flood Risk Review meetings before being mapped as special flood hazard areas.

In addition, bathymetry of the mainstem was not included in the scope, though a baseflow was represented in the terrain.  In future studies, bathymetry could be used to better represent the depth of the Brazos River.

# References

Chow, Ven T. "Development of Uniform Flow and It Formulas." Open Channel Hydraulics. Caldwell, NJ: Blackburn, 1959. 109-113. Print.

England, J.F., Jr., Cohn, T.A., Faber, B.A., Stedinger, J.R., Thomas, W.O., Jr., Veilleux, A.G., Kiang, J.E., and Mason, R.R., Jr., 2018, Guidelines for determining flood flow frequency—Bulletin 17C: U.S. Geological Survey Techniques and Methods, book 4, chap. B5, 148 p., https://doi.org/10.3133/tm4B5.

FEMA, "Guidance for Flood Risk Analysis and Mapping – Base Level Engineering (BLE) Analyses and Mapping", November 2021. (https://www.fema.gov/sites/default/files/documents/fema_base-level-engineering-guidance_112021.pdf).

Homer, C.G., Dewitz, J.A., Yang, L., Jin, S., Danielson, P., Xian, G., Coulston, J., Herold, N.D., Wickham, J.D., and Megown, K., 2015, Completion of the 2016 National Land Cover Database for the conterminous United States-Representing a decade of land cover change information. Photogrammetric Engineering and Remote Sensing, v. 81, no. 5, p. 345-354

National Oceanic and Atmospheric Administration, 2022. Datums for 8772471, Freeport SPIP, Freeport Harbor TX. https://tidesandcurrents.noaa.gov/datums.html?datum=NAVD88&units=0&epoch=0&id=8772471&name=Freeport+SPIP%2C+Freeport+Harbor&state=TX

Texas Department of Transportation, Research and Technology Implementation Office, CALCULATION OF AREAL REDUCTION FACTORS USING NEXRAD PRECIPITATION ESTIMATES, Report 0-4642-3, November 2006, Austin, TX, https://static.tti.tamu.edu/tti.tamu.edu/documents/0-4642-3.pdf

U.S. Department of the Interior, U.S. Geological Survey, In cooperation with the Texas Department of Transportation, Areal-Reduction Factors for the Precipitation of the 1-Day Design Storm in Texas, Water-Resources Investigations Report (WRIR) 99-4267, W.H. Asquith, Austin, TX 1999, https://doi.org/10.3133/wri994267

U.S. Geological Survey, 1996. Techniques to estimate generalized skew coefficients of annual peak streamflow for natural basins in Texas: Water-Resources Investigations Report 96-4117. Austin, Texas.

U.S. Geological Survey, 2018, PeakFQ version 7.2, https://water.usgs.gov/software/PeakFQ/.

United States Department of Agriculture, 2012. Chapter 5 of the NRCS National Engineering Handbook Part 630, Amended May 2012.

United States Department of Agriculture, Natural Resources Conservation Service, National Engineering Handbook, Chapter 7, Hydrologic Soil Groups, January 2009, https://directives.sc.egov.usda.gov/OpenNonWebContent.aspx?content=22526.wba

United States Department of Agriculture, Natural Resources Conservation Service, National Engineering Handbook, Chapter 4 Storm Rainfall and Distribution, August 2019, https://directives.sc.egov.usda.gov/OpenNonWebContent.aspx?content=43924.wba

United States Department of Commerce, Weather Bureau, Technical Paper-29, Rainfall Frequency Atlas of the United States for Durations from 30 Minutes to 24 Hours and Return Periods from 1 to 100 Years, June 1957-1960, https://www.weather.gov/owp/hdsc_publications

US Army Corps of Engineers Hydrologic Engineering Center, August 2022. HEC-RAS River Analysis System User's Manual, Version 6.3.

USDA, NRCS, Implementation of NOAA 14 Precipitation Atlas Volume 11 for State of Texas, 2021

USGS, "Estimating Magnitude and Frequency of Floods Using PeakFQ Program: USGS Fact Sheet", 2006.

USGS. Multi-Resolution Land Characteristics Consortium. National Land Cover Database 2019. (https://www.mrlc.gov/national-land-cover-database-nlcd-2016).

V.B. Sauer, Dimensionless Hydrograph Method of Simulating Flood Hydrographs, 1989, https://onlinepubs.trb.org/Onlinepubs/trr/1989/1224/1224-010.pdf

Wang, Hong, National Oceanic and Atmospheric Administration. NOAA Atlas 14, Volume 11, Precipitation-Frequency Atlas of the United States. Silver Spring, Maryland, 2018.

# Appendix A – Exhibits – Base Level Engineering Results

`