| Submitted By: | Doucet & Associates | Date: | 01/07/2022 |
| --- | --- | --- | --- |
| Reviewed By: | FTN Associates, Ltd. | Date: | 01/25/2022 |

Technical Review – Lower Brazos San Gabriel (HUC 12070205)

| Y | N | N/A | Item Description | Comments |
| --- | --- | --- | --- | --- |
| General |  |  |  |  |
| ☒ | ☐ | ☐ | Has the CNMS QC Tool been run, and any remaining error checked and addressed? | Provided shapefiles were uploaded to a database with the 2021 CNMS Schema to run the CNMS QC 2019 Tool. Errors were produced. Please see FY19_QCTool.txt and make necessary changes. Revised |
| ☐ | ☒ | ☐ | Does the CNMS database include a self-certification form? | No self-certification form was included. As this was something added late during the review process, please consider adding when complete. Included |
| CNMS Geodatabase Check |  |  |  |  |
| S_Studies_Ln (Zone AE, Detailed, and newly added unmapped reaches) |  |  |  |  |
| ☐ | ☒ | ☐ | Has the BLE for all reaches been updated and based on CNMS Tech reference? | BLE field should be populated with “BLE 2D” instead of “BLE TIER A” BLE has only been updated for X reaches. Please update for A and AE reaches as well. Revised |
| ☐ | ☒ | ☐ | Has the BLE_CASE_NO been added and up to date and based on CNMS Tech reference? | Since this is unknown at the time, this can be populated as needed. Noted |
| ☐ | ☐ | ☐ | Has the BLE_DATE been added and up to date and based on CNMS Tech reference? | BLE Date currently listed as 9/9/9999. Please update date. BLE Date has only been updated for X reaches. Please update for A and AE reaches as well. Revised |
| S_Studies_Ln (Zone A and X Reaches) |  |  |  |  |
| ☐ | ☒ | ☐ | Has the STATUS_TYPE field been populated with “BEING STUDIED”? | STATUS_TYPE is currently “TO BE STUDIED” and should be changed to “BEING STUDIED.” Revised |
| ☐ | ☒ | ☐ | Has the STATUS_DATE field been populated with the Current Date? | STATUS_DATE should be updated for Zone A reaches as well. Revised |
| ☐ | ☒ | ☐ | Has the BS_CASE_NO field been populated? | Revised |
| ☒ | ☐ | ☐ | Has the BS_ZONE field been populated with “A”? |  |
| ☐ | ☐ | ☐ | Has the BS_STDYTYP field been populated with “NEW OR UPDATED APPROXIMATE”? | All Zone A and X streams should be listed as “NEW OR UPDATED APPROXIMATE” instead of “NEW APPROXIMATE” or “UPDATED APPROXIMATE.” Revised |
| ☒ | ☐ | ☐ | Has the BS_HYDRO_M field been populated with “OTHER” and defined in the comment field? |  |
| ☒ | ☐ | ☐ | Has the BS_HYDRO_CMT field been populated? |  |
| ☒ | ☐ | ☐ | Has the BS_HYDRA_M field been populated with “HEC-RAS”? |  |
| ☐ | ☒ | ☐ | Has the BS_HYDRA_CMT field been populated? | Please populate the HYDRA_CMT field. Revised |
| ☐ | ☒ | ☐ | Has the BS_MODEL_2D field been populated with “T” for True/Yes? | Please populate the BS_MODEL_2D field. Revised |
| ☒ | ☐ | ☐ | Has the BS_FY_FUND field been populated? |  |
| ☒ | ☐ | ☐ | Has the PRELIM_DATE field been populated? |  |
| ☒ | ☐ | ☐ | Has the LFD_DATE field been populated? |  |

General Comments:

- S_Studiess_Ln and S_Unmapped_Ln will need to be copied into a database with the 2021 CNMS schema. This can be done by exporting the 2021 Schema as a .XML and constructing a new database using this exported schema. Revised

- Some of the fields in the CNMS database have domain codes. These will need to be followed to ensure everything is copied over correctly. See Section F.2. of the CNMS Technical Reference for the proper domain codes. Noted

- The Point of Contact table will need to be updated for every county the study falls in. The BLE_POC field will need to be updated for Zone X and A reaches (not AE?). Directions to generate the POC_ID can be found in Table F-6 of the CNMS Technical Reference for the proper domain codes.