| Submitted By: | Doucet & Associates | Date: | 03/02/2022 |
| --- | --- | --- | --- |
| Reviewed By: | FTN Associates, Ltd. | Date: | 03/03/2022 |

Technical Review – Lower Brazos San Gabriel (HUC 8 # 12070205)

HUC 10 # 1207020501

| Y | N | N/A | Item Description | Comments |
| --- | --- | --- | --- | --- |
| Data Completeness Check |  |  |  |  |
| ☒ | ☐ | ☐ | Hydrology model and/or spreadsheets included | No hydrology models or hydrology calculations included. HMS 4.8 and spreadsheet “LBSG_Precipitation” have been included in supplemental data for review. Rev1 - Ok |
| ☒ | ☐ | ☐ | Terrain dataset included |  |
| ☐ | ☒ | ☐ | Projection file included | The projection file that the model has been linked to is not included. The projection file has been added to each model folder. Rev1 - Unable to find projection file in model folder. Projection file added to the simulations folder |
| ☒ | ☐ | ☐ | 2D boundaries included |  |
| ☒ | ☐ | ☐ | 2D Breaklines included |  |
| ☒ | ☐ | ☐ | Land Cover dataset (Manning’s n-values) Included |  |
| ☒ | ☐ | ☐ | Infiltration dataset (Composite CN values) Included |  |
| ☐ | ☒ | ☐ | Regression analysis Included | No regression analysis included. FIS discharges have been used for comparison in lieu of regression equations. Rev1 - Noted |
| ☒ | ☐ | ☐ | Gage analysis Included |  |
| ☒ | ☐ | ☐ | Preliminary floodplain runs Included in RAS-Mapper |  |
| ☒ | ☐ | ☐ | Terrain, land cover, and infiltration data have been associated with correct geometry(s) |  |
| ☐ | ☒ | ☐ | All flood events included in the HEC-RAS model (10%, 4%, 2%, 1%, 0.2%, 1%+, 1%-, and any observed rainfall events) | Rev1 - 1% results layer is missing 1% results layer has been added. |
| ☒ | ☐ | ☐ | Modeling narrative included | The narrative will need to be finalized with the completion of the project. Understood. Draft report is included in draft submittal package. Rev1 - Noted |
| Hydrology |  |  |  |  |
| Rainfall |  |  |  |  |
| ☒ | ☐ | ☐ | Precipitation values are an aerial average of Atlas 14 values for the entire 2D area boundary for each storm event | Please provide details/back up calculations on how the average NOAA Atlas 14 depths were generated. Added to hydrology section of report and HMS and spreadsheet included in supplemental data. Rev1 - Ok |
| ☐ | ☐ | ☒ | Total precipitation depths for 1%+ and 1%- are calculated correctly | Will verify once precipitation calculations are received. |
| ☒ | ☐ | ☐ | Rainfall hyetographs are calculated for all storm events (10%, 4%, 2%, 1%, 0.2%, 1%+, and 1%-) | The is a small drop in the 50yr, 100yr, 500yr, 100m, and 100p rainfall hyetographs. Did not have the backup data to confirm, please investigate and make sure the dip in the hyetograph is OK. No data entry errors identified. Appears to be product of balanced, nested hyetograph generation. Rev1 - Ok |
| ☒ | ☐ | ☐ | Cumulative excess rainfall values are in increasing order from 10%, 4%, 2%, 1%, to 0.2% |  |

| Y | N | N/A | Item Description | Comments |
| --- | --- | --- | --- | --- |
| ☒ | ☐ | ☐ | Cumulative excess rainfall values for the 1%+ and 1%- fall in order with the storm events matching the total precipitation values |  |
| Inflow Hydrographs (if any) |  |  |  |  |
| ☒ | ☐ | ☐ | Correct DSS file paths are being used in HEC-RAS |  |
| ☐ | ☐ | ☒ | Contributing hydrographs from another HEC-RAS model applied at coincident locations |  |
| HEC-RAS Geometry |  |  |  |  |
| Terrain |  |  |  |  |
| ☒ | ☐ | ☐ | 10ft Imported Grid resolution used |  |
| ☐ | ☐ | ☐ | Topographic abnormalities corrected and no drops or rises from underlying data |  |
| Land Cover (Manning’s n) |  |  |  |  |
| ☒ | ☐ | ☐ | Land Cover classifications are sufficiently descriptive within the data table |  |
| ☒ | ☐ | ☐ | Manning’s n values generally match current Land Cover classifications | 0.06 is being used for the stream buffer. This seems high. According the HEC-RAS 2D User’s Manual, the stream channel should generally follow the same range as open water. Roughness based on calibration runs to dampen peaks. Rev1 - Noted |
| Infiltration |  |  |  |  |
| ☐ | ☒ | ☐ | CN values generally match current soils and land use data | It appears that simulations are using the CN -15 infiltration layer. Please update report to include those values. Add values to table along with ARC1.5 and ARC3 values. Rev1 - Tables don’t appear to been updated; still include unedited CN values. Unedited CN values included in section 2.2.4.2 of report and calibrated CN values are in 2.2.3 |
| ☒ | ☐ | ☐ | Appropriate abstraction ratio and minimum infiltration used for CN classifications |  |
| 2D Mesh |  |  |  |  |
| ☒ | ☐ | ☐ | 2D Boundary delineated along watershed boundary or buffer to ensure all contributing area is included | It appears a buffer has been applied. |
| ☒ | ☐ | ☐ | Nominal cell size appropriate for model detail? (generally 200 feet) |  |
| ☒ | ☐ | ☐ | No mesh errors |  |

| Y | N | N/A | Item Description | Comments |
| --- | --- | --- | --- | --- |
| ☐ | ☒ | ☐ | Where appropriate cells overlap terrain features (i.e., V-notch at bridges) | V-Notches appear to be present along the stream centerline, but there are some in surrounding areas that are missing. See QAQC_Comments_HUC01.shp for examples. Please review model for other instances. V-notches addressed using terrain modification tools or breaklines. Terrain modification shapefile included in supplemental data for quick reference. Rev1 – There are still some areas that should have v-notches or a terrain modification. See HUC01_QAQC.shp for occurrences. Additional V-notches and terrain modifications added. |
| ☐ | ☐ | ☒ | 2D Area Connections placed at applicable benchmarks and set to use normal 2D equations domains |  |
| ☒ | ☐ | ☐ | Refinement zone assigned appropriately |  |
| Breaklines |  |  |  |  |
| ☒ | ☐ | ☐ | Breaklines are spaced at least 200 ft (cell spacing) apart or breakline spacing parameters adjusted as necessary? | No parameters imputed, uses cell size. |
| ☐ | ☒ | ☐ | Breaklines sufficiently detailed and consistent with the terrain data? | Several minor brealines missing. See QAQC_Comments_HUC01.shp for locations. Please review model for other instances. Rev1 – There are still some areas that should have breaklines. See HUC01_QAQC.shp for occurrences. Additional breaklines have been added. |
| 2D Boundary Conditions |  |  |  |  |
| ☐ | ☐ | ☒ | Initial conditions used and set appropriately | No initial condition used. |
| ☒ | ☐ | ☐ | Internal and external boundaries connected to 2D mesh |  |
| ☒ | ☐ | ☐ | Boundary conditions everywhere flow could build up at the boundary of the 2D mesh | Because a buffer was applied to boundary, water is building up around edges. There appears to be areas where the water build up is connected to stream floodplains. Suggest adjusting perimeter to reduce amount trapped or adding boundary condition. Please review perimeter and correct as necessary. Added normal depth BC lines around perimeter with nominal slope matched to outflow normal depth. Rev1 – Note mapping present due to the buffered perimeter will need to be removed during the mapping stage. |
| ☒ | ☐ | ☐ | Boundary conditions have a precipitation or hydrograph data and assigned at correct locations |  |
| ☒ | ☐ | ☐ | Downstream boundary consistent with ground slope / known WSEL |  |
| 2D Model Execution |  |  |  |  |
| ☒ | ☐ | ☐ | Appropriate 2D parameters and options used |  |
| Y | N | N/A | Item Description | Comments |
| ☒ | ☐ | ☐ | Hydrograph Output Interval and Mapping Output Interval set at a reasonable interval to capture peaks | It is our understanding that it was decided a mapping output of 1 hour and a hydrograph/detailed output of 15 min. Current settings are fine for internal review, but will ultimately need to be change for file size management. Updated and consistent with all models Rev1 - Ok |
| ☒ | ☐ | ☐ | Model runtime is reasonable |  |
| 2D Model Results |  |  |  |  |
| Computation Checks |  |  |  |  |
| ☒ | ☐ | ☐ | Model runs with no cell errors |  |
| ☒ | ☐ | ☐ | Volume balance is within tolerance (ΔV < 1% is ideal, however 2% to 3% can be acceptable for rain on mesh) |  |
| Full Checks |  |  |  |  |
| ☒ | ☐ | ☐ | Computational time step meets ideal Courant condition (diffusive wave ≤ 2, up to 5) | Target time step for team was 30 seconds, but 20 second time step was used. Please confirm. Updated and consistent with all models with 30 sec time step. Rev1 - Ok |
| ☒ | ☐ | ☐ | Model simulation is long enough to pass the entire hydrograph through the model (minimum of 12 hours past the peak, preferably 24 hours) |  |
| ☒ | ☐ | ☐ | Hydrographs at internal and external boundaries are smooth |  |
| ☒ | ☐ | ☐ | Contributing areas are contained within 2D mesh or accounted for with inflow hydrographs |  |
| ☒ | ☐ | ☐ | No Flow trapped at 2D Boundary | There are some areas where water is pooling around the perimeter. If they are not connected to the stream floodplain they can be left as is and will be removed during the mapping phase. Added perimeter boundary condition. Some of them are not connected to the streams and will be removed during the mapping phase. Rev1 - Ok |
| Spot Checks |  |  |  |  |
| ☒ | ☐ | ☐ | WSELs appear reasonable within a single cell (check for visible spikes) |  |
| ☐ | ☐ | ☐ | Depth grids appear reasonable (no artificial dips or spikes) |  |
| ☒ | ☐ | ☐ | Flooding is connected downstream for all significant contributing areas (> 64 acres) |  |

| Y | N | N/A | Item Description | Comments |
| --- | --- | --- | --- | --- |
| ☒ | ☐ | ☐ | Velocity grids appear reasonable (no unusually high velocity areas / hot spots) | 100-year velocity grid was reviewed and looked mostly reasonable. There are some steep areas with velocity hots spots (See screenshot for example). Consider refining areas with a breakline. Breaklines added; most will be resolved by added stream centerlines. Some hot spots remain at inflow BCs; however, these are typically within model “overlap” areas. Rev1 - Noted |
| Flow Checks |  |  |  |  |
| ☒ | ☐ | ☐ | DSS files contain runs consistent with project | DSS file only includes 100-year event in HUC10 1207020505. Updated to include all events. Rev1 - Noted |
| ☐ | ☐ | ☒ | Peak modeled discharges comparable to gages, and regional regression equations | Peak modeled discharges appear to have been compared to stream gages, but not regional regression equations. Discharges also compared to FIS discharges which are regression-based or better. Rev1 - Noted |
| 2D Model Verification |  |  |  |  |
| Stream Gage Analysis |  |  |  |  |
| ☐ | ☐ | ☒ | Regression analysis performed at select locations in the watershed | No regression analysis included. Discharges also compared to FIS discharges which are regression-based or better. Rev1 - Noted |
| ☒ | ☐ | ☐ | Stream gage analysis performed on all gages in the watershed with sufficient data |  |
| Sensitivity Analysis |  |  |  |  |
| ☒ | ☐ | ☐ | Sensitivity analysis performed to evaluate which model parameters significantly impact results |  |
| ☒ | ☐ | ☐ | Based on the sensitivity analysis results did the results of the selected computation settings resulted in acceptable outcomes |  |
| Additional Comments |  |  |  |  |
| - Lake Georgetown Dam: Water is currently only able to leave through the emergency floodway. Is there confirmation that is how a major flood event would be handled? From Aerial imagery it appears there a primary spillway. Dam discharges compared to USACE published maximum releases included in supplemental data. Since discharges are limited to 3,000 cfs, no modifications were deemed necessary. Rev1 - Noted - Suggest deleted old plans and files associated with old plans (plan .hdf files, .b files, .bco files, IC.O files). This will help with file size management. Will do. Rev1 – plans are no longer present in ras mapper, but files are still located in model folder. Please remove. Files Removed - At some stream and road intersections, there are cells cross the breakline. This may skew the amount of water that flows through the structure. These occurrences have been reviewed and adjusted as appropriate. Rev1 – There are still some areas that should be adjusted. See HUC01_QAQC.shp for occurrences. Revised. - Rev1 – We got this error message when the model was opened. This can be fixed by editing the project file and removing the old plan. Old plan was removed from folder - Rev1 – Terrain in model does not appear to have any terrain modifications. Is the terrain file submitted different than the one the model simulations used? The same terrain file is used for all models. Model now has some terrain modifications. |  |  |  |  |