| Submitted By: | Doucet & Associates | Date: | 03/02/2022 |
| --- | --- | --- | --- |
| Reviewed By: | FTN Associates, Ltd. | Date: | 03/03/2022 |

Technical Review – Lower Brazos San Gabriel (HUC 8 # 12070205)

HUC 10 # 1207020505

| Y | N | N/A | Item Description | Comments |
| --- | --- | --- | --- | --- |
| Data Completeness Check |  |  |  |  |
| ☒ | ☐ | ☐ | Hydrology model and/or spreadsheets included | No hydrology models or hydrology calculations included. HMS 4.8 and spreadsheet “LBSG_Precipitation” have been included in supplemental data for review. Rev1 – Ok |
| ☒ | ☐ | ☐ | Terrain dataset included |  |
| ☒ | ☐ | ☐ | Projection file included | The projection file that the model has been linked to is not included. The projection file has been added to each model folder. Rev1 – Ok |
| ☒ | ☐ | ☐ | 2D boundaries included |  |
| ☒ | ☐ | ☐ | 2D Breaklines included |  |
| ☒ | ☐ | ☐ | Land Cover dataset (Manning’s n-values) Included | Layer is currently linked to LBSG_501 folder. Suggest copying layer into LBSG_505 model folder. Revised Rev1 – Ok |
| ☒ | ☐ | ☐ | Infiltration dataset (Composite CN values) Included | Layer is currently linked to LBSG_501 folder. Suggest copying layer into LBSG_505 model folder. Revised Rev1 – Ok |
| ☐ | ☐ | ☒ | Regression analysis Included | No regression analysis included. FIS discharges have been used for comparison in lieu of regression equations. Rev1 – Noted |
| ☒ | ☐ | ☐ | Gage analysis Included |  |
| ☒ | ☐ | ☐ | Preliminary floodplain runs Included in RAS-Mapper |  |
| ☒ | ☐ | ☐ | Terrain, land cover, and infiltration data have been associated with correct geometry(s) |  |
| ☒ | ☐ | ☐ | All flood events included in the HEC-RAS model (10%, 4%, 2%, 1%, 0.2%, 1%+, 1%-, and any observed rainfall events) |  |
| ☒ | ☐ | ☐ | Modeling narrative included | The narrative will need to be finalized with the completion of the project. Understood. Draft report is included in draft submittal package. Rev1 – Ok |
| Hydrology |  |  |  |  |
| Rainfall |  |  |  |  |
| ☒ | ☐ | ☐ | Precipitation values are an aerial average of Atlas 14 values for the entire 2D area boundary for each storm event | Please provide details/back up calculations on how the average NOAA Atlas 14 depths were generated. Added to hydrology section of report and HMS and spreadsheet included in supplemental data. Rev1 – Ok |
| ☒ | ☐ | ☐ | Total precipitation depths for 1%+ and 1%- are calculated correctly | Will verify once precipitation calculations are received. Rev1 – Ok |
| ☒ | ☐ | ☐ | Rainfall hyetographs are calculated for all storm events (10%, 4%, 2%, 1%, 0.2%, 1%+, and 1%-) | The is a small drop in the 50yr, 100yr, 500yr, 100m, and 100p rainfall hyetographs. Did not have the backup data to confirm, please investigate and make sure the dip in the hyetograph is OK. No data entry errors identified. Appears to be product of balanced, nested hyetograph generation. Rev1 – Ok |
| ☒ | ☐ | ☐ | Cumulative excess rainfall values are in increasing order from 10%, 4%, 2%, 1%, to 0.2% |  |

| Y | N | N/A | Item Description | Comments |
| --- | --- | --- | --- | --- |
| ☒ | ☐ | ☐ | Cumulative excess rainfall values for the 1%+ and 1%- fall in order with the storm events matching the total precipitation values |  |
| Inflow Hydrographs (if any) |  |  |  |  |
| ☒ | ☐ | ☐ | Correct DSS file paths are being used in HEC-RAS |  |
| ☒ | ☐ | ☐ | Contributing hydrographs from another HEC-RAS model applied at coincident locations |  |
| HEC-RAS Geometry |  |  |  |  |
| Terrain |  |  |  |  |
| ☒ | ☐ | ☐ | 10ft Imported Grid resolution used |  |
| ☒ | ☐ | ☐ | Topographic abnormalities corrected and no drops or rises from underlying data |  |
| Land Cover (Manning’s n) |  |  |  |  |
| ☒ | ☐ | ☐ | Land Cover classifications are sufficiently descriptive within the data table |  |
| ☒ | ☐ | ☐ | Manning’s n values generally match current Land Cover classifications | 0.06 is being used for the stream buffer. This seems high. According the HEC-RAS 2D User’s Manual, the stream channel should generally follow the same range as open water. Roughness based on calibration runs to dampen peaks. Rev1 – Ok |
| Infiltration |  |  |  |  |
| ☒ | ☐ | ☐ | CN values generally match current soils and land use data |  |
| ☒ | ☐ | ☐ | Appropriate abstraction ratio and minimum infiltration used for CN classifications |  |
| 2D Mesh |  |  |  |  |
| ☒ | ☐ | ☐ | 2D Boundary delineated along watershed boundary or buffer to ensure all contributing area is included |  |
| ☒ | ☐ | ☐ | Nominal cell size appropriate for model detail? (generally 200 feet) |  |
| ☒ | ☐ | ☐ | No mesh errors |  |

| Y | N | N/A | Item Description | Comments |
| --- | --- | --- | --- | --- |
| ☒ | ☐ | ☐ | Where appropriate cells overlap terrain features (i.e., V-notch at bridges) | Breaklines don’t include v-notches along stream centerlines or in surrounding areas that are missing. See QAQC_Comments_HUC05.shp for examples. Please review model for other instances. V-notches addressed using terrain modification tools or breaklines. Terrain modification shapefile included in supplemental data for quick reference. Rev1 – There are still some areas that should have v-notches or a terrain modification. See HUC04_QAQC.shp for occurrences. V-notches addressed using terrain modification tools or breaklines. Terrain modification shapefile included in supplemental data for quick reference. |
| ☐ | ☐ | ☒ | 2D Area Connections placed at applicable benchmarks and set to use normal 2D equations domains |  |
| ☒ | ☐ | ☐ | Refinement zone assigned appropriately |  |
| Breaklines |  |  |  |  |
| ☒ | ☐ | ☐ | Breaklines are spaced at least 200 ft (cell spacing) apart or breakline spacing parameters adjusted as necessary? | No parameters imputed, uses cell size. |
| ☒ | ☐ | ☐ | Breaklines sufficiently detailed and consistent with the terrain data? | Rev1 – There are still some areas that should have breaklines. See HUC04_QAQC.shp for occurrences. |
| 2D Boundary Conditions |  |  |  |  |
| ☐ | ☐ | ☒ | Initial conditions used and set appropriately | No initial condition used. Granger Lake terrain elevations consistent with conservation pool as discussed in report. Rev1 – Ok |
| ☒ | ☐ | ☐ | Internal and external boundaries connected to 2D mesh | Suggest moving inflow boundary conditions outside of mesh. Revised Rev1 – Ok |
| ☐ | ☒ | ☐ | Boundary conditions everywhere flow could build up at the boundary of the 2D mesh | Because a buffer was applied to boundary, water is building up around edges. There appears to be areas where the water build up is connected to stream floodplains. Suggest adjusting perimeter to reduce amount trapped or adding boundary condition. Please review perimeter and correct as necessary. Added normal depth BC lines around perimeter with nominal slope matched to outflow normal depth. Rev1 – It appears that some floodplains will be connected along the border with HUC 12070204. We suggest adjusting the perimeter to follow the high ground between the HUC10s and adding additional inflows to the HUC 12070205 model as necessary. Border will remain to follow required 500 ft buffer of HUC boundary. Additional ponding will be removed in the mapping phase. |
| ☒ | ☐ | ☐ | Boundary conditions have a precipitation or hydrograph data and assigned at correct locations |  |
| ☒ | ☐ | ☐ | Downstream boundary consistent with ground slope / known WSEL |  |
| 2D Model Execution |  |  |  |  |
| ☒ | ☐ | ☐ | Appropriate 2D parameters and options used |  |
| Y | N | N/A | Item Description | Comments |
| ☐ | ☒ | ☐ | Hydrograph Output Interval and Mapping Output Interval set at a reasonable interval to capture peaks | It is our understanding that it was decided a mapping output of 1 hour and a hydrograph/detailed output of 15 min. Current settings are fine for internal review, but will ultimately need to be change for file size management. Updated and consistent with all models Rev1 – Ok |
| ☒ | ☐ | ☐ | Model runtime is reasonable |  |
| 2D Model Results |  |  |  |  |
| Computation Checks |  |  |  |  |
| ☐ | ☒ | ☐ | Model runs with no cell errors | Maximum iteration errors present. |
| ☒ | ☐ | ☐ | Volume balance is within tolerance (ΔV < 1% is ideal, however 2% to 3% can be acceptable for rain on mesh) |  |
| Full Checks |  |  |  |  |
| ☐ | ☒ | ☐ | Computational time step meets ideal Courant condition (diffusive wave ≤ 2, up to 5) | Time step for all events is currently 1 minute. Please correct. Updated and consistent with all models with 30 sec time step. Rev1 – Ok |
| ☒ | ☐ | ☐ | Model simulation is long enough to pass the entire hydrograph through the model (minimum of 12 hours past the peak, preferably 24 hours) |  |
| ☐ | ☒ | ☐ | Hydrographs at internal and external boundaries are smooth | 50yr output flow is higher than 100yr flow. There are also some dips present in the hydrographs. Updated model runs show correct results now Rev1 – Ok |
| ☒ | ☐ | ☐ | Contributing areas are contained within 2D mesh or accounted for with inflow hydrographs |  |
| ☐ | ☒ | ☐ | No Flow trapped at 2D Boundary | There are some areas where water is pooling around the perimeter. If they are not connected to the stream floodplain they can be left as is and will be removed during the mapping phase. Breaklines added; most will be resolved by added stream centerlines. Some hot spots remain at inflow BCs; however, these are typically within model “overlap” areas. Rev1 – It appears that some floodplains will be connected along the border with HUC 12070204. We suggest adjusting the perimeter to follow the high ground between the HUC10s and adding additional inflows to the HUC 12070205 model as necessary. Border will remain to follow required 500 ft buffer of HUC boundary. Additional ponding will be removed in the mapping phase. |
| Spot Checks |  |  |  |  |
| ☒ | ☐ | ☐ | WSELs appear reasonable within a single cell (check for visible spikes) |  |
| ☒ | ☐ | ☐ | Depth grids appear reasonable (no artificial dips or spikes) |  |
| ☐ | ☒ | ☐ | Flooding is connected downstream for all significant contributing areas (> 64 acres) |  |

| Y | N | N/A | Item Description | Comments |
| --- | --- | --- | --- | --- |
| ☐ | ☐ | ☐ | Velocity grids appear reasonable (no unusually high velocity areas / hot spots) | 100-year velocity grid was reviewed and looked mostly reasonable. There are some steep areas with velocity hots spots (See QAQC_Comment_HUC04.shp for examples). Consider refining areas with a breakline Breaklines added; most will be resolved by added stream centerlines. Some hot spots remain at inflow BCs; however, these are typically within model “overlap” areas. Rev1 – Ok |
| Flow Checks |  |  |  |  |
| ☐ | ☒ | ☐ | DSS files contain runs consistent with project | Suggest using dss file generated by project instead of dividing it into multiple DSS files for each event. Updated to include all events. Rev1 – Ok |
| ☒ | ☐ | ☐ | Peak modeled discharges comparable to gages, and regional regression equations | Peak modeled discharges appear to have been compared to stream gages, but not regional regression equations. Discharges also compared to FIS discharges which are regression-based or better. Rev1 – Noted |
| 2D Model Verification |  |  |  |  |
| Stream Gage Analysis |  |  |  |  |
| ☐ | ☐ | ☒ | Regression analysis performed at select locations in the watershed | No regression analysis included. Discharges also compared to FIS discharges which are regression-based or better. Rev1 – Noted |
| ☒ | ☐ | ☐ | Stream gage analysis performed on all gages in the watershed with sufficient data | Peak modeled discharges appear to have been compared to stream gages, but not regional regression equations. Discharges also compared to FIS discharges which are regression-based or better. Rev1 – Noted |
| Sensitivity Analysis |  |  |  |  |
| ☒ | ☐ | ☐ | Sensitivity analysis performed to evaluate which model parameters significantly impact results |  |
| ☒ | ☐ | ☐ | Based on the sensitivity analysis results did the results of the selected computation settings resulted in acceptable outcomes |  |
| Additional Comments |  |  |  |  |
| - Granger Dam: Water is currently only able to leave through the emergency floodway. Is there confirmation that is how a major flood event would be handled? From Aerial imagery it appears there a primary spillway. Dam discharges compared to USACE published maximum releases included in supplemental data. Since discharges are limited to 3,000 cfs, no modifications were deemed necessary. Rev01 - Noted - Suggest deleted old plans and files associated with old plans (plan .hdf files, .b files, .bco files, IC.O files). This will help with file size management. Model files cleaned up - Rev01 – There are still some old plan .hdf files. Please review and remove as necessary. - Old plan files removed - It appears some stream breaklines don’t match up with the terrain. Please review and fix as necessary. Updated stream breaklines - Rev01- Suggest editing boundary with HUC 1207020503, so that it is more perpendicular to the floodplain. - Boundary follows terrain for cleaner tie-in to 503 boundary. - Rev01 – At some stream and road intersections, there are cells cross the breakline. This may skew the amount of water that flows through the structure, see HUC03_QAQC.shp file for occurrences. This usually occurs where two breaklines cross each other and will likely need to be fixed by hand. = - Revised. |  |  |  |  |