| Submitted By: | Doucet & Associates | Date: |  |
| --- | --- | --- | --- |
| Reviewed By: | FTN Associates, Ltd. | Date: | 10/29/2021 |

Technical Review

| Y | N | N/A | Item Description | Comments |
| --- | --- | --- | --- | --- |
| Topology Check |  |  |  |  |
| Bare Earth DEM |  |  |  |  |
| ☒ | ☐ | ☐ | Terrain datasets included in ‘Terrain’ folder? |  |
| ☒ | ☐ | ☐ | Terrain projection file included? |  |
| ☒ | ☐ | ☐ | DEM has 10-ft cells and is in the correct state plane projection? |  |
| ☒ | ☐ | ☐ | Bare_Earth_DEM Included? |  |
| ☐ | ☒ | ☐ | DEMs Included? – GeoTIFF | DEM is included as a GeoTIFF, but has not been broken up. DEM should be divided into pieces so that each GeoTIFF is between 2 GB and 500MB. Doucet: DEM has been broken up into 3 as requested. |
| ☐ | ☒ | ☐ | DEM Tile Index Included? – .SHP | The DEM tile index is present, but should be updated once the DEM is broken up. Shapefile is also missing gridID field. Doucet: DEM tile index has been updated and now includes GridID field. |
| Hydrologically Corrected DEMs |  |  |  |  |
| ☐ | ☒ | ☐ | Hydrologically Corrected DEMs Included? – Esri grid/GeoTIFF/ASCII grid | hdem folder is empty. Doucet: HDEM has been provided. |
| ☐ | ☒ | ☐ | HDEM Tile Index? – .SHP | hdem folder is empty. Doucet: HDEM tile index has been provided. |
| ☐ | ☐ | ☒ | Any additional elevation data collected for use in the preparation of this Flood Risk Project? – Format as received |  |
| ☒ | ☐ | ☐ | DEM has 10-ft cells and is in the correct state plane projection? |  |
| ☐ | ☒ | ☐ | DEM vertical units are in feet NAVD88? | It appears no vertical coordinate system has been applied. Doucet: Vertical coordinate system of US Ft has been added. |
| ☒ | ☐ | ☐ | DEM is buffered at least 1000 ft past HUC 8 watershed boundary? |  |
| ☒ | ☐ | ☐ | DEM does not have sinks filled in? |  |
| ☒ | ☐ | ☐ | DEM has no missing (null) data, gaps, slivers, or shelving? |  |
| ☒ | ☐ | ☐ | DEM hillshade indicates no issues? |  |
| ☒ | ☐ | ☐ | Topographic abnormalities corrected? (no drops or rises from underlying data errors) |  |
| ☐ | ☐ | ☒ | Terrain modification documented? (post-processing (hydroconditioning, V-notches at structures, added/removed features)) |  |
| ☐ | ☐ | ☒ | Any contour information available? |  |

| Y | N | N/A | Item Description | Comments |
| --- | --- | --- | --- | --- |
| Metadata Check |  |  |  |  |
| ☒ | ☐ | ☐ | Is the metadata file included with the submittal? |  |
| ☐ | ☐ | ☒ | Does the metadata pass validation checks? Has the documentation been provided? | Since there is no case number MIP validation was not run. Metadata was compared to past examples and appears to match. Doucet: No response required. |
| ☐ | ☒ | ☐ | Is the metadata correct in structure and information? | Metadate does not include terrain lidar sources. Doucet: Terrain sources have been added to metadata. |
| ☒ | ☐ | ☐ | Terrain Metadata Included? – .XML |  |
| ☒ | ☐ | ☐ | Are all required spatial data sets and tables included? |  |
| ☒ | ☐ | ☐ | Do all the files have the same and correct state plane projection? |  |
| Stream Centerline and Watershed Check |  |  |  |  |
| ☒ | ☐ | ☐ | Stream centerlines are delineated to 1 sq mi? |  |
| ☒ | ☐ | ☐ | Stream centerlines are smoothed, generalized, and follow terrain? | See general comments below. Doucet: Preliminary stream centerlines for CNMS development have been revised and will be included in draft CNMS submittal. |
| ☐ | ☐ | ☒ | Stream centerlines are cut off at the caprock/first playa? |  |
| ☐ | ☒ | ☐ | Watershed boundary is cut / divided at correct high points locations? | While the subbasin file is not essential to the terrain submittal, please include it for review. Doucet: Subbasin file will be provided in a separate submittal. |
| ☐ | ☒ | ☐ | Stream Centerline and Watershed is in the correct state plane projection? | Shapefile not included. Doucet: Subbasin file will be provided in a separate submittal. |
| Completeness Check (Final, General, Spatial_Files, Supplemental_Data) |  |  |  |  |
| ☐ | ☒ | ☐ | Are the folders structured according to Guidance? | Terrain folder is missing the Spatial_Files and Supplemental Data folders. Suggest moving the project narrative to the general folder. Doucet: Revised as requested. |
| ☐ | ☒ | ☐ | Are all required spatial data sets and tables included? | S_Submittal_Info and L_Source_Cit files are not included in the submittal. Doucet: Added as requested. |
| ☐ | ☒ | ☐ | Do files meet MIP upload standards? | See comments above. Doucet: No response required. |
| ☒ | ☐ | ☐ | Has documentation or BLE Report provided? | See comments in report. Doucet: Narrative revised as requested. |

General Comments:

- There are some reoccurring issues found with the stream centerline:

  - Steam centerline often jumps from the main channel near roadways.

  - Stream centerline upstream of dams, do not follow the channel.

  - Small polyline segments (generally span 1 to 2 cells) should be removed.

- We acknowledge that the stream centerline is not essential to the terrain submittal and many of the routing issues with be addressed during hydraulics, but an accurate stream centerline will be required for the final submittal.

Doucet: General comments on CNMS line work will be addressed in preliminary CNMS Round 1 QC submittal.