# Texas 2D BLE Analysis Audit Checklist

# Project Information

| HUC-8 Watershed Name & ID: | Lower Brazos- San Gabriel 12070205 |
| --- | --- |
| HUC-10 Watershed Name & ID: | 501, 502, 503, 504, 505 |
| Modeler: | FNI |
| Reviewer: | Halff |

This audit process includes only high-level review for items identified in the Detailed Review for another HUC for the same consultant. The consultant is responsible for performing a detailed review internally for base data, hydrology, hydraulics, and modeling results.

## Audit 1: Terrain, Modeling, Preliminary Mapping Review (QC Items 1-7)

| Modeler Initials: |  | Reviewer Initials: | SD |
| --- | --- | --- | --- |
| Submittal Date: |  | Review Submitted Date: | 07/11/22 |
| Review Response Date: |  | Backcheck Complete Date: |  |

## Audit 2: Completeness Review (QC Item 8)

| Modeler Initials: |  | Reviewer Initials: | AC/WF/JM |
| --- | --- | --- | --- |
| Submittal Date: |  | Review Submitted Date: | 10/20/2022 |
| Review Response Date: |  | Backcheck Complete Date: | 5/12/23; 7/3/23; 8/2/23; 8/24/23 |

(Modeler to populate with an “X” for items included in the QC submittal)

| Level of Study |  |  | Flood Events |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 2D BLE |  |  | 10% ACE (10-year) |  | 0.2% ACE (500-year) |
|  | Enhanced BLE |  |  | 4% ACE (25-year) |  | 1% plus |
|  |  |  |  | 2% ACE (50-year) |  | 1% minus |
|  |  |  |  | 1% ACE (100-year) |  | Observed Rainfall |

Instructions

Each section includes instructions to the modeler and reviewer regarding their responsibilities in the completion of this checklist. In the Methodologies / Data Sources tables, grey italic text is added as an example response. Additional sample responses are located in the Texas 2D BLE Guidelines Quick Guide.

## Submittal Checklist

Data submitted for each milestone should be carried through each subsequent milestone. All geospatial layers and models must have same defined projection system as terrain. Modeler to populate with an “X” for items included in the QC submittal.

| Base Map Data (QC items 1-6) |  |
| --- | --- |
|  | Location Map (PDF): HUC-8 and HUC-10 boundaries and/or model domain boundary if different from HUC-10s, stream gages, dams (if applicable), mainstem streams (for reference), background map |
|  | GIS Map (MXD or Map Package) with the following data included but not limited to: Stream centerlines, model breaklines, rainfall data, WSEL and Depth grids (10, 100, 500-yr), adjacent HUC mapping (available on the BLE viewer or currently studying), 2D connections for inflows/outflows, gages or points of flow comparisons (BFEs, highwater marks, etc.), profile lines, boundary condition lines, refinement regions, model domain boundary, model mesh, DFIRM mapping panels, base map data (roads, aerials, etc.), soils data, land use data, terrain |
| Technical Report / Documentation (QC items 1-6) |  |
|  | Technical Report / Project Notebook: assumptions, special considerations, and modeling concerns |
|  | Detailed explanation and support data of variances in methodologies and approaches from recommended guidelines (Should be included in Technical Report / Project Notebook) |
|  | Internal QA/QC Documentation: comments and responses |
|  | Flow comparison tables within the technical report |
| Hydrology (QC items 1-6) |  |
|  | Hydrologic Model (HEC-HMS) model with all associated DSS files for 7 required storm events |
|  | Hydrologic Data and Parameter Spreadsheets (rainfall calculations, curve number table, gage locations, gage analysis and flow validation) |
|  | Hydrologic modeling notebook including assumptions, notes, and special conditions (if not provided in report) |
| Hydraulics (QC items 1-6) |  |
|  | Hydraulic Model (HEC-RAS) with all associated files and 7 required storm events (remove non-final files)d |
|  | Hydraulic Data and Parameter Spreadsheets (Manning’s n-value table, infiltration calculations support data-Soils, Land Use, Curve Number table, other calculations/data used in model) |
|  | Manning’s n-value table |
|  | Precipitation data correctly linked to hydrologic model or hard coded from provided spreadsheet |
|  | Hydraulic modeling notebook including assumptions, notes, and special conditions |
| Preliminary (Raw) Mapping (QC item 7) |  |
|  | Preliminary 10%, 1%, 0.2% ACE mapping results (raster grids from HEC-RAS) |
|  | Modeler to complete topology validation as outlines in the guidelines |
|  | Modeler to verify mapping meets deliverable requirements as outlined in guidelines |
| Completeness Review (QC item 8) See Texas 2D BLE Guidelines Chapter 4 – Deliverables |  |
|  | Technical BLE Report |
|  | Final cleaned hydrologic model |
|  | Final cleaned hydraulic model |
|  | Flood risk database |
|  | CMNS updates |
|  | HAZUS results |
|  | Additional supplemental data |

# 1 – Model Terrain

Methodologies / Data Sources (Modeler to populate)

| Terrain Source(s): |  |
| --- | --- |
| Data Gaps: |  |
| Source Resolution: |  |
| Model Terrain Resolution: |  |
| Terrain Horizontal Projection: |  |
| Terrain Vertical Projection: |  |
| Z Values: |  |
| Terrain Buffer: |  |
| Other: |  |

## Technical Review (Reviewer to provide comments)

Reviewer is to utilize detailed QA/QC to identify general review items for this audit.

## Review Comments

- Terrain is modified with road crossings and other line. Appears correct.

# 2 – Hydrology

## Methodologies / Data Sources (Modeler to populate)

| Software: |  |
| --- | --- |
| Rainfall Source: |  |
| Rainfall Duration: |  |
| Depth Selection: |  |
| Rainfall Temporal Distribution: |  |
| Areal Reduction: |  |
| Loss Methodology: |  |
| Unit Hydrograph Transform Method: |  |
| Inflow Hydrograph Source: |  |
| Other: |  |

## Statistical Data (Modeler to populate)

| Stream Gage Name & ID: |  |
| --- | --- |
| Stream Gage Name & ID: |  |
| Stream Gage Name & ID: |  |

## Technical Review (Reviewer to provide comments)

Reviewer is to utilize detailed QA/QC to identify general review items for this audit.

## Review Comments

- BC line/ SA-2D connection line for 501 is missing some flow on the left side. Consider extending it.

  - Agree. Connection line extended.

No additional comments.

# 3 – Hydraulic Geometry

## Methodologies / Data Sources (Modeler to populate)

| Software: |  |
| --- | --- |
| Loss Methodology: |  |
| Soils Source: |  |
| Land Cover Source: |  |
| Manning's Roughness Source: |  |
| Nominal Mesh Cell Sizes: |  |
| Other: |  |

## Technical Review (Reviewer to provide comments)

Reviewer is to utilize detailed QA/QC to identify general review items for this audit.

## Review Comments

- Include descriptions in each model referencing project and connections to other models.

  - Descriptions added to models.

No additional comments.

- Some of the reference lines drawn at gage locations (e.g. USGS_N Fk SG_08104700 in model 501, USGS_S Fk SG_08104900 in model 502, etc.) are not enforced to the mesh. Any results measurements along such lines may give inaccurate results.

  - Breaklines added to reference lines to enforce mesh.

No additional comments.

- Comment for all Models: Once Final, please remove any unnecessary data (Geometry Files, Flow Files, Plans Files, .hdf files, etc.)

  - Agree. Resolved.

No additional comments.

- Projection file is missing in the “Simulation” folder.

  - Projection file added.

No additional comments.

# 4 – Hydraulic Boundary Conditions

## Technical Review (Reviewer to provide comments)

Reviewer is to utilize detailed QA/QC to identify general review items for this audit.

## Review Comments

- Comment for all models: the u/s models have different BC/reference lines lengths or locations than Model 505. Need to adjust.

  - Updated.

No additional comments.

- Provide documentation on how stream gage frequency values were calculated.

  - Documentation will be provided in engineering report.

No documentation is provided in the report. Table 9 shows that discharge values were extracted from USGS StreamStats. It is recommended to run Bulletin 17C analysis (instead of using USGS StreaStats) for gages to use local knowledge and incorporate station specific parameters.

  - Stream gage frequencies were obtained off of the USGS Streamstats website and the Williamson County 2019 FIS report supplemented with additional calibration locations.

Provide documentation within report. Response to report QAQC comments. Also provide gage analysis files under hydrologic model folder.

Halff Comment 8/7/20230: See report for unresolved comments.

# 5 – Hydraulic Model Execution

## Methodologies / Data Sources (Modeler to populate)

| Unsteady Plan Names |  |  |  |
| --- | --- | --- | --- |
| 10% ACE (10-year): |  | 0.2% ACE (500-year): |  |
| 4% ACE (25-year): |  | 1% plus: |  |
| 2% ACE (50-year): |  | 1% minus: |  |
| 1% ACE (100-year): |  |  |  |
| Other: |  |  |  |

## Statistical Data (Modeler to populate)

| Time Step: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Courant Value: |  |  | Simulation Length: |  | hours |
| Computation Interval: |  | seconds | Model Runtime: |  | hours |
| Hydrograph Output Interval: |  | minutes | Mapping Output Interval: |  | minutes |

## Technical Review (Reviewer to provide comments)

Reviewer is to utilize detailed QA/QC to identify general review items for this audit.

## Review Comments

- Consider keeping Mapping output interval and Hydrograph output interval the same to avoid any result discrepancies.

  - Updated.

No additional comments.

# 6 – Hydraulic Model Results

## Methodologies / Data Sources (Modeler to populate)

| Flow Validation/Gage Analysis |  |
| --- | --- |
| Software: |  |
| Methodology |  |
| Removal of Outliers and Justification |  |
| Validation Notes (describe parameter adjustments): |  |
| Other: |  |

## Technical Review (Reviewer to provide comments)

Reviewer is to utilize detailed QA/QC to identify general review items for this audit.

## Review Comments

- Check if hydrology method (calculating precipitation input for HEC-RAS 2D) used for this HUC is consistent with other HUCs. Justify if not. Consider providing similar tables and results for all HUCs.

  - Updated.

No additional comments.

- Check Table 4 (precipitation totals) in the report. They looked incorrect and do not match totals in the model.

  - Table 4 updated.

No additional comments.

- The report mentioned that gages with more than 20 years of data were used for calibration, but Table 9 showed few gages with 8/15 years of record. Check and remove them if not used.

  - Report was updated.

No additional comments.

- Provide justification on large differences in flows compared to gage analysis.

  - Justification provided in report.

Need additional documentation and/or justification on why current parameters were chosen with the sensitivity analysis and still there are large differences at other gage locations.

Response: Further explanation has been documented within the report.

No additional comments.

- No comparison of BLE results to FEMA effective SFHA/BFEs provided or explained in the report. Please add.

  - Updated.

There is a mention that the BLE results compare reasonably with FEMA effective SFHA. But, no additional comparison (table/figure) provided in the report.

Response: Comparison to USGS gage data and FIS reports were further documented. Since this study uses Atlas 14 rainfall data, it is not expected to be comparable to FEMA effective SFHA.

Not addressed. Provide comparison table to FEMA effective.

Response: Comparison points included in Appendix B. Table updated to clarify comparison sources.

- Some NRCS dams (particularly Upper Brushy Creek dams in urban areas of Round Rock, Cedar Park) have leaky cells for their embankment. Need to fix.

  - Breaklines at the dams were adjusted to remove leaky cells

No additional comments.

# 7 – Preliminary (Raw) Mapping Results

## Technical Review (Reviewer to confirm with Pass/Fail)

| Hydraulic Model |  |
| --- | --- |
|  | HEC-RAS model with results, pertinent data and required storm events |
| Preliminary Mapping 1% ACE |  |
|  | Preliminary 10%, 1%, 0.2% ACE mapping results (raster grids from HEC-RAS) to meet FEMA mapping requirements |
|  | Boundary interfaces align matching or identified as AOMI |

## Review Comments

- Raster grids /preliminary mapping is not provided for all models.

  - Rasters provided.

No additional comments.

- A quick check at model boundary (e.g. for 100yr near 504/505 boundary) shows more than 0.5’ Difference. All tie-ins for 10, 100, and 500-year should be within 0.5’. Check and revise models to meet this criteria.

  - Tie ins were further calibrated.

No additional comments.

Halff Comment 8/7/20230: No Further comment

3. Please provide raw raster grids from HEC-RAS

Response: Raw raster grids can be found in the HEC-RAS results folder and in included Map Package submittal.

No additional comments.

4. Please respond to QC comments shapefile. Previous QC file provided with backcheck and new comments.

Response: QC shapefile with responses has been included.

No additional comments.

Halff Comment 8/7/2023: Additional QC polygons added to QC shapefile. There are a few places where BLE_WSE_1PCT exist where BLE_WSE0_2PCT Grid does not.

Response2: Resolved. QC shapefile with responses has been included.

Halff Comment 8/23/2023: No further comments

# 8 – Completeness Review

## Technical Review (Reviewer to confirm with Pass/Fail)

| File Structure and Naming |  |
| --- | --- |
|  | Deliverables provided in TWDB/FEMA required file folder structure and naming convention |
|  | Data missing (if any) documented in report and ReadMe file within the required folder |
| Report |  |
|  | Technical 2D BLE report documents all assumptions, special considerations, etc. |
|  | Flow validation and difference discussed in report |
|  | Flow comparisons discussed in report |
| Hydrologic Model |  |
|  | HEC-HMS model with results, pertinent data and required storm events |
| Hydraulic Model |  |
|  | HEC-RAS model with results, pertinent data and required storm events |
| Flood Risk Database |  |
|  | All features fully populated per current FEMA guidelines |
|  | Mapping consistent between grids to polygons for all three required mapped events |
|  | 10yr smaller than 100yr smaller than 500yr |
|  | Holes filled in polygon, depth and WSE grids |
|  | Boundary interfaces align matching or identified as AOMI |
| CNMS |  |
|  | Updated correctly with new centerlines |
|  | A1-A5 checks complete |
|  | CNMS Post- Processing Complete |
| HAZUS |  |
|  | Run per FEMA guidance |

## Review Comments

- Feature classes are not using appropriate citations as listed within the L_SOURCE_CIT table. Please revise.

Response: Revised.

Halff Comment 8/7/2023: No further comments

- Update VERSION_ID to be consistent throughout all feature classes and tables.

Response: Revised.

There are still different VERSION_IDs throughout database.

Response: Revised.

No additional comments.

Halff Comment 8/7/2023: S_Pol_Ar still has different VERSION_ID than the rest of the features. These should be consistent between all feature classes within the EBFE feature dataset. Otherwise, all other features look good.

Response: Revised.

Halff Comment 8/23/2023: No further comments

- S_HUC_AR: Feature class is empty. Please populate.

Response: Revised.

Has been added. Is missing Case number

Response2: case number populated.

Shows as NP. Please populate case number

Response3: case number populated.

Halff Comment 8/7/2023: Case number for all HUCs that are part of Task 01 is “22-06-0047S”. Please update field with this Case number.

Response: Revised.

Halff Comment 8/23/2023: No further comments

- S_POL_AR:

- POL_AR_ID should be unique sequential numbers.

- SOURCE_CIT: should be filled out like others "BASE1" or similar, as this is a reference layer and not created from the study.

- L_COM_NFO_ID is usually populated with CID.

- File does not contain appropriate communities for entire HUC8 limits. Please populate and clip to HUC8 boundary limits.

Response: Revised.

No additional comments.

Halff Comment 8/7/2023: SOURCE_CIT should not be populated with “FIRM1” FIRM source type is used for features extracted from the existing FIRM. Using a BASE# is expected for these feature class. Please reference the FIRM Database Technical Reference below. Otherwise, everything else looks correct.

Response 8/18/23: Agree, Source_CIT updated to BASE1.

Halff Comment 8/23/2023: Changing to a BASE# is correct. However, the citation being referenced in the L_Source_Cit table is incorrect. Political area boundaries do not come from the “National Flood Hazard Layer”. This needs to be where you originally obtained the data for this feature class. Typically political areas come from TxDOT or a similar source. Keep this field as BASE1 and please update the L_Source_Cit with the correct Citation, Publisher, Title, ect…

Response: BASE1 updated to cite TXDOT in the L_Source_CIT table.

- WTR_AR:

- SOURCE_CIT: Should be a BASE layer as these lakes were not created from this study. Please update this field and the information within L_SOURCE_CIT table and metadata.

Response: Revised.

OK

Halff Comment 8/7/2023: BASE2 only references the Watershed boundaries currently. Although it is true that NHD waterbodies can be downloaded from the same location as HUC boundaries you will need to include that within the citation. It is common convention to create a separate BASE citation in the L_Source_Cit table for watershed boundaries and NHD waterbodies. Please update accordingly.

Response: Revised.

Halff Comment 8/23/2023: No further comments

- DTL_STUD_AR:

- Fields do not match correct database schema. Please take a look and revise.

- You should be able to merge the floodplains together as long as they are within the same FIRM panel. This will shrink the number of entries.

- Populate CASE_NO field.

Response: Revised.

It only has 2 entries now. It should be divided by panels. Case no is not populated. Version ID different than in others (HUC_AR). Extents do not cover BLE mapping.

Response2: Revised to cover mapping.

Halff Comment 8/7/2023: Polygons need to be split by FIRM panel boundaries. Polygons within each FIRM panel needs to be merged into a single feature for each FIRM panel. Each feature will need to have the correct SOURCE_CIT that correlates to the FIRM panel in the L_Source_Cit table.

Response3: Revised, features split by FIRM panel boundary and Source_Cit updated.

Halff Comment 8/23/2023: No further comments

- DTL_STUD_LN:

- Fields do not match correct database schema. Please take a look and revise.

- Populate CASE_NO field.

Response: Revised.

DTL_LN_ID is not a unique number, Will most likely need to reclip/reevaluate after changes to DTL_STUD_AR have been made. Populate CASE_NO

Response: Revised.

Shows as NP. Please populate case number

Response: case number populated.

Halff Comment 8/7/2023: Lines need to be split by FIRM panel boundary. Lines within each FIRM panel need to be merged into a single feature for each FIRM panel. Each feature will need to have the correct SOURCE_CIT that correlates to the FIRM panel in the L_Source_Cit table.

Response4: Revised, features split by FIRM panel boundary and Source_Cit updated.

Halff Comment 8/23/2023: No further comments

- TENPCT_FP: Please refer to the QC shapefile provided.

- Make ZONE_SUBTY, VEL_UNIT, AR_REVERT, and AR_SUBTRV empty string “”.

- V_DATUM, LEN_UNIT, VEL_UNIT are only populated if STATIC_BFE, DEPTH, or VELOCITY are populated. Make empty string “”.

- Remove ORIG_FID field.

- Domain values are not being used. Please revise.

Response: Revised.

OK. Verify the domains are still used after schema update.

Response: Verified.

No additional comments.

Halff Comment 8/7/2023: No further comments

- FLD_HAZ_AR: Terrain not provided. Unable to validate against raw mapping and Tie-ins.

- V_DATUM, LEN_UNIT, VEL_UNIT are only populated if STATIC_BFE, DEPTH, or VELOCITY are populated. Make empty string “”.

- Make ZONE_SUBTY, VEL_UNIT, AR_REVERT, and AR_SUBTRV empty string “” for Zone A polygons.

- Make VEL_UNIT, AR_REVERT, and AR_SUBTRV empty string “”.

- Domain values are not being used. Please revise.

- Fields do not match correct database schema. Please take a look and revise.

Response: Revised.

OK. Verify the domains are still used after schema update.

Response: Verified.

No additional comments.

Halff Comment 8/7/2023: No further comments

- BFE:

- Fields do not match correct database schema. Please take a look and revise.

- Domain values are not being used for BFE_LN_TYP or LEN_UNIT. Please revise.

Response: Revised.

Check Domain values. Unmapped is being populated instead of using “Mapped” “Not Mapped” domains. Verify these are used when updating with new schema.

Response: Revised.

No additional comments.

Halff Comment 8/7/2023: No further comments

- S_AOMI_PT:

- CID: Most of the cells are blank. Please populate.

- Domain values are not being used for AOMI_CLASS, AOMI_TYPE, AOMI_CAT.

- Populate CASE_NO field.

- SOURCE_CIT: These should not be STUDY1 if they are reference files. Please update all applicable files across submittal (tables, metadata, etc.).

- Transportation is spelled incorrectly within all entries for AOMI_SRCE.

- File should not be multipoint features.

There are still AOMI_PTs in the database. Need to update to new schema.

Removed as part of new schema. No additional comments.

Halff Comment 8/7/2023: No further comments

- S_AOMI_AR: Feature class is empty.

Response: S_AOMI_AR is a polygon feature class representing areas mitigation interest that should be noted. No such areas were found within the model areas so it was left blank.

AOMI_PTs will need to be added to AOMI_AR

Response: Added.

Add Aomi areas around the HUC8 boundary where floodplain spills outside the HUC limits. See example below:

Response: Added.

Halff Comment 8/7/2023: Polygons defining floodplain extending past HUC 8 boundaries are not on the HUC boundary. Please re-draw polygons on the HUC boundary where S_FLD_HAZ_AR spills outside the HUC limits.

Response: Revised. Added.

Halff Comment 8/23/2023: No further comments

- S_FRAC_AR:

- Fields do not match correct database schema. Please take a look and revise.

- Domain values are not being used. Please revise.

- SOURCE_CIT should be populated with REF1 as this is a reference file and HAZUS1 is not an appropriate citation by FEMA standards.

- ARV_BG_TOT and ARV_CN_TOT are not showing values in the thousands. Please multiply entries by 1000.

Response: Revised.

Need to update to new schema.

Response2: File follows schema provided on 4/20/23

Populate Case no.

Response3: case number populated.

Halff Comment 8/7/2023: Study1 is not the appropriate source from the L_Source_Cit table. These data come from HAZUS and should be cited as such. I recommend changing Study2 to REF1 and populating the Source_Cit  for S_FRAC_AR as REF1.

Response4: Updated. Hazus cited as REF2.

Halff Comment 8/23/2023: See new comment for L_Source_Cit table below. No additional comments.

Response5: Updated.

Halff Comment 8/7/2023:

- WTR_LN:

- Lines in WTR_LN should be from your BLE study. The lines currently in this feature class look identical to the current effective CNMS lines provided from the Region. Please confirm these lines were not copied from S_Studies_Ln feature class and were generated from your models.

Response: WTR_LN has been revised so proper model stream lines are reflected in feature.

Halff Comment 8/23/2023: No further comments

Halff Comment 8/7/2023:

- L_Source_Cit Table: The incorrect HUC name is being used in the Title field. Please change upper Sabine to San Gabriel.

Response: Updated.

Halff Comment 8/23/2023: STUDY1 was correct the way it was originally. Please change REF1 back to  STUDY1 and change REF2 to REF1. Go back through any features that were using these and update accordingly.

Response2: Revised.

New Folder Structure Comments:

- Hydraulic Models: For final submittal Input, Landcover, Output and Terrain can all go in each modeling area folder. Clean up folders to remove all blank results.

- CNMS files will be reviewed with separate CNMS submittal.

Preliminary comments provided to FNI by email on 10/7/2022:

San Gabriel

- RAS model BLE_LBSG_501 is unable to load and export results.

Response: Results were left in model and output file locations have been documented.

- RAS model BLE_LBSG_502 results are loading incorrectly.

Response: Revised.

No additional comments

Response: Revised.

No additional comments

- BLE_WSEL01PCT has WSEL Busts with almost 4 feet difference at multiple locations. Please verify against raw data.

Response: Revised.

No additional comments

- A lot pf places are missing mapping along the centerlines. Please confirm that no raw mapping is available. Few example locations below:

- S_AOMI_AR: Feature class is blank.

Response: S_AOMI_AR is a polygon feature class representing areas mitigation interest that should be noted. No such areas were found within the model areas so it was left blank.

See aOMI AR comment above.

- Fields within feature classes do not match schema. Please verify you have the latest schema to load.

Response: Revised.

No additional comments

Halff Comment 8/7/2023: No further comments

- S_HUC_AR: Feature class is blank.

Response: Revised.

v No additional comments

Halff Comment 8/7/2023: No further comments

- S_POL_AR: Feature class not covering the HUC extents.

Response: Revised.

No additional comments

Halff Comment 8/7/2023: No further comments

- Folders are missing Hydrology_DCS.gdb, Hydraulics_DCS.gdb and TerrainDCS.gdb from the folder structure.

Response: These folders are not required in the current folder submittal structure.

No additional comments

QA/QC Checklist Approval

This 2D Base Level Engineering Analysis QA/QC Review is in compliance with the contract requirements and all task checkpoints are complete.  The independent QC Team has reviewed the analysis, presented review comments to the Production Team Task Leader, and discussed any problems or issues.  The Task Leader, QC Manager, QC Reviewer, and Project Manager have signed the QA/QC Checklist to confirm that all comments have been received, addressed, and documented appropriately.

| QA/QC Team – [TWDB or company name] |  |  |  | Production Team – [add company name] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| QC Manager |  | Date |  | Task Lead |  | Date |
| QC Reviewer |  | Date |  | Project Manager |  | Date |