# Texas 2D BLE Analysis QA/QC Checklist

# Project Information

| HUC-8 Watershed Name & ID: | Middle Colorado: 12090106 |
| --- | --- |
| HUC-10 Watershed Name & ID: | Mustang Creek - Colorado River (1209010601) - Model 0601_A1 Mustang Creek - Colorado River (1209010601) - Model 0601_A2 Bull Creek – Colorado River (1209010602) – Model 0602_A3 Bull Creek – Colorado River (1209010602) – Model 0602_A4 Home Creek (1209010603) – Model 0603_A6 Cow Creek – Colorado River (1209010604) – Model 0608_A7 Clear Creek – Colorado River (1209010605) – Model 0605_A8 San Saba River – Colorado River (1209010606) – Model 0605_A9 |
| Modeler: | Atkins |
| Reviewer: | Halff Associates |

## Terrain and Modeling Review (QC Items 1-6)

| Modeler Initials: | SN / JG | Reviewer Initials: | SD |
| --- | --- | --- | --- |
| Submittal Date: | June 24, 2022 | Review Submitted Date: | 03/07/2023 |
| Review Response Date: | April 17, 2023 | Backcheck Complete Date: | 5/10/2023; 8/10/23 |

## Mapping Review (QC Item 7)

| Modeler Initials: | PC | Reviewer Initials: | NB |
| --- | --- | --- | --- |
| Submittal Date: | July 08, 2022 | Review Submitted Date: | 8/10/2023; 10/13/23; 12/07/2023; 5/23/2024, 6/07/2024 |
| Review Response Date: |  | Backcheck Complete Date: |  |

## Completeness Review (QC Item 8)

| Modeler Initials: |  | Reviewer Initials: | NB |
| --- | --- | --- | --- |
| Submittal Date: |  | Review Submitted Date: | 8/10/2023; 10/13/23; 12/07/2023, 6/07/2024 |
| Review Response Date: |  | Backcheck Complete Date: |  |

(Modeler to populate with an “X” for items included in the QC submittal)

| Level of Study |  |  | Flood Events |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| X | 2D BLE |  | X | 10% ACE (10-year) | X | 0.2% ACE (500-year) |
|  | Enhanced BLE |  | X | 4% ACE (25-year) | X | 1% plus |
|  |  |  | X | 2% ACE (50-year) | X | 1% minus |
|  |  |  | X | 1% ACE (100-year) |  | Observed Rainfall |

Instructions

Each section includes instructions to the modeler and reviewer regarding their responsibilities in the completion of this checklist. In the Methodologies / Data Sources tables, grey italic text is added as an example response. Additional sample responses are located in the Texas 2D BLE Guidelines Quick Guide.

Reviewer to indicate if Technical Review items generally comply using a “Pass”. If the technical review observes an issue of noncompliance or an item that requires additional attention, an “Fail” should be utilized.

## Submittal Checklist

Data submitted for each milestone should be carried through each subsequent milestone. All geospatial layers and models must have same defined projection system as terrain. Modeler to populate with an “X” for items included in the QC submittal.

| Base Map Data (QC items 1-6) |  |
| --- | --- |
| X | Location Map (PDF): HUC-8 and HUC-10 boundaries and/or model domain boundary if different from HUC-10s, stream gages, dams (if applicable), mainstem streams (for reference), background map |
| X (RAS Mapper) | GIS Map (MXD or Map Package) with the following data included but not limited to: Stream centerlines, model breaklines, rainfall data, WSEL and Depth grids (10, 100, 500-yr), adjacent HUC mapping (available on the BLE viewer or currently studying), 2D connections for inflows/outflows, gages or points of flow comparisons (BFEs, highwater marks, etc.), profile lines, boundary condition lines, refinement regions, model domain boundary, model mesh, DFIRM mapping panels, base map data (roads, aerials, etc.), soils data, land use data, terrain |
| Technical Report / Documentation (QC items 1-6) |  |
| X | Technical Report / Project Notebook: assumptions, special considerations, and modeling concerns |
| X | Detailed explanation and support data of variances in methodologies and approaches from recommended guidelines (Should be included in Technical Report / Project Notebook) |
| X | Internal QA/QC Documentation: comments and responses |
| X | Flow comparison tables within the technical report |
| Hydrology (QC items 1-6) |  |
|  | Hydrologic Model (HEC-HMS) model with all associated DSS files for 7 required storm events |
| X | Hydrologic Data and Parameter Spreadsheets (rainfall calculations, curve number table, gage locations, gage analysis and flow validation) |
| X (Report/ Results spreadsheet) | Hydrologic modeling notebook including assumptions, notes, and special conditions (if not provided in report) |
| Hydraulics (QC items 1-6) |  |
| X | Hydraulic Model (HEC-RAS) with all associated files and 7 required storm events (remove non-final files) |
| X | Hydraulic Data and Parameter Spreadsheets (Manning’s n-value table, infiltration calculations support data-Soils, Land Use, Curve Number table, other calculations/data used in model) |
| X | Manning’s n-value table |
| X | Precipitation data correctly linked to hydrologic model or hard coded from provided spreadsheet |
| X | Hydraulic modeling notebook including assumptions, notes, and special conditions |
| Mapping (QC item 7) |  |
|  | Flood risk database |
|  | Modeler to complete topology validation as outlines in the guidelines |
|  | Modeler to verify mapping meets deliverable requirements as outlined in guidelines |
| Completeness Review (QC item 8) See Texas 2D BLE Guidelines Chapter 4 – Deliverables |  |
|  | Technical BLE Report |
|  | Final cleaned hydrologic model |
|  | Final cleaned hydraulic model |
|  | Flood risk database |
|  | CMNS updates |
|  | HAZUS results |
|  | Additional supplemental data |

# 1 – Model Terrain

Methodologies / Data Sources (Modeler to populate)

| Terrain Source(s): | Please see technical memo for more detail: Texas West Central LiDAR (2018), Brown County LiDAR (2019), Hurricane LiDAR (2019), Lower Colorado San Bernard LiDAR (2018), and Lampasas LiDAR (2014) |
| --- | --- |
| Data Gaps: | None |
| Source Resolution: | Cell size of 1 x 1 meter |
| Model Terrain Resolution: | Cell size of 5 x 5 feet |
| Terrain Horizontal Projection: | North American Datum (NAD) 1983 (2011) Texas State Plane Zone 3 (Central) |
| Terrain Vertical Projection: | NAVD88 |
| Z Values: | Units of feet |
| Terrain Buffer: | 2,500 feet |
| Other: |  |

## Technical Review (Reviewer to confirm with Pass/Fail)

| General Review |  |
| --- | --- |
|  | LiDAR source matches the latest available data at time of project initiation |
|  | Large reservoir or lake extents included in model domain |
|  | Check visible seams, gaps or holes of LiDAR sources (if more than one source was used) |

## Review Comments

- There are two terrains in the model (one with modifications).  Remove the one that is not used. (applicable for all models)

Atkins Response (03/09/2023): Both terrains are used and serve different purposes. First terrain (Terrain.hdf) is created based on LIDAR DEM. The second Hydroenforced terrain (Terrain.HydroEnforced.hdf) is a clone of first terrain and includes terrain modifications. Since Hydro enforced terrain allows water to flow through culvert/ bridge openings, it is required for modeling. The Hydro enforced terrain can not be used for mapping as it would show all roadways overtopped at culvert/ bridge crossing at terrain modifications. The first terrain includes actual roadway elevations and would correctly map the FP at the roadway crossings depending on the computed water surface elevations. So, both terrains are required and can not be deleted from the model.

The “hydroenforced” terrain is used for modeling as well as mapping in RAS Mapper (see screenshot below) which is different than the response above. Confirm and document how these terrains are used in RAS and for mapping. Both terrains can be kept in the model if used as responded above but clearly document in the model inventory.

Atkins Response (06/03/2023): Approach described in the 03/09 response is correct and we will add documentation in the report (See the end of Section 1.1.1).  Additional details are added to model inventory and screenshot is included for reference.

The hydro enforced terrain contained the terrain modifications for bridge/culvert crossing locations at roadways and was therefore used for modeling. The terrain without modifications will be used for mapping. The mapping for the project has not been developed yet and will be developed after modeling is approved. The raw grids exported using terrain.hdf (not hydroenforced) were included in the submittal.

Raw exported 0.2% AEP grid shown below in the left figure does not show flow overtopping the roadway. Figure on the right shows hydro enforced terrain. If the raw mapping was based on the hydro enforced terrain then the mapping would be connected over the roadway, as there is a terrain modification which cut culvert opening through the roadway.

Raw mapping does not show roadway topping. 		Hydro enforced terrain with terrain modification

8/10/23: No additional comments.

# 2 – Hydrology

## Methodologies / Data Sources (Modeler to populate)

| Software: | HEC-RAS 6.2 |
| --- | --- |
| Rainfall Source: | NOAA Atlas 14 |
| Rainfall Duration: | 24 hours |
| Depth Selection: | Domain Average |
| Rainfall Temporal Distribution: | NRCS |
| Areal Reduction: | None |
| Loss Methodology: | Spatially varied CN |
| Unit Hydrograph Transform Method: | N/A |
| Inflow Hydrograph Source: | Inflow Hydrograph for O.H. Ivie Reservoir was developed using InFRM frequency peak flows at gage 08126380- Colorado River near Ballinger TX (please refer to the hydraulic technical memo) |
| Other: Precipitation spreadsheets are included with the submittal |  |

## Statistical Data (Modeler to populate)

| Stream Gage Name & ID: | Colorado River near Ballinger TX (08126380, located upstream) |
| --- | --- |
| Stream Gage Name & ID: | Colorado River near Stacy TX (08136700) |
| Stream Gage Name & ID: | Colorado River near San Saba TX (08147000) |

## Technical Review (Reviewer to confirm with Pass/Fail)

| Excess Rainfall |  |
| --- | --- |
|  | Precipitation values are an aerial average of Atlas 14 values for the entire 2D area boundary for each storm event or other appropriate rainfall application |
|  | CN values appropriately reflect land use and soils for entire 2D area boundary |
|  | Losses are correctly calculated to establish the excess rainfall hyetographs by checking land use, soils and infiltration layers or other relative information |
|  | Total precipitation depths for 1%+ and 1%- are calculated correctly |
|  | Excess rainfall hyetographs are calculated for all storm events (10%, 4%, 2%, 1%, 0.2%, 1%+, and 1%-) |
|  | Cumulative excess rainfall values are in increasing order from 10%, 4%, 2%, 1%, to 0.2% |
|  | Cumulative excess rainfall values for the 1%+ and 1%- fall in order with the storm events matching the total precipitation values |
| Inflow Hydrographs |  |
|  | Shape and time-to-peak of any inflow hydrographs resemble past documented large storm events |
|  | Hydrographs reasonable when compared to the other return periods. The shapes should be similar, and hydrographs from different events should not intersect unless there is an engineering reason. Peak flows from a lower return period event should not be higher than a higher return period event |
|  | Contributing hydrographs from another RAS model applied at coincident locations (Provide locations in base map data) |
| General Hydrology |  |
|  | Any flow regulating dams in the model area? (Yes/No) |
|  | Dam modeling methodologies applied and explained? (Yes/No). If yes, provide discharge rating curve calculations and gage analysis documenting regulated versus unregulated records, if applicable. |
|  | Model aligns with documentation of dam modeling approaches. |
| Other Notes: NA |  |

## Review Comments

- Per the TWDB guidance document “Texas Two-Dimensional Base Level Engineering Guidelines, version 1.0” dated May 21, 2022, the TWDB prefers precipitation hyetographs to be developed within HEC-HMS and any variation must be approved with prior coordination with TWDB. The submitted supplemental documentation shows development of hyetographs in excel based on duplication of Fortran code from WinTR-20.

Atkins Response (03/09/2023): The NRCS nested hydrograph method is another commonly used temporal distribution which Atkins had used successfully on previous projects. Our model development for the TO1 of this study was well underway before TWDB guidance was published so we had used NRCS distribution.  Additionally, in past monthly progress meetings we have discussed this topic with TWDB staff, and they agree that NRCS nested rainfall methodology is acceptable and can be used for 2D BLE model development. It is our understanding that this methodology will be added to the TWDB Guidance at some point in the near future.

No additional comment.

- TWDB recommends the use of areal reduction as a validation parameter; this was not mentioned in the Buchanan report. Clearly state in the report if areal reduction was applied or considered.

Atkins Response (03/09/2023):  Atkins was able to achieve flow validation using curve number and manning’s n updates so did not use aerial reduction factor as a validation parameter for this watershed. We have added a statement regarding this in Section 1.2.9 Model Validation of the report.

No additional comment.

- The TWDB recommends that base curves numbers be utilized with a separate percent impervious value (for developed areas). Explain variance from recommendations. This affects the “developed” conditions.

Atkins Response (03/09/2023):  For developed categories, we have applied the composite TR55 curve numbers equivalent to the values in the TWDB guidance. For example, for developed high intensity soil group D the TWDB guidance document listed a base CN value of 80 with 85 percent imperviousness. For this category, we have applied a corresponding TR55 composite value of 95 (prior to validation) from commercial and business land cover category, where impervious area of 85% is included in composite value calculation.

Per the meeting discussion on 03/27/2023, the composite curve numbers were utilized for all TO1 HUCs which will be changed for TO2 HUCs. Clear documentation of composite CN approach is required in the report.

Atkins Response (06/03/2023): Additional discussion of the of the CN values will be added to the report.

See Subsection 1.2.2.

8/10/23: No additional comments.

- There are a number of CNs in the models that do not meet plus or minus either the AMC – 1 or AMC – 2. Please revise.

Atkins Response (03/09/2023):  Published documents show that there is large variation in AMC conditions across Texas however, few sources are available which depict how AMC condition vary. Through some research Atkins was able to find following map from Page 23 of Engineering Technical Note No. 210-18-TX5 titled Estimating Runoff For Conservation Practices developed by SCS. The term Antecedent Runoff Condition (ARC) used in the older literature is analogous to Antecedent Moisture Condition (AMC), which is the term that is commonly accepted today.

During model validation process for some of the model domains the computed BLE flows were too high with 90% AMCII CN values and too low with AMCI CN values. After research of published documentation on the topic of AMC conditions in Texas, we obtained following map which shows gradual transition from AMCI to AMCII condition from west Texas to east Texas. By following this approach and watershed location, we weighted the CN values to by assigning 70% weight to AMCI and 30% weight to AMCII. This approach appeared technically superior to Atkins staff rather than continuing to adjust the base CN values more than 10%. Additional discussion added in the report Section 1.2.9 Model Validation. This graph was included in the appendix B of the word document of the draft BLE report.

No additional comment.

- Inflow for A1 model into OH Ivie – hydrograph crosses for 0.2% and 1%Plus. Verify source of inflow hydrographs.

Atkins Response (03/09/2023):  The source of the inflows used in the BLE model for the O. H. Ivie reservoir is InFRM RFA study and InFRM gage analysis. Please refer to the table and screenshots below, see additional discussion in Section 1.2.5 of the BLE report. For both Colorado River at Ballinger and Concho River at Paint Rock USGS gages, the InFRM 1% plus flow is higher than 0.2% AEP. This causes hydrographs to cross between 1% plus and 0.2% AEP.

| AEP | InFRM gage Analysis |  | Combined Peak Inflow rate (cfs) | InFRM RFA Analysis | Inflow applied in BLE model (cfs) |
| --- | --- | --- | --- | --- | --- |
|  | 08126380 -Colorado River near Ballinger (cfs) | 08136500 -Concho River at Paint Rock (cfs) |  | Total Release (cfs) |  |
| 10% | 9,730 | 13,500 | 23,230 | 1,500 | 23,230 |
| 4% | 14,900 | 23,000 | 37,900 | 32,700 | 32,700 |
| 2% | 19,200 | 32,100 | 51,300 | 52,500 | 52,500 |
| 1% | 24,000 | 43,300 | 67,300 | 67,350 | 67,350 |
| 0.2% | 36,400 | 78,400 | 114,800 | 94,945 | 94,945 |
| 1% plus | 64,000 | 127,000 | 191,000 | - | 191,000 |
| 1%Minus | 15,400 | 24,600 | 40,000 | - | 40,000 |

Screenshot from Draft InFRM report Page 146:

Screenshot from Draft InFRM report Page 149:

Screenshot from Draft RFA report Page 67:

If 1%+ peak is higher than the 0.2% peak, then the hydrograph generated for 1%+ should be adjusted to be consistently higher than that of 0.2%.

Atkins Response (06/03/2023): There are two separate peaks occurring along Colorado River at the different times namely, the peak from local rainfall over the watershed and peak generated from O. H. Ivie outflows. The peak 1% Plus outflow downstream O.H. Ivie was higher than the 0.2% outflow, however the 0.2% rainfall over the watershed is higher than 1% plus rainfall. Because of this contradiction, for domains immediately downstream of O. H. Ivie the 1% plus peak flows /elevations are higher than 0.2% peak flows and further downstream the 0.2% peak flows/elevations are higher than 1% plus. This item is discussed in detail in Challenges section of the report and some of the example figures from the report are included below for reference. No modeling updates were performed.

8/10/23: No additional comments.

# 3 – Hydraulic Geometry

## Methodologies / Data Sources (Modeler to populate)

| Software: | HEC-RAS 6.2 |
| --- | --- |
| Loss Methodology: | Spatially varied CN |
| Soils Source: | SSURGO |
| Land Cover Source: | NLCD 2019 |
| Manning's Roughness Source: | Assigned based on land use. Please see table in MidCo Model Overview and Results.xlxs |
| Nominal Mesh Cell Sizes: | 200 x200 except for channel refinement regions, breaklines and urban regions, where 100 ft x100 ft was considered. |
| Other: |  |

## Technical Review (Reviewer to confirm with Pass/Fail)

| 2D Mesh |  |
| --- | --- |
|  | 2D Boundary delineated along watershed boundary or buffer to ensure all contributing area is included |
|  | Nominal cell size appropriate for model detail (generally 200 feet, minimum 50 feet for refinement areas) |
|  | Computation points are at default locations or changes documented |
|  | Mesh Cell and Face tolerances are default or changes documented |
|  | No mesh errors |
|  | Cell face relationships are calculated |
| Breaklines |  |
|  | Breaklines are consistent with the terrain data |
|  | Breaklines sufficiently detailed to capture features (thalweg, banks, berms, etc.) |
|  | Cells do not overlap terrain features where appropriate (i.e., unwanted leaking) |
|  | Breakline cell sizing and parameters are appropriate |
|  | 2D Area Connections placed at applicable benchmarks, gages, or dams |
|  | 2D Area Connections set to use normal 2D equation domain for benchmarks and gages. Appropriate modeling approach used for dams. |
| Manning’s N-Values |  |
|  | Manning’s n-values associated to land cover data provided |
|  | Land Cover classifications are sufficiently descriptive within the data table |
|  | Manning’s n-value classifications generally match current aerial imagery |
|  | Manning’s n dataset is associated with the geometry files |
| General |  |
|  | All model datasets each have unique descriptive names |

## Review Comments

- Add plan descriptions for all domains and events (as well as main RAS window) per the TWDB guidance document (Section 2.5.2).

Atkins Response (04/14/2023):  Plan descriptions are now added to all domains and events Section 2.5.2 in the guidance. Example screenshot below.

No additional comment.

- Report final Manning’s values for each domain within the report (or appendices). Remove excess “landcover” layer if it is not referenced by any plans.

Atkins Response (03/09/2023):  Updated Table 7 and Table 9 to further clarify Manning’s n values used in the final model. For this watershed, no changes to overbank n values were made during validation which is shown in Overland n adjustment column in Table 9. Channel n adjustments validation are documented in Channel n Adjustment column in Table 9.  Details of model validation are also documented in the Model Overview and Results spreadsheets located under Task Documentation folder.

Deleted any excess backup landcover files. The Landcover.tif file used in the infiltration layer and Mannings.tif is linked to the geometry to define manning’s n values.

No additional comment.

- If 2D connections are used to carry flow from one domain to the next, make those 2D connection line at the same location and same size. (e.g. A2 out line doesn’t match A3 in line). Check and update for all models.

Atkins Response (03/09/2023):  We had documented this item in M1 submittals on previous watersheds. Following discission added to report:

## Length Adjustments to Inflow Boundary Condition Lines

At the request of TWDB’s independent technical reviewers, SA/2D connections were modeled to capture the outflows from one domain into another. The flows at these features are recorded in the model’s DSS output files. These connections are coincident with the inflow boundary location in the downstream model.

The lengths of the inflow lines were shortened to a location between the 1% and the 10% floodplain limits to prevent the full distribution of the flow across the valley width. When the inflow line was the same length as the SA/2D connection, floodplain and flow were erroneously added at locations that were dry in the overlap area of the upstream model. Figure 6 shows an example of an outflow SA/2D connection near the outlet of an upstream domain. The results with the full-length coincident inflow boundary condition line in the receiving domain in Figure 7 show an example of an expanded floodplain compared to the upstream model. Figure 8 shows an example of the results with the shortened inflow boundary condition that are more consistent with the results in the overlap area between the two model domains.

Figure 6. Example SA/2D connection and floodplain at the outflow of an upstream domain.

Figure 7. Example of full-length inflow boundary condition line for flow transfer to a receiving domain.

Figure 8. Example of shortened inflow boundary condition line for flow transfer to receiving domain.

Providing shorter inflow stage boundary condition lines does reduce the increased WSE inadvertently applied to the overbank however it would be more appropriate to have separate BC lines to capture channel and overbank flows separately. Review and adjust as necessary.

Atkins Response (06/03/2023): Where there are divided flows such as due major roadways, we use multiple SA/2D to distribute flows over the floodplain width. If the flow is not divided only single connection seem adequate rather than separating main channel from overbanks. If the flow is higher than channel capacity, it will spread to overbanks.

8/10/23: No additional comments.

- Why Inflow line at A8 is drawn along the channel which is different than A7 outflow and overall different than other models?

Atkins Response (03/09/2023):  See response to previous comment.

8/10/23: No additional comments.

# 4 – Hydraulic Boundary Conditions

## Technical Review (Reviewer to confirm with Pass/Fail)

| Boundary Review |  |
| --- | --- |
|  | External boundaries connected to 2D mesh |
|  | Boundary conditions everywhere flow could build up at the boundary of the 2D mesh |
|  | Boundary conditions have a hydrograph or elevation/slope data assigned |
|  | Input hydrographs consistent with hydrologic data (DSS, manual entry) |
|  | Precipitation data is consistent with hydrologic data (DSS, manual entry) |
|  | Downstream boundary consistent with ground slope or known downstream WSEL |
|  | Flow hydrographs applied at appropriate locations |
|  | Energy grade slope for hydrograph distribution is reasonable |
|  | Internal rating curves appropriate and documented for control structures (if applicable) |

## Review Comments

- Document boundary condition approach in report to justify the longer ND BCs seen in the models. Verify there is not unaccounted for flow transfer between the domains not at the main channel crossing. E.g. A1 model General Outflow 1 shows over 20,000 cfs for 0.2% event. If there are non-negligible flows, divide the boundary condition as necessary to better represent flow leaving the mesh. Applicable for all models.

Atkins Response (03/09/2023):  We have added a discussion in the Section 1.2.4 of the report. We have applied a buffer of 1000’ to the watershed boundary so the 2D mesh extends beyond the high ground at the watershed boundary. Therefore, flow captured in the perimeter boundary condition line of the model is not the overflow from one watershed into adjacent on the model. For this watershed there is topographic relief so the overflows leaving into adjacent watersheds are negligible. For locations where floodplain extends to the watershed boundary, we have drawn profile lines to verify flow is negligible and not need to be accounted into adjacent HUC8 model. Additionally, due to application of buffer minor overflows from adjacent watersheds are accounted for in the model without needing to add overflows as inflows.

This general approach is acceptable. But, add response to the comment particularly where outflow is significant like in example mentioned where it is over 20,000 cfs.

Atkins Response (06/03/2023): As discussed earlier, the model extent (2D perimeter) used a buffer of 1000’ feet over the watershed boundary. Since entire model has one single 2D area amount of rainfall applied over the watershed is also applied to the areas outside of the watershed located within the 1000’ buffer. The flow generated outside of the watershed boundary would drain out through the perimeter boundary conditions so high flow is expected at the perimeter boundary conditions, which is not an overflow. As an example, in this domain the contributing watershed area is 175 sq. mi and the model extent has area of 190 sq. mi. and the 0.2% rainfall excess (from HEC-RAS) is 7.2 inches. So, the ~25,000 cfs captured in outflow boundary condition 1 is resulting from 7.2 inches of rainfall excess over ~15 sq. mi area (190 - 175) outside watershed boundary, which is not an overflow out of Middle Colorado HUC8.

Please see screenshots below:

8/10/23: No additional comments.

- Document and justify in BLE report why ND was not used and provide justification/appropriateness of using stage hydrographs between model domains (e.g. between A1 and A2, A2 and A3). Per Section 2.3.6, the preferred downstream boundary condition is normal depth. Stage and flow hydrographs are primarily for coastal areas.

Atkins Response (03/09/2023):  Use of stage hydrograph as a downstream boundary is acceptable and it was used for two main reasons. First, stage hydrographs account for simulated stages in the downstream model, whereas north boundary condition may not mimic the stage-time variation in downstream model. Second, during tie-in process we found the stage hydrograph boundaries were less iterative than normal depth boundary conditions so stage hydrographs are used in this study.

Stage hydrograph BC is applicable in certain scenarios. If it is used here, provide a comparison table for inflows/outflows and WSELs to confirm that flow is not removed or generated within the model due to the stage-hydrograph BC.

Atkins Response (06/03/2023): We have added these tables to the report. See Subsection 1.2.4.

8/10/23: No additional comments.

- A2 model d/s BC line and 2D connection doesn’t incorporate all the flow leaving the domain. Revise.

Atkins Response (04/14/2023):  We will have adjusted the BCLine (blue line in screenshot) and mesh as shown in screenshot below per this comment.

No additional comment.

# 5 – Hydraulic Model Execution

## Methodologies / Data Sources (Modeler to populate)

| Unsteady Plan Names |  |  |  |
| --- | --- | --- | --- |
| 10% ACE (10-year): | NRCS_10_PCT | 0.2% ACE (500-year): | NRCS_0_2_PCT |
| 4% ACE (25-year): | NRCS_4_PCT | 1% plus: | NRCS_1 _PLU |
| 2% ACE (50-year): | NRCS_2_PCT | 1% minus: | NRCS_1 _MIN |
| 1% ACE (100-year): | NRCS_1_PCT |  |  |
| Other: |  |  |  |

## Statistical Data (Modeler to populate)

| Time Step: | Advanced time step based on Courant condition (7.5-60 Seconds) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Courant Value: | Min: 0.45 | Max: 2 | Simulation Length: | 66-168 | hours |
| Computation Interval: | 30 | seconds | Model Runtime: | 1-5.5 | hours |
| Hydrograph Output Interval: | 10 | minutes | Mapping Output Interval: | 60 | minutes |

## Technical Review (Reviewer to confirm with Pass/Fail)

| Execution Review |  |
| --- | --- |
|  | Appropriate 2D parameters are used (Boundary Conditions, Time Step, Simulation Length, etc.) |
|  | Hydrograph Output Interval and Mapping Output Interval set at a reasonable interval to capture peaks |
|  | Model runtime is reasonable (< 12 hours, but dependent upon model size) |

## Review Comments

- Simulation length, mapping output interval, runtime noted above are different for different models.

Atkins Response (04/14/2023):  The Simulation Length and Model Runtime are updated in table above to show the range of lengths and run times for all models in this watershed.

No additional comment.

# 6 – Hydraulic Model Results

## Methodologies / Data Sources (Modeler to populate)

| Flow Validation/Gage Analysis |  |
| --- | --- |
| Software: | N/A |
| Methodology | Regression flows were used in the absence of gage data and FIS flows; Draft InFIRM report was used to develop the inflow hydrograph for O.H. Ivie reservoir, pool frequency information obtained from RFA report is leveraged for reservoir modeling. Refer to the BLE report and the Model Overview and Results Spreadsheet. |
| Removal of Outliers and Justification | N/A |
| Validation Notes (describe parameter adjustments): Refer to the BLE Report and the MidCo Model Overview and Results.xlsx for more details of validation |  |
| Other: |  |

## Technical Review (Reviewer to confirm with Pass/Fail)

| Computation Checks |  |
| --- | --- |
|  | Model runs with no cell errors |
|  | Volume balance is within tolerance (ΔV < 1% is ideal, however 2% to 3% can be acceptable for rain on mesh) |
| Results Spot Checks Check at all critical structures, major confluences, a handful of headwater tributaries, downstream end, at benchmarks, and a handful of randomly selected locations. |  |
|  | No visible spikes in WSEL visibly present with a single cell |
|  | WSEL and depth grids appear reasonable (no artificial dips or spikes) |
|  | Visual verification flooding is connected downstream for all significant contributing areas (> 64 acres) |
|  | Velocity grids appear reasonable (no unusually high velocity areas or Courant values greater than 2) |
|  | No crossing profiles between all events |
|  | Inflow and outflow hydrograph timing is reasonable as compared to the peak |
|  | Spills into/from adjacent HUCs appropriately accounted for |
|  | WSEL tie-in between model domains and HUCs along major rivers/streams where inflows are applied |
| Results Full Checks |  |
|  | Computational time step meets ideal Courant condition |
|  | Diffusion Wave: V∆T∆X ≤2 (Up to 5) (Results are unit-less) |
|  | Model simulation is long enough to pass the entire hydrograph through the model (minimum of 12 hours past the peak, preferably 24 hours) |
|  | Hydrographs at internal and external boundaries are smooth |
|  | Contributing areas are contained with 2D mesh or accounted for with inflow hydrographs |
|  | No flow trapped at 2D boundary |
|  | Visual verification mapping is reasonable compared to effective |
| Flow Verification |  |
|  | Peak flow gage analysis performed on all gages in the watershed with sufficient data |
|  | All gages used have an appropriate period of record (10 to 20+ years) |
|  | All gages used have no extensive gaps in record which would impact the analysis |
|  | Gage analysis results validated to other sources (Regression, USGS, etc.) |
|  | Results at applicable benchmarks justified (gages, HWMs, effective BFEs) |
|  | Peak discharges comparable to gages, effective/prior studies, regional regression equations |

## Review Comments

- WSEL results in the O. H. Ivie reservoir may not be correct as the model include only part of the reservoir storage area. The inflow is not utilizing all of the available storage volume (outside/upstream of the model domain) causing higher WSE in the lake.

Atkins Response (03/09/2023):  The InFRM RFA study has established the pool frequencies for O.H. Ivie Reservoir, and goal of the BLE study was to obtain elevations similar to InFRM study without doing any detailed modeling for dam. Also, the O. H. Ivie is located at the confluence of the Colorado and Concho Rivers and portion of the reservoir extends into Concho River HUC8 and Middle Colorado-Elm HUC8, which are outside the scoped study area of Middle Colorado HUC8 watershed. So to obtain elevations, peak inflow rate into the reservoir was obtained the RFA Total Release or calculated by adding the InFRM gage analysis peak flow rates for USGS gages on Colorado River near Ballinger, TX (08126380) and Concho River at Paint Rock, TX (08136500). Using the inflow peak rate, a triangular inflow hydrograph shape was applied and hydrograph shape was iterated to obtain pool elevations similar to InFRM study. Detailed discussion on O.H. Ivie reservoir modeling is added to the 1.2.5 section of the BLE report.

Recommend adding O. H. Ivie reservoir modeling (including inflow adjustments/discrepancy, modeling correct storage volume, starting water elevation, etc.) to Challenges section in the report.

Atkins Response (06/03/2023): A brief discussion has been added to the Challenges section pointing the reader to Subsection 1.2.5 to learn how the reservoir was modeled.

8/10/23: No additional comments.

- The starting level/bottom elev at the reservoir OH Ivie is 1540. The outflow start only at 1551. Is this a correct starting elevation? Or should the starting elevation in the lake is assumed to be at the normal pool of the lake which is 1551?

Atkins Response (03/09/2023):  As explained in response to the previous comment the goal of this BLE effort was not establish the elevations on the O.H. Ivie reservoir but to obtain elevations similar to InFRM study without detailed modeling. So initial elevations for the lake were not applied.

See comment above.

Atkins Response (06/03/2023): A brief discussion has been added to the Challenges section pointing the reader to Subsection 1.2.5 to learn how the reservoir was modeled.

8/10/23: No additional comments.

- Either compute/update results layesr or remove them

Atkins Response (04/14/2023):  Reviewed RASMapper setup and deleted rasters with missing links from all models. To reduce model size we had deleted the saved rasters from model folder. The raw output grids are however included in the supplemental data folder.

No additional comment.

# 7 – Mapping Results

## Technical Review (Reviewer to confirm with Pass/Fail)

| Hydraulic Model |  |
| --- | --- |
|  | HEC-RAS model with results, pertinent data and required storm events |
| Flood Risk Database |  |
|  | All features fully populated per current FEMA guidelines |
|  | Mapping consistent between grids to polygons for all three required mapped events |
|  | 10yr smaller than 100yr smaller than 500yr |
|  | Holes filled in polygon, depth and WSE grids |
|  | Boundary interfaces align matching or identified as AOMI |

## Review Comments

- See Checkpoint 8 below for mapping and final deliverables comments.

Atkins Response (8/30/2023): Noted. Please see our responses in Checkpoint 8.

# 8 – Completeness Review

## Technical Review (Reviewer to confirm with Pass/Fail)

| File Structure and Naming |  |
| --- | --- |
|  | Deliverables provided in TWDB/FEMA required file folder structure and naming convention |
|  | Data missing (if any) documented in report and ReadMe file within the required folder |
| Report |  |
|  | Technical 2D BLE report documents all assumptions, special considerations, etc. |
|  | Flow validation and difference discussed in report |
|  | Flow comparisons discussed in report |
| Hydrologic Model |  |
|  | HEC-HMS model with results, pertinent data and required storm events |
| Hydraulic Model |  |
|  | HEC-RAS model with results, pertinent data and required storm events |
| Flood Risk Database |  |
|  | All features fully populated per current FEMA guidelines |
| CNMS |  |
|  | Updated correctly with new centerlines |
|  | A1-A5 checks complete |
|  | CNMS Post- Processing Complete |
| HAZUS |  |
|  | Run per FEMA guidance |

## Review Comments

- CNMS Review

- Not submitted as part of this review, CNMS will be reviewed at a later date coordinated with TWDB and Halff.

Atkins Response (8/30/2023): Noted. CNMS will be submitted following M2 approval.

No further comments (10/13/2023)

- Hazus Review

- No Comments

- FRD Review

- Base Dataset

- S_Pol_Ar -

- COM_NFO_ID-Blank, should contain Community Information Identification. Foreign key to L_Comm_Info table. This attribute links to the table L_Comm_Info that contains information about the specific community. Refer to FIRM Database Technical Reference.

Atkins Response (8/30/2023): S_POL_AR attributes have been updated.

No further comments (10/13/2023)

- S_HUC_Ar -

    - Missing geometry for HUC 10’s and 12’s, feature class only contains HUC 8 boundary.

Atkins Response (8/30/2023): S_HUC_AR has been updated.

No further comments (10/13/2023)

- EBFE Dataset

- WTR_LN -

- SOURCE_CIT-BASE1 is the same source used for S_Pol_Ar, WTR_LN’s should have a unique source since data is acquired differently. Update to match L_Source_Cit table.

Atkins Response:  Updated WTR_LN source cite to “STUDY1”.

No further comments (10/13/2023)

- Several streams seem to be missing? Streams should be mapped 1 SQM rural and 0.5 SQM urban. See examples below and QC polygons.

Atkins Response:  All 1 sq mi streams were already present in WTR_LN. Added two segments of 0.5 stream in Cities of Bangs and Goldthwaite.

No further comments (10/13/2023)

- WTR_AR -

- SOURCE_CIT-BASE1 is the same source used for S_Pol_Ar, WTR_AR’s should have a unique source/identifier since data is acquired differently. Update to match L_Source_Cit table.

Atkins Response:  Updated WTR_AR source cite to “BASE2”.

No further comments (10/13/2023)

- DTL_STUD_AR -

- Currently empty missing effective mapping in Brown County and San Saba County, Coleman County and Mills County review screenshot below referencing CoreLogic data and check QC area polygons. Review FEMA Map Service Center, National Flood Hazard Layer (NFHL) or CoreLogic for effective mapping.

Atkins Response: Screenshot above includes Zone A, which shouldn’t be included in the DTL_STUD_AR. Added a small Zone AE polygon to the south of the watershed.

CD_EMAIL field currently “Null” update to acceptable null values such as “NP” or blank string (10/13/2023)

Atkins Response (11/28/23): Updated field to “NP”

Halff Backcheck Comment 12/06/2023-

      - No Further Comments

- DTL_STUD_LN -

- Currently empty missing update with WTR_LNs that intersect effective mapping.

Atkins Response:  There are no DTL_STUD_LN going through the small piece of Zone AE polygon. No changes made.

No further comments (10/13/2023)

- TENPCT_FP -

- TENPCT_FP not contained in several areas within the 100yr FLD_HAZ_AR, update to contain 10yr within S_FLD_HAZ_AR should not extend outside of 100 or 500yr

Not contained within the 100yr

Atkins Response:  TENPCT_FP revised to be contained in the 100-yr polygon.

No further comments (10/13/2023)

- FLD_HAZ_AR –

  - EST_AR_ID-Currently contains NULL values, assign acceptable values (example sequential numbers)

Atkins Response:  EST_AR_ID has been updated.

ZONE_SUBT- Currently has Null values for Zone A, update to empty string. (10/13/2023)

EST_RISK- Currently has Null values update to appropriate Domain value for specific flood risk/zone. (10/13/2023)

Atkins Response (11/28/23): Updated Zone_SUBTY to Moderate or High based on FLD_Zone.

Halff Backcheck Comment 12/06/2023-

      - No Further Comments

STUDY_TYP- Currently has Null values update to empty string. (10/13/2023)

Atkins Response (11/28/23): Updated to “NP”

Halff Backcheck Comment 12/06/2023-

      - No Further Comments

DUAL_ZONE- Currently has Null values update to appropriate Domain value for specific flood risk/zone. (10/13/2023)

Atkins Response (11/28/23): Updated to “False”

Halff Backcheck Comment 12/06/2023-

      - No Further Comments

Model Tie-In Reviews

Mapping cut off at model domains

Mapping cut off at model domains

Atkins Response (8/30/2023): The watershed was reviewed. Tie-ins that were outside the 0.5-ft tolerance have been revised.

100YR outside of 500yr in mapping, does not match grids, still cutting off at model. (10/13/2023)

Issue with mapping FLD_HAZ_AR, appears in some areas throughout the HUC that the 500yr was removed/erased? Looks like this is occurring most within Models 0605_A8 and 0606_A9. Please review screenshots below:

Atkins Response (11/28/23): Revised the 0.2% and 1% floodplains so that the 1% floodplain completely resides within the 0.2% floodplain.

Revised Submittal for review:

Previous Submittal for review:

AtkinsRéalis Response (5/23/24): The floodplain mapping has been revised throughout the watershed for this submittal.  The 1% floodplain resides within the 0.2% floodplain.

- BFE_2D -

- No comments, BFEs match WSE grids appropriately

- S_AOMI_Ar -

- Since AOMI_Pt is not required/in the current schema, all AOMO_Pt features should be buffered and included in the AOMI_Ar.

Atkins Response (8/30/2023): Atkins has received approval from TWDB to include S_AOMI_PT in our FRDs for our TO1 HUCs. For all other TOs, Atkins will not include AOMI_PT in the FRD.

AOMI_INFO, HUC8_CODE and CASE_NO- fields currently empty for some entries populate to match other attributes. (10/13/2023)

Atkins Response (11/28/23): Fields updated as requested.

Halff Backcheck Comment 12/06/2023-

      - No Further Comments

- S_AOMI_Pt – This feature class is no longer being used all attributes should be buffered and added to S_AOMI_Ar.

- Streamflow constrictions missing at some road intersections, update to include. Examples in QC polygon

Atkins Response (8/30/2023): Atkins has received approval from TWDB to include S_AOMI_PT in our FRDs for our TO1 HUCs. For all other TOs, Atkins will not include AOMI_PT in the FRD.

No further comments (10/13/2023)

- S_FRAC_Ar –

    - No comments

- L_Source_Cit -

- No comments

- Grids Review

Grid Snapping:

  - BLE_WSE01PCT snapped correctly to BLE_WSE0_2PCT

  - BLE_DEP01PCT snapped correctly BLE_DEP0_2PCT

Atkins Response (8/30/2023): Noted.

Grid Extents

  - Several areas identified where the BLE_WSE01PCT grids and BLE_DEP01PCT extend outside of BLE_WSE0_2PCT and the BLE_DEP01PCT, review QC polygons for example areas

\

Atkins Response (8/30/2023): The watershed was reviewed. The grids have been revised so the 1% grids are within the 0.2% grids.

No further comments (10/13/2023)

  - BLE_WSE01PCT grid extends outside of BLE_WSE0_2PCT, review QC polygon for example areas

Atkins Response (8/30/2023): The watershed was reviewed. The grids have been revised so the 1% grids are within the 0.2% grids.

No further comments (10/13/2023)

  - BLE_DEP01PCT grid extends outside of BLE_DEP0_2PCT, review QC polygon for example areas

Atkins Response (8/30/2023): The watershed was reviewed. The grids have been revised so the 1% grids are within the 0.2% grids.

No further comments (10/13/2023)

Tie-Ins at Model Boundaries

- Along model boundary tie-in greater than 0.5 foot in grids across boundary.

Hole found in 0.2PCT grid recommend using interpolation method to fill hole in grid to match 500yr floodplain. See screenshot below of hole within BLE_WSE0_2PCT grid that is not in Zone X FLD_HAZ_AR.

Grids cut off at model boundary and tie-ins greater than 0.5ft in the 1PCT

Grids cut off at model boundary and tie-ins greater than 0.5ft in the 1PCT

Atkins Response (8/30/2023): The watershed was reviewed. Tie-ins that were outside the 0.5-ft tolerance have been revised.

Issues with Model boundaries still please review examples in screenshots below and QC polygons (10/13/2023):

Most issues along Model Boundary 0606_A9, 0.2PCT

Issue above along 0604_A7, 0.2PCT

Please review along all model boundaries above and QC polygons are just representative examples.

Tie-In issues at adjacent HUCs (10/13/2023) Jim Ned, Pecan, San Saba and Brady

Please review all tie-ins for proper connectivity screenshots and polygons are just represenative examples.

Atkins Response (11/28/23): Rasters updated between model boundaries as requested. The entire watershed was reviewed to confirm this issue was not present at other locations.

Halff Backcheck Comment 12/06/2023-

- Issues with tie-ins still present at Jim Ned please review QC polygon and screenshot below. If unable to tie in due to modeling differences and issues, please create S_AOMI_Ar polygons.

Tie-In issues at adjacent HUCs

AtkinsRéalis Response (5/23/24): The floodplain mapping has been revised throughout the watershed for this submittal. The tie-ins between the Middle Colorado and Jim Ned watersheds have been corrected. See the difference rasters, HEC-RAS model results, and split lines included with this submittal to validate the tie-ins along the Middle Colorado-Jim Ned border.

There is one location in the Jim Ned mapping database that the Halff team has where the mapping will look like it is cut-off. See the image below, which is in the FID=55 polygon in the ITR QC shapefile. Black is the Middle Colorado mapping (1%) and grey is the Jim Ned mapping (1%). Red is the HUC-8 boundary and purple is the ITR QC shapefile. We have deleted the Jim Ned mapping that is cut-off off by the HUC-8 boundary.

Halff Backcheck Comment 6/07/2024

- All tie-ins with Jim Ned have been updated and corrected, no further comments.

QA/QC Checklist Approval

This 2D Base Level Engineering Analysis QA/QC Review is in compliance with the contract requirements and all task checkpoints are complete.  The independent QC Team has reviewed the analysis, presented review comments to the Production Team Task Leader, and discussed any problems or issues.  The Task Leader, QC Manager, QC Reviewer, and Project Manager have signed the QA/QC Checklist to confirm that all comments have been received, addressed, and documented appropriately.

| QA/QC Team – [TWDB or company name] |  |  |  | Production Team – [add company name] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Seth Ahrens |  | 12/16/2024 |
| QC Manager |  | Date |  | Task Lead |  | Date |
|  |  |  |  | Seth Ahrens |  | 12/16/2024 |
| QC Reviewer |  | Date |  | Project Manager |  | Date |