# Texas 2D BLE Analysis Audit Checklist

# Project Information

| HUC-8 Watershed Name & ID: | Pecan Bayou - 12090107 |
| --- | --- |
| HUC-10 Watershed Name & ID: | Headwaters Pecan Bayou - 1209010701 Turkey Creek-Pecan Bayou - 1209010702 Red River-Pecan Bayou -1209010703 Lake Brownwood - 1209010703 Devils River-Pecan Bayou - 1209010704 Blanket Creek - 1209010705 Browns Creek-Pecan Bayou - 1209010706 |
| Modeler: |  |
| Reviewer: | Halff Associates, Inc. |

This audit process includes only high-level review for items identified in the Detailed Review for another HUC for the same consultant. The consultant is responsible for performing a detailed review internally for base data, hydrology, hydraulics, and modeling results.

## Audit 1: Terrain, Modeling, Preliminary Mapping Review (QC Items 1-7)

| Modeler Initials: | JG | Reviewer Initials: | KL |
| --- | --- | --- | --- |
| Submittal Date: | 05/30/2022 | Review Submitted Date: | 7/27/2022 |
| Review Response Date: |  | Backcheck Complete Date: |  |

## Audit 2: Completeness Review (QC Item 8)

| Modeler Initials: | JG | Reviewer Initials: | JM |
| --- | --- | --- | --- |
| Submittal Date: | 11/04/2022 | Review Submitted Date: |  |
| Review Response Date: |  | Backcheck Complete Date: | 6/21/23;8/21/24; 2/12/25 (M3) |

(Modeler to populate with an “X” for items included in the QC submittal)

| Level of Study |  |  | Flood Events |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| x | 2D BLE |  | x | 10% ACE (10-year) | x | 0.2% ACE (500-year) |
|  | Enhanced BLE |  | x | 4% ACE (25-year) | x | 1% plus |
|  |  |  | x | 2% ACE (50-year) | x | 1% minus |
|  |  |  | x | 1% ACE (100-year) |  | Observed Rainfall |

Instructions

Each section includes instructions to the modeler and reviewer regarding their responsibilities in the completion of this checklist. In the Methodologies / Data Sources tables, grey italic text is added as an example response. Additional sample responses are located in the Texas 2D BLE Guidelines Quick Guide.

## Submittal Checklist

Data submitted for each milestone should be carried through each subsequent milestone. All geospatial layers and models must have same defined projection system as terrain. Modeler to populate with an “X” for items included in the QC submittal.

| Base Map Data (QC items 1-6) |  |
| --- | --- |
| X | Location Map (PDF): HUC-8 and HUC-10 boundaries and/or model domain boundary if different from HUC-10s, stream gages, dams (if applicable), mainstem streams (for reference), background map |
| X (Ras Mapper) | GIS Map (MXD or Map Package) with the following data included but not limited to: Stream centerlines, model breaklines, rainfall data, WSEL and Depth grids (10, 100, 500-yr), adjacent HUC mapping (available on the BLE viewer or currently studying), 2D connections for inflows/outflows, gages or points of flow comparisons (BFEs, highwater marks, etc.), profile lines, boundary condition lines, refinement regions, model domain boundary, model mesh, DFIRM mapping panels, base map data (roads, aerials, etc.), soils data, land use data, terrain |
| Technical Report / Documentation (QC items 1-6) |  |
| X | Technical Report / Project Notebook: assumptions, special considerations, and modeling concerns |
| X | Detailed explanation and support data of variances in methodologies and approaches from recommended guidelines (Should be included in Technical Report / Project Notebook) |
| X | Internal QA/QC Documentation: comments and responses |
| X | Flow comparison tables within the technical report |
| Hydrology (QC items 1-6) |  |
| X (HEC-RAS) | Hydrologic Model (HEC-HMS) model with all associated DSS files for 7 required storm events |
| X | Hydrologic Data and Parameter Spreadsheets (rainfall calculations, curve number table, gage locations, gage analysis and flow validation) |
| X | Hydrologic modeling notebook including assumptions, notes, and special conditions (if not provided in report) |
| Hydraulics (QC items 1-6) |  |
| X | Hydraulic Model (HEC-RAS) with all associated files and 7 required storm events (remove non-final files)d |
| X | Hydraulic Data and Parameter Spreadsheets (Manning’s n-value table, infiltration calculations support data-Soils, Land Use, Curve Number table, other calculations/data used in model) |
| X | Manning’s n-value table |
| X | Precipitation data correctly linked to hydrologic model or hard coded from provided spreadsheet |
| X | Hydraulic modeling notebook including assumptions, notes, and special conditions |
| Preliminary (Raw) Mapping (QC item 7) |  |
| X | Preliminary 10%, 1%, 0.2% ACE mapping results (raster grids from HEC-RAS) |
| X | Modeler to complete topology validation as outlines in the guidelines |
| X | Modeler to verify mapping meets deliverable requirements as outlined in guidelines |
| Completeness Review (QC item 8) See Texas 2D BLE Guidelines Chapter 4 – Deliverables |  |
| X | Technical BLE Report |
| X | Final cleaned hydrologic model |
| X | Final cleaned hydraulic model |
| X | Flood risk database |
| X | CMNS updates |
| X | HAZUS results |
| X | Additional supplemental data |

# 1 – Model Terrain

Methodologies / Data Sources (Modeler to populate)

| Terrain Source(s): | Texas West Central LiDAR, Brown County LiDAR, Hurricane LiDAR, Brazos River Basin LiDAR, Bandera & Lampasas Counties LiDAR, Upper Clear Fork Brazos River LiDAR, and the USGS National Elevation Dataset (NED) LiDAR |
| --- | --- |
| Data Gaps: | None |
| Source Resolution: | Cell size of 1 x 1 meter |
| Model Terrain Resolution: | Cell size of 5 x 5 feet |
| Terrain Horizontal Projection: | North American Datum (NAD) 1983 (2011) Texas State Plane Zone 3 (Central) |
| Terrain Vertical Projection: | NAVD88 |
| Z Values: | Units of feet |
| Terrain Buffer: | 2,500 feet |
| Other: |  |

## Technical Review (Reviewer to provide comments)

Reviewer is to utilize detailed QA/QC to identify general review items for this audit.

## Review Comments

- Terrain was not reviewed with H&H modeling.

# 2 – Hydrology

## Methodologies / Data Sources (Modeler to populate)

| Software: | HEC-RAS 6.2 |
| --- | --- |
| Rainfall Source: | NOAA Atlas 14 |
| Rainfall Duration: | 24 hours |
| Depth Selection: | Domain Average |
| Rainfall Temporal Distribution: | NRCS |
| Areal Reduction: | None |
| Loss Methodology: | Spatially varied CN |
| Unit Hydrograph Transform Method: | N/A |
| Inflow Hydrograph Source: | Varies by model |
| Other: |  |

## Statistical Data (Modeler to populate)

| Stream Gage Name & ID: | 08143500 Pecan Bayou at Brownwood, TX |
| --- | --- |
| Stream Gage Name & ID: | 08143600 Pecan Bayou near Mullin, TX |
| Stream Gage Name & ID: |  |

## Technical Review (Reviewer to provide comments)

Reviewer is to utilize detailed QA/QC to identify general review items for this audit.

## Review Comments

- The curve numbers used in the HEC-RAS models should be rounded to whole numbers.

Response: The initial CN values for the HEC-RAS models were obtained from Table 2.4 of the TWDB’s 2D BLE Guidance. The initial CN values were whole rounded numbers. During validation process the curve numbers were adjusted within +/- 10% of initial CN values. Rounding curve number values would have slightly changed the % adjustment for each category, so the validated CN values were not rounded to have same consistent adjustment for all categories.

No additional comments.

- Computation spreadsheets provided for 1% plus/minus calculations do not align with the TWDB BLE Guidelines. Please provide documentation of equations/methodology used to compute the 1% plus/minus precipitation depths.

Response: The 1% plus and 1% minus rainfall totals are now calculated using TWDB 2D BLE Guidance and documentation included in the BLE report.

No additional comments.

- Methodology for rainfall hyetograph development not provided, please provide methodology discussion and HMS model used if applicable.

Response: To develop rainfall hyetograph basin averaged NOAA rainfall totals were calculated for 5-min to 24-hour durations (5, 10, 15, 30, 60 mins 2, 3, 6, 12 and 24hr). Then NRCS temporal distribution was applied to obtain rainfall hyetograph. The HEC-HMS was not used developing rainfall data and read me files added in the hydrology submittal folder.  Documentation included in the BLE report.

No additional comments.

# 3 – Hydraulic Geometry

## Methodologies / Data Sources (Modeler to populate)

| Software: | HEC-RAS 6.2 |
| --- | --- |
| Loss Methodology: | Spatially varied CN |
| Soils Source: | SSURGO |
| Land Cover Source: | NLCD 2019 |
| Manning's Roughness Source: | Assigned based on land use. Please see table in Brady Model Overview and Results.xlxs |
| Nominal Mesh Cell Sizes: | 200x200, except for Channel Refinement Regions, Breaklines and Urban Regions where 100x100 was considered. |
| Other: |  |

## Technical Review (Reviewer to provide comments)

Reviewer is to utilize detailed QA/QC to identify general review items for this audit.

## Review Comments

- According to the provided excel file titled “Pecan Bayou Model Overview and Results”, the manning’s value for grassland/herbaceous and cultivated crops should both be 0.035, but the models show these both as 0.045.

Response: The Manning’s n values in this revision have been updated to match the TWDB 2D BLE Guidance (Version 1). The cultivated crop n value was updated to 0.035 and grassland/herbaceous value of 0.038 was used per the guidance.

No additional comments.

- Consider smoothing the stream centerlines being used as breaklines in order to simplify the cell faces.

Response: The channel breaklines have been now smoothened and an example screenshot shown below.

No additional comments.

- Planarize the lines used for breaklines to simplify cell geometry.

Response: The breaklines have been planarized and an example screenshot shown below.

No additional comments.

- Remove conflicting breaklines to simplify cell geometry

Response: Removed conflicting breaklines where possible to simplify mesh.

No additional comments.

- Breaklines should be added along any results lines used as inflow data or flow comparisons/calibrations to ensure results lines align to cell faces.

Response: Breaklines are added and enforced at flow validation locations.

No additional comments.

- Any results lines (and corresponding breaklines) and 2D connections should be drawn perpendicular to the channel and floodplain.

Response:  We have reviewed and revised validation location profile lines and SA/2D connections to be perpendicular to flow.

No additional comments.

- Overlap areas between model domains are not consistently applying breaklines and or refinement regions along the streams.

Response:  Added missing refinement region for consistency in the overlapping areas. Since domains were processed separately the breaklines and refinement regions between adjacent domains in the overlap areas are very similar but may not be exactly same.

No additional comments.

# 4 – Hydraulic Boundary Conditions

## Technical Review (Reviewer to provide comments)

Reviewer is to utilize detailed QA/QC to identify general review items for this audit.

## Review Comments

- Upstream boundary condition line for US inflows needs to lie outside of the 2D area, otherwise flow will not all be directed DS as intended. Recommended that US models are extended to have sufficient overlapping area but the DS extents are limited to start exactly at the location of the 2D connection of the US model. This allows the US boundary condition to be external and not have any duplicate contributing rainfall areas upstream of the inflow location.

Response:  This change was not applied for following reasons:

- During ITR comment discission held on 05/04/2022 between Atkins and Halff staff, it was agreed to add inflows as internal boundary conditions for downstream model and this approach is used for Pecan Bayou and other watershed models developed in this task order.

- Difference between adding internal versus external BCLine is minimal

- Having small overlap upstream of SA/2D can help in some cases with smoother tie-in

No additional comments.

- DS end of 706_A7 boundary – either adjust the 2D extents to not include the connecting stream at all or adjust boundary conditions. Flow is currently allowed to leave from both the US and DS sections of the adjoining stream with the ND boundary condition wrapping around both ends.

Response:  Pulled the 2D extent back to just upstream of the Colorado River confluence. The downstream boundary condition type has been changed to stage hydrograph to account for Colorado River backwater. Please see screenshot below.

No additional comments.

- Number (1 and 2) from 705_A5 2D connection lines to 706_A7 Inflow boundary conditions do not match up in naming, make sure these names and hydrograph applications align.

Response:  BCLine names for the two 0704_A5 inflows into 706_A7 model domain are:

- Inflow_from_0704_A5_1

- Inflow_from_0704_A5_2

Since the inflow names are self-explanatory the BCLine names are not updated.

No additional comments.

- Outflow from 703_A4 at the reservoir is very different from the hydrographs used as inflows to 704_A5, the 500-year peak outflow is 46% higher than the peak inflow to the 704_A5.

Response:  Model setup for this area has been updated and outflow hydrograph from 0703_A4 domain is added as an inflow into 0704_A5 model domain.

No additional comments.

# 5 – Hydraulic Model Execution

## Methodologies / Data Sources (Modeler to populate)

| Unsteady Plan Names |  |  |  |
| --- | --- | --- | --- |
| 10% ACE (10-year): | NRCS_10_PCT | 0.2% ACE (500-year): | NRCS_0.2_PCT |
| 4% ACE (25-year): | NRCS_4_PCT | 1% plus: | NRCS_1.0_PLU |
| 2% ACE (50-year): | NRCS_2_PCT | 1% minus: | NRCS_1.0_MIN |
| 1% ACE (100-year): | NRCS_1.0_PCT |  |  |
| Other: |  |  |  |

## Statistical Data (Modeler to populate)

| Time Step: | 30 s - Advanced time step based on Courant condition (7.5-60 Seconds) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Courant Value: | 0.45 – 2.0 |  | Simulation Length: | 48 to 120 | hours |
| Computation Interval: | (variable) 30 | seconds | Model Runtime: | Generally 3 to 5 | hours |
| Hydrograph Output Interval: | 10 | minutes | Mapping Output Interval: | 10 | minutes |

## Technical Review (Reviewer to provide comments)

Reviewer is to utilize detailed QA/QC to identify general review items for this audit.

## Review Comments

- No additional comments

# 6 – Hydraulic Model Results

## Methodologies / Data Sources (Modeler to populate)

| Flow Validation/Gage Analysis |  |
| --- | --- |
| Software: | HEC-RAS v 6.2 |
| Methodology | HEC-RAS 2D – rain-on-grid |
| Removal of Outliers and Justification | Refer to BLE report |
| Validation Notes (describe parameter adjustments): Refer to BLE report and Model Overview and Results Spreadsheet |  |
| Other: |  |

## Technical Review (Reviewer to provide comments)

Reviewer is to utilize detailed QA/QC to identify general review items for this audit.

## Review Comments

- Floodplains are consistently noted to be smaller than the effective and flows are generally shown to be smaller than FIS flows, please provide an explanation for these decreases in the report.

Response:  The InFRM study is the best available hydrologic data for the Pecan Bayou watershed. At the two gage locations in the watershed, BLE flows compared favorably with the InFRM study. Effective flows are higher than BLE flows as review comment mentioned. However, please note that flows reported in the 2018 effective FIS appear to be 3 to 5 times higher than the previous effective study in 1980 and there is no documentation available for such significant flow increase.  Discussion added in the BLE report.

No additional comments.

# 7 – Preliminary (Raw) Mapping Results

## Technical Review (Reviewer to confirm with Pass/Fail)

| Hydraulic Model |  |
| --- | --- |
|  | HEC-RAS model with results, pertinent data and required storm events |
| Preliminary Mapping 1% ACE |  |
|  | Preliminary 10%, 1%, 0.2% ACE mapping results (raster grids from HEC-RAS) to meet FEMA mapping requirements |
|  | Boundary interfaces align matching or identified as AOMI |

## Review Comments

- WSEs do not seem to tie in within 0.5-ft of each other at the interface of 703_A3 and 703_A4 or at 705_A6 and 706_A7

Response:  Updated all model domains in Pecan Bayou watershed to tie-in within 0.5’ with adjacent model. Stage hydrograph downstream boundary condition is used to achieve tie-in.

No additional comments.

# 8 – Completeness Review

## Technical Review (Reviewer to confirm with Pass/Fail)

| File Structure and Naming |  |
| --- | --- |
|  | Deliverables provided in TWDB/FEMA required file folder structure and naming convention |
|  | Data missing (if any) documented in report and ReadMe file within the required folder |
| Report |  |
|  | Technical 2D BLE report documents all assumptions, special considerations, etc. |
|  | Flow validation and difference discussed in report |
|  | Flow comparisons discussed in report |
| Hydrologic Model |  |
|  | HEC-HMS model with results, pertinent data and required storm events |
| Hydraulic Model |  |
|  | HEC-RAS model with results, pertinent data and required storm events |
| Flood Risk Database |  |
|  | All features fully populated per current FEMA guidelines |
|  | Mapping consistent between grids to polygons for all three required mapped events |
|  | 10yr smaller than 100yr smaller than 500yr |
|  | Holes filled in polygon, depth and WSE grids |
|  | Boundary interfaces align matching or identified as AOMI |
| CNMS |  |
|  | Updated correctly with new centerlines |
|  | A1-A5 checks complete |
|  | CNMS Post- Processing Complete |
| HAZUS |  |
|  | Run per FEMA guidance |

## Review Comments

- FRD

  - The coordinate system for the feature datasets should be in a Geographic Coordinate System (GCS) not in a Projected Coordinate System (PCS).

Halff Backcheck 5/30/2023: Projected Coordinate Systems were correctly converted to Geographic Coordinate Systems. However, the feature datasets are using the 2011 NAD 83 Datum. BLE guidance specifies using the old NAD83 datum.

Atkins Response 6/5/2023: Feature dataset coordinate systems have updated to NAD83.

Halff Backcheck 6/14/2023: No further comment

  - BFE

    - For BFE_LN_TYP, lines within DTL_STUD_AR should be changed to “NOT MAPPED”

Atkins Response 02/07/2023:  BFE lines within DTL_STUD_AR locations were attributed with unmapped line type to ensure that BLE elevations will not be visible on the eBFE viewer in detail study areas. BFE_LN_TYP for all other features was set to ‘MAPPED’

Halff Backcheck 5/30/2023: No Further Comment

  - DTL_STUD_AR

    - DTL_LN_ID - needs to be updated with HUC8#_BLE

    - CD_POC - recommend populating with “NP”

    - CD_ADD1 - recommend populating with “NP”

    - CD_ADD2 - recommend populating with “NP”

    - CD_CITY - recommend populating with “NP”

    - CD_STATE - recommend populating with “NP”

    - CD_ZIP - recommend populating with “NP”

    - CD_PHONE - recommend populating with “NP”

    - CD_EMAIL - recommend populating with “NP”

    - LOMER_ID - recommend populating with “NP”

    - SCALE- is missing

    - STAUS - recommend populating with “NP”

Atkins Response 02/07/2023:  Updated fields identified in the comment with NP values and panel scale is now added.

Halff Backcheck 5/30/2023: No Further Comment

    - Recommend making polygons single part features

Atkins Response 02/07/2023:  All polygons were converted to single part. Where detail study span across multiple panels, polygons within same panel were merged to preserve the FIRM panel numbers. Please see the screenshot below.

Halff Backcheck 5/30/2023: No Further Comment

  - DTL_STUD_LN

    - DTL_LN_ID - needs to be updated with HUC8#_BLE

Atkins Response 02/07/2023:  This item was discussed in a meeting between TWDB, Halff, and Atkins on 02/10/2023 and ITR team clarified that either of HUC8# or HUC8#_BLE is acceptable but has to be consistent across all feature classes in the database. We have consistently used HUC8#for Pecan BLE database.

Halff Backcheck 5/30/2023: No Further Comment

    - CD_POC - recommend populating with “NP”

    - CD_ADD1 - recommend populating with “NP”

    - CD_ADD2 - recommend populating with “NP”

    - CD_CITY - recommend populating with “NP”

    - CD_STATE - recommend populating with “NP”

    - CD_ZIP - recommend populating with “NP”

    - CD_PHONE - recommend populating with “NP”

    - CD_EMAIL - recommend populating with “NP”

    - LOMER_ID - recommend populating with “NP”

    - SCALE- is missing

    - STAUS - recommend populating with “NP”

Atkins Response 02/07/2023:  Populated CD_POC, CD_ADD1, CD_ADD2, CD_CTY, CD_STATE, CD_ZIP, CD_PHONE, CD_EMAIL, LOMR_ID, and STATUS field values with NP, and panel scale added

Halff Backcheck 5/30/2023: No Further Comment

  - FLD_HAZ_AR

    - STUD_TYP - recommend populating with “NP”

Atkins Response 02/07/2023:  STUDY_TYP field is now updated with NP values.

Halff Backcheck 5/30/2023: No Further Comment

    - AR_REVERT – recommend populating with empty string “”

    - AR_SUBTRV - recommend populating with empty string “”

Atkins Response 02/07/2023:  Per the review comment discussion meeting between TWDB, Halff and Atkins on 02/10/2023, the AR_REVERT, AR_SUBTRV will use empty string “”. The values are updated.

Halff Backcheck 5/30/2023: Fields have not been updated yet. Please update with empty string (“”)

Atkins Response 6/5/2023: AR_REVRT and AR_SUBTRV updated with empty string

Halff Backcheck 6/14/2023: No further comment

  - TENPCT_FP

    - STUDY_TYP- recommend populating with “NP”

Atkins Response 02/07/2023:  STUDY_TYP field is now updated with NP values.

Halff Backcheck 5/30/2023: No Further Comment

    - AR_REVERT – recommend populating with empty string “”

    - AR_SUBTRV - recommend populating with empty string “”

Atkins Response 02/07/2023:  Per the review comment discussion meeting between TWDB, Halff and Atkins on 02/10/2023, the AR_REVERT, AR_SUBTRV will use empty string “”. The values are updated.

Halff Backcheck 5/30/2023: Fields have not been updated yet. Please update with empty string (“”)

Atkins Response 6/5/2023: AR_REVRT and AR_SUBTRV updated with empty string

Halff Backcheck 6/14/2023: No further comment

    - DUAL_ZONE – Change to “False (No)” for all

Atkins Response 02/07/2023:  DUAL_ZONE is now updated “False (No)”.

Halff Backcheck 5/30/2023: No Further Comment

  - S_AOMI_AR

    - Feature class is empty. Are there no AMOIs?

Atkins Response 02/07/2023:  Compared the NLCD 2019 land cover dataset to NLCD 2011 and identified development clusters. Added features to S_AOMI_Ar.

Halff Review (05/30/2023): Development clusters are fine as is. However, “for floodplain tie-in locations along the HUC8 boundary, where no floodplain or WSEL information is available from adjacent watershed”, they need to be put in S_AOMI_AR as a polygon not S_AOMI_Pt as per TWDB BLE guidance below.

Atkins Response 6/5/2023: Adjacent HUC tie-in comments are added in S_AOMI_Ar. Please note that mapping for Jim Ned HUC8 was completed before Pecan modeling was finalized. As a result, the perimeter mapping for Jim Ned Watershed will be updated along the Jim Ned - Pecan HUC8 boundary. So, no AOMI_Pts are added along this boundary.

Halff Backcheck 6/14/2023: No further comment

  - S_FRAC_AR

    - TOT_LOSS01 – re-calculate 0 to -9999 per TWDB request

Atkins Response 02/07/2023: Updated 0 to –9999 per request.

Halff Backcheck 5/30/2023: No Further Comment

    - SCENAR_ID – Recommend populating with sequential numbers

Atkins Response 02/07/2023:  Per the review comment discussion meeting between TWDB, Halff and Atkins on 02/10/2023, the values in SCENAR_ID field are updated with sequential numbers.

Halff Backcheck 5/30/2023: field has not been updated yet. Please update with sequential numbers.

Atkins Response 6/5/2023: Sequential number for SCENAR_ID are now added

Halff Backcheck 6/14/2023: No further comment

    - SOURCE_CIT – double check source citation. Should be a REF# not a Study#

Atkins Response 02/07/2023:  Source CIT value updated

Halff Backcheck 5/30/2023: No Further Comment

  - S_HUC_AR

    - SOURCE_CIT - double check source citation. Should be a Base# not a Study#

Atkins Response 02/07/2023:  Updated Source_Cit to BASE2.

Halff Backcheck 5/30/2023: No Further Comment

  - S_POL_AR

    - DFIRM_ID – Update by populating ST_FIPS + CO_FIPS

Atkins Response 02/07/2023:  Updated communities with DFIRM_ID to use ST_FIPS + CO_FIPS+”C format

Halff Backcheck 5/30/2023: No Further Comment

    - COM_NFO_ID – Populate with “CID” column

Atkins Response 02/07/2023:  COM_NFO_ID with CID

Halff Backcheck 5/30/2023: No Further Comment

  - L_Source_Cit

    - Should “STUDY2” be “REF2” since it’s HAZUS? Only one REF is needed for HAZUS even if it was run multiple times. This can probably be deleted entirely.

Atkins Response 02/07/2023:  Updated to REF1

Halff Backcheck 5/30/2023: No Further Comment

    - Recommend dropping “a” and “b” from (FEMA, 2022) citations. Since the “SOURCE_CIT” is different its ok it “CITATION” is the same.

Atkins Response 02/07/2023:  Letters dropped from CITATION

Halff Backcheck 5/30/2023: No Further Comment

- MAPPING

  - FLD_HAZ_AR

    - Mapping is missing for the Jim Ned HUC that is a part of this project

Atkins Response 02/07/2023:  Portion of Jim Ned was modeled in Pecan and Jim Ned mapping will be updated incorporate this additional floodplain in the near future.

Halff Backcheck 5/30/2023: Please coordinate with TWDB and Halff on the resubmittal and review of the completed Jim Ned mapping. Please note, fringe mapping tie ins should be reviewed between Pecan Bayou and Jim Ned to meet FEMA’s mapping requirements of ±0.5 ft.

Atkins Response 6/5/2023: We agree with your comment, and we plan to update fringe/ perimeter mapping for Jim Ned HUC8 to tie-in with Pecan in the near future.  Please note that mapping for Jim Ned HUC8 was completed before Pecan modeling was finalized so current Jim Ned FRD does not reflect Pecan mapping updates. As a result, the perimeter mapping for Jim Ned Watershed will be updated along the Jim Ned - Pecan HUC8 boundary where, no AOMI_Pts are added along this boundary.

Halff Backcheck 6/14/2023: No further comment

    - Recommend Deleting holes and islands that are smaller than 2 acres for 100yr and 500yr polygons

Atkins Response 04/10/2023:  Based on direction from TWDB on 02/21/2023, holes smaller than 2 acres were filled for 100yr and 500yr floodplain polygons (Fld_Haz_Ar). Islands in floodplain were mapped using the TWDB matrix, which selectively removed portion of the shallow flooding areas from the processed floodplain layer per the area of flood polygon and average flood depth (shown above).

Halff Backcheck 5/30/2023: No Further Comment

    - Mapping for 100 and 500 years need to be connected through well-defined spillways

Atkins Response 02/07/2023:  We have added breaklines in HEC-RAS model to capture flow over well-defined spillways. However, where there is not enough volume overtopping spillway or there is steep slope at the spillway mapping may not be continuous.

Atkins Response 04/10/2023: Based on direction from TWDB on 02/21/2023, disconnected 100-YR and 500-YR floodplains from the model at the secondary spillways were manually connected. The TWDB suggested only performing this revision for dams located within the NID database, which had normal pool storage of 200 ac-ft, to limit the manual revisions. The Fld_Haz_Ar layer, depth and WSE grids have been revised to match updated floodplain extent.

Halff Backcheck 5/30/2023: No Further Comment

  - TENPCT_FP

    - Recommend deleting features smaller than 0.5 acres

Atkins Response (04/10/2023):  Based on direction from TWDB on 02/21/2023, all islands smaller than 0.5 acres were deleted and holes smaller than 0.5 ac were filled for the TENPCT_FP layer. Since the 10 % floodplain is contained within the 100YR floodplain, comment about deleting islands smaller than 2 ac. outside 100yr boundary does not apply.

Halff Backcheck 5/30/2023: No Further Comment

    - Recommend deleting features smaller than 2 acres that are outside of the 100 year floodplain boundary

Atkins Response 04/10/2023:  Since the 10 % floodplain is contained within the 100YR floodplain, comment about deleting islands smaller than 2 ac. outside 100yr boundary does not apply.

Halff Backcheck 5/30/2023: No Further Comment

  - GRIDS

    - Areas exist where 10 percent grids extend further than 1 percent and where 1 percent extends further than 0.2 percent Grids.

Atkins Response 02/07/2023:  Merged 10-year floodplain into 100-year and used erase function to ensure there are no slivers. Similarly, the 100-year floodplain was merged into the 500-year checked for slivers.

Halff Backcheck 5/30/2023: No Further Comment

    - Some mapping tie-ins between Grids exceed the 0.5ft tolerance.

Atkins Response 02/07/2023:  We have checked the model WSELs at the model boundaries and updated mapping to tie-in within 0.5’. For floodplain tie-in locations along the HUC8 boundary, where no floodplain or WSEL information is available from adjacent watershed, were marked as AOMI locations and added to AOMI_Pt feature class. Please note that Jim Ned watershed mapping was finalized before Pecan, so perimeter areas where Pecan floodplain is wider the Jim Ned mapping will be updated to tie-in with Pecan.

Halff Backcheck 5/30/2023: model tie-ins still have areas that exceed 0.5ft tolerance. Please refer to QC polygon for these locations. Pull in the WSE GRIDs and turn on “Dynamic Range Adjustment (DRA) to see the differences more easily in cell values when zoomed in close.

Atkins Response 6/5/2023: We have updated the mapping and grids for two locations identified in the Pecan_MappingQC.shp to resolve the tie-in inconsistencies. The locations identified in the  Fringe_TieIns.shp are all along the Jim Ned – Pecan HUC8 boundary. As discussed earlier, mapping for Jim Ned HUC8 was completed before Pecan modeling was finalized so fringe/ perimeter mapping for Jim Ned HUC8 will revised match with Pecan in the near future. We have been using DRA option during grid review.

Halff Backcheck 6/14/2023: No further comment

  - S_STUDIES_LN

    - See CNMS comments Below…

  - WTR_LN

    - Delete lines floating outside of HUC 8 boundary

Atkins Response 02/07/2023:  Water lines outside of HUC8 boundary removed

Halff Backcheck 5/30/2023: No Further Comment

    - Recommend re-aligning lines inside of the floodplain

Atkins Response 02/07/2023:  WTR_LN features were reviewed and updated such that streamlines were inside floodplain expect at some of elevated roadway crossings or some dams.

Halff Backcheck 5/30/2023: No Further Comment

    - Recommend re-aligning lines through well-defined spillways around dams

Atkins Response 02/07/2023:  Based on recent direction from TWDB and Halff it is acceptable to put the streamline alignment through the main dam outlet and follow spillway.

Halff Backcheck 5/30/2023: No Further Comment

  - WTR_AR

    - Recommend deleting Features that are smaller than 5acres and un-named

Atkins Response 02/07/2023:  Removed water areas that were smaller than 5acres

Halff Backcheck 5/30/2023: No Further Comment

    - Recommend merging remaining features into floodplains 10/100/500 years

Atkins Response (04/10/2023):  Per the direction provided by TWDB on 02/21/2023, WTR_Ar features were merged into 100- and 500-year floodplains (Fld_Haz_Ar). It was directed not to merge WTR_AR features into 10-YR FP (TENPCT_FP).

Halff Backcheck 5/30/2023: No Further Comment

- CNMS

  - Halff Backcheck 5/30/2023: (General CNMS comment) – Due to questions regarding the inconsistencies between the FRD Domains and CNMS domains, it is the advice of Michael Johnson (FEMA Region 6 CNMS Lead) that the current best practice is to completely delete the CNMS features within the FRD and replace them with the S_Studies_Ln and S_Unmapped _Ln feature classes from the stand alone CNMS gdb. This will maintain the CNMS Schema and Domains for the CNMS feature classes. You may notice some similar/duplicate domain names with different code values in ArcCatalog but MJ has assured us that this is a known issue and that previous submittals done this way have not been an issue.

  - S_Studies_Ln

    - Geometry

      - Some disconnected lines floating. Recommend connecting.

Atkins Response 02/07/2023: New linework was generated for the wtr_ln feature class. That linework was added as Zone X to s_studies_ln and edited to connect to existing CNMS linework

Halff Backcheck 5/30/2023: No Further Comment

      - Unmapped lines not removed yet.

Atkins Response 02/07/2023:  unmapped lines are now removed

    - Table

      - “REASON” field for Zone X streams are inconsistent. Should all be the same since it’s the same study and project?

Atkins Response 02/07/2023: Updated Reason field for Zone X stream to be consistent.

Halff Backcheck 5/30/2023: No Further Comment

      - “FBS_CHKD” has 3 different dates. Should they be the same?

Atkins Response 02/07/2023: FBS_CHKD is only updated for new Zone X streams in this CNMS update and they were not updated for Zone A and Zone AE streams as part of this update. FBS dates for Zone A and Zone AE streams are typically updated after regulatory products are produced.

Halff Backcheck 5/30/2023: No Further Comment

      - “FB_CTYP” for all Zone X should be INDIVIDUAL REACH ATTRIBUTION

Atkins Response 02/07/2023: Updated FBS_CTYP is individual reach attribution for all Zone X streams

Halff Backcheck 5/30/2023: No Further Comment

      - “A5_CMT”, “A5_SRC”, “A5_URL” need to be populated

Atkins Response 02/07/2023: updated these fields

Halff Backcheck 5/30/2023: Fields not yet updated.

  - S_UnMapped

    - Dataset is empty.

Atkins Response 02/07/2023:  Since BLE floodplain mapping extend upstream of new Zone X and there were no locations where new Zone X fell outside the new BLE floodplain, the S_Unmapped_Ln layer was empty.

    - Cut out tails from S_Studies_Ln that extend past floodplain extents and add them to S_Unmapped. Lines that are completely within another streams backwater also need to be removed from S_Studies_Ln and moved to S_UnMapped

Atkins Response 02/07/2023:  Since BLE floodplain mapping extend upstream of new Zone X and there were no locations where new Zone X fell outside the new BLE floodplain, the S_Unmapped_Ln layer was empty.

Halff Backcheck 5/30/2023: S_Studies_Ln completely within other lines backwater should be moved to S_Unmapped or attributed withing S_Studies_Ln according to CNMS guidance. Please refer to QC polygon for examples.  look for other instances throughout.

- HAZUS

  - Couldn’t open .hpr file due to version differences but did a manual review of the S_FRAC_AR layer compared to the 1pct depth grid and 2020 census blocks. Everything looked good and seems to have been run correctly.

Atkins Response 02/07/2023:  Noted.

- Folder Structure

Halff Backcheck 5/30/2023: Please update folder structure to follow the EBFE deliverable format provided by TWDB.

  - Flood Risk Products

    - Task Documentation – Missing Metadata and Report

Atkins Response 04/10/2023: Added Metadata to task documentation folder. We are working on adding some additional details to the BLE report and will email it in next few days. There were no specific comments on the report in the last submittal.

  - Hydraulics

    - Missing Model Inventory

Atkins Response 04/10/2023: Added

    - Supplemental – Recommend providing raw grids

Atkins Response 04/10/2023: Raw grids are included in the supplemental data folder for mapping (\FRP\12090107\Final\Flood_Risk_Products\Supplemental_Data\RAS_output)

    - Hydraulics DCS missing (appears to be copy of FRD)

Atkins Response (4/10/2023): Per direction from TWDB and Halff from Mapping Workshop on 03/30/2023, the DCS layers will not be required. A copy of the FRD is included.

    - Task Documentation – Missing Metadata and Report

Atkins Response 04/10/2023: Added Metadata to task documentation folder. We are working on adding some additional details to the BLE report and will email it in next few days. There were no specific comments on the report in the last submittal.

  - Terrain

    - Missing Terrain DCS

Atkins Response 04/10/2023: Not required per Mapping Workshop on 03/30/2023.

    - Supplemental – Missing LiDAR Reports

Atkins Response 04/10/2023: Added LIDAR reports under supplemental data folder of terrain submittal.

    - Task Documentation – Missing Metadata

Atkins Response 04/10/2023: Not required per Mapping Workshop on 03/30/2023.

8/21/2024 – TENPCT_FP, FLD_HAZ_AR, and the Depth and WSE rasters for both the 1% and 0.2% events were re-built to address tie-in issues with adjacent HUC8s. The comments below represent the quality control review of these modified features.

- FLD_HAZ_Ar

Geometry:

- There are some features in Zone X with a NULL or empty geometry area size. These appear to be tiny slivers that were left behind after erasing the 0.2% with the 1%. I recommend deleting these features since their size is insignificant.

AtkinsRéalis Backcheck Response 01/03/2025: Features with null geometry removed.

Halff Backcheck 2/12/2025: Not addressed. Recommend creating a topology and removing features smaller than the cluster tolerance specified in the Region 6 BLE Submittal Guidance on page 30.

AtkinsRéalis Backcheck Response 04/17/2025:  Ran repair geometry, then topology, then repair geometry, then topology all again.  All features smaller than cluster tolerance (approx.. 50 records) have been removed.  Unique IDs recalculated.

Attribute Table:

- V_DATUM should be populated with Null or “ ” (empty string). This field is only populated when STATIC_BFE field is populated.

AtkinsRéalis Backcheck Response 01/03/2025: Fields populated with empty string

Halff Backcheck 2/12/2025: Corrected. No further comments.

- LEN_UNIT should be populated with Null or “ ” (empty string). This field is only populated when STATIC_BFE or DEPTH fields are populated.

AtkinsRéalis Backcheck Response 01/03/2025: Fields populated with empty string

Halff Backcheck 2/12/2025: Corrected. No further comments.

- TENPCT_FP

Geometry:

- No Comments

Attribute Table:

- ZONE_SUBTY field should be populated with Null or “ ” (empty string) not “NP” since it is not a required field.

AtkinsRéalis Backcheck Response 01/03/2025: Fields populated with empty string

Halff Backcheck 2/12/2025: Corrected. No further comments.

- V_DATUM should be populated with Null or “ ” (empty string). This field is only populated when STATIC_BFE field is populated.

AtkinsRéalis Backcheck Response 01/03/2025: Fields populated with empty string

Halff Backcheck 2/12/2025: Corrected. No further comments.

- LEN_UNIT should be populated with Null or “ ” (empty string). This field is only populated when STATIC_BFE or DEPTH fields are populated.

AtkinsRéalis Backcheck Response 01/03/2025: Fields populated with empty string

Halff Backcheck 2/12/2025: Corrected. No further comments.

- Grids

  - Internal modeling tie-in between Domain_0703_A4 and Domain_0704_A5 exceed 0.5ft tolerance. Please see QC_Polygon_20240813.shp for location.

AtkinsRéalis Backcheck Response 01/03/2025: BLE_WSE0_2PCT updated and corrected to meet the 0.5ft tie-in tolerance. Universal check to confirm no similar issues were found.

Halff Backcheck 2/12/2025: Corrected. No further comments.

- Halff Backcheck 2/12/2025 – External Tie-In was reviewed against Jim Ned HUC8. Tie-Ins between HUCs meet 0.5ft tolerance and we have no further comments.

- Note: Tie-ins to Middle Colorado HUC8 could not be complete at this time as they were not included with the M3 submittal.

QA/QC Checklist Approval

This 2D Base Level Engineering Analysis QA/QC Review is in compliance with the contract requirements and all task checkpoints are complete.  The independent QC Team has reviewed the analysis, presented review comments to the Production Team Task Leader, and discussed any problems or issues.  The Task Leader, QC Manager, QC Reviewer, and Project Manager have signed the QA/QC Checklist to confirm that all comments have been received, addressed, and documented appropriately.

| QA/QC Team – [TWDB or company name] |  |  |  | Production Team – [add company name] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| QC Manager |  | Date |  | Task Lead |  | Date |
| QC Reviewer |  | Date |  | Project Manager |  | Date |