# Texas 2D BLE Analysis Audit Checklist

# Project Information

| HUC-8 Watershed Name & ID: | Brady: 12090110 |
| --- | --- |
| HUC-10 Watershed Name & ID: | Upper Brady Creek: 1209011001 Lower Brady Creek: 1209011002 |
| Modeler: | Atkins |
| Reviewer: | Halff Associates |

This audit process includes only high-level review for items identified in the Detailed Review for another HUC for the same consultant. The consultant is responsible for performing a detailed review internally for base data, hydrology, hydraulics, and modeling results.

## Audit 1: Terrain, Modeling, Preliminary Mapping Review (QC Items 1-7)

| Modeler Initials: | JG | Reviewer Initials: | KL |
| --- | --- | --- | --- |
| Submittal Date: | 5/3/22 | Review Submitted Date: | 7/14/2022 |
| Review Response Date: | September 23, 2022 | Backcheck Complete Date: |  |

## Audit 2: Completeness Review (QC Item 8)

| Modeler Initials: | JG | Reviewer Initials: | JM |
| --- | --- | --- | --- |
| Submittal Date: | September 23, 2022 | Review Submitted Date: | 5/10/2023 |
| Review Response Date: |  | Backcheck Complete Date: |  |

(Modeler to populate with an “X” for items included in the QC submittal)

| Level of Study |  |  | Flood Events |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| x | 2D BLE |  | x | 10% ACE (10-year) | x | 0.2% ACE (500-year) |
|  | Enhanced BLE |  | x | 4% ACE (25-year) | x | 1% plus |
|  |  |  | x | 2% ACE (50-year) | x | 1% minus |
|  |  |  | x | 1% ACE (100-year) |  | Observed Rainfall |

Instructions

Each section includes instructions to the modeler and reviewer regarding their responsibilities in the completion of this checklist. In the Methodologies / Data Sources tables, grey italic text is added as an example response. Additional sample responses are located in the Texas 2D BLE Guidelines Quick Guide.

## Submittal Checklist

Data submitted for each milestone should be carried through each subsequent milestone. All geospatial layers and models must have same defined projection system as terrain. Modeler to populate with an “X” for items included in the QC submittal.

| Base Map Data (QC items 1-6) |  |
| --- | --- |
| x | Location Map (PDF): HUC-8 and HUC-10 boundaries and/or model domain boundary if different from HUC-10s, stream gages, dams (if applicable), mainstem streams (for reference), background map |
| X (Rasmapper) | GIS Map (MXD or Map Package) with the following data included but not limited to: Stream centerlines, model breaklines, rainfall data, WSEL and Depth grids (10, 100, 500-yr), adjacent HUC mapping (available on the BLE viewer or currently studying), 2D connections for inflows/outflows, gages or points of flow comparisons (BFEs, highwater marks, etc.), profile lines, boundary condition lines, refinement regions, model domain boundary, model mesh, DFIRM mapping panels, base map data (roads, aerials, etc.), soils data, land use data, terrain |
| Technical Report / Documentation (QC items 1-6) |  |
| x | Technical Report / Project Notebook: assumptions, special considerations, and modeling concerns |
| x | Detailed explanation and support data of variances in methodologies and approaches from recommended guidelines (Should be included in Technical Report / Project Notebook) |
| x | Internal QA/QC Documentation: comments and responses |
| x | Flow comparison tables within the technical report |
| Hydrology (QC items 1-6) |  |
| x | Hydrologic Model (HEC-HMS) model with all associated DSS files for 7 required storm events |
| x | Hydrologic Data and Parameter Spreadsheets (rainfall calculations, curve number table, gage locations, gage analysis and flow validation) |
| x | Hydrologic modeling notebook including assumptions, notes, and special conditions (if not provided in report) |
| Hydraulics (QC items 1-6) |  |
| x | Hydraulic Model (HEC-RAS) with all associated files and 7 required storm events (remove non-final files)d |
| x | Hydraulic Data and Parameter Spreadsheets (Manning’s n-value table, infiltration calculations support data-Soils, Land Use, Curve Number table, other calculations/data used in model) |
| x | Manning’s n-value table |
| x | Precipitation data correctly linked to hydrologic model or hard coded from provided spreadsheet |
| x | Hydraulic modeling notebook including assumptions, notes, and special conditions |
| Preliminary (Raw) Mapping (QC item 7) |  |
| x | Preliminary 10%, 1%, 0.2% ACE mapping results (raster grids from HEC-RAS) |
| x | Modeler to complete topology validation as outlines in the guidelines |
| x | Modeler to verify mapping meets deliverable requirements as outlined in guidelines |
| Completeness Review (QC item 8) See Texas 2D BLE Guidelines Chapter 4 – Deliverables |  |
| x | Technical BLE Report |
| x | Final cleaned hydrologic model |
| x | Final cleaned hydraulic model |
| x | Flood risk database |
| x | CMNS updates |
| x | HAZUS results |
| x | Additional supplemental data |

# 1 – Model Terrain

Methodologies / Data Sources (Modeler to populate)

| Terrain Source(s): | Please see technical memo for more detail: Texas West Central Lidar (2018), Brown County Lidar (2019), Hurricane Lidar (2019), Brazos River Basin Lidar (2016), Upper Clear Fork Brazos River Lidar (2014), USGS National Elevation Dataset 1/3 arcsecond |
| --- | --- |
| Data Gaps: | Gap between Upper Clear Fork, Brazos River Basin, and Texas West Central Lidar that was filled with the USGS NED data. There are seams in the terrain where the NED data are used. However, they are located near the upstream watershed boundary and should not impact the hydraulic modelling and mapping results. |
| Source Resolution: | Cell size of 1 x 1 meter |
| Model Terrain Resolution: | Cell size of 5 x 5 feet |
| Terrain Horizontal Projection: | North American Datum (NAD) 1983 (2011) Texas State Plane Zone 3 (Central) |
| Terrain Vertical Projection: | NAVD88 |
| Z Values: | Units of feet |
| Terrain Buffer: | 2,500 feet |
| Other: |  |

## Technical Review (Reviewer to provide comments)

Reviewer is to utilize detailed QA/QC to identify general review items for this audit.

## Review Comments

- No additional comments.

# 2 – Hydrology

## Methodologies / Data Sources (Modeler to populate)

| Software: | HEC-RAS 6.2 |
| --- | --- |
| Rainfall Source: | NOAA Atlas 14 |
| Rainfall Duration: | 24 hours |
| Depth Selection: | Domain Average |
| Rainfall Temporal Distribution: | NRCS |
| Areal Reduction: | None |
| Loss Methodology: | Spatially varied CN |
| Unit Hydrograph Transform Method: | N/A |
| Inflow Hydrograph Source: | HUC10 1209011001 – Model 1001_A2 HUC10 1209011001 – Model 1002_A3, 1002_A4 |
| Other: |  |

## Statistical Data (Modeler to populate)

| Stream Gage Name & ID: | Brady Creek at Brady, Texas, 08145000 |
| --- | --- |
| Stream Gage Name & ID: |  |
| Stream Gage Name & ID: |  |

## Technical Review (Reviewer to provide comments)

Reviewer is to utilize detailed QA/QC to identify general review items for this audit.

## Review Comments

- Computation spreadsheets provided for 1% plus/minus calculations do not align with the TWDB BLE Guidelines. Please provide documentation of equations/methodology used to compute the 1% plus/minus precipitation depths.

Response: The 1% plus and 1% minus rainfall totals are now calculated using TWDB 2D BLE Guidance and temporal distributions have been updated such that 1% plus and 1% minus profiles don’t cross other profiles.

No additional comments.

- Methodology for rainfall hyetograph development not provided, please provide methodology discussion and HMS model used if applicable.

Response: For each model domain, NOAA Atlas 14 precipitation statistics were downloaded and basin wide averages for 5 min to 24Hr durations were calculated. NRCS temporal distribution was then used to obtain nested hyetograph shapes, which were used as rainfall input for HEC-RAS model. Combined hydrologic and hydraulic analysis was performed in HEC-RAS 6.2 using spatially varying infiltration capabilities. Discussion of the hydrologic analysis is added to the Brady BLE report.  A readme file is included in the simulations folder of the hydrologic analysis to explain that HEC-HMS model was not developed.

Frequency storms are used for nested hyetograph which is also preferred method per BLE Guidelines. Provide justification for using NRCS temporal distribution.

Atkins Response: The NRCS nested hydrograph method is another commonly used temporal distribution which Atkins had used successfully on previous projects. Our model development was well underway for this watershed before TWDB guidance was published and had used NRCS distribution.  Additionally, in past monthly progress meetings we have discussed this topic with TWDB staff, and they agree that NRCS nested rainfall methodology is acceptable and can be used for 2D BLE model development. It is our understanding that this methodology will be added to the TWDB Guidance at some point in the near future.

- DAA not mentioned in provided methodology documentation, assuming it was not accounted for, provide justification for this decision.

Response: It was not clear what DAA acronym stands for.

DAA is Depth-Area-Analysis in HMS. This is to account for areal reduction.

Atkins Response: We were able to achieve flow validation using curve number and manning’s n parameter updates so did not use aerial reduction factor for this watershed.

3 – Hydraulic Geometry

## Methodologies / Data Sources (Modeler to populate)

| Software: | HEC-RAS 6.2 |
| --- | --- |
| Loss Methodology: | Spatially varied CN |
| Soils Source: | SSURGO |
| Land Cover Source: | NLCD 2019 |
| Manning's Roughness Source: | Assigned based on land use. Please see table in Brady Model Overview and Results.xlxs |
| Nominal Mesh Cell Sizes: | 200x200, except for Channel Refinement Regions, Breaklines and Urban Regions where 100x100 was considered. |
| Other: |  |

## Technical Review (Reviewer to provide comments)

Reviewer is to utilize detailed QA/QC to identify general review items for this audit.

## Review Comments

- According to the provided excel file titled “Brady Model Overview and Results”, the manning’s value for Developed, Medium Intensity is lower than Developed, Low Intensity.  It appears that these values are simply transposed in the spreadsheet, however, please review and revise.

Response: Manning’s N values (and curve numbers) have been updated to match the TWDB 2D BLE Guidance.

No additional comments.

- Channel n-value is not consistently applied along the entire length of the mainstem – US end of A4 is missing a channel n-value designation

Response: Channel refinement region is now extended at the upstream end of consistency.

No additional comments.

- Consider smoothing the stream centerlines being used as breaklines in order to simplify the cell faces.

Response: The channel breaklines have been now smoothened and an example screenshot shown below.

No additional comments.

- Planarize the lines used for breaklines to simplify cell geometry.

Response: The breaklines have been planarized and an example screenshot shown below.

No additional comments.

- Remove conflicting breaklines to simplify cell geometry

Response: Conflicting breaklines have been adjusted.

No additional comments.

- Cell adjustments need to be made a large structure crossing and embankments to allow flow to leak through the structure where results show large areas of backwater occurring.

Response: At major structures, terrain modifications were performed and burn lines were added to allow flow through the roadway. Areas of large backwater reviewed and added missing burns lines.  Along larger dams with defined structures breaklines are added through the dam crest which would cause some ponding behind the dam.

No additional comments.

- Breaklines should be added along any results lines used as inflow data or flow comparisons/calibrations to ensure results lines align to cell faces.

Response: At validation locations, breaklines have been added along the profile lines

No additional comments.

- Cells along refinement region boundaries are not being enforced.

Response: Cells along channel refinement region are now being enforced.

No additional comments.

4 – Hydraulic Boundary Conditions

## Technical Review (Reviewer to provide comments)

Reviewer is to utilize detailed QA/QC to identify general review items for this audit.

## Review Comments

- 1001_A1 DS boundary condition line should be extended a bit more to ensure it is wide enough for the entire 0.2% wetted cells.

Response: The DS boundary condition line have been extended.

No additional comments.

- Upstream boundary condition line for US inflows needs to lie outside of the 2D area, otherwise flow will not all be directed DS as intended.

Response: Per previous direction provided to Atkins by Halff, flow is calculated and applied at coincident locations between adjacent models. A SA/2D connection is added in the upstream model to compute flow hydrograph and a BCLine in co-incident location is added to apply inflow for the downstream model.

No additional comments.

- 2D Connection lines (SA2D Connections) should be enforced into the mesh of the US model at an ideal transitional location and used to assess the flow hydrograph for the DS area and the DS 2D extents established accordingly to ensure flow application at coincident locations.

Response: SA2D connections are now added and enforced for flow transfer.

No additional comments.

- At the end of domain A4, the mesh is extending beyond the confluence of Brady Creek and San Saba Creek. There is no inflow for San Saba Creek and Brady flow is allowed to leave through the US direction of San Saba Creek with the Perimeter_Outflow_2, an inflow should be accounted for or the mesh extents should be adjusted.

Response: At the end of domain A4, an inflow has been added based on San Saba BLE model, additionally downstream boundary for the main channel was changed from normal to stage hydrograph boundary.

No additional comments.

- Provide discussion and reasoning for using the gage analysis downstream of the reservoir as the reservoir outflow into 1002_A4, considering the additional drainage area between the gage location and the reservoir embankment.

Response: The reservoir operation data for Brady reservoir is not available, however majority of the outflow in larger storms occur through the uncontrolled secondary spillway. A SA2D connection has been added at the secondary spillway to capture dam outflow. The dam outflow is added as an inflow for the downstream domain and gage analysis is no longer used.

No additional comments.

# 5 – Hydraulic Model Execution

## Methodologies / Data Sources (Modeler to populate)

| Unsteady Plan Names |  |  |  |
| --- | --- | --- | --- |
| 10% ACE (10-year): | NRCS_10_PCT | 0.2% ACE (500-year): | NRCS_0.2_PCT |
| 4% ACE (25-year): | NRCS_4_PCT | 1% plus: | NRCS_1.0_PLU |
| 2% ACE (50-year): | NRCS_2_PCT | 1% minus: | NRCS_1.0_MIN |
| 1% ACE (100-year): | NRCS_1.0_PCT |  |  |
| Other: |  |  |  |

## Statistical Data (Modeler to populate)

| Time Step: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Courant Value: | Min 0.45 | Max 2.0 | Simulation Length: | 120 | hours |
| Computation Interval: | 30 | seconds | Model Runtime: | 2-4 | hours |
| Hydrograph Output Interval: | 10 | minutes | Mapping Output Interval: | 10 | minutes |

## Technical Review (Reviewer to provide comments)

Reviewer is to utilize detailed QA/QC to identify general review items for this audit.

## Review Comments

- No additional comments.

# 6 – Hydraulic Model Results

## Methodologies / Data Sources (Modeler to populate)

| Flow Validation/Gage Analysis |  |
| --- | --- |
| Software: | N/A |
| Methodology | Gage Analysis values from InFRM Report with priority for gage being highest followed by FIS flows and then regression analysis, refer to the hydraulic technical memo and the model plans and overview spreadsheet. |
| Removal of Outliers and Justification | N/A |
| Validation Notes (describe parameter adjustments): |  |
| Other: |  |

## Technical Review (Reviewer to provide comments)

Reviewer is to utilize detailed QA/QC to identify general review items for this audit.

## Review Comments

- Please provide a comparison of flows at USGS Gage 08145100 to the 1002_A4 results at the gage location.

Response: The flow and WSEL comparison at USGS gage are discussed and documented in the BLE report and Brady Model Overview and Result.xlsx spreadsheet included some additional information.

No additional comments.

- Please provide an explanation for the decrease in results flows at FIS comparison point 5 in 1002_A4 and for the results flows at FIS comparison points 2-5 being less that the pre-atlas 14 FIS flow values.

Response: Please refer to in Section 1.2.9 of the BLE report for discussion of this topic.

No additional comments.

# 7 – Preliminary (Raw) Mapping Results

## Technical Review (Reviewer to confirm with Pass/Fail)

| Hydraulic Model |  |
| --- | --- |
| Pass | HEC-RAS model with results, pertinent data and required storm events |
| Preliminary Mapping 1% ACE |  |
| Fail | Preliminary 10%, 1%, 0.2% ACE mapping results (raster grids from HEC-RAS) to meet FEMA mapping requirements |
|  | Boundary interfaces align matching or identified as AOMI |

## Review Comments

- Tie-in locations have significantly different WSEs at each of the 3 join areas between models. Overlaps and inflow implementation needs to be adjusted for WSEs to be within 0.5-foot at tie-in.

Response: The models have been updated to achieve tie-in within 0.5’. In most cases stage hydrograph boundary at the main channel outfall location from downstream model were used to achieve tie-in. In few cases, normal depths are revised to achieve tie in.

No additional comments.

# 8 – Completeness Review

## Technical Review (Reviewer to confirm with Pass/Fail)

| File Structure and Naming |  |
| --- | --- |
| x | Deliverables provided in TWDB/FEMA required file folder structure and naming convention |
| x | Data missing (if any) documented in report and ReadMe file within the required folder |
| Report |  |
| x | Technical 2D BLE report documents all assumptions, special considerations, etc. |
| x | Flow validation and difference discussed in report |
| x | Flow comparisons discussed in report |
| Hydrologic Model |  |
| n/a | HEC-HMS model with results, pertinent data and required storm events |
| Hydraulic Model |  |
| x | HEC-RAS model with results, pertinent data and required storm events |
| Flood Risk Database |  |
| x | All features fully populated per current FEMA guidelines |
| x | Mapping consistent between grids to polygons for all three required mapped events |
| x | 10yr smaller than 100yr smaller than 500yr |
| x | Holes filled in polygon, depth and WSE grids |
| x | Boundary interfaces align matching or identified as AOMI |
| CNMS |  |
| x | Updated correctly with new centerlines |
| x | A1-A5 checks complete |
| x | CNMS Post- Processing Complete |
| HAZUS |  |
| x | Run per FEMA guidance |

## Review Comments

- BFE:

- Domain values are not being used for BFE_LN_TYP or LEN_UNIT. Please revise.

Atkins Response:  Updated BFE_LN_TYP and LEN_UNIT fields to use domain values.

Backcheck 2/8: No further comment

- BFE_LN_TYPE: Field says Mapped instead of “Unmapped”

Atkins Response:  BFE lines within DTL_STUD_AR locations were attributed with unmapped line type to ensure that BLE elevations will not be visible on the eBFE viewer in detail study areas.

Backcheck 2/8: No further comment

- DTL_STUD_AR :

- Fields are blank. Please populate: CD_YN, CD_POC, CD_ADD1, CD_ADD2, CD_CTY, CD_STATE, CD_ZIP, CD_Phone, CD_Email, LOMR_ID, Scale and Status with “NP”.

Atkins Response: Updated field values with NP, and panel scale added

Backcheck 2/8: No further comment

- SOURCE_CIT should be “FIRM1” as this is effective information.  Please update and revise this feature class, L_SOURCE_CIT table and metadata.

Atkins Response: Added FIRM1 source cite for effective data

Backcheck 2/8: No further comment

- DTL_STUD_LN:

- Fields are blank. Please populate: CD_YN, CD_POC, CD_ADD1, CD_ADD2, CD_CTY, CD_STATE, CD_ZIP, CD_Phone, CD_Email, LOMR_ID, Scale and Status with “NP”.

Atkins Response:  Updated fields identified in the comment with NP values and panel scale is now added.

Backcheck 2/8: No further comment

- WTR_AR & WTR_LN:

- SOURCE_CIT: Written as BASE1 instead of “STUDY1”.  ONLY APPLIES TO WTR_LN. Update

Atkins Response:  Source citation for WTR_LN is updated to STUDY1

Backcheck 2/8: No further comment

- L_SOURCE_CIT table and metadata.

  - WTR_AR should be BASE7. Add to L_SOURCE_CIT table and metadata.

Atkins Response:  Since WTR_AR features are based on NHD, Base 2 source CIT is used for WTR_AR. Base 7 citation is for NLCD which does not apply for WTR_AR.

Backcheck 2/8: No further comment

- There are some disconnected floating stream segments. Please verify and revise as necessary. See marked red in the image below:

Atkins Response:  WTR_LN features were reviewed and updated such that there are no disconnected streams.

Backcheck 2/8: No further comment

- TENPCT_FP:.

- STUDY_TYP, ZONE_SUBTY are both blank. Please populate.

Atkins Response:  STUDY_TYP  and ZONE_SUBTY is now updated with NP values.

Backcheck 2/8: No further comment

- Make ZONE_SUBTY, VEL_UNIT, AR_REVERT, and AR_SUBTRV empty string “”.

Atkins Response:  Updated VEL_UNIT to blank. During previous M3 reviews for other watersheds on this project NP value was recommended for AR_REVERT and AR_SUBTRV so the value not updated to “” to maintain consistency.

Backcheck 2/8: No further comment

- V_DATUM, LEN_UNIT, VEL_UNIT are only populated if STATIC_BFE, DEPTH, or VELOCITY are populated. Make empty string “”.

Atkins Response:  V_DATUM, LEN_UNIT, and VEL_UNIT fields are now updated with empty string (“”).

Backcheck 2/8: No further comment

- Make Dual_Zone as "False (No)" coded value is 'F' instead of Unknown.

Atkins Response:  Updated Dual_Zone to False (No)" using coded value 'F'.

Backcheck 2/8: No further comment

- FLD_HAZ_AR:

- V_DATUM, LEN_UNIT, VEL_UNIT are only populated if STATIC_BFE, DEPTH, or VELOCITY are populated. Make empty string “”.

Atkins Response:  V_DATUM, LEN_UNIT, and VEL_UNIT fields are now updated with empty string (“”).

Backcheck 2/8: No further comment

- Make ZONE_SUBTY, VEL_UNIT, AR_REVERT, and AR_SUBTRV empty string “” for Zone A polygons.

Atkins Response:  ZONE_SUBTY and VEL_UNIT are updated with now updated with empty string (“”).

During previous M3 reviews for other watersheds on this project NP value was recommended for AR_REVERT and AR_SUBTRV so the value not updated to “” to maintain consistency.

Backcheck 2/8: No further comment

- Make VEL_UNIT, AR_REVERT, and AR_SUBTRV empty string “”.

Atkins Response:  VEL_UNIT is now updated with empty string (“”).

During previous M3 reviews for other watersheds on this project NP value was recommended for AR_REVERT and AR_SUBTRV so the value not updated to “” to maintain consistency.

Backcheck 2/8: No further comment

- V_DATUM, LEN_UNIT and STUDY_TYP are blank. Please populate.

Atkins Response:  V_DATUM, LEN_UNIT, are updated with empty string (“”) based on the first bullet in Comment no. 6.

STUDY_TYP values are populated with “NP”.

Backcheck 2/8: No further comment

- ZONE_SUBTY: NP is used for 100 year instead of leaving it blank.

Atkins Response:  Updated ZONE_SUBTY to blank for 100YR floodplain.

Backcheck 2/8: No further comment

- BLE_DEP01PCT, BLE_DEP0_2PCT, BLE_WSE01PCT & BLE_WSE02PCT:

- The HUCs that are surrounding Brady HUC-8 are in progress right now. Please make sure that the WSEL are tying in with those HUC 8s.

Atkins Response:  The model tie-in within 0.5’ with downstream San Saba HUC8. The San Saba and Middle Colorado BLE modeling and mapping is in progress and the floodplain tie-in within 0.5’ along the watershed boundary based on current data. Will have to doublecheck after modeling / mapping is further along.

Due to 2D rain-on-grid modeling/ mapping approach some of the tributary floodplains extend to the watershed divide at the upstream end of the tributaries. Brady does not have very flat topography (compared to some of the eastern Texas watersheds) and there are negligible overflows into adjacent HUC8 watersheds from Brady tributaries. in our opinion, unless there are major overflows between watersheds the tie-ins at the perimeter appear not important and, priority could be given to other important aspects of the BLE modeling and mapping.

Backcheck 2/8: No further comment

- Mapping is not restricted to 0.33ft but in the report its mentioned that it was restricted. Please verify and update accordingly.

Atkins Response:  The floodplain mapping for this project was carried out by using following TWDB mapping thresholds shown in the table below. When both area and depth thresholds are met then area is mapped and when depth and/or area criteria are not met then area is not mapped. For example, isolated flooding polygons less than 0.33 ac will not be mapped. If the polygon area is between 0.33 to 0.5 ac and average depth is greater than 0.33 feet then the area will be mapped.

TWDB had developed and provided mapping toolbox for processing raw HEC-RAS output grids based on the matrix, which Atkins used for floodplain cleanup for this watershed. We updated the report to clarify the mapping process.

Backcheck 2/8: No further comment

- S_AOMI_PT:

- Hazus Essential Facilities should be cited as “REF1”.

Atkins Response:  Updated source citation to “REF1” for AOMI_PT and added REF1 citation to L_Source_Cit

Backcheck 2/8: No further comment

- S_AOMI_AR:

- Feature class is blank.

Atkins Response:  Added AOMI area based on review of NLCD data.

Backcheck 2/8: No further comment

- S_HUC_AR:

- Showing HUC12 and HUC10 boundaries. Please remove.

Atkins Response:  The HUC 10 and HUC 12 boundaries are now removed from the database.

Backcheck 2/8: No further comment

- Update SOURCE_CIT to BASE2.  These boundaries were not created during this study and are part of reference files. Update L_SOURCE_CIT and metadata.

Atkins Response:  Updated Source_CIT field to BASE2 and updated in metadata.

Backcheck 2/8: No further comment

- S_POL_AR:

- COM_NFO_ID: Field is blank.  Please populate with CID.

Atkins Response:  Populated CID values

Backcheck 2/8: No further comment

- ANI_TF: Mention True (Yes) or False (No) instead of Unknown.

Atkins Response:  Added True / False values.

Backcheck 2/8: No further comment

- POL_AR_ID: Make DFIRM ID plus '_' and Letter. No nulls or blanks.

Atkins Response:  Assigned POL_AR_IDs. Slightly different format than suggested is used to populate values.

Backcheck 2/8: No further comment

- DFIRM_ID: Make five-digit number beginning with '48' and ending with “C”. No Nulls or Blanks.

Backcheck 2/8: Guidance has been revised. This field should be populated with 2-digit State FIPS code and 3-digit county code (i.e., 48 + 095 = 48095) + "C" = 48095C

Atkins Response (02/17/2023): The DFIRM IDs have been revised per the ITR comment on 02/08.

Halff Backcheck 5/10/2023: No further comments

- POL_NAME2, POL_NAME3, and ANI_FIRM should be populated with empty strings “”.

Atkins Response:  Left as is to be consistent with previous watersheds

Backcheck 2/8: No further comment

- S_STUDIES_LN:

- Domains are not being used for attributes. Please correct and revise.

Atkins Response:  The domain values between CNMS and BLE database are not same and in the previous submittal the domain vales in the BLE database were not correctly transferred. We have corrected this issue and updated to use FRD domain values.

Halff Backcheck 5/10/2023: It is correct that CNMS and BLE domains are not the same and causes some confusion that FEMA is working on addressing. It is the advice of Michael Johnson (FEMA Region 6 CNMS Lead) that the current best practice is to completely Delete the CNMS features within the FRD and replace them with the S_Studies_Ln and S_Unmapped _Ln feature classes from the stand alone CNMS gdb. This will maintain the CNMS Schema and Domains for the CNMS feature classes. You may notice some similar/duplicate domain names with different code values in ArcCatalog but MJ has assured us that this is a known issue and that previous submittals done this way have not been an issue.

Halff Backcheck 7/22/2024: Per coordination with TWDB, review of CNMS features will be completed and documented by Michael Johnson, Region 6 CNMS Lead. Therefore, Halff is no longer reviewing this portion of the Flood Risk Database deliverables.

*Updated note from Backcheck QC 2/8

For CNMS S_Studies_Ln and WTR_LNs review that both meet the following requirements:

- 1 SQM or 0.5 SQM in population centers (with population greater than 1,000)

- Coverage of FEMA effective S_WTR_LN

- Coverage of CNMS S_Studies_Ln

Atkins Response (02/17/2023): The CNMS features have been revised per the ITR comment on 02/08. WTR_LN and S_Studies_Ln have been extended to 0.5 sq. mi based population of 1000.  Using political boundaries shapefile obtained from TXDOT, added 0.5 SQMI streamlines to WTR_LN and S_Studies_Ln for populated areas with greater than 1000 population.  Coverage between WTR_LN and S_Studies_Ln have been checked and necessary changes are applied. The S_Studies_Ln have been extended where WTR_LN extended upstream of S_Studies_Ln and vice versa.

Halff Backcheck 5/10/2023: No further comments

  - Domains Comment- Review in BLE Database-CNMS_Dataset:

- FLD_ZONE field-Double check domains are being used for all Flood Zones currently it looks like zone AE has been added manually.

- TIER field does not appear to be using Domains update.

Double check other fields use/reference Domains from Domain Table Technical Reference.

Atkins Response: *see response above*

- Populate POC_ID for all rows.

Atkins Response:  This field was left blank in previous watersheds. However, reach IDs were used to populate this field.

Backcheck 2/8: POC_ID should contain the Point of Contact update with POC information for project.

Atkins Response (02/17/2023): Information added to POC table and POC_ID has been populated. Per the information provided FEMA Region 6 CNMS lead (Michael Johnson) the POC_ID field was populated using following format “County FIPS+05+XXXXX”.

Halff Backcheck 5/10/2023: No further comments

- Zone A Streams:

  - DATE_RQST and DATE_EFFECT should match base CNMS database.  Use STATUS_DATE and HYDRO_DATE_EFFCT respectively.

Atkins Response:  The values are updated per the suggestion

Backcheck 2/8: No further comment

*Updated note from Backcheck QC 2/8

  - A4_TECH field-currently all reaches except for one marked as “PASS” needs review. HYDRO and HYDRA_MDL model fields have “UNKNOWN” and have an effective date that were done in or prior to 1980 more than likely the study was not model backed and A4_TECH field should fail for all reaches.

Atkins Response (2/17/23): HYDRO_MDL and HYDRA_MDL fields were not updated as part of BLE CNMS updates, these fields are typically updated a regulatory study is performed. Per the ITR comment all reaches are now marked as FAIL in the A4_TECH field.

Halff Backcheck 5/10/2023: No further comment

  - A5_FOAPASS is empty.  Please double check fields match/align when populating over features from other databases.  Unable to verify validity without field populated.

Atkins Response:  The values are updated

Backcheck 2/8: Updated correctly no further comment

Halff Backcheck 5/10/2023: I see this was corrected in the previous iteration sent on 1/25/2023, but it looks like the most recent version sent on 4/27/2023 does not have this field populated. Please double check this is populated in the most up to date version before submitting.

Halff Backcheck 7/22/2024: Per coordination with TWDB, review of CNMS features will be completed and documented by Michael Johnson, Region 6 CNMS Lead. Therefore, Halff is no longer reviewing this portion of the Flood Risk Database deliverables.

- Zone AE Streams:

  - DATE_RQST and DATE_EFFECT should match base CNMS database.  Use STATUS_DATE and HYDRO_DATE_EFFCT respectively.

Atkins Response:  The values are updated per the suggestion

Backcheck 2/8: Updated correctly no further comment

- Zone X Streams:

  - WTR_NM should be “NP” for any streams without a name.

Atkins Response:  Null values are updated to “NP”

Backcheck 2/8: Updated correctly no further comment

  - REASON field is a required field and needs to be populated for all rows.

Atkins Response:  Updated reason field

Backcheck 2/8: Updated correctly no further comment

  - DATE_RQST should match STATUS_DATE. Please populate.

Atkins Response:  The values are updated per the suggestion

Backcheck 2/8: Updated correctly no further comment

- S_UNMAPPED_LN: File is empty. Please populate with appropriate attributes.

Atkins Response:  There were no locations where new Zone X fell outside the new BLE floodplain, the S_Unmapped_Ln layer was empty.

Backcheck 2/8: Some reaches currently in backwater and can be added to S_Unmapped or in S_Studies attributed appropriately in the “Reason” field. Review Brady_QC_Comments polygon for reaches that need to be added/as examples of backwater.

Atkins Response (02/17/2023): Reaches identified in the QC shapefile are added to S_unmapped_Ln, attributed appropriately

Halff Backcheck 5/10/2023: Reaches that were in backwater were moved to S_Unmapped_Ln correctly but still need to delete those lines out of S_Studies_Ln. There should not be overlapping features between S_Studies_Ln and S_Unmapped_Ln.

Halff Backcheck 7/22/2024: Per coordination with TWDB, review of CNMS features will be completed and documented by Michael Johnson, Region 6 CNMS Lead. Therefore, Halff is no longer reviewing this portion of the Flood Risk Database deliverables.

- S_FRAC_AR:

- Polygons does not cover entire HUC-8 limits. Please confirm.

Atkins Response:  HAZUS 5.1 only uses dasymetric data, which adjusted based on land cover area instead of homogeneous

Backcheck 2/8: No further comment

- SCENAR_ID: Field is blank.  Please populate with unique IDs.

Atkins Response:  The Flood Risk Database Technical Reference states that SCENARIO_ID is associated with levee or dam scenario identification for L_Levee Scneario or L_Dam_Scneario tables. Since such scenario do not apply to this study, the field was left empty.

Backcheck 2/8: No further comment

- SOURCE_CIT: Rename to “REF1”. Update L_SOURCE_CIT table and metadata.

Atkins Response:  Updated to REF1

Backcheck 2/8: Updated correctly no further comment

- L_SOURCE_CIT:

- Populate DFIRM_ID with attributes, not NP.

Atkins Response:  Populated FIRM1 and DFIRM ID shown in MSC is CID

Backcheck 2/8: The BLE submittal guidance says this is a required field. Because of this populate all DFIRM_IDs with an identical value (State + County Code+C)

Atkins Response (02/17/2023): Populated DFIRM_IDs in L_SOURCE_CIT and S_POL_AR with State + County Code + C values

Halff Backcheck 5/10/2023: No further comments

- Add BASE2, BASE7, and REF1 to table.  Update metadata accordingly.

Atkins Response:  Added BASE2, REF1 and FIRM1 and updated metadata

Backcheck 2/8: No further comment

7/18/2024 – TENPCT_FP, FLD_HAZ_AR, and the Depth and WSE rasters for both the 1% and 0.2% events were re-built to address tie-in issues with adjacent HUC8s. The comments below represent the quality control review of these modified features.

- FLD_HAZ_Ar

Geometry:

- No Comment

Attribute Table:

- AR_REVERT and AR_SUBTRV fields should be populated with Null or “ ” (empty string) not “NP” since they are not required fields.

- V_DATUM should only be populated if STATIC_BFE is populated. Populate with Null or “ ” (empty string) otherwise.

- LEN_UNIT should only be populated if STATIC_BFE or DEPTH is populated. Populate with Null or “ ” (empty string) otherwise.

Atkins Response 07/25/2024:  The attribute table fields have been updated in the FLD_HAZ_Ar feature class.

Halff Backcheck 8/7/2024: Addressed. No Further Comments

- TENPCT_FP

Geometry:

- Features smaller than 0.33 acres in size of any depth is not allowed according to TWDB mapping threshold matrix. Please see ‘QC_Polygon_20240718’ for example and search for other instances throughout the HUC.

Atkins Response 07/25/2024: All Features smaller than 0.33 acres that do not extend into adjacent watersheds were removed from the TENPCT_FP feature class.

Halff Backcheck 8/7/2024: Addressed. No Further Comments

Attribute Table:

- AR_REVERT and AR_SUBTRV, ZONE_SUBTY fields should be populated with Null or “ ” (empty string) not “NP” since they are not required fields.

- V_DATUM should only be populated if STATIC_BFE is populated. Populate with Null or “ ” (empty string) otherwise.

- LEN_UNIT should only be populated if STATIC_BFE or DEPTH is populated. Populate with Null or “ ” (empty string) otherwise.

Atkins Response 07/25/2024:  The attribute table fields have been updated in the TENPCT_FP feature class.

Halff Backcheck 8/7/2024: Addressed. No Further Comments

- Grids

Current Version

Previous Version

WSE Grids for both the 1% and 0.2% are very noisy compared to previous versions. See screen shots below.

Atkins Response 07/25/2024:  WSE Grids were replaced with WSEs that were generated directly from model output. Output is now smoother. We ran into this issue with our Llano submittal a while back and have applied the same solution to Brady as we did to Llano.

Halff Backcheck 8/7/2024: No Further Comments

Folder Structure:

Halff Backcheck 5/10/2023: For all HUCs which have not been completed/finalized, the EBFE Folder Structure provided with this review should be used (as discussed in the Workshop on March 30th). Please note, no additional data has to be created for the EBFE deliverable. The files already in your current submittal just need to be moved into the EBFE folder structure.

- FRP: Please revise the folder name to “Flood_Risk_Products_Submittal”

Atkins Response:  In past submittals to Halff, the ITR review team had issues accessing data due to long file paths when full names were used so the folder name was shortened. No change was made.

- Final:

- Task Documentation: Missing word & PDF of report.

Atkins Response:  Added word and pdf file under task documentation folder.

- Hydraulics: Please revise the folder name to “Hydraulics_Submittal”

Atkins Response:  In past submittals to Halff, the ITR review team had issues accessing data due to long file paths when full names were used so the folder name was shortened. No change was made.

- Brady_12090110:

- Supplimentary_Data: Missing readme.txt file and raw 10,100 and 500 WSEL& Depth rasters.

Atkins Response:  Added The raw HEC-RAS outputs were included under the supplemental data for flood risk products in path (FRP\12090110\Final\Flood_Risk_Products\Supplemental_Data\RAS_Output). The Readme file were only added in empty folder.

- Missing model inventory for all RAS models.

Atkins Response:  The model index shapefile and results and overview spreadsheets are located in Task Documentation folder of hydraulics submittal.

- Spatial Files:  Hydraulics_DCS.gdb file does not follow the FEMA schema.

Atkins Response:  Since this is a BLE submittal BLE database schema provided by TWDB was used for the spatial database.

- Simulation Models (All folders) :

- Land Classification: please only keep the final Land cover file and remove the rest.

- Backup: Delete any backup data.

- Remove the extra folders:

Atkins Response:  Extra folders in the are now deleted per the markup.

- Hydrology: Please revise the folder name “Hydrology_Submittal”

Atkins Response:  In past submittals to Halff, the ITR review team had issues accessing data due to long file paths when full names were used so the folder name was shortened. No change was made.

- Task Documentation:

- Missing word & PDF of report.

- Missing MetaData from folder.

Atkins Response:  Both hydrologic and hydraulic analysis were performing using HEC-RAS 6.2 and no separate hydrologic analysis (HEC-HMS analysis) was performed so all data (report, spatial files, supplemental data ) is included under the hydraulics. A readme file in hydrology folder has been included to point the users to hydraulics folder.

- Supplemental_Data: Any gage analysis files should be saved here.

- Atkins Response:  InFRM gage analysis is included under the supplemental data folder for hydraulics. A readme file in hydrology folder has been included to point the users to hydraulics folder.

- Spatial Files:  Hydrology_DCS.gdb file does not follow the FEMA schema.

Atkins Response:  Since this is a BLE submittal BLE database schema provided by TWDB was used for the spatial database.

- Terrain: Please revise the folder name to “Terrain_Submittal”

Atkins Response:  In past submittals to Halff, the ITR review team had issues accessing data due to long file paths when full names were used so the folder name was shortened. No change was made.

- Spatial_Files: Missing TerrainDCS.mdb

Atkins Response:  Since this is a BLE submittal BLE database schema provided by TWDB was used for the spatial database.

- Task Documentation: Missing Terrain MetaData

Atkins Response:  FRD metadata is located task documentation folder of FRP. This information has not been requested on previous watersheds and based on previous discussions with TWDB staff all BLE data will be uploaded to MIP under one task.

QA/QC Checklist Approval

This 2D Base Level Engineering Analysis QA/QC Review is in compliance with the contract requirements and all task checkpoints are complete.  The independent QC Team has reviewed the analysis, presented review comments to the Production Team Task Leader, and discussed any problems or issues.  The Task Leader, QC Manager, QC Reviewer, and Project Manager have signed the QA/QC Checklist to confirm that all comments have been received, addressed, and documented appropriately.

| QA/QC Team – [TWDB or company name] |  |  |  | Production Team – AtkinsRéalis |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Seth Ahrens |  | 12/16/2024 |
| QC Manager |  | Date |  | Task Lead |  | Date |
|  |  |  |  | Seth Ahrens |  | 12/16/2024 |
| QC Reviewer |  | Date |  | Project Manager |  | Date |