# Texas 2D BLE Analysis Audit Checklist

# Project Information

| HUC-8 Watershed Name & ID: | North Llano: 12090202 |
| --- | --- |
| HUC-10 Watershed Name & ID: | Upper North Llano River: 1209020201 Middle North Llano River: 1209020202 Lower North Llano River: 1209020203 |
| Modeler: | Atkins |
| Reviewer: | Halff Associates |

This audit process includes only high-level review for items identified in the Detailed Review for another HUC for the same consultant. The consultant is responsible for performing a detailed review internally for base data, hydrology, hydraulics, and modeling results.

## Audit 1: Terrain, Modeling, Preliminary Mapping Review (QC Items 1-7)

| Modeler Initials: | BV | Reviewer Initials: | LF |
| --- | --- | --- | --- |
| Submittal Date: | 5/10/2022 | Review Submitted Date: | 07/15/2022 |
| Review Response Date: | 10/07/2022 | Backcheck Complete Date: |  |

## Audit 2: Completeness Review (QC Item 8)

| Modeler Initials: | SN | Reviewer Initials: | PB |
| --- | --- | --- | --- |
| Submittal Date: | 10/07/2022 | Review Submitted Date: |  |
| Review Response Date: |  | Backcheck Complete Date: | 7/31/23 |

(Modeler to populate with an “X” for items included in the QC submittal)

| Level of Study |  |  | Flood Events |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| X | 2D BLE |  | X | 10% ACE (10-year) | X | 0.2% ACE (500-year) |
|  | Enhanced BLE |  | X | 4% ACE (25-year) | X | 1% plus |
|  |  |  | X | 2% ACE (50-year) | X | 1% minus |
|  |  |  | X | 1% ACE (100-year) |  | Observed Rainfall |

Instructions

Each section includes instructions to the modeler and reviewer regarding their responsibilities in the completion of this checklist. In the Methodologies / Data Sources tables, grey italic text is added as an example response. Additional sample responses are located in the Texas 2D BLE Guidelines Quick Guide.

## Submittal Checklist

Data submitted for each milestone should be carried through each subsequent milestone. All geospatial layers and models must have same defined projection system as terrain. Modeler to populate with an “X” for items included in the QC submittal.

| Base Map Data (QC items 1-6) |  |
| --- | --- |
| X | Location Map (PDF): HUC-8 and HUC-10 boundaries and/or model domain boundary if different from HUC-10s, stream gages, dams (if applicable), mainstem streams (for reference), background map |
| X (RASMapper) | GIS Map (MXD or Map Package) with the following data included but not limited to: Stream centerlines, model breaklines, rainfall data, WSEL and Depth grids (10, 100, 500-yr), adjacent HUC mapping (available on the BLE viewer or currently studying), 2D connections for inflows/outflows, gages or points of flow comparisons (BFEs, highwater marks, etc.), profile lines, boundary condition lines, refinement regions, model domain boundary, model mesh, DFIRM mapping panels, base map data (roads, aerials, etc.), soils data, land use data, terrain |
| Technical Report / Documentation (QC items 1-6) |  |
| X | Technical Report / Project Notebook: assumptions, special considerations, and modeling concerns |
| X | Detailed explanation and support data of variances in methodologies and approaches from recommended guidelines (Should be included in Technical Report / Project Notebook) |
| X | Internal QA/QC Documentation: comments and responses |
| X | Flow comparison tables within the technical report |
| Hydrology (QC items 1-6) |  |
| n/a | Hydrologic Model (HEC-HMS) model with all associated DSS files for 7 required storm events |
| X | Hydrologic Data and Parameter Spreadsheets (rainfall calculations, curve number table, gage locations, gage analysis and flow validation) |
| X | Hydrologic modeling notebook including assumptions, notes, and special conditions (if not provided in report) |
| Hydraulics (QC items 1-6) |  |
| X | Hydraulic Model (HEC-RAS) with all associated files and 7 required storm events (remove non-final files)d |
| X | Hydraulic Data and Parameter Spreadsheets (Manning’s n-value table, infiltration calculations support data-Soils, Land Use, Curve Number table, other calculations/data used in model) |
| X | Manning’s n-value table |
| X | Precipitation data correctly linked to hydrologic model or hard coded from provided spreadsheet |
| X | Hydraulic modeling notebook including assumptions, notes, and special conditions |
| Preliminary (Raw) Mapping (QC item 7) |  |
| X | Preliminary 10%, 1%, 0.2% ACE mapping results (raster grids from HEC-RAS) |
| X | Modeler to complete topology validation as outlines in the guidelines |
| X | Modeler to verify mapping meets deliverable requirements as outlined in guidelines |
| Completeness Review (QC item 8) See Texas 2D BLE Guidelines Chapter 4 – Deliverables |  |
| X | Technical BLE Report |
| X | Final cleaned hydrologic model |
| X | Final cleaned hydraulic model |
| X | Flood risk database |
| X | CMNS updates |
| X | HAZUS results |
| X | Additional supplemental data |

# 1 – Model Terrain

Methodologies / Data Sources (Modeler to populate)

| Terrain Source(s): | Lower Colorado San Bernard LIDAR |
| --- | --- |
| Data Gaps: | None |
| Source Resolution: | Cell size of 1 x 1 meter |
| Model Terrain Resolution: | Cell size of 5 x 5 feet |
| Terrain Horizontal Projection: | North American Datum (NAD) 1983 (2011) Texas State Plane Zone 3 (Central) |
| Terrain Vertical Projection: | NAVD88 |
| Z Values: | Units of feet |
| Terrain Buffer: | 2,500 feet |
| Other: |  |

## Technical Review (Reviewer to provide comments)

Reviewer is to utilize detailed QA/QC to identify general review items for this audit.

## Review Comments

- The terrain report states that the terrain was projected into North American Datum 1983 (2011) Texas State Plane Zone 3 (Central).  Section 1.1.7 of the TWDB BLE Guidance dated May 21, 2022 states that the correct horizontal datum for all study areas will be the North American 1983 (NAD83).

Response: Native projection of the TNRIS source LiDAR data was NAD 83 (2011), which was maintained during terrain development task for this study. During ITR comment discussion meeting held on July 08, 2022, between Atkins, Halff, and TWDB, this comment was discussed. TWDB staff directed not to reproject terrain to NAD83 for following reasons:

- TNRIS source LiDAR data used NAD83 (2011) projection

- Difference between NAD83 and NAD83 (2011) projections is very minor for the study area

- The terrain data was developed before the TWDB BLE guidelines were available

No additional comments.

# 2 – Hydrology

## Methodologies / Data Sources (Modeler to populate)

| Software: | HEC-RAS 6.2 |
| --- | --- |
| Rainfall Source: | NOAA Atlas 14 |
| Rainfall Duration: | 24 hours |
| Depth Selection: | Domain Average |
| Rainfall Temporal Distribution: | NRCS |
| Areal Reduction: | None |
| Loss Methodology: | Spatially varied CN |
| Unit Hydrograph Transform Method: | N/A |
| Inflow Hydrograph Source: | Varies by model |
| Other: |  |

## Statistical Data (Modeler to populate)

| Stream Gage Name & ID: | 08148500 N Llano Rv nr Junction, TX |
| --- | --- |
| Stream Gage Name & ID: |  |
| Stream Gage Name & ID: |  |

## Technical Review (Reviewer to provide comments)

Reviewer is to utilize detailed QA/QC to identify general review items for this audit.

## Review Comments

- Consider removing hydrologic files not used in the hydraulic modeling (2-, 5-, 200-, and 1000-year events)

Response: Removed rainfall data for 2-, 5-, 200-, and 1000-year events from the DSS file per the comment.

No additional comments.

- The memo provided states “The 1% plus and 1% minus AEP precipitation events were computed using the approach provided in Section 4.4 of FEMA’s “2D Watershed Modeling in HEC-RAS Recommended Practices” document dated November 2021.”  The procedure discussed in the TWDB BLE Guidelines should be used unless this variance has been approved by TWDB.

Response: The 1% plus and 1% minus rainfall totals are now calculated using TWDB 2D BLE Guidance and temporal distributions have been updated such that 1% plus and 1% minus profiles don’t cross other profiles.

No additional comments.

- The loss values in some models are not matching the table provided in spreadsheet “North Llano Model Overview and Results“.  Please review and revise as necessary.

Response: CN values in the BLE report as well as Infiltration Look-Up Table tab in North Llano Model Overview and Results spreadsheet list initial curve number values. For 4 out of 8 model domains in North Llano Watershed, initial antecedent moisture condition (AMC) II CN values were reduced by 10% during model validation process so the CN values in HEC-RAS model did not match values in the BLE Report / Infiltration Look-Up Table tab. For other four model domains, AMC I CN values have been used to achieve model validation goals. Since AMC I CNs were not adjusted further the values in HEC-RAS model matched BLE report / spreadsheet. Please refer to Section 1.2.9 of BLE report for additional discussion on model validation.

No additional comments.

- The report states that rather than running an updated gage analysis, the draft InFRM Watershed Assessment for the Lower Colordao River Basin (Fall 2021) was used.  The spreadsheet “North Llano Model Overview and Results” shows a comparison to the 1% and the 1%+/- flows.  The 1%+/- flows could not be found in the segment of the draft InFRM report provided.  How were these values obtained or calculated; in the same fashion as the regression 1%+/-? Please provide any calculations completed.

Response: The USACE’s InFRM Watershed Assessment has performed detailed gage analysis and it is best available data for the study area. Since InFRM analysis was more detailed than a typical scope of a BLE gage analysis, the study results were leveraged rather the performing new gage analysis. InFRM study had reported 95% bound flows and did not calculate the 84% bound flows so, the 95% bound values were used for 1% plus and 1% minus. The computed 1% BLE flows are within the 95% bounds so will be within the 84% bounds.

No additional comments.

- Please provide regression calculations.

Response: Regression flow calculations were performed geospatially in GIS so there is no calculation spreadsheet available that can be submitted. However, Section 1.2.7 of BLE report included discussion of how the regression analysis was performed.

No additional comments.

# 3 – Hydraulic Geometry

## Methodologies / Data Sources (Modeler to populate)

| Software: | HEC-RAS 6.2 |
| --- | --- |
| Loss Methodology: | Spatially varied CN |
| Soils Source: | SSURGO |
| Land Cover Source: | NLCD 2019 |
| Manning's Roughness Source: | Assigned based on land use. Please see table in Brady Model Overview and Results.xlxs |
| Nominal Mesh Cell Sizes: | 200x200, except for Channel Refinement Regions, Breaklines and Urban Regions where 100x100 was considered. |
| Other: |  |

## Technical Review (Reviewer to provide comments)

Reviewer is to utilize detailed QA/QC to identify general review items for this audit.

## Review Comments

- Profile lines were used within the modeling to compare results to FIS/regression flows.  SA2D lines should be used instead to allow a smoother cross section flow calculation, rather than the selection of the closest cell faces along the profile line.

Response: The SA/2D lines are not required to obtain flow hydrograph at validation locations and can lead to increased model run times so they were not added. Profiles lines were added and enforced at validation locations.  SA/2D connections, however, have been used for flow transfer between models.

No additional comments.

- According to the provided excel file titled “North Llano Model Overview and Results”, the manning’s value for Developed, Medium Intensity is lower than Developed, Low Intensity.  It appears that these values are simply transposed in the spreadsheet, however, please review and revise.

Response: Manning’s n values have been updated to match the TWDB 2D BLE Guidance and higher n value has been used for Developed, Medium Intensity (0.12) than Developed, Low Intensity (0.09) in the revised model.

No additional comments.

- Consider smoothing the stream centerlines being used as breaklines in order to simplify the cell faces.

Response: The channel breaklines have been now smoothened and an example screenshot shown below.

No additional comments.

- Duplicate the stream Breakline located at the A7/A8 domain boundary in each model.

Response: Since the tributary was modeled in domain 0203_A7 breakline was not added to 0204_A8 domain. However, agree that adding the breakline would have improved the 0204_A8 results for this tributary.

No additional comments.

- Planarize the lines used for breaklines to simplify cell geometry.

Response: The breaklines have been planarized and an example screenshot shown below.

No additional comments.

# 4 – Hydraulic Boundary Conditions

## Technical Review (Reviewer to provide comments)

Reviewer is to utilize detailed QA/QC to identify general review items for this audit.

## Review Comments

- Models A6 and A8 are not tying together at coincident locations.  Over the tie-in zone it appears that basin A6 is higher than basin A8.

Response: The models have been updated to achieve tie-in within 0.5’ and the 1% tie-in between 0202_A6 and 0203_A8 domains is illustrated in screenshots below.  For 0202_A5 domain the 1% WSEL of 1848.07’ is shown in the left screenshot and the 1% WSEL of 1847.68’ at the same location from domain 0202_A6 is shown in the right screenshot.

No additional comments.

- The transition period for A4 to A6 should be adjusted. Currently the SA2D lines being used to transfer flow are crossing the upstream and downstream end of the bridge and are not perpendicular to the primary flow path on the larger events.  Additionally, it does not appear that the mapping between A4 and A6 will tie together at the transition zone.  It is suggested that the hydrograph transition line and mapping transition zone be adjusted slightly downstream.  This will also dampen any effects of boundary conditions at the bridge here, which appears to be one of the larger and more sensitive transportation paths in the project area.  Small systems are incoming between the current transition of A4 to A6, and the point at which A5 enters A6.  It is not expected moving this transition line will significantly impact basin sizes.  Suggest moving to one of the three locations shown using red arrows in the below figures.

Response: The 0202_A4 model domain extent has been extended downstream per the ITR recommendation and the SA/2D connection is revised using suggested cross-section alignment provided in the markup above. A screenshot of the revised model setup for this area is shown in the screenshot below.

No additional comments.

# 5 – Hydraulic Model Execution

## Methodologies / Data Sources (Modeler to populate)

| Unsteady Plan Names |  |  |  |
| --- | --- | --- | --- |
| 10% ACE (10-year): | NRCS_10_PCT | 0.2% ACE (500-year): | NRCS_0.2_PCT |
| 4% ACE (25-year): | NRCS_4_PCT | 1% plus: | NRCS_1.0_PLU |
| 2% ACE (50-year): | NRCS_2_PCT | 1% minus: | NRCS_1.0_MIN |
| 1% ACE (100-year): | NRCS_1.0_PCT |  |  |
| Other: |  |  |  |

## Statistical Data (Modeler to populate)

| Time Step: | 30 s - Advanced time step based on Courant condition (7.5-60 Seconds) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Courant Value: | 0.45 – 2.0 |  | Simulation Length: | 36 | hours |
| Computation Interval: | (variable) 30 | seconds | Model Runtime: | Generally 1 | hours |
| Hydrograph Output Interval: | 10 | minutes | Mapping Output Interval: | 10 | minutes |

## Technical Review (Reviewer to provide comments)

Reviewer is to utilize detailed QA/QC to identify general review items for this audit.

## Review Comments

- No comments

# 6 – Hydraulic Model Results

## Methodologies / Data Sources (Modeler to populate)

| Flow Validation/Gage Analysis |  |
| --- | --- |
| Software: | HEC-RAS v 6.2 |
| Methodology | HEC-RAS 2D – rain-on-grid |
| Removal of Outliers and Justification | Refer to BLE report |
| Validation Notes (describe parameter adjustments): Refer to BLE report and Model Overview and Results Spreadsheet |  |
| Other: |  |

## Technical Review (Reviewer to provide comments)

Reviewer is to utilize detailed QA/QC to identify general review items for this audit.

## Review Comments

- Water surface elevations at the limit of the study (confluence of North and South Llano) were compared against the effective water surface elevations using an added profile line.  The water surface elevations reported were several feet lower than the effective profile (FIS rev. 11/05/1997).  It is understood that gage flows in this area are much lower than effective flows and this is impacting final study tie-ins.  However, two large streams are coming together at the end of the study and due to the nature of the confluence South Llano flows should be included in the model.  It is expected that this will raise water surface elevations around Junction, Texas and the study tie-in area. Please discuss the tie in at the downstream limit of study in the report, and provide reasoning for any remaining discrepancies in elevation and flow.

Response: The South Llano River inflow has been incorporated into the updated model for domain 0203_A8 and the downstream boundary condition for the 0203_A8 domain was changed from normal depth to stage hydrograph to achieve tie-in with Llano and North Llano HUC8 watersheds. Atkins is studying the North Llano and Llano HUC8s as part of this BLE study and used the Llano BLE model to obtain stage hydrograph for the North Llano downstream boundary. Updated WSEL comparison between effective and BLE model for cross-section B is shown in table below which shows a close match for 1% and 0.2% events. The discussion of the model boundaries and tie-ins has been added in the BLE report.

| AEP | Effective WSEL (ft) | BLE WSEL (ft) | Difference |
| --- | --- | --- | --- |
| 10% | 1686 | 1683.47 | -2.53 |
| 2% | 1695 | 1694.32 | -0.68 |
| 1% | 1698 | 1697.91 | -0.09 |
| 0.20% | 1706.5 | 1706.07 | -0.43 |

Regarding the peak flows at USGS gage, the InFRM analysis used much longer period of record than effective so better predicted flows compared to effective study. A comparison of gage flows is shown below:

| AEP | Effective | InFRM | BLE |
| --- | --- | --- | --- |
| 1% | 231,000 | 126,000 | 140,656 |

No additional comments.

# 7 – Preliminary (Raw) Mapping Results

## Technical Review (Reviewer to confirm with Pass/Fail)

| Hydraulic Model |  |
| --- | --- |
| Pass | HEC-RAS model with results, pertinent data and required storm events |
| Preliminary Mapping 1% ACE |  |
| Fail | Preliminary 10%, 1%, 0.2% ACE mapping results (raster grids from HEC-RAS) to meet FEMA mapping requirements |
| Fail | Boundary interfaces align matching or identified as AOMI |

## Review Comments

- See comment above regarding tie-in WSEL discrepancy.

Response: The models have been updated to achieve tie-in within 0.5’ over the tie-in zone.

No additional comments.

- FRD database and final mapping data were not provided for review.

Response: FRD is included in this submittal included mapping, Hazus and CNMS datasets.

See comments below in Section 8 Completeness Review

# 8 – Completeness Review

## Technical Review (Reviewer to confirm with Pass/Fail)

| File Structure and Naming |  |
| --- | --- |
| X | Deliverables provided in TWDB/FEMA required file folder structure and naming convention |
| X | Data missing (if any) documented in report and ReadMe file within the required folder |
| Report |  |
| X | Technical 2D BLE report documents all assumptions, special considerations, etc. |
| X | Flow validation and difference discussed in report |
| X | Flow comparisons discussed in report |
| Hydrologic Model |  |
| n/a | HEC-HMS model with results, pertinent data and required storm events |
| Hydraulic Model |  |
| X | HEC-RAS model with results, pertinent data and required storm events |
| Flood Risk Database |  |
| X | All features fully populated per current FEMA guidelines |
| X | Mapping consistent between grids to polygons for all three required mapped events |
| X | 10yr smaller than 100yr smaller than 500yr |
| X | Holes filled in polygon, depth and WSE grids |
| X | Boundary interfaces align matching or identified as AOMI |
| CNMS |  |
| X | Updated correctly with new centerlines |
| X | A1-A5 checks complete |
| X | CNMS Post- Processing Complete |
| HAZUS |  |
| X | Run per FEMA guidance |

## Review Comments

HAZUS QC

Cannot review HPR file due to difference in Hazus versions. Overall S_FRAC_AR matches GRID, Census blocks are sparse in some areas but this is expected since it’s a rural area.

S_FRAC_AR Comments/Fields to review:

SCENAR_ID- Should be a unique identifier assigned by table creator. Example sequential numbers with leading 0’s.

Response: - The Flood Risk Database Technical Reference states that SCENARIO_ID is associated with levee or dam scenario identification for L_Levee Scneario or L_Dam_Scneario tables. Since such scenario do not apply to this study, the field was left empty.  Please see the screenshot below:

Backcheck 2/10: Update with sequential numbers and leading zeros based off internal and regional comments received for Halff as discussed in meeting on 2/10.

Atkins Response (03/28/2023):  Scenario ID field is updated with sequential numbers and leading zeros.

Backcheck 6/12:  No further comments

SOURCE_CIT- Should be updated to HAZUS1 or REF1 according to guidance. See example below from Flood Risk Products Technical Reference.

Response: - The Source citation is now updated to REF1.

Backcheck 2/10:  No further comments

CNMS QC

S_Unmapped_Ln Comments/Fields to review:

Feature class populated incorrectly please read CNMS Technical guidance for direction. Currently S_Unmapped_Lns show linework for entire state of Texas and only lines outside of study area North Llano.

Response: - S_unmapped lines outside of the HUC have been deleted. Since BLE floodplain mapping extend upstream of new Zone X and there were no locations where new Zone X fell outside the new BLE floodplain, so the updated S_Unmapped_Ln layer was empty.

Backcheck 2/10: Correctly updated and removed S_Unmapped reaches outside of HUC boundary. Please review NorthLlano_CNMS_Backcheck QC polygons for reaches that currently can be added to S_Unmapped or attributed appropriately in S_Studies_Lns as backwater.

Atkins Response (03/28/2023): Addressed all QC comments in shapefile. Please see NorthLlano_CNMS_Backcheck_GISResponse for responses to each comment. The CNMS features have been revised per the ITR comment on 02/10. WTR_LN and S_Studies_Ln have been extended to 0.5 sq. mi based population of 1000 or more.  Coverage between WTR_LN and S_Studies_Ln have been checked and necessary changes are applied. The S_Studies_Ln have been extended where WTR_LN extended upstream of S_Studies_Ln and vice versa.

Backcheck 6/12:  Since CNMS can be decoupled form the other deliverables, we recommend coordination between Atkins, TWDB and Halff to establish a separate schedule for the completion of CNMS. CNMS backcheck was not completed with this review.

Atkins Response (06/15/2023): Per the direction from TWDB and Halff,  CNMS will be submitted separately for review.

Review CNMS Techinical Manual for guidance on generating S_Unmapped_Lns.

Response: - S_unmapped lines outside of the HUC have been deleted. Since BLE floodplain mapping extend upstream of new Zone X and there were no locations where new Zone X fell outside the new BLE floodplain, so the updated S_Unmapped_Ln layer was empty.

Backcheck 2/10: Correctly updated and removed S_Unmapped reaches outside of HUC boundary.

HUC8_KEY- Different values-All HUC8 keys should be the same. This is the HUC8 code for the watershed. Ex. North Llano 12090202

Atkins Response (5/1): HUC8 keys all calced with 12090202

Response: - The revised  S_Unmapped_Ln layer is empty after updates.

Backcheck 2/10: Please review NorthLlano_CNMS_Backcheck QC polygons for reaches that currently can be added to S_Unmapped or attributed appropriately in S_Studies_Lns as backwater.

Atkins Response (03/28/2023): Addressed all QC comments in shapefile. Please see NorthLlano_CNMS_Backcheck_GISResponse for responses to each comment. The CNMS features have been revised per the ITR comment on 02/10. WTR_LN and S_Studies_Ln have been extended to 0.5 sq. mi based population of 1000 or more.  Coverage between WTR_LN and S_Studies_Ln have been checked and necessary changes are applied. The S_Studies_Ln have been extended where WTR_LN extended upstream of S_Studies_Ln and vice versa. Backwater Reach identified in the ITR has been added to S_Unmapped_Ln.

Backcheck 6/12:  Since CNMS can be decoupled form the other deliverables, we recommend coordination between Atkins, TWDB and Halff to establish a separate schedule for the completion of CNMS. CNMS backcheck was not completed with this review.

Atkins Response (06/15/2023): Per the direction from TWDB and Halff, CNMS will be submitted separately for review.

S_Studies_Lns Comments/Fields to review:

Did not add all WTR_LNs from current BLE study, only shows current CNMS inventory and partial WTR_LNs. WTR_LNs should be added and attributed as Zone X. Any outside of BLE Fld_Haz_Ar should be added to S_Unmapped.

Response: - Wtr lines have been added, snapped to existing CNMS linework where necessary, and attributed as Zone X. No lines were outside of BLE floodplain, so none added to s_unmapped

Response: - For locations where WTR_LN feature extended upstream of previous S_Studies_Ln, the missing features are now added to S_Studies_Ln and they are extended to WTR_LN extent. Please note that all the Zone X streams in the S_Studies_Ln layer have been replaced using WTR_LN features.

Backcheck 2/10: Correctly updated S_Studies_Lns with missing reaches from WTR_LNs and extended. Some streams still need to be extended and or added from WTR_LNs feature class please review NorthLlano_CNMS_Backcheck QC polygons for reaches that currently can be added to S_Unmapped or attributed appropriately in S_Studies_Lns as backwater.

Atkins Response (03/28/2023): Addressed all QC comments in shapefile. Please see NorthLlano_CNMS_Backcheck_GISResponse for responses to each comment. The CNMS features have been revised per the ITR comment on 02/10. WTR_LN and S_Studies_Ln have been extended to 0.5 sq. mi based population of 1000 or more.  Coverage between WTR_LN and S_Studies_Ln have been checked and necessary changes are applied. The S_Studies_Ln have been extended where WTR_LN extended upstream of S_Studies_Ln and vice versa.

Backcheck 6/12:  Since CNMS can be decoupled form the other deliverables, we recommend coordination between Atkins, TWDB and Halff to establish a separate schedule for the completion of CNMS. CNMS backcheck was not completed with this review.

Atkins Response (06/15/2023): Per the direction from TWDB and Halff,  CNMS will be submitted separately for review.

STUDY_ID- Empty/Null suggested to populate with values in REAC_ID field or unique identifier.

Response: - Populated field using REAC_ID

Backcheck 2/10: Correctly updated, no further comments

FLOODWAY- Field currently empty- Suggested to update to acceptable domain value

Response: - Values in this field are now populated

Backcheck 2/10: Correctly updated, no further comments

VALIDATION_STATUS-Zone X and Zone AE correctly populated, Zone A should be populated based on A5 Pass/Fail. Currently fields marked as passed in REASON field marked as UNVERIFIED.

Response: - Values in this field are verified based on A5_COMPARE. For most Zone A streams A5_COMPARE failed so Validation Status set to Unverified for these streams. Where A5_COMPARE passed the validation status was set to valid.

If A5 is PASS = VALID

If A5 is FAIL = UNVERIFIED

If A5 is NULL, update based on A1-A4; any single failed check = UNVERIFIED

REASON- Incorrect for any lines that passed A5 check. Update to match A5 fields pass or fail.

Response: - Values in this field are verified based on A5_COMPARE. For most Zone A streams A5_COMPARE failed so Validation Status set to Unverified for these streams. For one stream A5_COMPARE passed so the Validation Status was set to VALID.

Backcheck 2/10: Correctly updated, no further comments

BLE_CASE_NO- Suggested update to MIP Case #

Response: - Added MIP case no for this BLE study.

Backcheck 2/10: Incorrectly updated in CNMS database currently has comment “CTP_FUNDED”

The BLE Case Number can be placed in both BLE_CASE_NO and BS_MIP_CASE_NUMBER. Currently BS_MIP_CASE_NUMBER has 22-06-0047S this can be placed in the BS_CASE_NO field.

Atkins Response (03/28/2023): Populated BLE_CASE_NO field with 22-06-0047S and BS_MIP_CASE_NUMBER for Being Studied fields with 22-06-0047S

Backcheck 6/12:  Since CNMS can be decoupled form the other deliverables, we recommend coordination between Atkins, TWDB and Halff to establish a separate schedule for the completion of CNMS. CNMS backcheck was not completed with this review.

Atkins Response (06/15/2023): Per the direction from TWDB and Halff, CNMS will be submitted separately for review.

BS_MIP_CASE_NUMBER- Suggested update to MIP Case #

Response: - Added MIP case no for this BLE study.

Backcheck 2/10: This field is populated correctly however check the MIP_CASE_NUMBER field in the CNMS database these two are often understandably confused. We have received comments from Michael Johnson FEMA Region 6 CNMS Lead to leave this field empty. This column should be empty and only contain any case numbers for Effective studies. Remove the case number added from this study and leave Null except for any effective studies that may have been filled out originally in the CNMS database. Make sure and do this in the BLE database deliverable as well remove/Null out MIP_CASE_NUMBER field.

Atkins Response (03/28/2023): Checked that MIP_CASE_NUMBER field is nulled except for case numbers that were previously in the database (there were none so all are null)

A1, CMT, SRC, URL- Populated correctly- All reaches should fail older study, TOPO_SRC not LiDAR.

A2, CMT, SRC, URL- Populated correctly- HYDRO_MDL UNKNOWN regression equations not known to be used.

A3, CMT, SRC, URL- Populated correctly- IS_URBAN field is "False," rural area.

A4, CMT, SRC, URL- Populated correctly- Older effective study not model backed.

A5, CMT, SRC, URL- Unable to verify if populated correctly A5 points not provided. Only one reach passed. Check how A5 points were generated and what methods were used and verify against CNMS Technical Manual.

Response: - CMT, SRC, and URL fields populated for A5 fields. The FBS audit points are now added under the supplemental data folder for FRP (\FRP\12090202\Final\Flood_Risk_Products\Supplemental_Data). For the FBS check the points were created every 100 feet along the effective floodplain and determined checked for vertical and horizontal check. Along with NFHL, Atkins acquired digitized effective floodplains from RMSI to get complete coverage of effective mapping.

Backcheck 2/10: Correctly updated, no further comments

FRD QC

- General comments

  - Consider using leading zeros for feature specific unique IDs. Not required, but good practice

Response: - Revised the IDs to have leading zero.

Backcheck 6/12:  No further comments.

  - Consider consistency with the HUC IDs. Either HUC8#_BLE or just HUC8#

Backcheck 6/12:  No further comments.

  - Is version ID of 01/2019 correct? This should reference the date of the standards being used for this study. Likely to be using 2022 standards.

Response: - Updated Version ID to the BLE_072021

Backcheck 2/10: Correctly updated, no further comments

  - Ensure all datasets are clipped to the HUC8 boundary

Response: - All datasets clipped to the HUC8 boundary

Backcheck 2/10: Correctly updated, no further comments

- BFE

  - EBFE_LN_ID- Consider using leading zeros

Response: - Revised the IDs to have leading zero.

Backcheck 2/10: there are ~500,000 records. EBFE_LN_ID should be ‘000001’ not ‘0001’. For reference, this is the Python we use within Field Calculator to create unique IDs with x number of digits.

rec=0

def autoIncrement():

global rec

pStart = 1

pInterval = 1

if (rec == 0):

rec = pStart

else:

rec = rec + pInterval

return str(rec).zfill(6)

autoIncrement()

Atkins Response (03/28/2023): Revised the IDs to have leading zero with correct number of digits

Backcheck 6/12:  No further comments.

  - There should be no BFEs outside the HUC8 boundary

Response: - BFEs are now clipped to the HUC8 boundary

Backcheck 2/10: Correctly updated, no further comments

- DTL_STUD_AR

  - DTL_LN_ID- in general, consider being consistent by either using HUC8#_BLE or just HUC8#

Response: - Revised to HUC8# consistently.

Backcheck 2/10: Correctly updated, no further comments

  - Source_CIT- Need to reference the detailed study/FIRM panel. Suggest “FIRM1”. Be sure to add citation to L_Source_Cit Table

Response: - Added FIRM1 to source_CIT field.

Backcheck 2/10: Correctly updated, no further comments

  - SCALE- field should not be blank. Typically populated with 1200 or 2400 depending on the detailed study information

Response: - Populated SCALE field

Backcheck 2/10: Correctly updated, no further comments

  - CD_POC field through CD_EMAIL, LOMR_ID, and STATUS- should be populated with NP

Response: - Populated field with NP value

Backcheck 2/10: Correctly updated, no further comments

  - Polygon area should be within/snapped to the HUC8 boundary. It should also closely follow the effective areas a little closer.

Response: - DTL_STUD_AR is now clipped to the HUC8 boundary

Backcheck 2/10: Correctly updated, no further comments

- DTL_STUD_LN

  - The goal of this layer is to identify stream reaches for areas within a watershed where detailed studies exist on the effective FIRM.  It is important that we refer inquirers to the effective detailed BFE in areas where detailed studies exist. Ensure that all NFHL Profile baselines and NFHL Waterlines in the DTL_STUD_AR are included.

Response: - Effective detailed study lines are added. We have used floodplain RMSI dataset in addition to NFHL to identify detail study areas.

Backcheck 2/10: Correctly updated, no further comments

  - CD_POC field through CD_EMAIL, LOMR_ID, and STATUS- should be populated with NP

Response: - Populated field with NP value

Backcheck 2/10: Correctly updated, no further comments

  - SCALE- should not be blank

Response: - Populated scale of 24000

Backcheck 2/10: Correctly updated, no further comments

- WTR_AR

  - Consider using leading zeros for WTR_AR_ID

Response: - Revised the IDs to have leading zero.

Backcheck 2/10: Correctly updated, no further comments

  - L_Source_Cit table says BASE1 is for TxDOT City Boundaries. Please add source citation for (NHD?) water bodies

Response: - Added BASE2 source_CIT for NHD and updated field.

Backcheck 2/10: Correctly updated, no further comments

  - Consider removing unnamed water bodies less than 5 acres.

Response: - WTR_AR features less than 5 acres have been removed from the database

Backcheck 2/10: Correctly updated, but the Acre field is still in there. Be sure to delete intermediary fields before submittal.

Atkins Response (03/28/2023): Acre field is now removed

Backcheck 6/12: Correctly updated, no further comments

  - Consider adding 10/100/500 year mapping polygons everywhere that there is a water body.

Response: - Ensured that floodplain mapping is present at WTR_AR features

Backcheck 2/10: Correctly updated, no further comments

- WTR_LN

  - Consider using leading zeros for WTR_LN_ID

Response: - Revised the IDs to have leading zero.

Backcheck 2/10: Correctly updated, no further comments

  - Many streams have long numbers. These would not be the names shown on the FIRM map. Recommend removing the numbers and leaving blank if there is no formal name. Make sure the main stem and major tribs are attributed correctly.

Response: - Stream name with numbers are now populated with NP

Backcheck 2/10: Correctly updated, no further comments

  - There are many disconnected lines, especially within the mainstem. Please review

Response: - Updated WTR_LN to be continuous

Backcheck 2/10: Correctly updated, no further comments

  - Source_Citation should be STUDY1, not BASE1 for TxDOT boundaries

Response: - Added source_CIT of Study1.

Backcheck 2/10: Correctly updated, no further comments

- FLD_HAZ_AR

  - Consider using leading zeros for EST_AR_ID

Backcheck 2/10: Correctly updated, no further comments

  - Zone_Subty should be populated with NP for 100yr

Response: - NP for Zone_Subty field in fld-haz-ar is not an acceptable value per tech reference and should be left blank. There have been previous ITR comments to populate this value as blank for 100yr FP

Backcheck 2/10: Correctly updated, no further comments

  - Study_Typ should be populated with NP

Response: - Updated NP values to Study_TYP field

Backcheck 2/10: Correctly updated, no further comments

- TENPCT_FP

  - Consider using leading zeros for EST_AR_ID

Backcheck 2/10: Correctly updated, no further comments

  - Zone_Subtyp should be populated with NP for 10yr

Response: - Updated NP values to Zone_subtyp field

Backcheck 2/10: Correctly updated to “”, no further comments

  - Study_Typ should be populated with NP

Response: - Updated NP values to Study_TYP field

Backcheck 2/10: Correctly updated, no further comments

  - Dual_Zone should be N-No

Response: - Updated with “F” (domain value)

Backcheck 2/10: Correctly updated, no further comments

- Unmapped_ln

  - Feature class is empty in FRD DB

Response: - S_unmapped lines outside of the HUC have been deleted. Since BLE floodplain mapping extend upstream of new Zone X and there were no locations where new Zone X fell outside the new BLE floodplain, so the updated S_Unmapped_Ln layer was empty.

- 1% Depth and WSE grids show holes where mapping polygons do not

Response: - grids  are now clipped to the floodplain polygons.

Backcheck 2/10: Correctly updated, no further comments

- AOMI_PT

  - Consider using leading zeros for AOMI_ID

Response: - Revised the IDs to have leading zero.

Backcheck 2/10: Correctly updated, no further comments

  - Consider changing STUDY2 to REF1 for HAZUS citation. STUDY is typically reserved for a whole BLE study, not a HAZUS reference dataset.

Response: - Added source citation of REF1 for Hazus

Backcheck 2/10: Correctly updated, no further comments

  - Consider adding schools to the "at risk essential facilities" from HAZUS, significant land use change, or NOAA storm events. Be sure to add to the source citation table too

Response: - There are no schools with HUC boundary (all are located just outside of the watershed boundary).

Backcheck 2/10: Correctly updated, no further comments

- AOMI_AR

  - Consider adding an AOMI area near the detailed study area or anywhere that warrants further investigation or research for later studies in this region.

Response: - Added significant landuse change areas in AOMI_AR and added source cite from NLCD

Backcheck 2/10: Correctly updated, no further comments

- S_FRAC_Ar

  - SCENAR_ID should have unique IDs

Response: - The Flood Risk Database Technical Reference states that SCENARIO_ID is associated with levee or dam scenario identification for L_Levee Scneario or L_Dam_Scneario tables. Since such scenario do not apply to this study, the field was left empty.

Backcheck 2/10: As discussed in the 2/10 meeting, this field should be used to store unique IDs

Atkins Response (03/28/2023): Added unique IDs to SCENAR_ID field

Backcheck 6/12: Correctly updated, no further comments

  - Consider changing STUDY2 to REF1 for HAZUS citation. STUDY is typically reserved for a whole BLE study, not a HAZUS reference dataset.

Response: - Added source citation of REF1 for Hazus

Backcheck 2/10: Correctly updated, no further comments

- S_HUC_AR

  - Consider removing HUC12s from dataset.

Response: - Deleted HUC10 and HUC12 from HUC_AR.

Backcheck 2/10: Correctly updated, no further comments

  - Source_Cit is incorrect

Response: - Updated  source citation to BASE2

Backcheck 2/10: Correctly updated, no further comments

  - Boundaries are particularly jagged. Consider using smooth NHD HUC boundaries. Be sure to add record to Source Citation table

Response: - Updated HUC boundary using USGS HUC8

Backcheck 2/10: Correctly updated, no further comments

- S_Pol_Ar

  - Consider using leading zeros for Pol_Ar_ID

Response: - Revised the IDs to have leading zero.

Backcheck 2/10: Correctly updated, no further comments

  - COM_NFO_ID is blank

Response: - COM_NFO_ID field is now populated with CID values

Backcheck 2/10: Correctly updated, no further comments

  - There is no BASE2 in the Source_Cit table. Be sure to have citations/descriptions for both cities and counties.

Response: - Both city and county boundaries were from same source (TxDot).  Updated metadata title to “TxDOT Political Boundaries”

Backcheck 2/10: Correctly updated, no further comments

- Metadata was not included for review

Response: Metadata is now included in the task documentation folder

Backcheck 6/12: FRD metadata provided. However, for the EBFE deliverable, the Hydraulics Metadata is required and not FRD. Please update to correct metadata with next submittal.

Atkins Response (06/15/2023): Updated metadata is now included.

Response (6/21) : Metadata is updated using Hydraulics metadata

Halff Backcheck 07/21/23: Complete submittal was not provided to verify metadata was revised to hydraulic metadata. Please provide complete package in next submittal.

Atkins Response (8/14/23): Metadata file has been revised as requested and included in this submittal.

Mapping QC

QC comments can be found in the included shapefile.

Please review shapefile for responses

Backcheck 2/14: Mapping does not tie in with Lower Llano BLE study within 0.5ft.

Atkins Response (03/28/2023): North Llano has tied in with South Llano at the HUC edge boundary and holes less than 2 acres for the FLD_HAZ_Ar have closed.  Please find screenshots after mapping revisions:

Backcheck 6/12/23:

- _WTR_LN do not cover CNMS S_Studies. Please provide coverage of S_Studies.

Atkins Response (06/15/2023): Since this comment is tied to CNMS linework, we will ensure the consistency between WTR_LN and CNMS as part of the CNMS submittal.

- Issues with mapping tie-ins with the adjacent South Llano HUC still exist, see example screen shots below. Variances in tie-in depths exceed 0.5 feet for the 100- and 500-yr frequencies. Please address for all areas along HUC fringe.

Atkins Response (06/15/2023): We have reviewed the fringe mapping and updated the FLD-HAZ_AR polygons and grids to resolve any tie-in inconsistencies where the flooding extents at the HUC8 boundary had differences or had more than 0.5’ in WSE difference. The responses to the comments are added to shapefile. Also copy of latest South Llano grids included to aid the review.

Halff Backcheck 07/21/23:  Refer to QC shapefiles “NLLanoBackcheck_06302023”, “DiffWSE500_Ovr05” and ….: for all subsequent comments for example areas.

Atkins Response 8/14/23: Responses have been added to the attribute table of the shapefile.

- In the grid modification process, the 0.2% grids now have a shift as compared to the 1% grids and previous submittal grids and don’t align with the mapping. Refer to QC shapefiles. The depth grid will have difference values with the new alignment in relation to the ground DEM.

Atkins Response 8/14/23: See below for a screenshot showing that this issue has been resolved.

0.2% polygon overlay on top with WSE 0.2%

1% polygon overlay on top with WSE 1%

- Updated 0.2% rasters do not meet 0.5-ft tie-in requirement. *Note: For QC purposes, a modified 0.2% grid shifted to the appropriate alignment (5-ft to the east) was created for assessment of remaining tie-in issues. Areas noted would remain an issue whether using a temporary shifted grid or the one submitted. Review and address for entire HUC fringe, not only areas of grid overlap where points were generated.

Atkins Response 8/14/23: See QC point shapefile showing that this issue has been resolved.

- Updated 0.2% raster values have changed from previous submission resulting in new areas that do not meet the 0.5-ft tie-in requirement. Review and address for entire HUC fringe, not only areas of grid overlap where points were generated.

Atkins Response 8/14/23: See QC point shapefile showing that this issue has been resolved.

- Tie in issue, 0.2% grids. Mapping may not be appropriate on S Llano side, areas where DEM showing a higher elevation that the WSE.

Atkins Response 8/14/23: See below for a screenshot showing that this issue on South Llano has been resolved.

- Tie in issues remain on the South Llano side.

Atkins Response 8/14/23: See below for a screenshot showing that this issue has been resolved.

Additional tie-in issues:

Atkins Response 8/14/23: See below for a screenshot showing that this issue has been resolved.

- Updated 1% raster values have changed from previous submission resulting in new areas that do not meet the 0.5-ft tie-in requirement. Review and address for entire HUC fringe, not only areas of grid overlap where points were generated. One example area below:

Atkins Response 8/14/23: See QC point shapefile showing that this issue has been resolved

QA/QC Checklist Approval

This 2D Base Level Engineering Analysis QA/QC Review is in compliance with the contract requirements and all task checkpoints are complete.  The independent QC Team has reviewed the analysis, presented review comments to the Production Team Task Leader, and discussed any problems or issues.  The Task Leader, QC Manager, QC Reviewer, and Project Manager have signed the QA/QC Checklist to confirm that all comments have been received, addressed, and documented appropriately.

| QA/QC Team – [TWDB or company name] |  |  |  | Production Team – [add company name] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| QC Manager |  | Date |  | Task Lead |  | Date |
| QC Reviewer |  | Date |  | Project Manager |  | Date |