# Texas 2D BLE Analysis QA/QC Checklist

# Project Information

| HUC-8 Watershed Name & ID: | Austin-Travis Lakes (12090205) |
| --- | --- |
| HUC-10 Watershed Name & ID: | Lake Marble Falls-Lake Travis (1209020501) - Model 0501_A1 Cow Creek-Lake Travis (1209020502) – Model 0502_A2 City of Austin-Colorado River (1209020503) – Model 0503_A3 City of Austin-Colorado River (1209020503) – Model 0503_A4 Onion Creek-Colorado River (1209020504) – Model 0504_A5 Onion Creek-Colorado River (1209020504) – Model 0504_A6 Onion Creek-Colorado River (1209020504) – Model 0504_A7 City of Austin-Colorado River (1209020503) – Model 0503_A8 City of Austin-Colorado River (1209020503) – Model 0503_A9 Onion Creek-Colorado River (1209020504) – Model 0504_A10 |
| Modeler: | Atkins |
| Reviewer: | Halff |

## Terrain and Modeling Review (QC Items 1-6)

| Modeler Initials: | MJ | Reviewer Initials: | KH & CR |
| --- | --- | --- | --- |
| Submittal Date: | June 24, 2022 | Review Submitted Date: | 3/27/2023 |
| Review Response Date: |  | Backcheck Complete Date: | 7/27/23 |

## Mapping Review (QC Item 7)

| Modeler Initials: | PC | Reviewer Initials: | NB |
| --- | --- | --- | --- |
| Submittal Date: | July 28, 2022 | Review Submitted Date: | 3/12/2024, 6/10/2024 |
| Review Response Date: |  | Backcheck Complete Date: | 3/21/2024, 6/21/2024, 6/27/2024 |

## Completeness Review (QC Item 8)

| Modeler Initials: |  | Reviewer Initials: | NB |
| --- | --- | --- | --- |
| Submittal Date: |  | Review Submitted Date: | 3/12/2024, 6/10/2024; 6/25/2024 |
| Review Response Date: |  | Backcheck Complete Date: | 3/21/2024, 6/21/2024, 6/27/2024 |

(Modeler to populate with an “X” for items included in the QC submittal)

| Level of Study |  |  | Flood Events |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| X | 2D BLE |  | X | 10% ACE (10-year) | X | 0.2% ACE (500-year) |
|  | Enhanced BLE |  | X | 4% ACE (25-year) | X | 1% plus |
|  |  |  | X | 2% ACE (50-year) | X | 1% minus |
|  |  |  | X | 1% ACE (100-year) |  | Observed Rainfall |

Instructions

Each section includes instructions to the modeler and reviewer regarding their responsibilities in the completion of this checklist. In the Methodologies / Data Sources tables, grey italic text is added as an example response. Additional sample responses are located in the Texas 2D BLE Guidelines Quick Guide.

Reviewer to indicate if Technical Review items generally comply using a “Pass”. If the technical review observes an issue of noncompliance or an item that requires additional attention, an “Fail” should be utilized.

## Submittal Checklist

Data submitted for each milestone should be carried through each subsequent milestone. All geospatial layers and models must have same defined projection system as terrain. Modeler to populate with an “X” for items included in the QC submittal.

| Base Map Data (QC items 1-6) |  |
| --- | --- |
| X | Location Map (PDF): HUC-8 and HUC-10 boundaries and/or model domain boundary if different from HUC-10s, stream gages, dams (if applicable), mainstem streams (for reference), background map |
| X (RAS Mapper) | GIS Map (MXD or Map Package) with the following data included but not limited to: Stream centerlines, model breaklines, rainfall data, WSEL and Depth grids (10, 100, 500-yr), adjacent HUC mapping (available on the BLE viewer or currently studying), 2D connections for inflows/outflows, gages or points of flow comparisons (BFEs, highwater marks, etc.), profile lines, boundary condition lines, refinement regions, model domain boundary, model mesh, DFIRM mapping panels, base map data (roads, aerials, etc.), soils data, land use data, terrain |
| Technical Report / Documentation (QC items 1-6) |  |
| X | Technical Report / Project Notebook: assumptions, special considerations, and modeling concerns |
| X | Detailed explanation and support data of variances in methodologies and approaches from recommended guidelines (Should be included in Technical Report / Project Notebook) |
| X | Internal QA/QC Documentation: comments and responses |
| X | Flow comparison tables within the technical report |
| Hydrology (QC items 1-6) |  |
|  | Hydrologic Model (HEC-HMS) model with all associated DSS files for 7 required storm events |
| X | Hydrologic Data and Parameter Spreadsheets (rainfall calculations, curve number table, gage locations, gage analysis and flow validation) |
| X (Memo/ results spreadsheet) | Hydrologic modeling notebook including assumptions, notes, and special conditions (if not provided in report) |
| Hydraulics (QC items 1-6) |  |
| X | Hydraulic Model (HEC-RAS) with all associated files and 7 required storm events (remove non-final files) |
| X | Hydraulic Data and Parameter Spreadsheets (Manning’s n-value table, infiltration calculations support data-Soils, Land Use, Curve Number table, other calculations/data used in model) |
| X | Manning’s n-value table |
| X | Precipitation data correctly linked to hydrologic model or hard coded from provided spreadsheet |
| X | Hydraulic modeling notebook including assumptions, notes, and special conditions |
| Mapping (QC item 7) |  |
| X | Flood risk database |
|  | Modeler to complete topology validation as outlines in the guidelines |
| X | Modeler to verify mapping meets deliverable requirements as outlined in guidelines |
| Completeness Review (QC item 8) See Texas 2D BLE Guidelines Chapter 4 – Deliverables |  |
|  | Technical BLE Report |
|  | Final cleaned hydrologic model |
|  | Final cleaned hydraulic model |
|  | Flood risk database |
|  | CMNS updates |
|  | HAZUS results |
|  | Additional supplemental data |

# 1 – Model Terrain

Methodologies / Data Sources (Modeler to populate)

| Terrain Source(s): | Please see technical memo for more detail: Hurricane Lidar (2019), Central Texas Lidar (2017), North and Central Lidar (2020) |
| --- | --- |
| Data Gaps: | None |
| Source Resolution: | Cell size of 1 x 1 meter |
| Model Terrain Resolution: | Cell size of 5 x 5 feet |
| Terrain Horizontal Projection: | North American Datum (NAD) 1983 (2011) Texas State Plane Zone 3 (Central) |
| Terrain Vertical Projection: | NAVD88 |
| Z Values: | Units of feet |
| Terrain Buffer: | 2,500 feet |
| Other: |  |

## Technical Review (Reviewer to confirm with Pass/Fail)

| General Review |  |
| --- | --- |
| P | LiDAR source matches the latest available data at time of project initiation |
| P | Large reservoir or lake extents included in model domain |
| F | Check visible seams, gaps or holes of LiDAR sources (if more than one source was used) |

## Review Comments

- On Domain A2 there is a small part of 2D mesh that extends past Terrain limits

Atkins Response (04/20/2023):  Only a very small portion of a cell appears to be missing. Adding the terrain will require model reruns and this change will not have any impact on the model results. The model extent includes a buffer Therefore, this change was not applied.

Halff Response (7/26/2023): No Further Comment.

# 2 – Hydrology

## Methodologies / Data Sources (Modeler to populate)

| Software: | HEC-RAS 6.2 |
| --- | --- |
| Rainfall Source: | NOAA Atlas 14 |
| Rainfall Duration: | 24 hours |
| Depth Selection: | Domain Average |
| Rainfall Temporal Distribution: | NRCS |
| Areal Reduction: | None |
| Loss Methodology: | Spatially varied CN |
| Unit Hydrograph Transform Method: | N/A |
| Inflow Hydrograph Source: | Inflows vary by model. Major inflows include Colorado River outflow from LBJ Lake, Pedernales River HUC8, Barton and Onion Creek and its tributaries |
| Other: Precipitation spreadsheets are included with the submittal. |  |

## Statistical Data (Modeler to populate)

| Stream Gage Name & ID: | Barton Ck at SH 71 nr Oak Hill, TX (08155200) |
| --- | --- |
| Stream Gage Name & ID: | Bull Creek at Loop 360 near Austin, TX (08154700) |
| Stream Gage Name & ID: | Barton Creek at Lost Creek Boulevard near Austin, TX (08155240) |
| Stream Gage Name & ID: | Barton Creek at Lost Creek Boulevard near Austin, TX (08155300) |
| Stream Gage Name & ID: | Colorado Rv at Austin, TX (08158000) |
| Stream Gage Name & ID: | Walnut Creek at Webberville Road, Austin TX; 08158600 |
| Stream Gage Name & ID: | Onion Creek at US Highway 183 – Gage #8159000 |

## Technical Review (Reviewer to confirm with  Pass/Fail)

| Excess Rainfall |  |
| --- | --- |
| F | Precipitation values are an aerial average of Atlas 14 values for the entire 2D area boundary for each storm event or other appropriate rainfall application |
| F | CN values appropriately reflect land use and soils for entire 2D area boundary |
| P | Losses are correctly calculated to establish the excess rainfall hyetographs by checking land use, soils and infiltration layers or other relative information |
| P | Total precipitation depths for 1%+ and 1%- are calculated correctly |
| P | Excess rainfall hyetographs are calculated for all storm events (10%, 4%, 2%, 1%, 0.2%, 1%+, and 1%-) |
| P | Cumulative excess rainfall values are in increasing order from 10%, 4%, 2%, 1%, to 0.2% |
| P | Cumulative excess rainfall values for the 1%+ and 1%- fall in order with the storm events matching the total precipitation values |
| Inflow Hydrographs |  |
| NC | Shape and time-to-peak of any inflow hydrographs resemble past documented large storm events |
| F | Hydrographs reasonable when compared to the other return periods. The shapes should be similar, and hydrographs from different events should not intersect unless there is an engineering reason. Peak flows from a lower return period event should not be higher than a higher return period event |
| F | Contributing hydrographs from another RAS model applied at coincident locations (Provide locations in base map data) |
| General Hydrology |  |
| Yes | Any flow regulating dams in the model area? (Yes/No) |
| No | Dam modeling methodologies applied and explained? (Yes/No). If yes, provide discharge rating curve calculations and gage analysis documenting regulated versus unregulated records, if applicable. |
| Yes | Model aligns with documentation of dam modeling approaches. |
| Other Notes: NA |  |

## Review Comments

- Per the TWDB guidance document “Texas Two-Dimensional Base Level Engineering Guidelines, version 1.0” dated May 21, 2022, the TWDB prefers precipitation hyetographs to be developed within HEC-HMS and any variation must be approved with prior coordination with TWDB. The submitted supplemental documentation shows development of hyetographs in excel based on duplication of Fortran code from WinTR-20.

Atkins Response (04/20/2023): The NRCS nested hydrograph method is another commonly used temporal distribution which Atkins had used successfully on previous projects. Our model development for the TO1 of this study was well underway before TWDB guidance was published so we had used NRCS distribution.  Additionally, in past monthly progress meetings we have discussed this topic with TWDB staff, and they agree that NRCS nested rainfall methodology is acceptable and can be used for 2D BLE model development. It is our understanding that this methodology will be added to the TWDB Guidance at some point in the near future.

Halff Response (7/26/2023): No Further Comment.

- Precipitation does not appear to be consistent with Atlas 14 per NOAA’s website for several domains. Tables A4, A8 and A9 are identical and do not match closely to precipitation data pulled from each domain’s centroid.

Atkins Response (04/20/2023): When we began the model development for this watershed model domains A4, A8 and A9 were part of one domain. There are large urban areas in this watershed and to reduce model run times this area was split into 3 smaller domains. Areas for model domain for A4, A8, and A9 are 129, 50, and 56 sq. mi. respectively. The rainfall depths are similar, so kept the same rainfall for the three domains.

Halff Response (7/26/2023): Clearly document the source of the rainfall within the report. Include within the documentation a discussion of local rainfall regulations (City of Austin) in order to demonstrate that they are different.

Atkins response (08/08/2023): Subsection 1.2.10 has been revised to say why rainfall for the three domains is identical. Text has added that states that City of Austin regulations were not applied.

- Clearly state in report how precipitation values were identified. Verify values for each domain.

Atkins Response (04/20/2023): Additional details of the precipitation methodology will be added to the report. The precipitation spreadsheets are included the supplemental data folder. The precipitation values are verified and correct.

Halff Response (7/26/2023): Unresolved. Text Not Provided. Please provide report.

Atkins response (08/08/2023): Subsection 1.2.10 has been revised to align with other BLE reports prepared for this task order.

- TWDB recommends the use of areal reduction as a validation parameter; this was not mentioned in the Buchanan report. Clearly state in the report if areal reduction was applied or considered.

Atkins Response (04/20/2023):  Atkins was able to achieve flow validation using curve number and manning’s n 	updates so did not use aerial reduction factor as a validation parameter for this watershed. We have added a 	statement regarding this in Section 1.2.9 Model Validation of the report.

Halff Response (7/26/2023): Unresolved. Text Not Provided. Please provide report.

Atkins Response (08/08/2023): A sentence saying that areal reduction factors were not applied has been added to Subsection 1.2.10.

- The TWDB recommends that base curves numbers be utilized with a separate percent impervious value (for developed areas). Explain variance from recommendations. This affects the “developed” conditions.

Atkins Response (04/20/2023):  For developed categories, we have applied the composite TR55 curve numbers equivalent to the values in the TWDB guidance. For example, for developed high intensity soil group D the TWDB guidance document listed a base CN value of 80 with 85 percent imperviousness. For this category, we have applied a corresponding TR55 composite value of 95 (prior to validation) from commercial and business land cover category, where impervious area of 85% is included in composite value calculation.

Halff Response (7/26/2023): No Further Comment.

- There are a number of CNs in the models that do not meet plus or minus 10% of either the AMC – 1 or AMC – 2 per section 1.2.2 (Domains A1 and A6). Please revise curve numbers or provide additional clarification in the report.

Atkins Response (4/20/2023): Published documents show that there is large variation in AMC conditions across Texas however, few sources are available which depict how AMC condition vary. Through some research Atkins was able to find the following map from Page 23 of Engineering Technical Note No. 210-18-TX5 titled Estimating Runoff for Conservation Practices developed by SCS. The term Antecedent Runoff Condition (ARC) used in the older literature is analogous to Antecedent Moisture Condition (AMC), which is the term that is commonly accepted today. During model validation process for some of the model domains the computed BLE flows were too high with 90% AMCII CN values. After research of published documentation on the topic of AMC conditions in Texas, we obtained the following map which shows gradual transition from AMCI to AMCII condition from west Texas to east Texas. So instead of lowering the AMC II CN values more than 10% the AMCI curve number values were used to validate flows. For two domains, we weighted the CN values to by assigning 70% weight to AMCI and 30% weight to AMCII. Additional discussion is added in the report Section 1.2.9 Model Validation. This graph was included in appendix B of the word document of the draft BLE report.

Halff Response (7/26/2023): No Further Comment.

- A number of inflow hydrographs have crossing profiles. These include: A1 and A10.

Atkins Response (04/20/2023):  Where there are crossing profiles between 1% plus and 0.2% AEP additional discussion will be added under challenges section of the report. For domains A1 and A2 the 1% plus and 0.2% profiles cross as the Pedernales River inflow for 1%plus flow of 734,000 cfs is higher than 0.2% AEP flow of 479,000 cfs. The Pedernales River inflows are established based on InFRM gage analysis are shown in screenshot below.

Screenshot from Draft InFRM report Page 152:

For A10 in the last model version we slightly varying normal depth slopes for each profile to tie-in with effective AE. However, they are now changed to same normal depth slope for all profiles of to eliminate crossing profiles.

Halff Response (7/26/2023): Unresolved. Text Not Provided. Please provide report. Per a similar discussion during the June 21, 2023 call regarding Middle Colorado, the crossing profiles is due to rain on mesh for the local tributaries and application of the InFRM Watershed Hydrology Assessment inflow hydrographs for the Colorado River mainstem. If this applies to the Austin Travis models, further discussion is required in the BLE report to document both the modeling approach, reason for the crossing profiles and discussed in challenges for future mapping partners.

Atkins Response (08/08/2023): The modeling approach for the Pedernales River has been revised. The WHA team recently provided the TWDB with an HEC-HMS mode of the Colorado River watershed. Although this model was received too late in the modeling workflow to be used extensively in the BLE assessment, Atkins did extract hydrographs for the Pedernales River and used them to establish the river’s discharges into the Colorado River. The report has been updated to document this modeling approach. See our response below in Comment 7 of the Hydraulic Boundary Conditions section for more information about the application of these HEC-HMS hydrographs. This figure shows the hydrographs that enter the Colorado River from the Pedernales River. Note that they no longer cross.

Following these changes to the Pedernales River, the crossing profiles out of A1 have been resolved.

For the inflows in to A10, there continue to be crossing profiles, but there is a good reason for this. These crossing profiles will be documented in the report.

The inflow hydrographs for A10 are here:

Note how the 10% hydrograph (red) eventually becomes aligned with the 4% hydrograph just above it. The reason this is happening is that the RFA for Lake Travis computed identical discharges out of Lake Travis for the 20%, 10%, and 4% events as documented in this table from the RFA:

The first peak in the hydrographs is due to local rainfall runoff. The second peak is driven by the outflows from Lake Travis. Since the RFA computed identical outflow discharges from the dam for the 10% and 4% events (30,000 cfs), the hydrographs for the 10% and 4% eventually align with each other in A10.

# 3 – Hydraulic Geometry

## Methodologies / Data Sources (Modeler to populate)

| Software: | HEC-RAS 6.2 |
| --- | --- |
| Loss Methodology: | Spatially varied CN |
| Soils Source: | SSURGO |
| Land Cover Source: | NLCD 2019 |
| Manning's Roughness Source: | Assigned based on land use. Please see table in Model Overview and Results.xlxs |
| Nominal Mesh Cell Sizes: | 200x200, except for Channel Refinement Regions, Breaklines and Urban Regions where 100X100 was considered. |
| Other: |  |

## Technical Review (Reviewer to confirm with Pass/Fail)

| 2D Mesh |  |
| --- | --- |
| F | 2D Boundary delineated along watershed boundary or buffer to ensure all contributing area is included |
| P | Nominal cell size appropriate for model detail (generally 200 feet, minimum 50 feet for refinement areas) |
| - | Computation points are at default locations or changes documented |
| F | Mesh Cell and Face tolerances are default or changes documented |
| P | No mesh errors |
| P | Cell face relationships are calculated |
| Breaklines |  |
| - | Breaklines are consistent with the terrain data |
| - | Breaklines sufficiently detailed to capture features (thalweg, banks, berms, etc.) |
| - | Cells do not overlap terrain features where appropriate (i.e., unwanted leaking) |
| P | Breakline cell sizing and parameters are appropriate |
| p | 2D Area Connections placed at applicable benchmarks, gages, or dams |
| F | 2D Area Connections set to use normal 2D equation domain for benchmarks and gages. Appropriate modeling approach used for dams. |
| Manning’s N-Values |  |
| F | Manning’s n-values associated to land cover data provided |
| P | Land Cover classifications are sufficiently descriptive within the data table |
| p | Manning’s n-value classifications generally match current aerial imagery |
| P | Manning’s n dataset is associated with the geometry files |
| General |  |
| F | All model datasets each have unique descriptive names |

## Review Comments

- The project notebook was not provided; therefore computation points changes could not be verified or identified.

Atkins Response (04/20/2023): The validation locations are documented in the Model Overview and Results spreadsheet (AustinTravis Model Overview and Results.xlsx) located under Task Documentation folder of hydraulics submittal. Additional details of validation are added in the BLE report.

Halff Response (7/26/2023): This comment is in reference to computation points located within the HEC-RAS model (cell centers). No further comment.

- It was noted in A4 that the solver core is different from default. Please confirm change was intentional.

Atkins Response (04/20/2023): Number of solver cores doesn’t affect model results. It was intentional change.

Halff Response (7/26/2023): No further comment.

- Domain A3 and A9 channel manning’s is 0.065 but on report channel manning’s range is 0.04 to 0.05. Clarify for consistency.

Atkins Response (04/20/2023): We will revise BLE report to correct the range to 0.065.

Halff Response (7/26/2023): Unresolved. Text Not Provided. Please provide report.

Atkins Response (08/08/2023): Subsection 1.2.2 has been revised to note that channel n ranges up to 0.065.

- Some of the Manning’s/ land use values for developed areas do not align to best available aerial imagery. Due to the application of NLCD land use data, these areas of land use change should be identified in the FRD feature dataset AOMI_AR as a location of significant land use change. Please discuss in the report in Section 2.0 Challenges.

Atkins Response (04/20/2023): We will add a discussion under challenges section and will add AOMI points in the FRD to capture significant land use change area

Halff Response (7/26/2023): Unresolved. Text Not Provided. Please provide report.

Atkins Response (08/08/2023): The Challenges section now includes the requested language.

- Report final Manning’s values for each domain within the report (or appendices).

Atkins Response (04/20/2023): We will update the Model Parameter adjustment summary table (Table 9) to clarify which n values were used and update validation section to further clarify parameter adjustment. However, n values for each domain will not be added as it would redundant. Details of validation are already captured in the model overview and results spreadsheet.

Halff Response (7/26/2023): Unresolved. Text Not Provided. Please provide report.

Atkins Response (08/08/2023): There is a table in Subsection 1.2.10 called Summary of Parameter Adjustments Made During Validation in the Austin-Travis Lakes Watershed that summarizes how the Manning’s n values were revised during the validation task. This is what we have done in our other BLE reports for this task order.

- Items in red highlight within the checklist above were not reviewed in detail. Please ensure these modeling assumptions/parameters have been reviewed and are appropriate. Also, review comments provided in previously completed detail review(s) and address accordingly as appropriate and applicable for this watershed.

# 4 – Hydraulic Boundary Conditions

## Technical Review (Reviewer to confirm with Pass/Fail)

| Boundary Review |  |
| --- | --- |
| P | External boundaries connected to 2D mesh |
| P | Boundary conditions everywhere flow could build up at the boundary of the 2D mesh |
| P | Boundary conditions have a hydrograph or elevation/slope data assigned |
| P | Input hydrographs consistent with hydrologic data (DSS, manual entry) |
| P | Precipitation data is consistent with hydrologic data (DSS, manual entry) |
| F | Downstream boundary consistent with ground slope or known downstream WSEL |
| F | Flow hydrographs applied at appropriate locations |
| P | Energy grade slope for hydrograph distribution is reasonable |
| P | Internal rating curves appropriate and documented for control structures (if applicable) |

## Review Comments

- Note on the review, only the 500-year event flow interface was verified. All storm events must match between domains. The following flow interfaces did not match between domains: A5_02-A6, A8_0-A10 and A8_1-A10. Review all boundary conditions for all storm events. The flows should match between domains by no greater than 5% difference. This can best be achieved using .dss files.

Atkins Response (04/20/2023): All flow transfers are within 5%, the flow values. A table documenting inflows and outflows will be added to the report. The models are setup using DSS and flows are not hard coded.

Halff Response (7/26/2023): At the boundary conditions above, the 500-year flows are NOT within 5% difference. For example, at the A5_2-A6 boundary condition, the A5_2 2D connector shows a flow of 987.5 cfs while the A6 inflow hydrograph in the unsteady flow editor shows a peak flow of 2436.98 cfs. This is a difference of 147%.

Additionally, the flow at the downstream of Lake Travis is 530,872 cfs in the model. The INFRM hydrograph applied at A4 downstream has a peak flow of 261,100 cfs. This is a significant difference showing poor validation in the model on the mainstem. Review the decoupled modeling approach to ensure that the inflows used align with the InFRM data. A decoupled modeling approach should provide two separate sets of models: One for the mainstem using the inflows/outflows from the InFRM study but no rain on grid since these inflows are already accounted for in the mainstem inflows within the InFRM study. The second set of plans should represent the rain on grid modeling for the tributaries but no inflow/outflows on the mainstem. This prevents double counting flows which could be resulting in the significantly higher inflow.

Atkins Response (08/08/2023): The current version of the models shows all flows within 5% at the tie-ins at the domain outlets for A5 and A8 (and other domains). The tie-in tables in the report have been updated to include these numbers. For domain A5, for main outfall connection the 500-yr flows are within 5% (166,856 cfs vs 164,009 cfs  - 1.74% difference ). The review comment noted the difference between the secondary overbank connection which are very small compared main channel outfall and combined main channel and overbank the flows are within 5%.

Regarding Lake Travis, we have reservoirs in several of our HUC-8 watersheds in this task order. We have not included 2 sets of plans for any of these reservoirs. At the beginning of the task order, Atkins met with the TWDB to discuss the application of a Joint Probability Analysis (JPA) at the reservoirs in the study area to produce more realistic discharges/volumes in the reservoirs. The JPA would have used two sets of plans, one pluvial and one fluvial. The TWDB asked Atkins not to apply this approach; therefore, only one set of plans was used in the BLE models. It was acknowledged at the time that this approach would generate high flows/volumes in the reservoirs, but the TWDB was concerned that multiple sets of plans would confuse end users of the BLE models.

- Document boundary condition approach in report to justify the longer ND BCs seen in the models. Verify there is not unaccounted for flow transfer between the domains not at the main channel crossing. If there are non-negligible flows, divide the boundary condition as necessary to better represent flow leaving the mesh.

Atkins Response (04/20/2023): We have applied a buffer of 1000’ to the watershed boundary so the 2D mesh extends beyond the high ground at the watershed boundary. Therefore, flow captured in the perimeter boundary condition line of the model is not the overflow from one watershed into adjacent on the model. For this watershed there is topographic relief so the overflows leaving into adjacent watersheds are negligible. For locations where floodplain extends to the watershed boundary, we have drawn profile lines to verify flow is negligible and not need to be accounted into adjacent HUC8 model. Additionally, due to application of buffer minor overflows from adjacent watersheds are accounted for in the model without needing to add overflows as inflows. Sample screenshot below.

Halff Response (7/26/2023): No further comment.

- Document and justify in BLE report why ND was not used and provide justification/appropriateness of using stage hydrographs between model domains. Per Section 2.3.6, the preferred downstream boundary condition is normal depth. Stage and flow hydrographs are primarily for coastal areas.

Atkins Response (04/20/2023):  Use of stage hydrograph as a downstream boundary is acceptable and it was used for two main reasons. First, stage hydrographs account for simulated stages in the downstream model, whereas north boundary condition may not mimic the stage-time variation in downstream model. Second, during tie-in process we found the stage hydrograph boundaries were less iterative than normal depth boundary conditions, so stage hydrographs are used in this study.

Halff Response (7/26/2023): No further comment.

- The WSEL between the domains A3-A10 do not match. Revise model to eliminate differences in WSEL greater than 0.5 feet. Review all tie-ins to ensure criteria is met.

Atkins Response (04/20/2023):  Assuming A9 and A10 tie-in since A3 and A10 are not adjacent. Models tie-in within 0.5. Tie-in table will be added to report.

Halff Response (7/26/2023): The following tie-ins do not meet criteria: A3-A4, A5-A6, A8-A9, A9-A10 and the tie in at Lake Travis Dam (A2-A4). There may be others that do not meet tie-in requirements. Review all tie-ins to ensure criteria is met. See A9-A10 100-year WSEL comparison sample below. The tie-in location for all storm events must occur at the same boundary line.

Atkins Response (08/08/2023): The tie-in between A3-A4, A5-A6, A8-A9, and A9-A10 do fall within 0.5ft. Screenshots and tables below aid in the review.

The tie-in between A3 and A4:

Table of the tie-in data between A3 and A4:

| Event | A3 WSE Elevation (ft) | A4 WSE Elevation (ft) | Difference (ft) |
| --- | --- | --- | --- |
| 1pct | 763.05 | 762.83 | 0.22 |
| 10pct | 751.97 | 751.83 | 0.14 |
| 0.2pct | 772.40 | 772.16 | 0.24 |
| 4pct | 756.19 | 755.95 | 0.24 |
| 2pct | 759.56 | 759.36 | 0.20 |
| 1pct min | 758.27 | 758.05 | 0.22 |
| 1pct plus | 767.11 | 766.88 | 0.23 |

The tie-in between A5 and A6:

Table of the tie-in data between A5 and A6:

| Event | A5 WSE Elevation (ft) | A6 WSE Elevation (ft) | Difference (ft) |
| --- | --- | --- | --- |
| 1pct | 610.69 | 610.44 | 0.25 |
| 10pct | 598.92 | 598.71 | 0.21 |
| 0.2pct | 619.55 | 619.27 | 0.28 |
| 4pct | 603.20 | 602.98 | 0.22 |
| 2pct | 606.93 | 606.69 | 0.24 |
| 1pct min | 606.61 | 605.38 | 0.23 |
| 1pct plus | 614.82 | 614.55 | 0.27 |

In the previous submittal we had inadvertently included a previous version of A10, which we have corrected. Additionally, we made a minor update to the A9 model to improve the tie-in the A8-A9 and A10 overlap area.

The tie-in data between A8 and A9:

Table of the tie-in data between A8 and A9:

| Event | A8 WSE Elevation (ft) | A9 WSE Elevation (ft) | Difference (ft) |
| --- | --- | --- | --- |
| 1pct | 434.51 | 434.44 | 0.07 |
| 0.2pct | 443.67 | 443.58 | 0.09 |
| 10pct | 422.91 | 423.13 | -0.22 |
| 4pct | 426.47 | 426.57 | -0.10 |
| 2pct | 431.88 | 431.88 | 0.00 |
| 1pct plus | 438.67 | 438.58 | 0.09 |
| 1pct min | 429.07 | 429.13 | -0.06 |

The tie-in data between A9 and A10:

Table of the tie-in data between A9 and A10:

| Event | A9 WSE Elevation (ft) | A10 WSE Elevation (ft) | Difference (ft) |
| --- | --- | --- | --- |
| 1pct | 440.03 | 440.00 | 0.03 |
| 0.2pct | 443.91 | 443.67 | 0.24 |
| 10pct | 433.98 | 434.05 | -0.07 |
| 4pct | 436.11 | 436.51 | -0.40 |
| 2pct | 438.20 | 438.34 | -0.14 |
| 1pct plus | 441.94 | 441.79 | 0.15 |
| 1pct min | 437.41 | 437.63 | -0.22 |

- The downstream end of the model at Domain A10 does not have a 2D Connector at the outfall. Please add one to aide in future HUC modeling.

Atkins Response (04/20/2023):  SA/2D connection is added for future modeling.

Halff Response (7/26/2023): There is no SA/2D connection in the models submitted 7/11/23. Please confirm the correct models were submitted for review. Outflows were identical to previously submitted models.

Atkins Response (08/08/2023): In the previous submittal we had inadvertently included a previous version of A10, which we have corrected. The SA2D connections are present and enforced in most recent version of the model. A Screenshot is shown below to aid the review.

- A8-A9-A10 interface is not well defined in BCs. A8 outfall is applied to both A9 and A10, however in A10 "0503_A8_Outfall1" is applied to a single cell as opposed to the longer distance that the flow is drawn from. Revise such that the applied flows from A8 matches between A10 and A9.

Atkins Response (04/20/2023):  We have shortened the length of BCLines compared to the SA/2D connections. We  apply the flow in the channel which can it enter overbanks depending on channel capacity. When BCLines extend entire length of SA/2D, the flow is distributed in the overbank areas, that are located outside the computed floodplain at SA/2D. We will add further discussion of the approach in the BLE report.

Halff Response (7/26/2023): Unresolved. Text Not Provided. Please provide report.

The cells in which flows are applied from A8 into A9 (for the same boundary conditions) are different than those from A8 into A10. Further, the flows applied are not the same.  Be consistent in application of upstream flows to the downstream.

|  | A8 (upstream) | A9 (middle) | A10 (Downstream) |
| --- | --- | --- | --- |
| 0503_A8_Outfall (channel) | 140077.6 | 140077.6 | 172617.9 |
| 0503_A8_Outfall1 (overbank) | 116576.1 | 116576.1 | 79055.78 |
| SUM | 256653.7 | 256653.7 | 251673.7 |

Atkins Response (08/08/2023): This comment was stemming from the review of the old version of A10. The flows are very similar the latest model.

|  | A8 (upstream) From SA/2D | A9 (middle) | A10 (Downstream) |
| --- | --- | --- | --- |
| 0503_A8_Outfall (channel) | 153,127 | 153,127 | 153,127 |
| 0503_A8_Outfall1 (overbank) | 97,822 | 97822 | 97822 |

The BCLine is for A8 outfall 1 is updated 0503_A9 model for consistency.  Screenshot below:

- Based on visual inspection, an offsite river is not applied in A2. Revise or document as necessary.

Atkins Response (04/20/2023):  We have extended the model beyond Austin Travis Lakes HUC8 boundary into Pedernales River HUC8. The inflow applied accounts represents total flow of Pedernales River, and the tributary inflow is insignificant compared to Pedernales River and Colorado river.

Halff Response (7/26/2023): Flows should be applied at the location they are “observed” in the upstream model. If the flows were collected from the model downstream of the tributary, then a 2D boundary condition downstream of the tributary should be applied.  Upstream regions may not be discounted as “insignificant” and thus removed from the model. Even if the immediate peak is not affected, downstream peaks and volumes may be. Move the boundary condition to clearly show that all contributing areas are included in the analysis (if appropriate).

Atkins Response (08/08/2023): The TWDB recently sent to Atkins an HEC-HMS model from the InFRM WHA for the Colorado River watershed. In general, this information was received too late in the modeling workflow to be included in the BLE assessment. However, we were able to pull hydrographs from this model that represented the inflows from the Pedernales River into the Colorado River watershed and insert them into the BLE modeling. Text about this HMS model and how it was applied has been placed in Subsection 1.2.4 of the report. These hydrographs include the contributions from Cypress Creek, which is the off-site river referenced in the initial ITR comment. We have also edited the mesh extent in this area so that along the Pedernales River, the mesh ends just downstream of the confluence of Cypress Creek and the river.

Keep in mind that the BLE mapping for the Austin-Travis Lakes watershed will not produce any mapping for the Pedernales River watershed since the river is outside of the Austin-Travis Lakes watershed.

- Several “general_outflow” boundary conditions have constant water surface elevations. This is likely a result of the boundary conditions being too long resulting in a poor representation of the boundary condition. Divide boundary conditions into smaller pieces that more accurately represent discharge from the model. Domains with this issue include: A1, A2, A4, A9 and A10.

Atkins Response (04/20/2023):  We have applied a buffer of 1000’ to the watershed boundary so the 2D mesh extends beyond the high ground at the watershed boundary. Therefore, flow captured in the perimeter boundary condition line of the model is not the overflow from one watershed into adjacent on the model. For this watershed there is topographic relief so the overflows leaving into adjacent watersheds are negligible. For locations where floodplain extends to the watershed boundary, we have drawn profile lines to verify flow is negligible and not need to be accounted into adjacent HUC8 model. Also, due to buffer application, minor overflows from adjacent watersheds are accounted for in the model without adding overflows as inflows. For this reason, suggestion of splitting the boundary condition lines into smaller sections will not add any value to the analysis. Flow and stage data in the perimeter boundary condition does not provide any useful information in terms of flows or stages. So, this change was not applied.

Halff Response (7/26/2023): Based on previous discussion, no further comment subject to meeting tie-in requirements.

- Further, several outflow boundary conditions have crossing or irregular water surface profiles. This can be the result of instabilities in the model. Review the model. Domains with this issue include: A1, A2, A3, A4, A6, A7 and A10. Flows between domains should match. Irregular flow or stage hydrographs are found in the following domains: A2 (General_Outflow1), A4 (General_Outflow3), A6 (General_Outflow3), and A7 (Drainage_Divide_1).

Atkins Response (04/20/2023):  Please refer to response to comment 8.

Halff Response (7/26/2023): Based on previous discussion, no further comment subject to meeting tie-in requirements.

# 5 – Hydraulic Model Execution

## Methodologies / Data Sources (Modeler to populate)

| Unsteady Plan Names |  |  |  |
| --- | --- | --- | --- |
| 10% ACE (10-year): | NRCS_10_PCT | 0.2% ACE (500-year): | NRCS_0.2_PCT |
| 4% ACE (25-year): | NRCS_4.0_PCT | 1% plus: | NRCS_1.0_PLU |
| 2% ACE (50-year): | NRCS_2.0_PCT | 1% minus: | NRCS_1.0_MIN |
| 1% ACE (100-year): | NRCS_1.0_PCT |  |  |
| Other: |  |  |  |

## Statistical Data (Modeler to populate)

| Time Step: | Advanced Time Step Based on Courant condition (7.5 - 60 Seconds) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Courant Value: | Min – 0.45 | Max – 2.0 | Simulation Length: | 4 to 7 | days |
| Computation Interval: | 30 | seconds | Model Runtime: | 5 to 12 | hours |
| Hydrograph Output Interval: | 10 | minutes | Mapping Output Interval: | 10 | minutes |

## Technical Review (Reviewer to confirm with Pass/Fail)

| Execution Review |  |
| --- | --- |
| P | Appropriate 2D parameters are used (Boundary Conditions, Time Step, Simulation Length, etc.) |
| F | Hydrograph Output Interval and Mapping Output Interval set at a reasonable interval to capture peaks |
| F | Model runtime is reasonable (< 12 hours, but dependent upon model size) |

## Review Comments

- The simulation length for A1 is not adequately long to capture the receding limb of the hydrograph. Lengthen the simulation to match that of A2 downstream.

Atkins Response (04/20/2023):  The model is run almost 2 days past peak flow and the end of simulation flows are significantly less than peak flows. We tried to reduce model run times so used 4-day simulation for this domain, as the extending simulation length won’t alter max WSE or floodplain extent.

Halff Response (7/26/2023): No further comment.

- The model run time for Domains A2 and A5 are excessively long (greater than 12 hours). Review the model for any inefficiencies.

Atkins Response (04/20/2023):  A2 models has a large size of 384 sqmi, contain urban areas, very large flows, and include Lake the Lake Travis, so the model run times are large. We couldn’t split the model in the middle of reservoir, which would be the only way to reduce run time.

For domain A5 we have tried to minimize the oscillations with different mesh configuration, however, could not resolve the oscillations in the flow hydrograph

Halff Response (7/26/2023): Review the BLE Guidelines Section 2.2.2. The guidelines state: “The TWDB prefers lower runtimes with an end user model runtime goal of under 12 hours and ideally under 6 hours. However, the TWDB recommends the runtime be established not based solely on runtime but also such that a Courant Value less than two (2) is achieved.” This further needs to be reviewed concurrently with appropriate courant values. No further comment on A2.

Atkins Response (08/08/2023): The A5 model domain is ~ 230 sq. mi. and has ~260K cells due to large number roadways and channel / urban area refinement regions. Additionally, we had ran this model on a slower computer is also a major factor on longer run time. Running it on a faster computer will reduce run times significantly. We have reviewed the max velocities and Courant numbers and they seem reasonable.

- The simulation length listed above is not true for all plans. Provide ranges as appropriate in the statistical data table above. Clarify the varied simulation lengths.

Atkins Response (04/20/2023):  Typical simulation length was used previously, simulation lengths in this HUC8 from 4 days to 7 days.

Halff Response (7/26/2023): No further comment.

- Downstream domains have longer simulation lengths than upstream domains. Because the inflow hydrographs will be shorter, the flows will be artificially extended from the time the upstream domain run time ends. This could result in inconsistent flows in the downstream domain. Clarify how hydrographs were extended in the report.

Atkins Response (04/20/2023):  To reduce the model run times we used shorter simulation times for some of the upstream domains, we ensured that model is at least a full day after the peak flow so, so shorter run not affect peak results in the downstream domains. Flow is generally look at the slope in the previous few time steps and gradually reduce flows to extend hydrograph.

No further comment. Please note if extended flows do affect peak conditions in surrounding models, extended modeling is necessary.

# 6 – Hydraulic Model Results

## Methodologies / Data Sources (Modeler to populate)

| Flow Validation/Gage Analysis |  |
| --- | --- |
| Software: | N/A |
| Methodology | Gage Analysis values from the InFRM Report with priority for gage being highest followed by FIS flows and then regression analysis, refer to the draft BLE Report and the Model Overview and Results spreadsheet. |
| Removal of Outliers and Justification | N/A |
| Validation Notes (describe parameter adjustments): Reduced the CNs and adjusted main channel overbank Manning’s n values. Please refer to Austin Travis Model Overview and Results spreadsheet. |  |
| Other: More details included in the hydraulics technical memo and model plans and overview spreadsheets. |  |

## Technical Review (Reviewer to confirm with Pass/Fail)

| Computation Checks |  |
| --- | --- |
| F | Model runs with no cell errors |
| P | Volume balance is within tolerance (ΔV < 1% is ideal, however 2% to 3% can be acceptable for rain on mesh) |
| Results Spot Checks Check at all critical structures, major confluences, a handful of headwater tributaries, downstream end, at benchmarks, and a handful of randomly selected locations. |  |
| - | No visible spikes in WSEL visibly present with a single cell |
| - | WSEL and depth grids appear reasonable (no artificial dips or spikes) |
| - | Visual verification flooding is connected downstream for all significant contributing areas (> 64 acres) |
| F | Velocity grids appear reasonable (no unusually high velocity areas or Courant values greater than 2) |
| F | No crossing profiles between all events |
| P | Inflow and outflow hydrograph timing is reasonable as compared to the peak |
| F | Spills into/from adjacent HUCs appropriately accounted for |
| F | WSEL tie-in between model domains and HUCs along major rivers/streams where inflows are applied |
| Results Full Checks |  |
| P | Computational time step meets ideal Courant condition |
| p | Diffusion Wave: V∆T∆X ≤2 (Up to 5) (Results are unit-less) |
| p | Model simulation is long enough to pass the entire hydrograph through the model (minimum of 12 hours past the peak, preferably 24 hours) |
| F | Hydrographs at internal and external boundaries are smooth |
| P | Contributing areas are contained with 2D mesh or accounted for with inflow hydrographs |
| P | No flow trapped at 2D boundary |
| - | Visual verification mapping is reasonable compared to effective |
| Flow Verification |  |
| P | Peak flow gage analysis performed on all gages in the watershed with sufficient data |
| P | All gages used have an appropriate period of record (10 to 20+ years) |
| P | All gages used have no extensive gaps in record which would impact the analysis |
| F | Gage analysis results validated to other sources (Regression, USGS, etc.) |
| F | Results at applicable benchmarks justified (gages, HWMs, effective BFEs) |
| F | Peak discharges comparable to gages, effective/prior studies, regional regression equations |

## Review Comments

- Overflow between domains is not adequately identified based on the current boundary conditions (extremely long). There is potential for missing interaction between the domains away from the main stem of the river. Please revise as necessary to appropriately capture flow transfers.

Atkins Response (04/20/2023):  Additional discussion of normal depth boundary condition approach is added to Section 1.2.4 of the BLE report. We have applied a buffer of 1000’ to the watershed boundary so the 2D mesh extends beyond the high ground at the watershed boundary. Therefore, flow captured in the perimeter boundary condition line of the model is not the overflow from one watershed into adjacent on the model. For this watershed there is topographic relief so the overflows leaving into adjacent watersheds are negligible. For locations where floodplain extends to the watershed boundary, we have drawn profile lines to verify flow is negligible and not need to be accounted into adjacent HUC8 model. Also, due to buffer application, minor overflows from adjacent watersheds are accounted for in the model without adding overflows as inflows.

Halff Response (7/26/2023): No further comment.

- There are cell errors at the downstream dam in A1 for all plans. Revise the model to eliminate the errors.

Atkins Response (04/20/2023):   The 2 errors are negligible with maximum error of 0.022’ for 1%.

Halff Response (7/26/2023): The errors are in a number of locations and range from 0.014 to 0.078 (in the 50-year event). All storm events have errors.

Atkins Response (08/08/2023): Small iteration errors such as those noted in the review comment (0.014 ft to 0.078 ft) can occur in 2D models. Errors of this magnitude are acceptable for the following reasons:

- They are only occurring at occurring at one or two cells among more than 250K cells

- They are only occurring during a few time steps in the entire simulation.

- The error values are less than 0.1 ft.

- There is disconnected ponding in roadways at the end of simulations. This can lead to volume loss in the model. Review and revise as needed.

Atkins Response (04/20/2023):   We have terrain modifications at roadway crossings by reviewing ponding at roadways. In the screenshot there appears to be a low area in the roadway so the flow is ponded. There is no stream or potential culvert crossing can be added as a terrain modification.

Halff Response (7/26/2023): No further comment.

- Velocity is high (35 ft/s) at the downstream end of the 2D mesh of Domain A2. Review model for instabilities. The velocities in A4 (downstream domain) are a max of about 9 ft/s. Revise to eliminate differences in velocities.

Atkins Response (04/20/2023):   The velocities have been lowered compared to previous version. Since this area downstream of the dam the flow velocities are expected to be high.

Halff Response (7/26/2023): The velocity downstream is spiking at greater than 100 fps immediately downstream of the dam.

Atkins Response (08/08/2023): The A2 model has been revised and the model is clipped at the dam so, this area is no longer in the model.

- Several locations throughout the domains have irregular hydrographs

Atkins Response (04/20/2023):   See response to comment 8 on Page 13.

Halff Response (7/26/2023): Based on previous discussion, no further comment subject to meeting tie-in requirements.

- For reviewer clarity, label USGS gages with their gage number in the pertinent tables similar to how USGS 08154700 was labeled.

Atkins Response (04/20/2023):  Gages have been labelled in the model overview and results spreadsheet. Will able the gages in the report in the BLE report.

Halff Response (7/26/2023): Unresolved. Text Not Provided. Please provide report. Further, the gages in “0503_A4 Plans (direct Flow)” have not been labeled with their gage number (see photo above as the screenshots would be identical). “0503_A9 Plans (Pluvial)” has the same issue.

Atkins Response (08/08/23): Validation summary table in Subsection 1.2.10 includes gage numbers. Summary Excel spreadsheet has been updated, too.

- Report “Model Validation” Section does not agree with “Buchanan_model_overview_and_Results” excel spreadsheet. Revise the documents as necessary to match each other and modeling.

Atkins Response (04/20/2023):  Will verify and update the report, might have been a roll over comment from Buchanan-LBJ lakes watershed.

Halff Response (7/26/2023): No further comment.

- Document each location and justify differences where the modeled results do not fall within the confidence interval for the available FIS.

Atkins Response (04/20/2023):  Will add details to the BLE report.

Halff Response (7/26/2023): Unresolved. Text Not Provided. Please provide report

Atkins Response (08/08/2023): The amount of research required to fulfill the reviewer’s request is extensive and beyond the scope of this task order. There are 106 locations in this watershed where effective flow data were available. 1% BLE outcomes are within 25% of the effective flow data at 71 locations and more than 25% different at 35 locations. There is too much effective flow data available in this watershed to understand why the BLE and effective discharges differ at this many locations. The BLE report already includes language explaining that the BLE outcomes may vary from other sources due to factors such as differing rainfall data, terrain data, and study methodologies. No changes made.

- Update sections of the report per comments by Shrinivas Kaulgud in submitted document.

Atkins Response (04/20/2023):  Will add details to the BLE report.

Halff Response (7/26/2023): Unresolved. Text Not Provided. Please provide report.

Atkins Response (08/08/2023): The report has been updated. Note that CNMS results will be updated once the modeling and mapping have been approved.

- When there is no FEMA study, FIS study, or gages to compare results to, use Fathom data to validate floodplain extent.

Atkins Response (04/20/2023):  Comment noted. For this particular watershed NFHL data is available.

Halff Response (7/26/2023): Unresolved. Text Not Provided. Please provide report

Atkins Response (08/08/2023): Report to be updated when mapping is available.

- Items in red highlight within the checklist above were not reviewed in detail. Please ensure these modeling assumptions/parameters have been reviewed and are appropriate. Also, review comments provided in previously completed detail review(s) and address accordingly as appropriate and applicable for this watershed

# 7 – Mapping Results

## Technical Review (Reviewer to confirm with Pass/Fail)

| Hydraulic Model |  |
| --- | --- |
|  | HEC-RAS model with results, pertinent data and required storm events |
| Flood Risk Database |  |
| Pass | All features fully populated per current FEMA guidelines |
|  | Mapping consistent between grids to polygons for all three required mapped events |
|  | 10yr smaller than 100yr smaller than 500yr |
|  | Holes filled in polygon, depth and WSE grids |
|  | Boundary interfaces align matching or identified as AOMI |

## Review Comments

- Mapping/Grid Comments –

- Grids – Holes and issues with tie-ins throughout HUC see representative screenshots below and reference QC polygons. Entire HUC should be reviewed and corrected.

Small holes in WSE and depth Grids 1PCT/0.2PCT:

There are tie-ins exceeding the 0.5ft FEMA tolerance for the WSE and Depth GRIDs for both 1% and 0.5%. please review comments below and review QC polygons for representative areas. Required full review of entire HUC 8.

Please ensure mapping tie ins are met to all adjacent study HUCs (Buchanan – LBJ Lakes for this study). AOMIs may be used for mapping tie ins to adjacent HUCs studied by others or completed with methods different from the current study (ex. 1D BLE v. 2D BLE).\

- Atkins Response 10/13/2023: Watershed was reviewed and small holes were filled as needed. Tie-ins were reviewed throughout the entire watershed and corrected as needed. When reviewing the tie-ins for Austin-Travis Lakes, please use our most recent Buchanan-LBJ Lakes M2 submittal to verify that the tie-ins along the Austin-Travis Lakes/Buchanan-LBJ Lakes watershed boundary are correct.

Backcheck Comments 11/1- Major issue in Grids causing waterfall effect and large variation in elevations at Model 0501_A1. Area causing issues with BFEs as well. From aerial looks like this could be a quarry if so possible AOMI recommended to account for difficulty in mapping/elevations. Please review screenshots below as well as review QC Polygons.

Atkins Response 11/27/2023:  BFE clustering issues were occurring because of a quarry.  AOMI_Ar were added to explain these anomalies.

WSE01PCT:

WSE0_2PCT

Backcheck Comments 11/1- Some small areas of the WSE0_2PCT grids are not within the FLD_HAZ_Ar, please remove or add to the FLD_HAZ_Ar polygons. Review screenshots below, QC polygons and entire HUC.

Atkins Response 11/27/2023:  Grids and polygons updated as needed to address this issue.

Backcheck Comments 11/29/2023-Addressed, no further comments.

Backcheck Comments 11/29/2023-Updated comment for issue at tie-ins at 0504_A6 and 0504_A7 please review screenshot below and QC polygon. Tie-In greater than the 0.5ft threshold in the BLE_WSE0_2PCT.

Please review GRIDs along this model boundary and double check others throughout HUC.

Atkins Response (11/30/2023): The tie-in the screen shot has been fixed. Model boundaries were reviewed to confirm no other tie-ins needed to be revised.

Backcheck Comments 12/11/2023- Issues still at same Domains 0504_A6 and 0504_A7 thresholds exceeding 0.5ft in the WSE0_2PCT grid and DEP0_2PCT

Please review and make corrections to grids along this domain boundary and all others to assure no more issues with tie-ins along boundaries.

Issue along 0504_A7 and 0504_A10.

Issue along model boundary 0503_A3 not meeting 0.5ft threshold in the WSE0_2PCT.

Issue along model boundary 0502_A2 not meeting 0.5ft threshold in the WSE01PCT.

Tie in issues addressed for all events across watershed.

AtkinsRéalis Response (03/12/2024) – Tie-in approach has been revised for this submittal. Split line approach has been applied and tie-ins are now clean. See For_ITR_Review folder for split lines, model domain boundaries, HEC-RAS WSE/depth rasters, and difference rasters to facilitate review. The split lines show where tie-ins are located throughout the watershed. Tie-ins with Buchanan (neighboring HUC-8 watershed) will be able to be checked by the ITR team when we submit revised Buchanan mapping.

Halff Backcheck (03/21/2024) – Reviewed new tie-ins no further comments issues have been addressed.

# 8 – Completeness Review

## Technical Review (Reviewer to confirm with Pass/Fail)

| File Structure and Naming |  |
| --- | --- |
|  | Deliverables provided in TWDB/FEMA required file folder structure and naming convention |
|  | Data missing (if any) documented in report and ReadMe file within the required folder |
| Report |  |
|  | Technical 2D BLE report documents all assumptions, special considerations, etc. |
|  | Flow validation and difference discussed in report |
|  | Flow comparisons discussed in report |
| Hydrologic Model |  |
|  | HEC-HMS model with results, pertinent data and required storm events |
| Hydraulic Model |  |
|  | HEC-RAS model with results, pertinent data and required storm events |
| Flood Risk Database |  |
|  | All features fully populated per current FEMA guidelines |
| CNMS |  |
|  | Updated correctly with new centerlines |
|  | A1-A5 checks complete |
|  | CNMS Post- Processing Complete |
| HAZUS |  |
|  | Run per FEMA guidance |

- FRD Review –

  - S_Pol_Ar- No Comments

  - S_HUC_Ar-

- SOURCE_CIT-References BASE1 looks like from the L_Source_Cit table it should be BASE2

Backcheck Comments 11/1-

Corrected, no further comments.

  - S_AOMI_Pts- This dataset should no longer be in the database. All features should be removed and buffered then added to the S_AOMI_Ar feature class.

Atkins Response 10/13/2023: Atkins has previously received approval from TWDB/ITR team to use S_AOMI_PT and S_AOMI_AR in this task order. For all other task orders, we will use only S_AOMI_AR.

Backcheck Comments 11/1-

- Source_CIT in table references STUDY3 for At Risk Essential Facilities, this source is not in the L_SOURCE_Cit table please update to match referenced sources in table.

Atkins Response 11/27/2023:  Source_Cit table has been updated. STUDY3 has been removed.

Backcheck Comments 11/29/2023- STUDY3 has been removed from the L_Source_Cit table but it is still within the S_AOMI_Pts. Please remove from S_AOMI_Pts and update with correct source for AOMI_TYP

Atkins Response (11/30/2023): STUDY3 has been removed from AOMI_PTS and AOMI_TYP has been updated.

Backcheck Comments 12/11/2023- No Further Comments

S_AOMI_Pts are currently outside of HUC 8 boundary please clip down to HUC8.

Atkins Response 11/27/2023:  Removed AOMI_Pts outside of HUC boundary.

Backcheck Comments 11/29/2023-Addressed, no further comments.

  - S_AOMI_Ar-

    - HUC8_Code-Currently several different HUCs listed (related to tie-ins of adjacent HUCs) however these should all be the same and should represent the Watershed being studied.

Backcheck Comments 11/1-

HUC8 Codes still differ, should be the same for watershed being studied not adjacent HUCs at tie-ins

Atkins Response 11/27/2023: Updated HUC8 Code.

Backcheck Comments 11/29/2023-Addressed, no further comments.

    - Add in buffered S_AOMI_Pts to include stream flow constrictions, structures and high water marks, low water crossings and at risk facilities.

Atkins Response 10/13/2023: Atkins has previously received approval from TWDB/ITR team to use S_AOMI_PT and S_AOMI_AR in this task order. For all other task orders, we will use only S_AOMI_AR.

Backcheck Comments 11/1- No further comments

  - WTR_LN-

- Geometry Comments- Several streams missing reminder 1 SQM Rural / 0.5 SQM Urban, refer to screenshots below and QC polygons for representative examples. Entire HUC should be reviewed.

Atkins Response 10/13/2023: Added 0.5-SQM water lines features in urban areas.

Backcheck Comments 11/1- No further comments

- Centerline not contained within 100 year floodplain, review screenshot below and QC polygon for representative areas, full review of HUC 8 required

Atkins Response 10/13/2023: Centerlines have been updated and reside within the  100yr floodplain.

Backcheck Comments 11/1- Centerline containment, most streams contained except area below WTR_LNs still outside of 1PCT FLD_HAZ_AR, please review screenshots and representative QC polygon and update. Full review of HUC required.

Atkins Response 11/27/2023: Stream centerlines have been updated so they are contained inside the 1% floodplain.

Backcheck Comments 11/29/2023-Addressed, no further comments.

- Attribute Table Comments

  - SOURCE_CIT- References BASE2, should be STUDY1 for the BLE study for the Watershed

Atkins Response 10/13/2023: No changes made. BASE2 is the source cite for a HUC8 Boundary from USGS.

Backcheck Comments 11/1- SOURCE_CIT references BASE1, in L_SOURCE_Cit table this represents TxDOT City Boundaries please update to correct source used for streams.

Backcheck Comments 11/29/2023-Addressed, no further comments.

Atkins Response 11/27/2023: Updated source cite for Wtr_Ln .

Backcheck Comments 11/29/2023-Addressed, no further comments.

  - TENPCT_FP-

    - Geometry Comments- Several areas of ten year outside of 100 year floodplain seen screenshots below and QC polygons for representative areas but review of entire HUC needed.

Atkins Response 10/13/2023: 10yr mapping has been revised. It is now contained within the 100yr floodplain.

Backcheck Comments 11/1- Areas below have been correctly updated, there are still some remaining small areas of TENPCT_FP outside of FLD_HAZ_AR 1PCT. Please review screenshot below and QC polygons, review of entire HUC necessary.

Atkins Response 11/27/2023: 10% polygon has been dissolved into the 1% polygon. 10% polygon is inside the 1% polygon.

Backcheck Comments 11/29/2023-Addressed, no further comments.

    - Attribute Table Comments- No Comments

  - FLD_HAZ_AR-

- Geometry Comments- Several areas of 100 year outside of 500 year see screenshot examples below and review QC polygons for representative examples, full review of HUC 8 needed.

Atkins Response 10/13/2023: 100yr mapping has been revised. It is now contained in the 500yr floodplain. Added Zone X where 500yr was missing.

Backcheck Comments 11/1- No further comments

Queried Zone X-Zone A available in area where Zone X is missing

- Attribute Table Comments- No Comments

Backcheck Comments 11/1- No further comments

  - BFE_2D-

    - Geometry Comments- Major issues with BFE lines cupping and clustering together this is causing issues between the raster and the elevations represented in the BFE line. See screenshots below and QC polygons for representative examples.

Atkins Response 10/13/2023: BFE linework has been corrected.

Backcheck Comments 11/1- See comments and screenshots below

There are over 1500 BFE lines clustered in this one small area with differing elevations from the 1PCT grid due to this clustering.

Major differences between the 1PCT grid and the BFE lines in areas of clustering:

More clustering/inconsistent elevations throughout HUC 8 review QC polygons for representative areas but full review needed:

Backcheck Comments 11/1- Most issues with BFEs addressed however there are some more areas of clustering causing differences in BFE’s of over 16 feet from the 1PCT WSE. See screenshots below and representative polygons. Review HUC8.

Atkins Response 11/27/2023: The clustered BFEs are occurring because there is a quarry. These areas have been identified and added to AOMI_Ar.

Backcheck Comments 11/29/2023- Addressed issues with S_AOMI_Ar polygons, no further comments.

Please also review BFE_2D attribute table there are several fields with NULLs that should be populated with appropriate domains, etc.

Backcheck Comments 11/29/2023- There are still some issues with the BFE_2D attribute table. Some missing data from fields and not all domains applied please see screen shot below:

LEN_UNIT field currently doesn’t have all fields populated some are empty string.

Please populate all fields with the appropriate domain (FT) like other attributes that have been populated.

Comment also applies to the V_DATUM field please update.

Comment also applies to SOURCE_CIT currently some fields are empty string please update to all include STUDY1.

Atkins Response (11/30/2023): The BFE attributes have been updated as requested.

Backcheck Comments 12/11/2023- No Further Comments

  - WTR_AR-

    - Geometry Comments- Missing waterbodies near dams these should be added into the WTR_AR feature class see representative screenshot and QC polygons review full HUC for any missing waterbodies near dams.

Atkins Response 10/13/2023: Water bodies over 5 acres have been added into FLD_HAZ_AR and included in the WSE/Depth grids.

Backcheck Comments 11/1- WTR_Ars still missing from original QC polygons these are near dams and should be added if capacity is greater than 200 ac-ft. Review and update feature as needed.

Atkins Response 11/27/2023: Adding dams with a storage area of 200 ac-ft is not documented in the guidance. After seeking clarification from Jenna Rao (TWDB) and Erika Nordstrom (Halff), the WTR_AR feature class has been populated as follows:

- Included all NHD waterbodies features that are 20 acres or larger or are named.

- Included all waterbodies larger than 20 acres captured in the LiDAR breaklines.

- Downloaded all dams from the NID with a surface area larger than 20 acres. If the ponded area shown on the aerial photography (i.e., base map aerials available in ArcPro), was 20 acres or larger, it was added to WTR_AR.

    - Attribute Table Comments- No Comments

  - DTL_STUD_AR-

    - Geometry Comments- Some DTL_STUD_AR polygons cover NFHL but do not fully encompass newly mapped BLE. Review screenshots and representative QC polygons as well as entire HUC where DTL_STUD_AR applies.

Atkins Response 10/13/2023: DTL_STUD_AR has been updated.

Backcheck Comments 11/1- No further comments

Tie-In at LBJ should be widened to appropriately join to NFHL:

    - Attribute Table Comments-

- SOURCE_CIT- References FIRM1, check these are all from the same study there are multiple counties within the HUC 8.

Atkins Response 10/13/2023: Source cite for effective studies was extracted from the State-wide NFHL database(NFHL_48_20230902).

Backcheck Comments 11/1- No further comments

  - DTL_STUD_LN-

    - Geometry Comments- Lines don’t align with BLE WTR_LNs, review screenshots below and QC polygons for representative examples full review of HUC needed.

DTL_STUD_LNs do not follow WTR_LNs, these should be generated with new BLE WTR_LNs and intersected with available effective mapping.

Looks like the DTL_STUD_LNs were generated from S_WTR_LNs from Effective Data (Below Travis County FIRM layers) these should be generated from current BLE study for Austin Travis Lakes newly generated WTR_LNs.

    - Attribute Table Comments-

      - SOURCE_CIT- References FIRM update to match L_SOURCE_CIT table, also check these are all from the same study there are multiple counties within the HUC 8.

Atkins Response 10/13/2023: Source cite for effective studies was extracted from State-wide NFHL database (NFHL_48_20230902).  Based on TWDB Guidelines document, DTL_STUD_LN should be derived from existing detailed studies contained within the NFHL layer and DFIRM datasets.

Backcheck Comments 11/1- No further comments

  - CNMS Comments- S_Studies_Ln and S_Unmapped_Ln currently empty, coordinate with Halff and TWDB to submit at a later date.

Atkins Response 10/13/2023: Noted.

  - S_FRAC_Ar- No Comments

  - L_Souce_Cit-

- SOURCE_CIT- References FIRM1, verify no other FIRMs used related to this watershed there are multiple counties within the HUC 8.

Atkins Response 10/13/2023: This has been verified.

Backcheck Comments 11/1- No further comments

Halff Backcheck (03/21/2024) – Please review that all FRD fields are properly populated some feature classes that were populated in previous submittal and are required to be are now empty. Please review and populate appropriately.

- S_AOMI_Ar

- DTL_STUD_AR

AtkinsRéalis Response (06/10/2024) – Attribution data have been added for S_AOMI_Ar and DTL_STUD_AR in the geodatabase.

Following additional discussion with the TWDB and Halff that clarified the expectations for the DTL-STUD-AR polygon, the DTL-STUD-AR polygon has been reviewed and extensively modified to better capture the BLE mapping that should be masked on the eBFE viewer.

Halff Backcheck 06/21/2024 – All feature classes have been updated and populated correctly, please see screenshots below and QC polygon for any remaining comments.

Halff Backcheck 06/21/2024 – One outstanding comment regarding DTL_STUD_AR, all look good just a couple extra polygons that need to be removed. Please see screenshot below and QC polygon:

Two DTL_STUD_AR polygons are circled below, these should be removed there is no effective mapping from Burnet County detailed study.

Atkins Response  06/25/2024 - DTL_STUD_AR polygons that inadvertently covered detailed effective (500 year) from adjacent HUC have been removed from DTL_STUD_AR layer.

Halff Backcheck 06/27/2024 – DTL_STUD_AR has been updated and corrected, no further comments.

QA/QC Checklist Approval

This 2D Base Level Engineering Analysis QA/QC Review is in compliance with the contract requirements and all task checkpoints are complete.  The independent QC Team has reviewed the analysis, presented review comments to the Production Team Task Leader, and discussed any problems or issues.  The Task Leader, QC Manager, QC Reviewer, and Project Manager have signed the QA/QC Checklist to confirm that all comments have been received, addressed, and documented appropriately.

| QA/QC Team – [TWDB or company name] |  |  |  | Production Team – AtkinsRéalis |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Seth Ahrens |  | 12/16/2024 |
| QC Manager |  | Date |  | Task Lead |  | Date |
|  |  |  |  | Seth Ahrens |  | 12/16/2024 |
| QC Reviewer |  | Date |  | Project Manager |  | Date |