2-D Base Level Engineering

Texas Water Development Board

Quality Control — Mapping

HUC 8 Reviewed:	Upper San Antonio

Submitted by: 		Aaron Raper

QC by:			Guillermo Simon

- Comparison with effective mapping

Compare water surface elevation and mapping extents, if possible. Of note, mapping extents may prove difficult to verify because BLE models rain-on-grid and effective maps show flow concentration.

- Mapping around HUC perimeter

Verify mapping near downstream boundary condition

- Tie-in with neighboring basins

Verify mapping continuity around the HUC’s perimeter

- Stream connectivity

Verify continuous mapping along conveying systems (creeks, channels, roads (?))

- Mapped depths

Depth use in mapping. Is minimum depth consistent with other HUCS?

- Mapping consistent with frequency.

Verify that mapping extents and flood elevations increase with lower frequencies (e.g., is 500-yr mapping “larger” than 100-yr mapping?)

- Miscellaneous comments

| QC Round | Date | Comment |
| --- | --- | --- |
| QC 1 sent |  |  |
| QC 1 complete | 9/13/2021 |  |
| QC 2 sent |  |  |
| QC 2 complete |  |  |

| QC Item | Comments and figures |
| --- | --- |
| 1. Comparison with effective mapping | 1.1 Upper San Antonio HUC includes numerous areas labeled as Zone AE. BLE is comparable-to-wider than effective mapping. Mapping differences highlight topography differences between BLE and effective studies. 1.2 BLE adds a sizable number of flood hazard areas, likely due to modeling of rain-on-grid boundary conditions. 1.3 The effective mapping of dams shows purposeful delineation near the structure and spillway. However, BLE mapping shows some discontinuities. I recommend matching the effective mapping in these areas. See examples in the figures below. |
| 2. Mapping near downstream boundary condition | 2.1 Mapping near the downstream appears correct. However, please review item 3.1. |
| 3. Mapping of interbasin flow transfer (tie-ins) | 3.1 Verify mapping for the San Antonio River near the downstream end. Upper SA HUC BLE mapping shows overflow into Lower SA. 3.2 Verify tie-ins around the HUC perimeter, as necessary, particularly with Cibolo and Lower SA. |
| 4. Stream continuity | 4.1 See item 1.3 (mapping near dams). 4.2 BLE mapping continuity looks good. I did not identify consistent discontinuity issues. |
| 5. Mapped depths | 5.1 Mapping adds significant flooding areas, where the 100-depth grid does not show any flooding at all. Figure below shows BLE mapping as hatched area. The figure below, for example, should only delineate flooding within the streets. It is unclear why we mapped the parcels (hatched areas show BLE mapping). This issue applies to “street” areas and streams features. |
| 6. Mapping consistent with frequency | 6.1 To verify frequency mapping consistency, we need to address item 5.1 first. |
| 7. Miscellaneous | 7.1 |