2-D Base Level Engineering

Texas Water Development Board

Quality Control — Mapping

HUC 8 Reviewed:	Lower San Antonio Bay

Submitted by: 		Gennadii Prykhodko

QC by:			Guillermo Simon

- Comparison with effective mapping

Compare water surface elevation and mapping extents, if possible. Of note, mapping extents may prove difficult to verify because BLE models rain-on-grid and effective maps show flow concentration.

- Mapping around HUC perimeter

Verify mapping near downstream boundary condition

- Tie-in with neighboring basins

Verify mapping continuity around the HUC’s perimeter

- Stream connectivity

Verify continuous mapping along conveying systems (creeks, channels, roads (?))

- Mapped depths

Depth use in mapping. Is minimum depth consistent with other HUCS?

- Mapping consistent with frequency.

Verify that mapping extents and flood elevations increase with lower frequencies (e.g., is 500-yr mapping “larger” than 100-yr mapping?)

- Miscellaneous comments

| QC Round | Date | Comment |
| --- | --- | --- |
| QC 1 sent |  |  |
| QC 1 complete | 9/2/2021 |  |
| QC 2 sent |  |  |
| QC 2 complete |  |  |

| QC Item | Comments and figures |
| --- | --- |
| 1. Comparison with effective mapping | 1.1 BLE mapping matches effective boundaries well (within BLE modeling limitations). Some exceptions include disconnected mapping or significantly larger or narrower areas. If BLE mapping will overwrite Zone As, we should verify connectivity, particularly in populated areas (see figure below). 1.2 Effective data provides more mapping than other HUCs which helps verify flowpaths and overall BLE mapping consistency. BLE mapping appears reasonable. Differences with respect to effective data and additional flooding areas are due to the rain-on-grid boundary condition and the model domain. 1.3 The NFHL layer provided for QC did not include Victoria County. Verify reasonableness, if possible. |
| 2. Mapping near downstream boundary condition | Because we split LSA HUC into two models, the review looks at two downstream boundary conditions. 2.1 Mapping between upper and lower LSA reaches shows some differences. The San Antonio River lower reach is wider than the upper reach. Ensure proper tie-in. If modeling results are consistent, I suggest tapering the mapping of two areas, that extends for a “significant” distance. See figures below. 2.2 Mapping at the downstream most end will tie-in with coastal flood hazards along the San Antonio River and Guadalupe River’s main channel, as necessary. Mapping of the peninsula west of Hynes Bay and San Antonio Bay (including Aransas National Wildlife Refuge and town of Austwell) looks correct. |
| 3. Mapping of interbasin flow transfer (tie-ins) | 3.1 Numerous flooding polygons cross the HUC boundary. Ensure mapping ties-in with surrounding HUCs, including five HUCS developed under this task order. 3.2 The NFHL layer provided for QC did not include Victoria County. Mapping extends well beyond the HUC boundary near the downstream end. This mapping may be all coastal flooding. However, I could not verify this because the NFHL layer provided for QC did not include Victoria County. Verify effective or preliminary coastal extents (see below). |
| 4. Stream continuity | 4.1 Continuity at dams. Series of dams below. 4.2 Ensure continuity of floodplains across US Highway 59 wherever effective mapping exists. This is the location where we broke up mapping of Lower San Antonio South. In general, mapping looks continuous, but take a closer look along the highway. |
| 5. Mapped depths | 5.1 Areas with topographic relief agree with BLE mapping well. Mapping includes areas shallower than 0.25 feet. Verify if this assumption meets the BLE mapping standard. 5.2 Some flat areas show inconsistent mapping with respect to grid depths (see figures below). Revise mapping depending on BLE standard consistent with other TWDB HUCs, if necessary. 5.3 The LSA south-south reach does not show elevations below 0.33 feet but the LSA south-north does not. Verify that we mapped the entire HUC consistently and following the standard for other TWDB HUCs. |
| 6. Mapping consistent with frequency | 6.1 Frequency mapping appears consistent. |
| 7. Miscellaneous | 7.1 N/A |