2-D Base Level Engineering

Texas Water Development Board

Quality Control — Mapping

HUC 8 Reviewed:	Cibolo

Submitted by: 		Gennadii Prykhodko

QC by:			Guillermo Simon

- Comparison with effective mapping

Compare water surface elevation and mapping extents, if possible. Of note, mapping extents may prove difficult to verify because BLE models rain-on-grid and effective maps show flow concentration.

- Mapping around HUC perimeter

Verify mapping near downstream boundary condition

- Tie-in with neighboring basins

Verify mapping continuity around the HUC’s perimeter

- Stream connectivity

Verify continuous mapping along conveying systems (creeks, channels, roads (?))

- Mapped depths

Depth use in mapping. Is minimum depth consistent with other HUCS?

- Mapping consistent with frequency.

Verify that mapping extents and flood elevations increase with lower frequencies (e.g., is 500-yr mapping “larger” than 100-yr mapping?)

- Miscellaneous comments

| QC Round | Date | Comment |
| --- | --- | --- |
| QC 1 sent |  |  |
| QC 1 complete | 9/1/2021 |  |
| QC 2 sent |  |  |
| QC 2 complete |  |  |

| QC Item | Comments and figures |
| --- | --- |
| 1. Comparison with effective mapping | 1.1 BLE mapping shows good agreement with effective boundaries. In many areas, BLE even appears to provide an improved delineation. BLE also delineates flood hazards along many (all?) tributaries. BLE provides good tie-ins with reaches mapped with detailed study methods. |
| 2. Mapping near downstream boundary condition | 2.1 Mapping at the downstream end (confluence with San Antonio River looks correct. However, as described in item 3.2, verify that mapping ties in with Lower San Antonio HUC. |
| 3. Mapping of interbasin flow transfer (tie-ins) | 3.1 BLE results do not show areas with significant interbasin flow transfers (except for item 3.2). Some tie-ins may have to occur at the upstream end of some tributaries. 3.2 The downstream end shows flows leaving the Cibolo HUC laterally, close to the confluence with San Antonio River. Verify that mapping extents with Lower San Antonio tie in. |
| 4. Stream continuity | 4.1 See discontinuities within spillways. 4.2 Some tributaries show as elongated “ponded” areas. These discontinuities are a result of steep terrain and narrow channels which may or may not have been captured by breaklines, but the current mapping looks odd. These features appear throughout the HUC. I recommend connecting to downstream reach, if possible. See figures below. |
| 5. Mapped depths | 5.1 Depth grid extents do not match the 1% delineation. Depths appear too shallow in the area shown below. Verify that mapping in this area meets standard. 5.2 In general, the 1% mapping does not follow the raw depth grid consistently. I cannot tell what the mapping standard is for depth, smoothing and general cleanup of small features. Many areas mapped depths <0.25 ft but others removed them. See the figures below and revise, if necessary (cyan color represents depths less than 0.25 ft—see scale). The following figure appears to have missed several deep flooding areas. |
| 6. Mapping consistent with frequency | 6.1 Mapping of the three flood hazards is inconsistent with their frequency (i.e., 0.2% > 1% > 10%) in many areas (see figures below). In most of the marked-up areas, the 10% is the widest mapped flood. There are also some areas where the 1% is wider than the 0.2% (see coordinates in figure). The area below was already noted. |
| 7. Miscellaneous | 7.1 N/A |