2-D Base Level Engineering

Texas Water Development Board

Quality Control — Mapping

HUC 8 Reviewed:	East San Antonio Bay

Submitted by: 		Bryden Coghlan

QC by:			Guillermo Simon

- Comparison with effective mapping

Compare water surface elevation and mapping extents, if possible. Of note, mapping extents may prove difficult to verify because BLE models rain-on-grid and effective maps show flow concentration.

- Mapping around HUC perimeter

Verify mapping near downstream boundary condition

- Tie-in with neighboring basins

Verify mapping continuity around the HUC’s perimeter

- Stream connectivity

Verify continuous mapping along conveying systems (creeks, channels, roads (?))

- Mapped depths

Depth use in mapping. Is minimum depth consistent with other HUCS?

- Mapping consistent with frequency.

Verify that mapping extents and flood elevations increase with lower frequencies (e.g., is 500-yr mapping “larger” than 100-yr mapping?)

- Miscellaneous comments

| QC Round | Date | Comment |
| --- | --- | --- |
| QC 1 sent |  |  |
| QC 1 complete | 7/22/2021 |  |
| QC 2 sent |  |  |
| QC 2 complete |  |  |

| QC Item | Comments and figures |
| --- | --- |
| 1. Comparison with effective mapping | 1.1 The only Zone AE I could find in this HUC-8 corresponds to the Guadalupe River. However, this Zone AE is a coastal flood risk area. The mxd may be missing effective data for other counties encompassed by this HUC. The area east of the industrial canal shows similar floodplain delineation extents, thanks to high banks. BLE extents appear higher in other areas. It is unclear if this difference is the result of using rain-on-grid boundary conditions. |
| 2. Mapping near downstream boundary condition | 2.1 I found no noticeable issues near the boundary condition with the bay. However, mapping appears “too coarse” to identify any potential issues. |
| 3. Mapping of interbasin flow transfer (tie-ins) | 3.1 Not reviewed at the moment. |
| 4. Stream continuity | 4.1 The mapping appears to lack refinement. The BLE mapping shows large areas of flooding, with little delineation (i.e., “everything floods”). The mapping does not look reasonable in some areas (see screenshot below). |
| 5. Mapped depths | 5.1 The 1% mapping includes a significant area, almost disregarding depths. The mapping shows many areas with depths less than 0.4 feet that should be removed from the flood hazard area, even for a BLE level of effort (see areas in blue and white below; hatched area indicates BLE mapping). Verify that we are using the same mapping threshold in all HUCS. |
| 6. Mapping consistent with frequency | 6.1 The 10% flooding extents are consistently smaller than the 1%. However, The 10% also appears to be very “coarsely” delineated. Review the standard for mapping shallow depths. The 0.2% flood hazard boundary shapefile is empty. |
| 7. Miscellaneous | 7.1 Many large areas with no flood depth or shallow depths (< 0.35 ft) are shown as flooded. We need to revise mapping for this HUC. |