2-D Base Level Engineering

Texas Water Development Board

Quality Control — Mapping

HUC 8 Reviewed:	Aransas Bay

Submitted by: 		Aaron Raper

QC by:			Guillermo Simon

- Comparison with effective mapping

Compare water surface elevation and mapping extents, if possible. Of note, mapping extents may prove difficult to verify because BLE models rain-on-grid and effective maps show flow concentration.

- Mapping around HUC perimeter

Verify mapping near downstream boundary condition

- Tie-in with neighboring basins

Verify mapping continuity around the HUC’s perimeter

- Stream connectivity

Verify continuous mapping along conveying systems (creeks, channels, roads (?))

- Mapped depths

Depth use in mapping. Is minimum depth consistent with other HUCS?

- Mapping consistent with frequency.

Verify that mapping extents and flood elevations increase with lower frequencies (e.g., is 500-yr mapping “larger” than 100-yr mapping?)

- Miscellaneous comments

| QC Round | Date | Comment |
| --- | --- | --- |
| QC 1 sent |  |  |
| QC 1 complete | 7/21/2021 |  |
| QC 2 sent |  |  |
| QC 2 complete |  |  |

| QC Item | Comments and figures |
| --- | --- |
| 1. Comparison with effective mapping | 1.1 BLE mapping of populated areas from Aransas Pass to Rockport displays significantly more areas of flood risk than effective maps. These differences stem from the use of rain-on-grid boundary conditions and lack of effective detailed studies. Some areas with coastal flood hazards expand which indicates that BLE identifies a higher risk. We can expect this “widening” to increase as we move away from the shoreline, but it may be something we need to consider in the display of data in the BLE viewer (see figure). 1.2 Mapping near a canal running mostly parallel to N. Main St near Ingleside compares consistently wider than the effective, but not unreasonably so. 1.3 Zone As for Copano Creek and Alameda Creek are narrower than BLE estimates. In some cases, the difference is significant. I do not know what the difference in elevation is. The same difference is visible in other Zone As. Of note, a review of flood depths indicates that most of the BLE maps depths less than 1 foot. Also, several Zone As do not follow topographic features well. |
| 2. Mapping near downstream boundary condition | 2.1 Mapping appears correct, but some areas do not connect to the bay. This disconnection appears to take place along steep shorelines, where coastal flood hazards are narrow. |
| 3. Mapping of interbasin flow transfer (tie-ins) | 3.1 These screenshots also apply to North Corpus Christi HUC-8 — BLE mapping along NCC and Aransas Bay shows significant differences. For a long section of the boundary, we may use a road as basin and mapping divide. Some areas simply require deletion of the neighboring HUC’s mapping. Other areas require more careful tie-ins. See screenshots below. |
| 4. Stream continuity | 4.1 Connectivity of 1% floodplains appears correct throughout the HUC-8, in part thanks to the wide floodplains. Some isolated “ponds” appear throughout the basin. Review if these ponds conform to the mapping standard. The 10% floodplain shows more discontinuities or ponding features (in my opinion, these depressions appear justified, given BLE level of detail). |
| 5. Mapped depths | 5.1 Mapping follows depths of ~0.4 feet. |
| 6. Mapping consistent with frequency | 6.1 Mapping of the three flood hazards is consistent with their frequency, i.e., 0.2% > 1% > 10%. |
| 7. Miscellaneous | 7.1 N/A |