2-D Base Level Engineering

Texas Water Development Board

Quality Control — Mapping

HUC 8 Reviewed:	Mission

Submitted by: 		Trish Burros

QC by:			Guillermo Simon

- Comparison with effective mapping

Compare water surface elevation and mapping extents, if possible. Of note, mapping extents may prove difficult to verify because BLE models rain-on-grid and effective maps show flow concentration.

- Mapping around HUC perimeter

Verify mapping near downstream boundary condition

- Tie-in with neighboring basins

Verify mapping continuity around the HUC’s perimeter

- Stream connectivity

Verify continuous mapping along conveying systems (creeks, channels, roads (?))

- Mapped depths

Depth use in mapping. Is minimum depth consistent with other HUCS?

- Mapping consistent with frequency.

Verify that mapping extents and flood elevations increase with lower frequencies (e.g., is 500-yr mapping “larger” than 100-yr mapping?)

- Miscellaneous comments

| QC Round | Date | Comment |
| --- | --- | --- |
| QC 1 sent |  |  |
| QC 1 complete | 9/9/2021 |  |
| QC 2 sent |  |  |
| QC 2 complete |  |  |

| QC Item | Comments and figures |
| --- | --- |
| 1. Comparison with effective mapping | 1.1 BLE mapping shows comparable-to-wider floodplains than effective boundaries. Some areas show additional significantly larger flood polygons (see figure below and item 5.1). This may be correct based on flood model results. See item 5.1 for more details regarding mapping of shallow flood depths. 1.2 BLE mapping covers effective 500-yr areas (see gray-shaded areas in the figure below). Mapping of BLE appears reasonable, but tie-ins may prove tricky. I recommend reviewing tie-ins more closely in the few, populated areas. 1.3 The area below shows significant differences with respect to effective flood hazard areas. I recommend ensuring that BLE mapping near the populated area is consistent with our model results. |
| 2. Mapping near downstream boundary condition | 2.1 Downstream mapping will tie-in with coastal flood hazards. |
| 3. Mapping of interbasin flow transfer (tie-ins) | 3.1 The lower half of Mission HUC shows large areas mapping into Aransas HUC. These “basin overflow” may complicate tie-ins with Aransas and Aransas Bay. At a minimum, verify proper connectivity along obvious channels. 3.2 If I recall correctly, the area below had significant overflow which we addressed during the modeling phase. Ensure proper tie-in. |
| 4. Stream continuity | 4.1 Stream continuity appears reasonable thanks, in part, to the wider BLE floodplains. Some discontinuities are normal for BLE mapping. |
| 5. Mapped depths | 5.1 In general, BLE mapping adds a substantial number of flood polygons throughout the lower half of the HUC. Many of these additional polygons represent shallow areas (less than 0.25 feet deep) in flat terrain. Ensure this criterion is consistent with other HUCs, as I do not recall adding so many flood areas in other HUCs. |
| 6. Mapping consistent with frequency | 6.1 The 10-yr flood hazard area appears spotty and too inconsistent, but this appears to be an issue that appears when I zoom in, only. Please review the file anyway. 6.2 I have not reviewed frequency consistency (0.2 > 1 > 10%). |
| 7. Miscellaneous | 7.1 Refreshing the 100-yr feature class while zoomed out is too slow. Assuming I have no network connectivity issues, please verify that we simplify the feature class (or shapefile) consistent with FEMA and TWDB standards. 7.2 The BLE 100-yr feature class shows pixelated boundaries. Ensure proper simplification to remove such “jaggedness.” Simplifying the delineation may also result in faster refreshing times. |