2-D Base Level Engineering

Texas Water Development Board

Quality Control — Mapping

HUC 8 Reviewed:	Aransas

Submitted by: 		Bryden Coughlan, Brian McNamara

QC by:			Guillermo Simon

- Comparison with effective mapping

Compare water surface elevation and mapping extents, if possible. Of note, mapping extents may prove difficult to verify because BLE models rain-on-grid and effective maps show flow concentration.

- Mapping around HUC perimeter

Verify mapping near downstream boundary condition

- Tie-in with neighboring basins

Verify mapping continuity around the HUC’s perimeter

- Stream connectivity

Verify continuous mapping along conveying systems (creeks, channels, roads (?))

- Mapped depths

Depth used in mapping. Is minimum depth consistent with other HUCS?

- Mapping consistent with frequency.

Verify that mapping extents and flood elevations increase with lower frequencies (e.g., is 500-yr mapping “larger” than 100-yr mapping?)

- Miscellaneous comments

| QC Round | Date | Comment |
| --- | --- | --- |
| QC 1 sent |  |  |
| QC 1 complete | 9/15/2021 |  |
| QC 2 sent |  |  |
| QC 2 complete |  |  |

| QC Item | Comments and figures |
| --- | --- |
| 1. Comparison with effective mapping | 1.1 BLE mapped boundaries appear comparable-to-larger than effective ones. In general, BLE appears to improve flood hazard identification over effective areas thanks to delineation on LiDAR. 1.2 An area near the town of Sinton, shows wide AE zones that do not match BLE well. However, effective mapping does not follow topography (except for the west side of the town where BLE and effective boundaries approximate well). I do not see any issues with BLE mapping, but please look and See figures below (hatched areas show BLE mapping). 1.3 BLE mapping is much wider than effective detailed mapping through Beeville. Like item 1.2, BLE mapping through this area appears correct, but we may want to verify how the tie-in with the effective, detailed boundary will look like. See figure below (hatched area shows BLE mapping). 1.4 “new” flooding polygons are attributable to the use of rain-on-grid boundary condition. |
| 2. Mapping near downstream boundary condition | 2.1 BLE mapping near the outflow point with Copano Bay looks correct. |
| 3. Mapping of interbasin flow transfer (tie-ins) | 3.1 Topography indicates that we may be able to use roads to cut off some polygons that flow out of Aransas HUC but verify that deep areas are properly tied-in with neighboring HUCs. See figure below showing examples of a necessary tie-ins with North Corpus Christi Bay and Mission HUCs. |
| 4. Stream continuity | 4.1 BLE mapping shows sufficient continuity throughout the HUC. 4.2 Mapping shows many “square” features which likely represent agricultural berms or canals. These agricultural features produce discontinuities, but mapping follows BLE standard. |
| 5. Mapped depths | 5.1 Delineation 1% flood hazards is consistent with depth grid. Mapping also removed small polygons and gaps correctly. |
| 6. Mapping consistent with frequency | 6.1 BLE mapping is consistent with their frequency, i.e., 0.2% > 1% > 10%. |
| 7. Miscellaneous | 7.1 I recommend smoothing out flood polygons to improve delineation and reduce polygon vertex count (this comment applies to all HUCs—I assume we will smooth out boundaries before completing the mapping process). |