2-D Base Level Engineering

Texas Water Development Board

Quality Control — Mapping

| HUC 8 | North Corpus Christi |
| --- | --- |
| Submitted by | Trish Burros |
| QC by | Guillermo Simon |

- Comparison with effective mapping

Compare water surface elevation and mapping extents, if possible. Of note, mapping extents may prove difficult to verify because BLE models rain-on-grid and effective maps show flow concentration.

- Mapping around HUC perimeter

Verify mapping near downstream boundary condition

- Tie-in with neighboring basins

Verify mapping continuity around the HUC’s perimeter – Addressed

- Stream connectivity

Verify continuous mapping along conveying systems (creeks, channels, roads (?)) Disconnections Align with project BLE standards

- Mapped depths

Depth use in mapping. Is minimum depth consistent with other HUCS?

- Mapping consistent with frequency.

Verify that mapping extents and flood elevations increase with lower frequencies (e.g., is 500-yr mapping “larger” than 100-yr mapping?)

- Miscellaneous comments

| QC Round | Date | Comment |
| --- | --- | --- |
| QC 1 sent | 6/11/2021 |  |
| QC 1 complete | 6/30/2021 |  |
| QC 2 sent |  |  |
| QC 2 complete |  |  |

| QC Item | Comments and figures |
| --- | --- |
| 1. Comparison with effective mapping | 1.1 Mapping extent is significantly larger than effective. Effective areas appear covered in BLE mapping. BLE mapping adds areas that likely flood (based on topography), but not shown on effective data. The following figure compares 1% AC flood near Ingleside. BLE depth mapping: Warm colors deeper flooding Cool colors shallower flooding Effective data 1.2 Mapping of BLE extents adds many areas not shown in the depth grid. The figure below shows, in pink, areas added as having a 1% AC. The figure below shows polygons that measure ~1.8 acres each, also within Ingleside. Response: Areas of mapping within the internal FP footprint not shown in the depth grids was due to an accidental filling of holes above our coastal mapping threshold of 2 acres. This has been corrected with holes 2 acres and above put back in. Our project threshold for detached flood polygons for coastal mapping includes areas above 0.33 acres, so the 1.8 is within our threshold. Ran check no detached polygons less than 0.33 acres for all frequencies. ~1.8 acres 1.3 BLE adds a significant amount of 1% AC areas, most of them showing depths less than 2 ft. Is this consistent with our expectation of BLE? See example area below, near Portland. |
| 2. Mapping near downstream boundary condition | 2.1 Verify mapping continuity for “large” channels discharging into the bay. See example area (near Portland). The areas shown below do not appear to interact with effective coastal mapping. Response: Drew in connectivity for large channels/conduits along coast for areas that discharge into Bay for 100 and 500-yr frequencies. Discontinuities Discontinuities at discharge points |
| 3. Mapping of basin flow transfer (tie-ins) | Pending |
| 4. Stream continuity | 4.1 Mapping shows continuity consistent with BLE level of effort. Mapping within ‘large” drainage channels appears correct, although some improvements are possible (see example below). 4.2 Mapping through culverts is inconsistent, but likely within the BLE standard. Some culverts show continued flooding, while others show gaps. Verify if this is consistent with BLE standards. Response: Consistent with project BLE Standards. Continuous mapping Discontinuity 4.3 Connect the areas shown below (Ingleside) Response: Areas connected. |
| 5. Mapped depths | 5.1 Mapping shows 1% AC depths shallower than 0.1 feet. |
| 6. Mapping consistent with frequency | 6.1 Mapping appears consistent with corresponding frequency, i.e., 0.2% > 1% > 10%. |
| 7. Miscellaneous | 7.1 The current delineation of 1 and 0.2% AC boundaries still shows “gridded” lines, or squares, albeit with rounded corners (see figure). Delineation of the 10% boundary is not smoothed at all, though this approach may be consistent with BLE standards. I recommend using a different smoothing technique in future mapping efforts. |