# Texas 2D BLE Analysis QA/QC Checklist

# Project Information

| HUC-8 Watershed Name & ID: | Lower Pecos- Red Bluff Reservoir (13070001) |
| --- | --- |
| HUC-10 Watershed Name & ID: | Red Bluff Reservoir (1307000101), Red Hills Draw (1307000105), Lower Salt Creek (1307000103), Upper Salt Creek (1307000102), Horsehead Draw-Pecos River (1307000106), Mcllvain Draw-Pecos River (1307000107), Rudd Draw-Soda Lake (1307000108), Narrow Bow Draw-Pecos River (1307000104), |
| Modeler: | AIG Technical Services, LLC |
| Reviewer: | AECOM |

## Terrain and Modeling Review (QC Items 1-6)

| Modeler Initials: | AIGTech | Reviewer Initials: | MM |
| --- | --- | --- | --- |
| Submittal Date: | 8/26/2024 (Milestone 1.5) | Review Submitted Date: | 08/29/2024 (Milestone 1.5) |
| Review Response Date: | NA | Backcheck Complete Date: | NA |

## Mapping Review (QC Item 7) – not included in this submittal

| Modeler Initials: |  | Reviewer Initials: | EDS |
| --- | --- | --- | --- |
| Submittal Date: |  | Review Submitted Date: | 11/7/2024 (sent 2/19/25) |
| Review Response Date: |  | Backcheck Complete Date: |  |

## Completeness Review (QC Item 8) – not included in this submittal

| Modeler Initials: |  | Reviewer Initials: | EDS |
| --- | --- | --- | --- |
| Submittal Date: |  | Review Submitted Date: | 11/7/2024 (sent 2/19/25) |
| Review Response Date: |  | Backcheck Complete Date: |  |

(Modeler to populate with an “X” for items included in the QC submittal)

| Level of Study |  |  | Flood Events |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| X | 2D BLE |  | X | 10% ACE (10-year) | X | 0.2% ACE (500-year) |
|  | Enhanced BLE |  | X | 4% ACE (25-year) | X | 1% plus |
|  |  |  | X | 2% ACE (50-year) | X | 1% minus |
|  |  |  | X | 1% ACE (100-year) |  | Observed Rainfall |

Instructions

Each section includes instructions to the modeler and reviewer regarding their responsibilities in the completion of this checklist. In the Methodologies / Data Sources tables, grey italic text is added as an example response. Additional sample responses are located in the Texas 2D BLE Guidelines Quick Guide.

Reviewer to indicate if Technical Review items generally comply using a “Pass”. If the technical review observes an issue of noncompliance or an item that requires additional attention, an “Fail” should be utilized.

## Submittal Checklist

Data submitted for each milestone should be carried through each subsequent milestone. All geospatial layers and models must have same defined projection system as terrain. Modeler to populate with an “X” for items included in the QC submittal.

| Base Map Data (QC items 1-6) |  |
| --- | --- |
|  | Location Map (PDF): HUC-8 and HUC-10 boundaries and/or model domain boundary if different from HUC-10s, stream gages, dams (if applicable), mainstem streams (for reference), background map |
|  | GIS Map (MXD or Map Package) with the following data included but not limited to: Stream centerlines, model breaklines, rainfall data, WSEL and Depth grids (10, 100, 500-yr), adjacent HUC mapping (available on the BLE viewer or currently studying), 2D connections for inflows/outflows, gages or points of flow comparisons (BFEs, highwater marks, etc.), profile lines, boundary condition lines, refinement regions, model domain boundary, model mesh, DFIRM mapping panels, base map data (roads, aerials, etc.), soils data, land use data, terrain |
| Technical Report / Documentation (QC items 1-6) |  |
|  | Technical Report / Project Notebook: assumptions, special considerations, and modeling concerns |
|  | Detailed explanation and support data of variances in methodologies and approaches from recommended guidelines (Should be included in Technical Report / Project Notebook) |
|  | Internal QA/QC Documentation: comments and responses |
|  | Flow comparison tables within the technical report |
| Hydrology (QC items 1-6) |  |
|  | Hydrologic Model (HEC-HMS) model with all associated DSS files for 7 required storm events |
|  | Hydrologic Data and Parameter Spreadsheets (rainfall calculations, curve number table, gage locations, gage analysis and flow validation) |
|  | Hydrologic modeling notebook including assumptions, notes, and special conditions (if not provided in report) |
| Hydraulics (QC items 1-6) |  |
|  | Hydraulic Model (HEC-RAS) with all associated files and 7 required storm events (remove non-final files) |
|  | Hydraulic Data and Parameter Spreadsheets (Manning’s n-value table, infiltration calculations support data-Soils, Land Use, Curve Number table, other calculations/data used in model) |
|  | Manning’s n-value table |
|  | Precipitation data correctly linked to hydrologic model or hard coded from provided spreadsheet |
|  | Hydraulic modeling notebook including assumptions, notes, and special conditions |
| Mapping (QC item 7) |  |
|  | Flood risk database |
|  | Modeler to complete topology validation as outlines in the guidelines |
|  | Modeler to verify mapping meets deliverable requirements as outlined in guidelines |
| Completeness Review (QC item 8) See Texas 2D BLE Guidelines Chapter 4 – Deliverables |  |
|  | Technical BLE Report |
|  | Final cleaned hydrologic model |
|  | Final cleaned hydraulic model |
|  | Flood risk database |
|  | CMNS updates |
|  | HAZUS results |
|  | Additional supplemental data |

Milestone 2 note to production team:  Please complete the info on Page 1 and the Submittal Checklist above. Looks like this has not been updated for M2 Reviews.

# 1 – Model Terrain

Milestone 1 Review approved 8/28/2024 for both model areas that make up the Lower Pecos-Red Bluff Reservoir HUC8 (Lower Pecos Lower and Lower Pecos Upper models).

Milestone 1 comments (from both LPLWR and LPUPR) that were related to content of the final BLE report were revisited and the draft BLE report reviewed to confirm that the requested Model Terrain content was adequately added – so those Milestone 1 responses are validated.

# 2 – Hydrology

Milestone 1 Review approved 8/28/2024 for both model areas that make up the Lower Pecos-Red Bluff Reservoir HUC8 (Lower Pecos Lower and Lower Pecos Upper models).

Milestone 1 comments (from LPUPR) that were related to content of the final BLE report were revisited and the draft BLE report reviewed to confirm that the requested Model Terrain content was adequately added – so those Milestone 1 responses are validated.

# 3 – Hydraulic Geometry

Milestone 1 Review approved 8/28/2024 for both model areas that make up the Lower Pecos-Red Bluff Reservoir HUC8 (Lower Pecos Lower and Lower Pecos Upper models).

Milestone 1 comments (from LPUPR) that were related to content of the final BLE report were revisited and the draft BLE report reviewed to confirm that the requested Model Terrain content was adequately added – so those Milestone 1 responses are validated.

# 4 – Hydraulic Boundary Conditions

Milestone 1 Review approved 8/28/2024 for both model areas that make up the Lower Pecos-Red Bluff Reservoir HUC8 (Lower Pecos Lower and Lower Pecos Upper models)

# 5 – Hydraulic Model Execution

Milestone 1 Review approved 8/28/2024 for both model areas that make up the Lower Pecos-Red Bluff Reservoir HUC8 (Lower Pecos Lower and Lower Pecos Upper models)

# 6 – Hydraulic Model Results

Milestone 1 Review approved 8/28/2024 for both model areas that make up the Lower Pecos-Red Bluff Reservoir HUC8 (Lower Pecos Lower and Lower Pecos Upper models).

Milestone 1 comments (from LPUPR) that were related to content of the final BLE report were revisited and the draft BLE report reviewed to confirm that the requested Model Terrain content was adequately added – so those Milestone 1 responses are validated.

# 7 – M1.5 Minor Post Processed Grids

## Review Comments – 8/29/2024

The comments below must be addressed, and revised M1.5 minor post processed grids resubmitted in order to allow TWDB to count Independence as “SMA”:

- Depth Grids: No comments - Noted

- WSEL Grids: No Comments - Noted

The notes below are for documentation and to potentially alert the production team to issue that may need to be addressed during Milestone 2 final BLE map production – they do NOT require response or revised submittal for this M1.5 process:

- Reviewer notes/confirmation:

-M1 backcheck is complete upon agreement with TWDB. Comments are closed out – Noted

-All files are present and viewable (no internal tie ins) – Noted

-Inland method was identified as threshold – Noted

-Post processed depth grids seem to have used inland threshold. However, it was noted through use of an in house script that some areas smaller than approximately 0.33-acres exist in the minor post processed depth grids. There were few enough that its likely due to clipping. Please just be aware during final map production.

M2 AtkinsRéalis response (10/18/2024): All polygons smaller than 0.33 acres are due to data clean up: clipping, smoothing, etc.

- Internal tie-ins are verified – Noted

-Extents of 1% WSE and Depth grids align, and extents of 0.2% WSE and Depth grids align – Noted

-Full mapping exists through entire HUC8 – Noted

- External tie ins are verified with difference raster’s provided. Note that final grid tie ins will need to be checked again during M2 review. – Noted

# 7 – Mapping Results

## Technical Review (Reviewer to confirm with Pass/Fail)

| Hydraulic Model |  |
| --- | --- |
| P | HEC-RAS model with results, pertinent data and required storm events |
| Flood Risk Database |  |
| F | All features fully populated per current FEMA guidelines |
| P | Mapping consistent between grids to polygons for all three required mapped events |
| P | 10yr smaller than 100yr smaller than 500yr |
| P | Holes filled in polygon, depth and WSE grids |
| P | Boundary interfaces align matching or identified as AOMI |

## Review Comments

- Please note that it doesn’t appear that the most up-to-date version of this QAQC ITR checklist has been used in the submittal, please plan to transfer all information from this copy to the updated/correct version upon resubmittal of Milestone 2 for backcheck.

- Modeler initials and submittal date need to be filled in for Mapping Review section at the beginning of this document

- The submittal checklist at the top of the document (Section 7/Section8) is incomplete, please fill out for backcheck submittal.

- All feature classes VERSION_ID field should be named as ‘BLE_MMYYYY’ based on the submittal date (note this issue discussed with Atkins on follow-up Toyah call – okay to respond/address consistent with that discussion).

- COM_NFO_ID should be populated with a unique value (often matches CID, but need something unique for the 48OTHR values

- TENPCT_FP – V_DATUM and LEN_UNIT fields should be empty or NULL

- Confirm all NHD features greater than 20 ac are included in WTR_AR. No editing/modification of polys added to WTR_AR are expected or typical.  Please reference the email sent from Sean Sutton at AECOM dated 2/17/25, as well as the previous emails from that thread, when assessing and responding to this comment, as it provides additional clarification on this topic.

- Add AOMI for WTR_LN_ID = 00671 upstream of the floodplain limits for the significant segment where there is no floodplain data (see Centerline_Containment feature class)

- Please review BFE_2D_Testpoints provided.  0.8% of the test points fail.  This is likely due to steep slope and cupping, but should be reviewed.

- Review attached bookmarks.  There are some visible waterbodies not showing full inundation (in the BLE fld haz ar layer) that should be corrected. Please reference the email sent from Sean Sutton at AECOM dated 2/17/25, as well as the previous emails from that thread, when assessing and responding to this comment, as it provides additional clarification on what the reviewer was looking at and commenting on.

# 8 – Completeness Review

## Technical Review (Reviewer to confirm with Pass/Fail)

| File Structure and Naming |  |
| --- | --- |
| F | Deliverables provided in TWDB/FEMA required file folder structure and naming convention |
| P | Data missing (if any) documented in report and ReadMe file within the required folder |
| Report |  |
| P | Technical 2D BLE report documents all assumptions, special considerations, etc. |
| P | Flow validation and difference discussed in report |
| P | Flow comparisons discussed in report |
| Hydrologic Model |  |
| P | HEC-HMS model with results, pertinent data and required storm events |
| Hydraulic Model |  |
| P | HEC-RAS model with results, pertinent data and required storm events |
| Flood Risk Database |  |
| F | All features fully populated per current FEMA guidelines |
| CNMS |  |
| F | Updated correctly with new centerlines |
| F | A1-A5 checks complete |
| F | CNMS Post- Processing Complete |
| HAZUS |  |
| F | Run per FEMA guidance |

## Review Comments

Hazus

- S_FRAC_Ar is clipped by the watershed.  According to the Flood Risk Assessment Guidance, exported Hazus data should include any census blocks that intersect the watershed and should not be clipped.

- Additionally, the census blocks that were considered inventory of the project footpring / watershed, but were not considered a loss, should have their loss populated as -8888 rather than 0.

File Structure and Naming

- Modeler initials and submittal date need to be filled in for Completeness Review section at the beginning of the document

Report

- The excel file “BLE3_Validation_Flow_Area_Plot” cannot be opened in the Reporting and Documentation folder. in the Reporting and Documentation folder.

Flood Risk Database

- See Comments Section 7.

CNMS

- S_Studies_Ln is empty

- Post Processing is incomplete with the empty S_Studies_Ln

- CNMS references/section incomplete in the report

.

QA/QC Checklist Approval

This 2D Base Level Engineering Analysis QA/QC Review is in compliance with the contract requirements and all task checkpoints are complete.  The independent QC Team has reviewed the analysis, presented review comments to the Production Team Task Leader, and discussed any problems or issues.  The Task Leader, QC Manager, QC Reviewer, and Project Manager have signed the QA/QC Checklist to confirm that all comments have been received, addressed, and documented appropriately.

| QA/QC Team – [TWDB or company name] |  |  |  | Production Team – [add company name] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| QC Manager |  | Date |  | Task Lead |  | Date |
| QC Reviewer |  | Date |  | Project Manager |  | Date |