# Texas 2D BLE Analysis QA/QC Checklist

# Project Information

| HUC-8 Watershed Name & ID: | Lower Pecos- Red Bluff Reservoir (13070001) |
| --- | --- |
| HUC-10 Watershed Name & ID: | Red Bluff Reservoir (1307000101), Red Hills Draw (1307000105), Lower Salt Creek (1307000103), Upper Salt Creek (1307000102), Horsehead Draw-Pecos River (1307000106), Mcllvain Draw-Pecos River (1307000107), Rudd Draw-Soda Lake (1307000108), Narrow Bow Draw-Pecos River (1307000104), |
| Modeler: | AIG Technical Services, LLC |
| Reviewer: | AECOM |

## M1 Review: Terrain, Modeling and Preliminary Tie-In Review (QC Items 1-6)

| Modeler Initials: | SM | Reviewer Initials: | MM |
| --- | --- | --- | --- |
| Submittal Date: | 8/26/2024 (Milestone 1.5) | Review Submitted Date: | 08/29/2024 (Milestone 1.5) |
| Review Response Date: | NA | Backcheck Complete Date: | NA |

## M2 Review: Mapping and Completeness Review (QC Item 7) – not included in this submittal

| Modeler Initials: | SM | Reviewer Initials: | EDS |
| --- | --- | --- | --- |
| Submittal Date: | 10/23/2024 | Review Submitted Date: | 11/7/2024 (sent 2/19/25) |
| Review Response Date: | 4/8/2025 | Backcheck Complete Date: | 4/30/2025 |

(Modeler to populate with an “X” for items included in the QC submittal)

| Level of Study |  |  | Flood Events |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| X | 2D BLE |  | X | 10% ACE (10-year) | X | 0.2% ACE (500-year) |
|  | Enhanced BLE |  | X | 4% ACE (25-year) | X | 1% plus |
|  |  |  | X | 2% ACE (50-year) | X | 1% minus |
|  |  |  | X | 1% ACE (100-year) |  | Observed Rainfall |

Instructions

Each section includes instructions to the modeler and reviewer regarding their responsibilities in the completion of this checklist. In the Methodologies / Data Sources tables, grey italic text is added as an example response. Additional sample responses are located in the Texas 2D BLE Guidelines Quick Guide.

Reviewer to indicate if Technical Review items generally comply using a “Pass”. If the technical review observes an issue of noncompliance or an item that requires additional attention, an “Fail” should be utilized.

## Submittal Checklist

Data submitted for each milestone should be carried through each subsequent milestone. All geospatial layers and models must have same defined projection system as terrain. Modeler to populate with an “X” for items included in the QC submittal.

| QA/QC Comments Addressed |  |
| --- | --- |
| X | M1 Comments have been addressed, we understand the comments are an example and not comprehensive of all the errors in the watershed. We have reviewed the entire HUC based on the QA/QC and revised our model accordingly. |
| X | M2 Comments have been addressed, we understand the comments are an example and not comprehensive of all the errors in the watershed. We have reviewed the entire HUC based on the QA/QC and revised our mapping accordingly. |
| M1: Base Map Data (QC items 1-6) |  |
| X | Location Map (PDF): HUC-8 and HUC-10 boundaries and/or model domain boundary if different from HUC-10s, stream gages, dams (if applicable), mainstem streams (for reference) |
| X | GIS Map (MXD or Map Package) with the following data included but not limited to: Stream centerlines, model breaklines, rainfall data, WSEL and Depth grids (10, 100, 500-yr), adjacent HUC mapping (available on the BLE viewer or currently studying), 2D connections for inflows/outflows, gages or points of flow comparisons (BFEs, highwater marks, etc.), profile lines, boundary condition lines, refinement regions, model domain boundary, model mesh, DFIRM mapping panels, base map data (roads, aerials, etc.), soils data, land use data, terrain |
| M1: Technical Report / Documentation (QC items 1-6) |  |
| X | Technical Report / Project Notebook: assumptions, special considerations, and modeling concerns |
| X | Detailed explanation and support data of variances in methodologies and approaches from recommended guidelines (Should be included in Technical Report / Project Notebook) |
| X | Internal QA/QC Documentation: comments and responses |
| X | Flow comparison tables within the technical report |
| M1: Hydrology (QC items 1-6) |  |
| X | Hydrologic Model (HEC-HMS) model with all associated DSS files for 7 required storm events |
| X | Hydrologic Data and Parameter Spreadsheets (rainfall calculations, curve number table, gage locations, gage analysis and flow validation) |
| X | Hydrologic modeling notebook including assumptions, notes, and special conditions (if not provided in report) |
| M1: Hydraulics (QC items 1-6) |  |
| X | Hydraulic Model (HEC-RAS) with all associated files and 7 required storm events run and results available in RASmapper (remove non-final files) Exported Raster Grids for 10, 100-, 500 yr events (using sloping render mode, unless previously coordinated with TWDB) |
| X | Hydraulic Data and Parameter Spreadsheets (Manning’s n-value table, infiltration calculations support data-Soils, Land Use, Curve Number table, other calculations/data used in model) |
| X | Manning’s n-value table |
| X | Precipitation data correctly linked to hydrologic model or hard coded from provided spreadsheet |
| X | Hydraulic modeling notebook including assumptions, notes, and special conditions (Draft BLE Report with sections related to terrain, modeling and validation) |
| M2: Mapping (QC item 7) |  |
| X | Flood risk database |
| X | Modeler to complete topology validation as outlined in the guidelines |
| X | Modeler to verify mapping meets deliverable requirements as outlined in guidelines |
| M2: Completeness Review (QC item 8) See Texas 2D BLE Guidelines Chapter 4 – Deliverables |  |
| X | Please submit items using the eBFE USGS format and MIP Terrain Folder (see example folder in BLE Resources) |
| X | Final Technical BLE Report |
| X | Final cleaned hydrologic model |
| X | Final cleaned hydraulic model |
| X | Flood risk database |
| X | CNMS updates |
| X | HAZUS results |
| X | Additional supplemental data: 1) ArcHydro outputs (if applicable), 2) Model domain shapefile, 3) LiDAR Extents Shp file, |

Milestone 2 note to production team:  Please complete the info on Page 1 and the Submittal Checklist above. Looks like this has not been updated for M2 Reviews.

# 1 – Model Terrain

Methodologies / Data Sources (Modeler to populate)

| Terrain Source(s): |  |
| --- | --- |
| Data Gaps: |  |
| Source Resolution: |  |
| Model Terrain Resolution: |  |
| Terrain Horizontal Projection: |  |
| Terrain Vertical Projection: |  |
| Z Values: |  |
| Terrain Buffer: |  |
| Other: |  |

## Technical Review (Reviewer to confirm with Pass/Fail)

| General Review |  |
| --- | --- |
|  | LiDAR source matches the latest available data at time of project initiation |
|  | Large reservoir or lake extents included in model domain |
|  | Check visible seams, gaps or holes of LiDAR sources (if more than one source was used) |

## Review Comments

| Modeler to Check Box of Acknowledgement during Response to Comments: | Acknowledge that review comments may be example comments and not a comprehensive of all required revisions. Consultant has reviewed provided comments and reviewed for applicability of comment throughout HUC-8. |
| --- | --- |

Milestone 1 Review approved 8/28/2024 for both model areas that make up the Lower Pecos-Red Bluff Reservoir HUC8 (Lower Pecos Lower and Lower Pecos Upper models).

Milestone 1 comments (from both LPLWR and LPUPR) that were related to content of the final BLE report were revisited and the draft BLE report reviewed to confirm that the requested Model Terrain content was adequately added – so those Milestone 1 responses are validated.

# 2 – Hydrology

## Methodologies / Data Sources (Modeler to populate)

| Software: |  |
| --- | --- |
| Rainfall Source: |  |
| Rainfall Duration: |  |
| Depth Selection: |  |
| Rainfall Temporal Distribution: |  |
| Areal Reduction: |  |
| Loss Methodology: |  |
| Unit Hydrograph Transform Method: |  |
| Inflow Hydrograph Source: |  |
| Any Deviations from TWDB Guidance? If yes, please describe |  |
| Other: |  |

## Statistical Data (Modeler to populate)

| Stream Gage Name & ID: |  |
| --- | --- |
| Stream Gage Name & ID: |  |
| Stream Gage Name & ID: |  |

## Technical Review (Reviewer to confirm with Pass/Fail)

| Excess Rainfall |  |
| --- | --- |
|  | Precipitation values are an aerial average of Atlas 14 values for the entire 2D area boundary for each storm event or other appropriate rainfall application |
|  | CN values appropriately reflect land use and soils for entire 2D area boundary |
|  | Losses are correctly calculated to establish the excess rainfall hyetographs by checking land use, soils and infiltration layers or other relative information |
|  | Total precipitation depths for 1%+ and 1%- are calculated correctly |
|  | Excess rainfall hyetographs are calculated for all storm events (10%, 4%, 2%, 1%, 0.2%, 1%+, and 1%-) |
|  | Cumulative excess rainfall values are in increasing order from 10%, 4%, 2%, 1%, to 0.2% |
|  | Cumulative excess rainfall values for the 1%+ and 1%- fall in order with the storm events matching the total precipitation values |
| Inflow Hydrographs |  |
|  | Shape and time-to-peak of any inflow hydrographs resemble past documented large storm events |
|  | Hydrographs reasonable when compared to the other return periods. The shapes should be similar, and hydrographs from different events should not intersect unless there is an engineering reason. Peak flows from a lower return period event should not be higher than a higher return period event |
|  | Contributing hydrographs from another RAS model applied at coincident locations (Provide locations in base map data) |
| General Hydrology |  |
|  | Any flow regulating dams in the model area? (Yes/No) |
|  | Dam modeling methodologies applied and explained? (Yes/No). If yes, provide discharge rating curve calculations and gage analysis documenting regulated versus unregulated records, if applicable. |
|  | Model aligns with documentation of dam modeling approaches. |
| Other Notes: NA |  |

## Review Comments

Modeler to Check Box of Acknowledgement during Response to Comments:

|  | Acknowledge that review comments may be example comments and not a comprehensive of all required revisions. Consultant has reviewed provided comments and reviewed for applicability of comment throughout HUC-8. |
| --- | --- |

Milestone 1 Review approved 8/28/2024 for both model areas that make up the Lower Pecos-Red Bluff Reservoir HUC8 (Lower Pecos Lower and Lower Pecos Upper models).

Milestone 1 comments (from LPUPR) that were related to content of the final BLE report were revisited and the draft BLE report reviewed to confirm that the requested Model Terrain content was adequately added – so those Milestone 1 responses are validated.

# 3 – Hydraulic Geometry

## Methodologies / Data Sources (Modeler to populate)

| Software: |  |
| --- | --- |
| Loss Methodology: |  |
| Soils Source: |  |
| Land Cover Source: |  |
| Manning's Roughness Source: |  |
| Nominal Mesh Cell Sizes: |  |
| Any deviations from TWDB Guidance? If yes, please describe. |  |
| Other: |  |

## Technical Review (Reviewer to confirm with Pass/Fail)

| 2D Mesh |  |
| --- | --- |
|  | 2D Boundary delineated along watershed boundary or buffer to ensure all contributing area is included |
|  | Nominal cell size appropriate for model detail (generally 200 feet, minimum 50 feet for refinement areas) |
|  | Computation points are at default locations or changes documented |
|  | Mesh Cell and Face tolerances are default or changes documented |
|  | No mesh errors |
|  | Cell face relationships are calculated |
| Breaklines |  |
|  | Breaklines are consistent with the terrain data |
|  | Breaklines sufficiently detailed to capture features (thalweg, banks, berms, etc.) |
|  | Cells do not overlap terrain features where appropriate (i.e., unwanted leaking) |
|  | Breakline cell sizing and parameters are appropriate |
|  | 2D Area Connections placed at applicable benchmarks, gages, or dams |
|  | 2D Area Connections set to use normal 2D equation domain for benchmarks and gages. Appropriate modeling approach used for dams. |
| Manning’s N-Values |  |
|  | Manning’s n-values associated to land cover data provided |
|  | Land Cover classifications are sufficiently descriptive within the data table |
|  | Manning’s n-value classifications generally match current aerial imagery |
|  | Manning’s n dataset is associated with the geometry files |
| General |  |
|  | All model datasets each have unique descriptive names |

## Review Comments

Modeler to Check Box of Acknowledgement during Response to Comments:

|  | Acknowledge that review comments may be example comments and not a comprehensive of all required revisions. Consultant has reviewed provided comments and reviewed for applicability of comment throughout HUC-8. |
| --- | --- |

Milestone 1 Review approved 8/28/2024 for both model areas that make up the Lower Pecos-Red Bluff Reservoir HUC8 (Lower Pecos Lower and Lower Pecos Upper models).

Milestone 1 comments (from LPUPR) that were related to content of the final BLE report were revisited and the draft BLE report reviewed to confirm that the requested Model Terrain content was adequately added – so those Milestone 1 responses are validated.

# 4 – Hydraulic Boundary Conditions

## Technical Review (Reviewer to confirm with Pass/Fail)

| Boundary Review |  |
| --- | --- |
|  | External boundaries connected to 2D mesh |
|  | Boundary conditions everywhere flow could build up at the boundary of the 2D mesh |
|  | Boundary conditions have a hydrograph or elevation/slope data assigned |
|  | Input hydrographs consistent with hydrologic data (DSS, manual entry) |
|  | Precipitation data is consistent with hydrologic data (DSS, manual entry) |
|  | Downstream boundary consistent with ground slope or known downstream WSEL |
|  | Flow hydrographs applied at appropriate locations |
|  | Energy grade slope for hydrograph distribution is reasonable |
|  | Internal rating curves appropriate and documented for control structures (if applicable) |
|  | Modeling Tie-ins: Check for flow continuity between upstream/downstream model domains for all profiles (e.g. matching flow hydrographs) |

## Review Comments

Modeler to Check Box of Acknowledgement during Response to Comments:

|  | Acknowledge that review comments may be example comments and not a comprehensive of all required revisions. Consultant has reviewed provided comments and reviewed for applicability of comment throughout HUC-8. |
| --- | --- |

Milestone 1 Review approved 8/28/2024 for both model areas that make up the Lower Pecos-Red Bluff Reservoir HUC8 (Lower Pecos Lower and Lower Pecos Upper models)

# 5 – Hydraulic Model Execution

## Methodologies / Data Sources (Modeler to populate)

| Unsteady Plan Names |  |  |  |
| --- | --- | --- | --- |
| 10% ACE (10-year): |  | 0.2% ACE (500-year): |  |
| 4% ACE (25-year): |  | 1% plus: |  |
| 2% ACE (50-year): |  | 1% minus: |  |
| 1% ACE (100-year): |  |  |  |
| Other: |  |  |  |

## Statistical Data (Modeler to populate)

| Time Step: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Courant Value: |  |  | Simulation Length: |  | hours |
| Computation Interval: |  | seconds | Model Runtime: |  | hours |
| Hydrograph Output Interval: |  | minutes | Mapping Output Interval: |  | minutes |

## Technical Review (Reviewer to confirm with Pass/Fail)

| Execution Review |  |
| --- | --- |
|  | Appropriate 2D parameters are used (Boundary Conditions, Time Step, Simulation Length, etc.) |
|  | Hydrograph Output Interval and Mapping Output Interval set at a reasonable interval to capture peaks |
|  | Model runtime is reasonable (< 12 hours, but dependent upon model size) |

## Review Comments

Modeler to Check Box of Acknowledgement during Response to Comments:

|  | Acknowledge that review comments may be example comments and not a comprehensive of all required revisions. Consultant has reviewed provided comments and reviewed for applicability of comment throughout HUC-8. |
| --- | --- |

Milestone 1 Review approved 8/28/2024 for both model areas that make up the Lower Pecos-Red Bluff Reservoir HUC8 (Lower Pecos Lower and Lower Pecos Upper models)

# 6 – Hydraulic Model Results

## Methodologies / Data Sources (Modeler to populate)

| Flow Validation/Gage Analysis |  |
| --- | --- |
| Software: |  |
| Methodology |  |
| Removal of Outliers and Justification |  |
| Review of Appropriateness of Gage Analysis results: - Check if there is a control structure upstream build in period of record, - Is the period of record sufficient, - Are there any gaps in the period of record, - Q vs A plot included - Wide confidence bands relative to flows? |  |
| Validation Notes (describe parameter adjustments): |  |
| Other: |  |

## Technical Review (Reviewer to confirm with Pass/Fail)

| Computation Checks |  |
| --- | --- |
|  | Model runs with no cell errors |
|  | Volume balance is within tolerance (ΔV < 1% is ideal, however 2% to 3% can be acceptable for rain on mesh) |
| Results Spot Checks Check at all critical structures, major confluences, a handful of headwater tributaries, downstream end, at benchmarks, and a handful of randomly selected locations. |  |
|  | No visible spikes in WSEL visibly present with a single cell |
|  | WSEL and depth grids appear reasonable (no artificial dips or spikes) |
|  | Visual verification flooding is connected downstream for all significant contributing areas (> 64 acres) |
|  | Velocity grids appear reasonable (no unusually high velocity areas or Courant values greater than 2) |
|  | No crossing profiles between all events |
|  | Inflow and outflow hydrograph timing is reasonable as compared to the peak |
|  | Spills into/from adjacent HUCs appropriately accounted for |
|  | WSEL tie-in between model domains and HUCs along major rivers/streams where inflows are applied |
| Results Full Checks |  |
|  | Computational time step meets ideal Courant condition |
|  | Diffusion Wave: V∆T∆X ≤2 (Up to 5) (Results are unit-less) |
|  | Model simulation is long enough to pass the entire hydrograph through the model (minimum of 12 hours past the peak, preferably 24 hours) |
|  | Hydrographs at internal and external boundaries are smooth |
|  | Contributing areas are contained with 2D mesh or accounted for with inflow hydrographs |
|  | No flow trapped at 2D boundary |
|  | Visual verification mapping is reasonable compared to effective |
| Flow Verification |  |
|  | Peak flow gage analysis performed on all gages in the watershed with sufficient data |
|  | All gages used have an appropriate period of record (10 to 20+ years) |
|  | All gages used have no extensive gaps in record which would impact the analysis |
|  | Gage analysis results validated to other sources (Regression, USGS, etc.) |
|  | Results at applicable benchmarks justified (gages, HWMs, effective BFEs) |
|  | Peak discharges comparable to gages, effective/prior studies, regional regression equations |

## Review Comments

Modeler to Check Box of Acknowledgement during Response to Comments:

|  | Acknowledge that review comments may be example comments and not a comprehensive of all required revisions. Consultant has reviewed provided comments and reviewed for applicability of comment throughout HUC-8. |
| --- | --- |

Milestone 1 Review approved 8/28/2024 for both model areas that make up the Lower Pecos-Red Bluff Reservoir HUC8 (Lower Pecos Lower and Lower Pecos Upper models).

Milestone 1 comments (from LPUPR) that were related to content of the final BLE report were revisited and the draft BLE report reviewed to confirm that the requested Model Terrain content was adequately added – so those Milestone 1 responses are validated.

# 7 – M1.5 Minor Post Processed Grids

## Review Comments – 8/29/2024

The comments below must be addressed, and revised M1.5 minor post processed grids resubmitted in order to allow TWDB to count Independence as “SMA”:

- Depth Grids: No comments - Noted

- WSEL Grids: No Comments - Noted

The notes below are for documentation and to potentially alert the production team to issue that may need to be addressed during Milestone 2 final BLE map production – they do NOT require response or revised submittal for this M1.5 process:

- Reviewer notes/confirmation:

-M1 backcheck is complete upon agreement with TWDB. Comments are closed out – Noted

-All files are present and viewable (no internal tie ins) – Noted

-Inland method was identified as threshold – Noted

-Post processed depth grids seem to have used inland threshold. However, it was noted through use of an in house script that some areas smaller than approximately 0.33-acres exist in the minor post processed depth grids. There were few enough that its likely due to clipping. Please just be aware during final map production.

M2 AtkinsRéalis response (10/18/2024): All polygons smaller than 0.33 acres are due to data clean up: clipping, smoothing, etc.

- Internal tie-ins are verified – Noted

-Extents of 1% WSE and Depth grids align, and extents of 0.2% WSE and Depth grids align – Noted

-Full mapping exists through entire HUC8 – Noted

- External tie ins are verified with difference raster’s provided. Note that final grid tie ins will need to be checked again during M2 review. – Noted

# 7 – Mapping Results

## Technical Review (Reviewer to confirm with Pass/Fail)

| Hydraulic Model |  |
| --- | --- |
| P | HEC-RAS model with results, pertinent data and required storm events |
| Flood Risk Database |  |
| P | All features fully populated per current FEMA guidelines |
|  | All features aligned and clipped to USGS HUC-8 boundary including: -Mapping should extend through entire HUC-8 -No mapping beyond HUC-8 boundary |
| P | Mapping (FLD_HAZ_AR) consistent between depth and WSE grids to polygons for all three required mapped events. The depth grid, water surface elevation grid and floodplain polygon extents must match per FEMA requirements, including the filling of holes, spillways, lake/pooled areas. |
| P | Containment within Events: (10% floodplain contained within 1% and 0.2%) |
|  | Centerline Containment within the 1% floodplain extents |
| P | Holes filled in polygon, depth and WSE grids: Holes Filled within the contiguous floodplain anything less than or equal to 2 acres in the 1- and 0.2% floodplain extents and anything less than or equal to 0.5 acres in the 10% floodplain extents. This should be clearly documented in the report. |
|  | Tie-ins Clarification: Tie-Ins realted to WSEL must be within 0.5ft for the following scenarios: 1) with currently studied HUCs (within the Team), 2) 2D HUCs available on the EBFE viewer, 3) Model Domain Tie-Ins must be 0.5 feet for all reoccurrence intervals 4)FEMA requires water surface elevation tie-ins within 0.5 ft for the mapped event of 10%, 1% and 0.2%. If mapping tie ins cannot be achieved: 1)Adjacent study not completed, 2) Adjacent study completed in 1D, or 3) Adjacent study completed with different modeling and mapping methodology causing tie in achievement to be difficult 1) Need to add AOMI area as AOMI_TYP = NP and AOMI_INFO = Spill area. In need of further engineering analysis. Where complex mapping tie ins between domains/HUCs exist, prior coordination with TWDB at M1 is recommended. (Ex. Braided channels) Please verify for Model Tie-Ins (Flow): o Each event needs to achieve flow continuity at the SA/2D connection. (E.g. Flow hydrographs must match at SA 2D connection between model domains). o A SA/2D connection line may be used to obtain a flow hydrograph between two model domains. The flow hydrograph obtained from the SA/2D connection line should match the Inflow Hydrograph Boundary condition to the downstream model domain. Please verify for Model/ Mapping Tie-Ins (WSEL): o Mapping tie ins (WSEL, Depth) are required for all modeled events across the entire floodplain’s width at one location in the model’s overlap region. This is validated using a raster calculation between both models’ WSEL grids. You must meet the WSEL tie in requirements and the SA/2D connection flow continuity between models. The WSEL tie ins requirement of ±0.5 ft. must be met in all model profiles (10, 25, 50, 100, and 500 yr.) and for 10, 100 and 500-yr for mapping products. Review of fringe mapping tie ins between study HUCs and other adjacent HUCs are within the ±0.5 ft. Both Model domains/HUC boundaries need to be in the ±0.5 ft. |
| P | Waterbodies Added (Lakes/Ponding Areas/Reservoirs) Water bodies of lake and reservoir inundation areas of 20 acres or greater or named (NHD waterbodies dataset) should be merged into final floodplains (FLD_HAZ_AR) and water areas (WTR_AR). |
| P | Boundary interfaces align matching or identified as AOMI |
| P | Centerline Containment (WTR_LN) within 1% floodplain extents |
| P | Schema |
| P | Review that DTL_STUD_AR and DTL_STUD_LN to ensure coverage of the NFHL effective detailed studies. |
| P | Reviewed that BFE feature values against the 1% annual chance event (ACE) water surface elevation (WSE) grid within +/- 1 foot |

## Review Comments

Modeler to Check Box of Acknowledgement during Response to Comments:

|  | Acknowledge that review comments may be example comments and not a comprehensive of all required revisions. Consultant has reviewed provided comments and reviewed for applicability of comment throughout HUC-8. |
| --- | --- |

- Please note that it doesn’t appear that the most up-to-date version of this QAQC ITR checklist has been used in the submittal, please plan to transfer all information from this copy to the updated/correct version upon resubmittal of Milestone 2 for backcheck.

Response 20250403:  QC Document Updated

Backcheck – Approved, EDS

- Modeler initials and submittal date need to be filled in for Mapping Review section at the beginning of this document

Response 20250403: QC Document Updated

Backcheck – Approved, EDS

- The submittal checklist at the top of the document (Section 7/Section8) is incomplete, please fill out for backcheck submittal.

Response 20250403: QC Document Updated

Backcheck – Approved, EDS

- All feature classes VERSION_ID field should be named as ‘BLE_MMYYYY’ based on the submittal date (note this issue discussed with Atkins on follow-up Toyah call – okay to respond/address consistent with that discussion).

Response 20250403: Agreed, no changes made

Backcheck – Approved, EDS

- COM_NFO_ID should be populated with a unique value (often matches CID, but need something unique for the 48OTHR values

Response 20250403: Appended _1 and _2 to the OTHR values.

Backcheck – Approved, EDS

- TENPCT_FP – V_DATUM and LEN_UNIT fields should be empty or NULL

Response 20250403: Made both fields empty.

Backcheck – Approved, EDS

- Confirm all NHD features greater than 20 ac are included in WTR_AR. No editing/modification of polys added to WTR_AR are expected or typical.  Please reference the email sent from Sean Sutton at AECOM dated 2/17/25, as well as the previous emails from that thread, when assessing and responding to this comment, as it provides additional clarification on this topic.

Response 20250403: Confirmed that all NHD waterbodies greater than 20 acres or named are 	included in WTR_AR

Backcheck – Approved, EDS

- Add AOMI for WTR_LN_ID = 00671 upstream of the floodplain limits for the significant segment where there is no floodplain data (see Centerline_Containment feature class)

Response 20250403: Added AOMI polygon.

Backcheck – Approved, EDS

- Please review BFE_2D_Testpoints provided.  0.8% of the test points fail.  This is likely due to steep slope and cupping, but should be reviewed.

Response 20250403: The test points were reviewed and all failed areas are related to steep slopes and cupping. As part of a review, additional thresholds were reviewed and the results are as follows:

Total Points:  674,531

Total above abs|1|:  1,026, 0.15%

Total above abs|2|:  83, 0.01%

Total above abs|5|:  0

Total above abs|10|:  0

Backcheck – Approved, EDS

- Review attached bookmarks.  There are some visible waterbodies not showing full inundation (in the BLE fld haz ar layer) that should be corrected. Please reference the email sent from Sean Sutton at AECOM dated 2/17/25, as well as the previous emails from that thread, when assessing and responding to this comment, as it provides additional clarification on what the reviewer was looking at and commenting on.

Response 20250403: Updated floodplain to include bookmarked waterbodies as well as waterbodies in S_WTR_AR

Backcheck – Approved, EDS

# 8 – Completeness Review

## Technical Review (Reviewer to confirm with Pass/Fail)

| File Structure and Naming |  |
| --- | --- |
| P | Deliverables provided in TWDB/FEMA required file folder structure and naming convention |
| P | Data missing (if any) documented in report and ReadMe file within the required folder |
| Report |  |
| P | Technical 2D BLE report documents all assumptions, special considerations, etc. |
| P | Flow validation and difference discussed in report |
| P | Flow comparisons discussed in report |
| Hydrologic Model |  |
| P | HEC-HMS model with results, pertinent data and required storm events |
| Hydraulic Model |  |
| P | HEC-RAS model with results, pertinent data and required storm events |
| Flood Risk Database |  |
| P | All features fully populated per current FEMA guidelines |
| CNMS |  |
| P | Updated correctly with new centerlines |
| P | A1-A5 checks complete |
| P | CNMS Post- Processing Complete |
| HAZUS |  |
| P | Run per FEMA guidance |

## Review Comments

- S_FRAC_Ar is clipped by the watershed.  According to the Flood Risk Assessment Guidance, exported Hazus data should include any census blocks that intersect the watershed and should not be clipped.

Response 20250403: FRAC is now selected by census blocks that intersect the HUC8 boundary, but not clipped by the boundary.

Backcheck – Approved, EDS

- Additionally, the census blocks that were considered inventory of the project footprint / watershed, but were not considered a loss, should have their loss populated as -8888 rather than 0.

Response 20250403: the census blocks that were considered inventory of the project footpring / watershed, but were not considered a loss populated as -8888 rather than 0

Backcheck – Approved, EDS

File Structure and Naming

- Modeler initials and submittal date need to be filled in for Completeness Review section at the beginning of the document

Response 20250403:  QC Document Updated

Backcheck – Approved.  Review of other M2 reviews state that the latest template no longer has this section, EDS

Report

- The excel file “BLE3_Validation_Flow_Area_Plot” cannot be opened in the Reporting and Documentation folder. in the Reporting and Documentation folder.

Response 20250403: The excel file was not included in the M3 submittal. Relevant validation data can be found in section 3.1.2 Validation of BLE Results Using Discharge-Drainage Area Curves of the report.

Backcheck –

Flood Risk Database

- See Comments Section 7.

Backcheck – Approved, EDS

CNMS

- S_Studies_Ln is empty

Response 20250403: S_Studies_Ln Populated.

Backcheck – Approved, EDS

- Post Processing is incomplete with the empty S_Studies_Ln

Response 20250403: BLE CNMS featureclasses match approved CNMS geodatabase

Backcheck – Approved, EDS

- CNMS references/section incomplete in the report

Response 20250403: CNMS references/section in the report have been completed

Backcheck – Approved, EDS

Terrain

- Source_Index does not fully cover all tiles delivered in ‘Bare_Earth_DEM’ raster. Please add necessary tiles to cover .

AR response 20250506: Updated

- S_Submittal_Info is much larger than the extents of the Source_Index and raster deliverable in ‘Bare_Earth_DEM’.  Please consider clipping to the revised Source_Index (see comment above)

AR response 20250506: Updated

- S_Submittal_Info should also follow FEMA Data Capture Guidance where TERRAIN CAPTURE tasks should not have overlapping polygons from different sources.

AR response 20250506: Updated

- Metadata comments

  - The <pubdate> in the <citation> and <timeperd> sections should be set to the MIP Submission Date.

AR response 20250506: pubdate updated

  - If necessary, update bounding coordinates based on changes made in comments above about Source_Index and S_Submittal_Info

AR response 20250506: Updated

  - <placekey> entries are missing some values.  Please ensure the REGION, STATE, COUNTY, COUNTY-FIPS, COMMUNITY, FEMA-CID and HUC8 code is entered for each community in the purchase geography. If not in purchase geography, they can be removed.

AR response 20250506: There is no FEMA issued CID for Reeves County  and Pecos County so the  <placekey>FEMA-CID entry was not included.

  - Replace miphelp@riskmapcds.com in all locations with updated MIPHelp address of fema-riskmap-ithelp@fema.dhs.gov

AR response 20250506: Updated

  - <srccontr> text should be input in the CONTRIB field in L_Source_Cit for each TOPO#

AR response 20250506: Text updated

  - In the <eainfo> section, an entry should be added for the Source_Index deliverable

AR response 20250506:

QA/QC Checklist Approval

This 2D Base Level Engineering Analysis QA/QC Review is in compliance with the contract requirements and all task checkpoints are complete.  The independent QC Team has reviewed the analysis, presented review comments to the Production Team Task Leader, and discussed any problems or issues.  The Task Leader, QC Manager, QC Reviewer, and Project Manager have signed the QA/QC Checklist to confirm that all comments have been received, addressed, and documented appropriately.

| QA/QC Team – [TWDB or company name] |  |  |  | Production Team – [add company name] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| QC Manager |  | Date |  | Task Lead |  | Date |
| QC Reviewer |  | Date |  | Project Manager |  | Date |