# Texas 2D BLE Analysis QA/QC Checklist

# Project Information

| HUC-8 Watershed Name & ID: | Delaware, 13070002 |
| --- | --- |
| HUC-10 Watershed Name & ID: | 1307000201, 1307000202, 1307000203 |
| Modeler: | Clint Kimball (EDGE Engineering) |
| Reviewer: | Dani Halloran (AECOM) |

## M1 Review: Terrain, Modeling and Preliminary Tie-In Review (QC Items 1-6)

| Modeler Initials: | CK | Reviewer Initials: | DH |
| --- | --- | --- | --- |
| Submittal Date: | 5/1/2024 | Review Submitted Date: | 5/20/2024 |
| Review Response Date: | 6/20/2024 | Backcheck Complete Date: | 6/28/2024 (sent 7/8/24) |
| Review Response Date: | 8/22/24 | Backcheck 2 Complete Date: | 8/22/2024 |

## M2 Review: Mapping and Completeness Review (QC Item 7)

| Modeler Initials: | CK | Reviewer Initials: | ES |
| --- | --- | --- | --- |
| Submittal Date: | 10/2/2024 | Review Submitted Date: | 10/28/2024 (sent 2/19/25) |
| Review Response Date: | 3/14/2025 | Backcheck Complete Date: | 3/24/2025 |

(Modeler to populate with an “X” for items included in the QC submittal)

| Level of Study |  |  | Flood Events |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| X | 2D BLE |  | X | 10% ACE (10-year) | X | 0.2% ACE (500-year) |
|  | Enhanced BLE |  | X | 4% ACE (25-year) | X | 1% plus |
|  |  |  | X | 2% ACE (50-year) | X | 1% minus |
|  |  |  | X | 1% ACE (100-year) |  | Observed Rainfall |

Instructions

Each section includes instructions to the modeler and reviewer regarding their responsibilities in the completion of this checklist. In the Methodologies / Data Sources tables, grey italic text is added as an example response. Additional sample responses are located in the Texas 2D BLE Guidelines Quick Guide.

Reviewer to indicate if Technical Review items generally comply using a “Pass”. If the technical review observes an issue of noncompliance or an item that requires additional attention, an “Fail” should be utilized.

## Submittal Checklist

Data submitted for each milestone should be carried through each subsequent milestone. All geospatial layers and models must have same defined projection system as terrain. Modeler to populate with an “X” for items included in the QC submittal.

| QA/QC Comments Addressed |  |
| --- | --- |
| X | M1 Comments have been addressed, we understand the comments are an example and not comprehensive of all the errors in the watershed. We have reviewed the entire HUC based on the QA/QC and revised our model accordingly. |
| X | M2 Comments have been addressed, we understand the comments are an example and not comprehensive of all the errors in the watershed. We have reviewed the entire HUC based on the QA/QC and revised our mapping accordingly. |
| M1: Base Map Data (QC items 1-6) |  |
| X (Included in RAS Mapper) | Location Map (PDF): HUC-8 and HUC-10 boundaries and/or model domain boundary if different from HUC-10s, stream gages, dams (if applicable), mainstem streams (for reference) |
| X | GIS Map (MXD or Map Package) with the following data included but not limited to: Stream centerlines, model breaklines, rainfall data, WSEL and Depth grids (10, 100, 500-yr), adjacent HUC mapping (available on the BLE viewer or currently studying), 2D connections for inflows/outflows, gages or points of flow comparisons (BFEs, highwater marks, etc.), profile lines, boundary condition lines, refinement regions, model domain boundary, model mesh, DFIRM mapping panels, base map data (roads, aerials, etc.), soils data, land use data, terrain |
| M1: Technical Report / Documentation (QC items 1-6) |  |
| X | Technical Report / Project Notebook: assumptions, special considerations, and modeling concerns |
| X | Detailed explanation and support data of variances in methodologies and approaches from recommended guidelines (Should be included in Technical Report / Project Notebook) |
| X | Internal QA/QC Documentation: comments and responses |
|  | Flow comparison tables within the technical report |
| M1: Hydrology (QC items 1-6) |  |
| X | Hydrologic Model (HEC-HMS) model with all associated DSS files for 7 required storm events |
| X (Explained in report) | Hydrologic Data and Parameter Spreadsheets (rainfall calculations, curve number table, gage locations, gage analysis and flow validation) |
| X (Explained in report) | Hydrologic modeling notebook including assumptions, notes, and special conditions (if not provided in report) |
| M1: Hydraulics (QC items 1-6) |  |
| X | Hydraulic Model (HEC-RAS) with all associated files and 7 required storm events run and results available in RASmapper (remove non-final files) Exported Raster Grids for 10, 100-, 500 yr events (using sloping render mode, unless previously coordinated with TWDB) Difference Raster for tie-ins? (for 10-, 100, 500yr events) (add some descriptions) |
| X (Explained in report) | Hydraulic Data and Parameter Spreadsheets (Manning’s n-value table, infiltration calculations support data-Soils, Land Use, Curve Number table, other calculations/data used in model) |
| X (Explained in report) | Manning’s n-value table |
| X | Precipitation data correctly linked to hydrologic model or hard coded from provided spreadsheet |
| X (Explained in report) | Hydraulic modeling notebook including assumptions, notes, and special conditions (Draft BLE Report with sections related to terrain, modeling and validation) |
| M2: Mapping (QC item 7) |  |
| X | Flood risk database |
| X | Modeler to complete topology validation as outlines in the guidelines |
| X | Modeler to verify mapping meets deliverable requirements as outlined in guidelines |
| M2: Completeness Review (QC item 8) See Texas 2D BLE Guidelines Chapter 4 – Deliverables |  |
| X | Please submit items using the eBFE USGS format and MIP Terrain Folder (see example folder in BLE Resources) |
| X | Final Technical BLE Report |
| X | Final cleaned hydrologic model |
| X | Final cleaned hydraulic model |
| X | Flood risk database |
| X | CNMS updates |
| X | HAZUS results |
| X | Additional supplemental data: 1) ArcHydro outputs (if applicable), 2) Model domain shapefile, 3) LiDAR Extents Shp file, |

## Review Comments

- The GIS Map Data has been checked above but majority of items listed have not been provided (I could only find the WSEL rasters, perimeter, and 2D Connections). Please provide these or strikethrough, and provide explanation for, all items that are not provided. Depth grids should at the very least be included.

Response: Agree. Additional GIS data provided.

Backcheck: DH passed

- Hydrology and hydraulics is explained in the BLE report. I would check the box and provide that explanation.

Response: Agree. Hydrology and hydraulics are covered in the BLE report. DH

Backcheck: DH passed

- The internal QC sheets need to be signed off and a reviewer name added.

Response: Agree. Signed off QC sheet with reviewer’s name provided. DH

Backcheck: DH passed

# 1 – Model Terrain

Methodologies / Data Sources (Modeler to populate)

| Terrain Source(s): | Please see technical memo for more detail: SE New Mexico LiDAR (2018), Texas West Central LiDAR (2018), and Pecos Dallas LiDAR (2019) |
| --- | --- |
| Data Gaps: | None |
| Source Resolution: | Cell size of 1 x 1 meter |
| Model Terrain Resolution: | Cell size of 10 x 10 feet |
| Terrain Horizontal Projection: | North American Datum (NAD) 1983, State Plane Texas Central, FIPS 4203 |
| Terrain Vertical Projection: | NAVD88 |
| Z Values: | Units of feet |
| Terrain Buffer: | 20,000 feet |
| Other: |  |

## Technical Review (Reviewer to confirm with Pass/Fail)

| General Review |  |
| --- | --- |
| P | LiDAR source matches the latest available data at time of project initiation |
| P | Large reservoir or lake extents included in model domain |
| P | Check visible seams, gaps or holes of LiDAR sources (if more than one source was used) |

## Review Comments

| Modeler to Check Box of Acknowledgement during Response to Comments: | Acknowledge that review comments may be example comments and not a comprehensive of all required revisions. Consultant has reviewed provided comments and reviewed for applicability of comment throughout HUC-8. |
| --- | --- |

# 2 – Hydrology

## Methodologies / Data Sources (Modeler to populate)

| Software: | HEC-HMS 4.11 |
| --- | --- |
| Rainfall Source: | NOAA Atlas 14 |
| Rainfall Duration: | 24 hours |
| Depth Selection: | Spatially Varied (See BLE Report) |
| Rainfall Temporal Distribution: | Frequency Storm |
| Areal Reduction: | None |
| Loss Methodology: | N/A (losses applied to hydraulic model) |
| Unit Hydrograph Transform Method: | N/A |
| Inflow Hydrograph Source: |  |
| Any Deviations from TWDB Guidance? If yes, please describe |  |
| Other: |  |

## Statistical Data (Modeler to populate)

| Stream Gage Name & ID: | Delaware River near Red Bluff, NM (08408500) |
| --- | --- |
| Stream Gage Name & ID: | Delaware River Trib near Orla, TX (08407800) |
| Stream Gage Name & ID: |  |

## Technical Review (Reviewer to confirm with Pass/Fail)

| Excess Rainfall |  |
| --- | --- |
| P | Precipitation values are an aerial average of Atlas 14 values for the entire 2D area boundary for each storm event or other appropriate rainfall application |
| P | CN values appropriately reflect land use and soils for entire 2D area boundary |
| P | Losses are correctly calculated to establish the excess rainfall hyetographs by checking land use, soils and infiltration layers or other relative information |
| P | Total precipitation depths for 1%+ and 1%- are calculated correctly |
| P | Excess rainfall hyetographs are calculated for all storm events (10%, 4%, 2%, 1%, 0.2%, 1%+, and 1%-) |
| P | Cumulative excess rainfall values are in increasing order from 10%, 4%, 2%, 1%, to 0.2% |
| P | Cumulative excess rainfall values for the 1%+ and 1%- fall in order with the storm events matching the total precipitation values |
| Inflow Hydrographs |  |
| N/A | Shape and time-to-peak of any inflow hydrographs resemble past documented large storm events |
| N/A | Hydrographs reasonable when compared to the other return periods. The shapes should be similar, and hydrographs from different events should not intersect unless there is an engineering reason. Peak flows from a lower return period event should not be higher than a higher return period event |
| N/A | Contributing hydrographs from another RAS model applied at coincident locations (Provide locations in base map data) |
| General Hydrology |  |
| N/A | Any flow regulating dams in the model area? (Yes/No) |
| N/A | Dam modeling methodologies applied and explained? (Yes/No). If yes, provide discharge rating curve calculations and gage analysis documenting regulated versus unregulated records, if applicable. |
| N/A | Model aligns with documentation of dam modeling approaches. |
| Other Notes: NA |  |

## Review Comments

Modeler to Check Box of Acknowledgement during Response to Comments:

|  | Acknowledge that review comments may be example comments and not a comprehensive of all required revisions. Consultant has reviewed provided comments and reviewed for applicability of comment throughout HUC-8. |
| --- | --- |

- Note (no comment back needed): HEC-RAS and HEC-HMS connections were verified. Specially Varying rainfall used and is applied to HEC-RAS using incremental precipitation at points.

# 3 – Hydraulic Geometry

## Methodologies / Data Sources (Modeler to populate)

| Software: | HEC-RAS 6.4.1 |
| --- | --- |
| Loss Methodology: | Spatially varied CN |
| Soils Source: | SSURGO |
| Land Cover Source: | NLCD 2019 |
| Manning's Roughness Source: | Assigned based on land use. |
| Nominal Mesh Cell Sizes: | 300 x300 except for channel refinement regions, breaklines and urban regions, where 150 ft x150 ft was considered. |
| Any deviations from TWDB Guidance? If yes, please describe. |  |
| Other: |  |

## Technical Review (Reviewer to confirm with Pass/Fail)

| 2D Mesh |  |
| --- | --- |
| P | 2D Boundary delineated along watershed boundary or buffer to ensure all contributing area is included |
| P | Nominal cell size appropriate for model detail (generally 200 feet, minimum 50 feet for refinement areas) |
| P | Computation points are at default locations or changes documented |
| P | Mesh Cell and Face tolerances are default or changes documented |
| P | No mesh errors |
| P | Cell face relationships are calculated |
| Breaklines |  |
| P | Breaklines are consistent with the terrain data |
| P | Breaklines sufficiently detailed to capture features (thalweg, banks, berms, etc.) |
| P | Cells do not overlap terrain features where appropriate (i.e., unwanted leaking) |
| P | Breakline cell sizing and parameters are appropriate |
| P | 2D Area Connections placed at applicable benchmarks, gages, or dams |
| P | 2D Area Connections set to use normal 2D equation domain for benchmarks and gages. Appropriate modeling approach used for dams. |
| Manning’s N-Values |  |
| P | Manning’s n-values associated to land cover data provided |
| P | Land Cover classifications are sufficiently descriptive within the data table |
| P | Manning’s n-value classifications generally match current aerial imagery |
| P | Manning’s n dataset is associated with the geometry files |
| General |  |
| P | All model datasets each have unique descriptive names |

## Review Comments

Modeler to Check Box of Acknowledgement during Response to Comments:

|  | Acknowledge that review comments may be example comments and not a comprehensive of all required revisions. Consultant has reviewed provided comments and reviewed for applicability of comment throughout HUC-8. |
| --- | --- |

- Please provide 1sqmi and 0.5sqmi streamlines. Unable to verify the streams and their extents.

Response: Agree. 1.0 and 0.5 sq. mi. streamlines were provided.

Backcheck: DH passed

- It doesn’t look like a buffer was applied to the perimeter. Please either add a buffer to the WA or explain why a buffer was not applied.

Response: Partially agree. It was agreed that the mesh was extended only where the 500-yr depth on the boundary is greater than 0.5’ to provide an overlap area for the tie-in with adjacent project boundaries.

Backcheck: DH- Makes sense Passed.

- Explain why a 300ft cell size was used in the report, as it differs from the general 200ft cell size laid out in the TWDB guidance.

Response: The TWDB guidance says, “When generating the computational cells within the mesh, the standard initial cell size for BLE is 200-feet X 200-feet with a maximum cell size of 300-feet X 300-feet.” Atkins decided to use the maximum to reduce model sizes and run times. EDGE also explained this matter to AECOM, and they agreed.

Backcheck:  DH passed

- There are many areas throughout the WA were the stream refinement regions look to have been merged together. I know this happens when trying to eliminate parts in the polygon so they can be added to RAS.  I would fix some of the larger areas where this occurs, example below.

Response: Disagree. HEC-RAS does not allow interior holes in refinement regions. Filling these holes produces higher resolution modeling without significantly increasing model run time, even when applied to larger areas such as this.

Backcheck: DH- Agree that is only adds resolution. In the past, I have gotten around the interior holes error in HECRAS by splitting the polygons, but I do agree that it adds more resolution. Passed

- CNs need to be documented in the report. Development of the CN values should also be included.

Response: Disagree. CN development is discussed in the BLE report in section 1.2.

Backcheck: DH - CNs match what is in the report. Passed.

- Mannings for “no data” values is 0.06 in RAS. The report states 0.115. Fix discrepancy.

Response: Agree. Discrepancy was fixed.

Backcheck: DH - I am still seeing the discrepancy. Report says 0.115 and model says 0.06.

Response 08/21/2024 – Since NoData land use type is not present within the model domain extent, this change was not applied in the model. However, we will update value in the BLE report as part of the M2 submittal.

Backcheck 2(RG): Passed, final BLE report to be reviewed during Milestone 2. M2

M2 AtkinsRéalis response (10/08/2024): Manning’s n value for NoData land use category is now updated to 0.06 in Table 2-5 of the Delaware HUC8 BLE report. Note that there are no locations in the model domain that have a land use value of NoData, so this value of n=0.06 is not actually used in any of the model calculations.

Backcheck (Milestone2) ITR: Passed

- Mannings value for channels is just set to 0.04. The TWDB guidance states that the mannings values should be set based on the NLCD Land Use Descriptions, see screen shot below. Provide explanation in report documentation.

Response: Disagree. TWDB Guidelines Table 2.3 on page 22 provides Manning’s values specifically for channels. A Manning’s n-value of 0.04 (Clean, winding, some pools, and shoals) was used for this analysis.

Backcheck: DH- Agreed. Passed, however, as noted in the Monument QAQC document, please just ensure that documentation of the approach and confirmation that good validation results were achieved are all documented in the final Delaware BLE report (to be reviewed during Milestone 2), as the single value method differs some from other model areas and BLE done for TWDB.

Backcheck (Milestone 2) ITR: Passed

- TWDB naming convention is not followed. The Names should be the HUC-8 + HUC-10 last 3 digits, and the Flow names should follow the 1pct, 10pct, etc. See the guidance and the screen shot below.

Response: Agree. HEC-HMS and HEC-RAS models have been renamed to follow TWDB/Atkins naming convention with the following agreed exceptions: The first 4 letters of the HUC 8 name were used for naming and HUC-8 number last 3 digits for numbering.

Backcheck: DH- Agreed. Passed.

- There are cell alignment issues throughout the model. Please review the model and fix appropriately.

Response: Agree. Fixed cell alignment issues throughout the model.

Backcheck: DH passed

# 4 – Hydraulic Boundary Conditions

## Technical Review (Reviewer to confirm with Pass/Fail)

| Boundary Review |  |
| --- | --- |
| P | External boundaries connected to 2D mesh |
| P | Boundary conditions everywhere flow could build up at the boundary of the 2D mesh |
| P | Boundary conditions have a hydrograph or elevation/slope data assigned |
| P | Input hydrographs consistent with hydrologic data (DSS, manual entry) |
| P | Precipitation data is consistent with hydrologic data (DSS, manual entry) |
| P | Downstream boundary consistent with ground slope or known downstream WSEL |
| P | Flow hydrographs applied at appropriate locations |
| N/A | Energy grade slope for hydrograph distribution is reasonable |
| N/A | Internal rating curves appropriate and documented for control structures (if applicable) |
|  | Modeling Tie-ins: Check for flow continuity between upstream/downstream model domains for all profiles (e.g. matching flow hydrographs) |

## Review Comments

Modeler to Check Box of Acknowledgement during Response to Comments:

|  | Acknowledge that review comments may be example comments and not a comprehensive of all required revisions. Consultant has reviewed provided comments and reviewed for applicability of comment throughout HUC-8. |
| --- | --- |

- How is the tie in with the downstream HUC8 Lower Pecos- Red Bluff Reservoir going to be applied? This needs to be added to the report and depth rasters included to verify the tie in.

Response: Agree. Tie-in was accomplished by adjusting the normal depth friction slope. Tie-in discussion of Delaware with Lower Pecos added to the report and depth rasters provided.

Backcheck: DH- Tie in was verified. Passed

# 5 – Hydraulic Model Execution

## Methodologies / Data Sources (Modeler to populate)

| Unsteady Plan Names |  |  |  |
| --- | --- | --- | --- |
| 10% ACE (10-year): | 10pct | 0.2% ACE (500-year): | 0_2pct |
| 4% ACE (25-year): | 4pct | 1% plus: | 1plu |
| 2% ACE (50-year): | 2pct | 1% minus: | 1min |
| 1% ACE (100-year): | 1pct |  |  |
| Other: |  |  |  |

## Statistical Data (Modeler to populate)

| Time Step: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Courant Value: | Min: 0.5 |  | Simulation Length: | 48 | hours |
| Computation Interval: | 20 | seconds | Model Runtime: | 1-3 | hours |
| Hydrograph Output Interval: | 15 | minutes | Mapping Output Interval: | 15 | minutes |

## Technical Review (Reviewer to confirm with Pass/Fail)

| Execution Review |  |
| --- | --- |
| P | Appropriate 2D parameters are used (Boundary Conditions, Time Step, Simulation Length, etc.) |
| P | Hydrograph Output Interval and Mapping Output Interval set at a reasonable interval to capture peaks |
| P | Model runtime is reasonable (< 12 hours, but dependent upon model size) |

## Review Comments

Modeler to Check Box of Acknowledgement during Response to Comments:

|  | Acknowledge that review comments may be example comments and not a comprehensive of all required revisions. Consultant has reviewed provided comments and reviewed for applicability of comment throughout HUC-8. |
| --- | --- |

- Update this QAQC sheet (table above) to match with the settings in HEC-RAS

Response: Agree. Tables above have been updated.

Backcheck: DH passed

# 6 – Hydraulic Model Results

## Methodologies / Data Sources (Modeler to populate)

| Flow Validation/Gage Analysis |  |
| --- | --- |
| Software: | N/A |
| Methodology | Refer to the BLE report. |
| Removal of Outliers and Justification | N/A |
| Review of Appropriateness of Gage Analysis results: - Check if there is a control structure upstream build in period of record, - Is the period of record sufficient, - Are there any gaps in the period of record, - Q vs A plot included - Wide confidence bands relative to flows? |  |
| Validation Notes (describe parameter adjustments): Refer to the BLE Report |  |
| Other: |  |

## Technical Review (Reviewer to confirm with Pass/Fail)

| Computation Checks |  |
| --- | --- |
| P | Model runs with no cell errors |
| P | Volume balance is within tolerance (ΔV < 1% is ideal, however 2% to 3% can be acceptable for rain on mesh) |
| Results Spot Checks Check at all critical structures, major confluences, a handful of headwater tributaries, downstream end, at benchmarks, and a handful of randomly selected locations. |  |
| P | No visible spikes in WSEL visibly present with a single cell |
| P | WSEL and depth grids appear reasonable (no artificial dips or spikes) |
| P | Visual verification flooding is connected downstream for all significant contributing areas (> 64 acres) |
| P | Velocity grids appear reasonable (no unusually high velocity areas or Courant values greater than 2) |
| P | No crossing profiles between all events |
| P | Inflow and outflow hydrograph timing is reasonable as compared to the peak |
| P | Spills into/from adjacent HUCs appropriately accounted for |
| P | WSEL tie-in between model domains and HUCs along major rivers/streams where inflows are applied |
| Results Full Checks |  |
| P | Computational time step meets ideal Courant condition |
| P | Diffusion Wave: V∆T∆X ≤2 (Up to 5) (Results are unit-less) |
| P | Model simulation is long enough to pass the entire hydrograph through the model (minimum of 12 hours past the peak, preferably 24 hours) |
| P | Hydrographs at internal and external boundaries are smooth |
| P | Contributing areas are contained with 2D mesh or accounted for with inflow hydrographs |
| P | No flow trapped at 2D boundary |
| P | Visual verification mapping is reasonable compared to effective |
| Flow Verification |  |
| P | Peak flow gage analysis performed on all gages in the watershed with sufficient data |
| P | All gages used have an appropriate period of record (10 to 20+ years) |
| P | All gages used have no extensive gaps in record which would impact the analysis |
| P | Gage analysis results validated to other sources (Regression, USGS, etc.) |
| P | Results at applicable benchmarks justified (gages, HWMs, effective BFEs) |
| P | Peak discharges comparable to gages, effective/prior studies, regional regression equations |

## Review Comments

Modeler to Check Box of Acknowledgement during Response to Comments:

|  | Acknowledge that review comments may be example comments and not a comprehensive of all required revisions. Consultant has reviewed provided comments and reviewed for applicability of comment throughout HUC-8. |
| --- | --- |

- Gage analysis not included and could not be verified. Report states that 13B analysis was used, please clarify and/or provide more detailed methodology description to allow for review to be completed.

Response: Agree. A description of the gauge analysis has been added to the Salt Draw report and text has been added to the Delaware report pointing the reader to the Salt Draw report.

Backcheck: DH – Thanks for adding the description! Passed.

- Report includes a Flow Area Relationship graph. The model included 2D Connections at locations other the gages, was regression run at these locations? Is there an excel document that compares the flow directly to gage flows, regression, etc.?

Response: Agree. See response to comment 1.

Backcheck: DH passed

- See Comment 4-1 regarding the flow transfer to the downstream model.

Response: Agree. Text has been added to the tie-in section describing the revised methodology. See response for comment 4-1.

Backcheck: DH – Information added and tie in looks good. Passed.

- Gaps in mapping exist where the channel is defined and there should be mapping. Please review entire model for these issues.

Response: Noted. During the mapping phase, AOMIs will be delineated around these areas indicating that future analysis should be performed to connect the mapping. Doing so in this model, would require excessive breakline extensions and excessively small grid cells, thus impacting model size and runtime.

Backcheck: DH passed

# 6.5 – M1.5 Minor Post Processed Grids

## Review Comments – 8/22/2024

The comments below must be addressed/responded to, and revised M1.5 minor post processed grids and/or other requested files resubmitted (if applicable), in order to allow TWDB to count Independence as “SMA”:

- Provide readme text or documentation on what threshold was used to produce post processed grids. Review of the depth raster suggests that an “inland” threshold was used.

M2 AtkinsRéalis response (10/08/2024): Inland mapping threshold metric was applied for this watershed. A readme file was submitted on 08/28/2024 following initial M1.5 mapping submittal package.

Backcheck: No Read me was found in the supplemental folder. No longer needed. Pass.

- Mapping is present outside of the Delaware HUC8 boundary as shown below. The M1.5 grids should be clipped to the HUC8 boundary.

Response:

M2 AtkinsRéalis response (10/08/2024): We had downloaded the HUC8 boundary from USGS WBD dataset so, our current grid extents appear correct. The USGS HUC8 boundary has changed for the area identified in the comment compared to the version on TNRIS website. Following screenshot shows UGGS WBD HUC8 boundary in green vs older TNRIS HUC8 boundary in red. We clipped the grids to the USGS WDB boundary, so the grid extent is slightly bigger for this area. This comment was resolved via email on 08/27/2024.

Backcheck: DH passed. Resolved via email.

# 7 – Mapping Results

## Technical Review (Reviewer to confirm with Pass/Fail)

| Hydraulic Model |  |
| --- | --- |
|  | HEC-RAS model with results, pertinent data and required storm events |
| Flood Risk Database |  |
| P | All features fully populated per current FEMA guidelines |
| P | All features aligned and clipped to USGS HUC-8 boundary including: -Mapping should extend through entire HUC-8 -No mapping beyond HUC-8 boundary |
| P | Mapping (FLD_HAZ_AR) consistent between depth and WSE grids to polygons for all three required mapped events. The depth grid, water surface elevation grid and floodplain polygon extents must match per FEMA requirements, including the filling of holes, spillways, lake/pooled areas. |
| P | Containment within Events: (10% floodplain contained within 1% and 0.2%) |
| P | Centerline Containment within the 1% floodplain extents |
| P | Holes filled in polygon, depth and WSE grids: Holes Filled within the contiguous floodplain anything less than or equal to 2 acres in the 1- and 0.2% floodplain extents and anything less than or equal to 0.5 acres in the 10% floodplain extents. This should be clearly documented in the report. |
| P | Tie-ins Clarification: Tie-Ins realted to WSEL must be within 0.5ft for the following scenarios: 1) with currently studied HUCs (within the Team), 2) 2D HUCs available on the EBFE viewer, 3) Model Domain Tie-Ins must be 0.5 feet for all reoccurrence intervals 4)FEMA requires water surface elevation tie-ins within 0.5 ft for the mapped event of 10%, 1% and 0.2%. If mapping tie ins cannot be achieved: 1)Adjacent study not completed, 2) Adjacent study completed in 1D, or 3) Adjacent study completed with different modeling and mapping methodology causing tie in achievement to be difficult 1) Need to add AOMI area as AOMI_TYP = NP and AOMI_INFO = Spill area. In need of further engineering analysis. Where complex mapping tie ins between domains/HUCs exist, prior coordination with TWDB at M1 is recommended. (Ex. Braided channels) Please verify for Model Tie-Ins (Flow): o Each event needs to achieve flow continuity at the SA/2D connection. (E.g. Flow hydrographs must match at SA 2D connection between model domains). o A SA/2D connection line may be used to obtain a flow hydrograph between two model domains. The flow hydrograph obtained from the SA/2D connection line should match the Inflow Hydrograph Boundary condition to the downstream model domain. Please verify for Model/ Mapping Tie-Ins (WSEL): o Mapping tie ins (WSEL, Depth) are required for all modeled events across the entire floodplain’s width at one location in the model’s overlap region. This is validated using a raster calculation between both models’ WSEL grids. You must meet the WSEL tie in requirements and the SA/2D connection flow continuity between models. The WSEL tie ins requirement of ±0.5 ft. must be met in all model profiles (10, 25, 50, 100, and 500 yr.) and for 10, 100 and 500-yr for mapping products. Review of fringe mapping tie ins between study HUCs and other adjacent HUCs are within the ±0.5 ft. Both Model domains/HUC boundaries need to be in the ±0.5 ft. |
| P | Waterbodies Added (Lakes/Ponding Areas/Reservoirs) Water bodies of lake and reservoir inundation areas of 20 acres or greater or named (NHD waterbodies dataset) should be merged into final floodplains (FLD_HAZ_AR) and water areas (WTR_AR). |
| P | Boundary interfaces align matching or identified as AOMI |
| P | Centerline Containment (WTR_LN) within 1% floodplain extents |
| P | Schema |
| P | Review that DTL_STUD_AR and DTL_STUD_LN to ensure coverage of the NFHL effective detailed studies. |
| P | Reviewed that BFE feature values against the 1% annual chance event (ACE) water surface elevation (WSE) grid within +/- 1 foot |
| P |  |

## Review Comments

Modeler to Check Box of Acknowledgement during Response to Comments:

|  | Acknowledge that review comments may be example comments and not a comprehensive of all required revisions. Consultant has reviewed provided comments and reviewed for applicability of comment throughout HUC-8. |
| --- | --- |

- Note: Confirmed all centerline containment features in the QAQC database are acceptable

Response: Noted.

Backcheck: N/A

- All feature classes VERSION_ID field should be named as ‘BLE_MMYYYY’ based on the submittal date (note this issue discussed with Atkins on follow-up Toyah call – okay to respond/address consistent with that discussion)

Response: Agree.

Backcheck:  Pass - Based on Toyah call, BLE072021 value is acceptable, EDS

- Please review BFE_2D_Testpoints provided.  14.9% of the test points fail.  This is likely due to steep slope and cupping, but should be reviewed.

Response: The points were reviewed and are due to steep slope and/or cupping.

Backcheck:  Pass, EDS

- Please fill this section out in ITR

Response: Agree. Section for QC Item 7 updated as requested. However, note that this checklist has been converted to the latest template and no longer has the section for QC Item 8.

Backcheck:  Confirmed, EDS

- The Submittal Checklist at top of QAQC document is incomplete for M2 (Section 7/Section 8), please populate fully upon resubmittal.

Response: Agree. See response to Comment #4.

Backcheck:  Confirmed, EDS

# 8 – Completeness Review

## Technical Review (Reviewer to confirm with Pass/Fail)

| File Structure and Naming |  |
| --- | --- |
| P | Deliverables provided in TWDB/FEMA required file folder structure and naming convention |
| F | Data missing (if any) documented in report and ReadMe file within the required folder |
| Report |  |
| P | Technical 2D BLE report documents all assumptions, special considerations, etc. |
| P | Flow validation and difference discussed in report |
| P | Flow comparisons discussed in report |
| Hydrologic Model |  |
| P | HEC-HMS model with results, pertinent data and required storm events |
| Hydraulic Model |  |
| P | HEC-RAS model with results, pertinent data and required storm events |
| Flood Risk Database |  |
| P | All features fully populated per current FEMA guidelines |
| CNMS |  |
| P | Updated correctly with new centerlines |
| P | A1-A5 checks complete |
| P | CNMS Post- Processing Complete |
| HAZUS |  |
| P | Run per FEMA guidance |

## Review Comments

HAZUS

- S_FRAC_Ar  is clipped by the watershed.  According to the Flood Risk Assessment Guidance, exported Hazus data should include any census blocks that intersect the watershed and should not be clipped.

Response: S_FRAC_AR was recreated, new version was not clipped to watershed and all census blocks that intersect the watershed were not clipped.

Backcheck:  Pass, EDS

- Additionally, the census blocks that were considered inventory of the project footprint / watershed, but were not considered a loss, should have their loss populated as -8888 rather than 0.

Response: S_FRAC_AR was recreated, new version have loss fields populated as –8888 in census blocks that were considered inventory of the project footprint / watershed, but were not considered a loss

Backcheck:  Pass, EDS

File Structure and Naming

- Small comment/recommendation to move the “AtkinsRealis_BLE3_Validation_Memo.pdf” to the Reporting and Documentation folder \QC, if more appropriate than under Spatial Data.

Response: AtkinsRealis_BLE3_Validation_Memo.pdf was moved to the QC folder

Backcheck: Validation Memo was not found in the QC folder. No longer needed. Pass

Report

- There are a couple of comments added into the report to address.

Response: All comments in report have been addressed

Backcheck:  All comments and additions Pass, EDS

04/21/2025 - The report also still has track changes on, which should be turned off in the final submittal.

AR response 20250506: All track changes and comments removed from report.

CNMS

- CNMS – S_Studies_Ln is empty

Response: S_Studies_Ln has been populated with S_Studies_Ln approved by MJ.

Backcheck:  Pass, EDS

- FIRM1 and FIRM2 not referenced in any of the submitted feature classes, remove from L_Source_Cit, or update source citations in the feature classes to indicate FIRM1 or FIRM2, if applicable.

Response: Agree. Source citations updated.

Backcheck:  Pass, EDS

- Unclear if CNMS A1-A4 is complete, A5 in the report is incomplete

Response: A1-A4 is complete, and A5 has been completed and approved by MJ since the M2 submittal

Backcheck:  Pass, EDS

Flood Risk Database

- See comments Section 7.

Response: Responses provided in section 7

Backcheck:  Pass, EDS

Terrain

- Source_Index does not fully cover all tiles delivered in ‘Bare_Earth_DEM’ raster. Please add necessary tiles to cover .

AR response 20250506: Source_Index corrected, tiles now cover full extent of ‘Bare_Earth_DEM’ raster

- S_Submittal_Info is much larger than the extents of the Source_Index and raster deliverable in ‘Bare_Earth_DEM’.  Please consider clipping to the revised Source_Index (see comment above)

AR response 20250506: Updated

- S_Submittal_Info should also follow FEMA Data Capture Guidance where TERRAIN CAPTURE tasks should not have overlapping polygons from different sources.

AR response 20250506: Updated

- Remove the NoData.txt file from the Supplemental_Data folder as there are data submissions included.

AR response 20250506: NoData.txt removed

- Metadata comments

  - The <pubdate> in the <citation> and <timeperd> sections should be set to the MIP Submission Date.

AR response 20250506: pubdate updated

  - Update bounding coordinates based on changes made in comments above about Source_Index and S_Submittal_Info

AR response 20250506: bounding coordinates updated

  - <placekey> entries are missing FEMA-CID entries.  When submitting to the MIP, the entries should match the Purchase Geography. If not included in the purchase geography, they should be removed from this section.

AR response 20250506: Updated

  - Replace miphelp@riskmapcds.com in all locations with updated MIPHelp address of fema-riskmap-ithelp@fema.dhs.gov

AR response 20250506: Updated

  - <srccontr> text should be input in the CONTRIB field in L_Source_Cit for each TOPO#

AR response 20250506: Updated

  - In the <eainfo> section, an entry should be added for the Source_Index deliverable

AR response 20250506: Updated

QA/QC Checklist Approval

This 2D Base Level Engineering Analysis QA/QC Review is in compliance with the contract requirements and all task checkpoints are complete.  The independent QC Team has reviewed the analysis, presented review comments to the Production Team Task Leader, and discussed any problems or issues.  The Task Leader, QC Manager, QC Reviewer, and Project Manager have signed the QA/QC Checklist to confirm that all comments have been received, addressed, and documented appropriately.

| QA/QC Team – [TWDB or company name] |  |  |  | Production Team – [add company name] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| QC Manager |  | Date |  | Task Lead |  | Date |
| QC Reviewer |  | Date |  | Project Manager |  | Date |